Acer ISDN 128 Surf USB

User`s Manual

###### Table of Contents

Chapter 1	1

Introduction

1.1 An Overview of the ACER ISDN 128 SURF USB	1

1.1.1 What is ISDN?	1

1.1.2 What is USB?	2

1.1.3 The ACER ISDN 128 SURF USB	2

1.2 Package Contents	4

1.3 ACER ISDN 128 SURF USB Features	4

Chapter 2	6

Installation

2.1 Before Installing	6

2.2 The Front Panel LEDs	7

2.3 Installing the ACER ISDN 128 SURF USB	8

2.4 ISDN Utility Installation	13

2.5 The Diagnostic Program	13

2.6 Using Multilink PPP	14

2.7 Uninstalling the ACER ISDN 128 SURF USB	19

Chapter 3	20

Troubleshooting

Apex A Specifications	24

Apex B Approvals	26

# Chapter 1

Introduction

## 1.1 An Overview of the ACER ISDN 128 SURF USB

### 1.1.1 What is ISDN?

ISDN is an abbreviation for Integrated Services Digital Network. ISDN transfers information digitally and integrates all telecommunication services (such as telephone, teletext, videotex, etc.) into a single network. Using ISDN, you can transfer text, voice, data files, images, and even video.

Before the use of ISDN, telecommunications operated using analog devices, which limited the quality of telecommunication transfers. With ISDN technology, the telephone lines can transmit data digitally with much greater speed and clarity of transmission than with analog transmissions. The ISDN is capable of transmitting all kinds of information at greatly accelerated rates. A typical modem transmission has a rate of 33.6Kbps (kilobits per second or thousand bits per second). ISDN digital technology allows transfer rates of up to 128Kbps.

Because ISDN transmits data digitally, the data is virtually error free, and the transmission is much clearer, with fewer interruptions and slowdowns in facsimile transmissions. With ISDN devices implemented throughout the world, a truly digital network will emerge, allowing everyone fast and easy access to the global information highway.

### 1.1.2 What is USB?

USB is an abbreviation for Universal Serial Bus. It is a new peripheral bus employing a single connector type for all low-speed and medium-speed devices, such as keyboard, mouse, monitor, floppy drive, printer, scanner, digital camera, modem and ISDN TA. It brings plug-and-play of computer peripherals outside the box, eliminating the need to install cards into dedicated computer slots and reconfigure the system. PCs equipped with USB allow computer peripherals to be automatically configured as soon as they are hot attached, without the need to reboot. USB also allows multiple devices, up to 127, to run simultaneously on a computer; some peripherals, such as monitors and keyboards, can act as additional plug-in hubs to make it easy for users to manage peripherals on the desktop. USB does offer unprecedented peripheral expandability and ease of use for PC users.

With better throughput of 12Mbps (megabits per second or million bits per second), USB is expected to rapidly become the preferred means of connecting ISDN TAs, though it will not replace the traditional PC ports (COM and LPT) overnight.

### 1.1.3 The ACER ISDN 128 SURF USB

The ACER ISDN 128 SURF USB is ideally suited to give your PC access to the world of the Integrated Services Digital Network. It is easy to install, and easy to configure because it is fully plug-and-play compatible. The Windows 98 or 2000 operating system will automatically install and configure the ACER ISDN 128 SURF USB.

With the ACER ISDN 128 SURF USB, you can have an answering machine, file transfer, G3/G4 Fax functions and Internet access. This will bring you more versatility and efficiency.

Figure 1-1: The ISDN System

## 1.2 Package Contents

The product package comes with the following accessories:

- The ACER ISDN 128 SURF USB

- The installation diskettes or CD of the ACER ISDN 128 SURF USB

- The installation diskettes or CD of the bundled application software programs

- One RJ-45 cable

- One USB cable

## 1.3 ACER ISDN 128 SURF USB Features

The ACER ISDN 128 SURF USB supports many standard and enhanced features, including the following:

- Fully plug-and-play compatible

- Hot attach and detach without rebooting the PC

- Bus-powered, no extra power adapter

- Supports both 16-bit and 32-bit CAPI drivers (V 2.0)

- Supports TAPI and NDIS WAN miniport drivers

- Supports Multilink PPP (MP) for 128Kbps (two B channels) internet access and data transfer

- Supports X.75, T.70NL, ISO8208, T.90, and HDLC transparent protocols on the B channel for T-Online, AOL, CompuServe, BTX, BBS, Eurofile transfer, and G4 Fax

- Has support for a bit-transparent driver on the B channel for answering machine, G3 Fax and soft-modem functions (with RVS-COM Lite Software)

- Supports both V.110 and V.120 rate adaptation

- Supports COM port simulation for modem-based application programs

# Chapter 2

Installation

## 2.1 Before Installing

Make sure that you have all you need to install the ACER ISDN 128 SURF USB. You should have the following:

- A PC that has USB connector and its USB controller is working properly as shown below.

Figure 2-1: Checking USB Controller in the PC

- Microsoft Windows 98 or 2000.

- At least 10MB of free disk space for the driver and software.

- An ISDN basic rate S0 interface released by a ISDN provider.

## 2.2 The Front Panel LEDs

| LED | Meaning |
| --- | --- |
| USB | When this LED is lit, it indicates that the USB port is connected to the PC and working properly. |
| ISDN | When lit it indicates that the ISDN S0 bus is activated. It will flash when there is message transfer in the D channel. |
| B1 | When lit it indicates that the ISDN “B1” channel is being accessed. It will flash when there is data transfer in the B1. |
| B2 | When lit it indicates that the ISDN “B2” channel is being accessed. It will flash when there is data transfer in the B2. |
| PC | When this LED is lit, it indicates that there is at least one application program using the ACER ISDN 128 SURF USB. |

## 2.3 Installing the ACER ISDN 128 SURF USB

Please refer to the following instructions and figures for installation procedures:

Figure 2-2: Connecting the ACER ISDN 128 SURF USB

- Connect your ISDN line to “ISDN S0” jack on the back of the ACER ISDN 128 SURF USB.

- Connect the USB cable to the “USB” jack on the back of the ACER ISDN 128 SURF USB. Connect the other end to the PC or to a self-powered USB hub device, e.g., monitor.

|  | You should not connect the ACER ISDN 128 SURF USB to a bus-powered USB hub device, such as keyboard, because there could be not enough power for the ACER ISDN 128 SURF USB. |
| --- | --- |

- All the LEDs will be lit and, then, extinguished one by one. Please refer to Chapter 3 Troubleshooting if any LED is not lit or extinguished normally.

- After that, Windows will detect it and pop-up the following screen after “Building driver information database”.

Figure 2-3: Windows detects the ACER ISDN 128 SURF USB

- Press “Next >”, then select “Search for the best driver for your device” and press “Next >” again.

- Insert the Driver Installation Diskette or CD into the appropriate drive (A:, B:, or D:, etc.) and specify the path of the driver.

- Press “Next >” twice. Windows will start copying the CAPI driver files to Windows sub-directories.

- While Windows is installing the NDIS WAN miniport driver, you might see the next screen.

Figure 2-4: Skipping the ISDN Configurations

- You can press “Next >” and “Finish” to skip the 4-page insignificant configurations.

- After Windows finishes installing all the drivers, you will be prompted to restart the computer.

Figure 2-5: Restarting the Computer

- Press “Yes” to restart your computer.

- After PC restarts, you can check the “Device Manager” to see if the ACER ISDN 128 SURF USB has been properly installed.

Figure 2-6: ACER ISDN 128 SURF USB Successfully Installed

- In addition, we recommend you to install the ISDN Utility and run the Diagnostic Program to make sure the ACER ISDN 128 SURF USB, its drivers and the ISDN line are all properly installed and connected. You can refer to the following sections for details.

## 2.4 ISDN Utility Installation

The ISDN Utility includes helpful programs for the ACER ISDN 128 SURF USB, such as diagnostic program, uninstallation program, and on-line manual. To install the ISDN Utility, please follow the instructions listed below.

- Insert the Utility Installation Diskette or CD into drive A: (B:, or D:, etc.).

- For diskette, press “Start” on the task bar and then click “Run”. Type “A:\setup” in the Run Dialog Box and press “OK”.

- The InstallShield Wizard will load the ISDN utility installation program.

Figure 2-7: The ISDN Utility Installation Wizard

- Press “Next>” to begin installing the ISDN Utility. Follow the instructions on the screen to complete the installation.

## 2.5 The Diagnostic Program

After you have installed your ACER ISDN 128 SURF USB, you are highly recommended to use the diagnostic program to make sure the ACER ISDN 128 SURF USB, its drivers and the ISDN line are all properly installed and connected. To run the diagnostic program please follow these instructions:

- Open the “Programs” folder on the “Start” menu and click on the “ISDN Utility” program folder.

- Click on the “Diagnostic Program” item to start the program.

- In the “Own ISDN Number” box, type in the subscriber number of your own ISDN line.

- Press “Loopback Test” to begin the diagnostic procedure. It will make a call from the B1 channel that returns through the B2 channel. The D and B channels will be tested for line integrity.

- Please refer to Chapter 3 Troubleshooting if you see any error or warning message.

Figure 2-8: The Loopback Test

## 2.6 Using Multilink PPP

|  | To have multilink PPP connections, the answering Internet Service Provider (ISP), online service or corporate LAN must provide multilink capabilities. |
| --- | --- |

To configure multilink support, please follow these steps.

- Double click on the “My Computer” icon on your Windows Desktop, then double click on the “Dial-Up Networking” (DUN) folder.

- The DUN folder displays an icon labeled “Make New Connection”, plus icons for each of the connections that you have already created. If you have not yet created a connection, double click on “Make New Connection” to define a connection to the dial up service you wish to use.

Figure 2-9: Creating a Multilink PPP Connection

- When you have created your connection, or if the connection you wish to use for multilink access already exists in the folder, use the mouse to right click on the connection icon. Choose the “Properties” option. This displays a dialog box with several category tabs at the top.

Figure 2-10: Dial-Up Networking Connection Properties

- Select the “Multilink” tab and click on the “Use additional devices” radio button.

- Press “Add…” to specify the device to be bundled together to form the multilink connection.

Figure 2-11: Configuring Multilink Devices

- Use the “Add…”, “Remove”, and “Edit” buttons to change the list of additional devices.

- You may enter a different phone number for each device, and the phone numbers will be stored. If you subsequently change the phone number for the connection icon, the phone numbers associated with additional devices on this page will not change.

After your additional devices are configured, you are ready to dial the multilink connection. Once the connection is established, you can view the status information by double clicking on the “communicating computers” icon displayed in the taskbar.

Figure 2-12: Dial-Up Networking Connection Status

The status information includes the number of bytes sent and received, the network protocols negotiated for use on the connection and a list box showing each of the additional devices. As you highlight a device in the list box, a “Suspend” or “Resume” button is displayed.

If a “Suspend” button is displayed, the additional device is now in use and bundled into the multilink connection. Clicking on the “Suspend” button disconnects that device and removes the additional device from the bundled connections.

If the “Resume” button is displayed, then click on “Resume” to dial that connection and add that additional device to the bundle. You may suspend and resume individual additional device without dropping the connection.

## 2.7 Uninstalling the ACER ISDN 128 SURF USB

The setup information for the ACER ISDN 128 SURF USB is saved by Windows. If you decide to permanently remove the driver and utility of the ACER ISDN 128 SURF USB from your system, you must run the uninstallation program from the ISDN Utility program group as follows:

- Open the “Programs” folder on the “Start” menu and click the “ISDN Utility” item.

- Click the “Uninstall DRV & UTL” item. The uninstallation program will run.

Figure 2-13: The Uninstallation Program

- Check the items that you want to completely remove from the Windows system and press “OK”.

# Chapter 3

Troubleshooting

|  | If your ACER ISDN 128 SURF USB does not work properly, run the Diagnostic Program to help you identify the problem. |
| --- | --- |

| Error Indication | Possible Cause | Remedy |
| --- | --- | --- |
| “USB” LED is not lit or “Fail” on USB interface panel or The CAPI2032.DLL file cannot start. | - The ACER ISDN 128 SURF USB is not ready. | - Make sure the USB cable is securely connected to PC or self-powered USB hub device, and the optional switch on the back of the TA has been turned on. |
|  | - The PC is not USB-ready. | - Examine your PC's hardware and software for USB capability. |
| The B1 LED is lit upon start-up. | - The RAM is out of order | Consult your dealer. |
| The B2 LED is lit upon start-up. | - The ISDN chip is out of order | Consult your dealer. |
| You are not allowed to run this program. | - The CAPI2032.DLL is not from the ISDN package. | Uninstall and then reinstall the ISDN device. |

| Error Indication | Possible Cause | Remedy |
| --- | --- | --- |
| A required .DLL file, CAPI2032.DLL, was not found. | - The CAPI2032.DLL is missing. | Uninstall and then reinstall the ISDN device. |
| CAPI driver is not loaded. | - The Registration key has been destroyed. | Uninstall and then reinstall the ISDN device. |
| Please input your own number | - The subscriber number of your own ISDN line has not been entered. | Key in the subscriber number of your ISDN line in the Own ISDN Number box. |
| Cannot activate S0 interface (“ISDN” LED is always OFF) | - The ISDN line is not connected. | Ensure that the ISDN S0 line is securely connected to the RJ-45 jack. |
| Cannot establish LAPD link | - The ISDN line may be malfunctioning. | Ensure that the ISDN S0 line is in good condition. |
| Connecting fail (error code = xxxx) | - Own ISDN Number is incorrect. | - Ensure that your ISDN number is correct. |
|  | - Other devices on the same S0 line are occupying the B channel. | - Be sure no other device is on the same S0 line and occupying the B channel. |
|  | - The ISDN protocol is wrong. | - Check to see if the ISDN protocol configuration is correct. |
| Data transfer error | The ISDN line is not in good condition | Check the ISDN line |

- Apex A	Specifications

- ISDN Standards: Euro-ISDN

- ISDN Interface: S/T interface, 2B+D

- USB Port

- Bus powered, no extra power adapter required

- “Hot-Plug-In” without rebooting the computer

- Data Protocols : PPP, Multilink PPP, X.75, T.70NL, ISO8208, V.110, V.120, HDLC-transparent and Eurofile Transfer

- Fax Modem function and COM-port emulation 	    (with RVS-COM Lite Software)

- API

- 32-bit WDM CAPI 2.0 driver

- NDIS driver for RAS and Dial-Up Networking

- OS

- Windows 98

- Windows 2000

- Mac OS 8.6 or above

- Operating Temperature: 0 to 50 degrees Celsius

- Relative Humidity: Up to 60%

- Apex B	Approvals

CE Approval

All CE approved ISDN devices may be connected to the Euro-ISDN in all countries of the EU.

The ACER ISDN 128 SURF USB has been approved by the German TÜV according to the new European procedure and may therefore be connected to the Euro-ISDN in all EU countries. The ACER ISDN 128 SURF USB therefore complies with:

- CTR-3 (ISDN Basic Rate Access) - Electromagnetic Compatibility Standards - Safety Standards

CE/EMC Mark Declaration of Conformance

The product complies with requirements of EN50081-1 and EN50082-1 under the scope of EMC Directive 89/336/EEC.