About this Quick Start Guide

Congratulations on your purchase of the AcerISDN T40 or AcerISDN T50. This manual describes both, the AcerISDN T40 and the AcerISDN T50 and,  for the sake of simplicity, refers to them as "T40/T50". What differentiate the T50 from the T40 is its additional 2 analog ports.

The T40/T50 is an external, stand-alone PABX System with an integrated ISDN access device for connecting your computer with the ISDN network from your local telephone company.

It provides a quick and easy "plug-and-play-pathway" to the Internet, to your company intranet or to any commonly known Online Provider like AOL or CompuServe with data transmission rates of up to 128 Kbit/s.

The T40/T50 is a complete PABX System (Private automatic branch exchange) providing 3 (T40) or 5 (T50) analog phone ports. You may connect any of your analog telephone equipment, e.g. analog phones, answering machines, fax machines or analog modems. The integration of an easy-to-use PABX System with an ISDN access device offers you a double advantage: you will profit from all modern ISDN features like call waiting and conferencing while maintaining to use standard analog devices.

Included in the delivery package you will find installation diskettes or a CD with "Acer Global Communication Office". This application will assist you to configure the T40/T50 interactively.

- Features

- Overview of the Installation

- Making a Physical Connection

- Installing the Driver

- Installing the Global Communication Office

- Internet Access

The following signs are used within the text:

This symbol appears where important advice, that you should follow at all costs, is given.

This symbol appears in front of additional information and hints.

A. Features

Analog Ports: There are 2 (T40) or 3 (T50) main ports and 1 (T40) or 2 (T50) auxiliary ports to which you can connect "traditional" analog devices like a phone, an answering machine, a fax machine, or an extra modem.
Intercom: All connected devices may communicate with each other without charge. (Port 21 vs. 22 and Port 31 vs. 32 are exclusive, please refer to section 1.4)
LCD Display: One-lined customized icons and two-lined display showing any necessary information, for example the cost of a call or Calling Line ID Presentation (CLIP, transmission of the caller's phone number)
Acer Global Communication Office: A simple application which includes online help and different comfortable options of file transfer and PABX system management.
Least Cost Routing: allows to take advantage of the most inexpensive phone company's services automatically at any specific time. This feature is not available in all countries, please contact the telephone company for details.

Logbook: The logbook keeps track of the last 10 incoming, outgoing and unanswered phone calls. The date, time and phone number of the caller (presuming he uses CLIP) are listed here.
Data Call Back: A caller may be called back immediately in order to take over his charges.
Call Forwarding: Any call may be forwarded to any other internal or external device.
Call Waiting, Call holding, Call Transfer, 3-Party-Conference: You can accept and manage more than one phone call at the same time on a single telephone.
Advice Of Charges / Display of Charges: Charges may be viewed for every single call or device.
Call Filter: Unwanted callers can be kept from being able to connect to any device.
Short Dial: Numbers that are frequently used and very important may be stored and activated using key combinations on your phone.
Special Ring: A special kind of ring may be assigned to chosen callers.
Morning Call: Any device can be programmed separately to ring at a certain time.
Internet Protocols: Uncomplicated internet access using an online service or a provider.
Automated Protocol Recognition: The connection protocol (X.75, PPP, V.120 and V.110) of incoming data call is recognized automatically.
Multilink Protocol: By combining 2 ISDN B-channels, the T40/T50 can achieve transfer rates of 128 KBit/s.
Configuration With Keypads: You can control most of your T40/T50's functions using your phone's keypad.

Two implementations of supplementary services (Call Waiting, Call Forward, Call Transfer, 3-party Conference, Advice of Charge listed above) by ISDN Switch, functional protocol and keypad protocol respectively. It is different country by country. The operation procedures and the display of Global Communication Office related to Call Forward, Charge Information described in chapter 3 conform to functional protocol only. For the countries that apply keypad protocol, you can use “*77” to be the prefix (pickup the handset, hear the dial-tone, press *77, hear the second dial tone) and then press specified keys to enable or disable specified features. Please contact the telephone company for the subscription and related information.

B. Overview of the Installation

Rear Panel

1  2          3    4  5       6

- Power switch (ON/OFF)

- Power Connector (DC only)

- RS-232, Serial Connector to the Serial Port of your computer

- ISDN Connector

- DIP Switch for the Terminating Resistor

- Analog ports, from left to right:

- Port 11

- Port 21

- Port 22

- Port 31 (only T50)

- Port 32 (only T50)

Front Panel

1                 2    3

- LCD Display

- Status LED (is ON if the T40/T50 power button is pressed)

- Push button for controlling the LCD Display

The installation steps :

- Make physical connection

- Connecting the T40/T50 to computer

- Connecting your analog equipment

- Connecting the ISDN line

- Connecting the T40/T50 to the power supply

- Install the Drivers and Global Communication Office

C. Making a Physical Connection

First Step: Connecting the T40/T50 to your Computer

Take the serial cable and connect it to the Serial Port of your computer. Connect the other end of the cable to connector nb3 on the rear panel.

Second Step: Connecting your analog equipment

The T40 has three analog ports for telephones, fax machines or any other analog device. The T50 has 5 analog ports. A distinct MSN (phone number) can be assigned to each of the analog ports .

Please keep in mind, that each channel may only be used by one phone device at the same time. 
So if, for example, someone uses a phone on port 21, port 22 will be unavailable. Port 21 is considered to be the so-called main port, while Port 22 is it's affiliated auxiliary port. The same limitation applies to Port 31 and Port32. Thus, Port 22 and Port 32 should be used particularly with less important devices, like a second fax machine or an analog fax modem which is only used part of the time. Nevertheless, if 5 distinct phone numbers (MSNs) have been assigned, each of the 5 phones may function as an ISDN telephone and will be accessible separately.

You may connect your analog devices to any analog  port of the T40/T50. Please keep in mind that Port 21 and Port 22 (as well as Port 31 and Port 32) can't be used at the same time. This limitation is due to the fact that the channels are being shared, using a technique called multiplexing.

Third step: Connecting the ISDN line

Please adjust the Termination Resistor (section 4.1) first if you would like to connect more than one ISDN access device to the same ISDN connector.
Take the ISDN cable out of the package. Connect the ISDN connector of the T40/T50 to the ISDN connector (NT-1 or NTBA) installed by your local telephone company.

Fourth step: Connecting the T40/T50 to the Power supply

Make sure the T40/T50 is turned OFF. Place the T40/T50 on a stable and secure surface. Connect the power adaptor to the power connector of the T40/T50. Under no circumstances should another power adaptor be used! Please use only the power adaptor provided with the T40/T50. Otherwise your T40/T50 might be severely damaged

C. Installing the Driver

Plug-and-Play for Windows 95, OSR2 or 98:

- Power on T40/T50 and then power on the computer. Windows will tell you that new hardware has been detected. You will see the followg screen for Windows 95 (Take T50 for example) :

Or you will see the following screen for OSR2 (Take T50 for example):

Or you will see the following screen for Windows 98 (Take T50 for example):

- Select "Driver from disk provided by hardware manufacturer" for Windows 95 or "<Next>" for OSR2 and Windows 98.

- Insert the Diskette (or CD) into the appropriate drive, select "\WIN95" as the source directory, click "OK" to start the installation.

- Now, the T40/T50 driver is installed for the COM port to which it is connected to.

- Installing through the Control Panel for Windows NT 4.0, 95, OSR2 or 98

- For Windows NT 4.0, you must be a member of the administrator group before you are allowed to install a new modem.

- For Windows 95/OSR2/98, follow the procedure below if you want to install the modem manually rather than through Plug-and-Play.

- Connect the T40/T50 to the serial port on your computer.

- Turn on the power to the T40/T50 and your computer.

- Click on Start, Settings, then Control Panel.

- Double-click Modem and click on Add.

- In the Install New Modem dialog box, check Don’t detect my modem; I will select it from a list.  Click Next.  Then click the Have Disk button.

- Insert the T40/T50 Diskette (or CD) into the appropriate disk drive.  If you are using Windows 95/OSR2/98, select "\WIN95".  If you are using Windows NT 4.0, select "WINNT40". Then click OK.

- Install the driver that met your communication environment, for example, if you want to use PPP for the ISP connection, please select “AIT-334S (PPP, 64K)” for T50 or “AIT-324S (PPP, 64K)” for T40. Then click Next.

The drivers listed in the Install New Modem screen refers to to different software drivers for the modem protocol profiles that you can set in Dial-Up Networking.

- Choose the COM port to which the AcerISDN T40/T50 is attached and click Next.

- When prompted, continue the installation by providing information such as country code, area code, etc.  Then click Next.

- Click Finish to complete the modem installation process.

- Select Close.

- Installing for Windows NT 3.51

- Before installation, make sure that the Remote Access Services (RAS) component is already installed under Windows NT 3.51. If not, please refer to the Windows NT 3.51 documentation to add the RAS component.

- Change to the directory containing the RAS (i.e., \WINNT35\SYSTEM\RAS). This directory should contain the MODEM.INF file.

- Append the contents of WINNT35.INF in the T40/T50 Diskette (or CD) to the end of MODEM.INF.

- After configuring RAS, you will find "AcerISDN T50" for T50 or "AcerISDN T40" for T40 listed in the modem listing.

- For detail on how to configure RAS, please refer to relevant Windows NT 3.51 documentation.

Installing for Windows 3.1

- Basically, there is no specific T40/T50 driver to install in Windows 3.1 environment. You might use Windows’ 3.X Terminal program to test if you can "Talk" to the ISDN modem.

- Go into Program Manager and open the Accessories group.

- Click on Terminal.

- Select Communications.

- Set the Baud rate (speed).

- Select the Connector (COM port) that you have connected the modem to.

- Click on OK to exit Communication.

- In Terminal, Type AT and press Enter.

- The modem should response OK.

E. Installing the Global Communication Office

Follow the instructions below, to install the application "Acer Global Communication Office", which will enable you to configure your AcerISDN T40/T50, on to your hard disk. The installation process is easy:

- Turn the T40/T50 on.

- Start your computer.

- Insert the installation diskette or the installation CD.

- For Windows 95, 98 and NT users, please run "setup.exe". For Windows 3.1, and 3.11 users, please run "setup16.exe".

- Quit the installer soon as the installation process is accomplished.

If the T40/T50 is connected correctly and turned ON, the status LED should be ON. 
If not, please check all cables and the power switch. Refer to the Troubleshooting (section 3.20) in order to find a possible solution to the problem.

Then, refer to the Examples (chapter 2) if you would like to get accustomed to the PABX and ISDN terminology or if you would like to read about some real-world environments where the T40/T50 is being used. 
Otherwise, you may double-click the "Acer Global Communication Office" and go straight to the Configuration (chapter 3) in order to set up your T40/T50 step by step and to configure it according to your needs.

F. Internet Access

The following dialog boxes will guide you how to set up "Dial-Up Networking" for Windows 95.

- Open Dial-up Networking in the My Computer folder.

- Open Make New Connection in Dial-up Networking.

- Click Next in the following frame.

- Please give a name to your Internet Connection.

- Select AIT-334S (PPP, 64K or Multilink PPP, 128K) for T50 or AIT-324S (PPP, 64K or Multilink PPP, 128K) for T40. Then click Next in the following frame.

- Enter the access number provided by your Internet Service Provider (ISP). If two different numbers are assigned (one for each B-channel) and a MP (128 Kbps) call is desired, enter the phone numbers as phone#1&phone#2.

- Enter the country code and click Next.

- Confirm the Setting before clicking Finish as follows.

- Return to the Dial-Up Networking folder. Click on the AcerISDN Connection icon and select Properties as follows:

- Take a look at the General folder of the AcerISDN Connection.

- Select the Server Types folder to set up the dial-up server as follows:

- Click TCP/IP to set up the IP.

- Normally, your ISP will assign a dynamic IP address when you access it. If you get a specific IP from your ISP, select Specify an IP address and key in the address.

- Frequently, your ISP will provide a specific address for Domain Name Server (DNS). Please enter this address. If your ISP provides you a dynamic DNS address to the contrary, please choose "Server assigned name server addresses".

Now, all you have to do to connect to your ISP is to invoke your newly created Dial-up Networking application and enter your user name and password when prompted. Once the connection is established, you can start your favorite Internet browser. Enjoy exploring the Internet!