# MPT OpenServer Driver Release Notes

# Version 1.03.28

# 8/22/2003

Release Notes and Restrictions:

- All Raid Volumes must be powered on and operational during bootup.  A Raid Volume can not be “Hot” added during normal operation.

Major Changes For Version 1.03.28:

General Changes

Functionality

- Check Nvram data and limit Domain Validation activities if requested.

Defect fixes

- Accommodate disk drives with capacity greater than 64 GB.

Major Changes For Version 1.03.27:

General Changes

Functionality

- Support for the 1035 operating in SCSI mode was added.

Defect fixes

- “Smart Host” recognition added.

Major Changes For Version 1.03.26:

General Changes

Functionality

- None.

Defect fixes

- Firmware upload on both channels of an onboard 1030 is now done.  This allows the firmware to utilize that memory for Message Frames, greatly increasing performance on both channels.

Major Changes For Version 1.03.25:

General Changes

Functionality

- None.

Defect fixes

- Commented out a debug message when the number of outstanding commands reaches its maximum when trying to start a driver queued command.

Major Changes For Version 1.03.24:

General Changes

Functionality

- None.

Defect fixes

- Multi-processor locks were added to one of the IOCTL routines before returning data to the application and releasing driver resources.  This prevents a hang condition on MP machines with SMP and ServeRaid running in a high intensity and sustained IO environment.

Major Changes For Version 1.03.23:

General Changes

Functionality

- None.

Defect fixes

- The ChainOffset field is explicitly zeroed in Message Frames used during Domain Validation, Event requests and Config requests.

Major Changes For Version 1.03.22:

General Changes

Functionality

- None.

Defect fixes

- The MsgFlags field is explicitly zeroed in Message Frames used during Domain Validation and Config requests.

- Device structure fields are initialized only during memory allocation, preserving previous values.  This is especially important during drive failure and hot spare utilization with Integrated Mirroring.

Major Changes For Version 1.03.21:

General Changes

Functionality

- None.

Defect fixes

- For ioctl calls that involve a RAID_SCSI_IO_PASSTHROUGH function, the PhysDiskId and PhysDiskBus numbers are obtained from the message frame rather than from the ioctl argument (that argument does not contain the correct values).  The corresponding SCSI Id and Bus numbers associated with this request are then obtained from IOC Page 3.

Major Changes For Version 1.03.20:

General Changes

Functionality

- None.

Defect fixes

- For ioctl calls that involve a SCSI_IO function, the id and bus numbers are obtained from the message frame rather than from the ioctl argument (that argument does not contain the correct values).

- Increased retry count from 3 to 9 when retries are desirable.

Major Changes For Version 1.03.19:

General Changes

Functionality

- None.

Defect fixes

- Commands are not retried if the device is a Sequential Device (Tape).

- The MOST byte in PCI Config Space is not modified if the 1030 revision number is C0 or later.

Major Changes For Version 1.03.18:

General Changes

Functionality

- None.

Defect fixes

- The “tunable” parameter mpt_TapeCommandTimeout is set for 24 Hours to allow sequential device commands to complete before being timed out and the bus reset.

- When a DV_NEEDED event occurs, IOC Page 3 will be read to obtain the current Raid configuration.  When the IOC Page 3 read is completed, the SCSI Id and Bus associated with the Physical Disk is obtained and Domain Validation is performed on that device.

Major Changes For Version 1.03.17:

General Changes

Functionality

- None.

Defect fixes

- A tunable parameter has been added to the space.c file.  This parameter, mpt_TapeCommandTimeout, contains the timeout value, in seconds, used with each command sent to a Sequential Device (usually tape drives).  The value is currently set for 1800 seconds (30 minutes) to allow those commands to complete before being timed out and the bus reset.

Major Changes For Version 1.03.16:

General Changes

Functionality*

- None.

Defect fixes

- The FWImageSize in IOCFacts will only be accessed when Bit 0 of the Flags field is set, indicating the host must upload the firmware image using the FWUpload request.  This FWImageSize will be used to allocate the memory for the FW upload.  Only one FWUpload will be done even though both channels will have Bit 0 set of the Flags field set.

Major Changes For Version 1.03.15:

General Changes

Functionality

- None.

Defect fixes

- Removed an extraneous WAIT when an event is received requesting that DV be performed as a result of a drive replacement in a Raid Volume.

Major Changes For Version 1.03.14:

General Changes

Functionality

- None.

Defect fixes

- Added code to check for Inactive Raid Volumes.

Major Changes For Version 1.03.13:

General Changes

Functionality

- None.

Defect fixes

- For Raid Devices, a DV Quiesce PhysDisk request is now issued before any other SCSI IO command, including a Test Unit Ready.  This is necessary for drive replacement in a Raid Volume during normal operation.

- A single routine was made for reading all the Config pages.  That routine is now in a common code file and is called by both the UnixWare and OpenServer drivers, for consistent operation.

Major Changes For Version 1.03.12:

General Changes

Functionality

- None.

Defect fixes

- Some Compaq disk drives report ANSI-Approved Version as 5 in their Inquiry data; the driver was changed to look at all 3 bits of this field so that this disk drive now operates properly.

Major Changes For Version 1.03.11:

General Changes

Functionality

- None.

Defect fixes

- The device type (byte 0 of Inquiry data) is saved for each device.  The  driver now checks that type and, if a Check Condition occurs with Unit Attention on a Sequential-access device, that sense data is sent back to the OS rather than retrying the command.  This prevents corrupt data from being on the tape when a Bus Reset occurs during a tape backup.  The type is also placed in bits 31-24 (previously an unused byte) of Target Info data during that Ioctl request.

Major Changes For Version 1.03.10:

General Changes

Functionality

- None.

Defect fixes

- The device’s Control field (containing flags for tagged/untagged and disconnecting/non-disconnecting) is OR’ed with the application’s IO request Control field before sending to the firmware.  This retains the device’s operating settings during Ioctl operations, allowing consistent device operation.

Major Changes For Version 1.03.09:

General Changes

Functionality

- None.

Defect fixes

- The location of the ioctl wait flag setting was changed to prevent a possible “forever” wait during ioctl processing.

- Memory allocations and frees during ioctl processing were changed to prevent a possible memory exhaustion situation from happening.

Major Changes For Version 1.03.08:

General Changes

Functionality

- None.

Defect fixes

- Added device structure and pointer for the HBA’s Id to allow IOCTL’s for this Id to be processed in the normal command flow.

Major Changes For Version 1.03.07:

General Changes

Functionality

- None.

Defect fixes

- Fixed a SMP locking problem when a request is received for a device

that was not present during the initialization device scan.

- Removed a redundant call to free Request frames if unable to obtain a chain buffer for SGE operation.

Major Changes For Version 1.03.06:

General Changes

Functionality

- None.

Defect fixes

- Fixed Domain Validation to allow Ultra320 and Ultra160 operation.

Major Changes For Version 1.03.05:

General Changes

Functionality

- None.

Defect fixes

- Fixed SCSI-1 devices to operate as untagged only.

- Added support for 929X and 919X.

Major Changes For Version 1.03.04:

General Changes

Functionality

- Added tunable default command timeout value.

Defect fixes

- None.

Major Changes For Version 1.03.03:

General Changes

Functionality

- Changed default command timeout period from 5 to 30 seconds.

Defect fixes

- None.

Major Changes For Version 1.03.02:

General Changes

Functionality

- None.

Defect fixes

- Return EFAULT if IOCTL request if for a non-existent device.

- Fixed potential problem of entering an invalid id into a “SMART HOST” event log.

Major Changes For Version 1.03.01:

General Changes

Functionality

- None.

Defect fixes

- Added Case statement for MPI_FUNCTION_SCSI_ENCLOSURE_PROCESSOR to process its reply from FW.  This removes the request from the running queue, preventing a timeout occurrence, and the application is notified of its completion.

- Added loop to search through the running queue in the switch statement’s default code (function number not present in case statement), looking for a request to match this reply.  If found, a message will be issued and the request will be removed from the running queue to prevent a timeout occurrence, and the application will be notified of the request’s completion.

- Modified location of where physical address of IOC Page 4 for each channel is saved.  This eliminates having the same physical address for both channels, allowing IOC Page 4 writes for channel B to done correctly.  This allows the IBM ServeRaid application to properly access SEP data when a SEP is attached to channel B of a 1030.

Major Changes For Version 1.03.00:

General Changes

Functionality

- None.

Defect fixes

- Added Case statement for MPI_FUNCTION_EVENT_ACK to process its reply from FW.  This removes the request from the running queue and prevents a timeout occurrence.

##### VERSION 1.01.00

Major Changes From Version 1.00.5:

General Changes

Functionality

- None.

###### Defect fixes

- Changed ProcessNonIOReply to explicitly check for all Functions that should be processed in this routine.  This was to prevent unsolicited and/or unexpected replies from entering the queue deletion area of code.

##### VERSION 1.00.53

Major Changes From Version 1.00.40:

General Changes

Functionality

- Added support for Integrated Mirroring and SEP devices.

Defect fixes

- Numerous changes to checkout the above features.

### NOTES, ISSUES and DETAILS

## No known issues.