# LSI CIM Solution Release Notes

# (Windows x86, x86_64, IA64)

# Version 3.06.00

# 08/01/2005

Defect Fixes:

| Issue | CIM does not recognize the lsi_scsi.sys driver and hence does not see any HABs or devices |
| --- | --- |
| Reported by | SW Test Lab (CR cospt#1766) |
| Steps to reproduce | Install a 1030 controller on Win2k3 running lsi_scsi.sys driver and run CIM Browser to view the controller and devices |
| Description of change | Updated LsiNative.dll to add support for STORPORT driver lsimpt.sys and split drivers lsi_scsi.sys and lsi_fc.sys. The CIM Browser requires LsiNativ.dll version 2.1_2.02.11 or newer |

| Issue | CIM does not see any devices or HABs when FC is in system |
| --- | --- |
| Reported by | SW Test Lab (CR cospt#1764) |
| Steps to reproduce | Install a 1030 as well as a FC in the same system and try starting CIM Browser to view the controllers and devices |
| Description of change | There was difference in PropertyDataType of ‘NetworkType’from MOF 2.5 to MOF 2.7. This has been fixed to use the latest data type as specified in MOF 2.7 |

| Issue | CIM does not display the controller if there is not at least one device attached to it |
| --- | --- |
| Reported by |  |
| Steps to reproduce | Install a 1030 controller with no devices attached to it. Run CIM Browser to try to view the controller |
| Description of change | With drivers that does not support a pseudo device, CIM Browser does not display controllers without any devices attached. With STORPORT and newer SCSI Port drivers that support pseudo devices, CIM Browser has been fixed to list controllers without any devices attached to it |

| Issue | The SCSI Address of the controller is wrongly displayed when there are no devices attached to it |
| --- | --- |
| Reported by | Self |
| Steps to reproduce | Install a 1030 controller without any devices, run CIM Browser and click on the controller without any devices. The ‘SCSI ID’ on the right hand tab shows -1 instead of the actual value. This requires a fix to list controllers without devices to be able to reproduce |
| Description of change | The ‘InitiatorId’ of the controller was populated through the device cache and this happens only when there is at least one device (or IR volume) is present. The Provider has been modified to set the ‘InitiatorId’ property in the SCSIController cache so the Browser can query for this field irrespective of if there is a device or not |

General Notes

- Please refer to MPT IR SCSI Firmware Release Notes for Version 01.03.11.00, as this document also applies to Integrated RAID.

- NVSRAM is required to run the Integrated Mirroring and/or Integrated Mirroring Enhanced firmware.

- The CIM Solution package by default installs both Provider and Browser.  The Provider package is needed on the system under management and the Browser package allows viewing of configuration information on local and remote systems.

Major Features

- Integrated RAID (IM, IS and IME) support, based on MPT 1.2 Specification.

- Hot Spare support for IM and IME RAID Volumes based on MPT 1.2 Specification.

- Inactive IR (IM, IS and IME) Volume support based on MPT 1.2 Specification.

- SMART error information for (real and not physical) disk drives is supported.

- Integrated Striping support, based on MPT 1.2 Specification.

- Context-sensitive on-line help is supported.

- A system tray icon is loaded to reflect the status of CIM

- Automatic pop-up of error messages from LSICIMProvider Service

- Management of Fibre Channel and SCSI within CIM Solution Package.

- Enhanced Provider support for all SCSI devices.  Event reporting to OS specific event logger.

- Enhanced Browser display resizing, error, status information and layouts.

- Browser is able to display configurations on remote systems. A browser on a Windows system is able to see configurations on Linux systems, and vice-versa.

I/O Protocol Devices Supported

- CIM Browser is designed to support LSI Ultra320 SCSI (SCSI1020/1030) and LSI Fibre Channel (FC929) controllers

Hardware Platforms

- This package of CIM Solution will run on any x86, x86_64 and IA64 platforms

Release Restrictions

- Installing the LSI CIM Solution requires the system have its video card set to 256 colors or greater.

- Installation to a network-shared device is unsupported.

- Multi-Lun devices do not always show media capacity information correctly.

- The Matrox MGI G4+M4A16DG Video Card is not supported.

- MediumChanger devices (Peripheral Device Type 8) are not supported.

- If you change the IP address or the hostname of a host or client system you must restart both the system running the provider and the system running the browser.

##### The LSI CIM Solution will not support SAFTE/SES processor devices.

##### Changes from 3.03.13.50

##### Fixed CIMBrowser update issue involving an IM Volume and HotSpare. If either primary/secondary drive fails, HotSpare kicks in as secondary, and the failed drive needs to be shown as failed. This information was not getting updated previously without a browser restart.

##### Changes from 3.03.13

##### When LSICIMProvider service is started with any kind of SAF-TE device connected to the host, it logs an event to the Application Log stating “Device does not respond to CIMProvider commands” against event ID: 6 unnecessarily and this has been fixed in this drop. With this new drop, user should not see this message in the event log and LSICIMProvider should gracefully throw away this SAF-TE target and not support it at all.

##### Changes from 3.03.12

##### None

##### Changes from 3.03.11

- When logging events in Windows on discovering a RAID Volume, NT Service was crashing due to a memory leak in underlying native dll and this has been fixed.

- Support for SAFTE processor devices has been disabled.

##### Changes from 3.03.10

- IS Volume was not logging events into the application log (in Windows) has been fixed.

### Known Issues

- Problem: If LSI CIM Solution is uninstalled with the CIMBrowser application kept opened, some of the jar files might get locked due to the uninstallation process trying to uninstall the files that were being used by CIMBrowser.

Workaround: Please close all the windows/dialogs that are opened in the system and then try uninstalling the application.

- Problem: Uninstallation will fail on some Windows NT systems. This is due to the PSAPI.dll file missing in the Windows NT installation (this file is included in most NT installations).

Workaround: Please download and install the PSAPI.dll file from the Microsoft website and then try uninstalling LSI CIM Solution. You may also manually stop the TaskBarIcon process through the Task Manager and ignore the error message about the PSAPI.dll file on uninstall.

- Problem: If a new F/w image is downloaded onto the card while CIM is running by using F/w download utility, CIMBrowser is not updating the F/w version number being reported for the HBA.

Workaround: Please restart the LSICIMProvider running as a service or as a daemon in Windows or Linux respectively.

- Problem:  PCI Slot Number attribute under HBA panel would report the right value only if the F/w image running on the LSI HBA is based on MPT 1.2 Addendum specifications, if not CIMBrowser is expected to return a value of 0.

Workaround: Upgrade the F/w image on the controller to the latest.

Detailed Feature Description

- Reporting of SMART Error

As per the SPC-3 SCSI Draft provided by www.t10.org, Informational Exceptions Control mode page – through which we extract the SMART errors encountered by a regular disk drive – shall apply only to informational exceptions that report an additional sense code of FAILURE PREDICTION THRESHOLD EXCEEDED (an asc value of 0x5d) or WARNING (an asc value of 0x0b) to the application client. Based on this information, only if the additional sense code is one of the above two values, it is considered to be a SMART error for all of our reporting purposes and not if otherwise.

If a SMART error is encountered by any of the devices after the LSICIMProvider Windows Service has started, it should be picked up by the CIMBrowser application in its next refresh-cycle (default is 4 minutes) and by the system tray icon application - “CIM Status Monitor” – in its next polling cycle (default is 8 minutes) and LSICIMProvider Windows Service should also pop-up an error message in its next polling cycle (default is 8 minutes).

- System Tray Icon

A system tray icon is also loaded to reflect the status of CIM and this application shall be installed in the StartUp directory of the OS. This application is intended to run in tandem with LSICIMProvider (running as a Windows Service) and in case of the Service being stopped due to some reason, the behavior of this application is not defined. The system tray icon also supports a mouse right-click pop-up menu item to “Launch the CIMBrowser” and to “Exit” this icon from the system tray. In selecting the latter menu item, LSICIMProvider will continue to run as a Windows Service and LSI CIMBrowser will not be closed, in case it is opened. In addition to the right-click, the system tray icon also supports mouse left-click, which should pop-up a message box describing the current status of CIM application. This pop-up message is localized in various languages same as in which the CIMBrowser application is localized. Any encountered warning/error condition in any of the connected devices should be reflected in the color of the system tray icon no later than the next polling cycle (default is 8 minutes).

- Automatic Pop-Up Error Messages

In the case of CIMBrowser application not yet launched and just the LSICIMProvider Service is up and running, automatic pop-up of error messages, incase of any warning/critical conditions detected for any of the devices associated with the controller, is supported. A SMART error encountered by a device is considered to be a critical condition for reporting purposes.

Since this automatic pop-up error message comes from the LSICIMProvider Windows service, this is not localized in various languages and only “English” text message is supported.

If any of the devices has encountered a warning/critical condition and if the CIMBrowser application is kept opened and if a manual-refresh or an auto-refresh action takes place in CIMBrowser, CIMBrowser might report the encountered warning/critical condition immediately after the refresh-action is completed but for the LSICIMProvider Windows Service and for the system tray icon application to report the same encountered warning/critical condition, it might take until the next polling cycle and the default is 8 minutes.