# LSI CIM Solution Release Notes

# Version 3.03.13.50

# 09/16/2003

#### General Notes

- Please refer to MPT IR SCSI Firmware Release Notes for Version 01.03.11.00, as this document also applies to Integrated RAID.

- NVSRAM is required to run the Integrated Mirroring and/or Integrated Mirroring Enhanced firmware.

- The CIM Solution package by default installs both Provider and Browser.  The Provider package is needed on the system under management and the Browser package allows viewing of configuration information on local and remote systems.

Major Features

- Integrated RAID (IM, IS and IME) support, based on MPT 1.2 Specification.

- Hot Spare support for IM and IME RAID Volumes based on MPT 1.2 Specification.

- Inactive IR (IM, IS and IME) Volume support based on MPT 1.2 Specification.

- SMART error information for (real and not physical) disk drives is supported.

- Integrated Striping support, based on MPT 1.2 Specification.

- Context-sensitive on-line help is supported.

- A system tray icon is loaded to reflect the status of CIM

- Automatic pop-up of error messages from LSICIMProvider Service

- Management of Fibre Channel and SCSI within CIM Solution Package.

- Enhanced Provider support for all SCSI devices.  Event reporting to OS specific event logger.

- Enhanced Browser display resizing, error, status information and layouts.

- Browser is able to display configurations on remote systems. A browser on a Windows system is able to see configurations on Linux systems, and vice-versa.

Release Restrictions

- Installing the LSI CIM Solution requires the system have its video card set to 256 colors or greater.

- Installation to a network-shared device is unsupported.

- Multi-Lun devices do not always show media capacity information correctly.

- The Matrox MGI G4+M4A16DG Video Card is not supported.

- MediumChanger devices (Peripheral Device Type 8) are not supported.

- If you change the IP address or the hostname of a host or client system you must restart both the system running the provider and the system running the browser.

- The LSI CIM Solution will not support SAFTE/SES processor devices.

### Known Issues

- Problem: If LSI CIM Solution is uninstalled with the CIMBrowser application kept opened, some of the jar files might get locked due to the uninstallation process trying to uninstall the files that were being used by CIMBrowser.

Workaround: Please close all the windows/dialogs that are opened in the system and then try uninstalling the application.

- Problem: Uninstallation will fail on some Windows NT systems. This is due to the PSAPI.dll file missing in the Windows NT installation (this file is included in most NT installations).

Workaround: Please download and install the PSAPI.dll file from the Microsoft website and then try uninstalling LSI CIM Solution. You may also manually stop the TaskBarIcon process through the Task Manager and ignore the error message about the PSAPI.dll file on uninstall.

- Problem: If a new F/w image is downloaded onto the card while CIM is running by using F/w download utility, CIMBrowser is not updating the F/w version number being reported for the HBA.

Workaround: Please restart the LSICIMProvider running as a service or as a daemon in Windows or Linux respectively.

- Problem:  PCI Slot Number attribute under HBA panel would report the right value only if the F/w image running on the LSI HBA is based on MPT 1.2 Addendum specifications, if not CIMBrowser is expected to return a value of 0.

Workaround: Upgrade the F/w image on the controller to the latest.

Detailed Feature Description

- Reporting of SMART Error

As per the SPC-3 SCSI Draft provided by www.t10.org, Informational Exceptions Control mode page – through which we extract the SMART errors encountered by a regular disk drive – shall apply only to informational exceptions that report an additional sense code of FAILURE PREDICTION THRESHOLD EXCEEDED (an asc value of 0x5d) or WARNING (an asc value of 0x0b) to the application client. Based on this information, only if the additional sense code is one of the above two values, it is considered to be a SMART error for all of our reporting purposes and not if otherwise.

If a SMART error is encountered by any of the devices after the LSICIMProvider Windows Service has started, it should be picked up by the CIMBrowser application in its next refresh-cycle (default is 4 minutes) and by the system tray icon application - “CIM Status Monitor” – in its next polling cycle (default is 8 minutes) and LSICIMProvider Windows Service should also pop-up an error message in its next polling cycle (default is 8 minutes).

- System Tray Icon

A system tray icon is also loaded to reflect the status of CIM and this application shall be installed in the StartUp directory of the OS. This application is intended to run in tandem with LSICIMProvider (running as a Windows Service) and in case of the Service being stopped due to some reason, the behavior of this application is not defined. The system tray icon also supports a mouse right-click pop-up menu item to “Launch the CIMBrowser” and to “Exit” this icon from the system tray. In selecting the latter menu item, LSICIMProvider will continue to run as a Windows Service and LSI CIMBrowser will not be closed, in case it is opened. In addition to the right-click, the system tray icon also supports mouse left-click, which should pop-up a message box describing the current status of CIM application. This pop-up message is localized in various languages same as in which the CIMBrowser application is localized. Any encountered warning/error condition in any of the connected devices should be reflected in the color of the system tray icon no later than the next polling cycle (default is 8 minutes).

- Automatic Pop-Up Error Messages

In the case of CIMBrowser application not yet launched and just the LSICIMProvider Service is up and running, automatic pop-up of error messages, incase of any warning/critical conditions detected for any of the devices associated with the controller, is supported. A SMART error encountered by a device is considered to be a critical condition for reporting purposes.

Since this automatic pop-up error message comes from the LSICIMProvider Windows service, this is not localized in various languages and only “English” text message is supported.

If any of the devices has encountered a warning/critical condition and if the CIMBrowser application is kept opened and if a manual-refresh or an auto-refresh action takes place in CIMBrowser, CIMBrowser might report the encountered warning/critical condition immediately after the refresh-action is completed but for the LSICIMProvider Windows Service and for the system tray icon application to report the same encountered warning/critical condition, it might take until the next polling cycle and the default is 8 minutes.