C51GM03-3.1 BIOS SPECIFICATION

Rev 0.8

Dec 08, 2005

| APPROVED | CHECKED | PREPARED |
| --- | --- | --- |
| Sean Chang | Sean Chang | Rex zhang |

- Content

.1	Product Summary	4

.2	BIOS Requirement	4

.2.1	 Standard BIOS Feature	4

.2.3	 Power Management	5

.2.4	 Language Support	6

.2.5  Logo for Specific Customer	7

.2.6  Hardware Monitor and PC Health Status Features	7

.3	BIOS Configuration Method	8

.4	Customer’s Features	8

.4.1	Preload Features	8

.4.2	SOW for DMI	9

.5	GPIO Definitions	10

.6	IRQ Routing Table	11

.7	BIOS Setup Options	12

- Scope

This document is a comprehensive listing of C51GM03-3.1 BIOS specific requirements for nVidia NForce Crush51 chipset product. The BIOS specification provides the project team with the information necessary to understand and design the product, and plan for project management. It provides a basis to plan for implementation, sales, and support.

- Revision History

| Revision | Revision History | Date |
| --- | --- | --- |
| 0.1 | Preliminary Release. | 2005/11/04 |
| 0.2 | Change ACPI Power LED state to ACER Aspire M6 | 2005/11/16 |
| 0.3 | Change LAN Chip to Marvell 88E1115 | 2005/11/29 |
| 0.4 | Change “Single Channel” to “Dual channel”. | 2005/11/30 |
| 0.5 | Add “System Fan Smart Fan Control” item. Change WOR wake up from S1 to S1/S3/S4 | 2005/12/01 |
| 0.6 | Remove “shutdown temperature” function. Delete "Manufacturing utility support: Dump/ Download CMOS table utility". Modify wake-up event: S3 RTC: Disabled. S4 PS/2: Keyboard: Disabled. | 2005/12/02 |
| 0.7 | Change PME(LAN) wake up from “by driver” to Disable. | 2005/12/07 |
| 0.8 | Change Power button / PS/2 K/B / USB K/B wake up from “Yes” to “Enable” | 2005-12-8 |

## Product Summary

The C51GM03 desktop board is a performance business desktop board with the following product features:

- μATX compliant form factor

- Supports the following AMD processors:

  - AMD Athlon 64/FX processors

  - AMD (Socket-939Pin) family processors

  - AMD (Socket-939Pin) dual core processors

  - HT speed up to 2000MT/s

- nVidia C51G-MCP51 chipset

- Winbond or SST 4M flash ROM(FWH)

- System memory:

- Dual channel four DIMM sockets

- Supports DDR266 /DDR333/DDR400 (4GB memory capacity)

- IDE intefrace

- Two IDE channels

- 2 Ultra DMA33/66/100/133 port supported

- SATA supported

- 4 1.5G SATA

- PCI-Express interface

- 1 PCI-Express x16 slot

- PCI interface

- 3 PCI request-grant pairs for support of 3 PCI Bus Masters

- Complies to PCI Specification Rev. 2.3

- Up to 8 USB Ports (Rear 4 USB + 3 Pin Header, 3rd pin header share with 2 ports under 1394)

- Audio on board, UAJ support (HD audio feature support on rear only) integrated audio Realtek ALC850

- ITE 8712F LPC super I/O with Hardware monitor supported.

- Marvell Gigabit Ethernet Transceiver, 88E1115 LAN

- Integrated 1394

- ACPI 1.0b, APM 1.2, PnP 1.0a, SMBIOS 2.3, WFM 2.0, USB 1.1, USB 2.0 Compliance.

- PC 99, PC99A and PC2001 Compliance

- WHQL certificated

- AMD Cool and Quiet

## BIOS Requirement

### .2.1		Standard BIOS Feature

- Phoenix Award BIOS

- Support AMD K8/939 Althon64FX, Althon64, Sempron processor detection and initialization

- AMD dual core processor detection and initialization

- nVidia C51G-MCP51 chipset initialization modules.

- Auto-detect keyboard and mouse

- Auto-detect of off-board serial/parallel ports, floppy ports, IDE controllers with auto-configuration of on-board devices

- Auto-detect of IDE drive type installed

- High performance IDE controller support with ATA-5 features, including LBA mode for large hard drives, advanced PIO modes 3 & 4 for faster PCI IDE disk access. BIOS will automatically configure drives for highest performance. Performance options set up on a per-drive basis

- Int 13h extensions with 48 bits LBA to support drives larger than 136.9GB

- Ultra DMA 33 Mode and Ultra ATA66/100/133 (33,66,100,133MB/s)

- ATAPI support for IDE CD-ROM and DVD-ROM drives

- LS-120 removable media support

- USB 1.1 and USB 2.0 compliance support (with support for legacy input devices)

- ACPI 1.0b compliance

- APM 1.2 compliance

- SMBIOS 2.3 compliance

- Wire For Management 2.0 (WFM) support

- Plug and Play 1.0a specification, support for Windows ‘95

- PCI add-in card auto-configuration, PCI 2.2compliance

- AC power loss state retention management

- User and administrator mode for system setup screens with 2 separate password-protected accesses.

- Maximum of 4GB system memory

- Four DIMM slots are provided for same memory configurations

- BIOS will not initialize installed memory greater than 4GB

- Single or double-sided DIMM configurations

- Mixed speed DIMM configuration will default to the slowest speed DIMM installed.

- .2.2		Device Boot Support

- FDD Boot <if installed>

- HDD Boot

- CDROM Boot

- Remote Network boot (PXE 2.0)

- USB (removable media) boot

- USB Flash Boot (Support USB-HDD, USB-FDD, USB-ODD and USB-ZIP standard)

### .2.3		Power Management

- Support APM and ACPI modes and all devices standby states as are listed bellow (Note S4 wake is OS dependent):-

| Devices | S1 | S3 | S4 | S5 |
| --- | --- | --- | --- | --- |
| Power Switch | Enable | Enable | Enable | Enable |
| PS2 Keyboard | Enable | Enable | Disable | Disable |
| USB Devices | Enable | Enable | Disable | no |
| PME(LAN) | Disable | Disable | Disable | Disable |
| RTC | Disable | Disable | Disable | Disable |
| PME(Ring Com Port) | Disable | Disable | Disable | no |

- ACPI LED Status State

- Bezel LED Status State – AcerPower M6 and Aspire E360

| LED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power LED |  |  | HDD LED | LAN LED | ODD LED |
| S0 | S1, S3 | S4, S5 |  |  |  |
| Blue | Blue Blinking | OFF | Blue Blinking | Blue Blinking | Blue Blinking |

- Bezel LED Status State – Aspire T160

| LED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Power LED |  |  | HDD LED | LAN LED | ODD LED |
| S0 | S1, S3 | S4, S5 |  |  |  |
| Green | Green Blinking | OFF | Red | Green | N/A |

### .2.4		Language Support

- English language support only.

### .2.5  Logo for Specific Customer

### .2.6  Hardware Monitor and PC Health Status Features

- Hardware Monitor and Fan Speed Control Implementation in BIOS PC Health Status Menu

(these Items will be updated later)

| Voltage | Description |
| --- | --- |
| Voltage CPU Vcore | The voltage of CPU core |
| Voltage 1.50V PCIExpress | PCI Express VDD 1.5V voltage |
| Voltage 3.30V Main Power | +3.3V voltage |
| Voltage 5.00V Main Power | +5V voltage |
| Voltage 12.0V Main Power | +12V voltage |
| Voltage 3.30V Standby | Standby 3.3V voltage |
| Voltage 2.60V Standby | Memory Vcc 2.6V voltage |
| Voltage 5.00V Standby | Standby 5V voltage |

| Temperature | Description |
| --- | --- |
| CPU Core Temperature | Current CPU core temperature |
| System Temperature | Current system temperature |

| FAN Speed | Description |
| --- | --- |
| CPU Fan Speed | Current CPU fan speed |
| Chipset Fan Speed | Current Chipset fan speed |
| System Fan Speed | Current system fan speed |

## BIOS Configuration Method

- Press DEL key to enter BIOS Setup Screen, F12 for Boot Menu

## Customer’s Features

### Preload Features

To meet customer pre-installation CD features, system BIOS implement functions as below specification.

4.1.1. INT 15 support

| BIOS Lock | DMS Solution |  |
| --- | --- | --- |
|  | INT15, Function AH=C0H Input AH=C0H BL=00H CX=4341H DX=5245H Output If function not support or function fail CY=1 AH=86H ELSE CY=0 AH=00H ES : The string segment address BX : The string offset address The string should be “ACR” and it address should be 16 byte boundary of F segment. |  |
|  | Other Out-Sourcing | To be combine the BIOS lock and Windows XP SLP in the same spec. |
|  | Use the same spec. describe in SLP section |  |
| SLP | For DMS Desktop/Server and Other OEM’s Desktop/Server/Notebook | 8K range, to shorten the search time to get better performance |
|  | Solution 1 “AcerSystem” and “ACR” string in BIOS 0xFE000H~0xFFFFFH |  |
|  | Solution 2 “AcerSystem” and “ACR” string in BIOS 0xF0000H~0xFDFFFH | 48K range, in case OEM customer can’t meet the first requirement |

4.1.2. BBS Support

Change boot sequence function for utilities

|  | The BIOS boot specification for utilities under 16-bit OS is to support the BIOS Boot Specification Version 1.01, which has been made available to the public and is the property of Compaq Computer Corporation (“Compaq”), Phoenix Technologies Ltd (“Phoenix”), and Intel Corporation (“Intel”). The 16-bit OS is DOS, Win95, Win98, or WinME. First of all, the system software (BIOS) must be a Plug and Play BIOS. That is, the Plug and Play installation check structure must be supported by system BIOS so utilities can get the entry point to these PnP BIOS functions. Pease refer Plug and Play BIOS Specification Version 1.0A for the details. Second, the system BIOS has to implement functions 60h through 66h defined in BIOS Boot Specification. Function 60h Get Version and Installation Check Function 61h Get Device Count Function 62h Get Priority and Table Function 63h Set Priority Function 64h Get IPL Device from Last Boot Function 65h Get Boot First Function 66h Set Boot First |  |
| --- | --- | --- |

### SOW for DMI

The purpose of this document is to provide the location in the BIOS area for DMI programming. It also contain the DMI information that to be programmed.

All these information get from DMI data area.

Please refer to the table below for the DMI programming info.

| Type | Component ID | Name | Max. Length | Description |
| --- | --- | --- | --- | --- |
| System Information | Manufacturer name |  | Standard |  |
| System Information | Product name | Product name | Standard | String 1 |
| System Information | Serial Number | System S/N | Standard | String 2 |
| Base Board | Manufacturer name |  | Standard |  |
| Base Board | Product name | Main Board ID | Standard | String 3 |
| Base Board | Serial number | Main Board S/N | Standard | String 4 |
|  |  | System BIOS Version |  | V6.0 |
|  |  | SMBIOS version |  | 2.3 |

DMI Programming Information:

Base Board Manufacturer 	= Acer

Base Board Product	 	= FC51GM

Base Board Version		= (Empty and just write a NULL terminator)

Base Board Serial Number	= (Follow on board Serial Number label)

System Manufacturer 		= Acer

System Product	 		= AST160/ASE360/APM6

System Version			= (Empty and just write a NULL terminator)

System Serial Number		= (Follow on board Serial Number label)

### Acer e-recovery SUPPORT

1. Check Alt+F10

AX=9610h

BL=18h

Return:

Carry = 0 => Successful

1 => Error

CX = 0 => Alt + F10 is pressed

1 => Alt + F10 is not pressed

Notice: Bios need set CX value when Alt+F10 is pressed

2. Check system resume from hibernation

AX=9610h

BL=15h

Return:

Carry = 0  => successful

= 1  => Error

CX = 0  => Resume from Hibernation

= 1  => others

If MBR check CX=0 , Not allow into hidden partition (press Alt+F10 no function)

3. Set BIOS boot sequence

AX=9610h

BL=30h

BH=00h => A -> C -> CD-ROM -> LAN

BH=01h => C -> CD-ROM -> LAN -> A

BH=02h => CD-ROM -> LAN -> A -> C

BH=03h => LAN -> A -> C -> CD-ROM

Return:

Carry = 0 => Successful

1 => Function not support or the selected device does not exist

## GPIO Definitions

| PIN NO | PWR WELL | USAGE | RESET | S3/S4/S5 | NOTES |
| --- | --- | --- | --- | --- | --- |
| NF4 C51GM03 |  |  |  |  |  |
| GPIO1 | +5.0V | N/A, TP | ----- | ----- |  |
| GPIO2 | +5.0V | Onboard LAN control | ----- | ----- |  |
| GPIO3 | +3.3V_DUAL | BIOS Write Protect | ----- | ----- |  |
| GPIO4 | +3.3V_DUAL | N/A | ----- | ----- |  |
| GPIO5 | +3.3V_DUAL | One button recovery | ----- | ----- |  |
| GPIO6 | +3.3V_DUAL | GPIO6 | ----- | ----- |  |
| GPIO7 | +3.3V_DUAL | LID# | ----- | ----- |  |
| GPIO8 | +3.3V_DUAL | GPIO8 | ----- | ----- |  |
| GPIO9 | +3.3V_DUAL | GPIO9 | ----- | ----- |  |
| GPIO10 | +3.3V_DUAL | GPIO10 | ----- | ----- |  |
| GPIO11 | +3.3V_DUAL | GPIO11 | ----- | ----- |  |
| GPIO12 | +3.3V_DUAL | GPIO12 | ----- | ----- |  |
| GPIO13 | +3.3V_DUAL | GPIO13 | ----- | ----- |  |
| GPIO14 | +3.3V_DUAL | GPIO14 | ----- | ----- |  |
| GPIO15 | +3.3V_DUAL | GPIO15 | ----- | ----- |  |
| GPIO16 | +3.3V_DUAL | GPIO16 | ----- | ----- |  |
| GPIO17 | +3.3V_DUAL | GPIO17 | ------ | ----- |  |
| GPIO18 | +3.3V_DUAL | GPIO18 | ------ | ------ |  |
| GPIO19 | +3.3V_DUAL | SPIO | ------ | ------ |  |
| GPIO20 | +3.3V_DUAL | RGMII_PWRDWN | ----- | ----- |  |
| GPIO21 | +3.3V_DUAL | GPIO21 |  |  |  |
| GPIO22 | +3.3V_DUAL | USB_OC1# | ----- | ----- |  |
| GPIO23 | +3.3V_DUAL | USB_OC2# | ----- | ----- |  |
| GPIO24 | +3.3V_DUAL | USB_OC3# | ----- | ----- |  |
| GPIO25 | +3.3V_DUAL | USB_OC4# | ----- | ----- |  |
| GPIO26 | +3.3V_DUAL | USB_OC5# | ----- | ----- |  |
| GPIO27 | +3.3V_DUAL | AC_SDATA_IN1 | ----- | ----- |  |
| GPIO28 | +3.3V_DUAL | SIO_PME# | ----- | ----- |  |
| GPIO29 | +3.3V_DUAL | EXT_SMI# | ----- | ----- |  |
| GPIO30 | +3.3V_DUAL | RI# | ----- | ----- |  |
| GPIO31 | +3.3V_DUAL | SUSCLK | ----- | ----- |  |
| GPIO32 | +3.3V_DUAL | AGP_PME# | ----- | ----- |  |
| GPIO33 | +3.3V_DUAL | EE_SEL | ----- | ----- |  |
| GPIO34 | +3.3V_DUAL | EE_CLK | ----- | ----- |  |
| GPIO35 | +3.3V_DUAL | EE_DATAO | ----- | ----- |  |
| GPIO36 | +3.3V_DUAL | EE_DATAI | ----- | ----- |  |
| GPIO37 | +3.3V_DUAL | PCI_PME# | ----- | ----- |  |
| GPIO38 | +3.3V_DUAL | USB_OC6# | ----- | ----- |  |
| GPIO39 | +3.3V_DUAL | USB_OC7# | ----- | ----- |  |
| GPIO40 | +3.3V_DUAL | SMB_ALERT# | ----- | ----- |  |
| GPIO41 | +3.3V_DUAL | SMB_CLK0 | ----- | ----- |  |
| GPIO42 | +3.3V_DUAL | SMB_DATA0 | ----- | ----- |  |
| GPIO43 | +3.3V_DUAL | SMB_CLK1 | ----- | ----- |  |
| GPIO44 | +3.3V_DUAL | SMB_DATA1 | ----- | ----- |  |
| GPIO45 | +3.3V | PCI_REQ4# | ----- | ----- |  |
| GPIO46 | +3.3V | PCI_GNT4# | ----- | ----- |  |
| GPIO47 | +3.3V | PCI_PERR# | ----- | ----- |  |
| GPIO48 | +3.3V | PCI_INTE# | ----- | ----- |  |
| GPIO49 |  |  | ----- | ----- |  |
| GPIO50 | +3.3V | A20GATE | ----- | ----- |  |
| GPIO51 | +3.3V | SPDIF | ----- | ----- |  |
| GPIO52 | +3.3V | FANRPM | ----- | ----- |  |
| GPIO53 | +3.3V | FANCTL0 | ----- | ----- |  |
| GPIO54 | +3.3V | FANCTL1 | ----- | ----- |  |
| GPIO55 | +3.3V | BUF_14MHZ | ----- | ----- |  |
| GPIO56 | +3.3V | CPU_SLP# |  |  |  |
| GPIO57 | +3.3V | SLP_S1# |  |  |  |
| GPIO58 | +3.3V | THERM# |  |  |  |
| GPIO59 | +3.3V | THERMTRIP# |  |  |  |
| GPIO60 | +3.3V | KBRDRSTIN# |  |  |  |
| GPIO61 | +3.3V | PCI_REQ3 |  |  |  |
| GPIO62 | +3.3V | PCI_GNT3 |  |  |  |
| GPIO63 | +3.3V | PCI_REQ5 |  |  |  |
| GPIO64 | +3.3V | PCI_GNT5 |  |  |  |

## IRQ Routing Table

|  | PCI1 | PCI2 | PCI3 | PCI4 | PCI5 | LAN |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INTA | C | A | D |  |  |  |  |  |  |  |
| INTB | D | B | A |  |  |  |  |  |  |  |
| INTC | A | C | B |  |  |  |  |  |  |  |
| INTD | B | D | C |  |  |  |  |  |  |  |
| INTE |  |  |  |  |  |  |  |  |  |  |
| INTF |  |  |  |  |  |  |  |  |  |  |
| INTG |  |  |  |  |  |  |  |  |  |  |
| INTH |  |  |  |  |  |  |  |  |  |  |
| REG/GNT | 4 | 2 | 1 |  |  |  |  |  |  |  |
| IDSEL | AD26 | AD24 | AD22 |  |  |  |  |  |  |  |

## BIOS Setup Options

- Full BIOS setup options are listed in Excel attachment bellow.

| Menu | Field | Display | Options | Default |
| --- | --- | --- | --- | --- |
| Product Information |  |  |  |  |
|  | Product Name | Show Only |  | AST160/ASE360/APM6 |
|  | Main Board ID | Show Only |  | FC51GM |
|  | System S/N | Show Only |  | N/a |
|  | Main Board S/N | Show Only |  | N/a |
|  | System Manufacture Name | Show Only |  | Acer |
|  | MB Manufacture Name | Show Only |  | Acer |
|  | System BIOS Version | Show Only |  | Customer determine |
|  | SMBIOS Version | Show Only |  | 2.3 |
|  | System BIOS ID | Show Only |  | Customer Determinie |
|  | BIOS Release Date | Show Only |  |  |
| Standard CMOS Features |  |  |  |  |
|  | Date (mm:dd:yy) | Show |  | N/a |
|  | Time (hh:mm:ss) | Show |  | N/a |
|  | IDE Channel 0 Master | Show |  | N/a |
|  | IDE Channel 0 Slave | Show |  | N/a |
|  | IDE Channel 1 Master | Show |  | N/a |
|  | IDE Channel 1 Slave | Show |  | N/a |
|  | IDE Channel 2 Master | Show |  | N/a |
|  | IDE Channel 3 Master | Show |  | N/a |
|  | IDE Channel 4 Master | Show |  | N/a |
|  | IDE Channel 5 Master | Show |  | N/a |
|  | Floppy Drive A | Show | None/ 360K , 5.25 in/ 1.2M , 5.25 in./ 720K , 3.5 in/ 1.44M, 3.5 in/ 2.88M, 3.5 in. | 1.44M 3.5in |
|  | Video | Show | EGA/VGA/ CGA 40/ CGA 80/ MONO | EGA/VGA |
|  | Halt On Setting | Show | All Errors/No Errors/All, But keyboard/All, But Diskette/All, But Disk/ key | All, But keyboard |
|  | Base Memory | Show only |  | N/a |
|  | Extended Memory | Show only |  | N/a |
|  | Total Memory | Show only |  | N/a |
| Advance BIOS Features |  |  |  |  |
|  | Hard Disk Boot Priority | Show | Press Enter |  |
|  | Virus Warning | Show | Enabled / Disabled | Disabled |
|  | Quick Power On Self Test | Show | Disabled / Enabled | Enabled |
|  | First Boot Device | Show | Floppy / LS120 / HardDisk / CDROM / ZIP100 / USB-FDD / USB-ZIP / USB-CDROM / Legacy LAN / Disabled | Floppy |
|  | Second Boot Device | Show | Floppy / LS120 / HardDisk / CDROM / ZIP100 / USB-FDD / USB-ZIP / USB-CDROM / Legacy LAN / Disabled | Hard Disk |
|  | Third Boot Device | Show | Floppy / LS120 / HardDisk / CDROM / ZIP100 / USB-FDD / USB-ZIP / USB-CDROM / Legacy LAN / Disabled | CDROM |
|  | Boot From Other Device | Show | Disabled/Enabled | Enabled |
|  | Swap Floppy Drive | Show | Disabled / Enabled | Disabled |
|  | Boot Up Floppy Seek | Show | Disabled / Enabled | Disabled |
|  | Boot Up NumLock Status | Show | Off/On | On |
|  | Gate A20 Options | Show | Normal/Fast | Fast |
|  | Typematic Rate Setting | Show | Disabled / Enabled | Disabled |
|  | Typematic Rate (Chars/Sec) | Show | 6/8/10/12/15/20/24/30 | 6 |
|  | Typematic Delay (Msec) | Show | 250/500/750/1000 | 250 |
|  | Security Options | Show | Setup/System | Setup |
|  | APIC Mode | Show | Disabled/Enabled | Enabled |
|  | MPS Version Control For OS | Show | 1.1/1.4 | 1.4 |
|  | OS Select For DRAM > 64MB | Show | Non-OS2/OS-2 | Non-OS2 |
|  | Silent Boot | Show | Disabled / Enabled | Enable |
|  | Small Logo (EPA) Show | Show | Disabled / Enabled | Disabled |
|  | Configuration Table | Show | Disabled / Enabled | Disabled |
|  | Bootblock write Protect | Show | Disabled / Enabled | Enable |
| Advanced Chipset Features |  |  |  |  |
|  | Dual Monitor Support | show | Disabled/Enabled | Disabled |
|  | Frame Buffer Size | Show | 16M/32M/64M/128/Disabled | 64M |
|  | CPU Spread Spectrum | Show | 200.0-300.0 | Down Spread |
|  | Err94 Enh | Show | auto/diabled | auto |
|  | Spread Spectrum | Show | Disabled/Enabled | Enabled |
|  | HT Spread Spectrum |  | Disabled/Center/Down Spread | Disabled |
|  | SSE/SSE2 Instructions | Show | Disabled/Enabled | Enabled |
|  | System BIOS Cacheable | Show | Disabled/Enabled | Enabled |
| Integrated Peripherals |  |  |  |  |
|  | IDE Function Setup | Show | Press Enter |  |
|  | OnChip IDE Channel0 | Show | Disabled / Enabled | Enabled |
|  | Primary Master PIO | Show | Auto/Mode 0/ Mode 1/ Mode 2/ Mode 3/ Mode 4 | Auto |
|  | Primary Slave PIO | Show | Auto/Mode 0/ Mode 1/ Mode 2/ Mode 3/ Mode 4 | Auto |
|  | Primary Master UDMA | Show | Disabled/Auto | Auto |
|  | Primary Slave UDMA | Show | Disabled/Auto | Auto |
|  | OnChip IDE Channel1 | Show | Disabled / Enabled | Enabled |
|  | Secondary Master PIO | Show | Auto/Mode 0/ Mode 1/ Mode 2/ Mode 3/ Mode 4 | Auto |
|  | Secondary Slave PIO | Show | Auto/Mode 0/ Mode 1/ Mode 2/ Mode 3/ Mode 4 | Auto |
|  | Secondary Master UDMA | Show | Disabled/Auto | Auto |
|  | Secondary Slave UDMA | Show | Disabled/Auto | Auto |
|  | IDE DMA transfer access | Show | Disabled/Enabled | Enabled |
|  | Serial-ATA 1 | Show | Enabled/Disabled | Enabled |
|  | Serial-ATA2 | Show | Enabled/Disabled | Enabled |
|  | IDE Prefetch Mode | Show | Disabled / Enabled | Enabled |
|  | IDE HDD Block Mode | Show | Disabled / Enabled | Enabled |
|  | SATA PORT Speed Settings | Show | Auto/Force GEN I/Force GEN II | Auto |
|  | Onboard Device Setup | Show | Press Enter |  |
|  | OnChip USB | Show | Disabled/ V1.1+V2.0/V1.1 | V1.1+V2.0 |
|  | USB Memory Type | Show | SHADOW/Base Memory(640K) |  |
|  | USB Keyboard Support | Show | Disabled / Enabled | Enabled |
|  | USB Mouse Support | Show | Disabled / Enabled | Enabled |
|  | AC97 Audio | Show | Disabled/Auto | Auto |
|  | MAC Lan | Show | Disabled/Auto | Enabled |
|  | MAC Lan Boot ROM | Show | Enabled/Disabled | Disabled |
|  | Onboard I/0 Chip Setup | Show | Press Enter |  |
|  | Onboard FDC Controller | Show | Disabled / Enabled | Enabled |
|  | Onboard Serial Port 1 | Show | Disabled/ 3F8/IRQ4 / 2F8/IRQ3 / 3E8/IRQ4 / 2E8/IRQ3 / Auto | 3F8/IRQ4 |
|  | Onboard Serial Port 2 | Show | Disabled/ 3F8/IRQ4 / 2F8/IRQ3 / 3E8/IRQ4 / 2E8/IRQ3 / Auto | 2F8/IRQ3 |
|  | UART Mode Select | Show | Normal/IrDA/ASKIR/SCR/ | Normal |
|  | UR2 Duplex Mode | Show only | Half | Half |
|  | Onboard Parallel Port | Show | Disabled/ 378/IRQ7 / 278/IRQ5 / 3BC/IRQ7 | 378/IRQ7 |
|  | Parallel Port Mode | Show | SPP/EPP/ECP/ECP+EPP | SPP |
|  | ECP Mode Use DMA | Show only | 3 | 3 |
| Power Management Setup |  |  |  |  |
|  | ACPI Function | Show | Enabled/ Disabled | Enabled |
|  | ACPI Suspend Type | Show | S1(POS)/S3(STR)/S1&S3 | S3(STR) |
|  | Power Management | Show | User Define / Min Saving / Max Saving | User Define |
|  | Video Off Method | Show | Blank Screen / V/H SYNC+Blank / DPMS Support | DPMS Support |
|  | HDD Power Down | Show | Disabled / 1 min / 2 min / 3 min / 4 min / 5 min / 6 min / 7 min / 8 min / 9 min / 10 min / 11 min / 12 min / 13 min / 14 min / 15 min | Disabled |
|  | HDD Down In Suspend | Show | Disabled / Enabled | Disabled |
|  | Soft-Off by Power Button | Show | Delay 4 Sec/Instant-Off | Delay 4 Sec |
|  | WOL(PME#) From Soft-off | Show | Disabled / Enabled | Disabled |
|  | WOR(RI#) From Soft-off | Show | Disabled / Enabled | Disabled |
|  | USB Resume from S1/S3 | Show | Disabled / Enabled | Disabled |
|  | Resume by Alarm | Show | Disabled / Enabled | Disabled |
|  | Day of Month Alarm | Show | 1-31 | 0 |
|  | Time (hh:mm:ss) Alarm | Show | 0-23:0-59:0-59 | 0:0:0 |
|  | Power ON Function | Show | Any KEY/BUTTON ONLY | BUTTON ONLY |
|  | PWRON After PWR-Fail | Show | Off/On/Former-Sts | Former-Sts |
| PnP/PCI Configurations |  |  |  |  |
|  | Init Display First | Show | PCI Slot/ PCIEx | PCIEx |
|  | Reset Configuration Data | Show | Disabled / Enabled | Disabled |
|  | Resources Controlled By | Show | Auto (ESCD) / Manual | Auto (ESCD) |
|  | IRQ Resources | Show | Press Enter |  |
|  | IRQ-5 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | IRQ-7 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | IRQ-9 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | IRQ-10 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | IRQ-11 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | IRQ-14 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | IRQ-15 assigned to | Show | PCI Device/Reserved | PCI Device |
|  | PCI/VGA Palette Snoop | Show | Disabled / Enabled | Disabled |
|  | Maximum Payload Size | Show | 128/256/512/1024/2048/4096 | 4096 |
| PC Health Status |  |  |  |  |
|  | Shutdown Temperature | Show | 80℃/176ºF; 85℃/185ºF; 90℃/194ºF; 95℃/203ºF | 90℃/194ºF |
|  | CPU Vcore | Show | Show only |  |
|  | +3.3V | Show | Show only |  |
|  | +5V | Show | Show only |  |
|  | +12V | Show | Show only |  |
|  | +5VSB | Show | Show only |  |
|  | Voltage Battery | Show | Show Only |  |
|  | Current CPU Temperature | Show | Show Only |  |
|  | Current SYSTEM Temperature | Show | Show Only |  |
|  | CPU Fan Speed | Show | Show Only |  |
|  | System Fan Speed | Show | Show Only |  |
|  | CPUFan SmartFan Control | Show | Enabled / Disabled | Enabled |
|  | System Fan Smart Fan Control | Show | Enabled/ Disabled | Enabled |
| Load Default Settings | Show |  |  |  |
| Set Supervisor Password | Show |  |  |  |
| Set User Password | Show only |  |  |  |
| Save & Exit Setup | Show |  |  |  |
| Exit Without Saving | Show |  |  |  |