Acer Incorporated

Acer ViiV

Aspire M5700

ME Firmware & System BIOS

Update SOP

Date: Oct, 2008

This document contains proprietary technical information, which is the property of the Acer Incorporated and shall not be disclosed to others in whole or in part, reproduced, copied, or used as the basis for design, manufacturing, or sale of apparatus without written permission of Acer Incorporated.

The information in this document, including products and software versions, is current as of the Release Date above. This document is subject to change without notice.

Prerequisite

  - Unplug AC Power Cable

  - Disable ME by short ‘MFG jumper’ on main board

(Please check jumper setting label on side cover of chassis)

In [DOS Mode]

  - Power on System and Boot to DOS

  - Key in ‘cd DosTool’  (Go to BIOS path: A://DosTool/)

  - Key in ‘flash4m.bat’

  - Press ‘enter’ and start to flash ME & System BIOS

  - Reboot system and Press "del" to enter BIOS setup menu.

  - Load default setting, and Save & Exit.

In [Windows Mode]

  - Run Dos command line as Administrator in Windows.

  - Go to WinTool folder in Dos prompt.

  - Key in ‘flash4mw.bat’.

  - Press ‘enter’ and start to flash ME & System BIOS

  - Reboot system and Press "del" to enter BIOS setup menu.

  - Load default setting, and Save & Exit.