Chausie_Viiv Project BIOS update SOP(From P24to 68AA1P25.BIN)

- Replug the AC power cord and power on the system.

- Press “Del” to enter AwardBIOS CMOS setup utility and load default settings.

3.Press “Alt+F4” once. Then select “Advanced BIOS Features”

4.Select “Bootblock Protection”.

5.Change the to “Disabled” for “Bootblock Protection”.

6.Save the setting and exit.

7.Boot to DOS or Windows XP.

8.If boot to DOS mode, run “flash.bat” in “DOS” folder to flash BIOS.

9.If boot to Windows mode, run”flash.bat” in “Windows” folder to flash BIOS.

10.After the flash process completed, shutdown and remove AC power cord once.

11.Replug the AC power cord and power on the system.

12.Press “Del” to enter AwardBIOS CMOS setup utility and load default settings.