ES1938/1941/1946 PCI AudioDrive®

# for Windows NT 4.0

# Version 4.05.15

# Software Release Notes

Dec. 21, 1998

# Contents

Contents	1

About This Release	2

Release Information	2

Release Description	2

KNOWN PROBLEMS	2

# About This Release

ESS Technology is providing a new Windows NT® driver for the ES1938 PCI AudioDrive® audio chip product in this release.

# Release Information

Product:	ES1938 PCI AudioDrive® for Windows NT® 4.0 Version 4.05.15

Drivers:	ESS reference design of ES1938/1941/1946

OS:	Microsoft Windows NT® 4.0 or later

Release Date:	Dec. 21, 1998

# Release Description

New Changes in Version 4.05.15

· Fixed suspend/resume hibernation.

Version 4.05.14

·        Fixed installation failure because of CPE bit being disabled.

·        Fixed shared IRQ issue.

New Changes in Version 4.05.13

·        Fixed L/R channel swap during 16 bit stereo recording.

·        Fixed the first little noise when starting recording.

Version 4.05.10

·        Fixed a pinball game problem.

Version 4.05.08

- Fixed Recording Peak Meter.

Version 4.05.06

- Fixed ACT98 wave driver test failure cases 14,17,19, 31 on Asus mother board.

Version 4.05.05

- Fixed ACT98 wave driver test FAIL cases # 2,17,14,19,31.

Version 4.05.04

- Removed "pop-up sound" when we stop recording with playback.

Version 4.05.03

- Support joystick driver.Allowed to enable/disable IIS    through “EnableIIS=0/1” from config.ini on ES1946.

- Record Monitor mode default value can be set by “RecordMonitorMode=0/1” from config.ini

- Support Power Management function based on Microsoft’s Laptop OAK(Oem Adaptation Kit).    Because Microsoft’s two different version “laptop.sys” behaves differently, a user may encounter some problem during suspend/resume when they use old laptop.sys (whose date is older than Jan 7. 1998).    To avoid such problem, user should change “New_Laptop.Sys=0” in config.ini when they install the driver. By default, this driver supports a newer version of laptop.sys(whose date is Jan, 7. 1998 or later).

Version 4.05.01

- Added Phone and PC Speaker line in mixer for ES1938.

- Fixed some clicking noises while playing 8 bit/sample wave file.

- Support lower sample rate of wave files such as 5.5K Hz.

- Support 48 K Hz sample rate.

# Known Problems

When we try to change MIDI device port from ESFM to external MIDI or vice versa (for example, from MIDI properties of media player), it does not take an effect immediately.    Rather, it takes an effect only after rebooting the system. It seems to be a    Microsoft NT 4.0 bug.