Most systems are already configured by the manufacturer or the dealer.  There is no need to run Setup when starting the computer unless you get a Run Setup message.

The Setup program loads configuration values into the battery-backed nonvolatile memory called CMOS RAM.  This memory area is not part of the system RAM.

|  | If you repeatedly receive Run Setup messages, the battery may be bad. In this case, the system cannot retain configuration values in CMOS. Ask a qualified technician for assistance. |
| --- | --- |

Before you run Setup, make sure that you have saved all open files.  The system reboots immediately after you exit Setup.

# 2.1	Entering Setup

To enter Setup, press the key combination ++.

|  | You must press ++ while the system is booting. This key combination does not work during any other time. |
| --- | --- |

The BIOS Utility Main Menu appears:

BIOS Utility

System Information Basic Configuration System Security Power Management Exit Setup Utility

¯¬® = Move highlight bar, ¿ = Select, Esc = Exit

The Main Menu screen displays the basic configuration level.  This level of configuration allows you to change the basic parameters of your system like the following:

- Changing the system date or time

- Setting power-saving modes and timers

- Setting system password

- Changing system boot drive

- Others

|  | The parameters on the screens show default values. These values may not be the same as those in your system. The grayed items on the screens have fixed settings and are not user-configurable. |
| --- | --- |

## 2.1.1	Advanced Configuration Level

To access the advance configuration level, you must press . The Advance Configuration options appears.

| BIOS Utility |  |  |  |
| --- | --- | --- | --- |
| System Information Basic Configuration System Security Power Management Advanced Configuration Exit Setup Utility |  |  |  |
| ¯¬® = Move highlight bar, ¿ = Select, Esc = Exit |  |  |  |
|  |  | The Advance Configuration level can only be access by pressing in the Main Menu. |  |
|  |  | The advanced configuration settings of your system BIOS is preset. Do not change any settings in the Advanced Configuration level if you are not a qualified technician. |  |

# 2.2	System Information

The following screen appears if you select System Information from the Main menu.

System Information Page 1/2

Processor Pentium Processor Speed 200 MHz L1 Cache Size 32 KB, Enabled L2 Cache Size 512 KB, Enabled Floppy Drive A 1.44 MB, 3.5-inch Floppy Drive B None IDE Primary Channel Master Hard Disk, xxx MB IDE Primary Channel Slave CD-ROM IDE Secondary Channel Master None IDE Secondary Channel Slave None Total Memory xxx MB 1st Bank SDRAM 2nd Bank None 3rd Bank None

PgDn/PgUp = Move Screen, Esc = Back to Main Menu

The System Information menu shows the current basic configuration of your system.

The command line at the bottom of the menu tells you how to move from one screen to another and return to the Main menu.

Press  to move to the next page or  to return to the previous page.

Press  to return to the Main menu.

The following screen shows page 2 of the System Information menu.

System Information Page 2/2

Serial Port 1 3F8h, IRQ 4 Serial Port 2 2F8h, IRQ 3 Parallel Port 378h, IRQ 7 PS/2 Mouse Installed USB Host Controller Disabled ØProduct Information

¯=Move Highlight bar,¿=Select,F1=Help,PgDn/PgUp=Move Screen

The following sections explain the parameters.

## 2.2.1	Processor

The Processor parameter specifies the type of processor currently installed in your system. The system supports Intel Pentium, AMD and Cyrix processors.

## 2.2.2	Processor Speed

The Processor Speed parameter specifies the speed of the processor currently installed in your system.  The system can support  Pentium processors running at 166, 200, and 233 MHz.

## 2.2.3	L1 Cache Size

This parameter specifies the first-level or the internal memory (i.e., the memory integrated into the CPU) size, and whether it is enabled or disabled.  For information on how to configure the system memory, see section 2.6.1.

## 2.2.4	L2 Cache Size

This parameter specifies the second-level cache memory size currently supported by the system.   The available cache sizes are 256 KB or 512 KB (manufacturing setting).  The board may also come without L2 cache. For information on how to configure the system memory, see section 2.6.2

## 2.2.5	Floppy Drive A

This parameter specifies the system’s current floppy drive A settings.  For information on how to configure the floppy drives, see section 2.3.2.

## 2.2.6	Floppy Drive B

This parameter specifies the system’s current floppy drive B settings. For information on how to configure the floppy drives, see section 2.3.2.

## 2.2.7	IDE Primary Channel Master

This parameter specifies the current configuration of the IDE device connected to the master port of the primary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.4.

## 2.2.8	IDE Primary Channel Slave

This parameter specifies the current configuration of the IDE device connected to the slave port of the primary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.4.

## 2.2.9	IDE Secondary Channel Master

This parameter specifies the current configuration of the IDE device connected to the master port of the secondary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.4.

## 2.2.10	IDE Secondary Channel Slave

This parameter specifies the current configuration of the IDE device connected to the slave port of the secondary IDE channel. For information on how to configure the IDE devices drives, see section 2.3.4.

## 2.2.11	Total Memory

This parameter specifies the total amount of onboard memory.   The memory size is automatically detected by BIOS during the POST.  If you install additional memory, the system automatically adjusts this parameter to display the new memory size.

### 1st Bank/2nd Bank/3rd Bank

The 1st Bank, 2nd Bank, and 3rd Bank parameters indicate the type of DRAM installed in DIMM sockets 1, 2, and 3.  The None setting indicates that there is no DRAM installed.  For the location of the DIMM sockets, refer to Figure 1-1.

## 2.2.12	Serial Port 1

This parameter shows the serial port 1 address and IRQ settings.

## 2.2.13	Serial Port 2

This parameter shows the serial port 2 address and IRQ settings.

## 2.2.14	Parallel Port

This parameter shows the parallel port address and IRQ settings.

## 2.2.15	PS/2 Mouse

The BIOS utility automatically detects if there is a mouse connected to your system.  If there is, this parameter displays the Installed setting.  Otherwise, this is set to None.

## 2.2.16	USB Host Controller

This parameter specifies whether the onboard USB controller is enabled or disabled.  For information on how to enable or disable USB, see section 2.6.7.

## 2.2.17	Product Information

The Product Information contains the general data about the system, such as the product name, serial number, BIOS version, etc.  These information are necessary for troubleshooting (may be required when asking for technical support).

The following figure shows how the Product Information screen appears.

Product Information Page 1/1

Product Name xxxxxxxxx System S/N xxxxxxxxx Main Board ID xxxxxxxxx Main Board S/N xxxxxxxxx System BIOS Version vx.xx

PgDn/PgUp = Move Screen, Esc = Back to Main Menu

### Product Name

This parameter specifies the official name of the system.

### System S/N

This parameter specifies the system’s serial number.

### Main Board ID

This parameter specifies the system board’s identification number.

### Main Board S/N

This parameter specifies the system board’s serial number.

### System BIOS Version

This parameter specifies the version of the BIOS utility.

# 2.3	Basic Configuration

Select Basic Configuration to input configuration values such as date, time, and disk types.

The following screen shows the Basic Configuration menu.

Basic Configuration Page 1/1

Date [WWW MMM DD, YYYY] Time [HH:MM:SS] Floppy Drive A [xx-MB, xx-inch] Floppy Drive B [xx-MB, xx-inch] LS-120 drive as [ Normal ] ØIDE Primary Channel Master ØIDE Primary Channel Slave ØIDE Secondary Channel Master ØIDE Secondary Channel Slave ØBoot Option

¯ = Move Highlight Bar, ¬ ® = Change Setting, F1 = Help

## 2.3.1	Date and Time

The real-time clock keeps the system date and time.  After setting the date and time, you do not need to enter them every time you turn on the system.  As long as the internal battery remains good (approximately seven years) and connected, the clock continues to keep the date and time accurately even when the power is off.

Date

Highlight the items on the Date parameter and press  or  to set the date following the month-day-year format.

Valid values for weekday, month, day, and year are:

- Weekday	Sun, Mon, Tue, Wed, Thu, Fri, Sat

- Month	1  to 12

- Day	1  to 31

- Year	1980  to 2099

Time

Highlight the items on the Time parameter and press  or  to set the time following the hour-minute-second format.

Valid values for hour, minute, and second are:

- Hour	00 to 23

- Minute	00 to 59

- Second	00 to 59

## 2.3.2	Floppy Drives

To enter the configuration value for the first floppy drive (drive A), highlight the Floppy Drive A parameter.  Press  or  key to view the options and select the appropriate value.

Possible settings for the Floppy Drive parameters:

- [        None        ]

- [360  KB,   5.25-inch]

- [1.2  MB,   5.25-inch]

- [720  KB,    3.5-inch]

- [1.44 MB,    3.5-inch]

- [2.88 MB,    3.5-inch]

Follow the same procedure to configure floppy drive B.  Choose None if you do not have a second floppy drive.

## 2.3.3	LS-120 Drive As

This parameter allows you not only to enable the LS-120 device installed in your system, but as well as specify the function of the device.  The setting affects how BIOS will detect the device.

Possible settings are:

- Normal   In this setting, BIOS does not support the LS-120 drive.  The drive needs the LS-120 device driver to operate.

- Drive A   BIOS recognizes the LS-120 drive as drive A.  If a standard diskette drive A exists, BIOS automatically identifies it as drive B.  If a standard diskette drive B exists, it automatically becomes inaccessible.  	

If two LS-120 drives exist, BIOS recognizes them as drive A and drive B, respectively..

- Drive B   BIOS recognizes the LS-120 drive as drive B.  If a standard diskette drive B exists, it becomes inaccessible.

- Hard Disk   BIOS recognizes the LS-120 drive as a hard disk.  In this setting, format the LS-120 drive as any other hard disk and assign it a drive letter C, D, E, and so on.  See the documentation that came with the LS-120 drive for more information.

## 2.3.4	IDE Drives

To configure the IDE drives connected to your system, select the parameter that represents the channel and port where the desired hard disk to configure is connected.   The options are:

### IDE Primary Channel Master

This parameter lets you configure the hard disk drive connected to the master port of IDE channel 1.

### IDE Primary Channel Slave

This parameter lets you configure the hard disk drive connected to the slave port of IDE channel 1.

### IDE Secondary Channel Master

This parameter lets you configure the hard disk drive connected to the master port of IDE channel 2.

### IDE Secondary Channel Slave

This parameter lets you configure the hard disk drive connected to the slave port of IDE channel 2.

The following screen appears if you select any of the IDE Drive parameters:

| IDE Primary/Secondary Channel Master/Slave Page 1/1 |  |  |  |
| --- | --- | --- | --- |
| Type [ Auto ] Cylinder [ XXXX ] Head [ XXXX ] Sector [ XXXX ] Size [ XXXX ] MB Hard Disk Size > 504MB [ Auto ] *Hard Disk Block Mode [ Auto ] *Advanced PIO Mode [ Auto ] *Hard Disk 32-Bit Access [Enabled ] *DMA Transfer Mode [ Auto ] *CD-ROM Drive DMA Mode [Disabled] |  |  |  |
| ¯ = Move Highlight Bar, ¬ ® = Change Setting, F1 = Help |  |  |  |
|  |  | The above parameters marked with ‘*’ will only appear if you access the advanced configuration level in the Main Menu. Please refer to section 2.1.1 for more information. |  |

#### Type

This parameter lets you specify the type of hard disk installed in your system.  If you want BIOS to automatically configure your hard disk, select Auto.   If you know your hard disk type, you can enter the setting manually.

Setting this parameter also sets the Cylinder, Head, Sector, and Size parameters.

#### CylinderS

This parameter specifies the number of cylinders of your hard disk, and is automatically set depending on your Type parameter setting.

#### head

This parameter specifies the number of heads of your hard disk, and is automatically set depending on your Type parameter setting.

#### SECTors

This parameter specifies the number of sectors of your hard disk, and is automatically set depending on your Type parameter setting.

#### Size

This parameter specifies the size of your hard disk, in MB.

### Enhanced IDE Features

#### Hard Disk Size > 504 MB

When set to Auto, the BIOS utility automatically detects if the installed hard disk supports the function.  If supported, it allows you to use a hard disk with a capacity of more than 504 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  However, enhanced IDE feature  works only under DOS and Windows 3.x, 95  environment.  Other operating systems require this parameter to be set to Disabled.

#### Hard Disk Block Mode

This function enhances disk performance depending on the hard disk in use.  If you set this parameter to Auto, the BIOS utility automatically detects if the installed hard disk drive supports the Block Mode function.  If supported, it allows data transfer in block (multiple sectors) at a rate of 256 bytes per cycle.  To disregard the feature, change the setting to Disabled.

#### Advanced PIO Mode

When set to Auto, the BIOS utility automatically detects if the installed hard disk supports the function.  If supported, it  allows for faster data recovery and read/write timing that reduces hard disk activity time.  This results to better hard disk performance.  To disregard the feature, change the setting to Disabled.

#### Hard Disk 32-bit Access

Enabling this parameter  improves system performance by allowing the use of the 32-bit hard disk access.  This enhanced IDE feature works only under DOS, Windows 3.x, Windows 95, and Novell NetWare.  If your software or hard disk does not support this function, set this parameter to Disabled .

#### DMA Transfer Mode

The Ultra DMA and Multi-DMA modes enhances hard disk performance by increasing the transfer rate. However, besides enabling these features in the BIOS Setup, both Ultra DMA and Multi-DMA modes require the DMA driver to be loaded. By setting this parameter to Auto, BIOS automatically sets the appropriate DMA mode for your hard disk.

#### CD-ROM Drive DMA Mode

Set this parameter to Enabled to enable the DMA mode for the CD-ROM drive.  This improves the system performance since it allows direct memory access to the CD-ROM.  To deactivate the function, set the parameter to Disabled .

## 2.3.5	Boot Options

This option allows you to specify your preferred setting for bootup.

The following screen appears if you select Boot Options from the Basic Configuration menu:

Boot Options Page 1/1

Fast Boot [ Auto ] Silent Boot [Enabled ] Num Lock After Boot [Enabled ] Memory Test [Disabled] Boot Sequence 1st [Floppy Disk] 2nd [Hard Disk ] 3rd [CD-ROM ] First Hard Disk Drive.. [IDE ] *Configuration Table [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

### Fast Boot

This parameter allows the system to boot faster by skipping some POST routines.  The default setting is Auto.

### Silent Boot

This parameter enables or disables the Silent Boot function.  When set to Enabled, BIOS is in graphical mode and displays only an identification logo during POST and while booting.  After which the screen displays the operating system prompt (such as DOS) or logo (such as Windows 95).  If any error occurred while booting, the system automatically switches to the text mode.

Even if your setting is Enabled, you may also switch to the text mode while booting by pressing  after you hear a beep that indicates the activation of the keyboard.

When set to Disabled, BIOS is in the conventional text mode where you see the system initialization details on the screen.

### Num Lock After Boot

This parameter allows you to activate the Num Lock function upon booting.  The default setting is Enabled.

### Memory Test

When set to Enabled, this parameter allows the system to perform a RAM test during the POST routine.  When set to Disabled, the system detects only the memory size and bypasses the test routine.  The default setting is Disabled.

### Boot Sequence

This parameter allows you to specify the boot search sequence during POST.

- 1st.  The system checks this drive first.

- 2nd. The system then check this drive if it can not boot from the 
        1st specified drive.

- 3rd. If  1st and 2nd search failed then boot from this drive.

The BIOS will display an error message if the drive(s) specified is not bootable.

### First Hard Disk Drive

If you have a SCSI drive installed, setting this parameter to SCSI allow the system to initialize the SCSI drive before the IDE drive.  When set to IDE, the system will initialize the IDE drive first. The default setting is IDE.

### Configuration Table

This option displays the configuration table during boot-up. The configuration table shows a summary of information concerning your system hardware configuration. The default setting is Disabled.

# 2.4	System Security

The Setup program has a number of security features to prevent unauthorized access to the system and its data.

The following screen appears if you select System Security from the Main menu:

System Security Page 1/1

Disk Drive Control Floppy Drive [ Normal ] Hard Disk Drive [ Normal ] Setup Password [ None ] Power-on Password [ None ] Operation Mode [ Normal ]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

## 2.4.1	Disk Drive Control

The disk drive control features allow you to control the floppy drive or the hard disk drive boot function to prevent loading operating systems or other programs from a certain drive while the other drives are operational (under DOS mode only).

Table 2-1 lists the drive control settings and their corresponding functions.

Table 2-1	Drive Control Settings

| Floppy Drive |  |
| --- | --- |
| Setting | Description |
| Normal | Floppy drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |
| Hard Disk Drive |  |
| Setting | Description |
| Normal | Hard disk drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |

## 2.4.2	Setup Password

The Setup Password prevents unauthorized access to the BIOS utility.

### Setting a Password

1.	Make sure that JP10 is set to 2-3 (bypass password).

|  | You cannot enter the BIOS utility if a Setup password does not exist and JP10 is set to 1-2 (password check enabled). By default, JP10 is set to 2-3 (bypass password). |
| --- | --- |

2.	Enter BIOS utility and select System Security.

3.	Highlight the Setup Password parameter and press the  or  key.  The password prompt appears:

4.	Type a password.  The password may consist of up to seven characters.

|  | Be very careful when typing your password because the characters do not appear on the screen. |
| --- | --- |

5.	Press .  A prompt asks you to retype the password to verify your first entry.

6.	Retype the password then press .

After setting the password, the system automatically sets the Setup Password parameter to Present.

7.	Press  to exit the System Security screen and return to the Main menu.

8.	Press  to exit the BIOS utility.  A dialog box appears asking if you want to save the CMOS data.

9.	Select Yes to save the changes and reboot the system.

10.	While rebooting, turn off the system then open the housing.

11.	Set JP10 to 1-2 to enable the password function.

The next time you want to enter the BIOS utility, you must key in your Setup password.

### Changing or Removing the Setup Password

Should you want to change your setup password, do the following:

1.	Enter the BIOS utility and select System Security.

2.	Highlight the Setup Password parameter.

3.	Press  or  to display the password prompt and key in a new password.

or

Press  or  and select None to remove the existing password.

4.	Press  to exit the System Security screen and return to the Main menu.

5.	Press  to exit the BIOS utility.  A dialog box appears asking if you want to save the CMOS data.

6.	Select Yes to save the changes.

### Bypassing the Setup Password

If you forget your setup password, you can bypass the password security feature by hardware.  Follow these steps to bypass the password:

1.	Turn off and unplug the system.

2.	Open the system housing and JP10 is set to 2-3 to bypass the password function.

3.	Turn on the system and enter the BIOS utility.  This time, the system does not require you to type in a password.

|  | You can either change the existing Setup password or remove it by selecting None. Refer to the previous section for the procedure. |
| --- | --- |

## 2.4.3	Power On Password

The Power On Password secures your system against unauthorized use.  Once you set this password, you have to type it whenever you boot the system.  To set this password, enter the BIOS utility, select System Security, then highlight the Power On Password parameter.  Follow the same procedure as in setting the Setup password.

|  | Make sure to set JP10 to 1-2 to enable the Power On password. |
| --- | --- |

### Operation Mode

This function lets you enable or disable the password prompt display.  When set to Normal, the password prompt appears before system boot.  When set to Keyboard Lock, the password prompt do not appear; however, the keyboard will be locked after system boot and will remain locked until the correct password is entered.

# 2.5	Power Management

The Power Management menu  lets you configure the system power-management feature.

The following screen shows the Power Management parameters and their default settings:

| Power Management Page 1/1 |  |  |  |
| --- | --- | --- | --- |
| Power Management Mode [Enabled ] IDE Hard Disk Standby Timer [OFF ] System Sleep Timer [OFF] Sleep Mode [ ----- ] Power Switch < 4 sec. . [Power Off] *ACPI BIOS [Enabled] System Wake-up event Modem Ring Indicator [Enabled ] |  |  |  |
| ¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help |  |  |  |
|  |  | The above parameters marked with ‘*’ will only appear if you access the advanced configuration level in the Main Menu. Please refer to section 2.1.1 for more information. |  |

## 2.5.1	Power Management Mode

This parameter allows you to reduce power consumption.  When this parameter is set to Enabled, you can configure the IDE hard disk and system timers.  Setting to Disabled deactivates the power-management feature and all the timers.

### IDE Hard Disk Standby Timer

This parameter allows the hard disk to enter standby mode after inactivity of 1 to 15 minutes, depending on your setting.  When you access the hard disk again, allow 3 to 5 seconds (depending on the hard disk) for the disk to return to normal speed.  Set this parameter to OFF if your hard disk does not support this function.

### System Sleep Timer

This parameter sets the system to the lowest power-saving mode after a specified period of inactivity.  Any keyboard or mouse action or any activity detected from the IRQ channels resume system operation.

#### Sleep Mode

This parameter lets you specify the power-saving mode that the system will enter after a specified period of inactivity. The options are Standby and Suspend Mode. This parameter becomes configurable only if the System Sleep Timer is enabled. Any keyboard or mouse action, or any enabled monitored activities occurring through the IRQ channels resume system operation.

## 2.5.2	Power Switch < 4 sec.

When set to Power-off, the system automatically turns off when the power switch is pressed.  When set to Suspend, the system enters the suspend mode.

## 2.5.3	ACPI BIOS

ACPI (Advanced Configuration and Power Interface) enables the operating system to control the amount of power given to each device attached to the computer. Using the OS (Operating System), ACPI can turn off peripheral devices , such as a CD-ROM drivers, when they're not in use. The default setting is Enabled.

## 2.5.4	System Wake-up Event

The system wake-up event allows the system to resume operation when the modem ring indicator is enabled.

### Modem Ring Indicator

When Enabled any fax/modem activity wake-up the system from suspend mode. The default setting is Enabled.

# 2.6	Advanced Configuration

The Advanced Configuration option allows you to configure the advanced system memory functions.

|  | To avoid damaging the system, do not change any settings in the Advanced Configuration if you are not a qualified technician. |
| --- | --- |

The following screen shows the Advanced Configuration parameters.

Advanced Configuration Page 1/1

Internal Cache [Enabled ] External Cache [Enabled ] Memory at 15MB-16MB Reserved for [System] C8000h-DFFFF Shadow [Disabled] ØOnboard Peripheral Configuration ØPnP/PCI System Configuration

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

## 2.6.1	Internal Cache

This parameter enables or disables the first-level or internal memory, that is, the memory integrated into the CPU.  The default setting is Enabled.

## 2.6.2	External Cache

This parameter enables or disables the external cache memory.  The external cache is incorporated in the CPU module.

## 2.6.3	Memory at 15MB-16MB Reserved For

To prevent memory address conflicts between the system and expansion boards, reserve this memory range for the use of either the system or an expansion board.

## 2.6.4	C8000h - DFFFF Shadow

The system reserves a portion of random access memory (RAM) for the shadow RAM function.  This parameter allows you to shadow the C8000h-DFFFF address range, enabling I/O ROM functions to run directly from the shadow RAM for faster operation.  When you set this parameter to Disabled, the functions run normally from ROM.

## 2.6.5	Onboard Peripheral Configuration

The Onboard Peripheral Configuration allows you to configure the onboard communication ports and the onboard devices.  Selecting this option from the Advanced Configuration menu displays the following screen:

Onboard Peripheral Configuration Page 1/1

Floppy Disk Controller [Enabled ] IDE Controller [Both ] PS/2 Mouse Controller [Enabled ] USB Host Controller [Enabled ] USB Legacy Mode [Disabled] Ø Onboard Serial/Parallel Port Settings

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help PgDn/PgUp = Move Screen

### Floppy Disk Controller

This parameter lets you enable or disable the onboard floppy disk controller.

### IDE Controller

Set this parameter to Primary to enable only the primary IDE controller; Both to enable both primary and secondary IDE controller; or Disabled to disable all IDE controllers.

### PS/2 Mouse Controller

This parameter enables or disables the onboard PS/2 mouse controller.

### USB Host Controller

This parameter lets you enable or disable the USB controller on board. When enabled, it activates the USB function of the system.  When disabled, it also deactivates the function.

#### USB Legacy Mode

This function, when enabled, lets you use a USB keyboard in DOS environment.  Set this to Disabled to deactivate the USB keyboard function in DOS environment.

### Onboard Serial/Parallel Port Settings

The Onboard Serial/Parallel Port Settings menu allows you to configure the onboard communication ports and the onboard devices.  Selecting this option from the Onboard Peripheral Configuration menu displays the following screen:

Onboard Serial/Parallel Port Settings Page 1/1

Serial Port 1 [Enabled ] Base Address [3F8h] IRQ [ 4 ] Serial Port 2 [Enabled ] Base Address [3F8h] IRQ [ 4 ] Parallel Port [Enabled ] Base Address [3BCh] IRQ [ 7 ] Operation Mode [ECP] ECP DMA Channel [ 3 ]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

#### Serial Port 1 and 2

This parameter allows you to enable or disable serial port 1 and/or 2.

Base Address

This function lets you set a logical base address for serial port 1 and 2.  The options are:

| - 3F8h | - 2F8h | - 3E8h | - 2E8h |
| --- | --- | --- | --- |

IRQ

This function lets you assign an interrupt for serial port 1 and 2.  The options for serial port 1 are IRQ 4 and 11. The option for serial port 2 are IRQ 3 and 10.

|  | The Base Address and IRQ parameters are configurable only if Serial Port 1 and/or 2 is enabled. |
| --- | --- |

#### Parallel Port

This parameter allows you to enable or disable the parallel port.

Base Address

This function lets you set a logical base address for the parallel port.  The options are:

| - 3BCh | - 378h | - 278h |
| --- | --- | --- |

IRQ

This function lets you assign an interrupt for the parallel port.  The options are IRQ 5 and 7.

|  | The Base Address and IRQ parameters are configurable only if Parallel Port is enabled. If you install an add-on card that has a parallel port whose address conflicts with the parallel port onboard, a warning appears on the screen. Check the parallel port address on the add-on card and change the address to one that does not conflict. |
| --- | --- |

Operation Mode

This item allows you to set the operation mode of the parallel port.  Table 2-2 lists the different operation modes.

Table 2-2	Parallel Port Operation Mode Settings

| Setting | Function |
| --- | --- |
| Standard Parallel Port (SPP) | Allows normal speed one-way operation |
| Standard and Bidirectional | Allows normal speed operation in a two-way mode |
| Enhanced Parallel Port (EPP) | Allows bidirectional parallel port operation at maximum speed |
| Extended Capabilities Port (ECP) | Allows parallel port to operate in bidirectional mode and at a speed higher than the maximum data transfer rate |

ECP DMA Channel

This item becomes active only if you select Extended Capabilities Port (ECP) as the operation mode.  It allows you to assign DMA channel 1 or DMA channel 3 for the ECP parallel port function (as required in Windows 95).

## 2.6.7	PnP/PCI System Configuration

The PnP/PCI System Configuration allows you to specify the settings for your PCI devices.  Selecting this option displays the following screen:

PnP/PCI System Configuration Page 1/1

PCI IRQ Setting [ Auto ] INTA INTB INTC INTD PCI Slot 1 [--] [--] [--] [--] PCI Slot 2 [--] [--] [--] [--] PCI Slot 3 [--] [--] [--] [--] PCI Slot 4 [--] [--] [--] [--] PCI IRQ Sharing [Yes] VGA Palette Snoop [Disabled] Plug and Play OS [Yes] Reset Resource Assignments [No ]

¯ = Move Highlight Bar, ® ¬ = Change Setting, F1 = Help

### PCI IRQ Setting

Select Auto to let BIOS automatically configure the plug-and-play (PnP) devices installed on your system. Otherwise, select Manual.

|  | Refer to your manual for technical information about the PCI card. |
| --- | --- |

#### PCI Slots

When you set the PCI IRQ Setting parameter to Auto, these parameters specify the auto-assigned interrupt for each of the PCI devices. If you set the PCI IRQ Setting parameter to Manual, you need to specify the interrupt that you want to assign for each PCI device installed in your system.

### PCI IRQ Sharing

Setting this parameter  to Yes allows you to assign the same IRQ to two different devices.  To disable the feature, select No.

|  | If there are no IRQs available to assign for the remaining device function, we recommend that you enable this parameter. |
| --- | --- |

### VGA Palette Snoop

This parameter permits you to use the palette snooping feature if you installed more than one VGA card in the system.

The VGA palette snoop function allows the control palette register (CPR) to manage and update the VGA RAM DAC (Digital Analog Converter, a color data storage) of each VGA card installed in the system.  The snooping process lets the CPR send a signal to all the VGA cards so that they can update their individual RAM DACs.  The signal goes through the cards continuously until all RAM DAC data have been updated.  This allows display of multiple images on the screen.

|  | Some VGA cards have required settings for this feature. Check your VGA card manual before setting this parameter. |
| --- | --- |

### Plug and Play OS

When this parameter is set to Yes, BIOS initializes only PnP boot devices such as SCSI cards.  When set to No, BIOS initializes all PnP boot and non-boot devices such as sound cards.

|  | Set this parameter to Yes only if your operating system is Windows 95. |
| --- | --- |

### Reset Resource Assignments

Set this parameter to Yes to avoid IRQ conflict when installing non-PnP or PnP ISA cards.  This clears all resource assignments and allows BIOS to reassign resources to all installed PnP devices the next time the system boots.  After clearing the resource data, the parameter resets to No.

# 2.7	Exit Setup Utility

To exit the BIOS utility, select Exit Setup Utility from the Main menu.  The following screen appears:

Exit Setup Utility Page 1/1

Save CMOS Settings and Exit Load Previous Settings and Exit Load Default Settings Save CMOS Settings Abort Settings Change

¯¬® = Move highlight bar, ¿ = Select,Esc = Back to Main Menu

## 2.7.1	Save CMOS Settings and Exit

Select this option if you want to save the current CMOS settings and exit the BIOS utility.

## 2.7.2	Load Previous Settings and Exit

Select this option to cancel the current changes made to the BIOS settings, reload the previous settings and exit the BIOS utility after reload.

## 2.7.3	Load Default Settings

This option loads the default settings for the optimized system configuration.  Press  to return to the Main menu.

## 2.7.4	Save CMOS Settings

Select this option to save the current BIOS settings, including your recent modifications.  Press  to return to the Main Menu.

## 2.7.5	Abort Settings Change

This option cancels all modifications that you have made in the system configuration and reloads your previous settings.  Press  to return to the Main Menu.