| Subject: Release AWARD BIOS S81M R01F3 |  |  | Part No.: Doc. No.: |  |  |  | Rev.: |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Project Code: 91.37P01.101 |  |  | Release Date:03/28/2002 |  |  |  | Page of |  |
| Project Name/Model No.: S81M FOR Veriton 7200 |  |  |  |  |  |  | Revision Status |  |
| Description: |  |  |  |  |  |  | Page |  |
| Release S81M R01-F3 BIOS |  |  |  |  |  |  |  |  |
| Reason for Release: |  |  |  |  |  |  |  |  |
| Fix : INT15.exe (pre-load utility) run at 32bit mode will fail. |  |  |  |  |  |  |  |  |
| Item | Contents |  |  | Q’ty | unit |  |  |  |
| 1 | Software R/N check list. |  |  | 1 | page |  |  |  |
| 2 | CD containing BIOS source code and binary file |  |  | 1 | set |  |  |  |
|  | ( 1 disk/set ) |  |  |  |  |  |  |  |
|  | Diskettes containing BIOS binary code |  |  | 1 | set |  |  |  |
|  | ( 1 disk/set ) |  |  |  |  |  |  |  |
| 3 | Files listing |  |  | 1 | page |  |  |  |
| 4 | Award BIOS checksum=EA00h |  |  |  |  |  |  |  |
| 5 | Check point list. |  |  | 5 | page |  |  |  |
|  | ECR/NO: |  |  |  |  |  |  |  |
|  | PF.FC.ACRBC.EC.R01F2 |  |  |  |  |  |  |  |
| Copy To(All Contents): |  |  |  |  |  |  |  |  |
| Copy (Cover Sheet): |  |  |  |  |  |  |  |  |
| Approved By/ |  | Reviewed By/ QA |  |  |  | Prepared By/ MRE2 #1646 |  |  |

- Documentation

| ##### Item | ### Check Item | ##### Yes | ##### No | ## Remark |
| --- | --- | --- | --- | --- |
| 1. | Does The Diskette Belong To Dos Media | ######  |  | #### If Not, Please to Fill Item 6 |
| 2. | Has The Diskette Re-formatted By Ms-DOS ?If it Does, Please Show The Version on “Remark” | ######  |  |  |
| 3. | Does The “Boot Sector” On Diskette Belong to Our Company Product? |  | ###### X |  |
| 4 | Does The “Hidden Files” on Diskette Belong to Our Company Product ? |  | X |  |
| 5. | Does The “Command.COM” on Diskette belong to Our Company Product? |  | X |  |
| 6. | Does The Content of Files On Diskette Relate Or Mention The Name of Other Company? | ######  |  |  |
| 7. | Does It List All The Files on Diskette? (Include The File Name, Size, Date…Etc.) | ######  |  |  |
| 8. | Does The File On Diskette Show “Acer Inc. Copyright 199X”? | ######  |  |  |
| 9. | Does The File On Diskette Need to Apply The Copyright? | ######  |  |  |
| 10. | Please Make Note In Remark : 1.The Method of Copy This Diskette. 2.The Type Of Media |  |  | - Disk Operation System - 80 Track 18 Sector 1440 KB |
| 11. | Is Release Hardware Follow Virus Installation Procedure? |  |  |  |
| 12. | Does The Virus Scan Find a Virus in The Diskette? |  | X | Scan File Name : OFFICESCAN Version : 246, Engine 6.100 |

- Conclusion

 Approved

 Rejected

- Remark

R01F3  EXE       4,411,865  03-21-02  15:38

R01F3  BIN         262,144  03-21-02  15:29

Information of On-board Components :

1. CPU BU Code Bundled –

Northwood CPU

MU4F12N.INC		A0 stepping

MU4F21N.INC		A1 stepping

MU4F23N.INC		A2 stepping

MU4F24N.INC		B0 stepping

Willamette CPU

MU1F12W.INC		D0 stepping

MU1F0AW.INC		C1 stepping

2. EEPROM Type Supported –

2Mbit

- SST FWH 49LF002A

- Winbond W49V002FAP

Appendix C. Check Points List

When POST executes a task, it uses a series of preset numbers called check points to be latched at port 80h, indicating the stages it is currently running.  This latch can be read and shown on a debug board.

Table D-1 describes the Acer common tasks carried out by POST.  A unique check point number represents each task.

POST Error Code Revision 1.0 11/03/1999 POST (hex); Description. CFh; Test CMOS R/W functionality.. C0h; Early chipset initialization: -Disable shadow RAM -Disable L2 cache (socket 7 or below) -Program basic chipset registers. C1h; Detect memory -Auto-detection of DRAM size, type and ECC. -Auto-detection of L2 cache (socket 7 or below). C3h; Expand compressed BIOS code to DRAM. C5h; Call chipset hook to copy BIOS back to E000 & F000 shadow RAM.. 0h1; Expand the Xgroup codes locating in physical address 1000:0. 02h; Reserved. 03h; Initial Superio_Early_Init switch.. 04h; Reserved. 05h; 1. Blank out screen 2. Clear CMOS error flag. 06h; Reserved. 07h; 1. Clear 8042 interface 2. Initialize 8042 self-test. 08h; - Test special keyboard controller for Winbond 977 series Super I/O chips. - Enable keyboard interface.. 09h; Reserved. 0Ah; - Disable PS/2 mouse interface (optional). - Auto detect ports for keyboard & mouse followed by a port & interface swap (optional). - Reset keyboard for Winbond 977 series Super I/O chips.. 0Bh; Reserved. 0Ch; Reserved. 0Dh; Reserved. 0Eh; Test F000h segment shadow to see whether it is R/W-able or not. If test fails, keep beeping the speaker.. 0Fh; Reserved. 10h; Auto detect flash type to load appropriate flash R/W codes into the run time area in F000 for ESCD & DMI support.. 11h; Reserved. 12h; Use walking 1’s algorithm to check out interface in CMOS circuitry. Also set real-time clock power status, and then check for override.. 13h; Reserved. 14h; Program chipset default values into chipset. Chipset default values are MODBINable by OEM customers.. 15h; Reserved. 16h; Initial onboard clock generator if Early_Init_Onboard_Generator is defined. See also POST 26h.. 17h; Reserved. 1Dh; Initial EARLY_PM_INIT switch.. 1Eh; Reserved. 1Fh; Load keyboard matrix (notebook platform). 20h; Reserved. 21h; HPM initialization (notebook platform). 22h; Reserved. 23h; - Check validity of RTC value: e.g. a value of 5Ah is an invalid value for RTC minute. - 2. Load CMOS settings into BIOS stack. If CMOS checksum fails, use default value instead.. 24h; Prepare BIOS resource map for PCI & PnP use. If ESCD is valid, take into consideration of the ESCD’s legacy information.. 25h; Early PCI Initialization: -Enumerate PCI bus number. -Assign memory & I/O resource -Search for a valid VGA device & VGA BIOS, and put it into C000:0. 26h; - If Early_Init_Onboard_Generator is not defined Onboard clock generator initialization. Disable respective clock resource to empty PCI & DIMM slots. - Init onboard PWM - Init onboard H/W monitor devices. 27h; Initialize INT 09 buffer. 28h; Reserved. 29h; - Program CPU internal MTRR (P6 & PII) for 0-640K memory address. - Initialize the APIC for Pentium class CPU. - Program early chipset according to CMOS setup. Example: onboard IDE controller. - Measure CPU speed.. 2Ah; Reserved. 2Bh; Invoke Video BIOS. 2Ch; Reserved. 2Dh; - Initialize double-byte language font (Optional) - Put information on screen display, including Award title, CPU type, CPU speed, full screen logo.. 2Eh; Reserved. 2Fh; Reserved. 30h; Reserved. 31h; Reserved. 32h; Reserved. 33h; Reset keyboard if Early_Reset_KB is defined e.g. Winbond 977 series Super I/O chips. See also POST 63h.. 34h; Reserved. 35h; Test DMA Channel 0. 36h; Reserved. 37h; Test DMA Channel 1.. 38h; Reserved. 39h; Test DMA page registers.. 3Ah; Reserved. 3Bh; Reserved. 3Ch; Test 8254. 3Dh; Reserved. 3Eh; Test 8259 interrupt mask bits for channel 1.. 3Fh; Reserved. 40h; Test 8259 interrupt mask bits for channel 2.. 41h; Reserved. 42h; Reserved. 43h; Test 8259 functionality.. 44h; Reserved. 45h; Reserved. 46h; Reserved. 47h; Initialize EISA slot. 48h; Reserved. 49h; - Calculate total memory by testing the last double word of each 64K page. - Program write allocation for AMD K5 CPU.. 4Ah; Reserved. 4Bh; Reserved. 4Ch; Reserved. 4Dh; Reserved. 4Eh; - Program MTRR of M1 CPU - Initialize L2 cache for P6 class CPU & program CPU with proper cacheable range. - Initialize the APIC for P6 class CPU. - On MP platform, adjust the cacheable range to smaller one in case the cacheable ranges between each CPU are not identical.. 4Fh; Reserved. 50h; Initialize USB Keyboard & Mouse.. 51h; Reserved. 52h; Test all memory (clear all extended memory to 0). 53h; Clear password according to H/W jumper (Optional). 54h; Reserved. 55h; Display number of processors (multi-processor platform). 56h; Reserved. 57h; - Display PnP logo - Early ISA PnP initialization -Assign CSN to every ISA PnP device.. 58h; Reserved. 59h; Initialize the combined Trend Anti-Virus code.. 5Ah; Reserved. 5Bh; (Optional Feature) Show message for entering AWDFLASH.EXE from FDD (optional). 5Ch; Reserved. 5Dh; - Initialize Init_Onboard_Super_IO - Initialize Init_Onbaord_AUDIO.. 5Eh; Reserved. 5Fh; Reserved. 60h; Okay to enter Setup utility; i.e. not until this POST stage can users enter the CMOS setup utility.. 61h; Reserved. 62h; Reserved. 63h; Reset keyboard if Early_Reset_KB is not defined.. 64h; Reserved. 65h; Initialize PS/2 Mouse. 66h; Reserved. 67h; Prepare memory size information for function call: INT 15h ax=E820h. 68h; Reserved. 69h; Turn on L2 cache. 6Ah; Reserved. 6Bh; Program chipset registers according to items described in Setup & Auto-configuration table.. 6Ch; Reserved. 6Dh; - Assign resources to all ISA PnP devices. - Auto assign ports to onboard COM ports if the corresponding item in Setup is set to “AUTO”.. 6Eh; Reserved. 6Fh; - Initialize floppy controller - Set up floppy related fields in 40:hardware.. 70h; Reserved. 71h; Reserved. 72h; Reserved. 73h; (Reserved. 74h; Reserved. 75h; Detect & install all IDE devices: HDD, LS120, ZIP, CDROM…... 76h; (Optional Feature) Enter AWDFLASH.EXE if: -AWDFLASH.EXE is found in floppy drive. -ALT+F2 is pressed.. 77h; Detect serial ports & parallel ports.. 78h; Reserved. 79h; Reserved. 7Ah; Detect & install co-processor. 7Bh; Reserved. 7Ch; Init HDD write protect.. 7Dh; Reserved. 7Eh; Reserved. 7Fh; Switch back to text mode if full screen logo is supported. -If errors occur, report errors & wait for keys -If no errors occur or F1 key is pressed to continue: Clear EPA or customization logo.. 80h; Reserved. 81h; Reserved. # E8POST.ASM starts. 82h; - Call chipset power management hook. - Recover the text fond used by EPA logo (not for full screen logo) - If password is set, ask for password.. 83h; Save all data in stack back to CMOS. 84h; Initialize ISA PnP boot devices. 85h; - USB final Initialization - Switch screen back to text mode. 86h; Reserved. 87h; NET PC: Build SYSID Structure.. 88h; Reserved. 89h; - Assign IRQs to PCI devices - Set up ACPI table at top of the memory.. 8Ah; Reserved. 8Bh; - Invoke all ISA adapter ROMs - Invoke all PCI ROMs (except VGA). 8Ch; Reserved. 8Dh; - Enable/Disable Parity Check according to CMOS setup - APM Initialization. 8Eh; Reserved. 8Fh; Clear noise of IRQs. 90h; Reserved. 91h; Reserved. 92h; Reserved. 93h; Read HDD boot sector information for Trend Anti-Virus code. 94h; - Enable L2 cache - Program Daylight Saving - Program boot up speed - Chipset final initialization. - Power management final initialization - Clear screen & display summary table - Program K6 write allocation - Program P6 class write combining. 95h; Update keyboard LED & typematic rate. 96h; - Build MP table - Build & update ESCD - Set CMOS century to 20h or 19h - Load CMOS time into DOS timer tick - Build MSIRQ routing table.. FFh; Boot attempt (INT 19h).

| 18h | Detect CPU information including brand, SMI type (Cyrix or Intel) and CPU level (586 or 686). |
| --- | --- |
| 19h | Reserved |
| 1Ah | Reserved |
| 1Bh | Initial interrupts vector table. If no special specified, all H/W interrupts are directed to SPURIOUS_INT_HDLR & S/W interrupts to SPURIOUS_soft_HDLR. |
| 1Ch | Reserved |
| 1Dh | Initial EARLY_PM_INIT switch. |
| 1Eh | Reserved |
| 1Fh | Load keyboard matrix (notebook platform) |
| 20h | Reserved |
| 21h | HPM initialization (notebook platform) |
| 22h | Reserved |
| 23h | - Check validity of RTC value: e.g. a value of 5Ah is an invalid value for RTC minute. 2. Load CMOS settings into BIOS stack. If CMOS checksum fails, use default value instead. |
| 24h | Prepare BIOS resource map for PCI & PnP use. If ESCD is valid, take into consideration of the ESCD’s legacy information. |
| 25h | Early PCI Initialization: -Enumerate PCI bus number. -Assign memory & I/O resource -Search for a valid VGA device & VGA BIOS, and put it into C000:0 |
| 26h | - If Early_Init_Onboard_Generator is not defined Onboard clock generator initialization. Disable respective clock resource to empty PCI & DIMM slots. - Init onboard PWM - Init onboard H/W monitor devices |
| 27h | Initialize INT 09 buffer |
| 28h | Reserved |
| 29h | - Program CPU internal MTRR (P6 & PII) for 0-640K memory address. - Initialize the APIC for Pentium class CPU. - Program early chipset according to CMOS setup. Example: onboard IDE controller. - Measure CPU speed. |
| 2Ah | Reserved |
| 2Bh | Invoke Video BIOS |
| 2Ch | Reserved |
| 2Dh | - Initialize double-byte language font (Optional) - Put information on screen display, including Award title, CPU type, CPU speed, full screen logo. |
| 2Eh | Reserved |
| 2Fh | Reserved |
| 30h | Reserved |
| 31h | Reserved |
| 32h | Reserved |
| 33h | Reset keyboard if Early_Reset_KB is defined e.g. Winbond 977 series Super I/O chips. See also POST 63h. |
| 34h | Reserved |
| 35h | Test DMA Channel 0 |
| 36h | Reserved |
| 37h | Test DMA Channel 1. |
| 38h | Reserved |
| 39h | Test DMA page registers. |
| 3Ah | Reserved |
| 3Bh | Reserved |
| 3Ch | Test 8254 |
| 3Dh | Reserved |
| 3Eh | Test 8259 interrupt mask bits for channel 1. |
| 3Fh | Reserved |
| 40h | Test 8259 interrupt mask bits for channel 2. |
| 41h | Reserved |
| 42h | Reserved |
| 43h | Test 8259 functionality. |
| 44h | Reserved |
| 45h | Reserved |
| 46h | Reserved |
| 47h | Initialize EISA slot |
| 48h | Reserved |
| 49h | - Calculate total memory by testing the last double word of each 64K page. - Program write allocation for AMD K5 CPU. |
| 4Ah | Reserved |
| 4Bh | Reserved |
| 4Ch | Reserved |
| 4Dh | Reserved |
| 4Eh | - Program MTRR of M1 CPU - Initialize L2 cache for P6 class CPU & program CPU with proper cacheable range. - Initialize the APIC for P6 class CPU. - On MP platform, adjust the cacheable range to smaller one in case the cacheable ranges between each CPU are not identical. |
| 4Fh | Reserved |
| 50h | Initialize USB Keyboard & Mouse. |
| 51h | Reserved |
| 52h | Test all memory (clear all extended memory to 0) |
| 53h | Clear password according to H/W jumper (Optional) |
| 54h | Reserved |
| 55h | Display number of processors (multi-processor platform) |
| 56h | Reserved |
| 57h | - Display PnP logo - Early ISA PnP initialization -Assign CSN to every ISA PnP device. |
| 58h | Reserved |
| 59h | Initialize the combined Trend Anti-Virus code. |
| 5Ah | Reserved |
| 5Bh | (Optional Feature) Show message for entering AWDFLASH.EXE from FDD (optional) |
| 5Ch | Reserved |
| 5Dh | - Initialize Init_Onboard_Super_IO - Initialize Init_Onbaord_AUDIO. |
| 5Eh | Reserved |
| 5Fh | Reserved |
| 60h | Okay to enter Setup utility; i.e. not until this POST stage can users enter the CMOS setup utility. |
| 61h | Reserved |
| 62h | Reserved |
| 63h | Reset keyboard if Early_Reset_KB is not defined. |
| 64h | Reserved |
| 65h | Initialize PS/2 Mouse |
| 66h | Reserved |
| 67h | Prepare memory size information for function call: INT 15h ax=E820h |
| 68h | Reserved |
| 69h | Turn on L2 cache |
| 6Ah | Reserved |
| 6Bh | Program chipset registers according to items described in Setup & Auto-configuration table. |
| 6Ch | Reserved |
| 6Dh | - Assign resources to all ISA PnP devices. - Auto assign ports to onboard COM ports if the corresponding item in Setup is set to “AUTO”. |
| 6Eh | Reserved |
| 6Fh | - Initialize floppy controller - Set up floppy related fields in 40:hardware. |
| 70h | Reserved |
| 71h | Reserved |
| 72h | Reserved |
| 73h | (Reserved |
| 74h | Reserved |
| 75h | Detect & install all IDE devices: HDD, LS120, ZIP, CDROM….. |
| 76h | (Optional Feature) Enter AWDFLASH.EXE if: -AWDFLASH.EXE is found in floppy drive. -ALT+F2 is pressed. |
| 77h | Detect serial ports & parallel ports. |
| 78h | Reserved |
| 79h | Reserved |
| 7Ah | Detect & install co-processor |
| 7Bh | Reserved |
| 7Ch | Init HDD write protect. |
| 7Dh | Reserved |
| 7Eh | Reserved |
| 7Fh | Switch back to text mode if full screen logo is supported. -If errors occur, report errors & wait for keys -If no errors occur or F1 key is pressed to continue: Clear EPA or customization logo. |
| 80h | Reserved |
| 81h | Reserved |
| # E8POST.ASM starts |  |
| 82h | - Call chipset power management hook. - Recover the text fond used by EPA logo (not for full screen logo) - If password is set, ask for password. |
| 83h | Save all data in stack back to CMOS |
| 84h | Initialize ISA PnP boot devices |
| 85h | - USB final Initialization - Switch screen back to text mode |
| 86h | Reserved |
| 87h | NET PC: Build SYSID Structure. |
| 88h | Reserved |
| 89h | - Assign IRQs to PCI devices - Set up ACPI table at top of the memory. |
| 8Ah | Reserved |
| 8Bh | - Invoke all ISA adapter ROMs - Invoke all PCI ROMs (except VGA) |
| 8Ch | Reserved |
| 8Dh | - Enable/Disable Parity Check according to CMOS setup - APM Initialization |
| 8Eh | Reserved |
| 8Fh | Clear noise of IRQs |
| 90h | Reserved |
| 91h | Reserved |
| 92h | Reserved |
| 93h | Read HDD boot sector information for Trend Anti-Virus code |
| 94h | - Enable L2 cache - Program Daylight Saving - Program boot up speed - Chipset final initialization. - Power management final initialization - Clear screen & display summary table - Program K6 write allocation - Program P6 class write combining |
| 95h | Update keyboard LED & typematic rate |
| 96h | - Build MP table - Build & update ESCD - Set CMOS century to 20h or 19h - Load CMOS time into DOS timer tick - Build MSIRQ routing table. |
| FFh | Boot attempt (INT 19h) |