This high-performance system board utilizes a 64-bit, 3.3V Intel Pentiumâ CPU running at 75/50, 90/60, 100/66, 120/60, 133/66, 150/60, 166/66, or 200/66 MHz.  It has three ISA and four PCI slots (with one PCI- and ISA-shared slot) for future expansion.  The onboard six 72-pin SIMM sockets allow memory upgrade to a maximum of 192 MB.  To further enhance system performance, you may also upgrade the integrated 256-KB pipelined-burst cache to 512 KB.

Standard features such as two serial ports, one parallel port, a diskette drive interface, and an embedded hard disk interface are also incorporated in the system board.  A Universal Serial Bus (USB) interface is added to the design to enable the system to support additional peripherals.

The system supports the power-management function that conforms to the power-saving standards of the U.S. Environmental Protection Agency (EPA) Energy Star program..  It also offers the Plug-and-Play feature.  This feature saves the user from configuration troubles, thus making the system more user-friendly.

The system is fully compatible with MS-DOS V6.X, OS/2, UNIX, Windows NT and Windows 95 operating systems.

# 1.1	System Board Layout

The system board has the following features and components:

- Supports 3.3V Intel Pentium CPU (75/50, 90/60, 100/66, 120/60, 133/66, 150/60, 166/66, 200/66 MHz)

- 192-MB maximum system memory

- Six 72-pin SIMM sockets that accept 4-, 8-, 16-, and 32-MB SIMMs, with or without Extended Data Output (EDO) and Error Checking and Correction (ECC) functions

- 256-KB/512-KB pipelined-burst second-level cache

- Integrates an enhanced PCI local bus IDE controller

- 128-KB Flash ROM for system BIOS

- Three ISA- and four PCI-expansion slots (one PCI- and ISA-shared slot)

- Dual 16C550 buffered serial ports and one ECP/EPP parallel port

- Optional USB interface that enables the system to support more peripherals

- PS/2 mouse and keyboard interface

- Plug-and-Play function

- Power-management function

Figure 1-1 shows the board layout and the locations of the important components.

1

2

3

4

5

6

7

8

9

10

11

12

13

14

1	Super I/O controller	9	Ether-SCSI card slot

2	RTC battery	10	PCI slots

3	SIMM sockets	11	ISA slots

4	Tag RAM	12	BIOS chip
5	256-KB pipelined-burst cache	13	PS/2 mouse connector

6	Cache module socket	14	PS/2 keyboard connector

7	CPU socket

8	Voltage regulators with heatsink

Figure 1-1	System Board Layout

|  | The heatsink becomes very hot when the system is on. NEVER touch the heatsink with any metal or with your hands. |
| --- | --- |

# 1.2	ESD Precautions

Electrostatic discharge (ESD) can damage your processor, disk drives, expansion boards, and other components.  Always observe the following precautions before you install a system component.

1.	Do not remove a component from its protective packaging until you are ready to install it.

2.	Wear a wrist grounding strap and attach it to a metal part of the system unit before handling components.  If a wrist strap is not available, maintain contact with the system unit throughout any procedure requiring ESD protection.

# 1.3	Pre-installation Instructions

Always observe the following before you install a system component:

1.	Turn off the system power and all the peripherals connected to the unit before opening it.

2.	Open the system according to the instructions in the housing installation manual.

3.	Follow the ESD precautions in section 1.2 before handling a system component.

4.	Remove any expansion boards or peripherals that block access to the SIMM sockets or CPU socket.

5.	See the following sections for specific instructions on the component you wish to install.

|  | Do not attempt the procedures described in the following sections unless you are a qualified service technician. |
| --- | --- |

# 1.4	Installing a CPU

## 1.4.1 	Installation Procedures

|  | Observe the ESD precautions when installing components. See section 1.2. |
| --- | --- |

The system board features a zero-insertion force (ZIF) socket for easy CPU installation.

Follow these steps to install a CPU:

1.	Pull up the socket lever.

2.	Insert the CPU, making sure that pin 1 (indicated by a notched corner) of the CPU connects to hole 1 of the socket.

3.	Pull down the socket lever to lock the CPU into the socket.

Notched corner

Figure 1-2	Installing a CPU

## 1.4.2	Installing an OverDriveä CPU

The OverDrive CPU comes with a heatsink already mounted.  To maintain proper airflow around the CPU and heatsink, follow the required clearances in Table 1-1.

Table 1-1	Clearances for OverDrive CPU and Heatsink

| Location | Minimum Clearance |
| --- | --- |
| Above the heatsink | 0.4 inches |
| Sides of the CPU (at least 3 of 4) | 0.2 inches |

Follow these steps to install an OverDrive CPU:

1.	Locate the CPU socket and pull up the socket lever.

2.	Remove the old CPU carefully from the socket and place it in an antistatic package.

3.	Take the upgrade CPU out of its antistatic package.

4.	Carefully insert the CPU into the socket such that pin 1 (indicated by a notched corner) of the CPU connects to hole 1 of the socket.

Notched Corner

Figure 1-3	Installing an OverDrive CPU

5.	Push down the socket lever to lock the CPU into the socket.

# 1.5	Jumpers and Connectors

Figure 1-4 shows the jumper and connector locations.

Figure 1-4	Jumper and Connector Locations

|  | The shaded pin indicates pin 1. |
| --- | --- |

The following tables list the jumper settings and their corresponding functions:

Table 1-2	System Board Jumper Settings

| Jumper | Setting | Function |
| --- | --- | --- |
| JP1 | 1-2 2-3 * | Check password Bypass password |
| JP3 | 1-2 2-3 * | 64-MB cacheable memory 512-MB cacheable memory |
| JP5 | 1-2 2-3 * | 3.3V operating voltage 3.6V operating voltage |
| JP6 | 1-2 2-3 * | 60 MHz DRAM refresh rate 66 MHz DRAM refresh rate |
| JP10 | 1-2 2-3 * | Enable SMI switch Enable Reset switch |
| JPX2 | 1-2 * 2-3 | Select 256-KB cache Select 512-KB cache |
| JPX3 | 1-2 2-3 | Support 2.5V CPU voltage Support 2.8V CPU voltage |

Table 1-3	CPU Type Select

| CN10 | CN11 | CNX1 | CPU Type |
| --- | --- | --- | --- |
| 1-3 2-4 | 1-3 2-4 | Open | P54C |
| Open | Open | 1-5, 2-6 3-7, 4-8 | P55C |

Table 1-4	CPU Frequency Select

| JP4 | JP6 | JP8 | JP9 | CN12 | CPU Freq. |
| --- | --- | --- | --- | --- | --- |
| 1-2 | 1-2 | 1-2 | 1-2 | 1-3, 2-4 | 75 MHz |
| 2-3 | 1-2 | 1-2 | 1-2 | 2-4 | 90 MHz |
| 2-3 | 2-3 | 1-2 | 1-2 | 1-3 | 100 MHz |
| 2-3 | 1-2 | 2-3 | 1-2 | 2-4 | 120 MHz |
| 2-3 | 2-3 | 2-3 | 1-2 | 1-3 | 133 MHz |
| 2-3 | 1-2 | 2-3 | 2-3 | 2-4 | 150 MHz |
| 2-3 | 2-3 | 2-3 | 2-3 | 1-3 | 166 MHz |
| 2-3 | 2-3 | 1-2 | 2-3 | 1-3 | 200 MHz |

Table 1-5	Second-level Cache Size Select

| JP7 | JPX1 | Cache Size |
| --- | --- | --- |
| 1-2 | 1-2 | Disable cache |
| 2-3 * | 1-2 * | 256 KB |
| 1-2 | 2-3 | 512 KB |
| 2-3 | 2-3 | Reserved |

Table 1-6 lists the onboard connectors.

Table 1-6	Onboard Connectors

| Connector | Function |
| --- | --- |
| CN1 | COM1 connector |
| CN2 | COM2 connector |
| CN3 | Parallel connector |
| CN5 | FDD connector |
| CN6 | Power connector |
| CN7 | USB connector |
| CN8 | HDD1 connector |
| CN9 | HDD2 connector |
| CN13 | Infrared (IR) function connector |
| CN15 | IDE LED connector |
| CN16 | Multifunction connector |
| CO1 | Two-pin fan connector |

The multifunction connector CN16 accommodates the front panel connectors for speaker, LEDs, keylock, reset, and turbo.  Figure 1-5 shows the CN16 pin assignments.

Figure 1-5	20-Pin Multifunction Connector  (CN16)

# 1.6	Installing Memory

The system memory is expandable to 192 MB by adding single in-line memory modules (SIMMs).  See Figure 1-1 for the location of the SIMM sockets.  Section 1.6.1 tells how to install SIMMs.

The six 72-pin SIMM sockets on the system board accept single- 
(4- and 8-MB) and double-density (16- and 32-MB) SIMMs, with 60 ns or less DRAM speed.  Table 1-7 lists the possible memory configurations.

Table 1-7	Memory Configurations

| Bank 0 |  | Bank 1 |  | Bank 2 |  | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Slot 1 | Slot 2 | Slot 3 | Slot 4 | Slot 5 | Slot 6 | Memory |
| 4 MB | 4 MB |  |  |  |  | 8 MB |
| 4 MB | 4 MB | 4 MB | 4 MB |  |  | 16 MB |
| 4 MB | 4 MB | 4 MB | 4 MB | 4 MB | 4 MB | 24 MB |
| 8 MB | 8 MB |  |  |  |  | 16 MB |
| 8 MB | 8 MB | 4 MB | 4 MB |  |  | 24 MB |
| 8 MB | 8 MB | 4 MB | 4 MB | 4 MB | 4 MB | 32 MB |
| 8 MB | 8 MB | 8 MB | 8 MB |  |  | 32 MB |
| 8 MB | 8 MB | 8 MB | 8 MB | 4 MB | 4 MB | 40 MB |
| 8 MB | 8 MB | 8 MB | 8 MB | 8 MB | 8 MB | 48 MB |
| 16 MB | 16 MB |  |  |  |  | 32 MB |
| 16 MB | 16 MB | 4 MB | 4 MB |  |  | 40 MB |
| 16 MB | 16 MB | 4 MB | 4 MB | 4 MB | 4 MB | 48 MB |
| 16 MB | 16 MB | 8 MB | 8 MB |  |  | 48 MB |
| 16 MB | 16 MB | 8 MB | 8 MB | 4 MB | 4 MB | 56 MB |
| 16 MB | 16 MB | 8 MB | 8 MB | 8 MB | 8 MB | 64 MB |
| 16 MB | 16 MB | 16 MB | 16 MB |  |  | 64 MB |
| 16 MB | 16 MB | 16 MB | 16 MB | 4 MB | 4 MB | 72 MB |

Table 1-7	Memory Configurations  (continued)

| Bank 0 |  | Bank 1 |  | Bank 2 |  | Total |
| --- | --- | --- | --- | --- | --- | --- |
| Slot 1 | Slot 2 | Slot 3 | Slot 4 | Slot 5 | Slot 6 | Memory |
| 16 MB | 16 MB | 16 MB | 16 MB | 8 MB | 8 MB | 80 MB |
| 16 MB | 16 MB | 16 MB | 16 MB | 16 MB | 16 MB | 96 MB |
| 32 MB | 32 MB |  |  |  |  | 64 MB |
| 32 MB | 32 MB | 4 MB | 4 MB |  |  | 72 MB |
| 32 MB | 32 MB | 4 MB | 4 MB | 4 MB | 4 MB | 80 MB |
| 32 MB | 32 MB | 8 MB | 8 MB |  |  | 80 MB |
| 32 MB | 32 MB | 8 MB | 8 MB | 4 MB | 4 MB | 88 MB |
| 32 MB | 32 MB | 8 MB | 8 MB | 8 MB | 8 MB | 96 MB |
| 32 MB | 32 MB | 16 MB | 16 MB |  |  | 96 MB |
| 32 MB | 32 MB | 16 MB | 16 MB | 4 MB | 4 MB | 104 MB |
| 32 MB | 32 MB | 16 MB | 16 MB | 8 MB | 8 MB | 112 MB |
| 32 MB | 32 MB | 16 MB | 16 MB | 16 MB | 16 MB | 128 MB |
| 32 MB | 32 MB | 32 MB | 32 MB |  |  | 128 MB |
| 32 MB | 32 MB | 32 MB | 32 MB | 4 MB | 4 MB | 136 MB |
| 32 MB | 32 MB | 32 MB | 32 MB | 8 MB | 8 MB | 144 MB |
| 32 MB | 32 MB | 32 MB | 32 MB | 16 MB | 16 MB | 160 MB |
| 32 MB | 32 MB | 32 MB | 32 MB | 32 MB | 32 MB | 192 MB |

## 1.6.1	Installing a SIMM

|  | Observe the ESD precautions when installing components. See section 1.2. |
| --- | --- |

Follow these steps to install a SIMM:

1.	Carefully slip a SIMM at a 45° angle into a socket making sure that the curved edge indicating the pin 1 of the SIMM matches pin 1 of the socket.

|  | A SIMM fits only in one direction. If you slip in a SIMM but would not completely fit, you may have inserted it the wrong way. Reverse the orientation of the SIMM. |
| --- | --- |

2.	Gently push the SIMM to a vertical position until the pegs of the socket slip into the holes on the SIMM, and the holding clips lock the SIMM into position.  The SIMM should be at a 90° angle when installed.

Hole

Peg

Pin 1 Indicator (curved edge)

1

2

Figure 1-6	Installing a SIMM

|  | See section 1.9 for the post-installation instructions. |
| --- | --- |

## 1.6.2	Removing a SIMM

Follow these steps to remove a SIMM:

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Move the SIMM to a 45° angle.

3.	Pull the SIMM out of the socket.

1

2

3

Holding Clip

Figure 1-7	Removing a SIMM

## 1.6.3	Reconfiguring the System

The system automatically detects the amount of memory installed.  Run Setup to view the new value for total system memory and make a note of it.

# 1.7	Upgrading the Second-level Cache

|  | Observe ESD precautions when installing components. See section 1.2. |
| --- | --- |

The system board comes with 256-KB second-level pipelined-burst cache and a cache module socket.  The cache socket allows you to upgrade the second-level cache to 512-KB.  See Figure 1-1 for the location of the 256-KB pipelined-burst cache and the cache module socket.

Upgrading the cache memory improves system performance.  To upgrade, simply insert the memory board into the cache socket.

Figure 1-8	Installing the Cache Module

|  | See section 1.9 for the post-installation instructions. |
| --- | --- |

# 1.8	Installing ISA Cards

Both PnP and non-PnP ISA cards require specific IRQs.  When installing ISA cards, make sure that the IRQs required by these cards are not previously assigned to PCI devices to avoid resource conflicts.

Follow these steps when installing ISA cards:

1.	Remove all PnP cards installed in the system, if any.

2.	Enter BIOS utility and set the Reset Resources Assignment parameter to Yes to clear the resource data assigned to the PnP devices.  Refer to section 2.4.5.

3.	Install non-PnP ISA cards.

4.	Turn on the system.

5.	Use Windows 95 or ICU to manually assign the appropriate IRQs to the cards.  This ensures that BIOS will not use the resources assigned to the non-PnP ISA cards.

|  | BIOS detects and configures only PnP cards. |
| --- | --- |

6.	Turn off the system.

7.	Install PnP ISA and PCI cards.

8.	Turn on the system.  This time PnP BIOS automatically configures the PnP ISA and PCI cards with the remaining free IRQs.

# 1.9	Post-installation Instructions

Observe the following after installing a system component:

1.	See to it that the components are installed according to the step-by-step instructions in their respective sections.

2.	Make sure you have set all the required jumpers.  See section 1.5 for the correct jumper settings.

3.	Replace any expansion boards or peripherals that you removed earlier.

4.	Replace the system cover.

5.	Connect the necessary cables and turn on the system.

# 1.10	Error Messages

In the event that you receive an error message, do not continue using the computer.  Note the message and take corrective action immediately.  This section describes the different types of error messages and suggests corrective measures.

There are two general types of error messages:

- Software

- System

## 1.10.1	Software Error Messages

Software error messages are returned by your operating system or application.  These messages typically appear after you boot the operating system or when you run your applications.  If you receive this type of message, consult your application or operating system manual for help.

## 1.10.2	System Error Messages

A system error message indicates a problem with the computer itself.  These messages normally appear during the power-on self-test, before the operating system prompt appears.  Table 1-8 lists the system error messages in alphabetical order.

Table 1-8	System Error Messages

| Error Message | Corrective Action |
| --- | --- |
| Bad CMOS Battery | Replace battery. Contact your dealer. |
| CMOS Checksum Error | Run Setup. |
| Diskette Drive Controller Error | Check and connect the cable to the diskette drive or controller. |
| Diskette Drive Error | Diskette may be bad. If not, check the diskette drive and replace if necessary. |
| DRAM Configuration Error | Check and modify DRAM configuration to agree with Table 1-7. |
| Equipment Configuration Error | Run Setup. |
| Hard Disk Controller Error | Check and connect the cable to the hard disk drive or controller. |
| Hard Disk 0 Error | Check all cable connections. Check the hard disk and replace if necessary. |
| Hard Disk 1 Error | Check all cable connections. Check the hard disk and replace if necessary. |
| Hard Disk 0 Extended Type Error | Run Setup. |
| Hard Disk 1 Extended Type Error | Run Setup. |
| I/O Parity Error | Contact your dealer. |
| Keyboard Error or No Keyboard Connected | Check and connect the keyboard to the system unit. |
| Keyboard Interface Error | Contact your dealer. |

Table 1-8	System Error Messages  (continued)

| Error Message | Corrective Action |
| --- | --- |
| Keyboard Locked | Unlock the keyboard. |
| Memory Error | Check SIMMs on the system board. Contact your dealer. |
| Memory Size Mismatch | Run Setup. |
| Serial 1 Conflict | Run Setup. Disable Onboard Serial 1. |
| Serial 2 Conflict | Run Setup. Disable Onboard Serial 2. |
| Parallel Port Conflict | Run Setup. Disable Onboard Parallel Port. |
| Pointing Device Error | Check or connect the pointing device. Contact your dealer. |
| Pointing Device Interface Error | Contact your dealer. |
| Press F1 key to continue or Ctrl-Alt-Esc for Setup | Press or . |
| Press Esc to turn off NMI, any key to reboot | Press to disregard NMI error. Press any key to reboot the system. |
| Protected Mode Test Fail | Contact your dealer. |
| RAM BIOS Error | Contact your dealer. |
| Real Time Clock Error | Run Setup. |
| Shadow RAM Fail | Contact your dealer. |
| System Memory Address Error | Check SIMMs on system board or contact your dealer. |

## 1.10.3	Correcting Error Conditions

As a general rule, the "Press F1 to continue" error message is caused by a configuration problem which can be easily corrected.  An equipment malfunction is more likely to cause a fatal error, i.e., an error that causes complete system failure.

Here are some corrective measures for error conditions:

1.	Run Setup.  You must know the correct configuration values for your system before you enter Setup, which is why you should write these values down when the system is correctly configured.  An incorrect Setup configuration is a major cause of power-on error messages, especially for a new system.

2.	Remove the system cover according to the directions in the system housing installation guide.  Check that the system board and any expansion boards are set correctly.

3.	If you cannot access a new disk, it may be because your disk is not physically formatted.  Physically format the disk using the FDISK and FORMAT commands.

4.	Check that all connectors and boards are secure.  Consult the system housing installation guide for assistance.

If you follow the corrective steps above and still receive an error message, the cause may be an equipment malfunction.

If you are sure that your configuration values are correct and your battery is in good condition, the problem may lie in a damaged or defective chip.  Contact an authorized service center for assistance.

* 	Default setting