The V55LA-2 is a high-performance system board with a 64-bit architecture.  It supports the Intel P54C and P55C CPUs running at 75/90/100/120/133/150/166/200 MHz.  It also supports Cyrix M1/M2 and AMD K5/K6 CPUs.  The system board utilizes the Peripheral Component Interconnect (PCI) local bus architecture that maximizes the system performance by enabling high-speed peripherals to match the speed of the microprocessor with its 120 MB or 132 MB per second transfer rate in burst mode.

The board incorporates a Sound Blaster Pro-compatible audio subsystem that consists of CD-audio, WaveTable, and fax/modem line-in interfaces.

A standard 1-MB video DRAM comes onboard and is upgradable up to 2 MB.  Two DRAM banks composed of four 72-pin sockets come with the board to support single- and double-density SIMMs for a maximum system memory of 128 MB.  The SIMM sockets accommodate both the standard page mode and extended data output (EDO) type SIMMs.  The board also supports 256-KB or 512-KB pipeline burst second-level cache.

The system board includes a 188-pin connector for the slot board that contains the PCI and ISA bus slots.  The two onboard PCI-enhanced IDE interfaces with a zero-wait state and 16.6 MB per second transfer rate support up to four IDE devices.  Onboard I/O interfaces comprise of two UART 16550 serial ports, a parallel port with ECP/EPP feature, and PS/2 keyboard and mouse ports.

# 1.1	Major Features

The system board has the following major features:

- A zero-insertion force (ZIF) socket for Intel P54C and P55C, Cyrix M1/M2, or AMD K5/K6

- Two DRAM banks composed of four 72-pin SIMM sockets that support 4/8/16/32-MB  60/70ns SIMMs

- 256-KB or 512-KB write-back pipeline burst second-level cache (manufacturing option)

- 256-KB boot block mode Flash ROM for system BIOS, VGA BIOS, and PnP ESCD

- Two PCI-enhanced IDE interfaces that support up to four IDE devices

- System clock/calendar with 256-byte CMOS RAM

- Interfaces for CD-audio, fax/voice modem, and WaveTable support

- Standard 1-MB video DRAM onboard plus two upgrade sockets for up to 2-MB video memory

- 188-pin connector for PCI/ISA slot board

- Feature socket for multimedia or Ethernet solution

- External ports:

- PS/2 keyboard and mouse ports

- Two buffered high-speed serial ports

- One ECP/EPP high-speed parallel port

- Video port

## 1.1.1	System Board

Figure 1-1 shows the locations of the system board major components.

| 1 Battery 2 Tag SRAM 3 Pipeline burst second-level cache 4 Pentium CPU socket 5 CPU voltage regulators 6 SIMM sockets 7 Buzzer 8 Switch 2 (SW2) 9 Video RAM upgrade sockets 10 Video RAM | 11 Video port 12 Parallel port 13 BIOS 14 Serial port 2 15 Slot board connector 16 Serial port 1 17 Mouse port 18 Keyboard port 19 USB connector |
| --- | --- |

Figure 1-1	System Board Layout

## 1.1.2	Slot Boards

The system board comes with a slot board already installed.  The slot board carries the PCI and ISA bus slots for system enhancements and future expansion.

The slot board may vary in size and layout depending on your system housing.  Figures 1-2 and 1-3 show the two kinds of slot boards.

Figure 1-2	2-PCI/3-ISA Slot Board (for desktop systems)

Figure 1-3	3-PCI/5-ISA Slot Board (for minitower systems)

# 1.2	Jumpers and Connectors

## 1.2.1	Jumper and Connector Locations

Figure 1-4 shows the jumper and connector locations on the system board.

Figure 1-4	System Board Jumper and Connector Locations

|  | The blackened pin of a jumper represents pin 1. |
| --- | --- |

## 1.2.2	Jumper Settings

Table 1-1 lists the system board jumpers with their corresponding settings and functions.

Table 1-1	System Board Jumper Settings

| Jumper | Setting | Function |
| --- | --- | --- |
| BIOS Type JP1 | 1-2 2-3 | For models with Acer BIOS For models with OEM BIOS |
| LED Function JP2 | 1-2 2-3 | LED for IDE and FDD LED for IDE only |
| Second-level Cache JP3, JP4 | 1-2, 1-2 1-2, 2-3 2-3, 2-3 | 256 KB 512 KB 1 MB |
| Regulator JP7 | Closed Open | For single-voltage CPUs (P54C, K5, M1) For dual-voltage CPUs (P55C, K6, M2) |
| SMM/Reset Switch JP11 | 1-2 2-3 | CN19 pins 19-20 support SMM switch Reserved |
| Software Shutdown JP16 | 1-2 2-3 | UPS enabled UPS disabled |
| CPU Voltage JP43 (for I/O) JP44 (for core) | 1-2 2-3 1-2 2-3 | 3.5V 3.3V 3.2V 2.8V |

Tables 1-2 to 1-4 show the settings of DIP switch 2 (SW2), JP7, JP43, and JP44 for CPU selection.

Table 1-2	SW2 Settings (CPU Selection)

| CPU Freq. (MHz) | Host Bus Freq. (MHz) | SW1 SW2 SW3 SW4 | JP7 | JP43 | JP44 |
| --- | --- | --- | --- | --- | --- |
| Intel P54C |  |  |  |  |  |
| P75 | 50 | On On Off Off | Closed | 2-3 | 2-3 |
| P90 | 60 | On Off Off Off | Closed | 2-3 | 2-3 |
| P100 | 66 | Off On Off Off | Closed | 2-3 | 2-3 |
| P120 | 60 | On Off On Off | Closed | 2-3 | 2-3 |
| P133 | 66 | Off On On Off | Closed | 2-3 | 2-3 |
| P150 | 60 | On Off On On | Closed | 2-3 | 2-3 |
| P166 | 66 | Off On On On | Closed | 2-3 | 2-3 |
| P200 | 66 | Off On Off On | Closed | 2-3 | 2-3 |
| Intel P55C |  |  |  |  |  |
| P150 | 60 | On Off On On | Open | 2-3 | 2-3 |
| P166 | 66 | Off On On On | Open | 2-3 | 2-3 |
| P200 | 66 | Off On Off On | Open | 2-3 | 2-3 |
| Cyrix M1 (6x86) |  |  |  |  |  |
| P120+ | 50 | On On On Off | Closed | 2-3 | 2-3 |
| P150+ | 60 | On Off On Off | Closed | 2-3 | 2-3 |
| P166+ | 66 | Off On On Off | Closed | 2-3 | 2-3 |
| Cyrix M1 (6x86L) |  |  |  |  |  |
| P120+ | 50 | On On On Off | Open | 2-3 | 2-3 |
| P150+ | 60 | On Off On Off | Open | 2-3 | 2-3 |
| P166+ | 66 | Off On On Off | Open | 2-3 | 2-3 |
| Cyrix M2 |  |  |  |  |  |
| PR166 | 66 | Off On On On | Open | 2-3 | 2-3 |
| PR180 | 60 | On Off Off On | Open | 2-3 | 2-3 |
| PR200 | 66 | Off On Off On | Open | 2-3 | 2-3 |

Table 1-2	SW2 Settings (CPU Selection) - continued

| CPU Freq. (MHz) | Host Bus Freq. (MHz) | SW1 SW2 SW3 SW4 | JP7 | JP43 | JP44 |
| --- | --- | --- | --- | --- | --- |
| AMD K5 |  |  |  |  |  |
| PR75 | 50 | On On Off Off | Closed | 1-2 | 2-3 |
| PR90 | 60 | On Off Off Off | Closed | 1-2 | 2-3 |
| PR100 | 66 | Off On Off Off | Closed | 1-2 | 2-3 |
| PR120 | 60 | On Off On Off | Closed | 1-2 | 2-3 |
| PR133 | 66 | Off On On Off | Closed | 1-2 | 2-3 |
| PR166 | 66 | Off On On On | Closed | 1-2 | 2-3 |
| AMD K6 |  |  |  |  |  |
| PR166 | 66 | Off On On On | Open | 2-3 | 1-2 |
| PR200 | 66 | Off On Off On | Open | 2-3 | 1-2 |
| PR233 | 66 | Off On Off Off | Open | 2-3 | 1-2 |

Table 1-3	SW2 Settings (Onboard Sound Chip)

| Setting 5 | Function |
| --- | --- |
| ON | Onboard sound chip disabled |
| OFF | Onboard sound chip enabled |

Table 1-4	SW2 Settings (Password Security)

| Setting 6 | Function |
| --- | --- |
| ON | Password bypass |
| OFF | Password check |

## 1.2.3	Connector Functions

Table 1-5 lists the different connectors on the system board and their respective functions.

Table 1-5	Connector Functions

| Connector | Function |
| --- | --- |
| CN1 | Feature connector for multimedia or Ethernet solution |
| CN2 | CD-audio line-in connector |
| CN3 | Power connector |
| CN5 | USB connector |
| CN6 | WaveTable connector |
| CN7 | PS/2 keyboard connector |
| CN8 | PS/2 mouse connector |
| CN9 | Audio I/O board connector |
| CN10 | Diskette drive connector |
| CN11 | IDE connector 2 |
| CN12 | IDE connector 1 |
| CN13 | Serial port 1 |
| CN14 | Serial port 2 |
| CN17 | Parallel port |
| CN18 | CPU fan connector |
| CN19 | Multifunction connector |
| CN21 | Video port |
| CN22 | Fax/modem connector |
| CN23 | ATI multimedia connector |

Table 1-5	Connector Functions  (continued)

| Connector | Function |
| --- | --- |
| CN31 | Internal line-in connector |
| JP10 | Hard disk and diskette drive connector |
| JP14 | Power-on switch connector (pins 1-2) |
| JP15 | Standby power connector |
| JP30 | External battery connector |

Figure 1-5 shows the cables you can connect to the multifunction connector CN19.

Figure 1-5	20-pin Multifunction Connector (CN19)

# 1.3	ESD Precautions

Always observe the following electrostatic discharge (ESD) precautions before installing a system component:

1.	Do not remove a component from its antistatic packaging until you are ready to install it.

2.	Wear a wrist grounding strap before handling electronic components.  Wrist grounding straps are available at most electronic component stores.

|  | Do not attempt the procedures described in the following sections unless you are a qualified technician. |
| --- | --- |

# 1.4	Memory Upgrade

The system board comes with four 72-pin SIMM sockets that support 4-MB and 16-MB single-density SIMMs and 8-MB and 32-MB double-density SIMMs.  Table 1-6 lists the possible 64-bit memory configurations.

Table 1-6	Memory Configurations (64-bit)

| Bank 0 |  | Bank 1 |  | Total |
| --- | --- | --- | --- | --- |
| SIMM-1 | SIMM-2 | SIMM-3 | SIMM-4 | Memory |
| 4 MB | 4 MB |  |  | 8 MB |
|  |  | 4 MB | 4 MB | 8 MB |
| 8 MB | 8 MB |  |  | 16 MB |
|  |  | 8 MB | 8 MB | 16 MB |
| 4 MB | 4 MB | 4 MB | 4 MB | 16 MB |
| 4 MB | 4 MB | 8 MB | 8 MB | 24 MB |
| 8 MB | 8 MB | 4 MB | 4 MB | 24 MB |
| 8 MB | 8 MB | 8 MB | 8 MB | 32 MB |
| 16 MB | 16 MB |  |  | 32 MB |
|  |  | 16 MB | 16 MB | 32 MB |
| 4 MB | 4 MB | 16 MB | 16 MB | 40 MB |
| 16 MB | 16 MB | 4 MB | 4 MB | 40 MB |
| 8 MB | 8 MB | 16 MB | 16 MB | 48 MB |
| 16 MB | 16 MB | 8 MB | 8 MB | 48 MB |
| 16 MB | 16 MB | 16 MB | 16 MB | 64 MB |
| 32 MB | 32 MB |  |  | 64 MB |
|  |  | 32 MB | 32 MB | 64 MB |
| 4 MB | 4 MB | 32 MB | 32 MB | 72 MB |
| 32 MB | 32 MB | 4 MB | 4 MB | 72 MB |
| 8 MB | 8 MB | 32 MB | 32 MB | 80 MB |
| 32 MB | 32 MB | 8 MB | 8 MB | 80 MB |
| 16 MB | 16 MB | 32 MB | 32 MB | 96 MB |
| 32 MB | 32 MB | 16 MB | 16 MB | 96 MB |
| 32 MB | 32 MB | 32 MB | 32 MB | 128 MB |

The system also supports 32-bit memory configurations.  This feature allows you to install only one SIMM in one configuration.  Table 1-7 shows the 32-bit configurations.

Table 1-7	Memory Configurations (32-bit)

| Bank 0 |  | Bank 1 |  | Total |
| --- | --- | --- | --- | --- |
| SIMM-1 | SIMM-2 | SIMM-3 | SIMM-4 | Memory |
| 4 MB* |  |  |  | 4 MB |
| 4 MB | 4 MB | 4 MB |  | 12 MB |
| 4 MB | 4 MB | 8 MB |  | 16 MB |
| 4 MB | 4 MB | 16 MB |  | 24 MB |
| 4 MB | 4 MB | 32 MB |  | 40 MB |
| 8 MB* |  |  |  | 8 MB |
| 8 MB | 8 MB | 4 MB |  | 20 MB |
| 8 MB | 8 MB | 8 MB |  | 24 MB |
| 8 MB | 8 MB | 16 MB |  | 32 MB |
| 8 MB | 8 MB | 32 MB |  | 48 MB |
| 16 MB* |  |  |  | 16 MB |
| 16 MB | 16 MB | 4 MB |  | 36 MB |
| 16 MB | 16 MB | 8 MB |  | 40 MB |
| 16 MB | 16 MB | 16 MB |  | 48 MB |
| 16 MB | 16 MB | 32 MB |  | 64 MB |
| 32 MB* |  |  |  | 32 MB |
| 32 MB | 32 MB | 4 MB |  | 68 MB |
| 32 MB | 32 MB | 8 MB |  | 72 MB |
| 32 MB | 32 MB | 16 MB |  | 80 MB |
| 32 MB | 32 MB | 32 MB |  | 96 MB |

## 1.4.1	Installing a SIMM

Follow these steps to install a SIMM:

1.	Carefully slip a SIMM at a 45° angle into a socket making sure that the curved edge indicating the pin 1 of the SIMM matches pin 1 of the socket.

|  | A SIMM fits only in one direction. If you slip in a SIMM but would not completely fit, you may have inserted it the wrong way. Reverse the orientation of the SIMM. |
| --- | --- |

2.	Gently push the SIMM to a vertical position until the pegs of the socket slip into the holes on the SIMM, and the holding clips lock the SIMM into position.  The SIMM should be at a 90° angle when installed.

Hole

Peg

Pin 1 Indicator (curved edge)

1

2

Figure 1-6	Installing a SIMM

## 1.4.2	Removing a SIMM

Follow these steps to remove a SIMM:

1.	Press the holding clips on both sides of the SIMM outward to release it.

2.	Move the SIMM to a 45° angle.

Holding Clip

3.	Pull the SIMM out of the socket.

1

2

3

Figure 1-7	Removing a SIMM

|  | Always remove SIMMs from the socket labeled SIMM-4, then SIMM-3, and so on. |
| --- | --- |

## 1.4.3	Reconfiguring the System

You must enter Setup after installing or removing SIMMs to reconfigure the system.

Follow these steps to reconfigure the system:

1.	Turn the system on.  A memory error message appears, indicating that the total memory does not match the value stored in CMOS.

2.	Press  +  +  to enter Setup.  A warning message appears indicating an incorrect memory configuration.

3.	Press  twice to exit and reboot the system.

The system boots with the new memory configuration.

# 1.5	IDE Hard Disk Support

The system board supports four IDE hard disks, or any other IDE devices, through the two onboard PCI IDE interfaces.  See Figure 1-1 for the location.

Follow the instructions in the housing installation manual on how to install a hard disk in the system.  Connect the cables according to the IDE hard disk configuration in Table 1-8.

Table 1-8	IDE Hard Disk Configuration

| IDE Connector | Master | Slave |
| --- | --- | --- |
| Channel 1 | Hard disk 0 | Hard disk 1 |
| Channel 2 | Hard disk 2 | Hard disk 3 |

# 1.6	CPU Installation

The system board comes with a zero-insertion force (ZIF) CPU socket for easy installation.

Follow these steps to install a Pentium CPU:

1.	Lift up the socket lever.

2.	Insert the CPU to the socket.  Make sure that the notched corner of the CPU matches the pin 1 indicator on the socket.

|  | Be careful not to bend any pins. |
| --- | --- |

3.	Pull down the socket lever.

STEP  1

STEP  2

STEP  3

Notched Corner

Pin 1 Indicator

Figure 1-8	Installing a Pentium CPU

4.	Set the CPU jumpers accordingly.  Refer to Table 1-2.

# 1.7	Video Memory Upgrade

Larger video memory allows you to display higher resolutions and more colors.  The system board comes with a 1-MB video memory onboard upgradable to 2 MB.

Follow these steps to upgrade the video memory:

1.	Locate the video DRAM upgrade sockets labeled U39 and U40 on the system board.  See Figure 1-1.

2.	Gently insert a 256K x 16, 60 ns EDO SOJ) chip into each of the upgrade sockets.

|  | Make sure that the pin 1 indicator on the chip matches the notched corner of the socket. |
| --- | --- |

Pin 1 Indicator

Notched Corner

Figure 1-9	Installing a Video Memory Chip

# 1.8	Second-level Cache Configuration

The system board supports either 256-KB or 512-KB pipeline burst second-level cache.  The cache size onboard is a manufacturing option.

Table 1-9 shows the second-level cache configurations.

Table 1-9	Second-level Cache Configurations

| Cache Size Onboard | Cache Type |
| --- | --- |
| 256 KB | 32K * 32 (7 ns) x 2 |
| 512 KB | 64K * 32 (6 ns) x 2 |

# 1.9	Audio Features

The system board supports Sound Blaster Pro-compatible sound system.  It has four connectors onboard to accommodate the audio-I/O board, WaveTable, CD-ROM, and fax-voice modem.

|  | You may disable the audio feature in the BIOS Utility. |
| --- | --- |

Figure 1-9 shows the four audio connectors on the system board.

CN9

CN6

CN2

CN22

1	Audio-I/O board connector (CN9)
2	WaveTable connector (CN6)
3	CD audio connector (CN2)
4	Fax/voice modem connector (CN22)

Figure 1-10	Audio Connectors Onboard

## 1.9.1	Installing the Audio-I/O Board

The connector CN9 on the system board accommodates the audio-I/O board.  Figure 1-10 shows how to install the board.

CN9

Figure 1-11	Installing the Audio-I/O Board

## 1.9.2	Audio-I/O Board Features

The audio-I/O board consists of microphone port, line-in port, line-out port, and MIDI/game port.  These ports accommodate the external audio devices.

Microphone Connector

Line-in Connector

Microphone Port

Line-in Port

Line-out Port

MIDI/Game Port

Figure 1-12	Audio-I/O Board

The internal connectors on the audio-I/O board are useful for system housings with built-in speakers, microphone, amplifier, or auxiliary devices.  With the internal audio connectors functioning exactly as the external audio ports, you can utilize all the sound features supported by the system board.

Since external speakers, microphone, and other audio devices give better sound quality, the external audio ports take higher priority than the internal connectors.  When you attach external audio devices to the ports, the internal audio devices are automatically disabled.

## 1.9.3	WaveTable Daughterboard (Optional)

|  | The system board supports a WableTable daughterboard as option. The WaveTable does not come with the basic system. |
| --- | --- |

The WaveTable daughterboard supports the same external connectors as the audio-I/O board but comes with enhanced audio features.  It bundles the Crystal chipsets that work together to produce better sound quality.

Figure 1-12 shows the WaveTable daughterboard layout.

Microphone Port

Line-in Port

Line-out Port

MIDI/Game Port

Figure 1-13	WaveTable Daughterboard

The WaveTable has two connectors, CN5 and CN6, located underside.  Match these two connectors with the audio-I/O connector (CN9) and WaveTable connector (CN6) on the system board.

Figure 1-14 shows how to install the WaveTable connector.

CN9

CN6

Figure 1-14	Installing the WaveTable Daughterboard

## 1.9.4	Audio-I/O Devices

Figure 1-15 shows the various devices that you can connect to the audio-I/O board.

Microphone

Synthesizer

CD Player

Stereo Amplifier

Head phones

Speakers

MIDI Adapter

Joystick

Figure 1-15	Audio I/O External Devices

# 1.10	Installing ISA Cards

Both PnP and non-PnP ISA cards require specific IRQs.  When installing ISA cards, make sure that the IRQs required by these cards are not previously assigned to PCI devices to avoid resource conflicts.

Follow these steps when installing ISA cards:

- Remove all PnP cards installed in the system, if any.

- Enter BIOS utility and set the Reset Resource Assignment parameter to Yes to clear the resource data assigned to the PnP devices.  Refer to section 2.6.5.

- Install non-PnP ISA cards.

- Turn on the system.

- Use Windows 95 or ICU to manually assign the appropriate IRQs to the cards.  This ensures that BIOS will not use the resources assigned to the non-PnP ISA cards.

|  | BIOS detects and configures only PnP cards. |
| --- | --- |

- Turn off the system.

- Install PnP ISA and PCI cards.

- Turn on the system.  This time PnP BIOS automatically configures the PnP ISA and PCI cards with the remaining free IRQs.

# 1.11	Error Messages

Do not continue using the computer if you receive an error message of any type.  Note the message and take corrective action.  This section explains the different types of error messages and corresponding corrective measures.

There are two general types of error messages:

- Software

- System

## 1.11.1	Software Error Messages

Software error messages are returned by your operating system or application.  These messages typically occur after you boot the operating system or when you run your applications.  If you receive this type of message, consult your application or operating system manual for help.

## 1.11.2	System Error Messages

A system error message indicates a problem with the computer itself.  A message of this type normally appears during the power-on self-test, before the operating system prompt appears.

Table 1-10 lists the system error messages.

Table 1-10	System Error Messages

| Message | Action |
| --- | --- |
| CMOS Battery Error | Replace the RTC chip or contact your dealer. |
| CMOS Checksum Error | Check the RTC chip and the necessary jumper. If the battery is still good, run Setup. |
| Display Card Mismatch | Run Setup |
| Diskette Drive Controller Error or Not Installed | Check and connect the control cable to the diskette controller. |
| Diskette Drive Error | Diskette may be defective. If not, replace the diskette drive. |
| Diskette Drive A Type Mismatch | Run Setup and select the proper drive type. |
| Diskette Drive B Type Mismatch | Run Setup and select the proper drive type. |
| Equipment Configuration Error | Modify the memory configuration to agree with one of the options in Tables 1-7 or 1-8. |
| Hard disk Controller Error | Run Setup. |
| Hard disk 0 Error | Check all cable connections. Replace hard disk. |
| Hard disk 1 Error | Check all cable connections. Replace hard disk. |
| Keyboard Error or No Keyboard Connected | Check and connect the keyboard to the system unit. |
| Keyboard Interface Error | Replace the keyboard or contact your dealer. |

Table 1-10	System Error Messages  (continued)

| Message | Action |
| --- | --- |
| Memory Error at: MMMM:SSSS:OOO (W:XXXX, R:YYYY) where: M: MB, S: Segment, O: Offset, X/Y: write/read pattern | Check SIMMs on the system board. Contact your dealer. |
| CPU Clock Mismatch | Run Setup. Check if the CPU clock is correct. If correct, exit Setup and reboot the system. If the error message reappears, ask for technical assistance. |
| Onboard Serial Port 1 Conflict | Run Setup and disable the port. |
| Onboard Serial Port 2 Conflict | Run Setup and disable the port. |
| Onboard Parallel Port Conflict | Run Setup and disable the port. |
| Pointing Device Error | Check and connect pointing device. |
| Pointing Device Interface Error | Replace the pointing device or contact your dealer. |
| Press key to continue or ++ for Setup | Press or ++ to enter Setup. |
| Real Time Clock Error | Check the RTC chip. If it is still good, run Setup. If not, replace the RTC chip. |

## 1.11.3	Correcting Error Conditions

As a general rule, if an error message says "Press F1 to continue," it is caused by a configuration problem, which can be easily corrected.  An equipment malfunction is more likely to cause a fatal error, i.e., an error that causes complete system failure.

Here are some corrective measures for error conditions:

1.	Run Setup.  You must know the correct configuration values for your system before you enter Setup, which is why you should write them down when the system is correctly configured.  An incorrect configuration is a major cause of power-on error messages, especially for a new system.

2.	Remove the system unit cover.  Check that the jumpers on the system board and any expansion boards are set correctly.

3.	If you cannot access a new disk, it may be because your disk is not properly formatted.  Format the disk first using the FDISK and FORMAT commands.

4.	Check that all connectors and boards are securely plugged in.

If you go through the corrective steps above and still receive an error message, the cause may be an equipment malfunction.

If you are sure that your configuration values are correct and your battery is in good condition, the problem may lie in a damaged or defective chip.

In both cases, contact an authorized service center for assistance.

*	May also be installed in SIMM-2, SIMM-3, or SIMM-4.