Most systems are already configured by the manufacturer or the dealer.  There is no need to run Setup when starting the computer unless you get a Run Setup message.

The Setup program loads configuration values into the battery-backed nonvolatile memory called CMOS RAM.  This memory area is not part of the system RAM.

|  | If you repeatedly receive Run Setup messages, the battery may be bad. In this case, the system cannot retain configuration values in CMOS. Ask a qualified technician for assistance. |
| --- | --- |

Before running Setup, have the following information ready:

- Diskette drive type  The standard type is either a 5.25-inch 1.2-MB or a 3.5-inch 1.44-MB diskette drive.

- IDE hard disk drive type  The drive information is on a label pasted on your IDE drive or in the documentation supplied by the vendor.  Refer also to the IDE drive table at the end of this chapter.

# 2.1	Entering Setup

To enter Setup, press the key combination ++ .

|  | You must press ++ while the system is booting. This key combination does not work during any other time. |
| --- | --- |

The BIOS Utility main menu then appears:

| BIOS Utility |  |  |  |
| --- | --- | --- | --- |
| Basic System Configuration Advanced System Configuration Power Saving Configuration System Security PCI System Configuration Load Setup Default Settings |  |  |  |
| ¯ = Move Highlight Bar, ¬ = Select, Esc = Exit and Reboot |  |  |  |
|  |  | The parameters on the screens show default values. These values may not be the same as those in your system. The grayed items (denoted with asterisks) on the screens have fixed settings and are not user-configurable. |  |

# 2.2	Basic System Configuration

Select Basic System Configuration to input configuration values such as date, time, and disk types.

The following screen shows the Basic System Configuration menu.

Basic System Configuration Page 1/2

Date [MM/DD/YY] Time [HH:MM:SS] Diskette Drive A [xx-MB xx-inch] Diskette Drive B [xx-MB xx-inch] Cylinder Head Sector IDE Drive 0 (xxx MB) [Auto] xx xx xx IDE Drive 1 (xxx MB) [Auto] xx xx xx IDE Drive 2 [Auto] IDE Drive 3 [Auto] *Base Memory [ xxx] KB *Extended Memory [ xxxx] KB *Total Memory [ xxxx] KB *Math Coprocessor [ Installed ] *Video Display [VGA/EGA]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

The command line at the bottom of the menu tells you how to highlight items, change settings, and move from one screen to another.

Press  or  on the cursor-edit keypad to highlight the desired parameter.

Press  or  to select the desired option for a parameter.

Press  to move to the next page or  to return to the previous page.

Press  to exit the configuration menu.

The following screen shows page 2 of the Basic System Configuration menu.

Basic System Configuration Page 2/2

Communication Settings Baud Rate [9600] BPS Parity [None] Stop Bits [1] Bits Data Length [8] Bits Enhanced IDE Features Hard Disk Block Mode [Enabled ] Advanced PIO Mode [Enabled ] Hard Disk Size > 504MB [Enabled ] Hard Disk 32-Bit Access [Enabled ] Num Lock After Boot [Enabled ] *Memory Test [Disabled] Auto Configuration Mode [Enabled ] Fast Boot Mode [Enabled ] Quiet Boot [Enabled ] Configuration Table [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

The following sections explain the different parameters and their settings.

## 2.2.1	Date and Time

The real-time clock keeps the system date and time.  After setting the date and time, you need not enter them every time you turn on the system.  As long as the internal battery remains good (approximately seven years) and connected, the clock continues to keep the date and time accurately even when the power is off.

Date

Highlight the items on the date parameter and press  or  to set the date following the month-day-year format.

Valid values for month, day, and year are:

- Month	1  to 12

- Day	1  to 31

- Year	00  to 99

Time

Highlight the items on the time parameter and press  or  to set the time following the hour-minute-second format.

Valid values for hour, minute, and second are:

- Hour	00 to 23

- Minute	00 to 59

- Second	00 to 59

## 2.2.2	Diskette Drives

To enter the configuration value for the first diskette drive (drive A), highlight the Diskette Drive A parameter.  Press  or  key to view the options and select the appropriate value.

Possible settings for the Diskette Drive parameters:

- [        None        ]

- [360  KB,   5.25-inch]

- [1.2  MB,   5.25-inch]

- [720  KB,    3.5-inch]

- [1.44 MB,    3.5-inch]

- [2.88 MB,    3.5-inch]

Follow the same procedure for Diskette Drive B.  Choose None if you do not have a second diskette drive.

## 2.2.3	IDE Drives

Move the highlight bar to the IDE Drive 0 parameter to configure the first IDE drive (drive C).  Press  or  to display the IDE drive types with their respective values.  Select the type that corresponds to your IDE hard disk drive.  Follow the same procedure for the other IDE drives, if any.  Choose None if you do not have other drives.

### Selecting the “Auto” Option

If you do not know the exact type of your IDE hard disk drive, select the option Auto.  During the power-on self-test (POST), when the system performs self-testing and self-initialization before loading the operating system and applications, the BIOS utility automatically determines your IDE drive type.  You can see the drive type and its values when you enter the BIOS Utility.

Cylinder	Head	Sector

IDE Drive 0 (xx MB)	[Auto]	xx	xx	xx

If desired, you can save the values under the option User.

Cylinder	Head	Sector

IDE Drive 0 (xx MB)	[User]	xx	xx	xx

The next time you boot the system, the BIOS utility does not have to auto-configure your IDE drive as it detects the saved disk information during POST.

|  | We recommend that you copy the IDE disk drive values and keep them in a safe place in case you have to reconfigure the disk in the future. |
| --- | --- |

Follow the same procedure to auto-configure other IDE drives.

### Selecting the “User” Option

There are cases when you cannot use the option Auto, instead you have to select User.  Choose the User option when you have installed an IDE hard disk that was previously formatted but does not use the disk native parameters or structure, that is, the disk type may be in the IDE hard disk types list but the number of cylinders, heads, and sectors differ.

Follow these steps to configure an IDE hard disk with the User option:

1.	Highlight an IDE drive parameter.

2.	Select the option User and press .

3.	Type in the number of cylinders, heads, and sectors of the drive under the appropriate columns.

|  | Be sure to have the correct IDE hard disk drive information beforehand. |
| --- | --- |

4.	Choose YES when asked if you want to save CMOS data.

## 2.2.4	System Memory

The system automatically detects the total amount of onboard memory during the POST and sets the memory parameters accordingly.  If you install additional memory, the system automatically adjusts the Total Memory parameter to display the new memory size.

## 2.2.5	Math Coprocessor

The CPU includes a math coprocessor so this parameter shows Installed  by default.

## 2.2.6	Video Display

The video display is the monitor on which the operating system prompt appears when you boot the system.  The system automatically detects the video mode of your primary display and sets the configuration value accordingly.  Values for this parameter are:

- [Monochrome]

- [CGA 40 columns x 25 rows]

- [CGA 80 columns x 25 rows]

- [VGA/EGA]

## 2.2.7	Communication Settings

The Communication Settings parameters allow you to set the baud rate, parity, stop bit and data length for the first serial port (COM 1).  The values for this parameter are:

- Baud rate	:	110 to 9600 bits per second (BPS)

- Parity	:	odd, even, or none

- Stop bit	:	1 or 2 stop bits

- Data length	:	7- or 8-bit data word

|  | The baud rate maximum value 9600 BPS applies only to POST under UNIX environment. The system I/O chipset SMC 37C665GT supports up to 115.2K BPS. |
| --- | --- |

There is one restriction on the options available for the communication status parameters.  If your data length parameter is an 8-bit data word, you must select one of the following combinations:

- 1 stop bit and odd or even parity

- 2 stop bits and no parity

## 2.2.8	Enhanced IDE Features

### Hard Disk Block Mode

This function enhances disk performance depending on the hard disk in use.  If you set this parameter to Enabled, it allows data transfer in block (multiple sectors).  If your system does not boot after enabling this parameter, change the setting to Disabled.  This parameter is normally set to Enabled.

### Advanced PIO Mode

Enabling this parameter allows for faster data recovery and read/write timing that reduces hard disk activity time.  This results to better hard disk performance.

To utilize this feature, your hard disk must support the advanced PIO mode.  If not, set this parameter to Disabled.  Refer to your hard disk documentation for information about the advanced PIO mode.

### Hard Disk Size > 504 MB

This enhanced IDE feature works only under DOS and Windows 3.x environments.  If enabled, it allows you to use a hard disk with a capacity of more than 504 MB.  This is made possible through the Logical Block Address (LBA) mode translation.  Other operating systems require this parameter to be set to Disabled.

To prevent data loss, keep this parameter set to Enabled if you are using a hard disk with more than 504 MB capacity that was previously configured through LBA mode.  When you use a hard disk configured through the cylinder-head-sector (CHS) mode, set this parameter to Disabled .

### Hard Disk 32-Bit Access

Enabling this parameter improves system performance by allowing the use of the 32-bit hard disk access.  This enhanced IDE feature works only under DOS, Windows 3.x and Novell NetWare.  If your software or hard disk does not support this function, set this parameter to Disabled .

## 2.2.9	Num Lock After Boot

This parameter allows you to activate the Num Lock function upon booting.  The default setting is Enabled.

## 2.2.10	Memory Test

When set to Enabled, this parameter allows the system to perform a RAM test during the POST routine.  When set to Disabled, the system detects only the memory size and bypasses the test routine.  The default setting is Disabled .

This item is fixed to Disabled and is not user-configurable if you enabled the Fast Boot Mode parameter on page 2 of the Basic System Configuration menu.  See section 2.2.12.

## 2.2.11	Auto Configuration Mode

When enabled, this parameter automatically sets the system configuration values to their optimized settings.  Set this parameter to the default Enabled if you do not know the hard disk drive parameters and the onboard communication port configurations.

## 2.2.12	Fast Boot Mode

When enabled, this parameter allows the system to boot faster by skipping some POST routines.  It bypasses memory test and enables the primary- and second-level cache.  This parameter is enabled by default.  When set to Enabled, this parameter causes the Memory Test parameter to be fixed to Disabled .

## 2.2.13	Quiet Boot

This parameter enables or disables the quiet boot function.  When set to Enabled, BIOS is in graphical mode and displays only an identification logo during POST and while booting.  After which the screen displays the operating system prompt (such as DOS) or logo (such as Windows 95).  If any error occurred while booting, the system automatically switches to the text mode.

Even if your setting is Enabled, you may also switch to the text mode while booting by pressing    after you hear a beep that indicates the activation of the keyboard.

When set to Disabled, BIOS is in the conventional text mode where you see the system initialization details on the screen.

## 2.2.14	Configuration Table

This parameter allows you to display the configuration table after POST but before booting.  The configuration table gives a summary of the hardware devices and settings that BIOS detected during POST.  Following is a sample configuration table.

| CPU/CLK : Pentium/133 MHz Math Coprocessor: Installed IDE Drive 0 : xxx MB IDE Drive 1 : xxx MB IDE Drive 2 : xxx MB IDE Drive 3 : xxx MB Diskette Drive A: xx-MB, xx-inch Diskette Drive B: None | Base Memory: : xxx KB Extended Memory: xxxx KB Shadow RAM : xxx KB Internal Cache : xxx KB, Enabled External Cache : xxx KB, Enabled Serial Port(s) : 3F8h, 2F8h Parallel Port : 378h Pointing Device: None |
| --- | --- |

# 2.3	Advanced System Configuration

The Advanced System Configuration option allows you to configure the advanced system memory functions.

|  | Do not change any settings in the Advanced Configuration if you are not a qualified technician to avoid damaging system. |
| --- | --- |

The following screen shows page one of the Advanced System Configuration parameters.

| Advanced System Configuration Page 1/1 |  |  |  |
| --- | --- | --- | --- |
| Shadow RAM *E0000h - FFFFFh (System BIOS) [Enabled ] *C0000h - C7FFFh (Video BIOS) [Enabled ] C8000h - CBFFFh [Disabled] CC000h - CFFFFh [Disabled] D0000h - D3FFFh [Disabled] D4000h - D7FFFh [Disabled] D8000h - DBFFFh [Disabled] DC000h - DFFFFh [Disabled] *Internal Cache (CPU Cache) [Enabled ] *External Cache [Enabled ] *Cache Scheme [ Write Back ] ECC/Parity Mode Selection [ ECC ] Memory at 15MB-16MB Reserved for [ System ] Use Plug & Play OS [Yes] |  |  |  |
| ¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit |  |  |  |
|  |  | The grayed parameters (denoted with asterisks) are not user-configurable. |  |

## 2.3.1	Shadow RAM

The system reserves 384 KB of random access memory (RAM) for the shadow RAM function.  This parameter has eight range addresses.  When you set these addresses to Enabled, the system BIOS, video BIOS, and I/O ROM functions run directly from the shadow RAM for faster operation.  When you set them to Disabled, the functions run normally from ROM.

The address range E0000h - FFFFFh is for shadowing the system BIOS.  This item is always set to Enabled and is not user-configurable.  The address range C0000h - C7FFFh is for shadowing the video BIOS.  This item is fixed to Enabled and is not user-configurable.  The remaining address ranges are for I/O ROM functions.

## 2.3.2	Internal Cache (CPU Cache)

This parameter enables or disables the internal cache memory.  This item is fixed to Enabled and is not user-configurable if you enabled the Auto configuration Mode and Fast Boot Mode parameters on page 2 of the Basic System Configuration menu.  Otherwise, you can choose to disable this item.

## 2.3.3	External Cache

This parameter enables or disables the external cache memory.  This item is fixed to Enabled and is not user-configurable if you enabled the Fast Boot Mode parameter on page 2 of the Basic System Configuration menu.  Otherwise, you can choose to disable this item.

### Cache Scheme

This parameter fixes the cache setting to Write-back mode.  Write-back updates the cache but not the memory when there is a write instruction.  It updates the memory only when these is an inconsistency between the cache and the memory.

## 2.3.4	ECC/Parity Mode Selection

This parameter allows you to enable or disable the ECC and parity check feature.  You must disable this parameter if you installed SIMMs without parity.

## 2.3.5	Memory at 15MB-16MB

To prevent memory address conflicts between the system and expansion boards, reserve this memory range for the use of either the system or an expansion board.  Before setting this parameter, check your add-on card manual to determine if your add-on card needs this memory space.  If not, set this parameter to System Use.

## 2.3.6	Plug & Play OS

Set this parameter to Yes if you installed the Windows 95 operating system.  Enabling this parameter allows Windows 95 to initialize and enable non-boot plug-and-play (PnP) devices such as audio or modem cards.  BIOS initializes only boot PnP devices such as SCSI cards.

Set this parameter to No if you do not use Windows 95.  With this parameter disabled, BIOS initializes all boot or non-boot PnP devices.

# 2.4	Power Saving Configuration

The Power Saving Configuration parameters are configurable only if your system supports the power management feature.

The following screens show the Power Saving Configuration parameters and their default settings:

Power Saving Configuration Page 1/1

Power Management Mode [Enabled ] *Monitor Power Saving Timer [15] Minute(s) *IDE Hard Disk Standby Timer [15] Minute(s) *System Standby Timer [OFF] *System Suspend Timer [OFF] Monitored Activities IRQ 2/IRQ 9 [Disabled] DRQ 0 [Disabled] IRQ 3 [Disabled] DRQ 1 [Disabled] IRQ 4 [Disabled] DRQ 3 [Disabled] IRQ 5 [Disabled] DRQ 5 [Disabled] IRQ 7 [Disabled] DRQ 6 [Disabled] IRQ 10 [Disabled] DRQ 7 [Disabled] IRQ 11 [Disabled] *IRQ 12 [Enabled ] *IRQ 15 [Enabled ]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.4.1	Power Management Mode

This parameter allows you to reduce power consumption.  When this parameter is set to Enabled, you can configure the monitor, IDE hard disk, and system timers.  Setting to Disabled deactivates the power management feature and all the timers.

### Monitor Power Saving Timer

This parameter sets the VESA DPMS-compatible display monitor to standby mode.  Any keyboard or mouse action returns the monitor to normal operation.

### IDE Hard Disk Standby Timer

This parameter allows the hard disk to enter standby mode after inactivity of 1 to 15 minutes, depending on your setting.  When you access the hard disk again, allow 3 to 5 seconds (depending on the hard disk) for the disk to return to normal speed.  Set this parameter to Off if your hard disk does not support this function.

### System Standby Timer

This parameter sets the system to a "fast-on" power saving mode.  It automatically enters the standby mode after a specified period of inactivity.  Any keyboard or mouse action, or any enabled monitored activities occurring through IRQ or DRQ channels, resume system operation.  See section 2.4.2.

### System Suspend Timer

This parameter supports the lowest power saving mode.  To resume system operation from suspend mode, you can either press a key, move the mouse or press the reset button.  If you run network, set this item to OFF.

## 2.4.2	Monitored Activities

The IRQ and DRQ items under this parameter allow you to monitor system activities occurring through the IRQ and DRQ channels to determine whether or not to enter power saving mode.

For example, if you assign IRQ 3 to a fax/modem and you set this item to Enabled, any fax/modem activity wakes up the system from standby mode.

# 2.5	System Security Setup

The Setup program has a number of security features to prevent unauthorized access to the system and its data.

Enter the Setup program and select System Security.  The following screen appears:

System Security Page 1/1

Disk Drive Control Diskette Drive [ Normal ] Hard Disk Drive [ Normal ] System Boot Drive [Drive A then C] Boot from CD-ROM [Disabled] Onboard Communication Ports Serial Port 1 Base Address [ 3F8h ] Serial Port 2 Base Address [ 2F8h ] Parallel Port Base Address [378 (IRQ 7)] Operation Mode [Enhanced Parallel Port (EPP)] Mode *ECP DMA Channel [-] Onboard PS/2 Mouse (IRQ 12) [Enabled ] Crystal Audio Chip [Enabled ] Setup Password [ None ] Power On Password [ None ]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.5.1	Disk Drive Control

The disk drive control features allow you to enable or disable the read/write functions of a disk drive.  These features can also control the diskette drive or the hard disk drive boot function to prevent loading operating systems or other programs from a certain drive while the other drives are operational.

Table 2-1 lists the drive control settings and their corresponding functions.

Table 2-1	Drive Control Settings

| Diskette Drive |  |
| --- | --- |
| Setting | Description |
| Normal | Diskette drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |
| Disabled | Disables all diskette functions |
| Hard Disk Drive |  |
| Setting | Description |
| Normal | Hard disk drive functions normally |
| Write Protect All Sectors | Disables the write function on all sectors |
| Write Protect Boot Sector | Disables the write function only on the boot sector |
| Disabled | Disables all hard disk functions |
| System Boot Drive |  |
| Setting | Description |
| Drive A then C | The system checks drive A first. If there is a diskette in the drive, the system boots from drive A. Otherwise, it boots from drive C. |
| Drive C then A | The system checks drive C first. If there is a hard disk (drive C) installed, the system boots from drive C. Otherwise, it boots from drive A. |
| C: | The system always boots from drive C. |
| A: | The system always boots from drive A. |

Table 2-1	Drive Control Settings  (continued)

| Boot from CD-ROM |  |
| --- | --- |
| Setting | Description |
| Enabled | The system for a bootable CD in the CD-ROM. If a CD is present, the system boots from the CD-ROM. Otherwise, it boots from the drive specified in the System Boot drive parameter. |
| Disabled | System boots from the drive specified in the System Boot Drive parameter. |

## 2.5.2	Onboard Communication Ports

### Serial Port 1 Base Address

This parameter allows you to set the serial port 1 logical base address.

Table 2-2	Serial Port 1 Settings

| Setting | Description |
| --- | --- |
| 3F8h | Serial port 1 with address 3F8h using IRQ4 |
| 2F8h | Serial port 1 with address 2F8h using IRQ3 |
| 3E8h | Serial port 1 with address 3E8h using IRQ4 |
| 2E8h | Serial port 1 with address 2E8h using IRQ3 |
| Disabled | Disables serial port 1 |

### Serial Port 2 Base Address

This parameter allows you to set the serial port 2 logical base address.

Table 2-3	Serial Port 2 Settings

| Setting | Description |
| --- | --- |
| 3F8h | Serial port 2 with address 3F8h using IRQ4 |
| 2F8h | Serial port 2 with address 2F8h using IRQ3 |
| 3E8h | Serial port 2 with address 3E8h using IRQ4 |
| 2E8h | Serial port 2 with address 2E8h using IRQ3 |
| Disabled | Disables serial port 2 |

|  | If you assign 3F8h to serial port 1, you may only assign 2F8h or 2E8h to serial port 2. If you assign 2F8h to serial port 1, you may only assign 3F8h or 3E8h to serial port 2. |
| --- | --- |

### Parallel Port Base Address

The system has one parallel port.  Table 2-4 lists the options for selecting the parallel port address.  You also have the option to disable the parallel port.

Table 2-4	Parallel Port Settings

| Setting | Function |
| --- | --- |
| 3BCh (IRQ 7) | Corresponds to the parallel port with address 3BCh |
| 378h (IRQ 7) | Corresponds to the parallel port with address 378h |
| 278h (IRQ 5) | Corresponds to the parallel port with address 278h |
| Disabled | Disables the parallel port |

To deactivate the parallel port, select the Disabled option.  If you install an add-on card that has a parallel port whose address conflicts with the parallel port onboard, the system automatically disables the onboard functions.

Check the parallel port address on the add-on card and change the address to one that does not conflict.

#### Operation Mode

This item allows you to set the operation mode of the parallel port.  Table 2-5 lists the different operation modes.

Table 2-5	Parallel Port Operation Mode Settings

| Setting | Function |
| --- | --- |
| Standard Parallel Port (SPP) | Allows normal speed one-way operation |
| Standard and Bidirectional | Allows normal speed operation in a two-way mode |
| Enhanced Parallel Port (EPP) | Allows bidirectional parallel port operation at maximum speed |
| Extended Capabilities Port (ECP) | Allows parallel port to operate in bidirectional mode and at a speed higher than the maximum data transfer rate |

#### ecp dma channel

This item becomes active only if you select Extended Capabilities Port (ECP) as the operation mode.  It allows you to select DMA channel 1 or DMA channel 3 (as required in Windows95).

## 2.5.3	Onboard PS/2 Mouse (IRQ 12)

This parameter enables or disables the onboard PS/2 mouse.  When set to Enabled, it allows you to use the onboard PS/2 mouse assigned with IRQ12.  When set to Disabled, it deactivates the mouse and frees IRQ12 for the use of other devices.

## 2.5.4	Crystal Audio Chip

This parameter allows you to enable or disable the onboard Crystal 4236 sound chip.  The default setting is Enabled .  If you install a sound card, disable this parameter to avoid conflict with the onboard chip.

## 2.5.5	Setup Password

The Setup Password prevents unauthorized access to the BIOS utility.

|  | Set jumper JP3 to pins 1-2 to enable the password function. |
| --- | --- |

### Setting a Password

1.	Highlight the Setup Password parameter and press the left- or right-arrow key.  The password prompt appears:

2.	Type a password.  The password may consist of up to seven characters.

|  | Exercise caution when typing your password because the characters do not appear on the screen. |
| --- | --- |

3.	Press .  A prompt asks you to retype the password to verify your first entry.

4.	Retype the password then press .

After setting the password, the system automatically sets the Setup Password parameter to Present.  The next time you want to enter the BIOS utility, you must key-in your Setup password.

### If You Forget the Password

If you forget your password, you must return the configuration values stored in CMOS to their default values.  Should this happen, call your dealer for assistance.

## 2.5.6	Power On Password

The Power On Password secures your system against unauthorized use.  Once you set this password, you have to type it whenever you boot the system.  To set this password, follow the same procedure as in setting the Setup password.

# 2.6	PCI System Configuration

The PCI System Configuration allows you to specify the settings for your PCI devices.

PCI System Configuration Page 1/1

PCI IRQ Setting [ Auto ] INTA INTB INTC INTD *PCI Slot 1 [--] [--] [--] [ 9] *PCI Slot 2 [--] [--] [--] [ 9] *PCI Slot 3 [--] [--] [--] [ 9] *Onboard PCI VGA [ 9] PCI IRQ Sharing [Yes] VGA Palette Snoop [Disabled]

¯ = Move Highlight Bar, ® ¬ = Change Setting PgDn/PgUp = Move Screen, F1 = Help, Esc = Exit

## 2.6.1	PCI IRQ Setting

This parameter allows for Auto or Manual configuration of PCI devices.  If you use plug-and-play (PnP) devices, you can keep the default setting Auto.  The system then automatically configures the PnP devices.  If your PCI device is not a PnP, you can manually assign the interrupt for each of the device.

|  | Refer to your manual for technical information about the PCI card. |
| --- | --- |

### PCI Slots

These parameters allow you to specify the appropriate interrupt for each of the PCI devices.  You can assign IRQ5, IRQ9, IRQ10, IRQ11, IRQ14, or IRQ15 to the slots.

Press  or  to move between fields.  Press  or  to select options.

## 2.6.2	PCI IRQ Sharing

When set to Yes, this parameter allows you to assign the same IRQ to more than one PCI device installed in the system.  When set to No, you must assign different IRQs to the PCI devices.

## 2.6.3	VGA Palette Snoop

PCI devices support the palette snooping technique that enables the device to control access to their palette registers.  Set this parameter to Enabled to activate the palette snooping function in the PCI VGA devices installed in your system.  Check your VGA card manual for more information about this function.

# 2.7	Load Setup Default Settings

Use this option to load the default settings for the optimized system configuration.  When you load the default settings, some of the parameters are grayed-out with their fixed settings.  These grayed parameters are not user-configurable.  If you want to change the settings of these items, disable the Fast Boot Mode parameter in the Basic System Configuration menu.

The following dialog box appears when you select Load Setup Default Settings from the main menu.

Load Setup Default Settings
  Are you sure

	[Yes]	[No]

Select [Yes]  to load the default settings.

# 2.8	Leaving Setup

Examine the system configuration values.  When you are satisfied that all the values are correct, write them down.  Store the recorded values in a safe place.  In the future, if the battery loses power or the CMOS chip is damaged, you will know what values to enter when you rerun Setup.

Press  to leave the system configuration setup.  If there is any change in the BIOS utility functions, the following screen appears:

Do you want to save CMOS data?

	[Yes]	[No]

Use the arrow keys to select your response.  Select Yes to store the new data in CMOS.  Select No to retain the old configuration values. Press  .

# 2.9	IDE Hard Disk Types

| Type | Cylinders | Heads | Sectors Per Track |
| --- | --- | --- | --- |
| None | (indicates SCSI or no hard disk) |  |  |
| 1 | 306 | 4 | 17 |
| 2 | 615 | 4 | 17 |
| 3 | 615 | 6 | 17 |
| 4 | 940 | 8 | 17 |
| 5 | 940 | 6 | 17 |
| 6 | 615 | 4 | 17 |
| 7 | 462 | 8 | 17 |
| 8 | 733 | 5 | 17 |
| 9 | 900 | 15 | 17 |
| 10 | 820 | 3 | 17 |
| 11 | 855 | 5 | 17 |
| 12 | 855 | 7 | 17 |
| 13 | 306 | 8 | 17 |
| 14 | 733 | 7 | 17 |
| 15 | (reserved) |  |  |
| 16 | 612 | 4 | 17 |
| 17 | 977 | 5 | 17 |
| 18 | 977 | 7 | 17 |
| 19 | 1024 | 7 | 17 |
| 20 | 733 | 5 | 17 |
| 21 | 733 | 7 | 17 |
| 22 | 733 | 5 | 17 |
| 23 | 306 | 4 | 17 |
| 24 | 612 | 4 | 17 |
| 25 | 306 | 4 | 17 |
| 26 | 612 | 4 | 17 |
| 27 | 698 | 7 | 17 |

| Type | Cylinders | Heads | Sectors Per Track |
| --- | --- | --- | --- |
| 28 | 976 | 5 | 17 |
| 29 | 306 | 4 | 17 |
| 30 | 611 | 4 | 17 |
| 31 | 732 | 7 | 17 |
| 32 | 1023 | 5 | 17 |
| 33 | 751 | 8 | 17 |
| 34 | 755 | 16 | 17 |
| 35 | 731 | 13 | 26 |
| 36 | 980 | 15 | 17 |
| 37 | 936 | 16 | 17 |
| 38 | 981 | 5 | 17 |
| 39 | 981 | 10 | 17 |
| 40 | 762 | 8 | 39 |
| 41 | 980 | 5 | 17 |
| 42 | 832 | 8 | 33 |
| 43 | 683 | 12 | 38 |
| 44 | 513 | 16 | 38 |
| 45 | 776 | 8 | 33 |
| 46 | 683 | 16 | 38 |
| 47 | 832 | 6 | 33 |
| 48 | 615 | 2 | 34 |
| 49 | 989 | 16 | 63 |
| 50 | 823 | 4 | 38 |
| 51 | 1001 | 15 | 17 |
| 52 | 1024 | 17 | 22 |
| 53 | 723 | 13 | 51 |
| 54 | 548 | 8 | 38 |
| 55 | 1013 | 4 | 41 |
| 56 | 929 | 15 | 17 |

| Type | Cylinders |  | Heads | Sectors Per Track |
| --- | --- | --- | --- | --- |
| 57 | 817 |  | 14 | 36 |
| 58 | 723 |  | 13 | 81 |
| 59 | 802 |  | 4 | 39 |
| 60 | 1024 |  | 9 | 17 |
| 61 | 895 |  | 5 | 55 |
| 62 | (reserved) |  |  |  |
| 63 | 966 |  | 10 | 34 |
| 64 | 1024 |  | 8 | 17 |
| 65 | 1024 |  | 11 | 17 |
| 66 | 918 |  | 11 | 17 |
| 67 | 965 |  | 14 | 49 |
| 68 | 1024 |  | 10 | 17 |
| 69 | 1024 |  | 12 | 17 |
| 70 | 1024 |  | 13 | 17 |
| 71 | 1024 |  | 14 | 17 |
| 72 | 1024 |  | 11 | 50 |
| 73 | 1024 |  | 16 | 17 |
| 74 | 918 |  | 15 | 17 |
| 75 | 1010 |  | 16 | 51 |
| 76 | 1024 |  | 5 | 17 |
| 77 | 1024 |  | 8 | 17 |
| 78 | (reserved) |  |  |  |
| 79 | 1001 |  | 15 | 32 |
| 80 | 1024 |  | 16 | 63 |
| 81 | 1024 |  | 7 | 17 |
| 82 | 988 |  | 16 | 52 |
| 83 | 1024 |  | 15 | 17 |
| 84 |  | 776 | 8 | 33 |
| 85 |  | 926 | 13 | 17 |

| Type | Cylinders |  | Heads | Sectors Per Track |
| --- | --- | --- | --- | --- |
| 86 |  | 805 | 4 | 26 |
| 87 |  | 976 | 5 | 17 |
| 88 |  | 685 | 16 | 38 |
| 89 |  | 1011 | 15 | 22 |
| 90 |  | 997 | 10 | 53 |
| 91 |  | 985 | 13 | 32 |
| 92 |  | 816 | 15 | 32 |
| 93 |  | 968 | 5 | 17 |
| 94 |  | 903 | 8 | 46 |
| 95 |  | 966 | 5 | 34 |
| 96 |  | 535 | 10 | 50 |
| 97 |  | 715 | 10 | 50 |
| 98 |  | 1016 | 16 | 63 |
| 99 |  | 996 | 16 | 63 |
| Auto or User |  | (User-defined) |  |  |