ME FW & SYSTEM BIOS Update

SOP

Date: Nov, 2010

This document contains proprietary technical information, which is the property of the Acer Incorporated and shall not be disclosed to others in whole or in part, reproduced, copied, or used as the basis for design, manufacturing, or sale of apparatus without written permission of Acer Incorporated.

The information in this document, including products and software versions, is current as of the Release Date above. This document is subject to change without notice.

In [DOS Mode]

  - Disable ME by setting HW jumper “ME_DISABLE”

  - Power on System and Boot to DOS

  - Key in ‘cd dostool’  (Go to BIOS path like “A:\DOSTOOL”)

  - Key in ‘flash8M.bat’ or ‘flash8M’

  - Press <Enter> and start to flash System BIOS and ME firmware.

  - Shutdown system and remove power core.

  - Enable ME by setting HW jumper “ME_DISABLE”

  - Clear CMOS.

1.9   Boot system.

In [Windows Mode]

By Windows command line

2.1 Run Command Prompt as Administrator in Windows.

Windows 32bit

    - Disable ME by setting HW jumper “ME_DISABLE”

    - Power on System and Boot to Windows

    - Key in ‘cd wintool\32’ (Go to BIOS path like “D:\WinTool\32”)

    - Key in ‘flash8M.bat’ or ‘flash8M’

    - Press <Enter> and start to flash ME firmware.

    - Shutdown system and remove power core.

    - Enable ME by setting HW jumper “ME_DISABLE”

    - Clear CMOS

    - Boot system.

Windows 64bit

    - Disable ME by setting HW jumper “ME_DISABLE”

    - Power on System and Boot to Windows

    - Key in ‘cd wintool\64’ (Go to BIOS path like “D:\WinTool\64”)

    - Key in ‘wflash8M.bat’ or ‘wflash8M’

    - Press <Enter> and start to flash ME firmware.

    - Shutdown system and remove power core.

    - Enable ME by setting HW jumper “ME_DISABLE”

    - Clear CMOS

    - Boot system.