BIOS Release Note

PROJECT NAME:          945P01

Date:       12/27/2005

| APPROVED | CHECKED | PREPARED |
| --- | --- | --- |
| Rick Lee | David Lee | Rex Chang |

| TITLE |  |
| --- | --- |
| Project Name | 945P01 |
| Software Version | 54DA1P42 |
| Date | 2005/12/27 |
| Checksum | 6300h |
| Software CORE Base | AWARD BIOS version 6.00 |
| Manufacturer/Customer | Foxconn/Acer |
| M/B (HW) Version | FAB-B3 |
| Engineer | Rex Chang |
| Project Manager | Happy shao |

| ON BOARD DEVICE FIRMWARE / APPENDANT SOFTWARE |  |
| --- | --- |
| On-Board LAN BIOS | 3.5.2.3 |
| On-Board VGA BIOS | LPG_1217.DAT |
| On-Board SATA BIOS | N/A |

| FILE DESCRIPTION |  |
| --- | --- |
| Source File Backup location | Md5biosserver\projects\945p01 |
| Binary/ROM File location | Md5biosserver\BIOSfile\945p01 |
| Binary/ROM File Name | 54DA1P42.bin |
| Attached Document | 54DA1P42_BIOS_HISTORY.TXT |

| BUGS FIXED AND CHANGES |  |  |
| --- | --- | --- |
| Following are the major changes from R01-C2 |  |  |
| 1.Fixed Intruder alarm can't be enable when shut down. |  |  |
| Note: Thermal profile implemented. |  |  |
| Single core CPU | Dual core CPU | Cedar Mill |
| Min CPU PWM =100 | Min CPU PWM =100 | Min CPU PWM =100 |
| CPU FAN low temperature =57∘C | CPU FAN low temperature =61∘C | CPU FAN low temperature =61∘C |
| CPU FAN high temperature = 67∘C | CPU FAN high temperature = 67∘C | CPU FAN high temperature = 67∘C |
| CPU Fan Range/Freq Value =9A | CPU Fan Range/Freq Value =6A | CPU Fan Range/Freq Value =9A |
| Min system PWM =49 | Min system PWM = 49 | Min system PWM = 49 |
| System FAN low temperature = 57∘C | System FAN low temperature = 61∘C | System FAN low temperature = 57∘C |
| System FAN high temperature = 67∘C | System FAN high temperature = 66∘C | System FAN high temperature = 67∘C |
| Sys Fan Range/Freq Value =9A | Sys Fan Range/Freq Value =6A | Sys Fan Range/Freq Value =9A |

IMPORTANT NOTES

1. Production version release

2. Base on AWARD BIOS 28th, Mar. 2005 code base.

3. Acer BIOS ID: R01-C3

4. Sign-on Message: This BIOS is exclusively for Acer only