BIOS Pre-Test Checklist

Function: BIOS

Project Name: 945P01

Document Revision: 54DA1P42

Issue Date: Dev.27, 2005

|  | ## APPROVED | CHECKED | PREPARED |
| --- | --- | --- | --- |
| ## BY | Rick Lee | David Lee | Iris Zou |
| DATE | 12/27/2005 | 12/27/2005 | 12/27/2005 |

This document contains proprietary and confidential information of Foxconn. All rights and titles hereto remain with Foxconn. The recipient shall keep this document in strict confidence and shall use this document in the manner and solely for purposes expressly agreed to by Foxconn. The recipient shall not disclose or allow access to this document, in whole or in part, by third parties, nor use it for the benefit of the recipient or third parties. The recipient shall not change this document in form and substance by any means, including without limitation, decompile, reproduce, revise, and amend.]

| TITLE |  |
| --- | --- |
| Project Name | 945P01 |
| BIOS Version | 54DA1P42 |
| BIOS Checksum | 6300H |
| Date | 12/27/2005 |
| Release Stage | □EVT □ DVT □ PVT ■ MP |
| Customer | FOXCONN/Acer |
| Engineer | Rex Zhang |

| OS / Module | Test Name | Procedure | Result | Comment |
| --- | --- | --- | --- | --- |
| General | POST | 1.Verify Logo is properly displayed at boot. | ■Pass □Fail □N/A |  |
|  |  | 2.Check if every summary screen show right or not. | ■Pass □Fail □N/A |  |
|  |  | 3.Check sign on message if BIOS revision information is correct or not. | ■Pass □Fail □N/A |  |
|  |  | 4.Check if Product information in BIOS setup is the same with DMI structure (Acer or Lenovo BIOS only) | ■Pass □Fail □N/A |  |
|  |  | 5.Verify that PnP ESCD & DMI structure is updated correctly during POST. (To change memory size or PCI slot configuration, ESCD update message should appear) | ■Pass □Fail □N/A |  |
|  | BIOS Setup Key | Verify that the BIOS can be accessed using the “Del” or special key. | ■Pass □Fail □N/A |  |
|  | BIOS Flash Utility | Flash system BIOS using the BIOS vendor’s flash Utility. Flash at least 3 times. | ■Pass □Fail □N/A |  |
|  | BIOS Setup Test | 1. Exercise all CMOS Default Changes (Optimized & fail safe default) | ■Pass □Fail □N/A |  |
|  |  | 2. Clear CMOS and verify that system defaults are loaded (check CMOS default table) | ■Pass □Fail □N/A |  |
|  |  | 3. Verify setup defaults by screen (first release P-BIOS only) | ■Pass □Fail □N/A |  |
|  | Boot Device | 1. Verify Boot Device order is the same as the setting of BIOS SETUP. | ■Pass □Fail □N/A |  |
|  |  | 2. Change First Boot Device to all possible choices and check functionality. | ■Pass □Fail □N/A |  |
|  | Power on Test | Perform 3 manual power on boots - watch for hangs and irregular boot messages | ■Pass □Fail □N/A |  |
|  | Cold Boot Test | Perform 3 reset button boots - watch for hangs and irregular boot messages | ■Pass □Fail □N/A |  |
|  | Warm Boot Test | Perform 3 manual warm boots (Ctrl-Alt-Del) – watch for hangs and irregular boot messages | ■Pass □Fail □N/A |  |
|  | System Health Monitor | 1. Verify BIOS report H/W health correctly. | ■Pass □Fail □N/A |  |
|  |  | 2. Over temperature causes Windows XP shutdown or BIOS Setup) | ■Pass □Fail □N/A |  |
| Windows XP | Power Manage ment Inactivity Timer | In WindowsXP, put system into standby mode, for 3 times. (Test both of S1 and S3) And test hibernation 3 times. | ■Pass □Fail □N/A |  |
| Windows XP | Sleep | 1. Wake with Mouse (PS/2) | □Pass □Fail ■N/A |  |
|  |  | 2. Wake with Keyboard (PS/2) | ■Pass □Fail □N/A |  |
|  |  | 3. Wake with LAN | ■Pass □Fail □N/A |  |
|  |  | 4. Wake with PME (PCI) | ■Pass □Fail □N/A |  |
|  |  | 5. Wake with RTC | ■Pass □Fail □N/A |  |
| Windows XP | Resources | Disable all possible devices and verify in Device Manager that the resources are made available when disabled in BIOS | ■Pass □Fail □N/A | COM port, LPT port, LAN,USB… |
| Record All PCI Register settings | Record All PCI Register settings | Check if all PCI Device register settings are the same as last version BIOS. | ■Pass □Fail □N/A |  |