BIOS Design Specification

Function: BIOS

Project Name: 945P01

Project Revision: Fab.B3

Document Revision: 0.4

Issue Date: 12/09/2005

|  | APPROVED | CHECKED | PREPARED |
| --- | --- | --- | --- |
| BY | rick | David | rEX |
| DATE | 12/09/2005 | 12/09/2005 | 12/09/2005 |

This document contains proprietary and confidential information of Foxconn. All rights and titles hereto remain with Foxconn. The recipient shall keep this document in strict confidence and shall use this document in the manner and solely for purposes expressly agreed to by Foxconn. The recipient shall not disclose or allow access to this document, in whole or in part, by third parties, nor use it for the benefit of the recipient or third parties. The recipient shall not change this document in form and substance by any means, including without limitation, decompile, reproduce, revise, and amend.

Content

.1	Product Summary	4

.1.1	The 945P01 desktop board is a performance business desktop board with the following product features.	……………………………………………………………………………………………………………………4

.2	BIOS Requirement	5

.2.1	Standard BIOS Feature	5

.2.2	Device Boot Support	6

.2.3	Power Management	6

.2.4	Hardware Monitor and PC Health Status Features	7

.2.5	Flash Memory Size	9

.2.6	BIOS ID	9

.2.7	 OEM MB features	9

.3	BIOS Configuration Method	9

.4	Customization	9

.4.1	Preload Features	9

.4.2	Language Support	9

.4.3	Logo for Specific Customer	9

.4.4	Sign-on Message	錯誤! 尚未定義書籤。

.4.5	DMI Structure	錯誤! 尚未定義書籤。

.5	SOW for DMI	10

.6	GPIO Definitions	14

.7	IRQ Routing Table	15

.8	BIOS Setup Options	16

Scope

This document is a comprehensive listing of 945P01 BIOS specific requirements for the Intel 945-G(Intel lakeport-G) chipset product. The BIOS specification provides the project team with the information necessary to understand and design the product, and plan for project management. It provides a basis to plan for implementation, sales, and support.

Revision History

| Revision | Revision History | Date |
| --- | --- | --- |
| 0.1 | Preliminary release. | 5/6/05 |
| 0.2 | Modify setup menu | 7/1/05 |
| 0.3 | Modify product name | 8/25/05 |
| 0.4 | Update INT15V1.5/Change P1-C OEM Logo | 12/09/05 |

## Product Summary

### The 945P01 desktop board is a performance business desktop board with the following product features.

- Pico BTX compliant form factor

- Supports the following Intel processors:

  - 533/800/1066MHz Front Side Bus

  - Support Hyper Threading Technology

  - Support Up to 3.8G (Hyper-threading)

  - Intel LGA775 family processors

    - 945-G (Intel Lakeport-G)

- Intel Lakeport-G on-die graphic (Enhanced Int. Gfx Core)solution

- DVMT technology support

    - I/O Controller Hub (ICH7)

    - Clock chip: ICS 954128

    - Winbond W49V49004FAP Firmware Hub

    - IDE intefrace:

- Single channels enhanced IDE Contoller from ICH7

- PIO Mode: 0/1/2/3/4

- ATA mode: 33/66/100

- Single channels enhanced IDE Contoller from SMSC LPC47M182-NR

  - Ultra DMA 133 Support

- Storage Type support:

- HDD/CD-ROM/CD-RW/DVD-ROM/DVD-RW/DVD+RW/DVD Dual/DVD SuperMultiPlus

- 2 ports Serial ATA interface (ICH7)

- PCI Express* supported:

- Support 1x PCIE x16 Gfx slot

- System memory:

- 2DIMM sockets

- 256MB to 1GB DDR2 400/667/533 Unbuffered ECC SDRAM module support

- Dual channel should be enabled always when plug-in 2 same memory size DDRII memory module

- Up to 2 GB maximum

- PCI interface:

- Complies to PCI Specification Rev. 2.3

  - 4 UHCI Host Controllers with 8 (rear:X4, front header:X2)USB Ports (ICH7) ,Rev 2.0 / 1.1

  - Audio Options:

- AC'97 Specification v2.3 compliant audio

- HD audio codec ALC880 HD codec 7.1  w/ S/PDIF out/in

- PCI-E Giga-Lan  Marvell 8052/53

  - H/W, F/W, and S/W should be able to support pin-2-pin compatible with ASF and TPM solution LAN chip

- SMSC LPC47M182-NR + Management ASIC Heceta 6e

- WOL Support

- TSB43AB22A chip for 1394a

- Support ACPI S3 STR power state for supported operating systems

- ACPI 1.0b, APM 1.2, PnP 1.0a, SMBIOS 2.3, WFM 2.0, USB 1.1, USB 2.0 Compliance.

- PC 99, PC99A and PC2001 Compliance

- WinXP MB Logo

- Support EM64T,EIST function for intel cpu of new generation .

- Monitor Functions

- Linear FAN control design in MB circuit, At least 4 steps FAN speed control by hardware monitor

- Designed in CPU burn protect circuit, CPU Over-temperature (over 120oC) power off protection

- Designed in System / CPU Over temperature warning/shut down circuit

## BIOS Requirement

### Standard BIOS Feature

- Phoenix Award BIOS

- Intel 775 Prescott and Celeron processor detection and initialization

- Intel Lakeport-G/GV chipset initialization modules, Intel memory sizing reference code.

- Auto-detect keyboard and mouse

- Auto-detect of off-board serial / parallel ports, floppy ports, IDE controllers with auto-configuration of on-board devices

- Auto-detect of IDE drive type installed

- High performance IDE controller support with ATA-5 features, including LBA mode for large hard drives, advanced PIO modes 3 & 4 for faster PCI IDE disk access. BIOS will automatically configure drives for highest performance. Performance options set up on a per-drive basis

- INT 13h extensions to support drives larger than 16GB

- Ultra DMA 33 Mode and Ultra ATA66/100 (33,66,100MB/s)

- ATAPI support for IDE CD-ROM and tape drives

- Support for 3 mode floppy devices for Japanese market

- LS-120 removable media support

- USB 1.1 and USB 2.0 compliance support (with support for legacy input devices)

- ACPI 2.0 compliance

- APM 1.2 compliance

- SMBIOS 2.3 compliance

- Wire For Management 2.0 (WFM) support

- ASF 2.0 Support

- LPC decoding for TPM support

- SIPP Support

- Plug and Play support

- PCI add-in card auto-configuration, PCI 2.3 compliance

- Restart on AC / Power Failure function support

- User and administrator mode for system setup screens with 2 separate password-protected accesses.

- BIOS Antivirus Support

- Maximum of 4GB system memory

- Four DIMM slots are provided for flexible memory configurations

- BIOS will not initialize installed memory greater than 4GB

- DDR II 400/533/667 MHz module supported

- Single or double-sided DIMM configurations

- Mixed speed DIMM configuration will default to the slowest speed DIMM installed.

- Wake up event support

- Acer eRecovery support

- Intel SIPP support

- Intel XD bit support

- Intel EIST support

- Intel EM64T support

- TPM support

- Intel DVMT support

### Device Boot Support

- FDD Boot

- HDD Boot

- CDROM Boot

- Remote Network boot (PXE 2.0)

- USB (removable media) boot

- USB Flash Boot (Support USB-HDD, USB-FDD, USB-ODD and USB-ZIP standard)

### Power Management

- Support APM and ACPI modes and all devices standby states as are listed bellow (Note S4 wake is OS dependent):

| Devices | S1 | S3 | S4 | S5 |
| --- | --- | --- | --- | --- |
| Power Button | Enabled | Enabled | Enabled | Enabled |
| PS2 Keyboard | Disabled | Disabled | Disabled | Disabled |
| USB Keyboard | Disabled | Disabled | N/A | N/A |
| PME | Disabled | Disabled | Disabled | Disabled |
| RTC | Disabled | Disabled | Disabled | Disabled |
| Ring | Disabled | Disabled | Disabled | Disabled |

| Wake up event | From State |
| --- | --- |
| Power Button | S1, S3, S4_OS, S5 |
| PS2 | S1, S3, S4_OS, S5 |
| USB Keyboard/Mouse | S1, S3, S4_OS |
| PCI (via PME# signal) | S1, S3, S4_OS, S5 |
| PCI Express | S1, S3, S4_OS, S5 |
| Serial Port (via SIO) | S1, S3 |
| RTC alarm | S1, S3, S4_OS, S5 |

MDA 2005

- Devices wake up from S3 should be less than 5s

- Devices wake up from S4 should be less than 25s

- Devices wake up from S5 should be less than 40s

- ACPI LED Status State

| Chassis | Bezel | LED |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Power LED |  |  | HDD LED | LAN LED | ODD LED |
|  |  | S0 | S1, S3 | S4, S5 |  |  |  |
| H200 | V251 | Blue | Blue Blinking | OFF | Blue | Blue | Blue |
|  | A251 | Blue | Blue Blinking | OFF | Blue | Blue | Blue |

### Hardware Monitor and PC Health Status Features

- Hardware Monitor and Fan Speed Control Implementation in BIOS PC Health Status Menu

| Voltage | Description |
| --- | --- |
| CPU VCore | CPU VCore PS Voltage Rails |
| +12V | +12V_SYS Voltage Rails |
| +5V | 5V_SYS Voltage Rails |
| +3.3V | 3.3V_SYS Voltage Rails |
| +1.5V | 1.5V_SYS Voltage Rails |

| Temperature | Description |
| --- | --- |
| Current CPU Temp | CPU Temperature |
| Current System Temp | System Temperature |

| Fan Speed | Description |
| --- | --- |
| CPU Fan1 Speed |  |

### Flash Memory Size

- 4Mbit Firmware Hub (FWH) in PLCC package

- 4Mbit symmetrical Flash

### BIOS ID

R01-A3

### OEM MB features

Meet OEM CUSTOMER BIOS SPEC (Legend Award Bios Spec V2.0)

## BIOS Configuration Method

- F12 for boot menu

- Press DEL key to enter BIOS Setup Screen

- Hidden Page design in BIOS is must, “Alt+F4” to switch both pages

## Customization

### Language Support

- English language support only.

- Spanish language support

### Logo for Specific Customer

For Veriton (P1-L & P1-C)

## SOW for DMI

| Veriton DMI data description/content/address |  |  |
| --- | --- | --- |
| Type 00 ==> BIOS Information; Handle:0000 |  |  |
| Description | Content | Offset Starting Address |
| Vendor Name: | Phoenix Technologies, LTD | 04H |
| BIOS Version: | same as BIOS ID | 05H |
| BIOS Starting Address Segment: | E000 | 06H |
| BIOS Build Date: |  | 08H |
| BIOS Characteristics: | Press [ENTER] for detail | 0AH |
| Size of BIOS ROM: | 0512K | 09H |
| BIOS Char Extension: | Press [ENTER] for detail | N/A |
| BIOS Char Extension 2: | Press [ENTER] for detail | N/A |
| Type 01 ==> System Information; Handle:0001 |  |  |
| Description | Content | Offset Starting Address |
| Manufacturer Name: | Acer | 04H |
| Product Name: | Veriton 2800 | 05H |
| Version: | same as BIOS ID | 06H |
| Serial Number: |  | 07H |
| UUID: | FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | 08H |
| Wake-up: | Power Switch | N/A |
| Type 02 ==> Base Board Information; Handle:0002 |  |  |
| Description | Content | Offset Starting Address |
| Manufacturer Name: | Acer | 04H |
| Product Name: | F945PL/F945PC | 05H |
| Version: |  | 06H |
| Serial Number: | Pls reserve for 22 bit at least | 07H |
| Type 03 ==> System Enclosure or Chassis; Handle:0003 |  |  |
| Description | Content | Offset Starting Address |
| Manufacturer Name: | Acer | 04H |
| Chassis lock: | No | N/A |
| Type: | Desktop | 05H |
| Version: |  | 06H |
| Serial Number: | Pls reserve for future usage | 07H |
| Asset Tag Number: | Pls reserve for future usage | 08H |
| Bootup State: | Unknown | N/A |
| Power Supply State: | Unknown | N/A |
| Thermal State: | Unknown | N/A |
| Security State: | Unknown | N/A |
| OEM Defined Data: | 0000 | N/A |

The purpose of this document is to provide the location in the BIOS area for DMI programming. It also contain the DMI information that to be programmed.

### 6.	Acer  erecovery  SUPPORT

1. Check One Button (through GPIO)

CMOS have to reserve 2 bits to set as "0" , "1" , "2"  that is the flag value.

When GPIO active, BIOS have to set the flag as "1" .

int15 ax=ca80

bl=0 set flag to 0  => bypass

bl=1 set flag to 1  => into recovery ui

bl=2 set flag to 2  => recovery system directly

others , no function

int15 ax=ca81(RETURN VALUE DL)

dl=0  => bypass

dl=1  => into recovery ui

dl=2  => recovery system directly

others , no function

There is a button connect to the M/B. With the button pressed, the CMOS bits is set to “1”

2. Check Alt+F10

AX=9610h

BL=18h

Return :

Carry = 0 => Successful

1 => Error

CX = 0 => Alt + F10 is pressed

1 => Alt + F10 is not pressed

Notice : Bios need set CX value when Alt+F10 is pressed

3. Check system resume from hibernation

AX=9610h

BL=15h

Return :

Carry = 0  => successful

= 1  => Error

CX = 0  => Resume from Hibernation

= 1  => others

If MBR check CX=0 , Not allow into hidden partition (press Alt+F10 no function)

4. Set BIOS boot sequence

AX=9610h

BL=30h

BH=00h => C -> CD-ROM -> LAN

BH=01h => CD-ROM -> LAN -> C

BH=02h => LAN -> A -> C -> CD-ROM

Return :

Carry = 0 => Successful

1 => Function not support or the selected device does not exist

5.Check/Set user password

Input parameters:

AX = 9610h

BL = 07h

BH = FunIndex

DS:SI = PWString[8] 	point to password string

CX = PW length

Output:

CF = 0	according to the BH, the return value means

BH = 	0: Password has been disabled

1: Password exist

2: Password has been set exactly

3: Password is consistent

6.Check/Set supervisor password

Input parameters:

AX = 9610h

BL = 08h

BH = FunIndex

DS:SI = PWString[8]	point to password string

CX = PWString

Output:

CF = 0	according to the BH, the return value means

BH = 	0: Password has been disabled

1: Password exist

2: Password has been set exactly

3: Password is consistent

7.Get/Set fast boot

Fast boot used for speeding up the system boot. It may ignore some detecting procedures, for example, memory test. This function provides the capability of application to enable/disable the fast boot behavior.

8.Get/Set boot sequence

Get/Set boot sequence let application can manage BIOS boot sequence in any combination. Application (caller) should prepare a 16-bytes array and pass the buffer to this function. For each byte indicates a boot device ID. The first byte has the highest boot priority. Here mapping the boot device ID to the device name:

Device ID:

00h	Floppy Devices

01h	Hard Drive

02h	CD-ROM/DVD Drive

03h	SCSI

04h	D2D recovery

05h	LS-120

06h	PCMCIA

07h	Network boot

08h	ZIP

09h	USB Removable Devices

0Ah	USB-HDD

0Bh	USB-FDD

0Ch	USB-ODD

0Dh	USB-ZIP

0Eh	USB Other Devices

0Fh	Other bootable devices

### 7   Intrusion Detection

Chassis Intrusion Detection test procedure (ICH6 intruder# used for intrusion detection flag)

BIOS Mode:

a)	First open the chassis and then close this chassis to ensure that Intrusion alarm scroll matched with the chassis cover.

b)	Power on EUT and enter into BIOS setup menu.

c)	Reset Intrusion Alarm, save this setting and exit.

d)	Reboot EUT to ensure that there is no "Warning! Your chassis has been opened" information displaying during BIOS Post.

e)	Enter into BIOS to ensure that <Intrusion alarm > is <Enabled>.

f)	Shutdown EUT and open the chassis.

g)	Power on EUT again to ensure that there is "Warning!  Your chassis has been opened" information displaying on the BIOS Post screen.

h)	Try to disable Intrusion Alarm in BIOS; there still has warning information displaying regardless open or close the chassis.

i)	Close this chassis and reset Intrusion Alarm again, the warning information will disappear. And then disable <Intrusion Alarm>, save this setting and exit.

j)	Open this chassis and power on EUT, there will no warning information displaying on the BIOS Post screen.

Windows XP Mode:

a)	First open the chassis and then close this chassis to ensure that Intrusion alarm scroll matched with the chassis cover.

b)	Power on EUT and enter into BIOS setup menu.

c)	Reset Intrusion Alarm, save this setting and exit.

d)  When system is on, open the chassis to ensure that EUT will enter into S4 automatically.

e)	Let EUT enter into S3 mode from Windows Start Menu.

f)	Open the chassis to ensure that EUT will automatically resume then enter into S4 mode

g)   Close this chassis, when system is on, the EUT will enter into S4 automatically if no reset <Intrusion Alarm> in BIOS.

### TPM Support

Open LPC interface decoding. Decoding port: 4700H

Open LPC Configuration port 2E/2F or 4E/4Fh

## GPIO Definitions

## IRQ Routing Table

|  | PCI1 | PCI2 | LAN | 1394 | ITE8212 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| INTA | D |  |  |  |  |  |  |  |  |  |
| INTB | A |  |  |  |  |  |  |  |  |  |
| INTC | B |  |  |  |  |  |  |  |  |  |
| INTD | C |  |  |  |  |  |  |  |  |  |
| INTE |  | C |  |  | A |  |  |  |  |  |
| INTF |  | B |  | A |  |  |  |  |  |  |
| INTG |  | A |  |  |  |  |  |  |  |  |
| INTH |  | D | A |  |  |  |  |  |  |  |
| IDSEL | AD18 | AD17 | AD19 | AD16 | AD20 |  |  |  |  |  |

## BIOS Setup Options

- Full BIOS setup options are listed in Excel attachment bellow

## BIOS Setup Options

- Full BIOS setup options are listed in Excel attachment bellow.

- Alt+F4 switches hidden-page to normal view.

| Menu |  | Field |  | Display | Options |  |  | Default |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Product Information |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System Product Name |  | Show | Veriton 2800 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MB Product Name |  | Show | F945PL/F945PC |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System S / N |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System Manufacturer Name |  | Show | ACER |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | MB Manufacturer Name |  | Show | ACER |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SMBIOS Version |  | Show | 2.4 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System BIOS Version |  | Show | 6.00 PG |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | System BIOS ID |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard CMOS Features |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Date |  | Show |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | Time |  | Show |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Channel 0 Master |  | Show |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Channel 0 Slave |  | Show |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Channel 1 Master |  | Show |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Channel 1 Slave |  | Show |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | 3 mode |  | Show | Enable / Disable |  |  | Disable |  |  |  |  |  |  |  |  |  |  |
|  |  | Video |  | Show | EGA / VGA / CGA 40 / CGA 80 / MONO |  |  | EGA / VGA |  |  |  |  |  |  |  |  |  |  |
|  |  | Halt On |  | Show | All Errors / No Errors / All, But Keyboard |  |  | All, But Keyboard |  |  |  |  |  |  |  |  |  |  |
|  |  | Base Memory |  | Show Only |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | Extended Memory |  | Show Only |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
|  |  | Total Memory |  | Show Only |  |  |  | N / a |  |  |  |  |  |  |  |  |  |  |
| Advanced BIOS Features |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Hard Disk Boot Priority |  |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Virus Warning |  |  | Show | Enabled / Disabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  | CPU L1 & L2 Cache |  |  | Hide | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  | CPU L3 Cache |  |  | Hide | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  | Silent Boot |  |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  | Quick Power On Self Test |  |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  | First Boot Device |  |  | Show | LS120 / HardDisk / CDROM / ZIP100 / LAN / Disabled |  |  | Hard Disk |  |  |  |  |  |  |  |  |  |  |
|  | Second Boot Device |  |  | Show | LS120 / HardDisk / CDROM / ZIP100 / LAN / Disabled |  |  | CDROM |  |  |  |  |  |  |  |  |  |  |
|  | Third Boot Device |  |  | Show | LS120 / HardDisk / CDROM / ZIP100 / LAN / Disabled |  |  | LAN |  |  |  |  |  |  |  |  |  |  |
|  | Boot Other Device |  |  | Show | Disabled / Enabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  | Boot Up Numlock Status |  |  | Show | Off / On |  |  | ON |  |  |  |  |  |  |  |  |  |  |
|  | Gate A20 Option |  |  | Show | Normal / Fast |  |  | Fast |  |  |  |  |  |  |  |  |  |  |
|  | Security Option |  |  | Show | Setup / System |  |  | Setup |  |  |  |  |  |  |  |  |  |  |
|  | APIC Mode |  |  | Showonly/(alt+F4) | Enabled/Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  | MPS Version Control for OS |  |  | Show | 1.1 / 1.4 |  |  | 1.4 |  |  |  |  |  |  |  |  |  |  |
|  | Configuration Table |  |  | Show | Disabled/Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  | HDD S.M.A.R.T Capability |  |  | Show | Disabled / Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  | Bootblock Write Protect |  |  | Hide/(alt+F4) | Enabled/Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
| Advanced Chipset Features |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DRAM Timing Selectable |  | Showonly | By SPD / Manual |  |  | By SPD |  |  |  |  |  |  |  |  |  |  |
|  |  | CAS Latency Time |  | Showonly | 2 / 2.5 / 3 |  |  | Depend on Memory |  |  |  |  |  |  |  |  |  |  |
|  |  | Active to precharge Delay |  | Showonly | 8 / 7 / 6 / 5 |  |  | Depend On Memory |  |  |  |  |  |  |  |  |  |  |
|  |  | DRAM RAS# to CAS# Delay |  | Showonly | 4 / 3 / 2 |  |  | Depend On Memory |  |  |  |  |  |  |  |  |  |  |
|  |  | DRAM RAS# Precharge |  | Showonly | 4 / 3 / 2 |  |  | Depend On Memory |  |  |  |  |  |  |  |  |  |  |
|  |  | Memory Frequency for |  | Hide | Auto / DDR400 / DDR533 |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | System BIOS Cacheable |  | Hide | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Video BIOS Cacheable |  | Hide | Enabled / Disabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | SLP_S4# Assertion Width |  | Showonly | 4 to 5 Sec./3 to 4 Sec./2 to 3 Sec./1 to 2 Sec. |  |  | 4 to 5 Sec |  |  |  |  |  |  |  |  |  |  |
|  |  | **** VGA Setting **** |  | show | Enabled / Disabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | On-chip Video Memory Size |  | show | Press Enter |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | On-Chip Frame Buffer size |  | show | 1MB / 8MB |  |  | 8MB |  |  |  |  |  |  |  |  |  |  |
|  |  | DVMT/FIXED Memory Size |  | Show | 64MB/128MB |  |  | 128MB |  |  |  |  |  |  |  |  |  |  |
|  |  | PEG/Onchip VGA Control |  | show | Auto/Onchip VGA/PEG Port |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | Hyper-Threading |  | show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Enhanced halt state |  | show | Auto/Disabled |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
| Integrated Peripherals |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | OnChip IDE Device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE DMA Transfer Access |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | On-Chip Secondary PCI IDE |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Primary Master PIO |  | Show | Auto/Mode 0/Mode 1/Mode 2/Mode 3/Mode 4 |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Primary Slave PIO |  | Show | Auto/Mode 0/Mode 1/Mode 2/Mode 3/Mode 4 |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Primary Master UDMA |  | Show | Auto/Disabled |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | IDE Primary Slave UDMA |  | Show | Auto/Disabled |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | On-Chip Serial ATA |  |  | Disabled/Auto/Combined Mode/Enhanced Mode/SATA Only |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | PATA IDE Mode |  | Show | Primary / Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | SATA Port |  | Showonly | P1,P3 is Secondary |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Serial ATA Port0 Mode |  | Hide | Primary Master/Primary slave/Secondary Master/Secondary Slave/SATA0 Master/SATA1 Master/ |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Serial ATA Port1 Mode |  | Hide | Primary Master/Primary slave/Secondary Master/Secondary Slave/SATA0 Master/SATA1 Master/ |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Onboard Device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | USB Controller |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | USB 2.0 Controller |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | USB Keyboard Support |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | USB Mouse Support |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Azalia Select |  | Show | Auto/ Disabled |  |  | Auto |  |  |  |  |  |  |  |  |  |  |
|  |  | Onboard 1394 |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Onboard LAN Function |  | Show | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Onboard Lan Boot ROM |  | Show | Enabled / Disabled |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Super IO Device |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | POWER ON Function |  | Show |  | Button Only / Any Key |  | Button Only |  |  |  |  |  |  |  |  |  |  |
|  |  | Onboard Serial Port 1 |  | Hide(alt+f40 |  | Disabled / 3F8+IRQ4 / 2F8+IRQ3 / 3E8+IRQ4 / 2E8+IRQ3 / Auto |  | 3F8+IRQ4 |  |  |  |  |  |  |  |  |  |  |
|  |  | Onboard Parallel Port |  | Show |  | Disabled / 378+IRQ7 / 278+IRQ5 / 3BC+IRQ7 |  | 378+IRQ7 |  |  |  |  |  |  |  |  |  |  |
|  |  | Parallel Port Mode |  | Hide(alt+f4) |  | SPP / EPP1.9+SPP / ECP / EPP1.9+ECP / Printer / EPP1.7+SPP / EPP1.7+ECP |  | Printer |  |  |  |  |  |  |  |  |  |  |
|  |  | ECP Mode Use DMA |  | Hide(alt+f4) |  | 1 / 3 |  | 3 |  |  |  |  |  |  |  |  |  |  |
| Power Management Setup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | PCI Express PM Function[Press Enter] |  | Show | PCI Express PME [Enabled/Disabled] |  |  | (Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | ACPI Funciton |  | hide(alt+F4) | Enabled / Disabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Sleep Mode |  | Show | S1(POS) / S3(STR)/S1&S3 |  |  | S3(STR) |  |  |  |  |  |  |  |  |  |  |
|  |  | Run VGABIOS if S3 Resume |  | Show | Atuo /Yes/No |  |  | Atuo |  |  |  |  |  |  |  |  |  |  |
|  |  | Soft-Off by PWB-BTTN |  | Show | Instant-Off/Delay 4 Sec |  |  | /Delay 4 Sec |  |  |  |  |  |  |  |  |  |  |
|  |  | PWRON After PWR-Fail |  | Show | Former-Sts/On/Off |  |  | Former-Sts |  |  |  |  |  |  |  |  |  |  |
|  |  | CPU THRM-Throttling |  | Show | 75.0%/50.0%/25.0%/ |  |  | 50.0% |  |  |  |  |  |  |  |  |  |  |
|  |  | Wake-up by PCI Card |  | Show | Disabled / Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Power on by Ring |  | Show | Disabled / Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | USB KB Wake-Up From S3 |  | Show | Disabled / Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Resume by Alarm |  | Show | Disabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Date(of Month) Alarm |  | Show | 0 |  |  | 0 |  |  |  |  |  |  |  |  |  |  |
|  |  | Time(hh:mm:ss) Alarm |  | Show | 0 :0 :0 |  |  | 0 :0 :0 |  |  |  |  |  |  |  |  |  |  |
| PnP / PCI Configurations |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Resources Controlled By |  | Hide | Auto (ESCD) / Manual |  |  | Auto (ESCD) |  |  |  |  |  |  |  |  |  |  |
|  |  | IRQ Resources |  | Hide | Press Enter |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | DMA Resources |  | Hide | Press Enter |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | PCI / VGA Pallette Snoop |  | Hide | Enabled / Disabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
| PC Health Status |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Smart Fan Function |  | show | Disabled / Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Intruder Detection |  | show | Disabled/Enabled/Reset |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Current CPU Temperature |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Current System Temperature |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Vcore |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | +12V |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | +3.3V |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | + 5V |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | +1.5V |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CPU fan Speed |  | Showonly | show only |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Shutdown Temperature |  | show(alt+f4) | 80°C/176°F /85°C/185°F /90°C/194°F /95°C/203°F |  |  | 90°C/194°F |  |  |  |  |  |  |  |  |  |  |
| Frequency Control |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | CPUID Maximuml Value Limit |  | Showonly | Disabled / Enabled |  |  | Disabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Auto Detect PCI Clk |  | Show | Disabled / Enabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
|  |  | Spread Spectrum |  | Show | Disabled / Enabled |  |  | Enabled |  |  |  |  |  |  |  |  |  |  |
| Load Default Settings |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Set Supervisor Password |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Set User Password |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Save & Exit Setup |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Exit Without Saving |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | Show |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |