| Product System(PS) |  |  | Others(x) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Subject: C3 Reliability Test Report |  |  |  |  |  |  | Part No.: |  |  |  |  |  |  | Rev.: |  |
|  |  |  |  |  |  |  | Doc. No.: |  |  |  |  |  |  | 0 |  |
| Project Code: |  |  |  |  |  |  | Release Date: August 29, 2001 |  |  |  |  |  |  | Page 1 of 1 |  |
| Project Name/Model No.: S81M/VT3300 |  |  |  |  |  |  |  |  |  |  |  |  |  | Revision Status |  |
| Description: |  |  |  |  |  |  |  |  |  |  |  |  |  | Page | Rev. |
| S81M/VT3300 C3 Reliability Test Report |  |  |  |  |  |  |  |  |  |  |  |  |  | 1 ~ 62 |  |
| Reason For Release: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| First Release |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Item | Contents |  |  |  |  |  |  |  |  |  | Qy | Unit |  |  |  |
|  | Reliability Test Report |  |  |  |  |  |  |  |  |  | 62 | Page |  |  |  |
| Copy To(All Contents): Copy (Cover Sheet): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Approved By/ |  | Review By/ |  |  |  |  |  |  |  | QA |  |  | Prepared By/ |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  | MNV3/ |  |  |

# Form No: 704-R001- 01( 20010719)

RELIABILITY TEST REPORT

PRODUCT: S81M/VT3300

Issue Date: August 29, 2001

| PREPARED BY: Kevin Ho Engineer Reliability Engineering Department | REVIEWED BY: Alan S.F. Chang Technical Manager Reliability Engineering Department |
| --- | --- |
| APPROVED BY: Homer Yu Director Component & Reliability Engineering Center | REVIEWED BY: Vincent Wang Associate Director Component Engineering Department |

Test Summary

1. Voltage Dips and Short Interruptions Immunity Test	Pass

2. Electric Fast Transient/Burst Immunity Test	pass

3. Surge Immunity Test	.. pass

4. Electrostatic Discharge Susceptibility Test	pass

5. DC Margin and DC Current Measurement	pass

6. Power Saving & AC Power Consumption Test	pass

7. Line Voltage and Frequency Test	pass

8. Power ON/OFF Characteristics	pass

9. Temperature/Thermal Profile Test	pass

10. Operating Temperature and Humidity Test	pass

11. Non-operating Temperature and Humidity Test	pass

12. Low Temperature & High Temperature Cold Start Test	pass

13. Operating Vibration Test	pass

14. Non-operating Random Vibration Test (Unpacked)	pass

15. Transportation Vibration Test	pass

16. Acoustic Noise Measurement	pass

16-1. Sound Pressure Level	pass

16-2. Sound Power Level	pass

17. Transportation Drop Test	pass

Table of Contents

TEST COVERAGE	3

PREFACE	5

1. Voltage Dips and Short Interruptions Immunity Test	11

2. Electric Fast Transient/Burst Immunity Test	14

3. Surge Immunity Test	18

4. Electrostatic Discharge Susceptibility Test	22

5. DC Margin and DC Current Measurement	26

6. Power Saving & AC Power Consumption Test	30

7. Line Voltage and Frequency Test	32

8. Power ON/OFF Characteristics	34

9. Temperature/Thermal Profile Test	36

10. Operating Temperature and Humidity Test	40

11. Non-operating Temperature and Humidity Test	43

12. Low Temperature & High Temperature Cold Start Test	45

13. Operating Vibration Test	47

14. Non-operating Random Vibration Test (Unpacked)	50

15. Transportation Vibration Test	52

16 Acoustic Noise Measurement	54

16-1. Sound Pressure Level	54

16-2. Sound Power Level	58

17. Transportation Drop Test	60

# TEST COVERAGE

| SKU-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Components | Vendor | Model Name | Key Spec. (refer to AVLC) | Remark |
| Main Board | Acer | S81M | -1 |  |
| Processor | Intel | P4 2.0 G | Willamette 2.0GHz, 400MHzFSB, 256K, D0 |  |
| CPU Fan Sink |  | --- | For Willamette |  |
| DIMM Socket 1 | Infineon | 64V16220GV-7.5(B) (ACER) | 128MB, SDRAM |  |
| DIMM Socket 2 | --- | --- | --- |  |
| DIMM Socket 3 | --- | --- | --- |  |
| HDD | Seagate | ST320410A | U6 20GB 5400RPM |  |
| CD-ROM | AOpen | CD952E | 52X |  |
| FDD | Panasonic | --- | 3 Mode for H80 350mm |  |
| SPS | FSP | --- | 160W PFC |  |
| VGA Card | Leadtek | Winfast 3D S325 | Winfast 3D S325 (M64) 32MB |  |
| Modem | Ambit | T62M154.00 | 56K HCF |  |
| LAN | --- | --- | --- |  |
| Keyboard | Darfon | --- | US version, 104keys |  |
| Mouse | Logitech |  | Wheel Logitec/930541-0000 S59P |  |
| Speaker | Charming | V-1 | Acer USB Powered Speaker |  |
| Housing | Acer | --- | Genesis H34a |  |
| SKU-2 |  |  |  |  |
| Components | Vendor | Model Name | Key Spec. (refer to AVLC) | Remark |
| Main Board | Acer | S81M | -1 |  |
| Processor | Intel | P4 1.9G | Willamette 1.9GHz, 400MHzFSB, 256K, D0 |  |
| CPU Fan Sink |  | --- | For Willamette |  |
| DIMM Socket 1 | Micron | MT8LSDT1664AG -133B1 | 128MB, SDRAM |  |
| DIMM Socket 2 | --- | --- | --- |  |
| DIMM Socket 3 | --- | --- | --- |  |
| HDD | Seagate | ST340810A | U6 40GB 5400RPM |  |
| DVD-ROM | Pioneer | --- | 16X DVD |  |
| FDD | Panasonic | --- | 3 Mode for H80 350mm |  |
| SPS | Delta | --- | 200W non-PFC |  |
| VGA Card | Leadtek | Winfast Geforce 2 MX | Winfast Geforce 2 MX 32M LP |  |
| Modem | --- | --- | --- |  |
| LAN | 3Com | 3C905CX-TXM | 3Com 10/100 Mbps AOL2 |  |
| Keyboard | Darfon | --- | US version, 104keys |  |
| Mouse | Logitech | 3D PS2 | Logitec/S48A S61 |  |
| Speaker | Charming | V-1 | Acer USB Powered Speaker |  |
| Housing | Acer | --- | Genesis H34a |  |

Table A. Test coverage based on AVL-C

# PREFACE

- Definitions:

  - Performance Definitions

    - Electrical Function

- The following definitions of the Performance Criteria in operation mode are referred to EN 50082-1 or
 			IEC 1000-4-X EMS Standard. On the basis of the operating conditions and functional specifications
			of the equipment under test, the test results shall be classified as in the following table, unless
			different specifications are given by product specifications. These error or performance criteria
			definitions will be also used in each test item's "test criteria" field.

      - Performance Criteria A

| - EN50082-1/ - IEC 1000-4-X - Description | - - IEC 1000-4-X: Normal performance within the specification limits. - - EN 50082-1: The apparatus shall continue to operate as intended. - No degradation of performance or loss of function is allowed below a performance level specified by the manufacturer, when the apparatus is used as intended. The performance level may be replaced by a permissible loss of performance. If the minimum performance level or the permissible performance loss is not specified by the manufacturer then either of these may be derived from the product description and documentation and what the user may reasonably expect from the apparatus if used as intended. |
| --- | --- |
| - Error Level | - No Noticeable Error |
| - Description of Acer Component & Reliability Engineering Center | - Normal performance within the specification limits |

Table B

      - Performance Criteria B

| - EN50082-1/ - IEC 1000-4-X - Description | - - IEC 1000-4-X: Temporary degradation or loss of function or performance which is self-recoverable - - EN 50082-1: The apparatus shall continue to operate as intended after the test. No degradation of performance or loss of function is allowed below a performance level specified by the manufacturer, when the apparatus is used as intended. The performance level may be replaced by a permissible loss of performance. During the test, degradation of performance is however allowed. No change of actual operating state or stored data is allowed. If the minimum performance level or the permissible performance loss is not specified by the manufacturer then either of these may be derived from the product description and documentation and what the user may reasonably expect from the apparatus if used as intended. |
| --- | --- |
| - Error Level | - Soft Error |
| - Description of Acer Component & Reliability Engineering Center | - Temporary degradation or loss of function or performance which is self-recoverable within 10 seconds. No any error defined in the "Minor re-start error", "Re-start error" and “Damage Error” is permitted - e.g. - * Without user intervention: - Screen flicker; video still for a few seconds; temporary but not periodic noise from speaker; HDD read/write retry and then continue to operate; FDD read/write retry and then continue to operate; CDROM access retry and then continue to operate; Modem/Network Card data transfer retry and then continue to operate, etc. |

Table C

      - Performance Criteria “Minor C”

| - EN50082-1/ - IEC 1000-4-X - Description | - N/A |
| --- | --- |
| - Error Level | - Minor Re-start Error |
| - Description of Acer Component & Reliability Engineering Center | - Temporary degradation or loss of function or performance which is easy to recover with user intervention within 30 seconds. No any error defined in the "Re-start error" and “Damage Error” is permitted - e.g. - * With user intervention: - With just ONE key pressed from keyboard or ONE click from mouse, touch-pad or some other pointing device or unplug and re-plug abnormal peripheral to recover to EUT's original operation status. More than one key/click to recover the system operation is not permitted either. |

Table D

      - Performance Criteria C

| - EN50082-1/ - IEC 1000-4-X - Description | - - IEC 1000-4-X: Temporary degradation or loss of function or performance which requires operator intervention or system reset - - EN 50082-1: Temporary loss of function is allowed, provided the function is self recoverable or can be restored by the operation of the controls |
| --- | --- |
| - Error Level | - Re-start Error |
| - Description of Acer Component & Reliability Engineering Center | - Temporary degradation or loss of function or performance that requires operator intervention or system reset. - e.g. - System halt - Keyboard , mouse, touch-pad, track point or other user-interface devices halt - HDD, FDD, LS120, Zip or other storage device fails to perform data transfer but no data loss or physical damage happened - CDROM/DVD-ROM, etc., data access or movie playback suspend - Unexpected noise from speaker which can only recovered after reboot the system - Abnormal screen (shiver, missing line, black or white block, wavy, distortion, etc.) - Modem, Network Card data transmission interruption - System Shut Down unexpectedly - System reboot unexpectedly - Others defined by Acer PA |

Table E

      - Performance Criteria D

| - EN50082-1/ - IEC 1000-4-X - Description | - IEC 1000-4-X: Degradation or loss of function which is not recoverable due to damage of equipment (components) or software, or loss of data. - EN 50082-1: N/A |
| --- | --- |
| - Error Level | - Damage Error |
| - Description of Acer Component & Reliability Engineering Center | - Degradation or loss of function which is not recoverable due to damage of equipment (components) or software - Loss of data which includes CMOS data loss and hard file data loss which is identified by diagnostic program (QAFE or PC Doctor) or Windows ScanDisk utility with fixing actions. - There is no any safety issue such as smoke, fire and electrical shock, etc. |

- Note: The meaning of character "A","B" and "C" for performance criteria is the same as in EN 50082-1 EMS Standard

Table F

    - Requirements of Appearance & Mechanical Function

      - No scratch (especially after drop or non-operating vibration test)

      - No dent (especially after drop, shock, non-operating vibration test or non-operating 						temperature/humidity test)

      - No crack (especially after drop, shock, non-operating temperature/humidity or non-operating 					vibration test

      - No mold speckle (especially after temperature/humidity test)

      - No unclear letter or missing painting (especially after non-operating 								temperature/humidity test)

      - No easily removed button (especially after drop, vibration test or non-operating temperature / 					humidity test, etc.)

      - No obvious fingerprint after touching the surface of system housing to degrade the quality 						image of the product.

      - Input and output terminals' connection must be easily matched (customer simulation 						reliability test)

      - No units wobbles

      - No migration materials in the set's plinths (e.g. foot of EUT dropped off after drop test)

      - No shift of top cover (especially after drop, shock, vibration, non-operating 								temperature/humidity test)

      - No migration of fan-sink, fan, heat sink (especially after drop, shock test)

      - No transfigure, deformity (especially after non-operating temperature/humidity test)

      - No stiffness or squeak for buttons when pressed

      - No warp or rust for metal parts (especially after non-operating temperature/humidity test)

      - No slack (e.g. screw loose)

      - No serious cracks to cushion (divided to more than 2 parts, especially after drop test)

      - No severe damage to carton box to eliminate customer’s acceptance to the product 						(carton broken; especially after drop test or package compression test)

      - No difficult assembly problem

      - No obvious enlarged mechanical gap to degrade the quality image of the product

      - No unexpected enlarged gap. (e.g. enlarged gap between top cover and front panel after 						drop test)

  - Diagnostic Program

    - Component Diagnostics Guideline

| - Component | - Brief Checked | - Description of the Operations |
| --- | --- | --- |
| - 1. MB | - Check for CPU, Cache, Video and Memory (kernel control and high speed data transfer) | - Use PC Doctor Diagnostic Program: - Choose the following test and run Normal Test Set for loops: - 1.Whole CPU Test - 2.Whole Memory Test - 3.Video Test: Perform “Data Transfer” and “Text Output operation” |
| - 2. Graphic | - Graphic Adapter Setting; Combined with CDROM/DVDROM video test and PC doctor testing. | - 1024*768*(8, 15 or 16 bpp; use possible biggest one), 75Hz ergonomic timing: PC99 graphic requirement |
| - 3. HDD | - Hard Disk Drive File Read/Write/Compare Repeatedly | - Make a HDD source folder and store the source hard files in it (total file size shall be over 200MB) - Record HDD test starting date/time to a log file - Mark date/time for the starting of the test loop - Copy files from HDD source directory to the HDD destination directory: This includes read and write operations - Binary compare files between the above two directories (read operation) and check if there is any comparison error. - Remove the destination directory - Mark date/time for the ending of the test loop - Record the test duration (ending time minus starting time) to the log file - Repeat step 3 to 8 in an endless loop. |
| - 4. FDD | - Floppy Disk Drive File Read/Write/Compare Repeatedly | - Make a FDD source folder in HDD and store the source files in it (total file size shall be over 1.2MB) - Record FDD test starting date/time to a log file - Mark date/time for the starting of the test loop - Copy files in HDD source directory to the FDD destination directory: This includes FDD write operations - Binary compare files between the above two directories (read operation) and check if there is any comparison error. - Remove the destination directory - Mark date/time for the ending of the test loop - Record the test duration (ending time minus starting time) to the log file - According to the above test duration, pause the FDD operation such that the duty cycle is about 25% - Repeat step 3 to 9 in an endless loop. |
| - 5. CDROM | - CDROM - Action Movie Playback | - Playback VCD action movie repeatedly; refer to "Graphic" component for the display setting. |
| - 6. DVDROM | - DVDROM - Action Movie Playback | - Playback DVD action movie repeatedly; refer to "Graphic" component for the display setting. |
| - 7. CD-RW | - CD-RW - Read/Write/Compare Repeatedly | - Format a CDRW test disc - Make a CDRW source folder and store the source files in it (total file size shall be over 300MB) - Record CDRW test starting date/time to a log file - Mark date/time for the starting of the test loop - Copy files in CDRW source directory to the CDRW destination directory: This includes read and write operations - Binary compare files between the above two directories (read operation) and check if there is any comparison error. - Remove the destination directory - Mark date/time for the ending of the test loop - Record the test duration (ending time minus starting time) to the log file - According to the above test duration, pause the CDRW operation such that the duty cycle is about 25% - Repeat step 4 to 10 in an endless loop |
| - 8. ZIP100 - ZIP250 - LS120 | - Large capacity flexible disk operation: - Read/Write/Compare Repeatedly | - Operation is like CDRW, test duty cycle is about 25%. |
| - 9. KB - & mouse | - Keyboard & Mouse | - Manually function test |
| - 10. Audio | - Audio file playback | - Playback a MIDI file repeatedly; VCD/DVD movie includes WAVE sound playback. |
| - 11. Network | - Network Card files Read/Write/Compare Repeatedly | - Map a folder of a network neighborhood computer to a local disk - (J: ) - Make a LAN source folder in J: (this is corresponding to remote computer) and store the source hard files in it (total file size shall be over 200MB) - Record LAN test starting date/time to a log file - Mark date/time for the starting of the test loop - Copy files from LAN source directory to the LAN destination directory in local HDD. - Binary compare files between the above two directories and check if there is any comparison error. - Remove the destination directory - Mark date/time for the ending of the test loop - Record the test duration (ending time minus starting time) to the log file - According to the above test duration, pause the LAN operation such that the duty cycle is about 50% - Repeat step 4 to 10 in an endless loop. |
| - 12. Modem | - Modem Card files Read/Write/Compare Repeatedly via dial-up server | - Connecting to a dial-up server and refer to diagnostic method of "Network" to perform a files transfer operation but the duty cycle is about 25%. |

- Table G: Component Diagnostics

    - Diagnostic Programs:

      - Scorpion I: Execute the following component diagnostics simultaneously in Windows 						OS environment (refer to component diagnostics guideline)

        - Main Board

        - Graphic

        - HDD

        - FDD/ZIP100/ZIP250/LS120

        - CDROM/DVDROM/CDRW

        - Modem (if applicable)

        - LAN (if applicable)

        - Audio

        - Keyboard & Mouse

      - Scorpion II: Scorpion I except FDD/ZIP100/ZIP250/LS120 test

      - Scorpion III: Disable power management and stay idle in Windows OS environment

      - Scorpion IV: Execute the following component diagnostics simultaneously in Windows 						OS environment (refer to component diagnostics guideline)

        - HDD

        - CDROM/DVDROM/CDRW

      - Scorpion V: Windows Control Panel / System: device driver check

      - QAFE Diagnostics Program (quick check):
					For "Acceptance Criteria of C3 Entry", "Acoustic Noise Measurement" and for quick 						hardware functional check.

      - HDD13.EXE: Test program for HDD random read operation

      - ONOFF.EXE: For EUT's system time record for "Power ON/OFF Characteristics"

      - Intel GTM (Generic Throttle Monitor) or Landmark 3.0: For CPU speed test

      - KPOWER.EXE: for thermal profile measurement

      - 3Dmark2000: for thermal profile measurement

      - Acer Re-Boot Test Program: For EUT's system time record for "Power ON/OFF 							Characteristics"

      - Drive Probe: FDD diagnostics before and after mechanical shock/drop test

  - Laboratory Climatic Conditions: 
		The tests shall be carried out in standard climate conditions in accordance with IEC 68-1 (Environmental 		Testing - General and Guidance), unless otherwise stated:

    - Temperature: 15C ~ 35C

    - Relative humidity: 25% ~ 75%

    - Barometric pressure: 86kPa ~ 106kPa (860 mbar ~ 1060 mbar)

# 1. Voltage Dips and Short Interruptions Immunity Test

- Basic Info.:
	Test Date				: 	8/3/2001
	Test Location			: 	ESD & Power Line Interference Test Room of Wistron Product 								  	Reliability Lab
	Lab. Ambient			: 	233C, 455 %RH 
	Test Equipment		: 	1. Keytek ECAT Dip Generator Test System
								2. Pacific Power Source 140TMX

- Field of Application

  - New PC system

  - SPS model Change

  - MB engineering change

  - SPS engineering change (optional; consult with CE engineers)

- Test Configuration (refer to test coverage table & the field of application):

  - SKU-1

  - SKU-2

- Test Specification:

  - if 230Vac/50Hz is applicable, then UT is 120Vac/60Hz & 230Vac/50Hz

  - if 230Vac/50Hz is not applicable (e.g. SPS for Japan market solo), UT is 120Vac/60Hz

| - Condition | - Test level - (% of UT) | - Duration (ms) | - Number of disturbance | - Polarity | - Time interval between shoots |
| --- | --- | --- | --- | --- | --- |
| - 1 | - 0 | - 5000 | - 3 | - 90 - and - 0, 180 | - Depends on the time - to enter windows - environment |
| - 2 | - 40 | - 100 | - 3 | - 90 - and - 0, 180 | - Depends on the time - to enter windows - environment |
| - 3 | - 70 | - 10 | - 6 | - 90 - and - 0, 180 | - 10s |
| - 4 | - 0 | - 10 | - 6 | - 90 - and - 0, 180 | - 10s |
| - 5 | - 0 | - 20 | - 6 | - 90 - and - 0, 180 | - 10s |

Table 1-1

- Test Criteria:

| - Condition | - Performance criterion (permitted) |
| --- | --- |
| - 1 | - C |
| - 2 | - C |
| - 3 | - A |
| - 4 | - A |
| - 5 | - B (for reference) |

Table 1-2

- Diagnostic Program:

  - Scorpion I during test for condition 3,4 and 5

  - Scorpion II during test for condition 1 and 2

  - QAFE quick check after test

- Test Result:

  - SKU-1:

    - UT: 230Vac/50Hz

| - Test level - % UT | - Voltage dip and short interruptions % UT | - Duration | - Performance Criteria | - Phase | - Actual - Performance | - Result |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 | - 5000 ms | - C | - 0 | - C | - PASS |
|  |  |  |  | - 90 | - C | PASS |
|  |  |  |  | - 180 | - C | PASS |
| - 40 | - 60 | - 100 ms | - C | - 0 | - A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 70 | - 30 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 20 ms | - B - (for reference) | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |

Table 1-3

    - UT: 120Vac/60Hz

| - Test level - % UT | - Voltage dip and short interruptions % UT | - Duration | - Performance Criteria | - Phase | - Actual - Performance | - Result |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 | - 5000 ms | - C | - 0 | C | PASS |
|  |  |  |  | - 90 | C | PASS |
|  |  |  |  | - 180 | C | PASS |
| - 40 | - 60 | - 100 ms | - C | - 0 | - A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 70 | - 30 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 20 ms | - B - (for reference) | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |

Table 1-4

  - SKU-2:

    - UT: 230Vac/50Hz

| - Test level - % UT | - Voltage dip and short interruptions % UT | - Duration | - Performance Criteria | - Phase | - Actual - Performance | - Result |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 | - 5000 ms | - C | - 0 | - C | PASS |
|  |  |  |  | - 90 | - C | PASS |
|  |  |  |  | - 180 | - C | PASS |
| - 40 | - 60 | - 100 ms | - C | - 0 | - B | PASS |
|  |  |  |  | - 90 | - B | PASS |
|  |  |  |  | - 180 | - B | PASS |
| - 70 | - 30 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 20 ms | - B - (for reference) | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |

- Table 1-5

    - UT: 120Vac/60Hz

| - Test level - % UT | - Voltage dip and short interruptions % UT | - Duration | - Performance Criteria | - Phase | - Actual - Performance | - Result |
| --- | --- | --- | --- | --- | --- | --- |
| - 0 | - 100 | - 5000 ms | - C | - 0 | C | PASS |
|  |  |  |  | - 90 | C | PASS |
|  |  |  |  | - 180 | C | PASS |
| - 40 | - 60 | - 100 ms | - C | - 0 | - A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 70 | - 30 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 10 ms | - A | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |
| - 0 | - 100 | - 20 ms | - B - (for reference) | - 0 | A | PASS |
|  |  |  |  | - 90 | A | PASS |
|  |  |  |  | - 180 | A | PASS |

- Table 1-6

# 2. Electric Fast Transient/Burst Immunity Test

- Basic Info,:
	Test Date				: 	7/23/2001
	Test Location			: 	ESD & Power Line Interference Test Room of Wistron Product 								  	Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	KeyTek ECAT Electric Fast Transient Generator Test 							  	  		System
							  	2.	Pacific Power Source 140TMX

- Field of Application:

  - New PC System

  - SPS model change

  - MB engineering change

  - SPS engineering change

  - Modem model change or engineering change

  - Network card model change or engineering change

  - Keyboard model change or engineering change

  - Mouse model change or engineering change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications:

  - Input AC power ports

    - Rise time of one pulse			: 5ns

    - Impulse duration			: 50ns

    - Repetition frequency of the impulses	: 5kHz

    - Charge voltage			: +/- 1kV

    - Duration of the test for each condition	: 1 minutes

    - Ports to be tested			: Line, neutral and chassis ground

  - Ports for signal lines and control lines (if modem or network card is added or built in the system)

    - Rise time of one pulse			: 5ns

    - Impulse duration			: 50ns

    - Repetition frequency of the impulses	: 5kHz

    - Charge voltage			: +/- 0.5kV

    - Duration of the test for each condition	: 1 minutes

- Note: Capacitive coupling clamp used

- Test Criteria:

  - Test on Input AC power port:

| - Error Level / Testing voltage | - 500V | - 750V | - 1000V | - 1250V |
| --- | --- | --- | --- | --- |
| - Performance criteria permitted | - A | - A | - B | - Margin Test for reference |

- Table 2-1

  - Test on ports for signal line or control line: (only for signal or control line which is more than 3 meters 		long)

| - Error Level / Testing voltage | - 250V | - 500V | - 750V |
| --- | --- | --- | --- |
| - Performance criteria permitted | - A | - B | - Margin Test for reference |

- Table 2-2

- Diagnostic Program

  - AC power port: Scorpion I and keyboard / mouse manual operation check

  - Ports for signal line or control line: Modem or network card data transmission check

- Test Result:

  - SKU-1:

    - Input AC power ports

| - Order | - Test Point | - Burst applied | - Performance during test | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - L | - + 500 V | - A | - PASS |
| - 2 | - N | - + 500 V | - A | - PASS |
| - 3 | - PE | - + 500 V | - A | - PASS |
| - 4 | - L | - - 500 V | - A | - PASS |
| - 5 | - N | - - 500 V | - A | - PASS |
| - 6 | - PE | - - 500 V | - A | - PASS |
| - 7 | - L | - + 750 V | - A | - PASS |
| - 8 | - N | - + 750 V | - A | - PASS |
| - 9 | - PE | - + 750 V | - A | - PASS |
| - 10 | - L | - - 750 V | - A | - PASS |
| - 11 | - N | - - 750 V | - A | - PASS |
| - 12 | - PE | - - 750 V | - A | - PASS |
| - 13 | - L | - + 1000 V | - B | - PASS |
| - 14 | - N | - + 1000 V | - B | - PASS |
| - 15 | - PE | - + 1000 V | - B | - PASS |
| - 16 | - L | - - 1000 V | - B | - PASS |
| - 17 | - N | - - 1000 V | - B | - PASS |
| - 18 | - PE | - - 1000 V | - B | - PASS |
| - 19 | - L (for reference) | - + 1250 V | - B | - PASS |
| - 20 | - N (for reference) | - + 1250 V | - B | - PASS |
| - 21 | - PE (for reference) | - + 1250 V | - B | - PASS |
| - 22 | - L (for reference) | - - 1250 V | - B | - PASS |
| - 23 | - N (for reference) | - - 1250 V | - B | - PASS |
| - 24 | - PE (for reference) | - - 1250 V | - B | - PASS |

- Table 2-3

- Note: Burst applied to L, N and PE is asynchronous

    - Signal line or control line

| - Order | - Test Point | - Burst applied | - Performance during test | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - Modem line | - + 250 V | - A | - PASS |
| - 2 | - Modem line | - - 250 V | - A | - PASS |
| - 3 | - Modem line | - + 500 V | - B | - PASS |
| - 4 | - Modem line | - - 500 V | - B | - PASS |
| - 5 | - Modem line - (for reference) | - + 750 V | - B | - PASS |
| - 6 | - Modem line - (for reference) | - - 750 V | - B | - PASS |
| - 7 | - Network line | - + 250 V | - A | - PASS |
| - 8 | - Network line | - - 250 V | - A | - PASS |
| - 9 | - Network line | - + 500 V | - B | - PASS |
| - 10 | - Network line | - - 500 V | - B | - PASS |
| - 11 | - Network line - (for reference) | - + 750 V | - B | - PASS |
| - 12 | - Network line - (for reference) | - - 750 V | - B | - PASS |

- Table 2-4

  - SKU-2:

    - Input AC power ports

| - Order | - Test Point | - Burst applied | - Performance during test | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - L | - + 500 V | - A | - PASS |
| - 2 | - N | - + 500 V | - A | - PASS |
| - 3 | - PE | - + 500 V | - A | - PASS |
| - 4 | - L | - - 500 V | - A | - PASS |
| - 5 | - N | - - 500 V | - A | - PASS |
| - 6 | - PE | - - 500 V | - A | - PASS |
| - 7 | - L | - + 750 V | - A | - PASS |
| - 8 | - N | - + 750 V | - A | - PASS |
| - 9 | - PE | - + 750 V | - A | - PASS |
| - 10 | - L | - - 750 V | - A | - PASS |
| - 11 | - N | - - 750 V | - A | - PASS |
| - 12 | - PE | - - 750 V | - A | - PASS |
| - 13 | - L | - + 1000 V | - B | - PASS |
| - 14 | - N | - + 1000 V | - B | - PASS |
| - 15 | - PE | - + 1000 V | - B | - PASS |
| - 16 | - L | - - 1000 V | - B | - PASS |
| - 17 | - N | - - 1000 V | - B | - PASS |
| - 18 | - PE | - - 1000 V | - B | - PASS |
| - 19 | - L (for reference) | - + 1250 V | - B | - PASS |
| - 20 | - N (for reference) | - + 1250 V | - B | - PASS |
| - 21 | - PE (for reference) | - + 1250 V | - B | - PASS |
| - 22 | - L (for reference) | - - 1250 V | - B | - PASS |
| - 23 | - N (for reference) | - - 1250 V | - B | - PASS |
| - 24 | - PE (for reference) | - - 1250 V | - B | - PASS |

- Table 2-5

- Note: Burst applied to L, N and PE is asynchronous

    - Signal line or control line

| - Order | - Test Point | - Burst applied | - Performance during test | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - Modem line | - + 250 V | - A | - PASS |
| - 2 | - Modem line | - - 250 V | - A | - PASS |
| - 3 | - Modem line | - + 500 V | - B | - PASS |
| - 4 | - Modem line | - - 500 V | - B | - PASS |
| - 5 | - Modem line - (for reference) | - + 750 V | - B | - PASS |
| - 6 | - Modem line - (for reference) | - - 750 V | - B | - PASS |
| - 7 | - Network line | - + 250 V | - A | - PASS |
| - 8 | - Network line | - - 250 V | - A | - PASS |
| - 9 | - Network line | - + 500 V | - B | - PASS |
| - 10 | - Network line | - - 500 V | - B | - PASS |
| - 11 | - Network line - (for reference) | - + 750 V | - B | - PASS |
| - 12 | - Network line - (for reference) | - - 750 V | - B | - PASS |

- Table 2-6

# 3. Surge Immunity Test

- Basic Info,:
	Test Date				: 	8/4/2001
	Test Location			: 	ESD & Power Line Interference Test Room of Wistron Product 									Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1. KeyTek ECAT Lightning Surge Generator
							  	2. Pacific Power Source 140TMX

- Field of Application:

  - New PC System

  - SPS model change

  - MB engineering change at power circuit

  - SPS engineering change (optional; consult with CE engineer)

  - Keyboard model change

  - Mouse model change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specification

| - Test Mode | - Test - Voltage | - Polarity | - Phase Shifting | - No. - of surges | - Source - Impedance | - Repetition Rate |
| --- | --- | --- | --- | --- | --- | --- |
| - Line to line - (differential mode) | - 1kV | - Positive & - negative | - 0, 90 and 270 | - 5 | - 2 | - 1 surge/minute |
| - Line to ground - (common mode) | - 2kV | - Positive & - negative | - 0, 90 and 270 | - 5 | - 12 | - 1 surge/minute |

Table 3-1

- Test Criteria:

  - AC power port: Line to Line

| - Error Level / Testing Voltage | - 750V | - 1000V | - 1250V |
| --- | --- | --- | --- |
| - Performance criteria permitted | - A | - B | - Margin Test for reference |

Table 3-2

  - AC power port: Line to ground

| - Error Level / Testing Voltage | - 1500V | - 2000V | - 2250V |
| --- | --- | --- | --- |
| - Performance criteria permitted | - A | - B | - Margin Test for reference |

Table 3-3

- Diagnostic Program
	Scorpion I

- Test Result

  - SKU-1:

    - Line to Line (No. of surges applied in each sub-item: 5)

| - Item | - Mode | - Phase | - Impulse Voltage | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - L to N | - 0 | - 750V | - PASS |
| - 2 | - L to N | - 90 | - 750V | - PASS |
| - 3 | - L to N | - 270 | - 750V | - PASS |
| - 4 | - L to N | - 0 | - -750V | - PASS |
| - 5 | - L to N | - 90 | - -750V | - PASS |
| - 6 | - L to N | - 270 | - -750V | - PASS |
| - 7 | - L to N | - 0 | - 1000V | - PASS |
| - 8 | - L to N | - 90 | - 1000V | - PASS |
| - 9 | - L to N | - 270 | - 1000V | - PASS |
| - 10 | - L to N | - 0 | - -1000V | - PASS |
| - 11 | - L to N | - 90 | - -1000V | - PASS |
| - 12 | - L to N | - 270 | - -1000V | - PASS |
| - 13 | - L to N | - 0 | - 1250V | - PASS |
| - 14 | - L to N | - 90 | - 1250V | - PASS |
| - 15 | - L to N | - 270 | - 1250V | - PASS |
| - 16 | - L to N | - 0 | - -1250V | - PASS |
| - 17 | - L to N | - 90 | - -1250V | - PASS |
| - 18 | - L to N | - 270 | - -1250V | - PASS |

Table 3-4

    - Line to PE: (No. of surges applied in each sub-item: 5; in case there is safety ground of the system)

| - Item | - Mode | - Phase | - Impulse Voltage | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - L to PE | - 0 | - 1500V | - PASS |
| - 2 | - L to PE | - 90 | - 1500V | - PASS |
| - 3 | - L to PE | - 270 | - 1500V | - PASS |
| - 4 | - N to PE | - 0 | - 1500V | - PASS |
| - 5 | - N to PE | - 90 | - 1500V | - PASS |
| - 6 | - N to PE | - 270 | - 1500V | - PASS |
| - 7 | - L/N to PE | - 0 | - 1500V | - PASS |
| - 8 | - L/N to PE | - 90 | - 1500V | - PASS |
| - 9 | - L/N to PE | - 270 | - 1500V | - PASS |
| - 10 | - L to PE | - 0 | - -1500V | - PASS |
| - 11 | - L to PE | - 90 | - -1500V | - PASS |
| - 12 | - L to PE | - 270 | - -1500V | - PASS |
| - 13 | - N to PE | - 0 | - -1500V | - PASS |
| - 14 | - N to PE | - 90 | - -1500V | - PASS |
| - 15 | - N to PE | - 270 | - -1500V | - PASS |
| - 16 | - L/N to PE | - 0 | - -1500V | - PASS |
| - 17 | - L/N to PE | - 90 | - -1500V | - PASS |
| - 18 | - L/N to PE | - 270 | - -1500V | - PASS |
| - 19 | - L to PE | - 0 | - 2000V | - PASS |
| - 20 | - L to PE | - 90 | - 2000V | - PASS |
| - 21 | - L to PE | - 270 | - 2000V | - PASS |
| - 22 | - N to PE | - 0 | - 2000V | - PASS |
| - 23 | - N to PE | - 90 | - 2000V | - PASS |
| - 24 | - N to PE | - 270 | - 2000V | - PASS |
| - 25 | - L/N to PE | - 0 | - 2000V | - PASS |
| - 26 | - L/N to PE | - 90 | - 2000V | - PASS |
| - 27 | - L/N to PE | - 270 | - 2000V | - PASS |
| - 28 | - L to PE | - 0 | - -2000V | - PASS |
| - 29 | - L to PE | - 90 | - -2000V | - PASS |
| - 30 | - L to PE | - 270 | - -2000V | - PASS |
| - 31 | - N to PE | - 0 | - -2000V | - PASS |
| - 32 | - N to PE | - 90 | - -2000V | - PASS |
| - 33 | - N to PE | - 270 | - -2000V | - PASS |
| - 34 | - L/N to PE | - 0 | - -2000V | - PASS |
| - 35 | - L/N to PE | - 90 | - -2000V | - PASS |
| - 36 | - L/N to PE | - 270 | - -2000V | - PASS |
| - 37 | - L to PE | - 0 | - 2250V | - FAIL |
| - 38 | - L to PE | - 90 | - 2250V | - FAIL |
| - 39 | - L to PE | - 270 | - 2250V | - FAIL |
| - 40 | - N to PE | - 0 | - 2250V | - FAIL |
| - 41 | - N to PE | - 90 | - 2250V | - FAIL |
| - 42 | - N to PE | - 270 | - 2250V | - FAIL |
| - 43 | - L/N to PE | - 0 | - 2250V | - FAIL |
| - 44 | - L/N to PE | - 90 | - 2250V | - FAIL |
| - 45 | - L/N to PE | - 270 | - 2250V | - FAIL |
| - 46 | - L to PE | - 0 | - -2250V | - FAIL |
| - 47 | - L to PE | - 90 | - -2250V | - FAIL |
| - 48 | - L to PE | - 270 | - -2250V | - FAIL |
| - 49 | - N to PE | - 0 | - -2250V | - FAIL |
| - 50 | - N to PE | - 90 | - -2250V | - FAIL |
| - 51 | - N to PE | - 270 | - -2250V | - FAIL |
| - 52 | - L/N to PE | - 0 | - -2250V | - FAIL |
| - 53 | - L/N to PE | - 90 | - -2250V | - FAIL |
| - 54 | - L/N to PE | - 270 | - -2250V | - FAIL |

Table 3-5

  - SKU-2:

    - Line to Line (No. of surges applied in each sub-item: 5)

| - Item | - Mode | - Phase | - Impulse Voltage | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - L to N | - 0 | - 750V | - PASS |
| - 2 | - L to N | - 90 | - 750V | - PASS |
| - 3 | - L to N | - 270 | - 750V | - PASS |
| - 4 | - L to N | - 0 | - -750V | - PASS |
| - 5 | - L to N | - 90 | - -750V | - PASS |
| - 6 | - L to N | - 270 | - -750V | - PASS |
| - 7 | - L to N | - 0 | - 1000V | - PASS |
| - 8 | - L to N | - 90 | - 1000V | - PASS |
| - 9 | - L to N | - 270 | - 1000V | - PASS |
| - 10 | - L to N | - 0 | - -1000V | - PASS |
| - 11 | - L to N | - 90 | - -1000V | - PASS |
| - 12 | - L to N | - 270 | - -1000V | - PASS |
| - 13 | - L to N | - 0 | - 1250V | - PASS |
| - 14 | - L to N | - 90 | - 1250V | - PASS |
| - 15 | - L to N | - 270 | - 1250V | - PASS |
| - 16 | - L to N | - 0 | - -1250V | - PASS |
| - 17 | - L to N | - 90 | - -1250V | - PASS |
| - 18 | - L to N | - 270 | - -1250V | - PASS |

Table 3-6

    - Line to PE: (No. of surges applied in each sub-item: 5; in case there is safety ground of the system)

| - Item | - Mode | - Phase | - Impulse Voltage | - Result |
| --- | --- | --- | --- | --- |
| - 1 | - L to PE | - 0 | - 1500V | - PASS |
| - 2 | - L to PE | - 90 | - 1500V | - PASS |
| - 3 | - L to PE | - 270 | - 1500V | - PASS |
| - 4 | - N to PE | - 0 | - 1500V | - PASS |
| - 5 | - N to PE | - 90 | - 1500V | - PASS |
| - 6 | - N to PE | - 270 | - 1500V | - PASS |
| - 7 | - L/N to PE | - 0 | - 1500V | - PASS |
| - 8 | - L/N to PE | - 90 | - 1500V | - PASS |
| - 9 | - L/N to PE | - 270 | - 1500V | - PASS |
| - 10 | - L to PE | - 0 | - -1500V | - PASS |
| - 11 | - L to PE | - 90 | - -1500V | - PASS |
| - 12 | - L to PE | - 270 | - -1500V | - PASS |
| - 13 | - N to PE | - 0 | - -1500V | - PASS |
| - 14 | - N to PE | - 90 | - -1500V | - PASS |
| - 15 | - N to PE | - 270 | - -1500V | - PASS |
| - 16 | - L/N to PE | - 0 | - -1500V | - PASS |
| - 17 | - L/N to PE | - 90 | - -1500V | - PASS |
| - 18 | - L/N to PE | - 270 | - -1500V | - PASS |
| - 19 | - L to PE | - 0 | - 2000V | - PASS |
| - 20 | - L to PE | - 90 | - 2000V | - PASS |
| - 21 | - L to PE | - 270 | - 2000V | - PASS |
| - 22 | - N to PE | - 0 | - 2000V | - PASS |
| - 23 | - N to PE | - 90 | - 2000V | - PASS |
| - 24 | - N to PE | - 270 | - 2000V | - PASS |
| - 25 | - L/N to PE | - 0 | - 2000V | - PASS |
| - 26 | - L/N to PE | - 90 | - 2000V | - PASS |
| - 27 | - L/N to PE | - 270 | - 2000V | - PASS |
| - 28 | - L to PE | - 0 | - -2000V | - PASS |
| - 29 | - L to PE | - 90 | - -2000V | - PASS |
| - 30 | - L to PE | - 270 | - -2000V | - PASS |
| - 31 | - N to PE | - 0 | - -2000V | - PASS |
| - 32 | - N to PE | - 90 | - -2000V | - PASS |
| - 33 | - N to PE | - 270 | - -2000V | - PASS |
| - 34 | - L/N to PE | - 0 | - -2000V | - PASS |
| - 35 | - L/N to PE | - 90 | - -2000V | - PASS |
| - 36 | - L/N to PE | - 270 | - -2000V | - PASS |

Table 3-7

# 4. Electrostatic Discharge Susceptibility Test

- Basic Info,:
	Test Date				: 	8/7/2001
	Test Location			: 	ESD & Power Line Interference Test Room of Wistron Product 									Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Electrostatic Discharge Tester (KeyTek Series 2000): 										KeyTek model ESD-1 ESD Simulator.

- 2.	IEC Discharge Network, KeyTek model DN-10 rated 150 										pF, 330 ohm with DT-2 (point) or DT-1(IEC Ball) discharge 										tip.

- Field of Application:

  - New PC system

  - Housing model change

  - SPS model change or engineering change

  - Front panel engineering change

  - CD-ROM/ DVD-ROM model change

  - FDD/LS-120/ZIP drive model change

  - Modem card/ network card model change

  - Audio card engineering change

  - Keyboard model change

  - Mouse model change

  - Main board engineering change near I/O port

  - Add on card change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications and Criteria

  - Climatic conditions shall be within the following ranges: (IEC 1000-4-2)

    - Ambient Temperature: 15C ~ 35C

    - Relative Humidity: 30% ~ 60%

    - Atmospheric Pressure: 86kPa to 106kPa

  - Details

|  | - ESD Voltage Level |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Acceptance Criteria | - Air Discharge | - Contact Discharge | - Air Discharge | - Contact Discharge | - Air Discharge | - Contact Discharge |
|  | - 4 kV | - 4 kV | - 8 kV | - 6 kV | - 15 kV | - 8 kV |
| - A | - Permitted |  | - Permitted |  | - Permitted |  |
| - B | - Not Permitted |  | - Permitted |  | - Permitted |  |
| - C | - Not Permitted |  | - Not permitted |  | - Permitted |  |
| - D | - Not Permitted |  | - Not Permitted |  | - Not Permitted |  |
| - Remark | - Note (1), (2) |  |  |  | - Note (3) |  |

- Table 4-1

- Note:

    - It is a minimum spec. of the add-on card which built in a PC system under an operation mode.

    - Following the spec also if the speakers with finger guard are exposed.

    - It is a minimum spec. of an operating PC whose add-on card under the idle mode.

  - At least 10 single discharges shall be applied to a pre-selected point

  - Time interval between successive single discharge: 1 second (standard) or 3 seconds (except 				communication cards).

  - Time interval between successive single discharge for test points on modem or LAN card: 3 seconds

  - If some selected test point can be susceptible for more than 20 successive single discharge at certain 			discharge type, level and parity, then the test point is said to pass that ESD test at the discharge type, 			level and parity.

- Diagnostic Program:
	Scorpion I and Windows 98's Game: Solitaire

- Test Result:

  - Attach 2 pictures indicating the ESD test points here (front view and rear view)

  - SKU-1

|  |  | - ESD Voltage Level |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Test results & Performance Criteria |  | - AIR DISCHARGE |  |  |  |  |  |  |
|  |  | -  4kV |  | -  8kV |  |  | -  15kV |  |
|  |  | - Performance - Level | - Result | - Performance - Level | - Result | - Performance - Level |  | - Result |
| - No. | - Test Location - (for example) | - A/B/C/D | - Pass/ - Fail | - A/B/C/D | - Pass/ - Fail | - A/B/C/D |  | - Pass/ - Fail |
| - 1 | - Power LED indicators | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 2 | - HDD LED indicators | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 3 | - FDD LED indicators | - --- | - --- | - --- | - --- | - --- |  | - --- |
| - 4 | - Push bottom of FDD | - --- | - --- | - --- | - --- | - --- |  | - --- |
| - 5 | - Push bottoms of CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 6 | - Headphone jack of CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 7 | - Volume control of CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 8 | - Aperture around power switch | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 9 | - Aperture around FDD | - --- | - --- | - --- | - --- | - --- |  | - --- |
| - 10 | - Aperture around CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 11 | - Headphone of front panel | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 12 | - Microphone of front panel | - A | - PASS | - B | - PASS | - B |  | - PASS |

- Table 4-2

|  |  | - ESD Voltage Level |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Test results & Performance Criteria |  |  | - CONTACT DISCHARGE |  |  |  |  |  |  |  |
|  |  |  | -  4kV |  |  | -  6kV |  |  | -  8kV |  |
|  |  | - Performance - Level |  | - Result | - Performance - Level |  | - Result | - Performance - Level |  | - Result |
| - No. | - Test Location - (for example) | - A/B/C/D |  | - Pass/ - Fail | - A/B/C/D |  | - Pass/ - Fail | - A/B/C/D |  | - Pass/ - Fail |
| - 1 | - Outer screws | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 2 | - SPS screws | - A |  | - PASS | - B |  | - PASS | - C |  | - PASS |
| - 3 | - PS2 keyboard - connector grilles | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 4 | - PS2 mouse - connector grilles | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 5 | - USB port - connector grilles | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 6 | - LAN port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 7 | - Print port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 8 | - Game port connector hoods | - A |  | - PASS | - B |  | - PASS | - C |  | - PASS |
| - 9 | - COM1 port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 10 | - COM2 port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 11 | - VGA Card bracket | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 12 | - MODEM Card bracket | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 13 | - I/O bracket | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |

- Table 4-3

  - SKU-2

|  |  | - ESD Voltage Level |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Test results & Performance Criteria |  | - AIR DISCHARGE |  |  |  |  |  |  |
|  |  | -  4kV |  | -  8kV |  |  | -  15kV |  |
|  |  | - Performance - Level | - Result | - Performance - Level | - Result | - Performance - Level |  | - Result |
| - No. | - Test Location - (for example) | - A/B/C/D | - Pass/ - Fail | - A/B/C/D | - Pass/ - Fail | - A/B/C/D |  | - Pass/ - Fail |
| - 1 | - Power LED indicators | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 2 | - HDD LED indicators | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 3 | - FDD LED indicators | - --- | - --- | - --- | - --- | - --- |  | - --- |
| - 4 | - Push bottom of FDD | - --- | - --- | - --- | - --- | - --- |  | - --- |
| - 5 | - Push bottoms of CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 6 | - Headphone jack of CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 7 | - Volume control of CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 8 | - Aperture around power switch | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 9 | - Aperture around FDD | - --- | - --- | - --- | - --- | - --- |  | - --- |
| - 10 | - Aperture around CDROM | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 11 | - Headphone of front panel | - A | - PASS | - B | - PASS | - B |  | - PASS |
| - 12 | - Microphone of front panel | - A | - PASS | - B | - PASS | - B |  | - PASS |

- Table 4-4

|  |  | - ESD Voltage Level |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| - Test results & Performance Criteria |  |  | - CONTACT DISCHARGE |  |  |  |  |  |  |  |
|  |  |  | -  4kV |  |  | -  6kV |  |  | -  8kV |  |
|  |  | - Performance - Level |  | - Result | - Performance - Level |  | - Result | - Performance - Level |  | - Result |
| - No. | - Test Location - (for example) | - A/B/C/D |  | - Pass/ - Fail | - A/B/C/D |  | - Pass/ - Fail | - A/B/C/D |  | - Pass/ - Fail |
| - 1 | - Outer screws | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 2 | - SPS screws | - A |  | - PASS | - B |  | - PASS | - C |  | - PASS |
| - 3 | - PS2 keyboard - connector grilles | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 4 | - PS2 mouse - connector grilles | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 5 | - USB port - connector grilles | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 6 | - LAN port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 7 | - Print port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 8 | - Game port connector hoods | - A |  | - PASS | - B |  | - PASS | - C |  | - PASS |
| - 9 | - COM1 port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 10 | - COM2 port connector hoods | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 11 | - VGA Card bracket | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 12 | - MODEM Card bracket | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |
| - 13 | - I/O bracket | - A |  | - PASS | - B |  | - PASS | - B |  | - PASS |

- Table 4-5

# 5. DC Margin and DC Current Measurement

- Basic Info,:
	Test Date				: 	6/5/2001
	Test Location			: 	Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.DC Power Supply (such as HP 6031A, HP 6542A, HP 										E3610A, HP 6651A bench power supply or Wistron laboratory 									tools)

- 2.	Digital Multi-meter

- Field of Application:

  - New PC system

  - Chipsets/ CPU/ Memory module model change

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - Add on card model change

  - Keyboard/mouse model change

  - Main board engineering change at power circuit

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications:

  - Product specification function test (for main board and key components)

| - DC Voltage on M.B. power socket |  |  |
| --- | --- | --- |
| - +5Vdc nominal | - +12Vdc nominal | - +3.3Vdc nominal |
| - +5% | - +5% | - +5% |
|  | - -5% |  |
| - -5% | - +5% |  |
|  | - -5% |  |
| - +5% | - +5% | - -5% |
|  | - -5% |  |
| - -5% | - +5% |  |
|  | - -5% |  |

- Table 5-1

  - Marginal test for main board only (key component use normal DC power)

| - DC Voltage on M.B. power socket |  |
| --- | --- |
| - +5Vdc nominal | - +3.3Vdc nominal |
| - 4.9 | - 3.2 |
| - 4.8 | - 3.1 |
| - 4.7 | - 3.0 |
| - 4.6 | - 2.9 |
| - 4.5 | - 2.8 |
| - 4.4 | - 3.4 |
| - 5.1 | - 3.5 |
| - 5.2 | - 3.6 |
| - 5.3 | - 3.7 |
| - 5.4 | - 3.8 |
| - 5.5 | - 3.9 |
| - 5.6 | - --- |
| - 5.7 | - --- |
| - 5.8 | - --- |
| - 5.9 | - --- |

- Table 5-2

- Note: +5Vdc nominal and +3.3V nominal marginal test is performed separately (no need to 			combine the two DC voltages).

  - Test Duration:

    - Product specification function test (for main board and key component)

      - 30 minutes at least for each test condition while Diagnostic Program is Scorpion I

      - Two loops of  QAFE quick check for each test item

    - Marginal test for main board

      - PC Doctor 2.0 for CPU full functional diagnostics for each sub test item

      - PC Doctor 2.0 for Memory full functional diagnostics for each sub test item

  - Test coverage for main board marginal test

    - Lightest loading for memory module: e.g. 64MB DIMM at DIMM socket 1 (for each memory vendor)

    - Heaviest loading for memory module: e.g. 128MB DIMM at ALL DIMM sockets (for each memory 				vendor)

    - All CPU with different manufacturing process should be tested.

- Test Criteria:

  - Product specification function test:
		Performance criteria A must be met for each test condition.

  - Marginal test for main board:

    - Performance criteria B shall be met for 10% deviation for each DC voltage.

    - For voltage deviation more than 10% and failure occurred case, EE shall provide the corresponding 				root cause for review.

- Diagnostic Program:

  - Scorpion I

  - PC Doctor 2.0

- Test Result:

  - SKU-1:

    - Product specification Function Test

| - DC Voltage |  |  | - Scorpion I Test Result | - Remark |
| --- | --- | --- | --- | --- |
| - +5Vdc | - +12Vdc | - +3.3Vdc |  |  |
| - +5% | - +5% | - +5% | - PASS |  |
| - +5% | - -5% | - +5% | - PASS |  |
| - -5% | - +5% | - +5% | - PASS |  |
| - -5% | - -5% | - +5% | - PASS |  |
| - +5% | - +5% | - -5% | - PASS |  |
| - +5% | - -5% | - -5% | - PASS |  |
| - -5% | - +5% | - -5% | - PASS |  |
| - -5% | - -5% | - -5% | - PASS |  |

- Table 5-3

    - Current consumption test: The current consumption of the EUT for each applied voltage are :

| - Current Consumption Each Voltage Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Condition | - + 5 V | - + 3.3 V | - +12 V | - ( * ) V |
| - Operating (max load) | - 1.202A | - 4.95A | - 4.2A |  |
| - Idle | - 0.978A | - 3.75A | - 1.04A |  |
| - Standby (if applicable) | - 0.334A | - 1.17A | - 0.615A |  |

- Table 5-4

    - Marginal Test for main board

| - DC Voltage |  | - Test Result of PC Doctor 2.0 - (CPU, Memory group test) | - Remarks |
| --- | --- | --- | --- |
| - +5Vdc - Nominal | - +3.3Vdc - Nominal |  |  |
| - 5.0 | - 3.3 | - PASS |  |
| - 4.9 | - 3.3 | - PASS |  |
| - 4.8 | - 3.3 | - PASS |  |
| - 4.7 | - 3.3 | - PASS |  |
| - 4.6 | - 3.3 | - PASS |  |
| - 4.5 | - 3.3 | - PASS |  |
| - 4.4 | - 3.3 | - PASS |  |
| - 5.1 | - 3.3 | - PASS |  |
| - 5.2 | - 3.3 | - PASS |  |
| - 5.3 | - 3.3 | - PASS |  |
| - 5.4 | - 3.3 | - PASS |  |
| - 5.5 | - 3.3 | - PASS |  |
| - 5.6 | - 3.3 | - PASS |  |
| - 5.7 | - 3.3 | - PASS |  |
| - 5.8 | - 3.3 | - PASS |  |
| - 5.9 | - 3.3 | - PASS |  |
| - 5.0 | - 3.2 | - PASS |  |
| - 5.0 | - 3.1 | - PASS |  |
| - 5.0 | - 3.0 | - PASS |  |
| - 5.0 | - 2.9 | - PASS |  |
| - 5.0 | - 2.8 | - PASS |  |
| - 5.0 | - 3.4 | - PASS |  |
| - 5.0 | - 3.5 | - PASS |  |
| - 5.0 | - 3.6 | - PASS |  |
| - 5.0 | - 3.7 | - PASS |  |
| - 5.0 | - 3.8 | - PASS |  |
| - 5.0 | - 3.9 | - PASS |  |

- Table 5-5

  - SKU-2:

    - Product specification Function Test

| - DC Voltage |  |  | - Scorpion I Test Result | - Remark |
| --- | --- | --- | --- | --- |
| - +5Vdc | - +12Vdc | - +3.3Vdc |  |  |
| - +5% | - +5% | - +5% | - PASS |  |
| - +5% | - -5% | - +5% | - PASS |  |
| - -5% | - +5% | - +5% | - PASS |  |
| - -5% | - -5% | - +5% | - PASS |  |
| - +5% | - +5% | - -5% | - PASS |  |
| - +5% | - -5% | - -5% | - PASS |  |
| - -5% | - +5% | - -5% | - PASS |  |
| - -5% | - -5% | - -5% | - PASS |  |

- Table 5-6

    - Current consumption test: The current consumption of the EUT for each applied voltage are :

| - Current Consumption Each Voltage Setting |  |  |  |  |
| --- | --- | --- | --- | --- |
| - Condition | - + 5 V | - + 3.3 V | - +12 V | - ( * ) V |
| - Operating (max load) | - 1.202A | - 4.95A | - 4.2A |  |
| - Idle | - 0.978A | - 3.75A | - 1.04A |  |
| - Standby (if applicable) | - 0.334A | - 1.17A | - 0.615A |  |

- Table 5-7

    - Marginal Test for main board

| - DC Voltage |  | - Test Result of PC Doctor 2.0 - (CPU, Memory group test) | - Remarks |
| --- | --- | --- | --- |
| - +5Vdc - Nominal | - +3.3Vdc - Nominal |  |  |
| - 5.0 | - 3.3 | - PASS |  |
| - 4.9 | - 3.3 | - PASS |  |
| - 4.8 | - 3.3 | - PASS |  |
| - 4.7 | - 3.3 | - PASS |  |
| - 4.6 | - 3.3 | - PASS |  |
| - 4.5 | - 3.3 | - PASS |  |
| - 4.4 | - 3.3 | - PASS |  |
| - 5.1 | - 3.3 | - PASS |  |
| - 5.2 | - 3.3 | - PASS |  |
| - 5.3 | - 3.3 | - PASS |  |
| - 5.4 | - 3.3 | - PASS |  |
| - 5.5 | - 3.3 | - PASS |  |
| - 5.6 | - 3.3 | - PASS |  |
| - 5.7 | - 3.3 | - PASS |  |
| - 5.8 | - 3.3 | - PASS |  |
| - 5.9 | - 3.3 | - PASS |  |
| - 5.0 | - 3.2 | - PASS |  |
| - 5.0 | - 3.1 | - PASS |  |
| - 5.0 | - 3.0 | - PASS |  |
| - 5.0 | - 2.9 | - PASS |  |
| - 5.0 | - 2.8 | - PASS |  |
| - 5.0 | - 3.4 | - PASS |  |
| - 5.0 | - 3.5 | - PASS |  |
| - 5.0 | - 3.6 | - PASS |  |
| - 5.0 | - 3.7 | - PASS |  |
| - 5.0 | - 3.8 | - PASS |  |
| - 5.0 | - 3.9 | - PASS |  |

- Table 5-8

# 6. Power Saving & AC Power Consumption Test

- Basic Info,:
	Test Date				: 	7/20/2001
	Test Location			: 	Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Frequency Converter

- 2.	True RMS power Meter

- Field of Application

  - New PC system

  - SPS model change

  - CDROM/DVD-ROM/CDRW model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - CPU model change

  - Fan model change

  - BIOS version change

  - Add on card model change

  - MB engineering change

  - OS change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications

  - Ambient Temperature		: 25  3 C

  - Input AC Voltage		: 1155 Vac RMS, 603Hz and 2305 Vac RMS , 503 Hz

  - Line Impedance		: less than 0.25 ohm

  - Total Harmonic Distortion (voltage)	: < 5%

- Test Criteria:
	To Meet USA EPA energy star program for computers:

  - Power management features must be activated before shipping with initial pre-set default (BIOS 			setting check).

  - 30 Watts maximum in low-power state.

  - The computer shall include one or more mechanisms through which it can activate the low-power 			modes of an ENERGY STAR monitor

  - When the system is awakened, the user shall be returned automatically to the same situation that 			existed prior to activation of the sleep mode

  - If the system is LAN on board or with a network card, the connection of the network should be 				remained after the system is awakened.

- Diagnostic Program

  - Scorpion I

  - Scorpion II

- Test Result

  - SKU-1:

    - AC Power measurement:

| - AC test condition - Configuration | - 230Vac, 50Hz - (Wrms) |
| --- | --- |
| - Windows 98 low power state - (within 30 minutes after system idle) | - 72 |
| - Scorpion I | - 132 |

- Table 6-1

    - AC Current measurement:

| - AC test condition - Configuration | - 230Vac, 50Hz - (Ampere) |
| --- | --- |
| - Windows 98 low power state - (within 30 minutes after system idle) | - 0.418 |
| - Scorpion I | - 0.756 |

- Table 6-2

  - SKU-2:

    - AC Power measurement:

| - AC test condition - Configuration | - 230Vac, 50Hz - (Wrms) |
| --- | --- |
| - Windows 98 low power state - (within 30 minutes after system idle) | - 72 |
| - Scorpion I | - 133 |

- Table 6-3

    - AC Current measurement:

| - AC test condition - Configuration | - 230Vac, 50Hz - (Ampere) |
| --- | --- |
| - Windows 98 low power state - (within 30 minutes after system idle) | - 0.614 |
| - Scorpion I | - 1.133 |

- Table 6-4

# 7. Line Voltage and Frequency Test

- Basic Info,:
	Test Date				: 	8/6/2001
	Test Location			: 	Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Frequency Converter
								2.	Power Meter
								3.	Multimeter

- Field of Application:

  - New PC system

  - SPS model change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications

  - Function Test I

    - Voltage

      - 100V/120V (-/+ 10%)					: 90~132V

      - 200V/240V (-/+ 10%)					: 180~264V

    - Frequency

      - 50Hz (-/+ 3Hz)						: 47~53Hz

      - 60Hz (-/+ 3Hz)						: 57~63Hz

  - Function Test II

    - Voltage variation (frequency is fixed to 50Hz or 60Hz)		:

      - 90V to 132V increasingly and 132V to 90V decreasingly

      - 180V to 264V increasingly and 264V to 180V decreasingly

    - Frequency variation (voltage is fixed to 115V or 230V)		: 47Hz to 63Hz and 63Hz to 47Hz

- Test Criteria:
	Performance Criteria A must be met for both test conditions

- Diagnostic Program: 
	Scorpion I

- Test Result:

  - SKU-1:

    - Function test I

| - Frequency | - 50Hz |  | - 60Hz |  |
| --- | --- | --- | --- | --- |
| - Voltage ( V ) | - 47 Hz | - 53 Hz | - 57 Hz | - 63 Hz |
| - 90 | - PASS | - PASS | - PASS | - PASS |
| - 132 | - PASS | - PASS | - PASS | - PASS |
| - 180 | - PASS | - PASS | - PASS | - PASS |
| - 264 | - PASS | - PASS | - PASS | - PASS |

Table 7-1

    - Function test II

| - Frequency | - Voltage |  |  |  | - Frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Voltage (V) or - Frequency(Hz) | - 90V~132V | - 132V~90V | - 264V~180V | - 118V~264V | - 47Hz~63Hz | - 63Hz~47Hz |
| - 115Vac | - --- | - --- | - --- | - --- | - PASS | - PASS |
| - 230Vac | - --- | - --- | - --- | - --- | - PASS | - PASS |
| - 50Hz | - PASS | - PASS | - PASS | - PASS | - --- | - --- |
| - 60Hz | - PASS | - PASS | - PASS | - PASS | - --- | - --- |

Table 7-2

  - SKU-2:

    - Function test I

| - Frequency | - 50Hz |  | - 60Hz |  |
| --- | --- | --- | --- | --- |
| - Voltage ( V ) | - 47 Hz | - 53 Hz | - 57 Hz | - 63 Hz |
| - 90 | - PASS | - PASS | - PASS | - PASS |
| - 132 | - PASS | - PASS | - PASS | - PASS |
| - 180 | - PASS | - PASS | - PASS | - PASS |
| - 264 | - PASS | - PASS | - PASS | - PASS |

Table 7-3

    - Function test II

| - Frequency | - Voltage |  |  |  | - Frequency |  |
| --- | --- | --- | --- | --- | --- | --- |
| - Voltage (V) or - Frequency(Hz) | - 90V~132V | - 132V~90V | - 264V~180V | - 118V~264V | - 47Hz~63Hz | - 63Hz~47Hz |
| - 115Vac | - --- | - --- | - --- | - --- | - PASS | - PASS |
| - 230Vac | - --- | - --- | - --- | - --- | - PASS | - PASS |
| - 50Hz | - PASS | - PASS | - PASS | - PASS | - --- | - --- |
| - 60Hz | - PASS | - PASS | - PASS | - PASS | - --- | - --- |

Table 7-4

# 8. Power ON/OFF Characteristics

- Basic Info,:
	Test Date				: 	7/20/2001
	Test Location			: 	Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	Electronic timer

- Field of Application:

  - New PC system

  - SPS model change

  - Main board engineering change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications

  - AC power line on/off:

    - Basic Function Test:

      - System ON Time 	: Booting OK and Entering Windows OS successfully

      - System OFF Time	: 20 seconds

      - ON/OFF Cycle		: 1000 times

    - Margin Test:

      - System ON/OFF time	: 1 second

      - ON/OFF Cycle 		: 100 times

  - Windows OS restart:

    - Restart Cycle		: 1000 times

- Test Criteria:

  - AC power line on/off:

    - Basic function Test:

      - Successfully boot to Windows OS environment at each power on/off cycle

      - No unacceptable RTC data loss

      - No physical damage after test

      - Function check OK using QAFE "quick check" after the test

    - Margin Test:

      - No unacceptable RTC data loss

      - No damage or safety problem after test

      - Function check OK using QAFE "quick check" after the test

  - Windows OS restart:

- Successfully restart to Windows OS at each cycle

- Diagnostic Program

  - Wistron Re-Boot test program

  - QAFE

  - Time marking program

- Test Result

  - SKU-1:

    - AC power line on/off test:

| - POWER ON/OFF Test |  | - Result |
| --- | --- | --- |
| - Basic - Function Test | - Boot successfully each cycle | - PASS |
|  | - No unexpected RTC data loss | - PASS |
|  | - No damage problem after test | - PASS |
|  | - QAFE quick check | - PASS |
| - Margin Test | - No unexpected RTC data loss | - PASS |
|  | - No damage problem after test | - PASS |
|  | - QAFE quick check | - PASS |

- Table 8-1

    - Windows restart test:

| - POWER ON/OFF Test | - Result |
| --- | --- |
| - Restart successfully each cycle | - PASS |

- Table 8-2

  - SKU-2:

    - AC power line on/off test:

| - POWER ON/OFF Test |  | - Result |
| --- | --- | --- |
| - Basic - Function Test | - Boot successfully each cycle | - PASS |
|  | - No unexpected RTC data loss | - PASS |
|  | - No damage problem after test | - PASS |
|  | - QAFE quick check | - PASS |
| - Margin Test | - No unexpected RTC data loss | - PASS |
|  | - No damage problem after test | - PASS |
|  | - QAFE quick check | - PASS |

- Table 8-3

    - Windows restart test:

| - POWER ON/OFF Test | - Result |
| --- | --- |
| - Restart successfully each cycle | - PASS |

- Table 8-4

# 9. Temperature/Thermal Profile Test

- Basic Info,:
	Test Date				: 	8/8/2001
	Test Location			: 	Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Temperature Recorder
								2.	Thermocouple
								3.	IR camera

- Field of Application:

  - New PC system

  - Housing model change

  - Front panel engineering change with thermal issue

  - SPS model change; SPS engineering change

  - Intake or exhaust of housing change

  - CPU, Chipsets, VGA, Audio, Modem, Lan, Power Chip, etc. model change

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - System fan RPM speed change

  - Heat sink thickness, shape or model change

  - CPU fan model change or engineering change

- Test Configurations:

  - SKU-1

- Test Specifications

  - Test Environment:

    - For “compact” housing with smaller volume such as IDB3, IDB3J or H20:

- EUT is tested in 35C/50% R.H. temperature/humidity chamber (smaller chamber should be
		used for this test) without wind flow for a period of four hours at least. (While the chamber
 		temperature has reached around 35C and thermal data are stable, stop the chamber to make a
		no-wind test environment. The steady state of chamber temperature will be around 35C but
		may not be 35C exactly)

    - For housing with large volume such as H61 or H70:

- EUT is tested in NORMAL (e.g. 25ºC, 	30%~60% RH) temperature/humidity environment 			without obvious wind flow for a period of four hours at least.

- Note: For larger volume housing thermal measurement, if the thermal result is fail under the
	normal temperature room environment, then the second round thermal measurement shall
 	be performed in 35ºC no-wind chamber. The second set of thermal data is used for pass/fail
 	judgment and the first set of thermal data is for reference only.

  - All thermal data measured must be converted to 35C equivalent by linear interpolation or  				extrapolation method

  - Graphic resolution: 1024*768*(8, 15 or 16 bpp) , 75Hz ergonomic timing

- Test Criteria:

  - Test Margin for each hot spot (exclude key components): 3ºC

  - Upper limit of measured case temperature

    - Key component:

| - No. | - Component | - Point | - Tc(max) C |
| --- | --- | --- | --- |
| - 1 | - HDD | - Top Surface of Actuator Position | - 60 (refer to databook) |
| - 2 | - CD-ROM | - Central surface on bottom surface | - 55 or 60 (refer to databook) |
| - 3 | - DVD-ROM | - Central surface on bottom surface | - 55 (refer to databook) |
| - 4 | - FDD | - Central surface on bottom surface | - 57 (refer to databook) |
| - 5 | - Other |  |  |

- Table 9-1

    - Component on main board

      - Major test points

| - No. | - Chip Name | - Point | - Tc(max) C |
| --- | --- | --- | --- |
| - 1 | - CPU | - 1. Use MAX1617 evaluation kit to measure the junction temperature of Intel CPU - 2.If the above test method is not applicable, attach thermocouple to Center of Surface of CPU and measure the case temperature | - Refer to databook |
| - 2 | - Power Regulator | - Center of Surface | - 105 or refer to databook (but 105C is the absolute maximum temperature on main board) |
| - 3 | - VGA Chip | - Center of Surface | - Refer to databook |
| - 4 | - Audio Chip | - Center of Surface | - Refer to databook |
| - 5 | - DRAM/SRAM | - Center of Surface | - Refer to databook |
| - 6 | - Chip Sets | - Center of Surface | - Refer to databook |
| - 7 | - Modem Chip | - Center of Surface | - Refer to databook |
| - 8 | - Lan Chip | - Center of Surface | - Refer to databook |
| - 9 | - Hot Components around voltage regulator module (VRM) | - Center of Surface | - 85C for aluminum electrolytic capacitor or refer to databook |
| - 10 | - DC-DC Converter | - Center of Surface | - Refer to databook |
| - 11 | - Chip whose temperature is relative high according to the survey of IR camera but the power consumption is less than 1 watt. | - Center of Surface | - 85ºC |
| - 12 | - Other chips whose temperature is relative high according to the survey of IR camera and the power consumption is more than 1 watt. | - Center of Surface | - Refer to databook |

Table 9-2

      - Step by step directions for each chip's Pass or Fail judgement:

        - If Tc(max) is available on component data sheet then the converted Tc should not be over 						Tc(max) and the pass/fail judgement is finished

        - If the steady DC power consumption of the chip is less than 1 watt and If Tc(max) is not 						available and Ta(max) is available on component data sheet then if the converted Tc is less 					than Ta(max)+15C, then the chip pass the test and the pass/fail judgement is finished.

        - If the relative thermal resistance is available from data sheet then use the following formula to 					check the result:

          - Parameter definition:

- Tj(max): maximum junction temperature which is provided by IC vendor (Typically, Tj(max)=125C)

- JC: Junction to case thermal resistance (C/Watt)

- JA: Junction to ambient thermal resistance (C/Watt)

- Pd: Device power dissipation (Watt). If the device power dissipation is not available, then the operating DC power consumption shall be used.

- Tc: measured and converted case temperature (35ºC equivalent)

- TA: Device operating ambient temperature

          - If JC , Tj(max) and Pd is available, then the following calculated Tj can 	not be over Tj 						(max) and the pass/fail judgement is finished.

- Tj = Tc  + Pd * JC

          - If JA, Tj (max) and Pd is available, then the calculated Tj can not be over Tj (max)

- Tj = TA  + Pd * JA  (Tc - 15) + Pd * JA

- Diagnostic Program:

  - Scorpion II (VCD [for CDROM] or DVD [for DVDROM] ACTION movie playback is the main test 			program to exercise the system; note: floppy disk drive test is not performed)

  - For Intel CPU: Maxpower test (2 threads; open Maxpower windows)
		For Non-Intel CPU: MadOnion.com/3DMark2000 Bench Mark test (Microsoft DirectX 7 should be 			installed first)

- Test Result:

  - SKU-1:

    - Test condition record

| - CPU speed test (GHz) | - SPS version |
| --- | --- |
| - 2.0 |  |

Table 9-3

      - Components on main board or other subsystem board using Tc(max) test criteria

| - COMPONENTS ON MAIN BOARD OR SUBSYSTEM BOARDS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Component location number - (Location number of board name) | - Component Name - (Vendor name; - model number) | - Tc measured - (C) | - 35C conversion - temperature - (C) | - Tc(max) - (C) | - Result |
| - CPU | - P4 2.0 GHz |  | - 72.3 | - 70 | - PASS |
| - U29 | - R01-BO | - 34.1 | - 44.1 |  | - PASS |
| - U20 | - ADI 1885 | - 44.8 | - 54.8 |  | - PASS |
|  | - SMSC LPC47M192-NC | - 41.2 | - 41.2 |  | - PASS |
| - U21 | - INTEL SECRET | - 39.4 | - 49.4 |  | - PASS |
| - U17 | - LD1085 | - 59.5 | - 69.5 |  | - PASS |
| - Q17 | - D0115 | - 43.4 | - 53.4 |  | - PASS |
| - U11 | - W83L5180 | - 44.6 | - 54.6 |  | - PASS |
| - U13 | - HIP66 | - 57.8 | - 67.8 |  | - PASS |
| - U10 | - ST75185C | - 51.4 | - 61.4 |  | - PASS |
| - U7 | - ST75185C | - 54.2 | - 64.2 |  | - PASS |
| - Q5 | - D0015 | - 53.3 | - 63.3 |  | - PASS |
| - U8 | - CYS8323PVC | - 56.0 | - 66.0 |  | - PASS |
| - Q4 | - T0038 | - 50.2 | - 60.2 |  | - PASS |
| - Q1 | - D0015 | - 49.5 | - 59.5 |  | - PASS |
| - Q12 | - T0038 | - 53.3 | - 63.3 |  | - PASS |
| - Q15 | - T0038 | - 45.5 | - 55.5 |  | - PASS |
| - U9 | - HIP66 | - 37.7 | - 47.7 |  | - PASS |
| - Q9 | - 9939 | - 43.4 | - 53.4 |  | - PASS |
|  | - Memory | - 52.1 | - 62.1 |  | - PASS |
| - TC10 |  | - 61.7 | - 71.7 |  | - PASS |
| - L13 |  | - 59.2 | - 69.2 |  | - PASS |
| - L2 |  | - 57.4 | - 67.4 |  | - PASS |
| - U12 | - SAMSUNG 052 | - 47.6 | - 57.6 |  | - PASS |
| - U13 | - SAMSUNG 052 | - 44.7 | - 54.7 |  | - PASS |

- Note:  This table shows only the components whose temperature rises above 50C.

Table 9-4

      - Components on main board or other subsystem board using Tj (max) test criteria

| - COMPONENTS ON MAIN BOARD OR SUBSYSTEM BOARDS |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| - Component location number - (Location number of board name) | - Component Name - (Vendor name; - model number) | - Tc measured - (C) | - Thermal resistance - JC or JA - (C/W) | - Pd - (W) | - Calculated Tj at 35C - Conversion value | - Tj(max) - (C) | - Result |
| - U15 | - VGA Card | - 49.4 |  |  | - 59.4 |  | - PASS |

Table 9-5

- Note:

        - This table shows only the components whose temperature rises above 50C.

        - Tj = Tc  + Pd * JC  or  Tj = TA  + Pd * JA  (Tc - 15) + Pd * JA

      - Key component

| - KEY COMPONENTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| - Component name | - Test Location | - Tc measured ( C) | - Tc 35C conversion - temperature - (C) | - Tc(max) - (C) | - Result |
| - HDD | - Top Center | - 43.7 | - 53.7 | - 60 | - PASS |
| - CDROM | - Top Center | - 44.8 | - 54.8 | - 55 | - PASS |

Table 9-6

# 10. Operating Temperature and Humidity Test

- Basic Info,:
	Test Date				: 	7/27/2001
	Test Location			: 	Temperature & Humidity Test Room of Wistron Product 									Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	Temperature/Humidity chamber

- Field of Application:

  - New PC system

  - Housing model change

  - MB engineering change

  - Fan engineering change

  - SPS model change

  - CPU/Chipsets/VGA model change

  - Memory module model change

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - Add-on card engineering change

  - Heat plate (sink) thickness, shape or model change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications

  - Temperature			: +10C ~ +35ºC

  - Humidity					: 20% ~ 80% R.H. (non-condensation)

  - Maximum Wet Bulb Temperature		: 30ºC

- Notes:

    - Temperature and humidity gradient should be less than 20C/hour and 20% RH/hour, 					respectively, to prevent condensation.

    - Provided that the maximum wet-bulb temperature is 30C at high temperature and high 					humidity condition.

  - Test Margin: During the test, temperature margin (5C) must be added if number for each test 	configuration is less than three (the corresponding wet-bulb temperature is also increased)

  - Test Profile:

| - Section | - Conditions(from) - Chamber setup value | - Conditions(to) - Chamber setup value | - Operation | - Duration - (hours) | - Test Point |
| --- | --- | --- | --- | --- | --- |
| - 1 | - 25ºC/50% RH | - 5ºC/ - 80%RH (high humidity) | - None - Power Off | - 2 | - --- |
| - 2 | - 5ºC/ - 80%RH (high humidity) | - 5ºC/ - 80%RH (high humidity) | - None - Power Off | - 4 | - 1 |
| - 3 | - 5ºC/ - 80%RH (high humidity) | - 5ºC/ - 80%RH (high humidity) | - Power on and Function Test | - 8 | - 1 |
| - 4 | - 5ºC/ - 80%RH (high humidity) | - 40ºC/20%RH | - Function test | - 3 | - --- |
| - 5 | - 40ºC/20%RH | - 40ºC/20%RH | - Function test | - 8 | - 2 |
| - 6 | - 40ºC/20%RH | - 5ºC/ - 20%RH (low humidity) | - Function test | - 2 | - --- |
| - 7 | - 5ºC/ - 20%RH (low humidity) | - 5ºC/ - 20%RH (low humidity) | - Function test | - 8 | - 3 |
| - 8 | - 5ºC/ - 20%RH (low humidity) | - 40ºC/69%RH | - Function test | - 2 | - --- |
| - 9 | - 40ºC/69%RH | - 40ºC/69%RH | - Function test | - 12 | - 4 |
| - 10 | - 40ºC/69%RH | - 33ºC/80%RH | - Function test | - 1 | - --- |
| - 11 | - 33ºC/80%RH | - 33ºC/80%RH | - Function test | - 12 | - 5 |
| - 12 | - 33ºC/80%RH | - 25ºC/50% RH | - Function test | - 2 | - --- |

- Table 10-1 Description of temperature/humidity profile

- Figure 10-1. Temperature/Humidity Test Point Location

- Figure 10-2 Test Profile for Operating Temperature/Humidity Test with Test Margin

- Test Criteria:

  - Performance criteria A must be met during and after the test

- Diagnostic Program:
	Scorpion I

- Test Result:

  - SKU-1:

| - Test Item | - Function Test | - Physical Check |
| --- | --- | --- |
| - Operating | - PASS | - PASS |

Table 10-2

  - SKU-2:

| - Test Item | - Function Test | - Physical Check |
| --- | --- | --- |
| - Operating | - PASS | - PASS |

Table 10-3

# 11. Non-operating Temperature and Humidity Test

- Basic Info,:
	Test Date				: 	7/4/2001
	Test Location			: Temperature & Humidity Test Room of Wistron Product Reliability 								Lab 
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	Temperature/Humidity chamber

- Field of Application

  - New PC system

  - SPS model change (optional)

  - Packing change (for storage package test only)

  - Add on card model change (optional)

  - CDROM/DVD-ROM model change (optional)

  - FDD/LS-120/ZIP model change (optional)

  - HDD model change (optional)

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications

  - Un-packed

    - Temperature			: -20C ~ 60ºC

    - Humidity			: 10% ~ 90% R.H.

    - Maximum Wet-bulb Temperature		: 40ºC

  - Storage Package

    - Temperature			: -20C ~ 60ºC

    - High temperature & high humidity		: 10% ~ 95% R.H.

    - Condensation			: Possibly on package

- Notes:

    - Temperature gradient (ramp rate) should be less than 20C /hour and 20%RH/hour to prevent 				condensation.

    - The humidity in un-packed case is non-condensing

    - The storage packaged temperature and humidity test is condensing possibly.

- Test Criteria:

  - Un-packed

    - Performance criteria A must be met after the test

    - No assembly problem

    - Appearance and mechanical function

      - No dent.

      - No crack

      - No mold speckle

      - No unclear letter, broken line

      - No easily removed button

      - Input and output terminals' connection must be easily matched

      - No units wobbles

      - No migration materials in the set's plinths (e.g. foot of EUT dropped off)

      - No letters or symbols on cabinet are missing

      - No transfigure, deformity

      - No stiffness or squeak for buttons when pressed

      - No warp or rust for metal parts

      - No slack (e.g. screw loose)

  - Storage Package

    - Performance criteria A must be met after the test

    - No assembly problem

    - Appearance and mechanical function

      - No scratch

      - No dent.

      - No crack

      - No mold speckle

      - No unclear letter, broken line

      - No easily removed button

      - No obvious fingerprint to degrade the quality image of the product.

      - Input and output terminals' connection must be easily matched

      - No units wobbles

      - No migration materials in the set's plinths (e.g. foot of EUT dropped off)

      - No letters or symbols on cabinet are missing

      - No transfigure, deformity

      - No stiffness or squeak for buttons when pressed

      - No warp or rust for metal parts

      - No slack (e.g. screw loose)

      - No cracks or severe damage to cushion

      - No severe damage or crack to carton box

- Diagnostic Program:
	QAFE quick check

- Test Result:

  - SKU-1:

| - Test Item | - QAFE - Quick Check | - Appearance/mechanical check | - Assembly Check | - Package Check |
| --- | --- | --- | --- | --- |
| - Unpacked | - PASS | - PASS | - PASS | - Not Applicable |
| - Storage Package | - PASS | - PASS | - PASS | - PASS |

Table 11-1

  - SKU-2:

| - Test Item | - QAFE - Quick Check | - Appearance/mechanical check | - Assembly Check | - Package Check |
| --- | --- | --- | --- | --- |
| - Unpacked | - PASS | - PASS | - PASS | - Not Applicable |
| - Storage Package | - PASS | - PASS | - PASS | - PASS |

Table 11-2

# 12. Low Temperature & High Temperature Cold Start Test

- Basic Info,:
	Test Date				: 	7/24/2001
	Test Location			: 	Temperature & Humidity Test Room of Wintron Product 									Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	Temperature/Humidity chamber

- Field of Application:

  - New PC system

  - SPS model change

  - CPU/Chipsets/VGA model change

  - Core power circuit engineering change

  - Memory module model change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications

  - Low Temperature	: +5C

  - High Temperature	: +40ºC

  - Humidity			: 35~50%R.H.

  - Cold start times	: 5 times for each configuration & for each test temperature

  - Test Profile:

    - Low temperature

| - Section | - Conditions(from) | - Conditions(to) | - Operation | - Duration - (hours) |
| --- | --- | --- | --- | --- |
| - 1 | - 25ºC/50% RH | - 5ºC | - Power Off | - 1 |
| - 2 | - 5ºC | - 5ºC | - Power Off | - > 2 |
| - 3 | - 5ºC | - 5ºC | - Cold Start 5 times | - 10 minutes |

- Table 12-1

    - High temperature

| - Section | - Conditions(from) | - Conditions(to) | - Operation | - Duration - (hours) |
| --- | --- | --- | --- | --- |
| - 1 | - 25ºC/50% RH | - 40ºC/35%RH | - Power Off | - 1 |
| - 2 | - 40ºC/35%RH | - 40ºC/35%RH | - Power Off | - > 2 |
| - 3 | - 40ºC/35%RH | - 40ºC/35%RH | - Cold Start 5 times | - 10 minutes |

- Table 12-2

- Test Criteria:

  - The system must function normally from POST to entering Windows OS environment at the first 5 			times cold boot for both low temperature & high temperature.

  - Each CPU on AVL-C shall be tested

  - Each memory module on AVL-C shall be tested for lightest load (e.g. 64MB on DIMM 1) and 				heaviest load condition (e.g. 256MB on DIMM1, DIMM 2 & DIMM3).

- Diagnostic Program:
	None

- Test Result:

  - SKU-1:

| - Cold Start Test - at low temperature | - Cold Start Test - at high temperature |
| --- | --- |
| - PASS | - PASS |

Table 12-3

  - SKU-2:

| - Cold Start Test - at low temperature | - Cold Start Test - at high temperature |
| --- | --- |
| - PASS | - PASS |

Table 12-4

# 13. Operating Vibration Test

- Basic Info,:
	Test Date				: 	8/7/2001
	Test Location			: 	Vibration, Drop & Pneumatic Shock Test Room of Wistron 									Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Vibration system (KingDesign KD-9363-EM-2000F-										50N200)

- 2.	Control system: Dactron LASER Vibration Control System

- Field of Application

  - New PC system

  - Housing model change

  - Housing engineering change

  - CDROM/DVD-ROM model change

  - HDD model change

  - FDD/LS120/ZIP model change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications:

  - Swept Sine Mode:
	The specifications below are applicable to most products. Some products, due to their peculiar design, 	may require different test specification and the corresponding specification is to be discussed.

    - When component level specification of the CDROM under test is NO less than 0.25G, then use 				the following specification:

| - Frequency Hertz | - Acceleration - (0-peak) | - Sweep Rate | - Endurance - Cycle | - Test Axis | - Test Time |
| --- | --- | --- | --- | --- | --- |
| - 5 ~ 250 | - 0.25 G | - 0.5 Octave/Minute | - 1 Cycles - ( 2 sweeps ) | - Z only - (But for all possible operating orientation) | - 23 minutes |

- Table 13-1

    - When component level specification of the CDROM under test is less than 0.25G, then use the 				following specification first:

| - Frequency Hertz | - Acceleration - (0-peak) | - Sweep Rate | - Endurance - Cycle | - Test Axis | - Test Time |
| --- | --- | --- | --- | --- | --- |
| - 5 ~ 250 | - 0.25 G | - 0.5 Octave/Minute | - 1 Cycles - ( 2 sweeps ) | - Z only - (But for all possible operating orientation) | - 23 minutes |

- Table 13-2

- If the performance of CDROM can not meet test criteria, then use the following specification:

| - Frequency Hertz | - Acceleration - (0-peak) | - Sweep Rate | - Endurance - Cycle | - Test Axis | - Test Time |
| --- | --- | --- | --- | --- | --- |
| - 5 ~ 250 | - 0.20 G | - 0.5 Octave/Minute | - 1 Cycles - ( 2 sweeps ) | - Z only - (But for all possible operating orientation) | - 23 minutes |

- Table 13-3

  - Random Mode:

    - PSD (power spectral density) break point, the unit is G2/Hz:

| - 5Hz | - 17Hz | - 500Hz |
| --- | --- | --- |
| - 8.010-7 | - 8.810-5 | - 8.810-5 |

- Table 13-4

    - Grms: 0.207

    - Test axes: X, Y, Z

    - Test duration: 20 minutes per axis

- Test Criteria:
	During and after the test:

  - Function:
		Performance criteria A, B or Minor C shall be met

  - Appearance & mechanical function

    - The EUT should not suffer permanent deformation or fracture.

    - No fixed part or assembly shall be loosening.

    - No moving or movable part of an assembly should become free or sluggish in the testing 					process.

    - No movable part should shift in setting, position or adjustment.

  - When problem occurred during the test:

- If the test fails while the Diagnostic Program is Scorpion II, then use HDD.BAT or CDROM/DVDROM 			playback Individually to double check the functionality of the failed component. If the system passed 			while the individual test program is used, then it is said to pass the test.

- Diagnostic Program:
	Scorpion IV (HDD.BAT & CDROM/DVDROM movie playback); no FDD operation is involved.

- Test Result:

  - SKU-1:

    - Result details:

      - Sine Mode:

| - Test Item | - Function Test Result | - Appearance & mechanical function Check Result | - Remarks |
| --- | --- | --- | --- |
| - Z orthogonal axis | - PASS | - PASS |  |

- Table 13-5

      - Random Mode:

| - Test Item | - Function Test Result | - Appearance & mechanical function Check Result | - Remarks |
| --- | --- | --- | --- |
| - X orthogonal axis | - PASS | - PASS |  |
| - Y orthogonal axis | - PASS | - PASS |  |
| - Z orthogonal axis | - PASS | - PASS |  |

- Table 13-6

  - SKU-2:

    - Result details:

      - Sine Mode:

| - Test Item | - Function Test Result | - Appearance & mechanical function Check Result | - Remarks |
| --- | --- | --- | --- |
| - Z orthogonal axis | - PASS | - PASS |  |

- Table 13-7

      - Random Mode:

| - Test Item | - Function Test Result | - Appearance & mechanical function Check Result | - Remarks |
| --- | --- | --- | --- |
| - X orthogonal axis | - PASS | - PASS |  |
| - Y orthogonal axis | - PASS | - PASS |  |
| - Z orthogonal axis | - PASS | - PASS |  |

- Table 13-8

# 14. Non-operating Random Vibration Test (Unpacked)

- Basic Info,:
	Test Date				: 	8/7/2001
	Test Location			: 	Vibration, Drop & Pneumatic Shock Test Room of Wistron 									Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Vibration system (KingDesign KD-9363-EM-2000F-										50N200)

- 2.	Control system: Dactron LASER Vibration Control System

- Field of Application

  - New PC system

  - Housing change

  - Housing engineering change

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications:

  - PSD (power spectral density) break point, the unit is G2/Hz:

| - Frequency (Hz) | - PSD level (G2/Hz) |
| --- | --- |
| - 2.0 | - 0.0005 |
| - 4.0 | - 0.0150 |
| - 8.0 | - 0.0150 |
| - 40.0 | - 0.0015 |
| - 55.0 | - 0.0050 |
| - 70.0 | - 0.0050 |
| - 200.0 | - 0.0005 |

- Table 14-1

  - Grms: 0.7354

  - Test axes and duration: 30 minutes for each axis of the three orthogonal axes.

- Test Criteria:
	After the test:

  - Function:
		Performance criteria A shall be met

  - Appearance & mechanical function

    - The EUT should not suffer permanent deformation or fracture.

    - No fixed part or assembly shall be loosening.

    - No moving or movable part of an assembly should become free or sluggish in the testing 					process.

    - No movable part should shift in setting, position or adjustment.

- Diagnostic Program
	QAFE quick check

- Test Result:

  - SKU-1:

| - Test Item | - Function Test - QAFE Quick Check | - Physical Check |
| --- | --- | --- |
| - X orthogonal axis | - PASS | - PASS |
| - Y orthogonal axis | - PASS | - PASS |
| - Z orthogonal axis | - PASS | - PASS |

- Table 14-2

  - SKU-2:

| - Test Item | - Function Test - QAFE Quick Check | - Physical Check |
| --- | --- | --- |
| - X orthogonal axis | - PASS | - PASS |
| - Y orthogonal axis | - PASS | - PASS |
| - Z orthogonal axis | - PASS | - PASS |

- Table 14-3

# 15. Transportation Vibration Test

- Basic Info,:
	Test Date				: 	8/7/2001
	Test Location			: 	Vibration, Drop & Pneumatic Shock Test Room of Wistron 									Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	Vibration system (KingDesign KD-9363-EM-2000F-										50N200)

- 2.	Control system: Dactron LASER Vibration Control System

- Field of Application

  - New PC system

  - Housing change

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - Package model change

  - Package engineering change

  - Cushion model change

  - Cushion engineering change

- Test Configurations:

  - SKU-1

  - SKU-2

- Test Specifications:

  - PSD (power spectral density) break point, the unit is G2/Hz:

| - 1Hz | - 4Hz | - 100Hz | - 200Hz |
| --- | --- | --- | --- |
| - 0.0001 | - 0.01 | - 0.01 | - 0.001 |

- Table 15-1

  - Grms: 1.146

  - Packaged, Non-operating

  - Test face/axes and test duration:

| - Test Face or Axis | - Bottom down (face) | - Top down (face) | - Left/Right (axis) | - Front/Rear (axis) |
| --- | --- | --- | --- | --- |
| - Test Duration - (minutes) | - 30 | - 10 | - 10 | - 10 |

- Table 15-2

- Test Criteria:
	After the test:

  - Function:
		Performance criteria A shall be met

  - Appearance & mechanical function

    - The EUT should not suffer permanent deformation or fracture.

    - No fixed part or assembly shall be loosening.

    - No moving or movable part of an assembly should become free or sluggish in the testing 					process.

    - No movable part should shift in setting, position or adjustment.

- Diagnostic Program

  - QAFE quick check

  - Drive Probe

- Test Result:

  - SKU-1:

| - Test Item | - Function Test | - Physical Check |
| --- | --- | --- |
| - X orthogonal axis | - PASS | - PASS |
| - Y orthogonal axis | - PASS | - PASS |
| - Z orthogonal axis | - PASS | - PASS |

- Table 15-3

  - SKU-2:

| - Test Item | - Function Test | - Physical Check |
| --- | --- | --- |
| - X orthogonal axis | - PASS | - PASS |
| - Y orthogonal axis | - PASS | - PASS |
| - Z orthogonal axis | - PASS | - PASS |

- Table 15-4

# 16. Acoustic Noise Measurement

## 16-1. Sound Pressure Level

- Basic Info,:
	Test Date				: 	8/7/2001
	Test Location			: 	Semi-anechoic Room of Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	B&K 2143 Real Time Frequency Analyzer
								2.	B&K 4190 Low Noise Free-Field 1/2 inch Microphone
								3.	B&K 2669B Microphone Pre-Amplifier
								4.	B&K 4231 Sound Level Calibrator
								5.	B&K 4179 1 inch Condenser Microphone

- Field of Application

  - New PC system

  - SPS fan model change

  - System fan model change

  - HDD model change

  - Housing model change

  - Audio card model change (for speaker noise measurement)

- Test Configurations:

  - SKU-1

  - SKU-2

- Specifications:

  - Details:

|  | - Noise Level (dBA) |  |  |  |
| --- | --- | --- | --- | --- |
| - Location | - IDLE | - FDD | - HDD | - CD-ROM or DVD-ROM |
| - Spec. | -  35 dB | -  46 dB | -  39 dB | -  47 dB |

- Table 16-1 Acoustic Noise Specifications (Sound Pressure Level)

- Note: If the system is not audio on-board or without audio add-on card when shipment, the speaker 	noise measurement shall be omitted.

  - System warm up period in idle mode before measurement: at least 1 hour

  - Number of test data points for each single test item: 2

  - EUT Exercise Mode:

    - Idle Mode I:

      - Without multimedia loudspeaker:

        - All possible power management is disabled (BIOS and Windows OS setup shall be checked).

        - Spindle motor of hard disk drive is rotating.

        - All fans on with their normal rotating speed at normal operating temperature (15~30ºC)

        - In Windows OS environment without running any applications and without user operation.

      - With multimedia loudspeaker:

        - For multimedia speaker noise test, use USB-powered active-type speaker for 						measurement if the AVL-C does not include loudspeaker component.

        - Both left and right speaker shall be connected to the system.

        - If there is volume control knob of the speaker, adjust the position of the knob to the "3 					o'clock" position.

        - If there is treble or bass tone control, adjust the tone control knob to its normal 						position.

        - If there is 3D sound effect control, disable it.

        - Maximize the playback WAVE volume control of Windows OS.

    - Idle Mode II:

      - Without multimedia loudspeaker connected to the system

      - All possible power management is disabled (BIOS and Windows OS setup shall 	be 						checked) except hard disk drive.

      - Spindle motor of hard disk drive is NOT rotating (HDD is in standby state).

      - All fans on with their normal rotating speed whether there is fan speed control or not.

      - In Windows OS environment without running any applications and without user 							operation.

    - Storage Device operating Mode:
			No need to test.

  - Measurement Position:

    - Operator Position (refer to Figure 16-1):
			The microphone shall be located 0.45m ±0.03m above the test table (0.75m high), and 0.50m0.03m 			horizontal from the center of the front face of the EUT.
			Note:

- In ISO7779 and ECMA 74, as for operator position, there is a statement as follows:
			For desktop equipment which normally has a detachable keyboard and which is tested without the 				keyboard, the distance from the front end of the reference box, for purposes of determining the 				operator position, shall be 0.50m  0.03m in front of such equipment.

    - Bystander Position (refer to Figure 16-2):
		The microphone shall be located 0.75m ±0.03m above the test table (0.75m high), and 1.0m±0.03m 			horizontal from the center of the front, rear, right, and left side of EUT respectively.

  - Measurement Unit Description:

    - 0 dB  20 Pa (1 Pa  1 N/m2) in this sound pressure level measurement

    - dBA: the dB value with A-weighting filter

  - The EUT must be in accordance with the configuration for shipment.

  - Measurement duration: 8 seconds

- Figure 16-1 Operator Position

- Figure 16-2 Bystander Position

- Test Criteria:

  - If equivalent number of test machine for each sub test item is less than 3 then the acoustic noise level 			should be below the specifications with 3dB test margin.

  - If equivalent number of test machine for each sub test item is more than 3 (included) then the 				arithmetic mean of all test data measured should be below the specifications without test margin.

- Diagnostic Program
	No specific test program; just in Windows OS environment.

- Test Result:

  - Background Noise Level:   19.5 dB

  - Details:

    - SKU-1:

      - Operator Position

|  |  | Noise Level ( dB ) LpA |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | - IDLE |  | - FDD | - HDD | - CD-ROM or DVD-ROM |
| Measure | 34.3 |  | 43.6 | 36.1 | 46.4 |
| Specification | -  35 dB |  | -  46 dB | -  39 dB | -  47 dB |
| Result | Pass |  | Pass | Pass | Pass |

- Table 16-2

- Note:

- Lpi = Noise measured at direction I

- n = Number of directions (Typically, n=4)

    - SKU-2:

      - Operator Position

|  |  | Noise Level ( dB ) LpA |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Location | - IDLE |  | - FDD | - HDD | - CD-ROM or DVD-ROM |
| Measure | 34.6 |  | 43.6 | 36.1 | 46.4 |
| Specification | -  35 dB |  | -  46 dB | -  39 dB | -  47 dB |
| Result | Pass |  | Pass | Pass | Pass |

## 16-2. Sound Power Level

- Basic Info,:
	Test Date				: 	8/7/2001
	Test Location			: 	Semi-anechoic Room of Wistron Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	1.	B&K 2143 Real Time Frequency Analyzer
								2.	B&K 4190 Low Noise Free-Field 1/2 inch Microphone
								3.	B&K 2669B Microphone Pre-Amplifier
								4.	B&K 4231 Sound Level Calibrator
								5.	B&K 4179 1 inch Condenser Microphone
								6.	A-BEX Unbalance TCD (0.25 g-cm)

- Field of Application

  - New PC system

  - SPS fan model change

  - System fan model change

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - Housing model change

- Test Configurations:

  - SKU-1

  - SKU-2

  - SKU-3

- Test Specifications

  - Upper limit of the declared A-weighted sound power level according to ISO-9296 or ECMA-109: 			refer to section 16-3 for more detail.

| - Mode | - LWAd |
| --- | --- |
| - Idle Mode I | - 4.6 B |
| - Idle Mode II | - 4.3 B |
| - HDD - Random Read | - 5.4 B |
| - CD-ROM/DVD-ROM - Sequential Read | - 5.4 B |
| - FDD - Sequential Seek/Verify | - 5.0 B |
| - LS120/ZIP100/ZIP250 - Random Verify | - 5.4 B |

- Table 16-18

  - Operating mode of storage device:

    - Operation with FDD: Performing Sequential Seek/Verify (average time: 64 sec; QAPlusFE) 					operation and other storage equipment is in idle mode.

    - Operation with HDD: Performing Random Read (average time: 32 sec; HDD13.EXE) operation 				and other storage equipment is in idle mode.

    - Operation with CD-ROM/DVDROM: Sequential Read at the inner radius of the disk such that 				the disk spins continuously at its fastest normal rotational speed. Repeat the above read process 				until measurement duration specified elapses (average time: 32 sec; QAPlusFE) and the other 				storage equipment is in idle mode. Note: unbalance CD of 0.25g-cm  10% should be used for 				this testing. (Refer to ECMA 74 6th edition - December 1999)

  - Idle Mode I & II: Refer to section 16-1

- Test Criteria:
	Declared sound power level is no greater than the values shown in the test specification.(refer to Section 	17-6 for acoustic noise declared values)

- Diagnostic Program

  - QAPlusFE 5.4

  - Acer PQA test program for HDD random read (HDD13.exe).

- Test Result:

  - Sound power calculation:

    - SKU-1:

| - Test Mode | - Measure (B) | - Specification | - Result |
| --- | --- | --- | --- |
| - Idle in WinME | 4.52 | 4.6 | - Pass |
| - HDD Random Access | 4.60 | 5.4 | - Pass |
| - CD-ROM/DVD-ROM Sequential Read | 5.10 | 5.4 | - Pass |
| - FDD Sequential Seek/Verify | 4.24 | 5.0 | - Pass |

    - SKU-2:

| - Test Mode | - Measure (B) | - Specification | - Result |
| --- | --- | --- | --- |
| - Idle in WinME | 4.52 | 4.6 | - Pass |
| - HDD Random Access | 4.60 | 5.4 | - Pass |
| - CD-ROM/DVD-ROM Sequential Read | 5.10 | 5.4 | - Pass |
| - FDD Sequential Seek/Verify | 4.24 | 5.0 | - Pass |

    - SKU-3:

| - Test Mode | - Measure (B) | - Specification | - Result |
| --- | --- | --- | --- |
| - Idle in WinME | 4.52 | 4.6 | - Pass |
| - HDD Random Access | 4.85 | 5.4 | - Pass |
| - CD-ROM/DVD-ROM Sequential Read | 5.10 | 5.4 | - Pass |
| - CD-ROM/DVD-ROM Sequential Read | 5.08 | 5.4 | - Pass |
| - FDD Sequential Seek/Verify | 4.24 | 5.0 | - Pass |

# 17. Transportation Drop Test

- Basic Info,:
	Test Date				: 	8/6/2001
	Test Location			: 	Vibration, Drop & Pneumatic Shock Test Room of Wistron 									Product Reliability Lab
	Lab. Ambient			: 	243C, 425 %RH 
	Test Equipment		: 	Drop Tester: King design KD-128; 40-150cm adjustable

- Field of Application

  - New PC system

  - CDROM/DVD-ROM model change

  - FDD/LS-120/ZIP model change

  - HDD model change

  - Package engineering change

  - Cushion engineering change

  - Fan-Sink model change

  - Fan-Sink engineering change

- Test Configurations:

  - SKU-3

- Test Specifications

  - Gross Weight and Drop Height

| - Packaged Gross Weight |  | - Drop Height |  |  |  | - No. of Drops |
| --- | --- | --- | --- | --- | --- | --- |
| - KG | - lb. | - Product Specification |  | - With Test Margin |  |  |
| - 0.45 ~ 9.52 | - 1 ~ 20 | - 76cm | - 30 in | - 84cm | - 33 in | - 10 |
| - 9.53 ~ 18.59 | - 21 ~ 40 | - 61cm | - 24 in | - 69cm | - 27 in | - 10 |
| - 18.60 ~ 27.66 | - 41 ~ 60 | - 46cm | - 18 in | - 54cm | - 21 in | - 10 |
| - 27.67 ~ 45.36 | - 61 ~ 100 | - 31cm | - 12 in | - 39cm | - 15 in | - 10 |
| - 10 drops : 1 corner , 3 edges and 6 surfaces |  |  |  |  |  |  |

- Table 17-1

  - Test Margin: 3 inches (about 8 cm)

- Test Criteria:

  - Test height should add 8-cm test margin during verification test.

  - Function requirement: performance criteria A must be met after the test

  - Appearance & Mechanical Function after the test

    - No scratch

    - No dent.

    - No crack

    - No mold speckle

    - No unclear letter, broken line

    - No easily removed button

    - No obvious fingerprint to degrade the quality image of the product.

    - Input and output terminals' connection must be easily matched

    - No units wobbles

    - No migration materials in the set's plinths (e.g. foot of EUT dropped off)

    - No letters or symbols on cabinet are missing

    - No transfigure, deformity

    - No stiffness or squeak for buttons when pressed

    - No warp or rust for metal parts

    - No slack (e.g. screw loose)

    - No severe cracks to cushion (no complete broken)

    - No severe damage to carton box

- Figure 17-1 Surface definition of EUT

- Diagnostic Program

  - QAFE

  - DriveProbe (Floppy Disk Drive test)

- Test Result:

  - SKU-1:

| - Drop Height 69 cm | - Physical Check |
| --- | --- |
| - The Weakest corner of product (corner 5-3-2) | - PASS |
| - The shortest edge radiating from the corner 5-3-2 | - PASS |
| - The next shortest edge radiating from the corner 5-3-2 | - PASS |
| - The longest edge radiating from the corner 5-3-2 | - PASS |
| - Surface 5 | - PASS |
| - Surface 6 | - PASS |
| - Surface 2 | - PASS |
| - Surface 4 | - PASS |
| - Surface 3 | - PASS |
| - Surface 1 | - PASS |

- Table 17-2

| - Check items | - Result |
| --- | --- |
| - QAFE HDD 5-cycle check | - PASS |
| - QAFE CDROM 5-cycle check | - PASS |
| - QAFE FDD 5-cycle check | - PASS |
| - Package appearance | - PASS |
| - Cushion appearance | - PASS |

- Table 17-3

  - SKU-2:

| - Drop Height 69 cm | - Physical Check |
| --- | --- |
| - The Weakest corner of product (corner 5-3-2) | - PASS |
| - The shortest edge radiating from the corner 5-3-2 | - PASS |
| - The next shortest edge radiating from the corner 5-3-2 | - PASS |
| - The longest edge radiating from the corner 5-3-2 | - PASS |
| - Surface 5 | - PASS |
| - Surface 6 | - PASS |
| - Surface 2 | - PASS |
| - Surface 4 | - PASS |
| - Surface 3 | - PASS |
| - Surface 1 | - PASS |

- Table 17-4

| - Check items | - Result |
| --- | --- |
| - QAFE HDD 5-cycle check | - PASS |
| - QAFE CDROM 5-cycle check | - PASS |
| - QAFE FDD 5-cycle check | - PASS |
| - Package appearance | - PASS |
| - Cushion appearance | - PASS |

- Table 17-5