Test Certificate

We herewith confirm that: The test data, data evaluation, test procedures, and equipment configurations shown in this test report (Ref. No. NEI-EMC-02049-1) carried out by the Laboratory comply with the requirements relating to EMC Directive (89/336/EEC).

| Project No. ： | 02E2129 |
| --- | --- |
| Equipment： | LCD Monitor |
| Model No.： | AL502 |
| Applicant ： | Acer Incorporated |
|  | 21F, 88, Sec. 1, Hsin Tai Wu Rd., Hsichih, |
|  | Taipei Hsien 221, Taiwan, R.O.C. |

| For the evaluation regarding to the Directive, following EEC harmonized standard(s) were applied: |  |  |
| --- | --- | --- |
| Product Family Standard | : | EN55022:1998 |
|  |  | CISPR 22:1997 |
|  |  | EN55024:1998 |
|  |  | EN61000-3-2:1995+Amd.1:1998+Amd.2:1998 |
|  |  | EN61000-3-3:1995 |
| Basic Standard | : | EN61000-4-2:1995 |
|  |  | EN61000-4-3:1996 |
|  |  | EN61000-4-4:1995 |
|  |  | EN61000-4-5:1995 |
|  |  | EN61000-4-6:1995 |
|  |  | EN61000-4-8:1993 |
|  |  | EN61000-4-11:1994 |

(Authorized Signature)

General Manager

(Title / Position)

Mar. 26, 2002

(Date)

NEMKO EMC Laboratory Aut.No. ELA-136

NEUTRON ENGINEERING INC.

No. 132-1, Lane 329, Sec. 2, Palain Road,

Shijr Jen Taipei, Taiwan, R.O.C.

TEL：(02) 2646-5426   FAX：(02) 2646-6815

EMC Measurement Report

| Issued Date ： | Mar. 26, 2002 |
| --- | --- |
| Project No. ： | 02E2129 |
| Equipment： | LCD Monitor |
| Model No.： | AL502 |

| Applicant ： | Acer Incorporated |
| --- | --- |
|  | 21F, 88, Sec. 1, Hsin Tai Wu Rd., |
|  | Hsichih, Taipei Hsien 221, |
|  | Taiwan, R.O.C. |

Neutron represents to the client that testing is done in accordance with standard procedures as applicable and that test instruments used has been calibrated with the standards traceable to National Measurement Laboratory(NML)of R.O.C., or National Institute of Standards and Technology(NIST) of U.S.A. Neutron reports apply only to the specific samples tested under conditions. It is manufacture’s responsibility to ensure that additional production units of this model are manufactured with the identical electrical and mechanical components. Neutron shall have no liability for any declarations, inferences or generalizations drawn by the client or others from Neutron issued reports. Neutron’s EMC laboratory quality assurance procedures are in compliance with ISO 25/EN 45001 requirements, certified according to the Nemko's ELA Scheme . Neutron’s report must not be used by the client to claim product endorsement by Nemko or any agency of the Norway Government. This report is the confidential property of the client . As a mutual protection to the clients, the public and Neutron-self, extracts from the test report shall not be reproduced except in full without Neutron’s authorized written approval.

NEMKO EMC Laboratory Aut.No. ELA-136

NEUTRON ENGINEERING INC.

No. 132-1, Lane 329, Sec. 2, Palain Road,

Shijr Jen Taipei, Taiwan, R.O.C.

TEL：(02) 2646-5426  FAX：(02) 2646-6815

Table of Contents

Page

| I. | Part 1. General Information …...................................................................................….......... | 3 |
| --- | --- | --- |
|  | 1 Applicant…................................................................................…..…................….............. | 4 |
|  | 2 Product Name….......................................................................…...........................…........... | 4 |
|  | 3 Brand / Model No. …...................................................................….........................………. | 4 |
|  | 4 Standard Compliance….............................................................….................................…... | 4 |
|  | 5 Test Facility…...........................................................................…....................................…. | 4 |
|  | 6 Measuring Instruments …........................................................….....................................…. | 4 |
|  | 7. Measuring Uncertainty .........................................................…………………………....... | 5 |
|  | 8 Product Descriptions …........................................................................…........................…. | 6 |
|  | 9 Electric Block Diagram….........................................…………………...........................….. | 7 |
|  | 10 EUT Modifications….....................................................................................................….. | 7 |
|  | 11 Photos of EUT …...........................................................................................................…… | 7 |
|  | 12 Information to the User …………………………………………………………………….. | 7 |
|  | 13 Conformity of Declaration…..............................................................………………………. | 7 |

| II. | Part 2. RFI Emissions Measurement..................................................................................….. | 8 |
| --- | --- | --- |
|  | 1 Standard Compliance......................................................................................................…… | 9 |
|  | 2 Limits of Emissions ........................................................................................................….. | 9 |
|  | 3 Test Methodology...........................................................................................................…… | 9 |
|  | 4 Sample(s) Tested.............................................................................................................…… | 9 |
|  | 5 System Tested Configuration...........................................................................................….. | 9 |
|  | 6 Conducted and Radiated Measurement Photos...............................................................….. | 14 |
|  | 7 Emissions Measurement Data .........................................................................................….. | 16 |

| III. | Part 3. Electromagnetic Immunity Test...............................................................................… | 21 |
| --- | --- | --- |
|  | 1 Standard Compliance.............................................................…………………………..….. | 22 |
|  | 2 Test Methodology..........................................................................................................…… | 23 |
|  | 3 Sample(s) Tested ...........................................................................................................…… | 23 |
|  | 4 Test Set-Up Configuration ..........................................................................................….. | 23 |
|  | 5 EUT Operating Condition ..............................................................................................….. | 32 |
|  | 6 EUT Tested Results ......................................................................................................…… | 32 |

| IV. | Part 4. Harmonics/Voltage Fluctuation and Flicker ............................................................ | 42 |
| --- | --- | --- |
|  | 1 Standards Compliance.....................................................................................................….. | 43 |
|  | 2. Limits ……………………………………………………………………………………… | 43 |
|  | 3 Test Methodology...........................................................................................................…… | 44 |
|  | 4 Sample(s) Tested ............................................................................................................…… | 44 |
|  | 5 Test Set-Up Configuration...............................................................................................….. | 44 |
|  | 6 EUT Operating Condition ..............................................................................................…… | 44 |
|  | 7 EUT Tested Results ........................................................................................................… | 44 |

| V. | Attachment ……..........................................………………………………………………... | 49 |
| --- | --- | --- |
|  | A. Measurement Instruments List.................................................................…………………. | 50 |
|  | B. Electric Block Diagram.................................................................………………………… | 54 |
|  | C. EUT Modification Description.................................................................…………………. | 55 |
|  | D. Labeling /User’s Manual Requirement .....................................................………….......... | 56 |
|  | E. Example of Declaration of Conformity...............................................…..........…………… | 58 |
|  | F. Laboratory Accreditation Certificate …………………….………………………….……. | 60 |
|  | G. EUT Photos .................................................................……………………………………. | 61 |

Part - I.  General Information

Table Contents

1. Applicant

2. Product Name

3. Brand / Model No.

4. Standard Compliance

5. Test Facility

6. Measuring Instruments

7. Product Descriptions

8. Electric Circuit Block Diagram

9. EUT Modifications

10. Photos of EUT

11. Information to the User

12. Conformity Declaration

PART - 1.  GENERAL INFORMATION

| 1. | Applicant |  |  |
| --- | --- | --- | --- |
|  | Name: |  | Acer Incorporated |
|  | Address: |  | 21F, 88, Sec. 1, Hsin Tai Wu Rd., Hsichih, Taipei Hsien 221, Taiwan, R.O.C. |

| 2. | Product Name |  |
| --- | --- | --- |
|  | LCD Monitor |  |

| 3. | Brand/Model Covered (if applicable) |  |
| --- | --- | --- |
|  | Model(s) covered in this test report is(are) : |  |
|  | acer Brand : AL502 |  |

| 4. | Standard Compliance (check one, if applicable) |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | A. Emission Regulation |  |  |  |  |  |  |
|  |  |  | ( X ) | EN55022:1998/CISPR 22:1997 |  |  |  |  |
|  |  |  | ( ) | EN60555-2:1987/IEC 555-2+Amd.1:1985+Amd.2:1986 |  |  |  |  |
|  |  |  | ( ) | EN60555-3:1987/IEC 555-3+Amd.1:1990 |  |  |  |  |
|  |  |  | ( X ) | EN61000-3-2:1995+A1:1998+A2:1998/IEC61000-3-2 |  |  |  |  |
|  |  |  | ( X ) | EN61000-3-3:1995/IEC61000-3-3 |  |  |  |  |
|  |  | B. Immunity Regulation |  |  |  |  |  |  |
|  |  |  | ( X ) | EN55024:1998 |  |  |  |  |
|  |  |  | ( X ) | EN61000-4-2:1995/IEC61000-4-2:1995 | ( X ) |  | EN61000-4-5:1995/IEC61000-4-5:1995 |  |
|  |  |  | ( X ) | EN61000-4-3:1996/IEC61000-4-3:1995 | ( X ) |  | EN61000-4-6:1996/IEC61000-4-6:1996 |  |
|  |  |  | ( X ) | EN61000-4-4:1995/IEC61000-4-4:1995 | ( X ) |  | EN61000-4-8:1993/IEC61000-4-8:1993 |  |
|  |  |  | ( X ) | EN61000-4-11:1994/IEC61000-4-11:1994 |  |  |  |  |
|  |  | C. Deviations from Standard Test Method |  |  |  |  |  |  |
|  |  |  | N/A |  |  |  |  |  |

| 5. | Test Facility |  |
| --- | --- | --- |
|  | The test facility used to collect the test data in this report is NEUTRON ENGINEERING INC. Laboratory located at No. 132-1, Lane 329, Sec. 2, Palain Rd., Shijr Jen, Taipei, Taiwan, R.O.C. This test laboratory has been certified according to the ELA(EMC Laboratory Authorization) Scheme by Nemko. Valid measuring scopes under this authorization, refer to Attachment-F. enclosed. |  |

| 6. |  | Measuring Instruments |
| --- | --- | --- |
|  | Valid measuring instruments used in this report refer to Attachment-A. enclosed. All those instruments has been calibrated with the standards traceable to National Measurement Laboratory (NML) of R.O.C. or National Institute of Standards and Technology (NIST) of U.S.A. |  |

| 7. | Measurement Uncertainty |  |
| --- | --- | --- |
|  | 7-1. | Radiated Emission Measurement |

|  |  | Uncertainty (dB) |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Contribution | Probability Distribution | Biconical Ant. 30-200 MHz |  | Log-periodic Ant. 200MHz-1GHz |  | Horn Ant. 1-18 GHz |
|  |  | 3 m | 10 m | 3 m | 10 m | 3 m |
| Antenna Factor Calibration | normal(k=2) | ±1.25 | ±1.25 | ±1.25 | ±1.0 | ±1.0 |
| Cable Loss Calibration | normal(k=2) | ±0.5 | ±0.5 | ±0.5 | ±0.5 | ±2.0 |
| RCV/SPA Specification | rectangular | ±2.0 | ±2.0 | ±2.0 | ±2.0 | ±2.0 |
| Antenna Directivity | rectangular | ±0.5 | ±0.3 | ±1.5 | ±0.5 | ±0.1 |
| Antenna Factor V.S Height | rectangular | ±2.0 | ±2.0 | ±0.5 | ±0.5 | 0 |
| Ant. Factor Freq. Interpolation | rectangular | ±0.2 | ±0.2 | ±0.2 | ±0.2 | ±0.25 |
| Measurement Distance | rectangular | ±0.5 | ±0.3 | ±0.5 | ±0.3 | ±0.5 |
| Site Imperfections | rectangular | ±2.0 | ±2.0 | ±2.0 | ±2.0 | ±1.5 |
| Combined Standard Uncertainty Uc(y) | normal | ±2.19 | ±2.17 | ±2.05 | ±1.785 | ±1.85 |
| Measuring Uncertainty for a Level of Confidence of 95 % U=2xUc(y) | normal(k=2) | ±4.38 | ±4.34 | ±4.10 | ±3.57 | ±3.70 |

|  | 7-2. | Conducted Emission Measurement |
| --- | --- | --- |

| Contribution | Probability Distribution | Uncertainty (dB) 150KHz - 30MHz |
| --- | --- | --- |
| Cable & I/P Attenuator Calibration | normal(k=2) | ±0.5 |
| RCV/SPA Specification | rectangular | ±2.0 |
| LISN Coupling Specification | rectangular | ±2.0 |
| Transducer Factor Frequency Interpolation | rectangular | ±0.2 |
| Combined Standard Uncertainty Uc(y) | normal | ±1.66 |
| Measuring Uncertainty for a Level of Confidence of 95 % U =2xUc(y) | normal(k=2) | ±3.32 |

| 8. | Product Descriptions |  |  |
| --- | --- | --- | --- |
|  | 8-1. | Specification, features, and/or product application if appropriate |  |
|  |  | The Acer Incorporated. Model: AL502 (referred to as the EUT in this report) is a 15", VESA compatible , TFT color LCD display monitor with following features: |  |
|  |  | (1) LCD Module | 15.0 Inch |
|  |  | (2) Native Resolution | 1024x768 |
|  |  | (3) Input Signal | Video : ANALOG 0.7 Vp-p/75 Ohms |
|  |  |  | Sync : Separate sync. TTL Level |
|  |  |  | Horizontal sync. Positive/Negative |
|  |  |  | Vertical sync. Positive/Negative |
|  |  | (4) Display Colors Analog input : | 16,194,277 |
|  |  | (5) Synchronization | Horizontal : 31.5Khz to 60KHz |
|  |  |  | Vertical : 56.2 Hz to 75.1Hz |
|  |  | (6) Resolutions Supported | 1024 x 768 at 60Hz to 75Hz |
|  |  | (7) Active Display Area | Vertical x Horizontal : 304 mmx 228mm |
|  |  | (8) Power Supply | AC100-240V/50-60Hz |
|  |  | (9) Dimensions | 370mm(W)x385mm(H)x172mm(D) |
|  |  | (10) Weight | 4.1kg |

More details of technical specification please refer to the User’s Manual.

|  | 8-2. | Connecting I/O Port(s) (check one, if applicable) |
| --- | --- | --- |
|  |  | Please refer to the User's Manual. |

|  | 8-3. | Products Covered |
| --- | --- | --- |

| The equipment under test covered in this test report includes the following sub-system, module, and/or accessory: |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Sub-system, Module, and/or Accessory |  |  | Model/Type No. |  |  | Int. Inst./ Ext. Cont. |
| Panel |  |  |  | HSD150MX41 |  | Int. |

|  | 8-4. | Model Difference (Series, Versions, if any) |
| --- | --- | --- |

N/A

| 9. | Electric Block Diagram |  |
| --- | --- | --- |
|  | See Attachment - B. enclosed. |  |

| 10. | EUT Modifications ( check one, if applicable) |  |  |  |
| --- | --- | --- | --- | --- |
|  | ( X ) |  | No any modification required for the EUT to comply with the standards mentioned in Part-I. Sec. 4. above during the measurement. |  |
|  | ( ) |  | To achieve compliance with the standards mentioned in Part-I. Sec. 4. above during the measurement, the modifications as Attachment-C. enclosed shall be implemented in all mass production models of this equipment for compliance to be maintained. |  |

| 11. | EUT Photos |  |
| --- | --- | --- |
|  | See Attachment - F. enclosed. |  |

| 12. | Information to the User |  |
| --- | --- | --- |
|  | See Attachment - D. enclosed. |  |

| 13. | Conformity of Declaration |  |
| --- | --- | --- |
|  | See Attachment - E. enclosed. |  |

Part - II.  RFI Emissions Measurement

Table Contents

1. Standard Compliance

2. Limits of Emissions

3. Test Methodology

4. Sample(s) Tested

5. System Tested Configuration.

6. Conducted and Radiated Measurement Photos

7. Emissions Measurement Data

PART - II.  RF EMISSIONS MEASUREMENT

| 1. | Standard compliance |
| --- | --- |
|  | EN 55022:1998/CISPR 22:1997 , ( ) Class A / ( X ) Class B |

| 2. | Limits of RF Emissions |  |
| --- | --- | --- |
|  | 2-1 | Conducted Emission Limits for the Frequency Range 150KHz-30MHz |

|  | Frequency | Mains Terminal |  | Mains Terminals |  |
| --- | --- | --- | --- | --- | --- |
|  | Range | Quasi-Peak Mode |  | Average Mode |  |
|  | (MHz) | (dBuV) |  | (dBuV) |  |
|  |  | ( ) Class A | ( X ) Class B | ( ) Class A | ( X ) Class B |
|  | 0.15 - 0.50 | 79 | 66-56 | 66 | 56-46 |
|  | 0.50 - 5.00 | 73 | 56 | 60 | 46 |
|  | 5.00 - 30.0 | 73 | 60 | 60 | 50 |

|  | 2-2 | Radiated Emission Limits for the Frequency Range 30MHz-1000MHz |
| --- | --- | --- |

|  | Frequency |  | Quasi-Peak |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Range |  | (dBV/m @ 10m) |  |  |  |
|  | (MHz) |  | ( ) Class A |  | (X ) Class B |  |
|  | 30 - 230 |  | 40 |  | 30 |  |
|  | 230 - 1000 |  | 47 |  | 37 |  |

| 3. | Test Methodology |
| --- | --- |
|  | Both conducted and radiated testing were performed according to the procedures in CISPR 22 (1997). Radiated testing was performed at an antenna to EUT distance 10 meters. |

| 4. | Sample(s) Tested |  |
| --- | --- | --- |
|  | The representative sample tested in this report is(are) model(s): | AL502 |
|  | Test results in this test report relate only to the sample(s) tested. |  |

| 5. | System Tested Configuration |  |  |  |
| --- | --- | --- | --- | --- |
|  | 5-1 |  | Tested System Details |  |
|  |  |  |  | The system was configured for testing in a typical fashion (as a customer would normally use) or in-accordance with the operating configuration specified in the user‘s manual. Figure 2-5-1 shown the Configuration of System Tested; Table 2-5-1 shown all equipment items, plus descriptions of the FCC IDs used in the system tested and Table 2-5-2 shown the descriptions of all cables used in this testing. |

Fig.  2-5-1  Configuration of Tested System

Table  2-5-1 Equipment Used in Tested System

| Item | Equipment | Mfr/Brand | Model/Type No. | Port Connected | FCC ID | Series No. | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| E-1 | LCD Monitor | acer | AL502 | VGA Port | N/A(3) | N/A | EUT |
| E-2 | PC | HP | 8801 |  | N/A | N/A |  |
| E-3 | Printer | SII | DPU-414 | Centronic Port | N/A(3) | 1045105A |  |
| E-4 | Keyboard | Forward | FDA-104GA | PS/2 Port | F4ZFDA-104G | FDKB8110136 |  |
| E-5 | Mouse | HP | M-S34 | PS/2 Port | DZL211029 | C3751-60201 |  |
| E-6 | Modem | ACEEX | DM-1414V | Com port | N/A | 8041708 |  |

| Note: |  |
| --- | --- |
| (1) | Unless otherwise denoted as EUT in『Remark』column , device(s) used in tested system is a support equipment. |
| (2) | Unless otherwise marked as ※ in『Remark』column, Neutron consigns the support equip- ment to the tested system. |
| (3) | The support equipment was authorized by Declaration of Confirmation. |

Table  2-5-2 Information of Interface Cable

| Item | I/O Cable | Device Connected | Shielded Type | Ferrite Core | Detachable/ Permanently | Length | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| C-1 | Video Cable | EUT-PC | Yes | Yes | Permanently attached on Monitor | 150cm | ※ |
| C-2 | Centronics Cable | PC-Printer | Yes | No | Part of Printer, Detachable | 200cm |  |
| C-3 | Keyboard Cable | PC-Keyboard | Yes | No | Permanently attached on Keyboard | 200cm |  |
| C-4 | Mouse Cable | PC-Mouse | Yes | No | Permanently attached on Mouse | 180cm |  |
| C-5 | Interface Cable | PC-Modem | Yes | No | Permanently attached on Mouse | 180cm |  |
| C-6 | Audio Cable | EUT-PC | Yes | No | Part of EUT, Detachable | 150cm | ※ |

| Note: |  |
| --- | --- |
| (1) | Unless otherwise marked as ※ in『Remark』column, Neutron consigns the support equip- ment to the tested system. |
| (2) | For detachable type I/O cable should be specified the length in cm in『Length』column. |

5-2. Justification

The system was configured for testing in a typical fashion (as a customer would normally use it). The EUT (LCD Monitor) was connected to a support equipment-Personal Computer. Peripherals of PC, such as mouse, keyboard, modem and printer were contained in this test system in order to comply with the ANSI C63.4/CISPR 22 (1997)Rules requirement. In addition, the EUT(LCD Monitor) was connected with a support equipment in follows:  the EUT Audio-In jack connected with PC to the audio output port.  a headphone connected with the EUT Earphone jack to support as an audio device.  Four USB KB/Mouse connected with the EUT USB connectors to support as an USB device. The system was investigated/evaluated by pre-scanning the pixel resolution in follows mode(s):  the default mode of 640×480/31.5KHz  the max. pixel resolution mode of 1024×768/60KHz Hsync. 75Hz refresh , NI  the highest hor. sweep rate mode of 1024×768/60KHz Hsync. 75Hz refresh , NI The system operated in following mode(s) was (were) found to be the worst case during the pre-scanning. This operating mode(s) was (were) tested and used to collect the data included.  Another resolution mode of 800×600 48KHz Hsync. 75Hz refresh, NI  the highest hor. sweep rate mode of 1024×768/60KHz Hsync. 75Hz refresh , NI

5-3. EUT Exercise Software

The EUT exercise program used during radiated and conducted testing was designed to exercise the various system components in a manner similar to a typical use. The software, contained on a 3-1/2 inch disk, was inserted into driver A and is auto-starting on power-up. Once loaded, the program sequentially exercises each system component in turn. The sequence used is: 1. Read (write) from (to) mass storage device (Disk). 2. Send "H" pattern to video port device (LCD Monitor). 3. Send " H " pattern to parallel port device (Printer). 4. Send " H " pattern to serial port device (Modem). 5. Repeated from 2 to 4 continuously. As the keyboard and mouse are strictly input devices, no data is transmitted to (from) them during test. They are, however, continuously scanned for data input activity.

| 6. | Conducted and Radiated Measurement Photos |  |
| --- | --- | --- |

6-1. Conducted Measurement Photos

| 6. | Conducted and Radiated Measurement Photos |  |
| --- | --- | --- |

6-2. Radiated Measurement Photos

| 7. | Emissions Measurement Data |  |
| --- | --- | --- |

7-1. Conducted Emission Data

The initial step in collecting conducted data is a spectrum analyzer peak scan of the measurement range. Significant peaks are then marked as shown on the above data, and these signals are then Quasi-Peaked. Data of Table 2-7-1. lists the significant emission frequencies , measured levels, limits and safe margins.

7-2. Radiated Emission Data

The initial step in collecting Radiated data is a spectrum analyzer peak scan of the measurement range. Significant peaks are then marked as shown on the above data, and these signals are then Quasi-Peaked. Data of Table 2-7-2. lists the significant emission frequencies, measured levels, correction factor(includes cable and antenna corrections), the corrected reading, as well as the limit, safe margins. Explanation of the Correction Factor is given in Section 7-3.

# Table  2-7-1.  Conducted Emission Data

| Project No. | 02E2129 |  |  | Model/Type No. |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 22.0 | ℃ | Relative Humidity： | 56.0 | ％ | Test Date | Mar. 05, 2002 |
| Standard No./Limit Apply： |  |  | CISPR 22/EN 55022，( ) Class A / ( X ) Class B |  |  |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |

| Judgement：Passed by | -3.41 | dB at | 2.93 | MHz |  |  | AVG |  | X | QP | X | Line |  | Neutral |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Remark： |  |
| --- | --- |
| (1) | Reading in which marked as QP means measurements by using are Quasi-Peak Mode with Detector BW=9KHz；SPA setting in RBW=100KHz,VBW =100KHz, Swp. Time = 0.3 sec./MHz。 Reading in which marked as AV means measurements by using are Average Mode with instrument setting in RBW=1MHz,VBW=10Hz, Swp. Time =0.3 sec./MHz。 |
| (2) | All readings are QP Mode value unless otherwise stated AVG in column of『Note』. If the QP Mode Measured value compliance with the QP Limits and lower than AVG Limits, the EUT shall be deemed to meet both QP & AVG Limits and then only QP Mode was measured, but AVG Mode didn‘t perform。In this case, a “ * ” marked in AVG Mode column of Interference Voltage Measured。 |
| (3) | Measuring frequency range from 150KHz to 30MHz。 |

| Tech. Mgr.： |  |  | Test Engr.： |  |
| --- | --- | --- | --- | --- |

## Table  2-7-2.  Radiated Emission Data

| Project No. | 02E2129 |  |  | Model/Type No. |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 24.0 | ℃ | Relative Humidity | 58.0 | ％ | Test Date | Feb. 27, 2002 |
| Standard No./Limit Apply： |  |  | CISPR 22/EN 55022，( ) Class A / ( X ) Class B，10m Ant. Distance |  |  |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |

| Judgement：Passed by | -2.35 | dB at | 251.52 | MHz |  | X | Peak |  |  | QP |  | Hor. | X | Vert. |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Remark： |  |
| --- | --- |
| (1) | Reading in which marked as QP or Peak means measurements by using are Quasi-Peak Mode or Peak Mode with Detector BW=120KHz；SPA setting in RBW=1MHz, VBW =1MHz, Swp. Time = 0.3 sec./MHz。 |
| (2) | All readings are Peak unless otherwise stated QP in column of『Note』. Peak denotes that the Peak reading compliance with the QP Limits and then QP Mode measure-ment didn‘t perform。 |
| (3) | Measuring frequency range from 30MHz to 1000MHz。 |
| (4) | If the peak scan value lower limit more than 20dB, then this signal data does not how in table。 |

| Tech. Mgr.： |  |  | Test Engr.： |  |
| --- | --- | --- | --- | --- |

7-3. Field Strength Calculation

The field strength is calculated by adding the Antenna Factor and Cable Factor and subtracting the Amplifier Gain (if any) from the measured reading. The basic equation with a sample calculation is as follows: FS = RA + AF + CL - AG Where FS = Field Strength RA = Receiver Amplitude AF = Antenna Factor (1) CL = Cable Attenuation Factor(Cable Loss) (1) AG = Amplifier Gain (1) (2) Remark : (1) The Correction Factor = AF + CL - AG, as shown in the data tables' Correction Factor column. (2) AG is not available for Neutron's Open Site Facility Example of Calculation: Assume a Receiver Reading of 23.7 dBuV is obtained with an Antenna Factor of 7.2 dB and a Cable Factor of 1.1 dB. Then: 1. The Correction Factor will be calculated by Correction Factor = AF + CL - AG = 7.2 + 1.1 - 0 = 8.3 (dB) as shown in the data tables' Correction Factor column. 2. The Field Strength will be calculated by FS = RA + Correction Factor = 23.7 + 8.3 = 32 (dBuV/m). FS is the value shown in the data tables' Corrected Reading column and RA is the value shown in the data tables' Receiver Reading column. The 32 dBuV/m value was mathe-matically converted to its corresponding level in uV/m as: Log-1【(32.0dBuV/m)/20】＝ 39.8 (uV/m)

7-4. Correction Factor VS Frequency

Frequency (MHz)       Antenna Factor (dB)           Cable Loss (dB)

| 30.00 | 11.10 | 0.20 |
| --- | --- | --- |
| 35.00 | 10.80 | 0.00 |
| 40.00 | 11.20 | 0.40 |
| 45.00 | 11.50 | 0.40 |
| 50.00 | 11.30 | 0.90 |
| 55.00 | 10.50 | 0.00 |
| 60.00 | 9.90 | 0.00 |
| 65.00 | 8.70 | 0.20 |
| 70.00 | 7.60 | 0.00 |
| 75.00 | 6.40 | 0.50 |
| 80.00 | 6.10 | 0.10 |
| 85.00 | 7.00 | 0.80 |
| 90.00 | 8.00 | 0.30 |
| 95.00 | 10.00 | 0.40 |
| 100.00 | 11.20 | 0.60 |
| 110.00 | 12.60 | 0.60 |
| 120.00 | 13.00 | 0.60 |
| 130.00 | 12.50 | 0.50 |
| 140.00 | 12.00 | 0.20 |
| 150.00 | 12.00 | 1.00 |
| 160.00 | 13.20 | 1.20 |
| 170.00 | 14.80 | 1.60 |
| 180.00 | 16.30 | 1.90 |
| 190.00 | 17.00 | 1.90 |
| 200.00 | 17.30 | 1.40 |
| 225.00 | 10.50 | 1.10 |
| 250.00 | 11.70 | 2.00 |
| 275.00 | 12.80 | 2.40 |
| 300.00 | 14.50 | 2.40 |
| 325.00 | 14.00 | 1.90 |
| 350.00 | 14.20 | 2.40 |
| 375.00 | 14.60 | 2.90 |
| 400.00 | 15.10 | 2.70 |
| 450.00 | 16.20 | 3.20 |
| 500.00 | 17.60 | 3.70 |
| 550.00 | 17.80 | 3.90 |
| 600.00 | 18.40 | 4.30 |
| 650.00 | 19.50 | 4.00 |
| 700.00 | 20.80 | 4.10 |
| 750.00 | 20.50 | 5.30 |
| 800.00 | 21.10 | 5.90 |
| 850.00 | 22.40 | 5.80 |
| 900.00 | 23.50 | 5.50 |
| 950.00 | 24.00 | 6.30 |
| 1000.00 | 24.80 | 5.20 |

Part - III.  Electromagnetic Immunity Test

Table Contents

1. Standard Compliance/Servrity Level/Criteria

2. Test Methodology

3. Sample(s) Tested

4. Test Set-Up Configuration

5. EUT Operating Condition

6. EUT Tested Results

PART - III.  Immunity Test

| 1. | Standard compliance/ Servility Level/Criteria |
| --- | --- |

|  | Tests Standard No. | Test Specification Servility Level | Test Mode Test Ports | Perform. Criteria | Remark |
| --- | --- | --- | --- | --- | --- |
|  | 1. ESD EN61000-4-2 (1995) IEC801-2 (1991) | 8KV air discharge 4KV contact discharge | Direct Mode | B |  |
|  |  | 4KV HCP discharge 4KV VCP discharge | Indirect Mode | B |  |
|  | 2. RS EN61000-4-3 (1996) | 80 MHz to 1000 MHz 3V/m(rms), 1 KHz, 80%, AM modulated | Enclosure | A |  |
|  | IEC801-3 (1984) | 27 MHz to 500 MHz 3V/m(rms), Un-modulated | Enclosure | A | N/A |
|  | 3. EFT/Burst EN61000-4-4 (1995) | 1.0KV(peak) 5/50ns Tr/Th 5KHz Repetition Freq. | Power Supply Port | B |  |
|  |  | 0.5 KV(peak) 5/50ns Tr/Th 5KHz Repetition Freq. | CTL/Signal Data Line Port | B | The cable less than 3m |
|  | 4. Surges EN61000-4-5 (1995) | 1 KV(5P/5N) 1.2/50(80/20) Tr/Th us | Line to Line Line to Neutral | B |  |
|  |  | 2 KV(5P/5N) 1.2/50(80/20) Tr/Th us | Line to PE Neutral to PE | B |  |
|  | 5 Injected Current EN61000-4-6 (1996) | 0.15 MHz to 80 MHz 3V(rms), 1KHz 80﹪, AM Modulated 150 source impedance | CTL/Signal Data Line | A | The cable less than 3m |
|  |  | 0.15 MHz to 80 MHz 3V(rms), 1KHz 80﹪, AM Modulated 150 source impedance | AC Power Port | B |  |
|  |  | 0.15 MHz to 80 MHz 3V(rms), 1KHz 80﹪, AM Modulated 150 source impedance | DC Power Port | B | No DC Power Port |
|  | 6. Power Frequency Magnetic Field EN 61000-4-8 (1993) | 50 Hz, 1A/m | Enclosure | A |  |
|  | 7. Volt. Interruptions | Voltage dig>95% | <5% | B |  |
|  | Volt. Dips EN61000-4-11 (1994) | Voltage dig 30% Interruption>95% | 70% <5% | C C |  |

* Remark:   N/A - denotes test is not applicable in this Test Report

| 2. | Test Methodology |  |  |  |
| --- | --- | --- | --- | --- |
|  | The testing was carried out according to the referenced Basic Standards listed in the follows: |  |  |  |
|  | Tests | Basic Standards | Provisions Referenced | Remark |
|  | 1. ESD | EN 61000-4-2 IEC 61000-4-2 | Clause 5.0, 7.0, 8.0 |  |
|  | 2. RF Electromagnetic Field Strength | EN 61000-4-3 IEC 61000-4-3 | Clause 5.0, 7.0, 8.0 |  |
|  | 3. EFT/Burst | EN 61000-4-4 IEC 61000-4-4 | Clause 5.0, 7.0, 8.0 |  |
|  | 4. Surges | EN 61000-4-5 IEC 61000-4-5 | Clause 5.0, 7.0, 8.0 |  |
|  | 5. Injected Current | EN 61000-4-6 IEC 61000-4-6 | Clause 5.0, 7.0, 8.0 |  |
|  | - Power Frequency Magnetic Field | EN61000-4-8 IEC 61000-4-8 | Clause 5.0, 7.0, 8.0 |  |
|  | 7. Volt. Interruptions Volt. Dips | EN 61000-4-11 IEC 61000-4-11 | Clause 5.0, 7.0, 8.0 |  |

Remark: * N/A - denotes testing is not applicable in this Test Report

| 3. | Sample(s) Tested |  |
| --- | --- | --- |
|  |  | The representative sample tested in this reports is the same as the statements of Part II. Sec. 4. unless otherwise a special model no. is specified in the record (Table of Test Results). |

| 4. | Test Set-Up Configuration |  |
| --- | --- | --- |
|  |  | The testing was carried out according to the Test Set-Up Configuration Figures in the follows: |

|  | Tests | Basic Standards | Test Set-Up Configuration Figures | Remark |
| --- | --- | --- | --- | --- |
|  | 1. ESD | EN 61000-4-2 IEC 61000-4-2 | Fig. 3-4-1 ESD Test Set-Up |  |
|  | - RF Electromagnetic Field Strength | EN 61000-4-3 IEC 61000-4-3 | Fig. 3-4-2 RS Test Set-Up |  |
|  | 3. EFT/Burst | EN 61000-4-4 IEC 61000-4-4 | Fig. 3-4-3 EFT/Burst Test Set-Up |  |
|  | 4. Surges | EN 61000-4-5 IEC 61000-4-5 | Fig. 3-4-4 Surges Test Set-Up |  |
|  | 5. Injected Current | EN 61000-4-6 IEC 61000-4-6 | Fig. 3-4-5 Injection Current Test Set-Up |  |
|  | - Power Frequency Magnetic Field | EN 61000-4-8 IEC 61000-4-8 | Fig. 3-4-6 Power Frequency Magnetic Field Test Set-Up |  |
|  | 7. Volt. Interruptions Volt. Dips | EN 61000-4-11 IEC 61000-4-11 | Fig. 3-4-7 Volt. Interruptions/Dips Test Set-Up |  |

Remark: * N/A - denotes test is not applicable in this Test Report

Fig. 3-4-1 ESD Test Set-Up Configuration

Reference Ground Plane(GRP) Bonded to PE

Nearest Wall

PVC

HCP

80cm

1m

EUT

VCP 50 cm x 50 cm

470K

470K

Non-Conductive Table

To AC Main

10 cm

ESD Generator

NSG 435

Discharge

Return Cable

to GRP

ESD Generator

NSG 435

Discharge Return

Cable to GRP

Fig. 3-4-2 RF Electromagnetic Field Strength Test Set-Up Configuration

Fiber

SG

3m

Meter

Power

Amplifier

GPIB

Keyboard

Mouse

System Controller

Computer

EUT

1.55 m(h)

0.8m(h)

Monitor

CCD

Fig. 3-4-3(A) EFT Test Set-Up Configuration for Power Supply Ports

Reference Ground Plane (GRP) Bonded to Earth

Nearest Wall

80 cm

1m

EUT

Non-Conductive Table

Coupling/

Decoupling

Network Device

EFT/Burst

Generator

To Earth

To AC Mains

Supply

Protective GND

Protective GND According

to System Installation

40 cm

L  1 m  * Remark

PEFT Junior

Remark：

If the manufacturer provides a non-detachable power cord more than 1m long

with the EUT, the excess length of this power cord shall be folded back and forth

forming a bundle 30-40 cm long and situated at a distance of 10 cm above the

reference ground plane(GRP).

Fig. 3-4-3(B) EFT Test Set-Up Configuration for CTL/Signal I/O Ports

- When both EUT are tested simultaneously:

L1=L2 < 1M between clamp and EUT to be tested

- When one EUT only is tested:

L2 min.  5 m or

L2 > 5 x L1 for de-coupling purposes

Nearest Wall

1m

AC Mains Supply

Capacitive

Coupling

Clamp

L1

L2

EUT

10 cm

10cm

Reference Ground Plane

(GRP) Bonded to Earth

GND Connection

According to the

MFR’s Specification

To Earth

EFT/Burst

Generator

Coupling/

Decoupling

Network Device

To AC Mains

Supply

PEFT Junior

Protective GND

To Earth

Fig. 3-4-4 Surge Test Set-Up Configuration

AC/DC Power Supply

and Surge Voltage

Coupling to EUT

Paurge 4.1

Combination Wave

Generator

To AC Mains

or DC Power

Supply

Decoupling

Network

Coupling

Network

EUT

Non-Metallic Table

Connect to Earth

Reference Ground Plane(GRP) Bonded to Earth

Fig. 3-4-5 Injection Current Test Set-Up Configuration

Simi-anechoie Chamber

25cm

D 50 cm

D  50 cm

S.G

EUT

CCD

CDN

Power

Amplifier

Insulation Support

10cm

GPIB

ground plane

System Control Computer

ground wire

Attenuator

6dB/25W

Keyboard

Mouse

Monitor

Fig. 3-4-6 Power Frequency Magnetic Field Test Set-Up Configuration

Fig. 3-4-7 Voltage Interruption/Dips Test Set-Up Configuration

Mains

Interference

Simulator

Pline 1610

To AC

Mains

Supply

Voltage Supply

to EUT

EUT

Non-Metallic Table

Reference Ground Plane(GRP) Bonded to Earth

| 5. | EUT Operating Condition |
| --- | --- |
|  | The EUT tested system was configured as the statements of Part II. Sec. 5. unless otherwise a special operating condition is specified in the follows during the testing. |

| 6. | EUT Tested Results |  |  |  |
| --- | --- | --- | --- | --- |
|  | Tests | Basic Standards | EUT Tested Results | Remark |
|  | 1. ESD | EN 61000-4-2 IEC 61000-4-2 | Table 3-6-1 ESD Testing |  |
|  | 2. RF Electromagnetic Field Strength | EN61000-4-3 IEC 61000-4-3 | Table 3-6-2 RS Testing |  |
|  | 3. EFT/Burst | EN 61000-4-4 IEC 61000-4-4 | Table 3-6-3 EFT/Burst Testing |  |
|  | 4. Surges | EN 61000-4-5 IEC 61000-4-5 | Table 3-6-4 Surges Testing |  |
|  | 5. Injected Current | EN 61000-4-6 IEC 61000-4-6 | Table 3-6-5 Injection Current Testing |  |
|  | - Power-frequency Magnetic-field | EN 61000-4-8 IEC 61000-4-8 | Table 3-6-6 Power Frequency Magnetic Field Testing |  |
|  | - Volt. Interruptions Volt. Dips | EN 61000-4-11 IEC 61000-4-11 | Table 3-6-7 Volt. Interruptions/ Dips Testing |  |

Remark: * N/A - denotes test is not applicable in this Test Report

Table 3-6-1-(A)  ESD Testing

| Project No. ： | 02E2129 |  |  |  |  | Model/Type No. ： |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 26.0 | ℃ | Relative Humidity： |  |  | 58.0 | ％ | Test Date： | Mar. 07, 2002 |
| Standard No. Apply： |  | ( ) IEC 801-2 |  |  |  | ( X ) IEC61000-4-2 |  |  |  |
| Discharge Mode： |  | ( X ) Direct Air / Contact |  |  |  |  |  | ( ) Indirect HCP/VCP |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |

| Mode | (X) Air / ( ) HCP Discharge |  |  |  |  |  |  |  | (X) Contact / ( ) VCP Discharge |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2KV |  | 4KV |  | 8KV |  | 15KV |  | 2KV |  | 4KV |  | 6KV |  | 8KV |  |
| Location | P | N | P | N | P | N | P | N | P | N | P | N | P | N | P | N |
| 1 | B | B | B | B | B | B |  |  | B | B | B | B |  |  |  |  |
| 2 | B | B | B | B | B | B |  |  |  |  |  |  |  |  |  |  |
| 3 | B | B | B | B | B | B |  |  |  |  |  |  |  |  |  |  |
| 4 | B | B | B | B | B | B |  |  |  |  |  |  |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Criteria | B |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |
| Judgment | PASS |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |
|  | 3) Test location(s) in which discharge to be applied illustrated by photos shown in next page(s). |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table 3-6-1-(B)  ESD Testing

| Project No. ： | 02E2129 |  |  |  |  | Model/Type No. ： |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 26.0 | ℃ | Relative Humidity： |  |  | 58.0 | ％ | Test Date： | Mar. 07, 2002 |
| Standard No. Apply： |  | ( ) IEC 801-2 |  |  |  | ( X ) IEC61000-4-2 |  |  |  |
| Discharge Mode： |  | ( ) Direct Air / Contact |  |  |  |  |  | ( X ) Indirect HCP/VCP |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |

| Mode | ( ) Air / (X) HCP Discharge |  |  |  |  |  |  |  | ( ) Contact / (X) VCP Discharge |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 2KV |  | 4KV |  | 6KV |  | 8KV |  | 2KV |  | 4KV |  | 6KV |  | 8KV |  |
| Location | P | N | P | N | P | N | P | N | P | N | P | N | P | N | P | N |
| 1 | B | B | B | B |  |  |  |  | B | B | B | B |  |  |  |  |
| 2 | B | B | B | B |  |  |  |  | B | B | B | B |  |  |  |  |
| 3 | B | B | B | B |  |  |  |  | B | B | B | B |  |  |  |  |
| 4 | B | B | B | B |  |  |  |  | B | B | B | B |  |  |  |  |
| 5 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 6 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 7 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 9 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Criteria | B |  |  |  |  |  |  |  | B |  |  |  |  |  |  |  |
| Judgment | PASS |  |  |  |  |  |  |  | PASS |  |  |  |  |  |  |  |

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |
|  | 3) Test location(s) in which discharge to be applied illustrated by photos shown in next page(s). |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Photo(s) shown the location(s) ESD evaluated

02E2129

02E2129

Table 3-6-2-(A)   EFT/Burst Testing

| Project No. ： | 02E2129 |  |  |  |  |  |  |  |  |  |  |  |  | Model/Type No. ： |  |  |  |  | AL502 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 26.0 |  | ℃ |  |  |  | Relative Humidity： |  |  |  |  |  |  | 58.0 |  | ％ |  |  | Test Date： |  | Mar. 07, 2002 |  |
| Standard No. Apply： |  |  |  | IEC 61000-4-4 |  |  |  |  |  |  | AC Mains Supply： |  |  |  |  |  |  |  | 230V, 50Hz , 1 |  |  |  |
| Voltage (Peak) ： |  | 1 KV |  |  |  | Repeat. Rate： |  |  |  |  |  |  | 5 KHz |  | Impulse Rise Time (Tr) ： |  |  |  |  |  |  | 5 ns±30％ |
| Impulse Duration (Th)： |  |  |  |  | 50 ns± 30％ |  |  |  |  |  |  |  |  |  |  |  | Burst Duration： |  |  | 15 ms±20％ |  |  |
| Burst Period： | 300 ms± 20％ |  |  |  |  |  |  |  |  | Duration of Test： |  |  |  |  |  |  | 1 min , 15 sec. rest between each testing |  |  |  |  |  |
| Test Mode： | ( X ) AC Power Line |  |  |  |  |  |  |  |  |  |  | ( ) DC Power Line |  |  |  |  |  |  | ( ) Signal/CTL Line |  |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Mode | ( X ) AC Power Line |  |  | ( ) DC Power Line |  | ( ) Signal/Control Line |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Port(s) | Polarity | Results | Polarity |  | Results | Polarity |  | Results |
| Line (L) | P | B | P |  |  | P |  |  |
|  | N | B | N |  |  | N |  |  |
| Neutral (N) | P | B | P |  |  | P |  |  |
|  | N | B | N |  |  | N |  |  |
| Ground (PE) | P | B | P |  |  | P |  |  |
|  | N | B | N |  |  | N |  |  |
| Criteria | B |  |  | N/A |  | N/A |  |  |
| Judgement | PASS |  |  | N/A |  | N/A |  |  |

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table 3-6-3  RS Testing

| Project No. ： |  | 02E2129 |  |  |  |  |  |  |  | Model/Type No. ： |  |  |  |  | AL502 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： |  | 26.0 |  | ℃ |  | Relative Humidity： |  |  |  | 58.0 |  | ％ |  |  | Test Date： |  | Mar. 07, 2002 |
| AC Mains Supply： |  |  |  |  | AC 230V, 50Hz , 1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard No. Apply： |  |  |  |  | ( ) IEC 801-3 |  |  |  |  |  | ( X ) IEC 61000-4-3 |  |  |  |  |  |  |
| Frequency Range： |  |  | ( ) 27 MHz - 500 MHz |  |  |  |  |  |  |  | ( X ) 80 MHz - 1000 MHz |  |  |  |  |  |  |
| Test Level： |  |  | ( X ) 3 V/m |  |  |  |  |  |  |  | ( ) 10 V/m |  |  |  |  |  |  |
| Modulation： | ( ) Un-Modulated |  |  |  |  |  |  | ( X ) Modulated |  |  |  |  |  | Signal Type： |  | 1KHz , 80% , AM |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Frequency Range (MHz) | RF Field Position | R.F. Field Strength | Results | Perform. Criteria | Judgment | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| 27MHz - 270MHz | Horizontal | 3 V(rms) |  | A |  | N/A |
| 270MHz - 500MHz |  |  |  |  |  |  |
| 27MHz - 270MHz | Vertical | 3 V(rms) |  | A |  | N/A |
| 270MHz - 500MHz |  |  |  |  |  |  |
| 80MHz - 500MHz | Horizontal | 3 V(rms) | A | A | PASS |  |
| 500MHz - 1000MHz |  |  | A |  | PASS |  |
| 80MHz - 500MHz | Vertical | 3 V(rms) | A | A | PASS |  |
| 500MHz - 1000MHz |  |  | A |  | PASS |  |

Remark: * N/A - denotes test is not applicable in this Test Report

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table-3-6-4  Surge Test Results

| Project No. ： | 02E2129 |  |  |  |  |  |  |  |  |  | Model/Type No. ： |  |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 26.0 | ℃ |  |  | Relative Humidity： |  |  |  |  |  | 58.0 | ％ |  | Test Date： | Mar. 07, 2002 |
| Mains Supply： | AC 230V, 50Hz , 1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Standard No. Apply： |  |  | ( ) IEC 801-5 |  |  |  |  |  |  | ( X ) IEC 61000-4-5 |  |  |  |  |  |
| Surge Wave form Generated： |  |  |  |  | Combination Wave, 1.2/50us-80/20us |  |  |  |  |  |  |  |  |  |  |
| Polarity and Numbers of Impulses： |  |  |  |  |  | 5 Pst/Ngt at each tested mode |  |  |  |  |  |  |  |  |  |
| Impulse Repetition Rate： |  |  |  | 5 sec. |  |  |  |  |  |  |  |  |  |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Wave Form EUT Ports Tested | 1.2/50(8/20)Ti/Th us |  |  |  | Perform. Criteria | Judgement |
| --- | --- | --- | --- | --- | --- | --- |
|  | Polarity | Phase | Voltage | Results |  |  |
| L - N | +/- | 0 | 1kV | B | B | PASS |
|  | +/- | 90 | 1kV | B | B | PASS |
|  | +/- | 180 | 1kV | B | B | PASS |
|  | +/- | 270 | 1kV | B | B | PASS |
| L - PE | +/- | 0 | 2kV | B | B | PASS |
|  | +/- | 90 | 2kV | B | B | PASS |
|  | +/- | 180 | 2kV | B | B | PASS |
|  | +/- | 270 | 2kV | B | B | PASS |
| N - PE | +/- | 0 | 2kV | B | B | PASS |
|  | +/- | 90 | 2kV | B | B | PASS |
|  | +/- | 180 | 2kV | B | B | PASS |
|  | +/- | 270 | 2kV | B | B | PASS |

Remark: * N/A - denotes test is not applicable in this Test Report

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table-3-6-5  Injection Current Test Results

| Project No. ： |  |  | 02E2129 |  |  |  |  |  |  |  |  |  |  |  | Model/Type No. ： |  |  |  |  |  | AL502 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： |  |  | 26.0 |  | ℃ |  | Relative Humidity： |  |  |  |  |  |  |  | 58.0 |  | ％ |  |  |  | Test Date： |  | Mar. 07, 2002 |
| Mains Supply： |  |  |  |  |  | ( X ) AC 230V, 50Hz , 1 |  |  |  |  |  |  |  |  |  | ( ) + DC 5 V |  |  |  |  |  |  |  |
| Standard No. Apply： |  |  |  |  |  | ( ) IEC 801-6 |  |  |  |  |  |  |  |  |  | ( X ) IEC 61000-4-6 |  |  |  |  |  |  |  |
| Frequency Range： |  |  |  | ( X ) 150 KHz - 80 MHz |  |  |  |  |  |  |  |  |  |  |  | ( ) 150KHz-230MHz |  |  |  |  |  |  |  |
| Test Level： |  |  |  | ( ) 1 V(rms) |  |  |  |  |  |  |  | ( X ) 3V(rms) |  |  |  |  |  |  |  |  | ( ) 10 V(rms) |  |  |
| Modulation： |  | ( ) Un-Modulated |  |  |  |  |  |  |  | ( X ) Modulated |  |  |  |  |  |  |  |  | Signal Type： |  |  | 1KHz , 80% , AM |  |
| Test Mode： | ( X ) AC Power Line |  |  |  |  |  |  |  |  |  | ( ) DC Power Line |  |  |  |  |  |  |  |  |  | ( ) Signal/CTL Line |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Test Ports (Mode) | Freq. Range MHz) | Field Strength | Perform. Criteria | Results | Judgement | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| Input/ Output AC. Power Port | 0.15 ---80 | 3V(rms) | A | A | PASS |  |
| Input/ Output DC. Power Port | 0.15 --- 80 | 3V(rms) | A |  |  | N/A |
| Signal / CTL Data Line | 0.15 --- 80 | 3V(rms) | A |  |  | N/A |

Remark: * N/A - denotes test is not applicable in this Test Report

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table-3-6-6   Power Frequency Magnetic Field Testing

| Project No. ： | 02E2129 |  |  |  |  |  |  | Model/Type No. ： |  |  |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 26.0 |  | ℃ | Relative Humidity： |  |  |  | 58.0 |  | ％ |  | Test Date： | Mar. 07, 2002 |
| Mains Supply： |  | ( X) AC 230V, 50Hz , 1 |  |  |  |  |  |  |  |  |  |  |  |
| Standard No. Apply： |  |  | ( X ) IEC 61000-4-8 |  |  |  |  |  |  |  |  |  |  |
| Test Level： |  |  | ( X )1A/m |  |  |  | ( )10A/m |  |  |  |  |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Test Mode | Test Level | Antenna aspect | Duration (s) | Perform Criteria | Results | Judgement |
| --- | --- | --- | --- | --- | --- | --- |
| Enclosure | 1 A/m | X | 30 s | A | A | PASS |
| Enclosure | 1 A/m | Y | 30 s | A | A | PASS |
| Enclosure | 1 A/m | Z | 30 s | A | A | PASS |

Remark: * N/A - denotes test is not applicable in this Test Report

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B – Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C – Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table-3-6-7   Tests of Voltage Interruption/DIPs

| Project No. ： | 02E2129 |  |  |  |  |  |  | Model/Type No. ： |  |  | AL502 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 26.0 | ℃ |  | Relative Humidity： |  |  |  | 58.0 | ％ |  | Test Date： | Mar. 07, 2002 |
| Mains Supply： | AC 230V, 50Hz , 1 |  |  |  |  |  |  |  |  |  |  |  |
| Standard No. Apply： |  |  | ( ) IEC 801-11 |  |  |  | ( X ) IEC 61000-4-11 |  |  |  |  |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |

| Test Mode | Test Level in &Ut | Duration (P) | Perform Criteria | Results | Judgement |
| --- | --- | --- | --- | --- | --- |
| Voltage dip ＞95% | <5% | 0.5 | B | B | PASS |
| Voltage dip 30% | 70% | 25 | C | C1 | PASS |
| Interruption＞95% | <5% | 250 | C | C1 | PASS |

Remark: * N/A - denotes test is not applicable in this Test Report

| Note: |  |
| --- | --- |
|  | 1) P/N denotes the Positive/Negative polarity of the output voltage. |
|  | 2) A/B/C denotes the performance criterion class: |
|  | A - No degradation of performance or loss of function is allowed below a performance level Specified by the manufacturer. |
|  | B - Temporary degradation of performance or loss of function, but no change of actual oper-ating state or stored data. |
|  | C - Temporary loss of function is allowed, provided the function is self-recoverable or can be restored by operation of the controls, or by any operation specified in the instructions for use. C1: before resetting C2: after resetting |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Part - IV.  Harmonics/Voltage Fluctuation and Flicker

Table Contents

1. Standards Compliance

2. Limits

3. Test Methodology

4. Sample(s) Tested

5. Test Set-Up Configuration

6. EUT Operating Condition

7. EUT Tested Results

PART - IV.  HARMONICS TEST

| 1. | Standard Compliance |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ( ) |  | EN 60555-2:1988/IEC 555-2 | ( ) | EN 60555-3:1988/IEC 555-3 |
|  | ( X ) |  | EN 61000-3-2:1995/IEC 61000-3-2 | ( X ) | EN 61000-3-3:1995/IEC1000-3-3 |

| 2. | Limits |  |
| --- | --- | --- |
|  | 2-1. Limits of Harmonic Current |  |

| IEC 555-2:1982+Amd. 1:1985+Amd. 2:1988 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Table - I |  |  | Table - II |  |  |
| Equipment Category | Harmonic Order n | Max. permissible harmonic current (in Ampers) | Equipment Category | Harmonic Order n | Max. permissible harmonic current (in Ampers) |
|  | odd | harmonics |  | odd | harmonics |
|  | 3 | 2.30 |  | 3 | 0.80 |
|  | 5 | 1.14 |  | 5 | 0.60 |
|  | 7 | 0.77 |  | 7 | 0.45 |
| Non | 9 | 0.40 | TV | 9 | 0.30 |
| Portable | 11 | 0.33 | Receivers | 11 | 0.17 |
| Tools | 13 | 0.21 |  | 13 | 0.12 |
| or | 15n39 | 0.15．15/n |  | 15n39 | 0.10．15/n |
| TV | even | harmonics |  | even | harmonics |
| Receivers | 2 | 1.08 |  | 2 | 0.30 |
|  | 4 | 0.43 |  | 4 | 0.15 |
|  | 8 | 0.30 |  |  |  |
|  | 8n40 | 0.23．8/n |  | DC | 0.05 |

Note: For Portable tools, a multiplication factor of 1.5 shall be applied to the limits specified

in Table - I.

| EN 61000-3-2:1995+A1:1998+A2:1998/IEC1000-3-2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Equipment Category | Max. permissible harmonic current (in Ampers) | Equipment Category | Harmonic Order n | Max. permissible harmonic current (in A) (mA/w) |  |
|  |  |  | 3 | 2.30 | 3.4 |
|  | Same as Limits |  | 5 | 1.14 | 1.9 |
| Class A | Specified in | Class D | 7 | 0.77 | 1.0 |
|  | 4-2.1, Table - I, |  | 9 | 0.40 | 0.5 |
|  | but only odd |  | 11 | 0.33 | 0.35 |
|  | harmonics required |  | 13n39 | see Table I | 3.85/n |
|  |  |  | only odd harmonics required |  |  |

|  | 2-2. Limits of Fluctuation and Flicker |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Tests | Limits |  | Descriptions |
|  |  |  | IEC555-3 | IEC 61000-3-2 |  |
|  |  | Pst |  1.0, Tp= 10 min. |  1.0, Tp= 10 min. | Short Term Flicker Indicator |
|  |  | Plt | N/A |  0.65, Tp=2 hr. | Long Term Flicker Indicator |
|  |  | dc |  3 % |  3 % | Relative Steady-State V-Chang |
|  |  | dmax |  4 % |  4 % | Maximum Relative V-change |
|  |  | d (t) | N/A |  3% for  200 ms | Relative V-change characteristic |

| 3. | Test Methodology |  |  |
| --- | --- | --- | --- |
|  |  | 3-1. Harmonic Current Test |  |
|  |  |  | Tests was performed according to the procedures specified in Clause 5.0 of IEC555-2 and/or Sub-clause 6.2 of IEC 61000-3-2 depend on which standard adopted for compliance measurement. |

|  | 3-2. Fluctuation and Flickers Test |  |
| --- | --- | --- |
|  |  | Tests was performed according to the Test Conditions/Assessment of Voltage Fluctu-ations specified in Clause 5.0/6.0 of IEC555-3 and/or Clause 6.0/4.0 of IEC61000-3-3 depend on which standard adopted for compliance measurement. |

|  | All types of harmonic current and/or voltage fluctuation in this report are assessed by direct measurement using flicker-meter, which compliance with the specification given in IEC868, connected as the test set-up configuration described in Section 6. |
| --- | --- |

| 4. | Sample(s) Tested |  |
| --- | --- | --- |
|  |  | The representative sample tested in this reports is the same as the statements of Part II. Sec. 4. unless otherwise a special model no. is specified in the record (Table of Test Results). |

| 5. | Test Set-Up Configuration |
| --- | --- |
|  | The test set-up configuration, including the auxiliary instruments, is sketched as block diagram of Fig. 4-6-1 in next page. |

| 6. | EUT Operating Condition |
| --- | --- |
|  | The EUT tested system was configured as the statements of Part II. Sec. 5. unless otherwise a special operating condition is specified in the follows during the testing. |

| 7. | EUT Tested Results |  |  |  |
| --- | --- | --- | --- | --- |
|  | Items | Tests | EUT Tested Results | Remark |
|  | 1. | Harmonics Current | Table 4-8-1 |  |
|  | 2. | Voltage Fluctuations/Flickers | Table 4-9-2 |  |

* Remark:   N/A - denotes test is not applicable in this Test Report

Fig. 4-6-1  Harmonics / Flicker Test Set-Up Configuration

Power Cord

L   N

Hi    Lo     Hi    Lo

L

N

L

N

Power Cord

GP-IB

Lo

Hi

Lo

Hi

Power Cord

Standard

Power Source

Chroma Model: 6530

E.U.T.

AC Mains Supply

        

        



V         A

Power

Analyzer

Chroma Model：6630

Connection Pad

Table 4-8-1   Harmonics Current Testing

| Project No. ： | 02E2129 |  |  |  |  |  |  | Model/Type No.： |  |  | AL502 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 25.0 | ℃ | Relative Humidity： |  |  |  |  | 55.0 | ％ |  | Test Date： |  |  | Mar. 07, 2002 |
| Test Condition (AC Input)： |  |  | 231.09 | V | 0.19505 |  | A | 19.6646 | W | 50 | Hz | P.F. |  | 0.43627 |
| Standard No. Apply： |  | ( ) IEC 555-2 |  |  |  |  |  | ( ) Table I |  |  |  |  | ( ) Table I x 1.5 |  |
|  |  | ( X ) IEC 61000-3-2 |  |  |  |  |  | ( ) Class A |  |  |  |  | ( X ) Class D |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Exemption：The EUT was exempted from Harmonics Current Test due to the equipment with an active input power  75W.

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Table 4-8-1   Harmonics Current Testing

| Project No. ： | 02E2129 |  |  |  |  |  |  | Model/Type No.： |  |  | AL502 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature： | 25.0 | ℃ | Relative Humidity： |  |  |  |  | 55.0 | ％ |  | Test Date： |  |  | Mar. 07, 2002 |
| Test Condition (AC Input)： |  |  | 231.09 | V | 0.19505 |  | A | 19.6646 | W | 50 | Hz | P.F. |  | 0.43627 |
| Standard No. Apply： |  | ( ) IEC 555-2 |  |  |  |  |  | ( ) Table I |  |  |  |  | ( ) Table I x 1.5 |  |
|  |  | ( X ) IEC 61000-3-2 |  |  |  |  |  | ( ) Class A |  |  |  |  | ( X ) Class D |  |
| Special Notes：(EUT Operation Mode or Test Configuration Mode, if applicable) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Exemption：The EUT was exempted from Harmonics Current Test due to the equipment with an active input power  75W.

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

| 4-9 | Test Results |  |  |  |
| --- | --- | --- | --- | --- |
| 4-9.1 | Flickers |  |  |  |
|  | Kind of Product | LCD Monitor | Model No.: | AL502 |
|  | Product Category: | N/A | Test Condition (AC Input) |  |
|  | AC Mains Rating | 220V, 50Hz, 1  | Voltage | 231.1V |
|  | Temperature: | 25.0 ℃ | Current | 0.687A |
|  | Relative Humidity: | 55.0 ％ RH | Watts | 69.69W |
|  | Test Result | Pass | Frequency | 49.999Hz |
|  | Special Note: | N/A | Power Factor | 0.439 |

| Datas Measured |  |  |  |
| --- | --- | --- | --- |
| Test Item | Std Limits | Test Reading | Test Result |
| Relative st-st Voltage Change (dc) | 3 ％ | 0.01 ％ | PASS |
| Max. Relative Voltage Change(dmax) | 4 ％ | 2.29 ％ | PASS |
| Duration＞3％ dV(d(t) for＞200ms) | 0.2 Sec. | 0.05 ％ | PASS |
| Short Term Flicker (Pst) | 1.00 | 0.69 | PASS |
| Long Term Flicker (Plt) | 0.65 | N/A | N/A |
| Remark： According to the test condition specified in A.11 of Application of limits and type test conditions for specific equipment of Annex A, IEC 61000-3-3:94, only the measurement of dmax. is made. |  |  |  |

| Tech. Mgr.: |  | Test Engr.: |  |
| --- | --- | --- | --- |

Attachment

Table Contents

A. Measurement Instrument List

A-1 EMI Measuring Instrument List

A-2 EMS Measuring Instrument List

A-3 Harmonics/Flickers Measuring Instrument

B. Electric Block Diagram

C. EUT Modification Description

D. Labeling /User’s Manual Requirement

E. Example of Declaration of Conformity

F. Laboratory Accreditation Certificate

G. EUT Photos Exhibition

Attachment - A-1

EMI Measuring Instrument List

| Item | Instruments | Mfr/Brand | Model/Type No. | Serial No. | Calibrated Date | Next Cali. Date | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | LISN | EMCO | 3825/2 | 9605-2539 | 2001-06-22 | 2002-06-21 |  |
| 2 | LISN | Rolf Heine | NNB-2/16Z | 98083 | 2001-10-20 | 2002-10-19 |  |
| 3 | LISN | Rolf Heine | NNB-2/16Z | 98053 | 2001-11-22 | 2002-11-21 |  |
| 4 | Pulse Limiter | Electro-Metrics | EM-7600 | 112644 | 2001-02-09 | 2002-02-08 |  |
| 5 | 50Ω Terminator | N/A | N/A | N/A | 2001-05-21 | 2002-05-20 |  |
| 6 | Test Cable | N/A | C01 | N/A | 2001-12-08 | 2002-12-07 |  |
| 7 | Log-Bicon Antenna | MESS-ELEKTRONIK | VULB 9160 | 3058 | 2001-10-27 | 2002-10-26 |  |
| 8 | Log-Bicon Antenna | MESS-ELEKTRONIK | VULB 9160 | 3060 | 2001-10-20 | 2002-10-19 |  |
| 9 | Log-Bicon Antenna | MESS-ELEKTRONIK | VULB 9161 | 4022 | 2001-07-04 | 2002-07-03 |  |
| 10 | Test Cable | N/A | 10M_OS01 | N/A | 2001-12-08 | 2002-12-07 |  |
| 11 | Test Cable | N/A | OS01-1/-2 | N/A | 2001-12-08 | 2002-12-07 |  |
| 12 | Test Cable | N/A | 10M_OS02 | N/A | 2001-12-08 | 2002-12-07 |  |
| 13 | Test Cable | N/A | OS02-1/-2/-3 | N/A | 2001-12-08 | 2002-12-07 |  |
| 14 | RF Switch | Anritsu | MP59B | M65982 | 2001-12-10 | 2002-12-09 |  |
| 15 | Quasi-Peak Adapter | HP | 85650A | 2521A00844 | 2001-09-24 | 2002-03-23 |  |
| 16 | RF Pre-Selector | HP | 85685A | 2648A00417 | 2001-09-24 | 2002-03-23 |  |
| 17 | Spectrum Analyzer | HP | 85680B | 2634A03025 | 2001-09-24 | 2002-03-23 |  |
| 18 | Spectrum Monitor | HP | 85662B | 2648A13616 | 2001-09-24 | 2002-03-23 |  |
| 19 | Pre-Amplifier | Anritsu | MH648A | M09961 | 2001-12-10 | 2002-12-09 |  |
| 20 | Spectrum Analyzer | ADVAN TEST | R3261C | 81720298 | 2001-08-17 | 2002-08-16 |  |
| 21 | Test Receiver | R&S | ESH3 | 860156/018 | 2001-10-23 | 2002-10-22 |  |
| 22 | Test Receiver | R&S | ESVP | 860687/009 | 2001-10-23 | 2002-10-22 |  |
| 23 | Test Receiver | MEB | SMV41 | 130 | 2001-12-05 | 2002-12-04 |  |
| 24 | Test Receiver | PMM | PMM 9000 | 4310J01002 | 2001-12-31 | 2002-12-30 |  |
| 25 | Horn Antenna | EMCO | 3115 | 9605-4803 | 2001-05-09 | 2002-05-08 |  |
| 26 | Test Receiver | R&S | ESMI | 843977/005 | 2001-11-14 | 2002-11-05 |  |
| 27 | Pre-Amplifier | R&S | ESMI-Z7 | 1045.5020 | 2001-05-21 | 2002-05-20 |  |
| 28 | Absorbing Clamp | R&S | MDS-21 | 841077/011 | 2001-08-18 | 2002-08-17 |  |
| 29 | Voltage Probe | R&S | ESH2-Z3 | 841.800/023 | 2001-08-20 | 2002-08-19 |  |
| 30 | Signal Generator | HP | 8648A | 3426A01034 | 2000-02-10 | 2002-02-09 |  |
| 31 | Antenna Mast | Chance Most | CMTB-1.5 | N/A | N/A | N/A |  |
| 32 | Turn Table | Chance Most | CMTB-1.5 | N/A | N/A | N/A |  |

Remark：

(1)   indicates the instrument used in this test report。

(2)  N/B denotes No Brand measurement facility。

Attachment - A-2

EMS Measuring Instrument List

| Item | Instruments | Mfr/Brand | Model/Type No. | Serial No. | Calibrated Date | Next Cali.Date | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | ESD Simulator | Schaffner | NSG 435 | ESD-001 | 2001-12-08 | 2002-12-07 |  |
| 2 | Signal Generator | IFR | 2023A | 202301/368 | 2001-03-25 | 2002-03-24 |  |
| 3 | Power Amplifier(RS) | M2S | AC8113-800/250A | 9904-113 | 2001-03-26 | 2002-03-25 |  |
| 4 | Antenna(500W) | MESS-ELEKTRONIK | VULB9161 | 4022 | 2001-07-04 | 2002-07-03 |  |
| 5 | EFT Burst Tester | Haefely | PEFT-Junior | 083 180-24 | 2001-12-05 | 2002-12-04 |  |
| 6 | Surge Tester | Haefely | PSURGE 4-1 | 083 665-01 | 2001-12-03 | 2002-12-02 |  |
| 7 | Power Amplifier(CS) | M2S | A0122-250 | 9902-111 | 2001-03-26 | 2002-03-25 |  |
| 8 | CDN | MEB | M3 | 13389 | 2001-05-22 | 2002-05-21 |  |
| 9 | CDN | MEB | M2 | 12127 | 2001-05-22 | 2002-05-21 |  |
| 10 | CDN | MEB | S1 | 14393 | 2001-05-22 | 2002-05-21 |  |
| 11 | CDN | MEB | S25 | 12426 | 2001-05-22 | 2002-05-21 |  |
| 12 | EM Clamp | MEB | KEMZ 801 | 14291 | 2001-05-22 | 2002-05-21 |  |
| 13 | Magnetic Field Tester | Haefely | MAG 100.1 | 083858-08 | 2001-05-24 | 2002-05-23 |  |
| 14 | DIP Generator | Haefely | PLINE 1610 | 083690-16 | 2001-12-05 | 2002-12-04 |  |
| 15 | Power Analyzer | Chroma | 6630 | 66300000120 | 2001-12-03 | 2002-12-02 |  |
| 16 | AC Source | Chroma | 6530 | 65300113 | 2001-12-04 | 2002-12-03 |  |

Remark: * N/A - denotes tests is not applicable in this Test Report

Attachment - A-3

Test Instrument for Harmonics/Flicker Measurements

Power Analyzer Model 6630

| Harmonics Analyzer |  | Flicker Measurements |
| --- | --- | --- |
| Compliance measurements according to IEC555-2 and EN 61000-3-2 |  | Compliance measurements according to IEC555-3 and EN 61000-3-3 |
| Testing: Preprogrammed limits according to standard for Pass /Fail testing. |  | Testing: Preprogrammed limits according to standard for Pass/Fail testing. |
| Method of Analysis: DFT& DSP technology for steady state and fluctuating harmonics |  | Method of Analysis: Implementation of IEC 868/EN60868 Flicker-meter Specification |
| Measurement: Harmonic order: 1 - 40 Freq. of fundamental: 40 - 70 Hz Freq. Source: Voltage or Current Data Source: Voltage or Current AD conversion: 18 bit resolution Measurement window: Rectangular Analyzed periods: 1 - 47 |  | Measurement: 1024 classify scales for flicker level 100/120Hz sample rate for 50/60 Hz fundamental Range Pst : 0.1 - 20 Plt: 0.1 - 20 d : 0 - 25 % |
| Synchronization uncertainty: 0.01 % Measurement time: 3.5s analyzing 6 periods Uncertainty: +/-(0.5% of rdg + 0.03% of range) |  | Freq. Range : 40 - 70 Hz Reference Impedance R + jX simulated in calculation, range of R and X 0.01 - 70 ohm Measurement Time: 1 - 15 min. No. of Measurements: 1-1100 Uncertainty: Pst 4 % of rdg for 0.5 < Pst < 20 Plt 4 % of rdg for 0.5 < Pst < 20 d: 2 % of rdg for dmax. > 0.1 % |
| Class D Waveform Detector for EN 61000-3-2 |  |  |
| Compliance Class D rectangular envelope detecting Display: Simultaneous presentation of V and I waveforms Measurement: Freq. Range 40 - 70 Hz fundamental, Frequency Source Voltage or Current Measurement Mode AC+DC |  |  |

Attachment - A-3

Test Instrument for Harmonics/Flicker Measurements

Power Analyzer Model 6630

| Technical Specifications of Power |  |  |
| --- | --- | --- |
| Output Ratings |  | Input Ratings |
| Phase: Single |  | Voltage Range: 190-254V, single Ph. Frequency Range: 47 - 63 Hz |
| Power: 3000VA |  | Current: 23 A Max. PF: 0.98 Min. under full load |
| Voltage: Range 150V/300V Auto |  | Measurements |
| Accuracy 0.5 % of F. S(15-45 Hz) |  |  |
| 0.2% of F. S(>45Hz-2KHz) |  | Voltage: |
| Resolution 0.1 V |  | Range 0 - 150V/ 0 - 300V Accuracy(RMS) 0.1% F.S + 0.25 % |
|  |  | Resolution 0.1 V |
| Distortion 1% (15 Hz - 45 Hz) |  |  |
| 0.5%(>45Hz - 500Hz) |  | Current: |
| 1 % (>500 Hz - 1KHz) |  | Range(peak) 0-140A |
| 2 % (> 1Khz - 2 KHz) |  | Accurnacy |
|  |  | (RMS) 0.1 % F.S. + 0.4 % |
| Line Regulation 0.1 % Load Regulation 0.1 % Temp. Coefficient 0.2 % per C drg. Max. Currents: RMS 30A/15A(150V/300V range) Peak 90A/45A/(15Hz-100Hz) 75A/38A(>100Hz-1KHz) 60A/30A(>1KHz - 2 Khz) Frequency: Range 15Hz - 2 Khz Accuracy 0.15 % Resolution 0.01 Hz (15 - 99.99 Hz) 0.1 Hz (100 - 999.9 Hz) 0.2 Hz (1 Khz - 2 KHz) |  | (Peak) 0.2 % F. S. + 0.4 % Resolution 0.01A Power: Accurancy 1 % F.S. (CF<6) Resolution 0.01W Frequency: Range 15 - 2000 Hz Accuracy 0.1 % + 2 count. Resolution 0.01 Hz Others Efficiency 80 % typical Protection OPP, OLP, OPT, Fan, Fail |

Attachment - B.

Electric Block Diagram

Attachment - C.

EUT Modification Description

(Intentionally Left Blank)

Attachment - D.

Information to the User

Shall be given in the

User's Manual and Label

Important Notice to the Applicant

重要訊息提示，請務必詳加閱讀

- CE-Mark 標示 (CE-Marking Requirement)

- 為符合相關自我宣告(DOC)要求，將來 貴公司的產品在出廠前一定一個明顯、易讀、可讀之CE Mark識別標示。

- 這個CE Mark識別標示可為貼上去之標簽(或稱銘牌)、或直接印製於產品上。

- 如果 貴公司的產品要標示CE Mark，必需確認是否已同時符合EMC及LVD或其它相關指令？否則不可標示CE Mark。若您不清楚 貴公司的產品是否已符合EMC或LVD或其它相關指令？歡迎電話詢問。

- 這個CE Mark標示的位置，可為下列之一:

- 產品本體上

- 包裝上

- 說明書/使用手冊上

- 保證書上

- 有關此產品識別標示之詳細要求及規定，請參考下頁的提示。

- 說明書(Information to the User shall be given in the Manual)

- 請將 貴公司的產品使用說明書/使用手冊(User's Manual)附於本報告之後，成為TCF之一部份。

- 請確認 貴公司的產品使用說明書內所述及之產品規格、特性與本報告內之內容相符合。

- 為符合相關自我宣告(DOC)之要求，上述這個產品的User's Manual至少應包含有本報告內所述及之Model No.。

CE Marking Requirements

- All apparatus covered by the Directive (89/336/EEC) in accordance with the protection requirements and accompanied by one of the means of certification provided for in Article 10 must bear the CE conformity marking as following.

- The CE conformity marking is affixed by the manufacturer or his authorized representative established within the Community on the device in a conspicuous location or else to the packing, instructions for user or guarantee certificate when the device is so small or for such use that it is not practicable to place the CE conformity marking on it.

3. The CE conformity marking should be in a size of type large enough to be visibly, legibly, and indelibly consistent with the dimensions of the equipment. However, the type size is required to be larger than 5 mm.

4. Where the product is covered by other Directives providing for the CE conformity marking , application of the CE marking also indicates that the apparatus conforms to the provisions of the other directives applicable to it.

5. It is prohibited to affix any marks or inscriptions that are likely to mislead third parties as to the verbal or pictorial significance of the CE conformity marking.

Attachment - E.

Declaration of Conformity

Important Notice to the Applicant

重要訊息提示，請務必詳加閱讀

- 為符合自我宣告(DOC)之要求，將來 貴公司的產品在出廠前一定要有一個宣告書以宣告 貴公司的產品可以符合CE 相關指令(Directive)。

- 假如您的客戶(Buyer/Importer)已備有他們自己的宣告書，請用他們的宣告書完成宣告。否則您可以參考本公司為您準備的宣告書範本來完成必要的宣告。(請參考下頁所附的內容提示)

- 若 貴公司的宣告書要與產品使用說明書/手冊(User's Manual)放在一起，成為User's Manual之一部份也可以。

- 為符合相關自我宣告(DOC)之要求，這個宣告書內的產品Model No.必需與本報告內所述及之Model No.一致。

Company Letter Head

| CE Declaration of Conformity |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| For the following equipment: |  |  |  |  |  |
| Product Name |  |  |  |  |  |
| Model Designation |  |  |  |  |  |
| Is herewith confirmed to comply with the requirements set put in the Council Directive on the Approximation of the Lows of the Member States relating to Electromagnetic Compatibility Directive (89/336/EEC) and Low-Voltage Directive(73/23/EEC). For the evaluation regarding the Directives, the following standards were applied: |  |  |  |  |  |
| The following importer/manufacturer are responsible for this declaration: |  |  |  |  |  |
|  | (Company Name , Importer) |  |  | (Company Name , Manufacturer) |  |
|  | (Person Responsible , Importer) |  |  | (Person Responsible , Manufacturer) |  |
|  | (Position/Title , Importer) |  |  | (Position/Title , Manufacturer) |  |
|  | (Legal Signature , Importer) |  |  | (Legal Signature , Manufacturer) |  |
|  | (Place , Importer) |  |  | (Place , Manufacturer) |  |
|  | (Date) |  |  | (Date) |  |

Attachment - F.

Laboratory Accreditation Certificate

Attachment - G.

EUT Photos Exhibition

1.  Photo # 1  Front View / Rear View

2.  Photo # 2-9  Unit Partially Disassembled