YAMAHA Corporation Jan. 29th, 1997

OPL3-SAx driver technical document

How to change default values of mixer

YAMAHA Corporation

System Development Dept.

Default values of mixer can be changed without compiling  in this OPL3-SAx driver. This document describes how we can do that. Following the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA32.DLL” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string table. There are various error messages and names in the string table. Each string has ID number. Mixer default values are from ID 321 to ID 347 and from ID 370 to 387. The following table shows the connections between each ID number and mixer setting.

| ID number | mixer setting | Value |
| --- | --- | --- |
| 370 | Left Master Volume | 0  65535 |
| 371 | Right Master Volume | 0  65535 |
| 372 | Left Line Volume | 0  65535 |
| 373 | Right Line Volume | 0  65535 |
| 374 | Left Wave Volume | 0  65535 |
| 375 | Right Wave Volume | 0  65535 |
| 376 | Left Synth Volume | 0  65535 |
| 377 | Right Synth Volume | 0  65535 |
| 378 | Left CD Volume | 0  65535 |
| 379 | Right CD Volume | 0  65535 |
| 380 | Mic Output Volume | 0  65535 |
| 381 | Mono In Volume | 0  65535 |
| 382 | Left 3D Wide Volume | 0  65535 |
| 383 | Right 3D Wide Volume | 0  65535 |
| 384 | 3D Enhanced Wide Mute | 0 (not mute), 1(mute) |
| 385 | 3D Enhanced Bass | 0  65535 |
| 386 | 3D Enhanced Treble | 0  65535 |
| 387 | 3D Enhanced Wide Enable | 0 (disable), 1(enable) |
| 321 | Recording Monitor Volume | 0  65535 |
| 322 | Left Line Recording Volume | 0  65535 |
| 323 | Right Line Recording Volume | 0  65535 |
| 324 | Mic Recording Volume | 0  65535 |
| 325 | Left CD Recording Volume | 0  65535 |
| 326 | Right CD Recording Volume | 0  65535 |
| 327 | Left Loop Recording Volume | 0  65535 |
| 328 | Right Loop Recording Volume | 0  65535 |
| 329 | Left Line Voice Volume | 0  65535 |
| 330 | Right Line Voice Volume | 0  65535 |
| 331 | Mic Voice Volume | 0  65535 |
| 332 | Left CD Voice Volume | 0  65535 |
| 333 | Right CD Voice Volume | 0  65535 |
| 334 | Left Loop Voice Volume | 0  65535 |
| 335 | Right Loop Voice Volume | 0  65535 |
| 336 | Master Mute | 0 (not mute), 1(mute) |
| 337 | Line Mute | 0 (not mute), 1(mute) |
| 338 | Wave Mute | 0 (not mute), 1(mute) |
| 339 | Synth Mute | 0 (not mute), 1(mute) |
| 340 | CD Mute | 0 (not mute), 1(mute) |
| 341 | Mic Output Mute | 0 (not mute), 1(mute) |
| 342 | Mono In Mute | 0 (not mute), 1(mute) |
| 343 | Recording Monitor Mute | 0 (not mute), 1(mute) |
| 344 | Recording Source Select | Aux , LineIn , MIC , Feedback |
| 345 | Voice Source Select | Aux , LineIn , MIC , Feedback |
| 346 | ZV Port Select | 0 (ZV disable), 1(ZV enable) |
| 347 | Mic +20dB | 0 (normal), 1(+20dB) |

N.B.: If OPL3-SA3 driver, you can use all ID.

However, if OPL3-SA2, Left 3D Wide Volume(ID 382), Right 3D Wide Volume(ID 383), 3D Enhanced Wide Mute(ID 384), 3D Enhanced Bass(ID 385), 3D Enhanced Treble(ID 386), 3D Enhanced Wide(ID 387) can not be used.

And if OPL3-SA driver, Mono In Volume(ID 381), Mono In Mute (ID 342) and ZV Port Select (ID 346) can not be also used.

4. Change values and save. You will get a modified driver with new default settings.

Additional, you must use suitable language resource file for operation system when you want to set value of mixer custom.