YAMAHA Corporation Jan. 29th, 1997

OPL3-SAx driver technical document

How to change maximum levels of volumes

YAMAHA Corporation

System Development Dept.

Maximum levels of volumes can be changed without compiling in this driver. This document describes how we can do that. Follow  the instructions as below.

1. Execute resource editor. Generally resource editor is included in the software development package. (ex. Developer Studio in Visual C++ 4.0,  App Studio in Visual C++ 1.xx,  Resource Workshop in Borland C++)

2. Open “OPL3SA32.DLL” . We can edit resources (such as dialog box, bitmap and string table) included in the EXE or DLL using resource editor.

3. See string  table. There are various error messages and names in there. Each string has ID number. Maximum levels of volumes are from ID 390 through ID 405. The following table shows the connections between each ID number and maximum level setting. Maximum value means maximum level which can be set to the hardware.

4. Change values and save. You will get a modified driver with new maximum level  settings.

| ID number | mixer setting | Value |
| --- | --- | --- |
| 390 | Output Master Volume | 0  63 |
| 391 | Line Output Volume | 0  31 |
| 392 | Wave Output Volume | Not implemented yet |
| 393 | Synth Output Volume | 0  31 |
| 394 | CD Volume | 0  31 |
| 395 | Mic Output Volume | 0  31 |
| 396 | Mono In Volume | 0  15 |
| 397 | Recording Monitor Volume | 0  63 |
| 398 | Line Recording Volume | 0  15 |
| 399 | Mic Recording Volume | 0  15 |
| 400 | CD Recording Volume | 0  15 |
| 401 | Loop Recording Volume | 0  15 |
| 402 | Line Voice Volume | 0  15 |
| 403 | Mic Voice Volume | 0  15 |
| 404 | CD Voice Volume | 0  15 |
| 405 | Loop Voice Volume | 0  15 |

- ID 390 Output Master Volume

| Value | MAX Level |
| --- | --- |
| 6360 | 0dB |
| 5956 | -2dB |
| 5552 | -4dB |
| 5148 | -6dB |
| 4744 | -8dB |
| 4340 | -10dB |
| 3936 | -12dB |
| 3532 | -14dB |
| 3128 | -16dB |
| 2724 | -18dB |
| 2320 | -20dB |
| 1916 | -22dB |
| 1512 | -24dB |
| 118 | -26dB |
| 74 | -28dB |
| 30 | -30dB |

- ID 391Line VolumeID393Synth Output VolumeID 394CD VolumeID395Mic Output Volume

| Value | MAX Level |
| --- | --- |
| 31 | 12.0dB |
| 30 | 10.5dB |
| 29 | 9.0dB |
| 28 | 7.5dB |
| 27 | 6.0dB |
| 26 | 4.5dB |
| 25 | 3.0dB |
| 24 | 1.5dB |
|  | ¥¥¥ |
| 6 | -25.5dB |
| 5 | -27.0dB |
| 4 | -28.5dB |
| 3 | -30.0dB |
| 2 | -31.5dB |
| 1 | -33.0dB |
| 0 | -34.5dB |

ID 396Mono In Volume

| Value | MAX Level |
| --- | --- |
| 15 | 0dB |
| 14 | -3.0dB |
| 13 | -6.0dB |
| 12 | -9.0dB |
| 11 | -12.0dB |
| 10 | -15.0dB |
| 9 | -18.0dB |
| 8 | -21.0dB |
| 7 | -24.0dB |
| 6 | -27.0dB |
| 5 | -30.0dB |
| 4 | -33.0dB |
| 3 | -36.0dB |
| 2 | -39.0dB |
| 1 | -42.0dB |
| 0 | -45.0dB |

- ID398Line Recording VolumeID 399Mic Recording VolumeID 400(CD Recording Volume)ID401(Loop Recording VolumeID 402Line Voice VolumeID403(Mic Voice Volume)ID404CD Voice VolumeID 405Loop Voice Volume

| Value | MAX Level |
| --- | --- |
| 15 | 22.5dB |
| 14 | 21.0dB |
| 13 | 19.5dB |
| 12 | 18.0dB |
| 11 | 16.5dB |
| 10 | 15.0dB |
| 9 | 13.5dB |
| 8 | 12.0dB |
| 7 | 10.5dB |
| 6 | 9.0dB |
| 5 | 7.5dB |
| 4 | 6.0dB |
| 3 | 4.5dB |
| 2 | 3.0dB |
| 1 | 1.5dB |
| 0 | 0.0dB |

- ID 397Recording Monitor Volume

| Value | MAX Level |
| --- | --- |
| 63 | 0dB |
| 62 | -1.5dB |
| 61 | -3.0dB |
| 60 | -4.5dB |
| 59 | -6.0dB |
| 58 | -7.5dB |
| 57 | -9.0dB |
| 56 | -10.5dB |
| ¥¥¥ | ¥¥¥ |
| 6 | -85.5dB |
| 5 | -87.0dB |
| 4 | -88.5dB |
| 3 | -90.0dB |
| 2 | -91.5dB |
| 1 | -93.0dB |
| 0 | -94.5dB |