DCL56 & CL53’s

InsydeFlash

And

ROMPack

Document

Insyde Software Corp.

6F, No. 137, Sec. 2, Chien Kuo N. Rd.

Taipei, Taiwan, R.O.C.

Tel: 886-2-2506-1289  Fax: 886-2-2506-3001

Table of Contents

Overview	3

System Requirement	3

Installing InsydeFlash and RomPack Utility	3

Configuration	3

Using InsydeFlash and RomPack Utility	5

Overview

InsydeFlash utility is a ROM flash application under windows 2000/XP. The user can load the updated ROM image and update it though the UI of this application. However, the updating operating should be as easy as possible for most end users. In order to achieve that, InsydeFlash ROMPack will pack the flash utility with platform.ini and related drivers together within one executable file for Compal. Compal’s people will need to pack the executable file with the target BIOS image into a compressed file by themselves.

# System Requirement

- Platform: Compal DCL56 & CL53 laptop.

- Operating System: Windows 2000/XP only.

- Support Flash part: SST 39VF040 only.

- Supported language: English version only.

Installing InsydeFlash and RomPack Utility

- InsydeFlash and RomPack utility doesn’t need to install, user can execute it directly. Before use it user must configure platform.ini file.

# Configuration

Platform.ini is a configuration file. It configures utility version display, platform check switch, ac adapter check switch, ec and system boot block configuration switch. It includes:

- [Version]

- Version=00

- Winflash utility will Display the value at top

- [Platform_Check]

- flag=1

- This flag is the switch of comparing bios project id with rom file project id. When flag is 1, utility will check project id. If id doesn't match, utility will close the application. When flag is 0, utility won't check project id.

- [AC_Adapter]

- flag=0

- This flag is the switch of checking AC. When flag is 1, utility will check AC. If no AC plug in, utility will display warning message. When flag is 0, utility won't check AC.

- [Bios_Version_Check]

- flag=1

- This flag is the switch of checking Bios Version. When flag is 1, utility will check Bios Version. If rom file Version is older than Bios, utility will display warning

- message and close application. When flag is 0, utility won't check rom file version.

- [BootBlock]

- EC_flag=1

- SYSTEM_flag=1

- EC_flag and SYSTEM_flag are the switches of controlling boot block protection. When EC_flag is 1, EC boot block will be protected. When EC_flag is 0, EC boot block will be programmed. When SYSTEM_flag is 1, SYSTEM boot block will be protected. When SYSTEM_flag is 0, SYSTEM boot block will be programmed.

- [4M_Flash_Configuration]

- EC_ ENE910_BOOTBLOCK_Address_Start=0

- EC_ ENE910_BOOTBLOCK_Address_End=24

- EC_ NS591_BOOTBLOCK_Address_Start=0

- EC_ NS591_BOOTBLOCK_Address_End=16

- SYSTEM_BOOTBLOCK_Address_Start=480

## SYSTEM_BOOTBLOCK_Address_End=512

- The size of SST39VF040 is 512K bytes. EC boot block range is from 0 to 16 or 0 to 24 which depends on different platform. SYSTEM boot block range is from 480 to 512. Winflash utility will use these boot block configuration to decide the location of boot block protection and check that the location of rom file isn't all 0x00 or 0xff.

Using InsydeFlash and RomPack Utility

- Setup platform.ini file.

- Use RomPack Utility to pack InsydeFlash utility. Source directory points to InsydeFlash utility’s directory. Destination directory points to an executable file. RomPack utility will pack all InsydeFlash utility’s files to an executable file.

- Put rom file and executable file in the same directory.

- Run this executable file. It will extract InsydeFlash Utility to c:\program files\inside and auto run insydeflash.exe to load rom file and flash bios.

- After flash bios successfully, system will auto reboot.

- Fixed issue:

- Rompack will pack insydeflash utility and configures the extraction path in c:\windows\temp.

- Add check EC and System Boot Block function.