Yamaha LSI Corporation.

Jan/09/1998

Release Note for Yamaha OPL3-SAx device driver.

Operation System: Win95

Driver Information: Ver4.03.8335

Different from previous version ver4.03.8329:

1. Problem: Play wave file after hibernation the wave speed became faster, when use

speed CPU such as MMX233~266MHz.

Sol: Previous driver didn't consider when so high speed CPU being used, so the

initiate time for YMF715x is too short when resume the system. Add a clock

detector to monitor the CPU speed, then the initiate time can meet with

different CPU speed.

Bug:

No known bug.