Intel SpeedSteptm Technology Applet

v2.3

Installation Guide

Intel Confidential

DO NOT DUPLICATE

Rev 1.00

April 1, 2002

WARNING:	4

1	Introduction:	4

1.1	New Features	5

1.2	Known Issues	8

2	Installation:	10

2.1	Installing for the first time	11

2.2	Over-installing	11

2.3	Uninstalling the Applet:	11

2.4	Installing on a non- Intel SpeedStep technology capable system:	12

2.5	Installing on Windows XP:	12

3	Operation:	12

3.1	Transition Control:	12

3.2	Selection of the EventRegister Port:	12

3.3	Expected Operation:	13

4	External Intel SpeedStep technology Interfaces:	17

4.1	PRPC_UpdateSettings External Interface	18

4.2	PRPC_QuerySMIVersion External Interface	18

5	Registry Keys:	19

5.1	AppletTimeout	20

5.2	DBSPeriod	21

5.3	DBSThreshold	21

5.4	EventRegister	22

5.5	ExitCode	22

5.6	Flags	23

5.7	MBPeriod	24

5.8	MBPoll	24

5.9	MBPrefs	24

5.10	MBThreshold	25

5.11	PollPeriod	26

5.12	SetStateRetry	26

5.13	SystemPrefs	26

5.14	UserPrefs	27

5.15	W2KTimer	28

5.16	AppExitCode	28

This Intel® SpeedStep™ technology Applet Installation Guide as well as the software described in it is furnished under license and may only be used or copied in accordance with the terms of the license. The information in this manual is furnished for informational use only, is subject to change without notice, and should not be construed as a commitment by Intel Corporation. Intel Corporation assumes no responsibility or liability for any errors or inaccuracies that may appear in this document or any software that may be provided in association with this document.

Except as permitted by such license, no part of this document may be reproduced, stored in a retrieval system, or transmitted in any form or by any means without the express written consent of Intel Corporation.

## WARNING:

You are authorized and licensed to install this application and driver ONLY on a Licensed PC that has System Management Mode (SMM) BIOS support installed.  This driver may permanently damage any system that does not meet these requirements. A Licensed PC is a computer which (1) is capable of seamlessly and automatically transitioning among multiple performance states (potentially operating at different efficiency ratings) based upon power source changes, end user preference, processor performance demand, and thermal conditions; and (2.) includes an Intel mobile Pentium II processor, Intel mobile Pentium III processor, Intel mobile Pentium 4 processor, or any other future Intel processors that incorporates the capability to transition between different performance states by altering some or any combination of the following processor attributes: core voltage, core frequency, bus frequency, number of processor cores available, or any other attribute that changes the efficiency (instructions/unit time-power) at which the processor operates.

## Introduction:

The Intel® SpeedStep™ technology (IST) Applet consists of a user-mode user interface and kernel-mode driver supplied to OEMs for installation.  The Intel SpeedStep technology Applet is installed using the supplied InstallShield setup program.  Please refer to the Installing for the first time: section for details on installing the Applet in different languages.

All installation files are delivered in password-encrypted .zip files.  One contains the OEM installation files and the others include the language specific end-user installation files.

This file, the Intel SpeedStep technology Applet Installation Guide (Readme.doc), is available to OEMs from your Intel field service representative.

Once installed, the complete Intel SpeedStep technology Applet installation includes the following files:

- Prpcui.exe – User interface executable

- Prpcui.dll – User interface support library

- Prpclang.dll – User interface language resource library (unique for each language)

- Prpcui.hlp – Context-sensitive help file (unique for each language)

- Prpc.sys – Kernel-mode driver for Windows 2000, Windows 98 SE, Windows Millennium Edition (ME) and Windows NT 4.0.

- Prpc.vxd – Kernel-mode driver for Windows 95.

- Prpc.cat – Signature file assigned by Microsoft WHQL group (Installed on all Windows 98, Windows ME and Windows 2000 installations)

- Prpc.inf – Setup file for associating the signature file with the program.

### New Features

#### Features of the v2.3 release

The Intel SpeedStep technology Applet v2.3 release features the following:

- The v2.3 release is logo’d for Windows 2000, Windows 98 SE, Windows ME and Windows NT 4.0.  Signature files are provided for all logo’d operating systems.

- The v2.3 release fixes the issue where the system was getting set to the incorrect performance state upon resume from suspend/hibernate after processing the system resume suspend message.

- The v2.3 release also fixes the issue where both Automatic and Max Battery modes (DBS) would be disabled upon resume from suspend/hibernate under Windows 98SE and Windows ME operating systems and on Windows 2000 operating system when Timer/TSC mechanism is used to calculate CPU utilization. This method is used by the applet only if the OS API's used to compute utilization fail or if the registry entry "W2KTimer" is set to 0 (zero).

- The v2.3 release also fixes the issue that when PRPC_UpdateSettings() external interface is called, both Automatic and Max Battery modes (DBS) would get disabled under Windows 98SE and Windows ME operating systems and on Windows 2000 operating system when Timer/TSC mechanism is used to calculate CPU utilization. This method is used by the applet only if the OS API's used to compute utilization fail or if the registry entry "W2KTimer" is set to 0 (zero).

#### Features of the v2.2a release

The Intel SpeedStep technology Applet v2.2a release adds the following fix:

- The v2.2a fixes the language override feature that did not work with the v2.2 release. Please refer to the Language override description under section 2.1 “Installing for the first time” for updated instructions for use of this feature.

#### Features of the v2.2 release

The Intel SpeedStep technology Applet v2.2 release features the following:

- The v2.2 release is logo’d for Windows 2000, Windows 98 SE, Windows ME and Windows NT 4.0.  Signature files are provided for all logo’d operating systems.

- The Applet v2.2 was specifically developed to support Enhanced Speedstep™ (Demand Based Switching) on Windows 98 and ME operating systems for the Pentium® 4 family of processors. The current Intel® SpeedStep™ technology (IST) Applet v2.1 uses a method of computing processor utilization that would not work for Pentium® 4 family of processors under Windows 98 and ME.

- The v2.2 release has added code to request transition ownership when processing the system resume from suspend message along with the system critical resume message.  When a critical hibernate occurs, the Applet does not receive any system level notification.  It therefore does not transfer transition control ownership to the BIOS.  Upon resume, the Applet assumed that ownership still resided with the Applet.  Some OS/BIOS implementations are unable to support this behavior.  The change forces the Applet to re-acquire transition ownership upon resume from a critical hibernate whether the OS broadcasts the PBT_APMRESUMESUSPEND or PBT_APMRESUMECRITICAL message.

- The v2.2 release does not allow disabling Automatic mode in the “Disable Modes” Flags registry value. The reasoning behind this is that if one disables Automatic mode, he/she would also want to disable the Maximum performance mode. This can be achieved independently for AC and Battery using the following registry entries.

Flags[11:8] - AC power resource - Max performance level supported

0 = No AC power selection is supported

1 = Only Battery Optimized Performance and Maximum Battery are supported

2 = All modes are supported

Flags[3:0] - Battery power resource - Max performance level supported

0 = No Battery power selection is supported

1 = Only Battery Optimized Performance and Maximum Battery are supported

2 = All modes are supported

So, Setting Flags[19:16] = 2 is not allowed anymore.

- The v2.2 release fixes an issue when the user comes back from suspend in automatic state. The current version string in the IST dialog box used to show BOP instead of AUTOMATIC.

#### Features of the v2.1 release

The Intel SpeedStep technology Applet v2.1 release features the following:

- The v2.1 release is logo’d for Windows 2000, Windows 98, Windows 98 SE and Windows ME.  Signature files are provided for all logo’ed operating systems.

- The v2.1 release adds an external interface that returns the SMI interface revision mode under which the Applet is operating.  This is the result of a customer request.  Please see the PRPC_QuerySMIVersion section for details.

- The v2.1 release adds the ability to configure the SystemPrefs registry value to prevent the “(Recommended)” string from appearing after any of the choices in the user interface drop-down selection boxes.  This is the result of a customer request.  Please see the SystemPrefs section for details.

- The v2.1 release adds enhanced error handling to the Transition Control SMI command when processing a system suspend message.  When the Applet receives a PBT_APMSUSPEND message, it releases ownership of transitions back to the BIOS prior to suspend.  We found that some BIOS implementations may fail this call.  If the call failed, the Applet assumed that ownership of transitions remained with the Applet.  This however was not always true.  The change forces the Applet to re-acquire transition ownership upon resume.

- The v2.1 release adds code to request transition ownership when processing the system critical resume message.  When a critical hibernate occurs, the Applet does not receive any system level notification.  It therefore does not transfer transition control ownership to the BIOS.  Upon resume, the Applet assumed that ownership still resided with the Applet.  Some OS/BIOS implementations are unable to support this behavior.  The change forces the Applet to re-acquire transition ownership upon resume from a critical hibernate.

- The v2.1 release fixes a user interface bug.  If the Flags[3:0] registry value was less than two and the user selected “Maximum Battery” for their “Running on batteries” preference and closed the user interface dialog, the Applet v2.0 release would set the preference to “Battery Optimized Mode” when the user reopened the user interface dialog box.  This behavior only occurs if the Flags[3:0] registry value is less than two.

- The v2.1 release fixes an unintended behavior.  If the Flags[3:0] registry value was less than two.  When running on AC, if the user could open the tray icon’s pop-up menu and removed AC before making a menu selection, the user could still select “Automatic” mode.  The Applet would be placed into “Automatic” mode although BOP to MP transitions would be blocked.  The change disallows selection of this non-supported mode.

- The IST Applet v2.0 supports Windows 98, Windows Millennium Edition (ME) and Windows 2000 only.  Support for Windows 95 and Windows NT 4.0 required the IST Applet v1.3 and earlier versions.  Due to the added complexity of supporting multiple Applets for the variety of Windows operating systems available, the IST Applet v2.1 adds static transition support for Windows 95 and Windows NT 4.0.  Static transition support means that AC insertion/removal events trigger transitions and Maximum Performance and Battery Optimized Performance are the only supported modes on Windows 95 and Windows NT 4.0.  Due to the system requirements for Automatic and Maximum Battery modes, these modes are not supported on Windows 95 or Windows NT 4.0.  Under Windows 95 and Windows NT 4.0, the IST Applet v2.1 will look and feel just like the IST Applet v1.3 and earlier versions.

- The v2.1 InstallShield installer has been modified so that it does not cause and error if a handle is open to the prpcui.dll.  When a handle is open, the file is considered locked.  The installer program sets up the wininit.ini to replace the locked file on the next reboot.

#### Features of the v2.0 release

The Intel SpeedStep technology Applet v2.0 release features the following:

- The v2.0 release is logo’ed for Windows 2000, Windows 98, Windows 98 SE and Windows ME.  Signature files are provided for all logo’ed operating systems.

- The v2.0 release adds two additional operating modes: Automatic and Maximum Battery.  The Automatic mode switches between Maximum Performance and Battery Optimized modes based upon processor load.  The Maximum Battery mode is a configurable selection that can be used to increase battery life.

- The v2.0 release will behave much like previous Applet releases on systems that have older BIOS’s (SMI interface revision 0.0.9.0).

- The v2.0 release includes enhanced thermal cycling support.  Thermal cycling now supports number of states = 0.  When the Applet receives zero states available, it places the system into Battery Optimized Performance and applies throttling (if supported).  Please refer to the Thermal Cycling section for more information.

- Disable Enhanced Modes: The Applet has a new configuration parameter in the registry value Flags.  The Disable Modes setting allows the OEM to selectively suppress either or both of the Automatic or Maximum Battery modes.  If a mode is suppressed, the option does not appear in the user interface drop down selection or in the tray icon’s pop-up menu.  The Maximum Performance and Battery Optimized Performance modes cannot be disabled.  Please refer to the Flags registry value in the Registry Keys section for more information.

NOTICE: Most new features of the Intel SpeedStep technology Applet v2.x releases require BIOS changes.  If you have questions about these changes, please contact your local Intel Corporation representative.

### Known Issues

#### Known Issues with the v2.3 release

The Intel SpeedStep technology Applet has the following known issues that may or may not be considered features:

- Windows exception access violation when upgrading from version 2.1 on Windows NT4.0. This was an issue with the removal process in the previous version of the applet. This has been fixed in the v2.3 such that future versions will be able to perform the upgrade successfully. This issue could also potentially occur on other flavors of Windows. The resolution is to completely uninstall any previous version of the applet before installing version 2.3.

- “Cannot change performance” dialog appears occasionally after resuming from Critical Hibernate on Windows 2000. If the user clicks on okay, the applet will continue to function normally. There is an option to suppress the message box in the SetStateRetry Bits[31:28].

- IST flag icon might show disabled even after enabling and rebooting. This is an error but the icon will get fixed as soon as the first attempt to transition occurs. This issue has been seen intermittently on Windows NT4.0 only. It has not been seen on other version of Windows.

#### Known Issues with the v2.2a release

The Intel SpeedStep technology Applet has the following known issues that may or may not be considered features:

- Windows exception access violation when upgrading from version 2.1 on Windows NT4.0. This was an issue with the removal process in the previous version of the applet. This has been fixed in the v2.2 such that future versions will be able to perform the upgrade successfully. This issue could also potentially occur on other flavors of Windows. The resolution is to completely uninstall any previous version of the applet before installing version 2.2.

- “Cannot change performance” dialog appears occasionally after resuming from Critical Hibernate on Windows 2000. If the user clicks on okay, the applet will continue to function normally. There is an option to suppress the message box in the SetStateRetry Bits[31:28].

- IST flag icon might show disabled even after enabling and rebooting. This is an error but the icon will get fixed as soon as the first attempt to transition occurs. This issue has been seen intermittently on Windows NT4.0 only. It has not been seen on other version of Windows.

#### Known Issues with the v2.2 release

The Intel SpeedStep technology Applet has the following known issues that may or may not be considered features:

- Windows exception access violation when upgrading from version 2.1 on Windows NT4.0. This was an issue with the removal process in the previous version of the applet. This has been fixed in the v2.2 such that future versions will be able to perform the upgrade successfully. This issue could also potentially occur on other flavors of Windows. The resolution is to completely uninstall any previous version of the applet before installing version 2.2.

- “Cannot change performance” dialog appears occasionally after resuming from Critical Hibernate on Windows 2000. If the user clicks on okay, the applet will continue to function normally. There is an option to suppress the message box in the SetStateRetry Bits[31:28].

- IST flag icon might show disabled even after enabling and rebooting. This is an error but the icon will get fixed as soon as the first attempt to transition occurs. This issue has been seen intermittently on Windows NT4.0 only. It has not been seen on other version of Windows.

- The language override feature does not work as intended in this release. The –l switch is apparently reserved by Installshield v5.5 and hence it does not pass this on to the execution script. Therefore the install script resorts to the default and installs the language of the Operating system as opposed to the language indicated by the switch. This issue has been fixed in the v2.2a release.

#### Known Issues with the v2.1 release

The Intel SpeedStep technology Applet has the following known issues that may or may not be considered features:

- Disabling Automatic mode in the “Disable Modes” Flags registry value keeps the Applet from entering the Maximum Battery mode.

The Flags registry value has a setting that allows the OEM to disable either the Automatic or Maximum Battery modes.  If the registry value is set to disable the Automatic mode only (i.e. Setting Flags[19:16] = 2), the Applet will not successfully enter Maximum Battery mode.

Workaround:  If the OEM wants to disable the use of Automatic mode for any reason, then Maximum Performance (MP) mode is also a concern because Automatic mode uses less power on average and will never use more power than MP mode.  

Therefore, for whichever power source the OEM desires to limit Automatic and MP modes, the Flags registry value allow the OEM to limit the Maximum performance level supported for each power source.

Flags[11:8] - AC power resource - Max performance level supported

0 = No AC power selection is supported

1 = Only Battery Optimized Performance and Maximum Battery are supported

2 = All modes are supported

Flags[3:0] - Battery power resource - Max performance level supported

0 = No Battery power selection is supported

1 = Only Battery Optimized Performance and Maximum Battery are supported

2 = All modes are supported

For example, if Automatic and MP modes are a concern when running on Battery, setting the Flags[3:0] = 1 will remove these selections from the GUI drop down box for running on battery and the user will not be able to select these options from the tray icon pop-up menu when running on battery.

This issue does not effect disabling the Maximum Battery mode (i.e. Setting Flags[19:16] = 1).

- When installing, the installation program will determine the target language and copy the appropriate language support library, named prpclang.dll.  The OEM version of the installation program gives the OEM the ability to override the target language selection process.  For example, the OEM can install the French language version on a Russian system. On a Right-To-Left (RTL) language operating system (Arabic or Hebrew), if a non-RTL language is installed, the sentence punctuation may be reversed.  This appears to be an issue with the manner in which the operating system interprets the strings in the dialog boxes.  It is recommended that RTL languages be installed on RTL operating systems.

- When the Intel SpeedStep technology user interface is displayed by double-clicking the tray icon, the user interface may not have focus.  The user may need to click on the user interface to give it focus or use the keyboard Alt-Tab command to give the user interface focus.  This behavior is not considered a failure.

NOTICE: The Intel SpeedStep technology Applet v2.1 adds “Automatic” and “Maximum Battery” operational modes to Windows 98, Windows 98 SE, Windows ME and Windows 2000 only.  Only “Maximum Performance” and “Battery Optimized Performance” are supported under Windows 95 or Window NT 4.0.

## Installation:

The Intel SpeedStep technology Applet uses the InstallShield setup program.  The installation media includes the OEM installation program and 26 End-User installation programs.

OEM Installation Program: The OEM installation program is only available in English.  This installation program allows the OEM to install language specific files on a target system.  The OEM installation program also allows silent (unattended) installs.

Since the OEM installation program is only available in English, systems installed with the OEM installation program, when uninstalled, will display uninstall program dialogs in English.

NOTICE: If an OEM requires the uninstall program to be localized, they are required to use the localized End-User installation program to install the Intel SpeedStep technology Applet.

End-User Installation Programs: Each of the 26 End-User installation programs include a ‘click-to-accept’ end-user license agreement and do not have language override or silent install options.  25 of the End-User installation programs are localized to the specific language – Arabic and Hebrew use an International English install program because these languages do not have localized InstallShield support.  The default operation of the End-User installation is that the install program determines the language on the target system and installs the corresponding language specific files.  If the installation program cannot determine the language of the target system or the target language is not supported, the default language will be installed.  For example, if installing the French version on a Vietnamese operating system, the installation program will install the French language files because Vietnamese is not supported.  Table 1 describes the languages supported and their associated two-letter language ID’s:

| Language | ID | Language | ID |
| --- | --- | --- | --- |
| Arabic | Ar | International English | Ie |
| Chinese (Simplified) | Cs | Japanese | Jp |
| Chinese (Traditional) | Ct | Korean | Ko |
| Czech | Cz | Norwegian | Nw |
| Danish | Da | Polish | Po |
| Dutch | Dt | Portuguese (Iberian) | Ip |
| English (U.S.) | En | Portuguese (Brazilian) | Bp |
| Finnish | Fn | Russian | Ru |
| French | Fr | Slovenian | Sv |
| German | Gr | Spanish | Sp |
| Greek | Gk | Swedish | Sw |
| Hebrew | Hb | Thai | Th |
| Hungarian | Hu | Turkish | Tu |
| Italian | It |  |  |
| Table 1. – Supported languages and associated IDs. |  |  |  |

### Installing for the first time

Run x:\setup, where x is the drive on which the Intel SpeedStep technology Applet setup files are located.  On Windows 2000 systems, the user must have administrator privileges.

Silent Install (OEM installation only):

To use the silent install of the OEM installation, invoke the setup.exe program with a /s or –s switch.  If you use both the Silent install and Language override options, the Language override option must precede the Silent install option.

Language override (OEM installation only):

The default operation of the OEM installation is that the install program determines the language on the target system and installs the corresponding language specific files.  To override the default operation, invoke the setup.exe program with the /olXx or –olXx switch, where /ol or –ol is the lower-case letter O followed by the lower-case letter L and Xx is the two-letter language ID from Table 1. If you use both the Silent install and Language override options, the Language override option must precede the Silent install option.

Registry.ini:

The Registry.ini file, included on in each installation package, contains OEM configurable registry entries.  This file may be edited to allow an OEM to customize the initial registry values to fit their specific needs.  The OEM modified Registry.ini file can then be included in their platform specific releases, updates or recovery media to keep the registry entries correct for each system.  Please refer to the Registry Keys section of this guide for information about the specific registry keys and value encoding.

### Over-installing

The Intel SpeedStep technology Applet’s installation program uninstalls any previously installed version of the Applet.  This is accomplished by first checking for a running IST Applet.  If the IST Applet is running, the installation program issues a close message to the running IST Applet.  This forces the IST Applet to exit in a well-behaved manner.  The installation program then checks the Windows Registry for an “Intel SpeedStep technology Applet” uninstall string.  If one exists, the installation program invokes it with an option that performs a silent uninstall.  Once the previous version is uninstalled, the new version is installed.

Run x:\setup, where x is the drive on which the Intel SpeedStep technology Applet setup files are located.  On Windows 2000 systems, the user must have administrator privileges.

NOTICE: For correct operation when over-installing, OEMs must take care to close any handle their applications may open to the Intel SpeedStep technology Applet’s User Interface support library: prpcui.dll. Additionally, please review the list of known issues before performing an upgrade.

### Uninstalling the Applet:

Open Control Panel | Add Remove Programs and select the “Intel SpeedStep technology Applet” entry and press Add/Remove.

### Installing on a non- Intel SpeedStep technology capable system:

NOTICE: For OEMs that plan to install a single operating system pre-load on systems that are both Intel SpeedStep technology capable and non-Intel SpeedStep technology capable, the system BIOS must support the ISTGetStatus SMM interface command.  The Intel SpeedStep technology Applet determines whether a system is capable of supporting Intel SpeedStep technology from the return of the ISTGetStatus command.  When the IST Applet determines that a system cannot support Intel SpeedStep technology, the application removes itself by invoking a silent InstallShield uninstall.  The Intel SpeedStep technology Applet will not load again unless it is re-installed.  The system BIOS on a non-Intel SpeedStep technology enabled system can simply return the static value EDI=0x1FF01 when the ISTGetStatus SMM command is called.

### Installing on Windows XP:

Starting with Windows XP, control of the Intel SpeedStep technology will be performed by the operating system.  The IST Applet should not be installed on this operating system.  The installation package will abort if attempting to install on Windows XP.  On systems that are upgraded to Windows XP, the IST Applet will be removed as part of Microsoft’s upgrade process.  The “Designed for Windows XP” logo requires the use of OS native control of processor performance states.

WARNING: Do not install any Intel SpeedStep technology Applet on a system running the Windows XP operating system.  This combination will cause unintended consequences.

## Operation:

### Transition Control:

NOTICE: The Intel SpeedStep technology Applet will issue a ISTControl SMM Interface call with ECX=0 during initialization to gain exclusive control of Intel SpeedStep technology transitions within the system.  Once the SMM code has received and successfully returned the ISTControl call, transitions are not allowed by SMM code except as directed by the Applet using the SetISTState SMM Interface call.  There is one exception to this rule: If the system reaches a thermal or battery critical situation, the BIOS can force the system into Battery Optimized Performance if and only if the BIOS sets the number of states available to one.  The number of states available is encoded in the EDI[23:16] register of the ISTGetStatus SMI interface call.  See the BIOS Writer’s Guide for further information.  If the number of states available remains two, the Applet may reset the processor performance level to Maximum Performance.

### Selection of the EventRegister Port:

NOTICE: For OEMs that plan to use the option EventRegister port, care must be taken when selecting a proper port address.  The selected port must be verified on operating systems to ensure that there are no conflicts between the selected port and normal OS operation.

### Expected Operation:

The following section explains the expected operation on a properly configured system.

#### Driver and Applet Initialization

When the system is powered on, the system BIOS is responsible for setting the processor performance level.  This processor performance level is a factor of the BIOS settings and the power source, AC or battery.  The operating system then initializes at the processor performance level set by the BIOS.

The Intel SpeedStep technology driver gets initialized early in the OS initialization sequence.  This statically loaded driver initializes and exposes an entry point to receive requests from the user-mode Applet.  The driver issues an ISTGetStatus SMM interface call to check 1.) Is the system is capable of supporting Intel SpeedStep technology? and 2.) Has the User BIOS disabled Intel SpeedStep technology?  If either of these conditions is true or an error occurs, the driver writes the appropriate ExitCode into the registry and exits without completing initialization.

The operating system processes the entries in the Run registry key and starts each of the programs listed.  The Intel SpeedStep technology Applet is one of the applications listed, so Windows launches the Applet.

The Applet first reads in the pertinent registry values (i.e. Flags, System and User preferences, driver exit code).  It then checks the driver’s exit code to ensure proper driver initialization.  If the driver exited because the system does not support Intel SpeedStep technology, the Applet removes itself by silently running the InstallShield uninstaller.  This action completely removes the Applet and driver from the Windows registry and deletes the Applet object files.  This will keep the driver and Applet from ever loading again.  If the BIOS has disabled the Applet, the tray icon is removed and the Applet exits without completing initialization.

The Applet then checks the Enable bit in the Flags registry value to see if the Applet is to be loaded enabled or disabled.  If the Applet is to load disabled, it puts the disabled icon in the system tray and then completes initialization without error.

If the Applet is not disabled, it will call ISTGetStatus – This is the first SMM call made by the Applet.  This ensures that the Applet has connected to the driver and can retrieve data.  If the Applet is unable to communicate with the driver, it again checks the driver’s exit code.  If there is no indication of an abnormal driver initialization, the Applet will read the AppletTimeout registry value and attempt to contact the driver every second until the AppletTimeout period expires before returning an error.

If the polling interval is not zero, the Applet will start the polling timer.  This polling timer checks the status for AC insertion or removal.  It also checks for a change in the number of states currently available that indicates that the processor performance level should be changed to Battery Optimized Performance due to unique conditions, such as thermal cycling.

The Applet now uses the ISTControl SMI command to tell the BIOS that the Applet will take exclusive control of Intel SpeedStep technology transitions.  The driver first issues the ISTControl command with the 2000 copyright string.  If this fails due to an invalid authentication string, the driver will issue the same command with the 1999 copyright string.  This feature allows the IST Applet to initialize properly on older BIOS’s.  If native OS support is installed, it will be reported in this call and the Applet will immediately remove itself by silently running the InstallShield uninstaller.  This action completely removes the Applet and driver from the Windows registry and deletes the Applet object files.  This will keep the driver and Applet from ever loading again.

The Applet queries the BIOS for the SMM Interface revision number.  If the revision is 00.0010.00, a flag is set indicating that the Demand-Based Switching modes (DBS) modes are supported by the BIOS.  DBS modes include Automatic and Maximum Battery modes.

NOTICE: If the revision returned by the BIOS is anything other than 00.0010.00, the system will revert back to legacy operation (i.e. static transitions only).

If the user does not have restricted privileges (on Windows 2000 only), the Applet then synchronizes the user preferences to the BIOS settings if they are different.  Each time the Applet initializes, it queries the User’s BIOS setup using the ISTGetStatus call.  If the user has changed their BIOS setting since the last boot, the Applet will adjust the user’s SpeedStep preferences to match.

The Applet then initializes the DBS support if the system has the capability.  The Applet queries the OEM configuration details and gets processor information from the BIOS.  If there are failures in any of these areas, such as a failure to get processor information, the Applet reverts back to legacy operation (i.e. static transitions only).  If the platform supports Automatic mode but does not support Maximum Battery mode due to lack of throttle control, the Applet will place the processor into Battery Optimized Performance when the user selects Maximum Battery.

#### The User Interface

The User Interface is accessed either from the system tray icon (a “flag”) or from the power management icon in the Control Panel. If the user does not have restricted privileges (on Windows 2000 only), the User Interface allows the user to select their processor performance level preference for when the computer is running on AC or on battery.  The number of performance levels available for each power source is configurable by the OEM.  If the system reports DBS capability, the “Automatic” and “Maximum Battery” options are available in addition to the “Maximum Performance” and “Battery Optimized Performance” selections.

The User Interface allows for advanced options, such as disabling the Applet.  Disabling the Applet only disables the user’s ability to set preferences and control transitions; it does not disable Intel SpeedStep technology in the BIOS.  The Applet never issues a SetISTSetup SMI command with ECX = 0xFF to disable control.  The only way to completely disable Intel SpeedStep technology is via the BIOS setup.

The behavior of the Applet when a user disables it is governed by OEM configuration settings in the registry value, Flags.  By default, the Applet sets the processor performance level to Battery Optimized Performance when the user disables the Applet.  The OEM may choose to have the Applet set the processor performance level to Maximum Performance or to leave it alone.  Please refer to the Flags registry value for further information about this setting.

As stated in the previous section, during Applet initialization, the Applet checks the user BIOS settings during Applet initialization and changes the Applet’s user preferences to match.  If the user makes a change to their preference for how the system runs when on battery or on AC, the Applet will save these changes and issue a SetISTSetup SMI command to synchronize the BIOS to the user’s preference.  The mapping of BIOS user preferences is described in the SystemPrefs registry value – Bit[7:4] describe the AC preference and Bit[3:0] describe the Battery preference.  The value 0xFF means that the BIOS user preference is to disable Intel SpeedStep technology.

When the Applet issues a SetISTSetup SMI command, the BIOS may reject the request by setting EAX=0 (Successful SMI) and EDI=1 (Setup not saved) when the SMI returns.  This has the effect of allowing the user’s preference setting to remain in effect until the next reboot, at which time the Applet will read the BIOS setup and set the user’s preferences to match the BIOS settings.

On legacy systems – those that do not support the new interface, the Applet interprets the BIOS user settings and issues SetISTSetup SMI commands as they are defined in the legacy interface.

#### Standby and Hibernate

Windows operating systems issue several messages to all running applications before entering a standby or hibernate state.  When the Intel SpeedStep technology Applet receives the PBT_APMSUSPEND message, it saves the current processor performance level and sets the processor into Battery Optimized Performance.  It then issues an ISTControl SMI command to return control of transitions to the BIOS and bypasses the polling timer that checks status changes.  Upon resuming from standby or hibernate, the operating system issues a PBT_APMRESUMESUSPEND or PBT_APMRESUMEAUTOMATIC message.  When the Applet receives any of these messages, it issues an ISTControl SMI command to request control of transitions and again begins monitoring the system for status changes.  If the processor performance level was changed before entering the suspend state, the Applet will restore the processor to its pre-suspend state.

Windows operating systems do not issue suspend messages during a critical suspend operation.  A critical suspend operation can occur due to critical battery level or thermal conditions.  Under these cases, the Intel SpeedStep technology Applet will not issue the ISTControl SMI command to return control of transitions to the BIOS.

#### Restricted User limitations

With some Windows operating systems, like Windows 2000, a user account can be created that has restricted access to the operating system and its environment.  While the IST Applet will run correctly, a restricted user cannot modify the system level preferences.  System level preferences include the ability to disable the Applet and the ability to set processor performance level preferences for AC and battery.  These choices will be grayed out.  User preferences will not be affected.

#### Applet MB and DBS Algorithms

The following flowchart describes the DBS algorithms that are included with the v2.0 release.  These algorithms are subject to change in subsequent releases.  Note that Maximum Battery (MB) Algorithm “B” uses both the MB Algorithm “A” and the Automatic algorithm.

NOTICE: Configuring the Applet to use MB Algorithm “B” will result in shorter battery life than MB Algorithm “A” and may result in shorter battery life than Battery Optimized Performance depending upon the work load.

#### Thermal Cycling

Thermal cycling is defined as when the ISTGetStatus SMI command returns the “Number of States currently supported” (EDI[23:16]) less than 2.  Thermal cycling is used by the OEM to limit the power consumption and thereby the heat dissipation of the processor.  This is accomplished by returning the Number of states currently supported of 1 or 0.  For each processor performance level, Table 2 below describes the behavior when the Number of States currently supported (#States) is changed.  #States of 2 is normal operation.

| Processor performance level | #States | No Load | Load |
| --- | --- | --- | --- |
| Maximum Performance (MP) | 2 | MP | MP |
| MP | 1 | BOP | BOP |
| MP | 0 | BOP + throttling (BT) | BT |
| Battery Optimized Performance (BOP) | 2 | BOP | BOP |
| BOP | 1 | BOP | BOP |
| BOP | 0 | BT | BT |
| Automatic | 2 | BOP | MP |
| Automatic | 1 | BOP | BOP |
| Automatic | 0 | BT | BT |
| Maximum Battery (MB) Algorithm A | 2 | BOP | BT |
| MB Algorithm A | 1 | BOP | BT |
| MB Algorithm A | 0 | BT | BT |
| MB Algorithm B | 2 | BOP | BT->BOP->MP |
| MB Algorithm B | 1 | BOP | BT->BOP |
| MB Algorithm B | 0 | BT | BT |
| Table 2. – Thermal Cycling behavior |  |  |  |

NOTICE: Thermal cycling should only be used when absolutely necessary and for the shortest period possible.  During periods of thermal cycling the Applet grays out the processor performance modes of the tray icon’s pop-up menu and does not allow the user to change their preferences for AC and battery operation.

## External Intel SpeedStep technology Interfaces:

The Intel SpeedStep technology Applet provides an external interface that allows an OEM to force the Applet to update its configuration settings at run-time.  Configuration settings are contained within the Registry. Configuration settings that may be updated at run-time include the settings contained in the Flags, PollPeriod, SystemPrefs, UserPrefs, DBSPeriod, DBSThreshold, MBPrefs, MBPeriod and MBThreshold registry values.  (See Registry Keys section for more details)  The OEM can make modifications to these registry values programmatically and then force the Intel SpeedStep technology Applet to update its configuration settings by calling the PRPC_UpdateSettings() API.

If the Intel SpeedStep technology Applet is disabled, special care must be taken.  The Applet may have been disabled in the current session (Flags[31:28] = 0 and ExitCode = 0x00000000) or disabled in a previous session (Flags[31:28] = 0 and ExitCode = 0x00000102).

If the Applet was disabled in the current session, the OEM can modify the registry items, set Flags[31:28] = 1 and use the External Interface to enable the Applet and update the configuration settings.

If the Applet was disabled in a previous session, a reboot will be required to update the configuration settings and greater care must be taken when modifying the AC and Battery power system preferences (SystemPrefs[7:4] and SystemPrefs[3:0]).  If the OEM modifies the system’s preferences, the OEM must ensure that the user BIOS setup reflects the same settings as the Applet’s user preferences.  If this is not done correctly, upon reboot the BIOS setup value will be used to reset the Applet’s user preferences as described in the Expected Operation section.

NOTICE: It is critical that all programs that open a handle to the Intel SpeedStep technology support library, prpcui.dll, release the handle and free the library after calling PRPC_UpdateSettings().  Failure to release the handle and free the library will result in errors during subsequent over-install operations.

### PRPC_UpdateSettings External Interface

The following code sample can be used as an example of how to call the PRPC_UpdateSettings() external interface:

#define PRPC_UI_LIB	“PRPCUI.DLL”

typedef DWORD (CALLBACK* LPFNDLLFUNC)(void);

HINSTANCE		hUILib = NULL;                  // Handle to the user interface

LPFNDLLFUNC		lpfnDllFunc;

DWORD			dwReturn = 1;

/*

// Load the user interface library

*/

hUILib = LoadLibrary(PRPC_UI_LIB);

if(NULL != hUILib)

{

lpfnDllFunc = (LPFNDLLFUNC)GetProcAddress(hUILib, “PRPC_UpdateSettings”);

if(lpfnDllFunc)

{

/*

//  dwReturn will return 0 for success, 0xFFFFFFFF for error

*/

dwReturn = lpfnDllFunc();

if(0xFFFFFFFF == dwReturn)

{

/* Appropriate error handling */

}

}

else

{

/* Appropriate error handling */

}

FreeLibrary(hUILib);

}

else

{

/* Appropriate error handling */

}

### PRPC_QuerySMIVersion External Interface

The following code sample can be used as an example of how to call the PRPC_QuerySMIVersion() external interface:

#define PRPC_UI_LIB		“PRPCUI.DLL”

#define PRPC_EXE_NOT_LOADED	0x1000

#define PRPC_INVALID_SMI      0x2000

typedef DWORD (CALLBACK* LPFNDLLFUNC)(DWORD *);

HINSTANCE		hUILib = NULL;// Handle to the user interface

LPFNDLLFUNC		lpfnDllFunc;

DWORD			dwReturn = 1;

DWORD			dwVersion = 0;

/*

// Load the user interface library

*/

hUILib = LoadLibrary(PRPC_UI_LIB);

if (NULL != hUILib)

{

lpfnDllFunc = (LPFNDLLFUNC)GetProcAddress(hUILib,

“PRPC_QuerySMIVersion”);

if(lpfnDllFunc)

{

/*

//  dwReturn will return 0 for success, 0xFFFFFFFF for error

*/

dwReturn = lpfnDllFunc(&dwVersion);

if (0 != dwReturn)

{

/*

* Possible error codes:

* PRPC_EXE_NOT_LOADED – prpcui.exe is not running.

* PRPC_INVALID_SMI – An SMI version has not been found.

*/

/* Appropriate error handling */

}

}

else

{

/* Appropriate error handling */

}

/* Always close the handle immediately after use */

FreeLibrary(hUILib);

}

else

{

/* Appropriate error handling */

}

## Registry Keys:

The following registry keys and values are added to the system registry when Intel SpeedStep technology Applet is installed.

Registry Key: HKEY_LOCAL_MACHINE\Software\Intel\PRPC

Values:

| Type | Name | Data |
| --- | --- | --- |
| DWORD | AppletTimeout | Timeout value, in seconds, that that Applet may delay to establish communication with the driver. See description below. |
| DWORD | Command | SMI Command – default value is 0x82. Valid values are 0x00-0xFF. This number is supplied by BIOS developers and should be considered Reserved. |
| DWORD | DBSPeriod | High Word: DBS Up Period – Valid values are 0x0000-0xFFFF. See description below. Low Word: DBS Down Period – Valid values are 0x0000-0xFFFF. See description below. |
| DWORD | DBSThreshold | High Word: DBS Up Threshold – Valid values are 1 – 100. See description below. Low Word: DBS Down Threshold – Valid values are 1 – 100. See description below. |
| DWORD | EventRegister (Optional) | Low Word: Event Register address – default value is zero. Valid values are 0x0001-0xFFFF. This number is supplied by BIOS developers and should be considered Reserved. See description below. High Word: Event Register Mask – default value is zero. Valid values are 0x00-0xFF. This number is supplied by BIOS developers and should be considered Reserved. See description below. |
| DWORD | ExitCode | The driver writes this entry to indicate a successful initialization (0) or an error exit code indicating the specific error that caused the failure. See description below. |
| DWORD | Flags | Flags describing OEM configuration. See description below. |
| DWORD | MBPeriod | High Word: MB to BOP Period – See description below. Low Word: MB to MP Period – See description below. |
| DWORD | MBPoll | Maximum Battery sampling interval – See description below. |
| DWORD | MBPrefs | OEM Maximum Battery Preferences – See description below. |
| DWORD | MBThreshold | High Word: MB Throttle Threshold – See description below. Low Word: MB Up Threshold – See description below. |
| DWORD | PollPeriod | Polling period, in milliseconds, for the Applet to check status. See description below. |
| DWORD | Port | SMI Command Port – default value is 0xB2. Valid values are 0x0000-0xFFFF. This number is supplied by BIOS developers and should be considered Reserved. |
| DWORD | SetStateRetry | Number of ISTSetState SMI attempts the Applet will make before reporting an error. See description below. |
| DWORD | SystemPrefs | System level preferences. See description below. |
| DWORD | UserPrefs | OEM version of the default User preferences. This value gets propagated to each new user account. See description below. |
| DWORD | W2KTimer | Choose timer scheme on Windows 2000 – See description below. |

Registry Key: HKEY_CURRENT_USER\Software\Intel\PRPC

Values:

| Type | Name | Data |
| --- | --- | --- |
| DWORD | AppExitCode | The Applet writes this entry to indicate a successful initialization (0) or an error exit code indicating the specific error that caused the failure. See description below. |
| DWORD | UserPrefs | User preferences. These map to specific user-selectable preferences that are unique for each user account. See description below. |

### AppletTimeout

The AppletTimeout element is a 32-bit DWORD value.

This value indicates the maximum time, in seconds, that the Applet may delay when attempting to establish communications with the driver.

Under certain conditions, the Applet may initialize before the driver has been given an opportunity to load.  An example of this condition: Under Windows NT4.0, with a fully installed network that requires DHCP to assign an IP address to the system.  If the network card remains plugged in, but the network cable is removed, the DHCP service takes a very long time to time-out.  While the DHCP service waits to time-out, other drivers are not allowed to load.  The user is still allowed to login, thus causing the IST Applet to load before the driver.  Under this condition, the Applet may delay up to AppletTimeout seconds before exiting with an error condition.

Under normal conditions, if the Applet is unable to establish communications due to an IST driver error that occurs during driver initialization, the Applet will not incur any delay.

### DBSPeriod

The DBSPeriod element is a 32-bit DWORD that is encoded as follows:

| [31:16] DBS Up Period | [15:0] DBS Down Period |
| --- | --- |

DBSPeriod[31:16] – DBS Up Period.  How long, in mSec, processor demand must be over the DBS Up Threshold before transitioning to Maximum Performance.  This value cannot be less than 100.

DBSPeriod[15:0] – DBS Down Period.  How long, in mSec, processor demand must be below the DBS Down Threshold before transitioning to Battery Optimized Performance.  This value cannot be less than 100.

This value and the DBSThreshold value are tightly coupled.  See the explanation for DBSThreshold below.

The default for this value is 0x012c03e8.  If the DBSPeriod value is zero, the default value of 0x012c03e8 will be used.

### DBSThreshold

The DBSThreshold element is a 32-bit DWORD that is encoded as follows:

| [31:24] Reserved | [23:16] DBS Up Threshold | [15:8] Reserved | [7:0] DBS Down Threshold |
| --- | --- | --- | --- |

DBSThreshold[31:24] – Reserved – must be set to zero

DBSThreshold[23:16] – DBS Up Threshold.  Valid values are 1 – 100.  Percent of processor demand that will trigger a transition to Maximum Performance.

DBSThreshold[15:8] – Reserved – must be set to zero

DBSThreshold[7:0] – DBS Down Threshold. Valid values are 1 – 100.  Percent of processor demand that will trigger a transition to Battery Optimized Performance.

These processor demand percentages are based upon a percentage of the BOP frequency of the processor.  The default is 95% for both up and down transitions.  The down transition percentage is scaled to the MP frequency of the processor.  For example: On a 750/600 processor, if the processor is in BOP mode and CPU utilization exceeds 95% of 600Mhz for the Up Period (See DBSPeriod above), the Applet will transition the processor to MP.  Once in MP, if demand drops below 95% of the BOP frequency (76% of MP frequency) for the Down Period, the Applet will transition the processor to BOP.

This value and the DBSPeriod value are tightly coupled.  See the explanation for DBSPeriod above.

The default for this value is 0x005f005f.  If the DBSThreshold value is zero, a default value of 0x005f005f will be used.

### EventRegister

The EventRegister element is a 32-bit DWORD that is encoded as follows:

| [31:16] Event Register Mask | [15:0] Event Register Address |
| --- | --- |

EventRegister[31:16] – Event Register Mask.  The lower 8-bits of this word represent a mask for interpreting the value that is read from the Event Register address.  The default value of 0x0000 is equivalent to an Event Register Mask of 0xFF.  For example, a mask of 0x03 indicates that only the least significant 3-bits of the value are used.  A mask of 0xFF indicates that all 8-bits should be used.

EventRegister[15:0] – Event Register Address.  If this value is non-zero, then this address is read immediately before an ISTGetStatus SMM Interface call.  The address is read using the assembly instruction ‘in al, dx’, where dx is the Event Register address.  If the value read at the address, after being masked by the Event Register Mask, has changed since the last read, it indicates that the system status has changed and an ISTGetStatus SMM interface call is made.  If the value has not changed, then no SMM interface call is made and the values from the last successful ISTGetStatus SMM interface call are returned.

### ExitCode

The ExitCode element is a 32-bit DWORD value that is written by the driver during its initialization to indicate the success or failure of the driver load.  The Exit code is interpreted as follows:

0x0000 – Initialization was successful.

0x0101 – An error occurred when querying the registry for the Flags value.

0x0102 – The Enable Intel SpeedStep technology bit of the Flags value is zero.

0x0104 – The first SMI Interface call (ISTGetStatus) returned an error.

0x0105 – The ISTGetStatus call reports that the system is not Intel SpeedStep technology Capable.

0x0106 – The ISTGetStatus call reports that the BIOS has disabled Intel SpeedStep technology.

0x0107 – The IoCreateSymbolicLink() call was unsuccessful.

0x0108 – The IoCreateDevice() call was unsuccessful.

0x0109 – The Operating System disabled Intel SpeedStep technology through native support.

0x010A – The IoCreateNotificationEvent() call was unsuccessful.

The following Driver Exit Codes are considered normal:

0x0000 – Initialization was successful.

0x0102 – The Enable Intel SpeedStep technology bit of the Flags value is zero.

0x0106 – The ISTGetStatus call reports that the BIOS has disabled Intel SpeedStep technology.

The OS native support Driver Exit Code (0x0109) and System not capable Exit Code (0x105) will cause the Intel SpeedStep technology Applet to uninstall itself and remove all associated registry keys.  Therefore the OEM/end-user should never see this condition.

### Flags

The Flags element is a 32-bit DWORD that is encoded as follows:

| [31:28] Enable Flag | [27:20] Reserved | [19:16] Disable Modes | [15:12] Disable Behavior | [11:8] AC Max Level | [7:4] Legacy OS Config | [3:0] Batt Max Level |
| --- | --- | --- | --- | --- | --- | --- |

Flags[31:28] – Enable Intel SpeedStep technology (1 = Enable; 0 = Disable)

Flags[27:20] – Reserved – must be set to zero

Flags[19:16] – Disable Modes

0 = Enable both Maximum Battery and Automatic modes (Default)

1 = Disable Maximum Battery mode only

2 = Unsupported (Previously this was Disable Automatic mode only)

3 = Disable both Maximum Battery and Automatic modes

NOTE: Setting Flags[19:16] = 2 also keeps the Applet from successfully entering Maximum Battery mode.  Please refer to the Know issues section for further information and a workaround.

Flags[15:12] – Disable behavior:

0 = Set system to Battery Optimized Performance when the Applet is disabled (Default)

1 = Set system to Maximum Performance when the Applet is disabled

2 = Do not change the processor performance level when the Applet is disabled

Flags[11:8] – AC power resource – Max performance level supported

0 = No AC power selection is supported (Not recommended)

1 = Only Battery Optimized Performance and Maximum Battery are supported

2 = All modes are supported

Flags[7:4] – Legacy OS configuration:

0 = Do not allow two-way transitions.

1 = Allow Two-way transitions on Windows 95.

2 = Allow Two-way transitions on Windows NT 4.0.

3 = Allow Two-way transitions on Windows 95 and NT 4.0 (Default).

Flags[3:0] – Battery power resource – Max performance level supported

0 = No Battery power selection is supported (Not recommended)

1 = Only Battery Optimized Performance and Maximum Battery are supported

2 = All modes are supported

For example, if Intel SpeedStep technology is enabled and that system supports all processor performance modes on AC (Max processor performance level of 2) and only Battery Optimized Performance and Maximum Battery on battery (Max processor performance level of 1), the Flags would be 0x10000231.  The default setting for this value is 0x10000232 indicating that all processor performance modes are supported on both AC and battery.

When the Disable Modes bits are set, care must be taken that the SystemPrefs registry values do not conflict with the disabled modes.  For example, the default recommended mode for battery is the Automatic mode.  If this mode is disabled, change the recommended mode to one of the remaining modes.

It is not recommended to set Flags[11:8] or Flags[3:0] to zero because it severely limits the Applet functionality.

### MBPeriod

The MBPeriod element is a 32-bit DWORD that is encoded as follows:

| [31:16] MB to BOP Period | [15:0] MB to MP Period |
| --- | --- |

MBPeriod[31:16] – MB to BOP Period in milliseconds.  Used by Applet Maximum Battery (MB) algorithm “B”.  This value determines how long the CPU utilization must be above the MB Up Threshold before transitioning from Maximum Battery to Battery Optimized Performance (i.e. disengage throttling).  This value cannot be less than the MBPoll value.

MBPeriod[15:0] – MB to MP Period in milliseconds.  Used by Applet MB algorithm “B”.  This value determines how long the CPU utilization must be above the MB Up Threshold before transitioning from Maximum Battery to Maximum Performance.  This measures the total period and includes the duration of the MB to BOP Period described above.  This value cannot be less than the 3 * MBPoll value.

These values and the MBThreshold are tightly coupled.  See the explanation for MBThreshold below.

The default for this value is 0x07d01388.  This translates to 2000 ms from MB to BOP and 5000 ms total time from MB to MP.  For example, using the default value, when CPU utilization reaches 100%, the MB algorithm “B” will throttle for 2 seconds (MB to BOP Period), switch to BOP for an additional 3 seconds (MB to BOP Period + MB to MP Period) before transitioning to MP.

If the MBPeriod value is zero, or if either MBPeriod[31:16] or MBPeriod[15:0] are zero, the Intel SpeedStep technology Applet will default to Maximum Battery Algorithm “A” regardless of the MBPrefs settings.

### MBPoll

The MBPoll element is a 32-bit DWORD that is encoded as follows:

| [31:16] Reserved | [15:0] MB Sampling Interval |
| --- | --- |

MBPoll [31:16] – Reserved – must be set to zero

MBPoll [15:0] – MB Sampling Interval in milliseconds.  Used by Applet algorithm “A” and “B”.  This value sets the sampling interval for checking CPU utilization when using the Applet’s built-in Maximum Battery algorithms.  This value cannot be less than 100.  If the MBPoll value is zero, a default value of 300 will be used.

The default for this value is 0x0000012c (300 milliseconds).

### MBPrefs

The MBPrefs element is a 32-bit DWORD that is encoded as follows:

| [31:17] SMI Generation Period | [16] OEM Flags 2 | [16:2] Reserved | [1:0] OEM Flags 1 |
| --- | --- | --- | --- |

MBPrefs[31:17] – SMI Generation Period.  Period, in milliseconds, between invocations of the ISTGetStatus SMI Command.  Used only when OEM Generate SMI bit is set (See below).  This value cannot be less than 100.

MBPrefs[16] – OEM Flags 2

Bit[16] – OEM Generate SMI bit (1 = Generate SMIs; 0 = Do not generate SMIs)  Used only if Bit[0] is set.

MBPrefs[16:2] – Reserved – must be set to zero.

MBPrefs[1:0] – OEM Flags 1

Bit[1] – Applet MB Algorithm select bit (0 = Algorithm “A”; 1 = Algorithm “B”)

Bit[0] – Applet vs. OEM MB Algorithm select bit (0 = Applet Algorithm; 1 = OEM Algorithm)

The OEM has the ability to choose between three different Maximum Battery (MB) algorithms.  These MB algorithms are:

- Applet MB algorithm “A” – This algorithm uses BOP and throttling to extend battery life at the expense of performance.

- Applet MB algorithm “B” – This algorithm uses throttling, BOP and MP performance levels to extend battery life somewhat but allow MP when demand is sustained.

- OEM MB algorithm – This algorithm is OEM designed and implemented within the BIOS code.  The IST Applet is used simply to signal the BIOS to enter this MB state by issuing an ISTSetState SMI command with ECX = 0x80.  When in this OEM MB state, the IST Applet can optionally be used to generate periodic ISTGetStatus SMI commands as a heartbeat to the BIOS.

The default for this value is 0x00000000.

### MBThreshold

The MBThreshold element is a 32-bit DWORD that is encoded as follows:

| [31:24] Reserved | [23:16] MB Throttle Threshold | [15:8]: Reserved | [7:0] MB Up Threshold |
| --- | --- | --- | --- |

MBThreshold[31:24] – Reserved – must be set to zero

MBTheshold[23:16] – MB Throttle Threshold.  Valid values are 1 – 100.  Used by Applet MB algorithms “A” and “B”.  When processor utilization exceeds this threshold, the IST Applet signals the BIOS to throttle the processor.  When demand drops below this threshold, the IST Applet signals the BIOS to disengage throttling.

MBThreshold[15:8] – Reserved – must be set to zero

MBThreshold[7:0] – MB Up Threshold.  Valid values are 1 – 100.  Used by Applet MB algorithm “B”.  When processor utilization exceeds this threshold for MB to BOP Period, the IST Applet signals the BIOS to disengage throttling to allow the system to run in BOP mode.  If processor utilization continues to exceed this threshold for MB to MP Period, the IST Applet switches to MP.

This value is tightly coupled to the MBPeriod value.  If the MBThreshold[23:16] value is zero, a default value of 50 will be used.  If the MBThreshold[7:0] value is zero, the Intel SpeedStep technology Applet will default to Maximum Battery Algorithm “A” regardless of the MBPrefs settings.

The default for this value is 0x0032005a.

### PollPeriod

The PollPeriod element is a 32-bit DWORD value.

The value indicates the Processor Poll Period.  This number (in mSec) sets how often the Applet checks for status changes.  Status changes include power source changes and changes to the number of processor states currently available.  The number of states currently available can indicate thermal cycling conditions.  The Applet checks by making an ISTGetStatus SMM interface call.  If the optional EventRegister has been implemented, an SMI is generated only if the EventRegister value has changed.  A value of zero (0), disables power source queries and therefore relies upon PBT_APMPOWERSTATUSCHANGE messages to trigger transition evaluations.

The default setting for this value is 2000 (0x7D0) for a 2 second polling period.

### SetStateRetry

The SetStateRetry element is a 32-bit DWORD that is encoded as follows:

| [31:28] Suppress Message flag. | [27:0] Retry Count |
| --- | --- |

SetStateRetry[31:28] – Suppress Message Flag: This 4-bit value indicates whether a Windows MessageBox with the following message is displayed when the ISTSetState SMI command returns with an error.

“The system was unable to change performance.
To change performance, you must shutdown and restart your computer.”

0 = Display the message if the ISTSetState SMI command returns an error. (Default)
1 = Suppress the message if the ISTSetState SMI command returns an error.

SetStateRetry[27:0] – Retry Count: This 28-bit value contains the number of times the driver should issue the ISTSetState SMI command before returning an error.  Prior to release v1.1, this was hard-coded to 1024 (0x400).  Valid values are 0x0000001-0xFFFFFFF.

A SetStateRetry value of zero means retry forever.

The default for this value is 0x00000400.

### SystemPrefs

The SystemPrefs element is a 32-bit DWORD that is encoded as follows:

| [31:16] Reserved | [15:12] AC Rec | [11:8] Batt Rec | [7:4] AC Pref | [3:0] Batt Pref |
| --- | --- | --- | --- | --- |

SystemPrefs[31:16] – Reserved – must be set to zero

SystemPrefs[15:12] – AC recommended selection

0x0 = Maximum Performance

0x1 = Battery Optimized Performance

0x2 = Maximum Battery

0x3 = Automatic

0xF = Do not display the “(Recommended)” string

SystemPrefs[11:8] – Battery recommended selection

0x0 = Maximum Performance

0x1 = Battery Optimized Performance

0x2 = Maximum Battery

0x3 = Automatic

0xF = Do not display the “(Recommended)” string

SystemPrefs[7:4] – AC power resource user preference

0x0 = Maximum Performance

0x1 = Battery Optimized Performance

0x2 = Maximum Battery

0x3 = Automatic

SystemPrefs[3:0] – Battery power resource user preference

0x0 = Maximum Performance

0x1 = Battery Optimized Performance

0x2 = Maximum Battery

0x3 = Automatic

This value decodes the OEM configurable recommendations.  Bits 15:12 and 11:8 decode which processor performance level is recommended for each power resource.  The string, “(Recommended)”, will be appended to the selection in the user interface drop-down boxes.  The user preferences, Bits [7:4] and [3:0] reflect the selections the user has chosen for each power resource.  On Windows 2000, a user with restricted access cannot change the user preferences.

The default value of 0x00000303 appends the “(Recommend)” string to the Maximum Performance option for AC and to the Automatic option for Battery.  It also sets these options as the initial user preferences.

NOTE: The mappings for Maximum Performance and Battery Optimized Performance have changed.  Prior versions mapped 1 to Maximum Performance and 0 to Battery Optimized Performance.

The default for this value is 0x00000303.

### UserPrefs

The UserPrefs element is a 32-bit DWORD that is encoded as follows:

| [31:4] Reserved | [3:0] User Pref |
| --- | --- |

UserPrefs[31:4] – Reserved – must be set to zero

UserPrefs[3:0] – User preference

Bit[3] = Notification beep (0 = Off; 1 = On)

Bit[2] = Taskbar icon display (0 = No display; 1 = display)

Bit[1] = Ask the user before transitioning (0 = Don’t ask; 1 = ask)

Bit[0] = Transition on power source change (0 = Don’t transition; 1 = transition)

There are two versions of this registry value.  The OEM default value is found in the HKEY_LOCAL_MACHINE\Software\Intel\PRPC key.  This setting propagated to each user account when the IST Applet is first run in each user account environment.  The User version is found in the HKEY_CURRENT_USER\Software\Intel\PRPC key.

The default for this value is 0x00000005.

### W2KTimer

The W2KTimer element is a 32-bit DWORD that is encoded as follows:

| [31:8] Reserved | [7:0] Timer selection |
| --- | --- |

W2KTimer[31:8] – Reserved – must be set to zero

W2KTimer [7:0] – Timer selection.  If the Intel SpeedStep technology Applet is running on a Windows 2000 platform, the Applet has a choice of two different timing mechanisms for calculating CPU utilization.  One choice is the mechanism used under Windows 98 and ME platforms.  This mechanism uses the native OS high performance counter and the CPU’s Time Stamp Counter (TSC) to calculate utilization.  The second choice uses native Windows 2000 performance data.  In most cases, both mechanisms report similar utilization results.  On non-Windows 2000 platforms, this registry value is ignored.

W2KTimer[7:0] = 0: Use the Timer/TSC mechanism.

W2KTimer[7:0] = 1: Use the native Windows 2000 mechanism.

The default for this value is 0x00000001.

### AppExitCode

The AppExitCode element is a 32-bit DWORD value.

The value is written by the Applet during its initialization to indicate the success or failure of the Applet initialization.  This value is found in the HKEY_CURRENT_USER\Software\Intel\PRPC key because the Applet is always guaranteed to have read/write privilege for this registry key.  The Exit code is interpreted as follows:

0x0000 – Initialization was successful.

0x0101 – An error occurred when initializing the OEM and user configuration data.

0x0102 – An error occurred when attempting to register the application’s window class.

0x0103 – An error occurred when using the CreateWindow() API to create the application.

0x0104 – An error occurred while initializing the application.

0x0105 – An error occurred when attempting to gain control of Intel SpeedStep technology transitions.

0x0201 – While initializing the application, the program was unable to communicate with the underlying Ring-0 driver.

0x0202 – While initializing the application, the program detected that the BIOS had disabled Intel SpeedStep technology.

0x0203 – While initializing the application, the program was unable to create the polling timer that checks for AC insertion/removal status.

Copyright © Intel Corporation 1999 - 2001. *Third-party brands and names are the property of their respective owners.