O2Micro CardBus Controller Software Installation

The following setup instructions are for Notebook systems containing an O2Micro CardBus controller.  For installations of O2Micro CardBus PCI Demo Boards, please refer to “Utilities for O2Micro CardBus PCI Demo Boards” for initialization instructions before continuing with these setup instructions.

Windows95 (release 1):

This original release of Windows 95 only supports Legacy PCIC mode (16-bit PCMCIA) by default.  CardBus mode is not supported by this release.  To utilize Legacy PCIC support, the BIOS must configure the CardBus controller in a Legacy PCIC mode.  You will have to check with your notebook documentation or manufacturer if not sure about this issue.  If your BIOS does not initialize the CardBus controller in Legacy mode you may have to contact the notebook manufacturer for an updated BIOS or configuration utility.

In order to utilize CardBus support, you will need to install third party “Socket Services” available from various vendors such as Phoenix, SystemSoft, or Award.  As stated above, you may need to ensure that the controller is placed in Legacy mode before installing the third party software.  Refer to the documentation provided by the software vendor for this requirement.

To install Legacy PCIC support under Windows95 release 1:

In "Control Panel" do "Add Hardware", "Pcmcia", "PCIC", and exit

In "Control Panel" run "Pcmcia" to enable 32 bit support

Reboot Windows to load the new PCIC driver

Windows95 OSR2 and Windows98:

Both Windows 95 OSR2 and Windows 98 support CardBus mode by default.  However, not all controllers are known to these operating systems.  Even if Windows knows about the particular controller and installs drivers for it you should run the “O2Setup” utility.  Unlike Windows 95 release 1, the controller may be in either Legacy or CardBus mode prior to booting the OS.

To Setup O2Micro CardBus Controllers:

NOTE: Ensure there are no PC-Cards in any slot before setting up CardBus support!  You will also be required to have the Windows Setup CD available.

Insert the O2Micro Setup Diskette in drive A: (or floppy drive of your choice)

Run “O2SETUP.EXE”

Follow the instructions that appear on the screen
(note: you may have to reboot a few times to complete the installation)

If, and when, prompted for the “O2Micro Setup Disk” to locate any drivers, you will have to “Browse”, or manually direct, Windows to find the drivers on the appropriate sub-directory on the setup diskette for the version of the OS being installed.

The three sub-directories available on the diskette are:

“WIN95”		- Drivers for Win95 OSR2

“WIN98”		- Drivers for Win98 (Original Release)

“WIN98SE”	- Drivers for Win98 (Second Edition)

Windows 3.1 or DOS:

Windows 3.1, and DOS for that matter, does not include any support for CardBus or Legacy PCIC.  You will need to install third party “Socket Services” available from various vendors such as Phoenix, SystemSoft, or Award to allow CardBus support.  As with Win95, you may need to ensure that the controller is placed in Legacy mode before installing the third party software.  Refer to the documentation provided by the software vendor for this requirement.

Also, some PC-Card vendors may provide “Direct Enablers” for use in Windows 3.1 or DOS that will configure the CardBus controller in order to support the particular PC-Card.  If no “Socket Services” or “Direct Enablers” are available, you can’t use CardBus or PCMCIA cards under Windows 3.1 or DOS.

WindowsNT 4.0

WindowsNT 4.0, like Windows 3.1 is not a Plug&Play OS.  This OS only has Legacy PCIC support by default.  In order to utilize Legacy PCIC support the BIOS must place the controller in a Legacy PCIC mode.

In order to utilize CardBus support, you will need to install third party “Socket Services” available from various vendors such as Phoenix, SystemSoft, or Award.  As stated above, you may need to ensure that the controller is placed in Legacy mode before installing the third party software.  Refer to the documentation provided by the software vendor for this requirement.

To enable Legacy PCIC support under WindowsNT 4.0:

- Open “Control Panel”, “Devices”

- Find "Pcmcia" entry

- Ensure “Started” at “Boot”

If the “Pcmcia” device can’t be started then the most likely cause is that the BIOS hasn’t placed the controller in a Legacy PCIC mode.

Utilities for O2Micro CardBus PCI Demo Boards

The following section is for installations of O2Micro CardBus PCI demo boards only.  These utilities are for the explicit use of initializing O2Micro CardBus PCI demo boards.  Unless otherwise requested by the Notebook manufacturer, the initialization utilities contained in the “UTIL” sub-directory should not be used on any Notebook.

Utility files on this disk:

Root <directory>

O2SETUP.EXE		- O2Micro Setup Utility

O2SETUP.INI			- O2Micro Setup Initialization file

O2MEMCHK.EXE		- O2Micro Memory Resource Checker

README.DOC		- This document

WIN95 <directory>

O2MCB.INF		- O2Micro Win95 INF file (O2Micro Drivers)

O2MW95.INF		- O2Micro Win95 INF file (Win95 Drivers)

CB725A01.W95	- O2Micro CardBus Driver for Windows 95

CBSSO2.VXD		- Actual destination name of O2Micro driver

CBSS.VXD		- Microsoft CardBus Driver for Windows 95

PCCARD.VXD	- Microsoft PC-Card Driver for Windows 95

PCI.VXD			- Microsoft PCI Driver for Windows 95

WIN98 <directory>

O2MCB.INF		- O2Micro Win98 INF file (O2Micro Drivers)

O2MW98.INF		- O2Micro Win98 INF file (Win98 Drivers)

CB511B22.W98	- O2Micro CardBus Driver for Windows 98

CBSSO2.VXD		- Actual destination name of O2Micro driver

WIN98SE <directory>

O2MCBSE.INF	- O2Micro Win98SE INF file (O2Micro Drivers)

O2MW98SE.INF	- O2Micro Win98SE INF file (Win98SE Drivers)

CB423706.W98	- O2Micro CardBus Driver for Windows 98SE

CBSSO2.VXD		- Actual destination name of O2Micro driver

UTIL <directory>

NOTE: The files in this directory are for “O2Micro PCI DEMO Board” use only.  These files are not required for Notebook systems and should not be used unless specified by the Notebook manufacturer.

O2INIT.DOC		- O2Micro Initialization Utility Documentation

O2INIT.SYS		- O2Micro Initialization Utility Device Driver

O2INIT.COM		- O2Micro Initialization Utility Program

PREG.EXE		- O2Micro PCI Register Dump Utility

O2.EXE			- O2Micro PCMCIA ExCA Register Dump Utility

INT19.COM		- INT19 Soft Reboot program

O2ACPI.EXE		- O2Micro ACPI Mode Utility

O2SW.EXE		- O2Micro Zoom-Video Switch/Enable Utility

O2Micro CardBus PCI Demo Board Initialization Data Files

These data files are only used for initializing O2Micro CardBus PCI Demo Boards!

OZ6812A.DAT	- OZ6812 Rev A

OZ6812B.DAT	- OZ6812 Rev B

OZ6812C.DAT	- OZ6812 Rev C

OZ6812D.DAT	- OZ6812 Rev D

OZ6812D1.DAT	- OZ6812 Rev D1

OZ6832C.DAT	- OZ6832 Rev C

OZ6832C2.DAT	- OZ6832 Rev C2

OZ6833A.DAT	- OZ6833 Rev A

OZ6833B.DAT	- OZ6833 Rev B

OZ6833C.DAT	- OZ6833 Rev C

OZ6833D.DAT	- OZ6833 Rev D

OZ6833D1.DAT	- OZ6833 Rev D1

OZ6860A.DAT	- OZ6836 Rev C and OZ6860 Rev A

OZ6860C.DAT	- OZ6860 Rev C

OZ6912A.DAT	- OZ6912 Rev A

OZ6922A.DAT	- OZ6922 Rev A

OZ6933A.DAT	- OZ6933 Rev A

OZ6933B.DAT	- OZ6933 Rev B

Utility File Descriptions:

NOTE: These files are for “O2Micro PCI DEMO Board” use only.  These files are not required for Notebook systems and should not be used unless specified by the Notebook manufacturer.  As described below,

O2INIT.DOC:

O2Micro CardBus controller initialization utility documentation.

O2INIT.SYS, O2INIT.COM:

O2Micro CardBus controller initialization utility in device driver and executable forms.  The particular CardBus controller (O2Micro demo board) under test must be initialized prior to use.  This includes loading any “Socket Services”, booting any OS or using any “Direct Enabler”, to name a few.  The “O2Init.Sys” statement should be placed in “Config.Sys” as the first line in the file.  This will ensure that the controller is initialized BEFORE any other drivers that may want to access PC-Cards in the slots.

The syntax is as follows:

Device=O2INIT.SYS <appropriate initialization data file>

(the appropriate data file must be used to correctly initialize the controller)

PREG.EXE:

A DOS utility used to view/edit PCI configuration space of any PCI device.

O2.EXE:

A DOS utility used to view/edit PCMCIA ExCA registers.

O2SW.EXE:

A Windows utility used to control O2Micro Zoom Video features.

O2ACPI.EXE:

A Windows utility used to set ACPI modes of an ACPI CardBus controller.

INT19.COM:

A DOS utility used to “Soft-Reboot” the system after controller initialization.

Debugging an O2Micro CardBus controller

A CardBus controller is a PCI device with standard PCI registers.  The CardBus controller registers are memory mapped at the location set in PCI register 10h.  Under Win95/98, the device manager properties can be examined to find that “Physical” memory location.  However, Win9x maps this physical memory address to a “Logical” memory address.  This makes debugging the CardBus controller a bit tricky.  The easiest way to debug CardBus controller problems is to use SoftIce from Numega.

The O2Micro Cardbus Controller is a single chip with 2 PCI devices (functions).  Windows9x views the chip as 2 separate PCI devices so there are two O2Micro entries in the Device Manager.  As such, each function (slot) will have a separate memory range assigned to it.  These two memory ranges are not necessarily contiguous.  Also as PC-Cards are inserted and removed, these memory ranges will move.  This makes tracking/trapping Windows accesses to various CardBus or ExCA registers a little tough.