# Introduction

The modem software drivers described in this note were built on April 2, 1999.

This is a release candidate version of V.90 drivers for Apollo ISA platform and Mars PCI platform.

Apollo - This version of drivers will run on DSP1642-CC, DSP1641-CC, DSP1643-CH, DSP1644-CH, DSP1641B-CH  ROM code. 56K and V.90 will work only on the CH ROM codes; this is a run time check option.

Mars - this version of drivers will run on DSP1645 and DSP1646 - AB ROM code.

This version of drivers does not require the external SRAMS for K56flex or V.90.

# Changes Since Version 5.43 for Apollo and Mars

99/03/16 15:47:00 atv

Vendor Specific Change : Vendor ID 0x0E11 :   Transmit level parameter # 2f decreased by 2. The change earlier

did not take effect for CTR21.

99/03/17 10:48:42 sarkar

Vendor Specific Change : Win NT :  ACER BIOS machines, modified driver to enable reading of Comm Address

in Windows NT from the BIOS rather than the “ltports.inf” file.

99/03/18 20:32:35 sarkar

LTMSG.EXE : Program to display messages for ‘invaild country’ and ‘over current’. Bug reported in Win NT. Modified to check message is enabled in ‘lthomol.ini’ before launching in Windows NT.

99/03/19 14:03:47 vr

Vendor Specific Change : Vendor ID 0x1436 :   For only subsystem id 0x0440 DLP, digital line protection is enabled,  for other id's DLP is disabled.

99/03/19 16:30:16 trevor

Vendor Specific Change : Vendor ID 0x1179 : Win NT Toshiba build, modified the function to update PCI resources. The driver was  updating the first  PCI  I/O window to the standard com I/O space. The driver now uses the I/O window allocated to us,  and does not modify it.

99/03/22 11:25:03 atv

Busy Detection : Problem reported in the field for North America, busy tone not detection when the modem is behind a 1AESS central office switch.  Changed busy detection cadence to cover the frequencies from a 1AESS switch.

99/03/22 11:25:03 atv

Homologation : Italy Dial Tone Detection : Problem reported in the field for Italy, dial tone not detected. The dial tone of cadence of  200/200/600/1000 for Italy was good, but the cadence  600/100/200/200 did not work with TAS. It was observed that with the second sequence the last 200 ms off cadance was longer than expected. This may be

TAS problem, changing cadence limits in the code fixed the problem.

99/03/23 19:04:55 sarkar

Win 95 : Country Selection for Win 95. The driver now supports different countries with the TAPI dialing properties,  the same as Win 98. The country select application provided with earlier drivers could be used instead of the TAPI method.

99/03/24 09:26:27 atv

Vendor Specific Change : Vendor ID 0x1179 : D3 Cold support is not provided by the modem driver.

99/03/24 09:38:30 atv

Homologation : Added new country id’s for Argentina (0x33), Bolivia (0x34), Chile(0x35), Colombia(0x36), Ecuador(0x37), Panama(0x38), and Peru (0x39).

99/03/25 11:54:14 atv

Vendor Specific Change : Vendor ID 0x1668 : For subsystem id 0x0440 and 0x0500 the ring detection frequency range changed to detect ring with half and full wave detector.

99/03/25 12:07:10 edeng

PC 99 : AT commands : Added the following AT command and their responses. For details please see the new AT command specification or the V.25ter specification.  Microsoft’s V.250 modem INF file sends the following init string to the modem, “AT+ILRR=1;+MR=1;+ER=1;+DR=1<cr>”

+DR – “AT+DR=1” or “AT+DR=0”, for compression on/off reporting.

+MR – “AT+MR=1” or “AT+MR=0” for modulation on/off reporting.

+ER – “AT+ER=1” or  “AT+ER=0” for error control of/off reporting.

+ILRR – “AT+ILRR=1” or “AT+ILRR=0” for DTE rate on/off reporting.

+ES – will determine the error correction mode.

+ES=1,0,1 – buffered mode

+ES=0,1,0 – direct mode

+ES=4,4,6 – MNP or disconnect mode

+ES=2,3,5 – LAPM or disconnet mode

+ES=3,3,5 – LAPM or disconnect mode

+ES=4,0,6 – MNP or buffered mode

+ES=3,0,2 – LAPM, MNP, or buffered mode (default)

+ES=2,0,2 – LAPM or buffered mode

+ES=3,2,4 – LAPM, MNP or disconnect mode

+IFC – will determine the local flow control method.

+IFC=0,0 – no flow control

+IFC=1,1 – software flow control, XON/XOFF characters processed by modem

+IFC=3,1 – software flow control , XON/XOFF characters passed to DTE

+IFC=2,2 – hardware flow control

+DS – will determine the data compression method.

+DS=3,0 – V42bis

+DS=0,0 – no compression

+DS=0 – no compression

99/03/26 09:05:20 vr

Vendor Specific Change : Vendor ID 0x8086, Subsystem ID 0x0034 :  Vendor ID 0x115D, Subsystem ID 0x0005 Added support for Apollo modem behind Intel 82559 chip. The following configuration works for Win 95/98 but not for Win NT yet.

99/03/26 15:02:03 atv

Homologation : Fixed blacklisting problem with TAPI  dialing. When the call disconnected the modem driver is re-initialized so the blacklisting table and all timers got reset. Now the blacklist & timer is done only once when the modem drivers are first loaded.

99/03/29 16:02:02 atv

Homologation : For Poland, Czech, and Slovakia 900 msec pulse dial inter-digit delay is used.

99/03/30 09:31:06 trevor

Vendor Specific Change : Vendor ID 0x1179 : Win 95/98 Toshiba build, the modem drivers do not virtualize the

standard COM I/O and IRQ for DOS box applications. The modem driver virtualizes the actual resources allocated by Win 95/98.

99/03/30 09:57:00 trevor

Homologation : Win NT - Modified the default country setting logic. We now use the default country setting  (LTHOMOL.INI default Country ID), if the update flag is set or if the country setting in the modem driver is zero. The NT4 driver works the same way as the Win 95/98 driver.

99/03/31 15:22:18 edeng

V.90 – Bug reported in the field when dropping back to V.34 from a V.90 connection. The modem sometimes keeps falling back or falling forward due to lapm errors or if the MSE is bad during the call. A previous change to fix a problem with an old Rockwell modem was done in version 5.43 release. This change or fix need not be done when in V.90 mode or when falling back to V.34 from V.90. This fixes this problem.

99/04/01 14:21:04 atv

Mars – Problem reported in the field with certain boards with Mars 2 chips, the board does not wake up properly on power up.  Modified the modem initialization routine to properly reset the Mars chip.

99/04/02 09:36:37 sarkar

LTMSG.EXE : Program to display messages for ‘invaild country’ and ‘over current’. Bug reported in  these languages. Fixed the Simplified Chinese, Traditional Chinese and Korean sections.

99/04/02 10:31:26 haolinxu

LTREMOVE.Exe – This program will remove the modem, and delete all files and registry entries for the modem. This program will work for Win 95/98 and Win NT.

99/04/02 10:31:26 haolinxu

Setup Program  – Version 2.29 for English and Version 3.29 for Global. The following changes were made to the modem setup program :

- silent install  will use the same setup.iss file for Win 95/98 and Win NT.

- The homecountry option will support Win 95/98 and Win NT.

- Polish language is added to the global setup program

99/04/02 12:23:29 vr

Mars/Apollo V.90 Version 5.44

# Open Issues/Bugs

Lucent will update the list of all open issues/bugs etc.  Testing is in progress.

**** END ****