Released Notes

Title

BIOS V1.13 for Canary(2 Spindle)/TMC300

Release Notice Date

2005/01/06

BIOS Release Date

2005/01/06

Originator

Eddie Wang

Eddie Wang @wistron.com.tw

Product Data

Media Type : Floppy

Support System List : Canary

LEVEL: MP

Version : TMC300 V1.13

BUILD ID : CQB113WW

CHECK SUM :  0x7AB74328 (CRC -L)

Virus Check : OfficeScan (Pattern No. 2.333.00)

-- Special Notes, Restrictions/Causions, Known Issues

1. For Canary 2 spindle.

2. For 5705 Lan chip machines

3. (1) MUST use /BBL to flash this BIOS, or your system maybe not able to BOOT!

(2) Don't use WflashAP to flash older BIOS (Can-V0.28 or before), or your

system maybe not able to BOOT and NVS data will be lost!

-- How to clean install:

DOS Mode:

1. Create a bootable disk.

2. Copy the ZIP file to your HDD. The recommended directory is C:\drivers\BIOS.

3. Then unzip it with -d option to store the sub-directories.ex.)PKUNZIP -d C:\drivers\BIOS\*.ZIP C:\drivers\BIOS\

4. Copy all the files which in directory C:\drivers\BIOS\ to the bootable disk.

5. Put the bootable disk into Canary machines, then boot.

Windows Mode:

1. Copy the ZIP file to your HDD. The recommended directory is C:\drivers\BIOS.

2. Then unzip it with –d option to store the sub-directories.

Ex.)PKUNZIP –d C:\drivers\BIOS\*.ZIP C:\drivers\BIOS

3. Boot to Windows mode. In directory C:\drivers\BIOS\, run “Can-V113.exe”.

-- How to over install:

It is the same as clean install.

-- How to uninstall:

No way.

-- How to check version:

When system POST, press F2 key to show information on screen.

-- How to do the unattended install:

No way.

-- Update Information

| Version | Comments |
| --- | --- |
| 0.41 | (1) Combine with BroadCom 5705 GigaLAN BootROM * B57BASE.BIN, 10/22/2002 * B.LOM, 12/06/2002 (2) always set cmosMGMxCFG to 0 (auto) for the issue that POST will not show error message when set CMOS to 0FFh. (3) Modify "Advanced" page of SETUP. - remove some items (4) Modify the assigned IRQ of PCI devices after POST |
| 0.42 | (1) Remove the small Phoneix LOGO in POST LOGO by Customer's request (2) Modify SETUP menu - modify default setting - modify the gray item when enter SETUP by USER Password - modify "Enable/Disable" to "Enabled/Disabled" for HDD Password |
| 0.43 | (1) Modify the algorithm for disable F2/F12 when S4 resume -> for some boards will go and keep S4 path in the beginning without boot into OS (2) CanaryII just support XP, BIOS doesn't need to change CPU speed when T4 (3) When issue Int15h with AX=9612h/BL=2Fh, the AutoDim function should active immediately (4) Remove the WA about _BST for Coldboot MBA. (5) When checking G768 setting . Dual-battery support should be disabled and set KBC SMbus to BatteryA. |
| 0.44 | (1) A. Implement new AutiDim algorithm for Annie. B. When issue Int15h with AX=9612h/BL=2Fh, the AutoDim function doesn't need to active immediately. (2) Remove "Parallel port:" in "Advanced" menu of SETUP - when port-replicator is attached, POST set LPT to ENABLED - when port-replicator is not attached, POST set LPT to DISABLED (3) Combine with new VGA BIOS for the issue that Power-DVD hangs up - MTG30566.CAN, 08/11/2003 - MBI30566.CAN, 09/26/2003 |
| 0.45 | (1) Combine with BroadCom 5705 GigaLAN BootROM for the String of "Boot LAN" in "Boot" menu of SETUP * B57BASE.BIN, 10/22/2002 * B57UNDI1.LOM, 09/03/2003 (2) add 1s delay to prevent the status of LID/TbletButton change in short time (3) There will be a strange "CLICK" of Digitizer when LightSensor is ENABLED. |
| 0.46 | (1) Implement the solution of "Port-Replicator" declaration in ACPI (2) Set the default setting of "AutoDim" to "DISABLED" |
| 0.47 | (1) Set the default setting of "Wireless" to "ENABLED" |
| 1.00 | (1) Fix boot-up with Tablet coverup closed, touchpad will be disabled always! |
| 1.01 | (1) Modify the Bar Image displayed with POST LOGO (2) Set Index 30h[0] = 0b of SIO LPT when LPT is disabled under POST (3) With new Intel 802.11 wireless cards that non-defined before for Canary - * Int15h with AX=9610h/BX=0235h -> return AL=0 for 802.11b/a+b/a+g/g non-exist * Int15h with AX=9610h/BX=A235h -> return AL[0]=1b for 802.11x wireless exist (4) When HDD password is Enabled and modify password by Notebook Manager, HDD will be locked! (5) Phase in the following thermal solution * G768 Index F1h can't be set any value. * Set G768 Index 09h to the FIXED value 92h. * Set KBC RAM bank 5 index 99h=9Ah=0 * Only check G768 Index 09h = 92h, and retry 2 times. If it is not equal, then power off the system. * Still set G768 Index 0Ah to the value 07h. |
| 1.02 | (1) POST with battery only, system sometimes auto-shutdown when press F2 continuously. (about 6/1000). OK with AC. |
| 1.03 | (1) New Intel mini-PCI Wireless LAN cards and the DID is not 1043h -> Only check Intel VID 8086 to identify Intel mini-PCI Wireless LAN cards. |
| 1.04 | (1) Support Dothan A1/A2 stepping CPU - modify speedstep table - combine with new Bucode DothanA2.ASM |
| 1.05 | (1) Fix UUID with 00h is wrong in NBM and DMI table. |
| 1.06 | (1) Annie Int15 SPEC - AX=9610h/BL=35h When BH=02, AL = Bit0=0: no 802.11b device =1: 802.11b device exist Bit1=0: no 802.11a+b device =1: 802.11a+b device exist Bit2=0: no 802.11a+g device =1: 802.11a+g device exist Bit3=0: no 802.11g device =1: 802.11g device exist Bit4=0: no 802.11b+g device =1: 802.11b+g device exist Notes : The current supported Wireless LAN Cards: VID DID SVID SSID Intel 802.11b : 8086 1043 8086 2527 Intel 802.11b : 8086 1043 8086 2522 (new) Intel 802.11a+b: 8086 1043 8086 2524 Intel 802.11a+b: 8086 1043 8086 2521 Intel 802.11b+g: 8086 4220 Ambit 802.11b : 1260 3872 1648 0200 Ambit 802.11a+b : 168C 0012 168C 2022 Ambit 802.11g : 1260 3890 1468 0210 Ambit 802.11a+g : 168C 0013 1468 0404 (2) Modify EC.ASI to disable WACOM (set 30h[0]=0b) when toggle Display to CRT only and enable WACOM (set 30h[0]=1b) when toggle display to LCD or BOTH. |
| 1.07 | (1) Support Dothan B1 stepping CPU (CPUID=6D6h). (2) Remove Dothan A0 stepping CPU. (3) Update Banias and Dothan CPU P-State Table Ver 1.1. |
| 1.08 | (1) Key sometimes crazy with SoftButton / NootbookManager. - modify the algorithm to return BrightnessLevel for Int15h with AX=9612h/BX=0007h/0207h. |
| 1.09 | (1) Support Dothan Celeron M CPU (CPUID=6D8h). - modify CPU BU code - modify Intel OSB - modify ACPI table - modify BIOS SETUP Information - modify DMI type 4 |
| 1.10 | (1) Light Sensor cannot change brightness after BIOS V1.08. - modify the Int15h with AX=9612h/BX=0107h/0307h. |
| 1.11 | (1) Support Banias Celeron M B1 stepping CPU (CPUID=695h). - remove BU code of Banias A1 stepping (for reducing code size) - modify CPU BU code - modify Intel OSB |
| 1.12 | (1) Update Intel OSB 2.82 (1024x768x16). - A7N4ICPM.BMP for Celeron M - A7N4PMP.BMP for Pentium M - A7N4CMT.BMP for Centrino (2) System will hang/abnormal when moving or clicking USB Mouse during BIOS POST or booting to OS. |
| 1.13 | (1) Add thermal table for Tj85 CPU. |