## BR10 BIOS 1.40A Release Check List

Test Purpose : Base on BIOS version 1.40A to check basic function no bad than old version and confirm fixed items.

Test Configuration: Please refer to the system configuration below:

# System Configuration  			Release Date: 09/14/2002

| Test System | Compal Model Name → BR10 Customer Model Name →Aspire 1400 |  |  |  |
| --- | --- | --- | --- | --- |
| CPU Type | Pentium 4 1.6 GHz | Chipset Software Installation utility | V3.40.1002 |  |
| L2 Cache | 256 KB | VGA Driver | ATI M6 6.13.10.6015 |  |
| Memory Size | 256 MB | Audio Driver | Crystal WDM Audio Codec 6.13.10.4103 |  |
| Hard Disk | IC25N040ATCS04-0 | Modem Driver | Smart Link 56K 2.92.4.1599 |  |
| DVD-ROM | Toshiba SD-R2502 | LAN Driver | Intel PRO Network Adapter V5.41.35.0 |  |
| Internal FDD | Panasonic | Wireless LAN Driver | Gemtek Mini PCI V1.7.37.0 |  |
| Battery info | Li-Ion 12 cells | Easy Button | Dritek CR1.006 |  |
| System BIOS | V 1.40A | Touch Pad | ALPS 5.3.201.9 |  |
| Keyboard BIOS | V 1.40A | Panel Type | IBM 15.0” SXGA+ |  |
| Operation System |  | Windows XP Professional Edition |  |  |

Please refer to the tables below for testing procedure, criteria and result.

| Flash BIOS |  |  |  |  |
| --- | --- | --- | --- | --- |
| No. | ### Check Item | Pass | Fail | Remark |
| 1. | Make sure BIOS ROM check sum match BIOS release note. (Check sum：B20A) |  |  |  |
| 2. | PHLASH /B=BR10ALL5.BIN BR10140A.ROM /X /mode=3 /c /PF="NRBT" |  |  |  |
| 3. | #### Check system will shut down after flash BIOS |  |  |  |
| 4. | Check BIOS version during POST procedure |  |  |  |

| BIOS Release Note (Solved Issues) Check List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | Issue No. | ### Check Item | Pass | Fail | Remark |
| 1. |  | ### Fix: Can't change boot sequence of NETWORK item on the system including PC87591S C2 version chip. |  |  | SYS |
| 2. |  | ### Update CPU Microcode for NW C1-Step CPU. |  |  | SYS |
| 3. |  | ### Fix system sometimes abnormal shutdown |  |  | EC |

| System Function Check List |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No. | ### Boot Test |  | Pass | Fail | Remark |
| 1. | Cold Boot from internal FDD |  |  |  |  |
| 2. | Warm Boot from internal FDD (Ctrl+Alt+Del) |  |  |  |  |
| 3. | Cold Boot from HDD |  |  |  |  |
| 4. | Warm Boot from HDD |  |  |  |  |
| 5. | Press “C” key to force system booting from CD-ROM/DVD/CD-RW/Combo during POST of cold boot/warm boot. (Without any error message) |  |  |  |  |
| 6. | Make sure the MDP cold boot time is less than 40 seconds (If double byte OS is 45 seconds) |  |  |  |  |
| 7. | Make sure the MDP warm boot time is less than 40 seconds (If double byte OS is 45 seconds) |  |  |  |  |
| 8. | Make sure the MDP standby resume time is less than 5 seconds. |  |  |  |  |
| 9. | Make sure the MDP hibernation resume time is less than 25 seconds. |  |  |  |  |
| No. | ### Power On Test by Manually |  | Pass | Fail | Remark |
| 1. | Press the Power Button to power system on with AC adapter only. (3 times) |  |  |  |  |
| 2. | Press the Power Button to power system on with Main Battery only. (3 times) |  |  |  |  |
| No. | ### Operating System/Driver Installation |  | Pass | Fail | Remark |
| 1. | Install the Operating System by manually. |  |  |  |  |
| 2. | Install all drivers without any error. (No “!” or “?” mark in Device management) |  |  |  |  |
| 3. | Make sure the DMI string can be written into system. |  |  |  |  |
| 4. | Restore the Operating System from the recovery CD without any error or hang up. |  |  |  |  |
| No. | ### Hot Key Function Check |  | Pass | Fail | Remark |
| 1. | Fn + ESC | Master Volume On/Off |  |  |  |
| 2. | Fn + F3 | Standby |  |  |  |
| 3. | Fn + F4 | Hibernation |  |  |  |
| 4. | Fn + F5 | Power on display(Support TV-out) Switch sequence : LCD only, CRT only, TV only, LCD+TV, LCD+CRT, CRT+TV, LCD+CRT+TV (If CRT or TV not be applied, all of CRT or all of TV items will be ignored.) |  |  |  |
| 5. | Fn + F6 | Decrease brightness |  |  |  |
| 6. | #### Fn + F7 | #### Increase brightness | ####  |  |  |
| 7. | #### Fn + F8 | #### Wireless LAN Switch | ####  |  |  |
| 8. | Press Fn+F5 without monitor connection, the display mode should not do any change. |  |  |  |  |
| 9. | Check sticky key function for above hot keys. |  |  |  |  |
| 10. | Resume from Standby/Hibernate, and then check sticky key function for above hot keys. |  |  |  |  |
| No. | ### Dritek Easy Button Function Check |  | Pass | Fail | Remark |
| 1. | #### Easy Button function check with Dritek Easy Button program installed under OS. (Eject/Stop, Play/Pause) |  |  |  |  |
| 2. | #### Check Easy Button function after resume from standby/Hibernate/Restart/Cold boot. (Eject/Stop, Play/Pause) |  |  |  |  |
| 3. | Direct CD function check when system is off. (Eject/Stop, Play/Pause…) |  |  |  |  |
| No. | ### Peripheral Device Function Check (Check two times) |  | Pass | Fail | Remark |
| 1. | Check all storage devices (FDD, USB FDD, HDD, CD/DVD-ROM/CD-RW etc.) are working properly. |  |  |  |  |
| 2. | Make sure all storage devices (FDD, USB FDD, HDD, CD/DVD-ROM etc.) are working fine after resume from standby/Hibernate. |  |  |  |  |
| 3. | Check the USB keyboard and mouse are working properly. |  |  |  |  |
| 4. | Make sure the USB keyboard and mouse are working fine after resume from standby /Hibernate/Restart/Cold boot. |  |  |  |  |
| 5. | Make sure 1394 GUID can be read/written from system. (Write a 1394 GUID into system, and then read for 3 times to check.) |  |  |  | N/A |
| 6. | Check the 1394 NIC is working fine. |  |  |  | N/A |
| 7. | Make sure the 1394 NIC are working fine after resume from standby /Hibernate/Restart/Cold boot. |  |  |  | N/A |
| 8. | Check the 1394 device (MO) is working properly. |  |  |  | N/A |
| 9. | Make sure the 1394 Device (MO) is working fine after resume from Standby /Hibernate/Restart/Cold boot. |  |  |  | N/A |
| 10. | Check the MDC Modem is working properly. |  |  |  |  |
| 11. | Make sure the MDC Modem is working fine after resume from Standby/ Hibernate/Restart/Cold boot. |  |  |  |  |
| 12. | Check the on board LAN is working properly. |  |  |  |  |
| 13. | Make sure the on board LAN card is working fine after resume from Standby/ Hibernate/Restart/Cold boot |  |  |  |  |
| 14. | Check the Wireless LAN is working properly. |  |  |  |  |
| 15. | Make sure Wireless LAN card is working fine after resume from Standby/ Hibernate/Restart/Cold boot |  |  |  |  |
| 16. | Check the LPT port (HP LaserJet 5000) function is working fine. (Without any error message) |  |  |  |  |
| 17. | Make sure the LPT port (HP LaserJet 5000) is working fine after resume from Standby/Hibernate/Restart/Cold boot. (Without any error message) |  |  |  |  |
| 18. | Check the FIR function by transfer files between two FIR systems. |  |  |  | N/A |
| 19. | Make sure the FIR transfer can work between FIR systems after resume from Standby/Hibernate/Restart/Cold boot. |  |  |  | N/A |
| 20. | Notices that fan have been turned on when system power on. |  |  |  |  |
| No. | ### ACPI Power Management (Standby, Hibernation/Resume) Check |  | Pass | Fail | Remark |
| 1. | Standby by Power Button (Total 10 times, after Standby for 5 times do cold boot once.) |  |  |  |  |
| 2. | Standby by close panel for 10 seconds. (Lid switch set to “Stand By “) (Total 10 times, after Standby for 5 times do cold boot once.) |  |  |  |  |
| 3. | Standby by Start Menu \Shut Down \Standby. (Total 10 times, after Standby for 5 times do cold boot once.) |  |  |  |  |
| 4. | Standby by timeout (Total 10 times, after Standby for 5 times do cold boot once.) |  |  |  |  |
| 5. | Hibernate by Power Button (Total 10 times, after Hibernation for 5 times do cold boot once.) |  |  |  |  |
| 6. | Hibernate by close panel for 10 seconds. (Lid switch set to “Hibernate”) (Total 10 times, after Hibernate for 5 times do cold boot once.) |  |  |  |  |
| 7. | Hibernate by Start Menu \Shut Down \Shift -> Hibernate. (Total 10 times, after Hibernate for 5 times do cold boot once.) |  |  |  |  |
| 8. | Hibernate by timeout (Total 10 times, after Hibernate for 5 times do cold boot once.) |  |  |  |  |
| 9. | Wake on Ring (Use Hyper terminal to test. After system was waked, system still can work on call out without disconnect and re-create a new connection.) |  |  |  |  |
| 10. | Wake on LAN |  |  |  |  |
| 11. | Check all LED indicators are working fine and correct. |  |  |  |  |
| 12. | Check all LEDs are working properly after resume from Standby, Hibernate |  |  |  |  |
| 13. | Force the system entering standby by close panel (Lid switch set to Standby) during shutdown the system. |  |  |  |  |
| 14. | Force the system entering Hibernate by close panel (Lid switch set to Hibernate) during shutdown the system. |  |  |  |  |
| 15. | Make sure the low/critical battery alarm tone was generated and keep auto update remaining capacity of the battery. |  |  |  |  |
| No. | ### PCMCIA Function Check (Please follow procedures as below) |  | Pass | Fail | Remark |
| 1. Make sure the system can auto-detect the device while hot insert the PCMCIA card. 2. Make sure the PC card driver have setup complete without any error or hang up. 3. Make sure no exclamation mark in device manager when PC card was inserted. 4. Check the PCMCIA card function is working properly. 5. Make sure the both upper socket and lower socket are working properly. 6. Make sure no exclamation mark in device manager after resume from Standby/Hibernate with PC card inserted. 7. Make sure the both upper socket and lower socket are working properly after resume from Standby/Hibernate. 8. Make sure the device does not list in device management after plug out the PC card. 9. Plug in two difference type of PC card into upper socket and lower socket, and then check item 1 to item 8. |  |  |  |  |  |
| 1. | Check the APA – 1480 SCSI Card (Rev. A) function |  |  |  |  |
| 2. | #### Check the REX-R231P SCSI Card (Ultra SCSI III 16 bit) function |  |  |  |  |
| 3. | #### Check the 3Com Megahertz 10/100 LAN+56K Modem (3CCFEM556B) function |  |  |  |  |
| 4. | #### Check the Xircom CBE2-100 Card Bus Ethernet card function |  |  |  |  |
| 5. | #### Check the Hagiwara SYS-COM Smartmedia Adapter (HPC-SVO3A) function |  |  |  |  |

Conclusion:

Base on above test result, this version BIOS V1.40A is better than old version. We agree to release BIOS V1.40A

Department : Software Application Dept.

Engineer : Kesin Chen