Kilimanjaro1.0

| File Link | Checksum | Date | EC BIOS | VBIOS | Release Note | Source Code |
| --- | --- | --- | --- | --- | --- | --- |
| V110.zip | 09E048CC | 2008/07/08 | 310 | 1533 | 1. Fix GM black screen and PM yellow bans issue. |  |
| V109.zip | 09E15306 | 2008/07/03 | 310 | 1533 | 1. Update EC code to 310. |  |
| V108.zip | 09DFD9CE | 2008/06/19 | 309 | 1533 | 1. Update EC code to 309. |  |
| V107.zip | 09DFD9D4 | 2008/05/19 | 308 | 1533 | 1. Update EC to 3.08. 2. Solve blue screen on M86 (Disable SERR_EN). |  |
| V106.zip | 09E14A8F | 2008/04/03 | 306 | 1533 | 1. Fix Panel no display when user close lid immediately after power on. 2. EC update to .306 for LTN160HT01-A02 |  |
| V105.zip | 09E16882 | 2008/03/28 | 305 | 1533 | 1. Solve sometimes POST hang at B3h with Penryn CPU. 2. Make machine auto-reboot after flash BIOS in DOS mode. 3. LCD-EDID renew 4. EC update to .305 |  |
| V104.zip | 09E1AE9B | 2008/03/21 | 304 | 1533 | 1.Change back password on boot default setting to disable 2.EC update to .304 |  |
| V103.zip | 09E1B29A | 2008/03/20 | 303 | 1533 | 1. Re-fixed Kili_CS_GM Monitor power status test Fail. 2. Fix setup menu Network boot item enable/disable no function 3. Intel VBIOS modified by BMP, renew Panel #16 minimum brightness to 44/255. | 03/20 |
| V102.zip | 09E1D799 | 2008/03/03 | 302 | 1533 | 1. Remove all USB ports _PRW and _PSW method. 2. EC update to .302 |  |
| V101.zip | 09E1B4E4 | 2008/02/29 | 301 | 1533 | 1. Change acer logo which is requested from acer S/W PM sandy. 2. Fix there are no Manufactory name and Device name in acer battery utility when using 4400m battery. 3. Fix sometimes brightness can not adjust to max. or min. on GM SKU. 4. Update EC to EcTeton.301 |  |
| V100.zip | 0994FF34 | 2008/02/27 | 300 | 1533 | 1. Fix after Arcade button wake S3, restart function will enter IO Arcade instead of Vista. 2. Fix battery information 3. Modify JMB38x 197B/2382h offset DEh[7:4] setting to 1010b 4. Modify Tjmax check rule |  |