Teton

| File Link | Checksum | Date | EC BIOS |  | VBIOS |  | Release Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| V106.zip | 09E1A686h | 2008/04/18 | EcTeton.307 |  | Extern VGA | 1.Update Ec to 307,support WXGA panel 2.Teton support ATI VGA |  |
| V105.zip | 09E10721h | 2008/03/28 |  | EcTeton.305 | Extern VGA | 1. Solve sometimes POST hang at B3h with Penryn CPU. 2. Make machine auto-reboot after flash BIOS in DOS mode. 3. LCD-EDID renew 4.EC update to .305 |  |
| V104.zip | 09E22A13h | 2008/03/21 |  | EcTeton.304 | Extern VGA | 1.Change back password on boot default setting to disable 2.EC update to .304 |  |
| V103.zip | 09E23B12h | 2008/03/20 |  | EcTeton.303 | Extern VGA | 1. Fix setup menu Network boot item enable/disable no function 2. Intel VBIOS modified by BMP, renew Panel #16 minimum brightness to 44/255. |  |
| V102.zip | 09E28014h | 2008/03/03 |  | EcTeton.302 | Extern VGA | 1. Remove all USB ports _PRW and _PSW method. 2. EC update to .302 |  |