Cheela 2.0

| File Link | Checksum | Date | EC BIOS | VBIOS | Release Note | Source Code |
| --- | --- | --- | --- | --- | --- | --- |
| V120.zip | 08E3BD2B | 2007/01/07 | V3.07 | Extern VGA | 1. Update Penryn CPU reset workaround. |  |
| V119.zip | 08E39F28 | 2007/01/02 | V3.07 | Extern VGA | 1. Add L7300/L7500 CPU support. 2. Remove S3 restore brightness level under Vista. |  |
| V117.zip | 08E9C17A | 2007/12/21 | V3.07 | Extern VGA | 1. Solve Audio lag when using T9300/T9500 CPUs. 2. Workaround code for Intel Penryn CPU false Thermaltrip. |  |
| V116.zip | 08E36DF5 | 2007/12/14 | V3.07 | Extern VGA | 1.Fix setup screen password garbage issue. 2.Fix WHQL SMBIOS item fail. 3.Fix PenrynCPU lag issue. |  |
| V115.zip | 08E43608 | 2007/12/10 | V3.07 | Extern VGA | 1.Update new camera HardwareID for Suyin and Bison. 2.Update Penryn CPU(CPU Signature 10676h, Revision ID 607h) microcode. |  |
| V114.zip | 08EC5FDB | 2007/11/21 | V3.07 | Extern VGA | 1.Fix External VGA/DVI output issue. |  |
| V113.zip | 08F238CC | 2007/10/16 | V3.07 | Extern VGA | 1.Fix set South Bridge throttle function not match WMI spec. 2.Update EC to V3.07 |  |
| V112.zip | 08F36828 | 2007/09/29 | V3.06 | Extern VGA | 1. Support Penryn processor in Santa Rosa. 2. Update PPM code support C6 and Penryn processor. 3. Update Penryn processor SV Tj,max increased to 105c 4. Update Penryn CPU(CPU Signature 10671h, Revision ID 106h) microcode. 5. Clovertown CPU L2 cache size incorrect on post and DMI type7. 6. Update Penryn CPU(CPU Signature 10674h, Revision ID 405h) microcode. 7.Update EC to V3.06 |  |
| V11101.zip The BIOS only test Penryn CPU bootleg | 08F2D3B1 | 2007/09/11 | V3.05 | Extern VGA | 1. Support Penryn processor in Santa Rosa. 2. Update PPM code support C6 and Penryn processor. 3. Update Penryn processor SV Tj,max increased to 105c 4. Update Penryn CPU(CPU Signature 10671h, Revision ID 106h) microcode. 5. Clovertown CPU L2 cache size incorrect on post and DMI type7. 6. Update Penryn CPU(CPU Signature 10674h, Revision ID 405h) microcode. | V11101.zip |
| V111.zip | 09058458 | 2007/09/10 | V3.05 | Extern VGA | 1. Fix cheela20 4G memory issue. 2.Add compiler option for Cheela20 when S3 resume |  |
| V110.zip | 09055F80 | 2007/08/31 | V3.05 | Extern VGA | 1.Replace V109.rom 2.Support SST25VF080B flash part |  |