Acer-suspend-to-fixed-disk program

������������������������������������

Version 1.3 R 1.1

This program locates free contiguous disk spaces and allocates them for

storing BASE MEMORY, EXTENDED MEMORY, VIDEO MEMORY, and SM RAM.  The  disk

spaces found are reserved for system use  and  are no longer accessible to

the user. If the program cannot find the required disk spaces,it shows the

error message  "Not enough disk space for allocation".   The disk may have

enough free spaces but these free spaces exist as small fragments. In this

case,  you  can  use  some tools such as SPEEDDISK in PCTOOLS or DEFRAG in

DOS 6.0 or above to compact your free disk spaces.   Then you can run this

program again.

Why does this program need to allocate disk spaces?    When most of the

power is removed from the system, (e.g. due to suspend timer time-out) the

system  swaps all memory images  and  saves all system status  information

into these disk spaces. When you resume the system from suspend mode,  the

system will return to its previous state.   If you do not run this program

to allocate the required disk spaces,  this function will not work .  Once

this program is executed,  the system will have the 'Suspend-to-Hard-Disk'

feature and you do not need to run it  again  except that the data in CMOS

is lost or corrupted. (For example, system configuration changed by adding

This utility,  ASTDK, is provided for both the  DOS-based user and  the

Windows-based user. The command line syntax is described as below.

This program has four paramaters:

/HELP or /?  : Shows a help screen briefly describing how to use the

parameters.

/CREATE=size : Allocates contiguous disk spaces for swapping memory image

(unit=KB)     and saving system registers.  We suggest that you allocate

the  required  disk spaces before you start using your PC.

You may specify the size of the disk space which you  want

to allocate.   If the size value you assigned is less than

the  system  memory size,  the program will use the system

memory size instead.The default value is the system memory

size.The program will add additional disk spaces for VIDEO

MEMORY,  SM RAM,  and  HEADER information  whose sizes are

described in NOTICE below.       If you want to resize the

allocated disk spaces, you have two options:

[1]  First execute this program with /DELETE parameter

described  below and then run this program with this

parameter  again;

[2]  Directly execute ASTDK with this parameter to  resize

the needed disk spaces automatically.

If the data in CMOS is lost or corrupted and you run this

program with /CREATE parameter, the program will auto-detect

and then allocate the disk spaces.         The program will

reallocate the disk space based on the new size you specify.

/DELETE      : Use this parameter to free the disk space allocated by the

program.

Notice :

1. If you do not specify any paramater when you run ASTDK under DOS

prompt, and you have already generated reserved area in your hard

disk, this utility will prompt you current size of reserved area.

2. Reserved areas are stored in the format of SYETEM/HIDDEN/READ_ONLY

File in Hard Disk.

3. While using /CREATE parameter for creating reserved disk spaces, this

program will add more values,    including VIDEO MEMORY, SM RAM  and

HEADER information, to the size value that you assign. Defaule values

for these items are:

VIDEO MEMORY : 1024KB

SM RAM       :   64KB

HEADER       : 1 sector of the hard disk used

4. This utility runs under the DOS, Windows 3.1 DOS Prompt, and Win95 DOS

Prompt environment only.

(Example : OS/2 is not supported.)

5. The installation steps are as follows :

(1) Physical format (if needed)

(2) Logical format (if needed)

(3) Install DOS (without using the DoubleSpace disk Compression

utility)

(4) copy ASTDK.EXE [and README.DOC] to your root directory

(e.g. C:>copy a:\astdk\*.*)

(5) At DOS prompt, execute this program.

(example: C:\ASTDK /C)

(6) Add the "C:\ASTDK" into "autoexec.bat" file under your boot

directory

6. If your hard disk with reserved area is switched to be used by

another machine,  you can use ASTDK command directly on that machine

to make SYSTEM aware of the entry point for the reserved area.

7. If on-board memory size is changed since the last time you executed

this program, please resize the allocated disk spaces by either one

of the following two methods.

a. Using '/C' parameters of this program to release all allocated

disk spaces and then reallocate the appropriate disk spaces

automatically.

b. Using '/D' parameter of this program to reclaim the allocated

disk spaces and then '/C' parameter to create the appropriate

disk spaces.

8. You should take care with DBLSPACE environment while using ASTDK.

If you find that your compressed drive does not contain contiguous

memory for reserved area in Hard Disk, you can use the following

command to enlarge or reduce the size of a compressed drive.

DBLSPACE /SIZE[size1 | /RESERVE=size2] drive: