AFLASH.EXE

*****************************************************************************

To use AFlash, do the following:

1. Copy the MSG.DAT and AFLASH.EXE files from the aflash & ecu

utility diskette into a subdirectory of your choice.

2. From that subdirectory, type:

aflash <Enter>

3. A help message appears. Press any key to continue.

4. The main menu appears. Use the up- or down-arrow keys to

highlight the options. Press Enter to select.

5. If you want to save a copy of the current BIOS into a file,

select SAVE BIOS TO A DISK FILE.

6. Select LOAD BIOS FILE to load the BIOS file into memory.

7. Select PROGRAM FLASH ROM to erase the current BIOS, and

program flash ROM.

** WARNING! ** Do not turn off system power while flash BIOS is

programming. This will destroy the BIOS.

8. Reboot the system.

*****************************************************************************

A quicker way to load the BIOS file into memory, and program the

Flash ROM is to type:

Aflash  <filename>   <Enter>

The program automatically performs the loading and programming

functions, then reboots the system.

If the program cannot find the BIOS file, it shows the main menu

and flashes this message:

"Can't read this file !!!      Press any key to continue...."

Follow the procedures for loading and programming the BIOS file

using the main menu.

** WARNING! ** Do not install memory-related drivers (XMS, EMS, DPMI)

when Aflash is used.

*****************************************************************************