PCMCIA   CARDSOFT

Chap 1. Installing CardSoft

Chap 2. CardSoft PCMCIA Software Suite

Chap 3. Configuring CardSoft Parameters

Chap 4. CardSoft Parameters

Chap 5. Beep Code Definitions

Chap 6. PCMCIA Trouble-Shooting Related to AN750 Series Notebook Computers

������������������������������������������������������������������������

**** If you want to have a quick view to set your CardSoft parameters, SKIP

**** directly to Chap 6 to follow those setup values. And you can make your

**** AN 750 work well with PCMCIA.

������������������������������������������������������������������������

Chap 1. Installing  CardSoft

The Installation Utility provides step-by-step instructions for installing

the software. Although the installation procedure is easy to follow , the

following instructions are provided for your convenience.

�  BEFORE YOU BEGIN

If you want to allow hot insertion and removal of communications and memory

cards within Windows, you will need to install Windows support for PCMCIA

after you have installed the CardSoft software. However, before doing so, you

must inform Windows to exclude certain memory regions, because some PC Cards

use memory windows. The range you exclude must match the range specified by

the driver for the communications or memory card. if you have a two-slot

PCMCIA adapter, you should exclude memory region D000-DFFF.

Memory regions can be excluded by adding a line to your CONFIG.SYS file.

For example, if you are using the Windows Memory Manager (EMM386), you should

add the following line to your CONFIG.SYS file to exclude the memory region

D000-DFFF:

EMMEXCLUDE=D000-DFFF

If you are using a memory manager other than EMM386, refer to the documen-

tations that you received with the memory manager to determine how to exclude

a memory region.

After adding the memory exclusion line to your CONFIG.SYS file, reboot your

system. After rebooting, you can begin the CardSoft installation process.

�  INSTALLATION DISKETTE

The installation diskette contains the install program files in the sub-

directory /PCMCIA.DOS of system utility diskette.

All CardSoft-specific files are stored in the /PCMCIA.DOS/CARDSOFT direc-

tory of the installation diskette.

�  CARDSOFT INSTALLATION

1. Insert the diskette containing the CardSoft software.

2. At the DOS prompt, type A:, then press <Enter>.

3. At the DOS prompt, type CD PCMCIA.DOS, then press <Enter>.

4. Type INSTALL and press <Enter>.

5. Select "First Time Install" or "Upgrade" (whichever is appropriate), then

press <Enter>.

6. Select "Standard Installation" or "Custom Installation".

7. A window appears prompting you to choose the number that corresponds to

the number of PCMCIA sockets that are in the system. Use the # or # keys

to select the appropriate number. Then press <Enter>.

8. Another window appears displaying all available drives on the system. Use

# or # keys to select the drive on which you want to install CardSoft,

then press <Enter>.

9. A window appears with the name of the directory into which the CardSoft

files will be iinstalled (the default is CARDSOFT). To keep the default,

simply press <Enter>. To change the destination directory, use the Back-

space key to delete the directory name, then type in the directory name

you want to use. Press <Enter> when done.

10. A confirmation screen appears. Press <Enter> to confirm the destination

drive/directory, or type N and press <Enter> to return to the Drive

Selection screen (then repeat Steps 8 and 9).

11. If you selected the Standard Installation :

a. The utility copies all the CardSoft files to the selected destination,

displaying messages as it does so.

b. The utility prompts you to automatically modify your CONFIG.SYS file by

adding the appropriate DEVICE line to it. (Your original CONFIG.SYS

will be saved and given a new name, which is displayed on the screen).

Press <Enter> to automatically update your CONFIG.SYS file, or press

Esc to abort the installation process. After the utility modifies

CONFIG.SYS, press <Enter> to continue.

c. At this point, the installation program runs CSALLOC to determine ava-

ilable system resources. When the message "You must reboot your system

for CardSoft to become active" appears, press <Enter>. When the DOS

prompt appears, remove the installation diskette and reboot your sys-

tem.

If you selected the Custom Installation :

a. A list of all the CardSoft components is displyed. Select YES for the

components you want to install, then press <Enter>.

b. If you are upgrading your software, additional  component  selection

screens may be displyed. Select YES for the components  you want to ins-

tall, then press <Enter>.

c. After all components have been installed,  the installation utility

prompts you to automatically update CONFIG.SYS by adding the appropriate

DEVICE lines to it. Type Y and press <Enter> to have the installation

utility update CONFIG.SYS, or type N and press <Enter> if you want to

make the changes manually. If you select N, a list of the DEVICE lines

you have to add to CONFIG.SYS is displayed. Write these down, then

press <Enter> to continue.

d. At this point, the installation program runs CSALLOC to determine avail-

able system resources. When the message "You must reboot your system

for CardSoft to become active" appears, press <Enter>. When the DOS

prompt appears, remove the installation diskette and reboot your system.

������������������������������������������������������������������������

Chap 2. CardSoft PCMCIA Software Suite

The CardSoft software suite provides the end user with a complete "plug

and play" system software solution. This solution consists of the following

drivers and utilities.

�  SOCKET SERVICES (SSDBOOK.EXE)

Socket Services provide a standard software interface to host controller

DataBook TCIC-2/N and isolates the socket hardware from higher level software

. Socket Services includes functions such as configuring a socket for an I/O

or memory interface and controlling socket power voltages.

�  CARD SERVICES (CS.EXE)

The Card Services driver manages competition for system resources and man-

ages adapter and card resources and configuration.

�  CARD IDENTIFICATION (CARDID.EXE)

This client device driver detects the insertion and removal of PCMCIA PC

cards, automatically determines the card type upon insertion, and then con-

figures the card and socket/adapter. This driver communicates with PCMCIA-

aware device drivers.

�  MEMORY CARD DRIVER (MEMDRV.EXE)

It provides the interface to the Flash File System that is provided through

MS-FLASH.SYS (not included in this diskette) .

�  SRAM CARD DRIVER (SRAMDRV.EXE)

This device driver recognizes and supports SRAM cards, and is used in place

of MEMDRV.EXE and MS-FLASH.SYS if you are using SRAM cards only. If you want

to use a combination of SRAM and Flash Memory cards, you must use MEMDRV.EXE

�  IDE/ATA SUPPORT (ATADRV.EXE and ATAINIT.EXE)

ATADRV.EXE is a block device driver that supports ATA/Type 3 hard drive

PCMCIA cards. The ATAINIT.EXE utility places a hard disk partition table onto

a blank ATA drive. This utility initializes the ATA drive in the same manner

as FDISK does with a standard hard drive.

�  0 VOLT SUSPEND DRIVER (CS_APM.EXE)

CS_APM.EXE is a TSR that supports APM connection awareness. It can automa-

tically reconfigure PCMCIA card when system resumes from 0 Volt suspend mode.

�  CARDSOFT UTILITIES

System Utilities Diskette provides the following utilities along with the

CardSoft components.

�  Installation Program

This easy-to-use program installs all or some of the CardSoft components

onto the system. Users have the option of a standard or custom installation.

�  Configuration Utility

the basic Caonfiguration Utility enables you to

a. Define the general configuration settings for fax/modem, modem, ATA

drive, and network cards.

b. Configure CardSoft to your specific system requirements by modifying the

various parameters for the CardSoft executable files (making it unnece-

ssary to configure CardSoft with command line/device line switches)

c. Quickly view and change the system resources that are made available to

Card Services.

In addition, a Configuration Utility for advanced users enables you to

create or modify card records that define configuration and recognition para-

meters for specific PC Cards, and save these records in a Card Library.

�  Resource Allocation Utility (CSALLOC.EXE)

CSALLOC.EXE is a DOS utility that scans the system for available memory,

I/O Port, and IRQ resources. It then writes this information to the file

CSALLOC.INI, which is used by Card Services to determine what system resour-

ces are available for PC Card configuration.

CSALLOC can be run in two modes, both of which must be run from the DOS

command line (they can not be run from within a Microsoft Windows DOS Window)

:

CSALLOC /g  Runs CSALLOC in non-display mode. When you enter this command at

the DOS prompt, you are prompted to remove any PC Cards that are

present in the system. Remove any inserted PC Cards, then press

any key. CSALLOC then scans the system for available resorces,and

writes information similar to the following to the CSALLOC.INI

file.

(Example)

MEM=C000-CFFF,D000-DFFF

IOP=108-16F,178-1EF,1F8-1FF,208-36F,380-3BF,3E0-3EF

RIOP=170-177,370-377,3F0-3FF

CSALLOC /g/d Runs CSALLOC in display mode. When you add the /d switch to the

command, CSALLOC displays a series of messages as it scans

system resources, then lists the available resources on the

screen (in the same format as above).

�  WHEN TO RUN CSALLOC

CSALLOC is automatically run as part of the CardSoft installation. During

installation, a CSALLOC.INI file is created. As a result, you should run the

CSALLOC utility only when you have changed I/O Port, IRQ, or memory settings

on your system (for example, if you change your COM Port settings in Windows,

or you enable the EMM386 memory manager). This creates an updated CSALLOC.INI

file without having to run the installation program again.

�  Card Information Utility (CARDINFO.EXE)

CARDINFO is a DOS utility that scans the PCMCIA sockets on the system and

lists their contents on the screen. It can be run in two modes, both of which

must be run from the DOS commandline (they cannot be run from within a Micro-

soft Windows DOS window):

CARDINFO  Runs CARDINFO in non-verbose mode. When you enter this command at

the DOS prompt, information similar to the following is displayed:

Slot 1

Card Type = Token Ring

Manufacturer = <card vender name>

Product Name = Token Ring

Slot 2

Card Type Unavailable

CARDINFO /v  Runs CARDINFO in verbose mode, which displys more extensive in-

formation about the PCMCIA slots. (We recommand that you use the

DOS "more" switch with this command, since the information may

require more than one screen to be displayed). Following is a

sample of CARDINFO information using the /v switch. The type of

information that is displayed depends on the type of cards that

are present in the slots.

CARDINFO /c  Provides the following additional Card Services information:

- Card Services Release Number

- Vendor Revision Number

- Number of slots

- Vendor copyright information

CARDINFO /?  Displays information about CARDINFO switches.

Sample CARDINFO Information

Client Information for handle 7263:

Client Revision = 1.05

CS Support Level = 2.01

Revision Date = 7-30-93

Client Name = "CardID"

Vendor Name = "XXXXXXX"

Client Information for handle 7242:

Client Revision = 3.01

CS Support Level = 2.01

Revision Date = 8-04-93

Client Name = "SRAM Driver"

Vendor Name = "XXXXXXX"

Slot 1:

[Card Information]

Card Type = "Ethernet"

manufacturer = "D-Link"

Product Name = "DE-650"

[Configuration Info]

Configuring client handle is 7263

Memory+I/O interface, Vcc 50, Vpp1 50, Vpp2 50

Config Base 0008, Config values:

Option value: 60

Status value: 00

Copy value: 00

First I/O range 300-30F, 8-bit

Second I/O range 310-31F, 16-bit

Assigned IRQ is 5 (enabled)

[Memory Information]

Window D6000 - D7FFF, 8-bit attribute memory

Latest Error = NO_MORE_ITEMS (1F), on Function

GET_NEXT_TUPLE (OA)

Slot 2:

Slot 2 is empty

������������������������������������������������������������������������

Chap 3. Configuring CardSoft Parameters

Parameters can be modified using either of the following methods:

Run the Configuration Utility (this is the recommended method)

Add or modify the device line parameters associated with the device

drivers in CONFIG.SYS

Running the Configuration Utility

The Configuration Utility is an easy-to-use program for changing the

CardSoft components. Its interface is a windowed type with pull down menus.

Users have the option of using the keyboard, a pointing device, or both to

maneuver around the screen.

The Configuration Utility:

Configures and recognizes various PC cards.

Modifies the CardSoft executable files.

Determines available system resources.

To run the Configuration Utility, change to the directory containing the

utility (i.e., cd CARDSOFT). There are two ways to run the Configuration

Utility. If you want to be able to modify all of the CardSoft executable

files as well as edit and create existing and new card libraried/records,

type CONFIG -ALL. If you only want to edit the card configuration settings

or run the Resource Allocation utility, type CONFIG. The second method is

the one that you should normally use, unless you need to configure a PC

Card for read recognition by Card Services software. Depending upon the

method used to start the Configuration Utility, one of the following

screens is displayed.  Notice that without the -ALL switch, the Edit pull

down menu is not available.

����������������������������������������������������������������������Ŀ

�                 CardSoft<TM>  Configuration Utility                  �

�  File    Utility    Display                                          �

�                                                                      �

�                                                                      �

�                                                                      �

������������������������������������������������������������������������

����������������������������������������������������������������������Ŀ

�                 CardSoft<TM>  Configuration Utility                  �

�  File    Utility    Utility   Display                                �

�    Current  File                    Card  Library                    �

�  ����������������������������ͻ ���������������������������������Ŀ  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  �                            � �                                 �  �

�  ����������������������������ͼ �����������������������������������  �

������������������������������������������������������������������������

The initial Configuration Utility screen consists of a menu bar at the top

and a help text section at the bottom. The middle of the screen is blank.

When a Card Library file is selected, the card records in that file are dis-

played on the left in the Current File list box. A second Card Library can be

opened in view/copy mode; the records in this file are displayed on the right

in the Card Library list box.

�  ACCESSING MENU ITEMS

The method for accessing menu items is the same for each Configuration Uti-

lity. Each menu bar item contains a pull down menu with various items to

choose from. To access each pull down menu, click the left mouse button on

the desired item or press Alt + the highlighted key. For example, to access

the File menu, click on the word File or press <Alt-F> from the keyboard.

When the pull down menu appears, select the desired item by:

Clicking on it with the left mouse button

Pressing the down arrow key to highlight the item and then pressing

<Enter>, or

Pressing the key that corresponds to the highlighted letter.

�  DEFINING PARAMETERS FOR CARDSOFT EXECUTABLE FILES

The Binary Adaptation function enables you to define the parameters for

each of the CardSoft executable components.    This section describes the

process for defining the parameters. For information on each of these para-

meters, their default values, and the possible values that can be assigned

to each, refer to Chap 4.

To modify the parameters for CardSoft executable components:

1. Select Binary Adaptation from the Utility menu, then select Open from the

dialog box that is displayed. A list of the CardSoft executable files is

displayed.

����������������������������������������������������������������������Ŀ

�                        Open  File                                    �

�                                                                      �

�   ��������������������������������������Ŀ                           �

�   � SSDBOOK.EXE������������������������� �        <    OK    >       �

�   � CS.EXE                               �                           �

�   � CSALLOC.EXE                          �        <  Cancel  >       �

�   � ATADRV.EXE                           �                           �

�   � CARDID.EXE                           �        <   Help   >       �

�   � CARDINFO.EXE                         �                           �

�   ����������������������������������������                           �

������������������������������������������������������������������������

2. Select the file whose parameters you want to modify, then select <OK>.

A Parameter List screen similar to the following appears. The list of

parameters differs for each executable file. The sample screen shown

below is the Parameter List for the CardSoft executable file (CS.EXE).

����������������������������������������������������������������������Ŀ

�                        CS Parameter List                             �

�   ������������������������������������������������Ŀ                 �

�   � Power-On Delay�������������������������      6 �  <    OK    >   �

�   � Event Processing Delay                       2 �                 �

�   � Card-Status-Change IRQ                      15 �  <  Cancel  >   �

�   � Card-Status-Change Interrupt Mask          FFh �                 �

�   � Language                                     0 �  <   Help   >   �

�   � Block INT 1A Calls To Socket Services      YES �                 �

�   � /VERSIONTEST                                1h �  <   Edit   >   �

�   � /HEAPSIZE                                   2h �                 �

�   ��������������������������������������������������  <   Dump   >   �

������������������������������������������������������������������������

3. To change a parameter, highlight the parameter and select <Edit>. If the

parameter is a numeric value, an Edit Parameter screen similar to the

following is displayed. If the parameter is a Yes/No Value, the screen

that is displayed enables you to select the Yes or No value.

Type the new value here.

����������������������������������������������������������������������Ŀ

� �                      Edit  Parameter                               �

� �                                                                    �

� � Card-Status-Change Interrupt IRQ Mask                              �

� �                                                 <    OK    >       �

� ����� Default    DEB8                                                �

�                                                   <  Cancel  >       �

�      �                                                               �

�      �Minimum     0h                              <   Help   >       �

� ����Ĵ                                                               �

� �    �Maximum  FFFFh                                                 �

� �    �                                                               �

������������������������������������������������������������������������

Valid range of values is displayed here.

4. After editing the parameter, select <OK>. The new value should now be

displayed. Select <Save> to save your changes (or select <Done>, then

select <Yes>). If you want to cancel the changes you made, select <Done>

, then select <No>.

�  DUMPING THE PARAMETER VALUES INTO CONFIG.LST

The Binary Adaptation function allows you to save the executable file pa-

rameters into the file CONFIG.LST, which enables you to easily display or

print out the list of configuration settings (using a TYPE CONFIG.LST or

PRINT CONFIG.LST command). You can overwrite the existing CONFIG.LST file,

or you can append the parameters to the end of the file.

To dump the parameters into CONFIG.LST, select <Dump> from the Parameter

List screen. If CONFIG.LST already exists, a dialog box is displayed, giving

you the option to Append to the end of the file (select <Append>) or Replace

the existing file (select <Replace>).

�  CHANGING THE CONFIGURATION UTILITY DISPLAY MODE

The Configuration Utility can be displayed in three modes:color,monochrome

, or LCD. To change the display mode, select the Display pull down menu,

then select the type of display you want to use. The current display mode is

indicated by a check mark (�).

������������������������������������������������������������������������

Chap 4. CardSoft  Parameters

�  SOCKET SERVICES

Socket Services provides the standard software interface to the hardware

that controls PCMCIA-compatible slots for PCMCIA cards. The following section

describes all configuration settings available for Socket Services, both from

the Configuration Utility and the device line.

Card-Status-Change Interrupt IRQ Mask            /SIRM

Range:  1h - FFFFh                               range: 1h - FFFFh

Default: 4CF8h                                   Default: 4CF8h

Specifies the 16-bit mask of available IRQs that can be assigned as card-

status-change interrupt IRQs. The least significant bit represents IRQ 0 and

the most significant bit represents IRQ 15. A value of 5EB8h indicates that

IRQs 14, 12, 11, 10, 9, 7, 5, 4 and 3 are valid for assignment as card-status

-change IREQs.

PC Card IREQ Mask                                /IIRM

Range:  1h - FFFFh                               Range: 1h - FFFFh

Default: 4CF8h                                   Default: 4CF8h

Specifies the 16-bit mask of available IRQs that can be assigned as valid

PC Card IREQs. The least significant bit represents IRQ 0 and the most signi-

ficant bit represents IRQ 15. A value of 5EB8h indicates that IRQs 14, 12, 11

, 10, 9, 7, 5, 4, and 3 are valid for assignment as PC Card IREQ's.

PCMCIA Adapter Base Port Address                 /PORT

Range: 0h - 3FFh                                 Range: 0h - 3FFh

Default: 240h                                    Default: 240h

Specifies the base port address of the PCMCIA adapters. This is the I/O

address used by CardSoft to communicate with the PCMCIA Adapter(s).

Voltage Stabilization Delay                      /PWRON

Range: 1 through 65535                           Range: 01h - FFFFh

Default: 6667                                    Default: 1A0B

Specifies the amount of time, in 15-�s units, to wait for the voltage to

stabilize after a voltage change on a slot. This occurs when a card is inser-

ted into a slot and when a Flash memory card is written to or erased. The

default is 100ms.

Install As Adapter Zero                          /ADA

Range: Yes or No                                 Range: 0 or 1

Default: No                                      Default: 1

Forces installation of Socket Services as Adapter Zero. This feature may

be used to override the presence (e.g., in the BIOS) of any previously loaded

non-compliant PCMCIA implementations. SSDBOOK installs as the next available

Adapter when this parameter is not specified.

Number Of Supported Slots                        /SKT

Range: 1 - 2                                     Range: 1 - 2

Default: 2                                       Default: 2

Specifies the number of PCMCIA slots that SSDBOOK supports. On machines

that have a PCMCIA adapter that can support more slots than are present in

the machine, this value should be set to the exact number of slots present.

HIO16 and HMEM16 Control Type                    /TOTEM

Range: 0 or 1                                    Range: 0 or 1

Default: 0                                       Default: 0

Specifies type of control for HIO16 and HMEM16 ISA bus signals. 0 enables

open drain control. 1 enables totem-pole control.

�  CARD SERVICES

Card Services software coordinates access to PC Cards, PCMCIA slots, and

system resources among multiple clients. These clients may be device drivers,

system utiltities, or application programs. Card Services makes access to

slot hardware through Socket Services software.

Power-On Delay                                   /PWR

Range: 1 - 255                                   Range: 01h - FFh

Default: 6                                       Default: 0Ch

Specifies the power-on delay, in MS-DOS timer ticks, from the application

of power during card-insertion processing, until any attemp to access the

card. The default is

Event Processing Delay                           /TTD

Range: 1 - 255                                   Range: 01h - FFh

Default: 2                                       Default: 02h

Specifies a delay, in MS-DOS timer ticks,for processing interrupt-generated

events. An event is only processed if the delay expires without recurrence of

the same event. If the event recurs prior to the timeout, the earlier event

is discarded.

Card-Status-Change IRQ                           /IRQ

Range: 1 - 255                                   Range: 01h - FFh

Default: 15                                      Default: 0Fh

Specifies the prefered IRQ number to be used for the card-status-change

interrupt. If Card Services is unable to determine a better IRQ number to use

for the card-status-change, then this value is used.

Card-Status-Change Interrupt Mask                /SCIM

Range: 0h - FFh                                  Range: 0h - FFh

Default: FFh                                     Default: FFh

Specifies the Card-Status-Change interrupts that can be enabled. This value

should be left as FFh unless unusual hardware conditions exist with the Card-

Status-Change interrupt. For such circumstances, a vendor specific function

exists to allow all Card-Status-Change interrupts to be enabled on a slot.

Language                                         /LANG

Range: 0 - 4                                     Range: 0 - 4

Default: 0                                       Default: 0

The Card Services driver may be configured to display messages in five

different languages: 0 = English, 1 = French, 2 = German, 3 = Italian,

4 = Spanish.

Block INT 1A Calls to Socket Services            /BLOCKSS

Range: Yes or No                                 Range: 0h or 1h

Default: Yes                                     Default: 1h

Specifies that direct calls to Socket Services must NOT be allowed via the

INT 1A interface. The caller must retrieve an entry point from Card Services

to make calls to Socket Services.

/VERSIONTEST                                     /VERSIONTEST

Range: 0h - 1h                                   Range: 0h - 1h

Default: 1h                                      Default: 1h

/HEAPSIZE                                        /HEAPSIZE

Range: 2h - Ah                                   Range: 2h - Ah

Default: 2h                                      Default: 2h

Specifies the amount of memory (in KB) that is reserved for Card Services

for use as an internal heap, which is used to store information that Card

Services needs to operate (for example, the number and type of cards inserted

into PCMCIA slots).

�  CARDID

This component is the client driver that provides detection and configu-

ration services for supported PCMCIA cards. CardID recognizes when a PC-Card

is installed or removed. When a card is inserted, CardID interrogates it to

determine what type of card it is. Once the card type has been determined,

CardID initializes it according to the parameters that were specified within

the Configuration Utility or those that were specified on the device line in

CONFIG.SYS. The following lists the Configuration Utility parameters (on the

left) as well as the device line parameters (on the right).

Language                                         /LANG

Range: 0 - 4                                     Range: 0 - 4

Default: 0                                       Default: 0

The CardID driver may be configured to display messages in five different

languages: 0 = English, 1 = French, 2 = German, 3 = Italian, 4 = Spanish.

�  CSALLOC

This is a DOS utility that scans the system for available memory, I/O Port,

and IRQ resources. It then writes this information to the file CSALLOC.INI,

which is used by Card Services to determine what system resources are avail-

able for PC Card configuration.

/PPSIG0                                          /PPSIG0

Default: 4324h                                   Default: 4324h

Undocumented

/PPSIG1                                          /PPSIG1

Default: 5150h                                   Default: 5150h

Undocumented

/IOALIASES                                       /IOALIASES

Default: 3h                                      Default: 3h

Undocumented

/SCANFF                                          /SCANFF

Default: 1h                                      Default: 1h

Undocumented

�  MEMORY CARD DRIVER

MEMDRV is a device driver that supports both SRAM and Flash Cards. It also

provides the interface to Microsoft's Flash File System. MEMDRV suppors the

following parameters.

Memory Card Memory Window Base Address            /BAS

Range: C0h - EFh                                  Range: C0h - EFh

Default: Doh                                      Default: D0h

Specifies the base address in system memory of a memory window to be used

for accessing memory cards. This address, along with the window size for the

memory card, is used by the memory card drivers to communicate with memory

cards. The value is stated in units of 4K (D0=D000:0000). The default value

of D0h indicates a memory window starting at segment D000:0h

Memory Card Memory Window Size                    /SIZ

Range: 1 - 16                                     Range: 01h - 10h

Default: 4                                        Dafault: 04h

Specifies the size of the system memory window to be used for accessing

memory cards. The size, along with the base address for the memory card, is

used by the memory card drivers to communicate with memory cards. The value

is stated in units of 4K (1 = 4K, 2 = 8K, 3 = 12K, or 01h = 4K, 02 = 8K,

03h = 12K, etc.), with 64K  being the maximum size. The default value of 4

represents a 16K window.

Card Services Memory Window Base Address          /CSBAS

Range: C0 - EFh                                   Range: C0h - EFh

Default: D4h                                      Default: D4h

Specifies the base address in system memory of a window area reserved for

Card Services for its own internal use. This address, along with the size

specified for the Card Services memory window, is used by Card Services to

communicate with PC cards. The value is stated in units of 4K (D4=D400:0000).

The default value of D4h indicates a memory window starting  at  segment

D400:0h.

Card Services Memory Window Size                  /CSSIZ

Range: 1 - 16                                     Range: 1 - 16

Default: 4                                        Default: 4h

Specifies the size of a window area reserved for Card Services for its own

internal use. The size, along with the address for Card Services memory win-

dow, is used by Card Services to communicate with PC cards. The value is

stated in units of 4K (1 = 4K, 2 = 8K, 3 = 12K, etc.) with 64K being the maxi-

mum size. The default value of 4 represents a 16K window.

Number of Concurrent Erasures                     /QUEUE

Range: 1 - 9                                      Range: 1h - 9h

Default: 1                                        Default: 1h

Specifies the number of concurrent erasures for cards that support back-

ground erasure.

Resident Flash Array                              /RFA

Range: Yes or No                                  Range: 0h - 1h

Default: No                                       Default: 0h

Specifies the presence of a resident flash array.

Maximum Partitions Per Card                       /QUEUE

Range: 1 - 8                                      Range: 1h - 8h

Default: 1                                        Default: 1h

Specifies the maximum number of partitions per card.

Language                                          /LANG

Range: 0 -4                                       Range: 0 - 4

Default: 0                                        Default: 0

The Memory Card driver may be configured to display messages in five diff-

erent languages:0 = English, 1 = French, 2 = German, 3 = Italian, 4 = Spanish.

/CARDOFF                                          /CARDOFF

Range: 1h - 10h                                   Range: 1h - 10h

Default: 1h                                       Default: 1h

Informs MEMDRV to map memory pages on a 16K boundary.

/NOCSALLOC

Default: 0h

Undocumented

Beep on Card Detect Change

Range: 0h - 3h

Default: 3h

Specifies whether or not the Card Installation Client produces beeps on the

system speaker when PC cards are inserted or removed.

0 = Disable all beeps

1 = Allow beeps in DOS only

2 = Allow beeps in Windows only

3 = Allow beeps in both DOS and Windows

/NOPOWEROFF

Default: 0h

Undocumented

�  SRAM DRIVER

SRAMDRV is a device driver that recognizes and supports SRAM cards. Use

this driver in place of MEMDRV.EXE and MS-FLASH.SYS if you are using SRAM

cards only. If you are using Flash cards and SRAM cards, use MEMDRV.EXE and

MS-FLASH.SYS instead. You cannot install all three drivers in CONFIG.SYS.

Memory Card Memory Window Base Address            /BAS

Range: C0h - EFh                                  Range: C0h - EFh

Default: Doh                                      Default: D0h

Specifies the base address in system memory of a memory window to be used

for accessing memory cards. This address, along with the window size for the

memory card, is used by the memory card drivers to communicate with memory

cards. The value is stated in units of 4K (D0=D000:0000). The default value

of D0h indicates a memory window starting at segment D000:0h

Memory Card Memory Window Size                    /SIZ

Range: 1 - 16                                     Range: 01h - 10h

Default: 4                                        Dafault: 04h

Specifies the size of the system memory window to be used for accessing

memory cards. The size, along with the base address for the memory card, is

used by the memory card drivers to communicate with memory cards. The value

is stated in units of 4K (1 = 4K, 2 = 8K, 3 = 12K, or 01h = 4K, 02 = 8K,

03h = 12K, etc.), with 64K  being the maximum size. The default value of 4

represents a 16K window.

Card Services Memory Window Base Address          /CSBAS

Range: C0 - EFh                                   Range: C0h - EFh

Default: D4h                                      Default: D4h

Specifies the base address in system memory of a window area reserved for

Card Services for its own internal use. This address, along with the size

specified for the Card Services memory window, is used by Card Services to

communicate with PC cards. The value is stated in units of 4K (D4=D400:0000).

The default value of D4h indicates a memory window starting  at  segment

D400:0h.

Card Services Memory Window Size                  /CSSIZ

Range: 1 - 16                                     Range: 1 - 16

Default: 4                                        Default: 4h

Specifies the size of a window area reserved for Card Services for its own

internal use. The size, along with the address for Card Services memory win-

dow, is used by Card Services to communicate with PC cards. The value is

stated in units of 4K (1 = 4K, 2 = 8K, 3 = 12K, etc.) with 64K being the maxi-

mum size. The default value of 4 represents a 16K window.

Number of Suppoerted Slots                        /MAX

Range: 1 - 8                                      Range: 1 - 8

Default: 1                                        Default: 1

Specifies the maximum number of PCMCIA slots in the machine.

Verbose Mode                                      /DEBUG

Range: Yes or No                                  Range: 0h - 1h

Default: No                                       Default: 1h

Specifies that more detailed information be displayed during driver initia-

lization.

Language                                          /LANG

Range: 0 - 4                                      Range: 0 - 4

Default: 0                                        Default: 0

The SRAM driver may be configured to display messages in five different

languages: 0 = English, 1 = French, 2 = German, 3 = Italian, 4 = Spanish.

�  IDE/ATA DEVICE DRIVER

ATADRV.EXE is a hard disk configuration device driver for removable PCMCIA

devices that emulate an IDE type hard disk. ATADEV.EXE is strictly a hard

disk driver that communicates to industry-standard hard disk controllers.

ATADRV.EXE can manage up to eight PCMCIA devices.

ATADRV communicates with the CardID component to obtain information on card

insertion and removal. Once a card is inserted and recognized by CardID,

ATADRV communicates with the card as if it were an IDE hard disk. ATADRV must

be installed before the CARDID.EXE driver. ATADRV supports the following

parameters:

Configuration Parameters

Slave Mode

Range: No or Yes

Default: No

Allows the driver to be slaved to a master control driver for drive letter

selection. The Static Ram Driver (SRAMDRV.EXE) is the only master control

driver currently available.

Number of Drive Units

Range: 1 - 8

Default: 2

Specifies the number of drives that the system supports when installed

either as a block device driver or as a slave device driver.

Language                                          /LANG

Range: 0 - 4                                      Range: 0 - 4

Default: 0                                        Default: 0

The driver/utility may be configured to display messages in five different

languages: 0 = English, 1 = French, 2 = German, 3 = Italian, 4 = Spanish.

Device Line Parameter

/D

Range: 1 - 8

Default: 2

Installs the ATADRV driver as a block device driver for the specified num-

ber of drives.

/S

Range: 1 - 8

Default: 2

Installs the ATADRV device driver as a slave to another block device driver.

It also specifies the number of drives it can support.

�  DISK PARTITION UTILITY

ATAINIT.EXE is a disk partitioning utility that must be used to create a

partitioin on any drive supported by ATADRV. When a new drive is inserted into

a socket, it is not recognized since there is no CMOS or other common method

to find out its physical parameters. ATAINIT interrogates the drive to find

the physical parameters to use.

Language                                          /LANG

Range: 0 - 4                                      Range: 0 - 4

Default: 0                                        Default: 0

The disk utility may be configured to display messages in five different

languages: 0 = English, 1 = French, 2 = German, 3 = Italian, 4 = Spanish.

CREATING A PARTITION ON A SUPPORTED ATADRV DRIVE

To format a disk managed by ATADRV, type

ATADRV <drive letter>

then press <Enter>

then follow the prompts on the screen.

After running ATAINIT, you must immediately format the drive with the DOS

FORMAT utility, using the '/U' option (for example, format d:/u).

ATAINIT only works with drives that are supported by the ATADRV driver.

������������������������������������������������������������������������

Chap 5. Beep Code Definitions

Beep code definitions are as follows:

Insertion:  660 Hz, then 880 Hz     Card was recognized and successfully

configured.

Insertion:  440 Hz                  Card was not recognized.

Insertion:  220 Hz                  Card was recognized but the configura-

tion attemp failed.

Removal:    880 Hz, then 660 Hz     Card was removed from the slot.

������������������������������������������������������������������������

Chap 6. PCMCIA Trouble-Shooting Related to AN750 Series NoteBook Computers

���������������������������������������������������������������������������

�                                                                         �

�                                                                         �

�    Version of SystemSoft Driver (CardSoft) : Ver 2.07                   �

�                                                                         �

�    SystemSoft Driver Version (using vcb.exe) :                          �

�                                                                         �

�         ssdbook.exe   Version 2.01.14                                   �

�              cs.exe   Version 2.04.16                                   �

�         csalloc.exe  Number is 2137-01                                  �

�          atadrv.exe  Number is 2133-02                                  �

�          memdrv.exe   Version 2.01.06                                   �

�          cardid.exe  Number is 2082-02                                  �

�        cardinfo.exe  Number is 2090-03                                  �

�          cs_apm.exe   Version 1.11                                      �

�             vcb.exe   Version 0.10.00                                   �

�                                                                         �

���������������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                   SSDBOOK Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �Card-Status-Change Interrupt IRQ Mask               5EB8h �        �

� �PC Card IREQ Mask                                   4CF8h �        �

� �PCMCIA Adapter Base Port Address                     240h �        �

� �Voltage Stabilization Delay                          6667 �        �

� �Install As Adapter Zero                                NO �        �

� �Number Of Supported Sockets                             2 �        �

� �HIO16 and HMEM16 Control Type                           0 �        �

� �                                                          �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                        CS Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �Power-On Delay                                          6 �        �

� �Event Processing Delay                                  2 �        �

� �Card-Status-Change IRQ                                 11 �        �

� �Card-Status-Change Interrupt Mask                     FFh �        �

� �Language                                                0 �        �

� �Block INT 1A Calls To Socket Services                 Yes �        �

� �/VERSIONTEST                                           1h �        �

� �/HEAPSIZE                                              2h �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                   CSALLOC Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �/PPSIG0                                             4324h �        �

� �/PPSIG1                                             5150h �        �

� �/IOALIASES                                             3h �        �

� �/SCANFF                                                1h �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                    ATADRV Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �Slave Mode                                             NO �        �

� �Number of Drive Units                                   2 �        �

� �Language                                                0 �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                   ATAINIT Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �Language                                                0 �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                    MEMDRV Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �Memory Card Memory Window Base Address                D0h �        �

� �Memory Card Memory Window Size                          4 �        �

� �Card Services Memory Window Base Address              D4h �        �

� �Card Services Memory Window Size                        4 �        �

� �Number of Concuerrent Erasures                          1 �        �

� �Resident Flash Array                                   NO �        �

� �Maximum Partitions Per Card                             1 �        �

� �Language                                                0 �        �

� �/CARDOFF                                               1h �        �

� �/NOCSALLOC                                             0h �        �

� �Beep On Card Detect Change                             1h �        �

� �/NOPOWEROFF                                            0h �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                    CARDID Parameter List                            �

� ����������������������������������������������������������Ŀ        �

� �Language                                                0 �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                    Card Library                                     �

� ����CARDID.CLB (Card Library)�����������������������������Ŀ        �

� �Generic Modem                                             �        �

� �Sundisk 5MB                                               �        �

� �Generic ATA Disk                                          �        �

� �Xircom CreditCard LAN                                     �        �

� �Intel Ethernet Lan Card (old)                             �        �

� �Intel Ethernet Lan Card (new)                             �        �

� �D-Link Ethernet Lan Card (bad CIS)                        �        �

� �D-Link Ethernet Lan Card (bad CIS)                        �        �

� �D-Link Ethernet Lan Card (new)                            �        �

� �IBM 3270                                                  �        �

� �IBM Token Ring                                            �        �

� �IBM Ethernet                                              �        �

� �Canon Floppy Disk Card                                    �        �

� �Compag Floppy Disk Card                                   �        �

� �                                                          �        �

� ������������������������������������������������������������        �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                      Modem Card Configuration                       �

�                                                                     �

� ��� Com Port Order �����Ŀ        ��� Com Port Definition �����Ŀ   �

� �    First:  Port  2     �        �   Port 1:  3F8, IRQ  4      �   �

� �   Second:  Port  1     �        �   Port 2:  2F8, IRQ  3      �   �

� �    Third:  Port  3     �        �   Port 3:  3E8, IRQ  4      �   �

� �   Fourth:  Port  4     �        �   Port 4:  2E8, IRQ  3      �   �

� ��������������������������        �������������������������������   �

�                                                                     �

�  COM Port                         ��� Other Settings ����������Ŀ   �

�  (.) Insertion Order              � [X]  Enable Audio           �   �

�  ( ) Socket Order                 � [X]  Enable Ring Indicate   �   �

�                                   �������������������������������   �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                    Network Card Configuration                       �

�                                                                     �

�                                                                     �

�      Port 300     Irq 5    Memory 1 d800    Memory 2 dc00           �

�                                                                     �

�                       [X]   Fast Token Ring                         �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                       ATA Parameters                                �

�                                                                     �

�     [X] Try Primary ATA Address                                     �

�     [X] Try Secondary ATA Address                                   �

�     [X] Try Any Linear ATA Address                                  �

�                                                                     �

�����������������������������������������������������������������������

���������������������������������������������������������������������Ŀ

�                  Card Insertion Parameters                          �

�                                                                     �

�     [X] Beep in DOS                                                 �

�     [X] Beep in Windows                                             �

�     [X] Beep on configuration failure                               �

�     [ ] Beep on unrecognized card                                   �

�     [ ] Beep on rejected card                                       �

�     [ ] Continue on configuration failure                           �

�                                                                     �

�                                                                     �

�����������������������������������������������������������������������

PCMCIA CARD FUNCTION VERIFICATION LIST

���������������������������������������������������������������������������

�                                                                         �

�                   PCMCIA Fax/Data Modem Card                            �

���������������������������������������������������������������������������

�  AMT STARCARD Fax Modem 2442fx

�  AMT STARCARD Fax Modem 2400fx

�  GVC Fax/Data Modem

�  Megahertz Data/Fax Modem 2400/9600

�  E-TECH Fax/Data Modem C9624RX

�  DATARACE RediCARD Data/Fax Modem

�  ROHM Fax/Data Modem 9600/2400

�  Intel Fax 9600

�  AT&T Paradyne Fax/Data Modem 3761-B1-001

�  IBM 14.4/14.4 Data/Fax Modem

�  ATI FAX-MODEM 14400 ETC-EXPRESS

�  SMART FAX/MODEM SmartExchange 1414

���������������������������������������������������������������������������

�                                                                         �

�                      PCMCIA  Memory  Card                               �

���������������������������������������������������������������������������

�  I.C.Card Inc. Flash 1MB

�  Maxell SRAM 256KB

�  Intel Flash 4MB Series 2

�  ASIS SRAM 1MB

���������������������������������������������������������������������������

�                                                                         �

�                      PCMCIA  ATA Drive Card                             �

���������������������������������������������������������������������������

�  Maxtor 105MB MXL-105-III

�  Calluna 81.1MB CT80MC

�  SUNDISK 10MB SDP-10

�  SUNDISK 20MB SDP-20

���������������������������������������������������������������������������

�                                                                         �

�                      PCMCIA  LAN Card                                   �

���������������������������������������������������������������������������

�������������������������������������������������������������������������Ŀ

�      Card        �      Item        �   Socket   0    �   Socket   1    �

�������������������������������������������������������������������������Ĵ

�                  �  Connectivity    �      Pass       �      Pass       �

�                  ������������������������������������������������������Ĵ

�                  �                                                      �

�                  �   Contents of net.cfg:                               �

�                  �     Link Driver PCMDMCS                              �

�     IBM          �          port 300                                    �

�     Ethernet     �          int 5                                       �

�     PCMCIA       �          mem D8000                                   �

�     Card         �          Frame Ethernet_802.3                        �

�                  �   Procedure:                                         �

�                  �    >lsl                                              �

�                  �    >pcmdmcs                                          �

�                  �    >ipxodi                                           �

�                  �    >netx                                             �

�                  �                                                      �

�������������������������������������������������������������������������Ĵ

�                  �  Connectivity    �      Pass       �      Pass       �

�                  ������������������������������������������������������Ĵ

�                  �                                                      �

�                  �   Contents of net.cfg:                               �

�                  �     Link Driver PCMDMCS                              �

�     GVC          �          port 300                                    �

�     Ethernet     �          int 5                                       �

�     PCMCIA       �          mem D8000                                   �

�     Card         �          Frame Ethernet_802.3                        �

�                  �   Procedure:                                         �

�                  �    >lsl                                              �

�                  �    >pcmdmcs                                          �

�                  �    >ipxodi                                           �

�                  �    >netx                                             �

�                  �                                                      �

�������������������������������������������������������������������������Ĵ

�                  �  Connectivity    �      Pass       �      Pass       �

�                  ������������������������������������������������������Ĵ

�                  �                                                      �

�                  �   Contents of net.cfg                                �

�                  �     Link Driver PCMDMCS                              �

�     National     �          port 300                                    �

�     Semiconductor�          int 5                                       �

�     PCMCIA       �          mem D8000                                   �

�     Ethernet     �          Frame Ethernet_802.3                        �

�     Card         �   Procedure:                                         �

�                  �    >lsl                                              �

�                  �    >pcmdmcs                                          �

�                  �    >ipxodi                                           �

�                  �    >netx                                             �

�                  �                                                      �

�������������������������������������������������������������������������Ĵ

�                  �  Connectivity    �      Pass       �      Pass       �

�                  ������������������������������������������������������Ĵ

�                  �                                                      �

�                  �   Contents of net.cfg                                �

�                  �     link driver ceodi                                �

�     Xircom       �        frame ethernet_802.3                          �

�     Ethernet     �        frame ethernet_II                             �

�     PCMCIA       �          int 5                                       �

�     Card         �          port 300                                    �

�   *Note          �          mem d8000                                   �

�                  �          iowordsize 16                               �

�                  �   Procedure:                                         �

�                  �    >lsl                                              �

�                  �    >ceodi                                            �

�                  �    >ipxodi                                           �

�                  �    >netx                                             �

�������������������������������������������������������������������������Ĵ

�                  �  Connectivity    �      Pass       �      Pass       �

�                  ������������������������������������������������������Ĵ

�                  �                                                      �

�                  �                                                      �

�                  � ������������������������������������������������Ŀ   �

�                  � �        Network Card Configuration              �   �

�                  � �                                                �   �

�                  � � Port 300   Irq 5  Memory 1 d800  Memory 2 d000 �   �

�                  � �                                                �   �

�                  � �                    [X]   Fast Token Ring       �   �

�                  � �                                                �   �

�                  � ��������������������������������������������������   �

�                  �                                                      �

�                  �                                                      �

�                  � ��������������������������������������������������Ŀ �

�                  � �              MEMDRV Parameter List               � �

�                  � �  ���������������������������������������������Ŀ � �

�                  � �  �                                             � � �

�                  � �  � Memory Card Memory Window Base Address   DCh� � �

�     IBM          � �  � Memory Card Memory Window Size             4� � �

�     Token Ring   � �  � Card Services Memory Window Base Address DCh� � �

�     16/4         � �  � Card Services Memory Window Size           4� � �

�     PCMCIA       � �  � Number of Concuerrent Erasures             1� � �

�     Card         � �  � Resident Flash Array                      NO� � �

�                  � �  � Maximum Partitions Per Card                1� � �

�                  � �  � Language                                   0� � �

�                  � �  � /CARDOFF                                  1h� � �

�                  � �  � /NOCSALLOC                                0h� � �

�                  � �  � Beep On Card Detect Change                1h� � �

�                  � �  � /NOPOWEROFF                               0h� � �

�                  � �  ����������������������������������������������� � �

�                  � ���������������������������������������������������� �

�                  �                                                      �

�                  �    Contents of net.cfg:                              �

�                  �      Link Support                                    �

�                  �        BUFFERS  1  9000                              �

�                  �      Link Driver TOKEN                               �

�                  �        int 5                                         �

�                  �        DATA RATE 16                                  �

�                  �                                                      �

�                  �    Procedure:                                        �

�                  �      >lsl                                            �

�                  �      >token                                          �

�                  �      >ipxodi                                         �

�                  �      >netx                                           �

�������������������������������������������������������������������������Ĵ

�                                                                         �

�  *Note                                                                  �

�      If you want to use modem card and Xircom lan card simultaneously,  �

�   you should delete xircom from cardid.clb by using config -all, then   �

�   run the following procedure:                                          �

�                                                                         �

�   >lsl                                                                  �

�   >cecfg driver=odi                                                     �

�   >ceodi                                                                �

�   >ipxodi                                                               �

�   >netx                                                                 �

���������������������������������������������������������������������������