Suspend-to-hard-disk program :

��������������������������������

This program locates free contiguous disk spaces and allocate them for

saving  base and extended memory as well as system registers.    The disk

spaces found are reserved for system use and are no longer accessible  to

the user. If the program cannot find the required disk spaces,   it shows

an error message  "Not enough disk space for allocation".    The disk may

have  enough  free spaces but these free spaces exist as small fragments.

In this case,  you  can  use  some tools such as  SPEEDDISK in PCTOOLS to

compact your free disk spaces.  Then you can run this program again.

Why does this program need to allocate disk spaces ?  When most of the

power is removed from the system (example, due to suspend timer time-out)

the system swaps all memory images and save all system status information

into these disk spaces. When you turn on the system again, the system will

be returned to its previous state before the power was shut off. If you do

not run this program to allocate the required disk spaces,  this  function

will not work. Therefore, it is advisable to run this program before using

the system for the first time.  Once this program is executed,  the system

will have the 'Suspend-to-Hard-Disk' feature and you do not need to run it

again until the data in CMOS is lost or corrupted.   (For example,  system

configuration changed by adding on-board memory, replacing hard disk, etc.)

Please refer to the following description.

This program has four paramaters :

/HELP or /?  : Shows a help screen briefly describing how to use the

parameters.

/CREATE=size : Allocates contiguous disk spaces for swapping memory image

and saving system registers.  We suggest that you allocate

the  required  disk spaces  before you start using your PC.

you may specify the size of the disk space which you  want

to allocate.   If the size value you assigned is less than

the  system  memory size,  the program will use the system

memory size instead. The default value is the system memory

size.  The  program  needs additional disk space for header

information.   If  you  want  to  resize the allocated disk

spaces,  you can  run  this  program and use this parameter

again.  The program will reallocate the disk space based on

the new size you specify.

/DELETE      : Use this parameter to free the disk space allocated by the

program.   If  you  need to allocate a new disk space, run

this program and use the /CREATE parameter.

/RECOVER     : This program keeps its status information in CMOS. When the

data  in  CMOS  is corrupted, you can use this parameter to

free the  allocated disk spaces on your hard disk.

Notice :

1. This utility runs under the DOS environment only.

(Example : OS2 is not supported.)

2. The installation steps are as follows :

(1) Physical format (if needed)

(2) Logical format (if needed)

(3) Install DOS (without using the DoubleSpace disk Compression

utility)

(4) At DOS prompt, execute this program.

(example: A:\ASTDK\ASTDK /C)

3. If on-board memory size or hard disk are changed since the  last

time you executed this program, please resize the allocated disk

space by using '/R' and '/C' parameters of this program.