���������������������������������ͻ

�				       �

�	   ERROR CODE LIST (V2.20)     �

�				       �

���������������������������������ͼ

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�  1. Main Board Test:							      �

�									      �

�     -  CPU Chip Test							      �

�	     <1>. 8088 compatible mode error			      01111   �

�									      �

�     -  Interrupt Controller Test					      �

�	     <1>. IMR R/W test	(8259 A)			      01211   �

�	     <2>. IMR R/W test	(8259 B)			      01212   �

�	     <3>. IRQ0 test					      01221   �

�	     <4>. Unexpected IRQ test  (8259 A) 		      01231   �

�	     <5>. Unexpected IRQ test  (8259 B) 		      01232   �

�									      �

�     -  DMA Controller Test						      �

�	     <1>. DMA0 current address register R/W test	      01300   �

�	     <2>. DMA0 current word count register R/W test	      01301   �

�	     <3>. DMA1 current address register R/W test	      01310   �

�	     <4>. DMA1 current word count register R/W test	      01311   �

�									      �

�     -  System Timers Test						      �

�	     <1>. NO IRQ0 generated				      01410   �

�	     <2>. Timer 1 test error (RREQ times too low )	      01411   �

�	     <3>. Timer 1 test error (RREQ times too high)	      01412   �

�	     <4>. Timer 2 latch test error			      01421   �

�	     <5>. Timer 3 (watchdog) error		      01431 - 01433   �

�									      �

�     -  Keyboard Controller Test					      �

�	     <1>. Read command byte test			      01501   �

�	     <2>. Write command byte test			      01502   �

�	     <3>. Selftest test 				      01511   �

�	     <4>. Keyboard interface test			      01512   �

�	     <5>. Auxiliary device intercface test		      01513   �

�	     <6>. Disable KBD interface test			      01521   �

�	     <7>. Enable KBD interface test			      01522   �

�	     <8>. Disable AUX interface test			      01523   �

�	     <9>. Enable AUX interface test			      01524   �

�									      �

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�									      �

�     -  RTC/CMOS RAM Test						      �

�									      �

�	  A. Type 58167 						      �

�	     <1>. Counter or latch reset error			      01611   �

�	     <2>. Latch R/W error				      01612   �

�	     <3>. Counter R/W error				      01613   �

�	     <4>. Counter Roll_Over error			      01614   �

�	     <5>. Interrupt error				      01615   �

�									      �

�	  B. Type 146818						      �

�	     <1>. 146818 R/W test				      01621   �

�	     <2>. No interrupt test				      01622   �

�	     <3>. 146818 REG. C test  (without INT.)		      01623   �

�	     <4>. 146818 REG. C test  (with INT.)		      01624   �

�	     <5>. Update count test				      01625   �

�	     <6>. RTC_TMR0 INT. count too low			      01626   �

�	     <7>. RTC_TMR0 INT. count too high			      01627   �

�	     <8>. CMOS RAM R/W type1 test			      01628   �

�	     <9>. CMOS RAM R/W type2 test			      01629   �

�	    <10>. Expanded CMOS RAM test (MCA ) 		      01630   �

�	    <11>. Expanded CMOS RAM test (EISA) 		      01631   �

�									      �

�     -  Speed Switching Test						      �

�	   <1>. 8088-based series speed switching error 	      01712   �

�	   <2>. 1033-based series speed switching error 	      01713   �

�	   <3>. 910/915-based series speed switching error	      01721   �

�	   <4>. 915P/915V-based series speed switching error	      01722   �

�	   <5>. 80386-6MHZ switching error			      01730   �

�	   <6>. 80386-8MHz switching error			      01731   �

�	   <7>. 80386-Top mode switching error			      01732   �

�	   <8>. 80386-10MHZ switching error			      01734   �

�	   <9>. 80386-12MHZ switching error			      01735   �

�	   <10>.80386-PC mode switching error			      01737   �

�	   <11>.80386-Smart mode switching error		      01738   �

�	   <12>.Bios - top speed switching error		      01740   �

�	   <13>.Bios - low speed switching error		      01741   �

�	   <14>.Bios - smart mode switching error		      01742   �

�									      �

�����������������������������������������������������������������������������Ĵ

�									      �

�  2. Memory Test : ON BOARD MEMORY					      �

�									      �

�	     <1>. Data bus test 				      02001   �

�	     <2>. Address bus test				      02002   �

�	     <3>. Refresh test					      02003   �

�									      �

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�									      �

�  3. Keyboard Test:							      �

�						     XT       1030	AT    �

�	     <1>. Controller reset test 	    03011    03111    03213   �

�		  Receive Incorrect Code	    03012     NONE     NONE   �

�	     <2>. KB 084 every key test 	    03041    03141    03241   �

�	     <3>. KB 097 every key test 	    03042    03142    03242   �

�	     <4>. KB 101 every key test 	    03043    03143    03243   �

�	     <5>. KB 102 every key test 	    03044    03144    03244   �

�	     <6>. KB 123 every key test 	    03045     NONE    03245   �

�	     <7>. KB  82 every key test 	     NONE     NONE    03482   �

�	     <8>. KB  83 every key test 	     NONE     NONE    03483   �

�	     <9>. KB  82+keypad every key test	     NONE     NONE    03499   �

�	    <10>. KB  83+keypad every key test	     NONE     NONE    03400   �

�	    <11>. KB  85 every key test 	     NONE     NONE    03485   �

�	    <12>. KB  86 every key test 	     NONE     NONE    03486   �

�	    <13>. KB  89 every key test 	     NONE     NONE    03489   �

�	    <14>. KB  85+keypad every key test	     NONE     NONE    03402   �

�	    <15>. KB  86+keypad every key test	     NONE     NONE    03403   �

�	    <16>. KB  89+keypad every key test	     NONE     NONE    03406   �

�									      �

�����������������������������������������������������������������������������Ĵ

�									      �

�  4. Display Test :							      �

�	     <1>. VIDEO CONTROL CHIP			    04101	      �

�		  VGA  SEQUENCER PATTERN TEST		    04102	      �

�		  VGA  GRAPHIC	 PATTERN TEST		    04103	      �

�		  VGA  ATTRIBUTE PATTERN TEST		    04104	      �

�		  Vertical Sync. bit test		    04105	      �

�		  GD62x5  Extension  Register Test	    04106	      �

�									      �

�	     <2>. VIDEO RAM Data Line Test		    04201	      �

�		  VIDEO RAM Address Line Test		    04202	      �

�									      �

�	     <3>. VGA RAMDAC TEST			    04301	      �

�									      �

�									      �

�						      CRT    LCD    SimulSCAN �

�									      �

�	     <4>. 40X25 MODE			      04005  04205  04405     �

�	     <5>. 80X25 MODE			      04006  04206  04406     �

�	     <6>. PAGES 40X25			      04012  04212  04412     �

�	     <7>. PAGES 80X25			      04013  04213  04413     �

�	     <8>. 320X200 PALETTE0 4 COLOR MODE       04014  04214  04414     �

�	     <9>. 320X200 PALETTE1 4 COLOR MODE       04015  04215  04415     �

�	    <10>. 320X200 16 color MODE 	      04016  04216  04416     �

�	    <11>. 320X200 256/256K MODE 	      04017  04217  04417     �

�	    <12>. 640X200 B/W MODE		      04018  04218  04418     �

�	    <13>. 640X350 16 color		      04023  04223  04423     �

�	    <14>. 640X480 B/W MODE		      04024  04224  04424     �

�	    <15>. 640X480 16 color MODE 	      04025  04225  04425     �

�	    <16>. GRAPHIC 800X600 16 COLORS	      04043		      �

�	    <17>. GRAPHIC 800X600 256 COLORS	      04046		      �

�	    <18>. GRAPHIC 1024X768 16 COLORS	      04047		      �

�	    <19>. GRAPHIC 640X480 256 COLORS	      04048  04248  04448     �

�									      �

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�									      �

�  5. Parallel Port LPTx Test:		 PORT 3BCH:	  378H:       278H:   �

�									      �

�     -  Internal Data Port Test	      05111	  05211       05311   �

�									      �

�     -  Internal Command Port Test					      �

�	     <1>. Strobe		      05121	  05221       05321   �

�	     <2>. Autofeed		      05122	  05222       05322   �

�	     <3>. Initialize		      05123	  05223       05323   �

�	     <4>. Select		      05124	  05224       05324   �

�									      �

�     -  Printing Test							      �

�	     <1>. No printer connector	      05161	  05261       05361   �

�	     <2>. Printer cannot be selected  05162	  05262       05362   �

�	     <3>. Initialization error	      05163	  05263       05363   �

�	     <4>. Paper busy		      05164	  05264       05364   �

�	     <5>. Printer busy		      05165	  05265       05365   �

�	     <6>. Unexpected error	      05166	  05266       05366   �

�	     <7>. Printing error	      05167	  05267       05367   �

�	     <8>. Unknown error 	      05168	  05268       05368   �

�									      �

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�									      �

�  6. Serial Port COM Test:				  COM1:       COM2:   �

�									      �

�     -  UART Chip Test 						      �

�	     <1>. Line control register 		  06111       06211   �

�	     <2>. Interrupt enable register		  06112       06212   �

�	     <3>. Modem status register 		  06113       06213   �

�	     <4>. Interrupt identification register	  06114       06214   �

�	     <5>. Relation between MCR and MSR		  06115       06215   �

�	     <6>. Modem control register		  06116       06216   �

�	     <7>. Internal INT test			  06117       06217   �

�     -  Internal Loopback Test 			  06121       06221   �

�     -  Baud Rate Test 						      �

�	     <1>. 9600 bps				  06141       06241   �

�	     <2>. 4800 bps				  06142       06242   �

�	     <3>. 2400 bps				  06143       06243   �

�	     <4>. 1200 bps				  06144       06244   �

�	     <5>.  600 bps				  06145       06245   �

�									      �

�����������������������������������������������������������������������������Ĵ

�									      �

�  7. Floppy Disk Drive Test:				 DRIVE A:   DRIVE B:  �

�									      �

�     -   FDC reset test				  07111       07211   �

�     -   Last track head F/W/R/C/V test		  0712x       0722x   �

�							  (   x=1 to x=4  )   �

�     -   Sequential R/W test				  0713x       0723x   �

�							  (   x=1 to x=2  )   �

�     -   Random W/R/C test				  07141       07241   �

�     -   Change line test				  07171       07271   �

�	  Write protect test				  07172       07272   �

�	  Read write-protection test			  07173       07273   �

�     -   Allocate memory fail				  07099 (DRIVER A&B)  �

�									      �

�	     Notice : x --> 1 = Write		       ? --> = item (1 -- 7)  �

�			    2 = Read					      �

�			    3 = Compare 				      �

�			    4 = Format					      �

�			    5 = Verify					      �

�									      �

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�									      �

�  8. Fixed Disk Drive Test:				 Drive 0:    Drive 1: �

�									      �

�     -  Controller Test						      �

�      <1>. RAM Diagnostic Fail (8088,8086 only)	  08111       08211   �

�      <2>. Drive Diagnostic Fail (8088,8086 only)	  08112       08212   �

�      <3>. Internal Diagnostic Fail (80X86 only)	  08113       08213   �

�      <4>. Reset Fail					  08114       08214   �

�									      �

�     -  Recalibrate Test				  08121       08221   �

�									      �

�     -  Head Write/Read/Compare Test					      �

�      <1>. Allocation Memory Fail			  08130       08230   �

�      <2>. Backup Data of Head 0,Sector 10 Fail	  08131       08231   �

�      <3>. Write Head Pattern Fail			  08132       08232   �

�      <4>. Read Head Pattern Fail			  08133       08233   �

�      <5>. Compare Head Pattern Error			  08134       08234   �

�      <6>. Write Test Pattern Fail			  08135       08235   �

�      <7>. Read Test Pattern Fail			  08136       08236   �

�      <8>. Compare Test Pattern Error			  08137       08237   �

�      <9>. Restore Data of Head 0,Sector 10 Fail	  08138       08238   �

�									      �

�     -  Cylinder 0 Test						      �

�      <1>. Read ID field Fail				  08141       08241   �

�      <2>. Read data field Fail			  08142       08242   �

�									      �

�     -  Verify Format Test						      �

�      <1>. Read ID field Fail				  08151       08251   �

�      <2>. Read Data Field Fail			  08152       08252   �

�      <3>. Defect Exceed Acceptable Range		  08153       08253   �

�      <4>. Write Defect to Floppy Disk Fail		  08154       08254   �

�									      �

�     -  Crescendo Seek Test						      �

�      <1>. Seek Fail					  08161       08261   �

�      <2>. Seek Time out (Exceed 1 Second)		  08162       08262   �

�									      �

�     -  Random Write/Read/Compare Test 				      �

�      <1>. Can't Find Enough Tracks to Test              08170       08270   �

�      <2>. Write Test Pattern Fail			  08171       08271   �

�      <3>. Read Test Pattern Fail			  08172       08272   �

�      <4>. Compare Test Pattern Error			  08173       08273   �

�      <5>. Allocate Memory Fail			  08174       08274   �

�      <6>. Backup Original Data Fail			  08175       08275   �

�      <7>. Restore Original Data Fail			  08176       08276   �

�									      �

�������������������������������������������������������������������������������

�����������������������������������������������������������������������������Ŀ

�	     ITEM						       CODE   �

�����������������������������������������������������������������������������Ĵ

�									      �

�  9. Pointing Device Test :						      �

�									      �

�	     <1>. Initialization - default value test		      11001   �

�				 - enable/disable mouse test	      11002   �

�				 - sample rate test		      11003   �

�				 - resolution test		      11004   �

�				 - scaling test 		      11005   �

�	     <2>. Left button test				      11011   �

�		  Right button test				      11012   �

�	     <3>. Movement test 				      11021   �

�									      �

�����������������������������������������������������������������������������Ĵ

�									      �

� 10. Cache Test :							      �

�									      �

�	     <1>. Cache Controller Test 			      13001   �

�	     <2>. Snoop Test					      13002   �

�	     <3>. Cache Memory Test				      13003   �

�	     <4>. LRU Test					      13004   �

�									      �

�����������������������������������������������������������������������������Ĵ

� 11. Modem Test :							      �

�							  COM1	     COM2     �

�									      �

�     <1>. Check CD Signal				  14x01      14x51    �

�     <2>. AT&F(&W)					  14x02      14x52    �

�     <3>. ATX						  14x03      14x53    �

�     <4>. ATS						  14x04      14x54    �

�     <5>. +++						  14x07      14x57    �

�     <6>. ATH						  14x09      14x59    �

�     <7>. ATZ0(ATZ1)					  14x11      14x61    �

�     <8>. ATDP 					  14x16      14x66    �

�     <9>. ATDT 					  14x17      14x67    �

�    <10>. ATS0=?					  14x18      14x68    �

�    <11>. ATS0&W					  14x19      14x69    �

�    <12>. AT&T0					  14x20      14x80    �

�    <13>. AT&T1					  14x21      14x81    �

�    <14>. AT&T6					  14x23      14x83    �

�									      �

�									      �

�	    x = 2   Non-volatile Memory Test Fail			      �

�	      = 3   Local Analog Loopback Test Fail			      �

�	      = 4   Relay Speaker Test Fail				      �

�									      �

�������������������������������������������������������������������������������