CardSoft 3.1

Software

User's Guide

Copyright (c) 1992-1994, SystemSoft  Corporation.   All

Rights Reserved.

SystemSoft Corporation

313 Speen Street

Natick MA 01760

No  part   of  this   publication  may   be  reproduced,

transmitted,  transcribed, stored in a retrieval system,

or translated into any language or computer language, in

any  form,  or  by any  means,  electronic,  mechanical,

magnetic,  optical,   chemical,   manual  or  otherwise,

without  the  prior  written  permission  of  SystemSoft

Corporation.

Disclaimer

SystemSoft  Corporation  provides  this document and the

programs "as  is"  without warranty of any kind,  either

expressed or implied, including, but not limited to, the

implied warranties of merchantability  or  fitness for a

particular purpose.

This document  could contain  technical inaccuracies  or

typographical errors.   Changes are periodically made to

the   information   herein;  these   changes   will   be

incorporated  in  future  revisions  of  this  document.

SystemSoft Corporation  is under no obligation to notify

any person of the changes.

The following trademarks are used in this document:

CardID is a trademark of SystemSoft Corporation.

CardSoft is a trademark of SystemSoft Corporation.

Microsoft is a registered trademark and Windows is a

trademark of Microsoft Corporation.

PCMCIA is a trademark of PCMCIA Corporation.

SystemSoft is a registered trademark of SystemSoft

Corporation.

CardSoft 3.1

Software

User's Guide

Revision: 0.1

May 1994

-ii-

Preface

This document  is  written  for  users  of  SystemSoft's

CardSoft 3.1  and CardBoot  PCMCIA products.  The manual

contains a CardSoft User section and a CardSoft Advanced

User  section  (which  includes  instructions  on  using

CardBoot).

This manual is organized as follows:

Part 1: CardSoft User's Guide

Chapter 1  describes how  to install  the  CardSoft  and

Flash  File   System  software,  and  defines  the  beep

sequences that  are heard when you insert a PC Card into

a slot.    It  also  describes  how  to  configure  your

system's memory  manager  (if  you  are  using  one)  to

prevent a conflict with CardSoft.

Chapter 2  describes the  CardSoft commands, CSALLOC and

CARDINFO, that  are provided  to help  you configure and

manage your PCMCIA system.  It also describes how to use

the CardSoft Configuration Utility to change the generic

configuration settings for modem, network, and ATA Drive

cards.

Chapter 3  describes how  to prepare  and use  removable

storage cards (ATA Drive cards and SRAM cards.

Part 2: CardSoft Advanced User's Guide

Chapter 4  describes the CardBoot product, which enables

you to  create bootable  SRAM cards.   It also describes

how to boot your system from a bootable SRAM card.

Chapter 5  describes how  to customize  your  system  by

modifying CONFIG.SYS and CSALLOC.INI.  It also describes

how to  use the  Resource  Allocation  function  of  the

Configuration  utility   to  change  how  resources  are

assigned on your system.

-iii-

-iv-

Table of Contents

Preface                                                    iii

Part 1: CardSoft 3.1 User's Guide

Chapter 1

Introduction                                               1

Installing the Software                               1

Before You Begin                                 1

How Much Memory Should You Exclude?         2

What's Next?                                3

Copying SYSTEM.INI                          3

Installation Diskette                            3

CardSoft Installation                            4

Initial Screen                              4

Installation Type                           4

Number of PCMCIA Sockets                    4

Destination Drive                           4

Destination Directory                       5

Socket Services Driver                      5

CardSoft Components Detected                5

Component Selection Screen                  6

CONFIG.SYS Update Screen                    6

Exit Screen                                 6

Installing Windows Support for CardSoft          7

System Beeps                                          7

Using TDK CD02x Ethernet Cards                        7

System Xircom CE-10BT and CE-10B2 Ethernet LAN Cards  8

Advanced Power Management                             8

Removing CardSoft From Your System                    8

Chapter 2

Using the CardSoft Utilities                               11

Using CSALLOC                                              11

When to Run CSALLOC                                   11

How to Run CSALLOC                                    12

Using CARDINFO                                             13

When to Run CARDINFO                                  13

How to Run CARDINFO                                   14

Sample CARDINFO Information                      15

Using the Configuration Utility                            16

Configuration Utility Help                            16

Running the Configuration Utility                     16

Accessing Menu Items                             17

Editing PC Card Configuration Settings                17

Modem Card Configuration                         19

COM Port Assignment Order                   19

COM Port Definition                         20

COM Port Order                              20

Audio                                       21

Ring Indicate                               21

Network Card Information                         21

ATA Card Configuration                           22

Saving PC Card Configuration                     22

Changing the Configuration Utility Display Mode       23

-v-

Chapter 3

Using Removable Storage Cards                              25

Using ATA Hard Disk and ATA Flash Disk Cards          25

ATAINIT                                          25

ATA Drive Cards and Drive Letters                25

Formatting an ATA Drive                          27

Using SRAM Cards                                      28

SRAM Cards and Drive Letters                     28

Formatting SRAM Cards                            29

Creating a Bootable SRAM Card                    30

Part 2: CardSoft 3.1 Advanced User's Guide

Chapter 4

CardBoot                                                   31

Boot Extension ROM                                    31

ENDBOOTR.COM                                          31

Boot Messages                                         32

Chapter 5

Customizing Your System                                    33

Using Keywords to Modify System Resource Assignments  34

MEM, IO, and IRQ Lines in CSALLOC.INI            34

IOEXCLUDE                                        35

IOINCLUDE                                        35

IRQEXCLUDE                                       36

IRQINCLUDE                                       36

MEMEXCLUDE                                       36

MEMINCLUDE                                       37

RIO                                              37

RIRQ                                             38

RMEM                                             38

SIO                                              38

SIRQ                                             39

SMEM                                             39

Using CONFIG to Modify Resource Assignments           39

Editing Modified Resources List                  40

Using a Customized .INI File                          42

Configuring System Beeps                              42

Customizing CONFIG.SYS                                43

Required DEVICEHIGH Lines                        44

DEVICEHIGH Lines Required for ATA Card Support   44

DEVICEHIGH Lines Required for SRAM Card Support  44

DEVICEHIGH Lines Required for Flash Memory

Card Support                                  45

If You Have DOS 6.0 ...                          45

-vi-

Part 1

CardSoft 3.1

User's Guide

Chapter 1

Introduction

Installing the Software

The   Installation    Utility   displays    step-by-step

instructions  for   installing  the  CardSoft  software.

Although  the   procedure  is   easy  to   follow,   the

instructions in  this section provide additional details

on considerations  to be  taken into  account during the

CardSoft installation process.  If you received CardSoft

with   optional   Flash   File   System   support,   the

installation procedure  is the  same (with the exception

that   different   driver   files   are   installed   in

CONFIG.SYS).

Note      Some  computer   manufacturers   install   the

CardSoft  software   at  the   factory.    The

standard default  directory is cardsoft.  Some

manufacturers  may   install  CardSoft   in  a

different directory.    Please  refer  to  the

documentation that  came with your computer to

determine if  this is  the  case.    To  check

whether or  not CardSoft  is already installed

on your  computer, type  cd cardsoft  from the

DOS prompt  (C:\), then  press Enter.   If the

message  "Invalid   directory"  is  displayed,

follow  the  procedures  in  this  chapter  to

install  the   software.     If   the   prompt

C:\cardsoft  appears, it means the software is

already  installed;  read  the  next  section,

"Before You Begin", then go to Chapter 2.

Before You Begin

Note      If you  do not  use a  memory manager (such as

EMM386 or QEMM) on your computer, you can skip

this section.

Before using  the CardSoft  software, you  may  have  to

inform your  system's memory manager software to exclude

a certain  memory address  range, because  some PC cards

require a  particular address  range for  use as  memory

windows.   During the  installation procedure, if one of

the following  memory managers  is detected,  a  warning

screen similar  to the one shown below is displayed, and

the appropriate  DEVICE line  changes  for  your  memory

manager are also shown on the screen:

-1-

EMM386    QEMM      386MAX

��������������������������������������������������������������������Ŀ

�  The CardSoft 3.1 Install Utility has detected the presence of the �

�  EMM386 memory manager. In order for CardSoft 3.1 to function      �

�  properly, certain ranges of memory need to excluded from this     �

�  memory manager. If you wish to ensure proper functionalist of the �

�  CardSoft 3.1 drivers we suggest you add the switch X=D000-DFFF.   �

�  After the installation is complete, please edit the line in your  �

�  CONFIG.SYS file to appear as below                                �

�                                                                    �

�        DEVICE=C:\DOS\EMM386.EXE NOEMS x=D000-DFFF                  �

� ���������������������������������������������������ͻ              �

� �                                                   �              �

� ���������������������������������������������������ͼ              �

����������������������������������������������������������������������

If you see this screen (or a similar screen), write down

the displayed  DEVICE line.   When  the installation has

completed, change the memory manager device line in your

CONFIG.SYS file  to match  the  one  displayed  on  this

screen.   To edit  or display your CONFIG.SYS file, type

the following command at the DOS prompt:

edit config.sys

If you  are using  the EMM386 memory manager, look for a

line similar  to the following (if you do not see a line

like this,  then  you  are  not  using  EMM386  on  your

system):

device=emm386.exe

To exclude the address range D000-DFFF, change this line

as  shown  here  (this  is  what  is  displayed  on  the

installation screen if EMM386 is detected):

device=emm386.exe noems x=d000-dfff

If you  are using  a memory  manager  other  than  those

listed, refer  to the  manual that you received with the

memory manager  for instructions  on how  to exclude  an

address range.

How Much Memory Should You Exclude?

As a  rule of  thumb, if  you  have  a  one-slot  PCMCIA

adapter in your system, you should exclude memory region

D000-D7FF (device=emm386  noems x=d000-d7ff).    If  you

have a PCMCIA adapter with two or more slots, you should

exclude memory  region  D000-DFFF.    Depending  on  the

address space required for the PC card(s) you are using,

you may  be able  to exclude  a smaller  address  range;

refer to  the manual  you received with your PC cards to

determine if  they require a particular address range to

work properly.

-2-

What's Next?

If you  received CardSoft  with Windows  support or  you

have  the   optional  CardView   3.1  for   Windows  3.1

interface, and  you plan  to install the Windows support

files, read  the  next  section,  "Copying  SYSTEM.INI."

Otherwise, restart  your system.   After restarting, you

can begin installing (or using) CardSoft.

After installation,  if your  system emits  a single low

tone beep  when you  insert a  card, try  increasing the

address range  being excluded,  until you  hear a medium

tone followed  by a higher tone beep when you insert the

card.   For example,  if you  originally excluded  D000-

D7FF, and  the card  is not  working, try increasing the

range to  D000-DBFF.   If the card is still not working,

increase the range to D000-DFFF.  If you are using a LAN

card, check  the documentation that came with it for the

manufacturer's recommended configuration setting.

Note      Always restart  your system  when  you  change

CONFIG.SYS so  that  the  changes  can  become

effective.

Copying SYSTEM.INI

Before installing  the Windows  support  files  via  the

INSTALL program, it is a good idea to make a backup copy

of your  SYSTEM.INI file, since INSTALL modifies it.  If

you want  to de-install  CardSoft, you  can  delete  the

modified SYSTEM.INI  file and replace it with the backup

copy.

To copy  your SYSTEM.INI file:, at the DOS prompt (C:\),

type the command

copy C:\windows\system.ini C:\windows\system.bak

then press Enter.

Installation Diskette

The installation  diskette contains  the install program

files in the root directory (A:\), and the program files

in  the   cardsoft  directory  (A:\cardsoft).    If  you

received CardSoft  with Windows support, a setup program

is  also   included  in   the  root   directory  of  the

installation  diskette  (A:\  setup),  and  the  Windows

support files  are contained  in the  cardview directory

(A:\cardview).  If you received CardSoft with Flash File

System support,  the installation  program automatically

installs the  Flash File  System along with the CardSoft

files.

-3-

CardSoft Installation

1. Insert the diskette containing the CardSoft software.

2. At the DOS command line, type a:, then press Enter.

3. At the  A:\ prompt,  type install  and  press  Enter.

Then follow  the on-screen instructions.  If you need

help with a particular screen, refer to the following

sections for  more information.  On most screens, use

the   or  arrow key to highlight your selection, then

press Enter.

4. After the  software is  installed, reboot your system

to activate the CardSoft drivers.

Initial Screen

First Time  Install -  Select this  option if this is

there are  no previous versions of CardSoft installed

on your system.

Upgrade -  Select this  option if there is a previous

version of  CardSoft on  your system, and you want to

upgrade to this release.

De-installation -  Select this  option if you want to

remove CardSoft from your system.

Exit - Select this option to exit the Install utility

without installing CardSoft.

Installation Type

Standard Installation  - Select  this option  if  you

want to  install all  components, and  you  want  the

Install  utility   to   automatically   modify   your

CONFIG.SYS file.

Custom Installation  - Select this option if you want

to select  the components  that are  installed, OR if

you want to manually update your CONFIG.SYS file with

the required DEVICEHIGH lines.

Exit - Select this option to exit the Install utility

without installing CardSoft.

Number of PCMCIA Sockets

Select the  EXACT number of PCMCIA slots on your system.

If the number you select is too low, some of your PCMCIA

slots will  not work.   If  the number you select is too

high, you will be wasting system resources.

Destination Drive

Select the  drive  you  want  to  install  the  CardSoft

software on.

The drive  letters that  are displayed  depend  on  your

system's drive configuration, and whether or not you are

connected to a network.  In most cases, you may only see

Drive C (or Drive C and Drive D).

-4-

Destination Directory

Specify the  directory in  which you want to install the

CardSoft software.    We  recommend  that  you  use  the

default, CARDSOFT.   To  change  the  default,  use  the

backspace key  to delete  CARDSOFT,  then  type  in  the

desired directory name.

Socket Services Driver

This screen  appears only  if  the  Install  utility  is

unable to  detect the  type of PCMCIA controller in your

system.   Refer to your system manual to determine which

Socket Services  driver you  should install.   Highlight

the appropriate  driver, use the Space Bar to toggle the

NO to  YES, then  press Enter  to install the driver and

continue.    Select  "Do  Not  Install  Socket  Services

Driver" only  if a proprietary Socket Services driver is

already installed, or if the driver your system requires

is not provided with CardSoft.

CardSoft Components Detected

This screen  appears only  if  CardSoft  components  are

already present  in the specified destination directory.

It indicates that CardSoft has already been installed on

your system.

������������������������������������������������������������������Ŀ

�           PLEASE NOTE THE FOLLOWING WARNING !                    �

�                                                                  �

�  Some CardSoft 3.1 Install Utility has detected pre-existing     �

�  PCMCIA software installed on your computer. If this software    �

�  is a version of CardSoft, then choose De-install to remove      �

�  these drivers from loading.                                     �

�                                                                  �

�  However, if this software is another type of PCMCIA software,   �

�  it is recommended to refer to your system's manual for          �

�  instructions on how to remove the drivers.                      �

�                                                                  �

�  Note that if you do not de-install the current PCMCIA software  �

�  before installing the software on this diskette, you may        �

�  encounter configuration issues with the new software.           �

�                                                                  �

�                ���������������������Ŀ                           �

�                �   De-Installation   �                           �

�                �   Exit              �                           �

�                �   Continue Anyway   �                           �

�                �����������������������                           �

��������������������������������������������������������������������

De-Installation -  Select this option to remove existing

CardSoft  drivers  from  your  system.    Then  run  the

Installation utility again to install the software.

-5-

Exit -  Select this  option to abort the Install utility

and return to DOS.  No CardSoft files will be installed.

If you  want to  overwrite the existing version, run the

Install utility  again,  and  select  Upgrade  from  the

Initial screen.

Continue Anyway  - Select  this option  to continue with

the standard  installation.  Any existing CardSoft files

on the  system will be overwritten with the files on the

installation diskette.   If  you  have  PCMCIA  software

other than CardSoft on the system, we recommend that you

not choose  this option.  If you do, you may end up with

an incompatible  mix of  PCMCIA drivers  on your system,

which  could  cause  configuration  problems  with  your

PCMCIA cards.

Component Selection Screen

This screen  appears only  during a Custom installation.

It enables  you to  select the  components you  want  to

install.   For example, if you don't use SRAM cards, you

can elect  not to install the driver for SRAM cards.  To

cancel  the   installation   of   unneeded   components,

highlight the  component, then  press the  Space Bar  to

toggle the  YES to  NO.   The  list  that  is  displayed

depends on  the CardSoft  optional packages you received

(you may  or may  not  have  optional  CardView  Windows

support files,  and you may or may not have the optional

Microsoft  Flash  File  System).    If  there  are  more

components than  can be  displayed on  this screen,  the

remaining components can be displayed by using the  key.

CONFIG.SYS Update Screen

The prompt  to update  your CONFIG.SYS file appears only

if you  selected a  Custom Installation.   The  standard

installation will  automatically modify your CONFIG.SYS.

Select YES  to automatically  update CONFIG.SYS with all

the required DEVICEHIGH lines for CardSoft, or select NO

if you  want to  manually update  CONFIG.SYS.    If  you

select NO,  a sample  CONFIG.SYS file  is created in the

CARDSOFT directory;  you can  use the  sample to  update

your system CONFIG.SYS file.

Exit Screen

A  screen   similar  to   the  above  appears  when  the

installation is  complete.   Press  Enter  to  exit  the

installation program  and return  to the DOS prompt.  If

your installation  diskette  does  not  contain  Windows

support  files,  then  your  installation  is  complete.

Reboot your  system so  that the  CardSoft settings take

effect.   If your Installation diskette contains Windows

support files,  you need to run the Setup program within

Windows to  copy the  appropriate files  to your system.

See below.

-6-

Installing Windows Support for CardSoft

1. Make sure  the installation  diskette containing  the

CardSoft Windows  support files  is in  the  diskette

drive.     (The  files  on  the  diskette  are  in  a

compressed  format.      The   installation   program

decompresses  the   files  that  are  copied  to  the

destination drive.)

2. Run Windows as you normally do.

3. From  Windows   Program  Manager,   select  the  File

pulldown menu and click on "Run".

4. Type a:setup  in the  Run dialog  box that appears on

the screen.

5. Click on the <OK> button.

The installation  program provides  you with easy-to-

follow  instructions.     When  the  installation  is

complete, make sure you close Windows and restart the

system so that the CardSoft settings take effect.

System Beeps

When you insert a card, your system emits a beep code to

let you  know whether  or not  the CardSoft software was

able to  recognize and  configure the  card.  These beep

codes are:

Medium tone followed    The PC  card was  recognized and

by high tone beep       successfully   configured   when

inserted.

Single low tone beep    The PC  card was recognized, but

not successfully configured when

inserted.     When  you  hear  a

single low  tone, it  means  the

card will  not work, and you may

have  to   change  your   system

configuration    or     CardSoft

configuration to get the card to

work.

Also, when  you remove  a card,  you should  hear a high

tone followed  by a  medium tone.   This  indicates that

CardSoft is aware that the card has been removed.

Using TDK CD02x Ethernet Cards

If you  wish to  use the  TDK CD02x  Ethernet card  with

CardSoft 3.1,  it is  necessary to make one small change

to enable this card.  In the subdirectory where CardSoft

is installed,  a file  exists called  CARDID.INI.   Edit

this file using any text edit and add the following line

to the [Libraries] section:

CardLib=TDKLAN2.CLB

Save the CARDID.INI file and restart your system.

-7-

Using  Xircom CE-10BT and CE-10B2 Ethernet LAN Cards

The CardSoft 3.1 package provides support for the Xircom

CE-10BT and  CE-10B2 Ethernet cards.  In most cases, you

should use  the software  drivers supplied  by Xircom to

enable this  LAN card.   In  the  event  that  you  want

CardSoft 3.1  to  configure  the  Xircom  cards,  it  is

necessary to make one small change.  In the subdirectory

where  CardSoft  is  installed,  a  file  exists  called

CARDID.INI.   Edit this  file using  any text editor and

add the following line to the [Libraries] section:

CardLib=XIRCOM.CLB

Save the CARDID.INI file and restart your system.

Advanced Power Management

If your  system is  designed to  conserve  battery  life

through the use of Advanced Power Management drivers, it

may be  necessary to  load the  CardSoft CS_APM  driver.

Contact your  system vendor  to determine if your system

supports Advanced Power Management.

To install  CS_APM.EXE edit your CONFIG.SYS file and add

the following  line after  the other  CardSoft 3.1 files

listed:

INSTALL=C:\CARDSOFT\CS_APM.EXE

Save the CONFIG.SYS file and reboot your system.

Removing CardSoft From Your System

Follow the  instructions in  this section if you want to

remove the CardSoft software from your system.

To de-install CardSoft:

1. Insert the CardSoft installation diskette in drive A.

2. At the DOS command line, type a:, then press Enter.

3. At the A:\ prompt, type install and press Enter.

4. Press Enter from the copyright screen.

5. Select  "De-installation",  then  follow  the  screen

prompts.

6. If you  installed the  Windows support files, and you

made a  backup copy  of your SYSTEM.INI file, replace

SYSTEM.INI with the backup copy:

copy c:\windows\system.bak c:\windows\system.ini

If you  did not  make a  backup of SYSTEM.INI, change

the following lines in SYSTEM.INI:

In the [boot] section:

comm.drv=drive:\path\sscomm.drv

comm.drv=comm.drv

-8-

In the [386Enh] section:

;device=*vcd         device=*vcd

You must  also delete  the following  lines from  the

[386Enh] section in SYSTEM.INI.

device=drive:\path\ssvrdd.386

device=drive:\path\ssvcd.386

Note      The drive  and  path  (drive:\path)  represent

where the  files necessary  for using CardSoft

with windows  are located.    The actual drive

letter and  location  were  determined  during

installation.  Unless you specified otherwise,

the drive  and path  are c:\cardview.   During

installation,  the   directory  "cardview'  is

created on  Drive C:  and the  files necessary

for using  CardSoft with  windows  are  placed

there.

-9-

-10-

Chapter 2

Using the CardSoft Utilities

This chapter describes how to use the CardSoft utilities

and commands (CONFIG, CSALLOC, and CARDINFO) to help you

configure and manage PC cards on your system.

Using CSALLOC

CSALLOC is  a DOS  program that  scans  the  system  for

available memory  (MEM), I/O  Port (IOP),  and Interrupt

Request Line  (IRQ) resources.  It is also able to write

this information  to the file CSALLOC.INI, which is used

by CardSoft  to determine  what system  resources can be

used by your PC cards.

For example,  other hardware  or software on your system

may need  to use  certain system  resources in  order to

work properly.  If CardSoft were to use these resources,

then the hardware (or software) might not work.  CSALLOC

makes sure that CardSoft does not use resources that are

needed for something else.

In addition, CSALLOC:

Scans upper memory for Read Only Memory (ROM)

Scans for the Extended Memory driver

Checks for the presence of HIGH memory

Checks for  the availability  of Upper  Memory Blocks

(UMBs)

Checks for the presence of the XMS driver.

Enables you to display the current resource status of

CardSoft.

When to Run CSALLOC

CSALLOC runs  automatically run  as part of the CardSoft

installation  and  also  when  you  start  your  system.

During installation,  a CSALLOC.INI file is created.  As

a result,  you need  to run  CSALLOC from  the  CardSoft

subdirectory (c:\cardsoft\csalloc) only when you need to

display which  system resources are being made available

to CardSoft.   Also,  if you have changed I/O Port, IRQ,

or memory  settings on  your system (for example, if you

change your  COM Port settings in Windows, or you enable

the XMS  memory manager  driver), you should run CSALLOC

to scan the system for available resources and create an

updated  CSALLOC.INI.    You  should  also  run  CSALLOC

whenever you  install  new  hardware  or  software  that

requires  specific   system  resources,   so  that   the

resources required  by the  newly-installed item  do not

conflict with CardSoft.

-11-

How to Run CSALLOC

CSALLOC can  only be  run from the DOS command line.  To

run it,  change to the CardSoft directory (cd cardsoft),

then type  one of  the following commands (shown in bold

type) and press Enter:

csalloc /s

You should use this command only if your system contains

a  Plug-and-Play  (PnP)  BIOS.    When  you  enter  this

command, CSALLOC  forces a  scan  of  system  resources,

instead of  relying on the PnP BIOS to inform CSALLOC as

to how system resources are assigned.

csalloc /r

When  you  enter  this  command,  CSALLOC  displays  the

current status  of memory (MEM), I/O Port address (IOP),

and IRQ  resources on  your  system,  as  shown  in  the

following example.   Resources  marked with  an (R)  are

reserved for  use by  your  PC  cards  or  other  system

component (for  example,  3F8-3FF  is  always  reserved,

since it  is assigned  to system COM Port 1).  Resources

marked with  an (A)  are already  allocated for  use  by

CardSoft.   Resources marked  with  an  (S)  are  shared

resources (that is, they can be shared by several system

components without creating a conflict).

Example:

MEM:

D000-DFFF

IOP:

108-16F, 170-177(R), 178-1EF, 1F8-2E7, 2E8-2EF(R),

2F0-2F7, 2F8-2FF(R), 300-36F, 370-377(R), 380-3BF,

3E8-3EF(R), 508-5EF(R), 5F8-6F6(R), 6F7-777(R),

780-7BF(R), 7E8-7EF(R), 908-9EF(R), 9F8-AF6 (R),

AF7-B77(R), B80-BBF(R), BE8-BEF(R), D08-DEF(R),

DF8-EF6(R), EF7-F77(R), F80-FBF(R), FE8-FEF(R)

IRQ:

3, 5, 9, A, B, C(R), D

csalloc /d

When you  enter this  command at the DOS prompt, you are

prompted to remove any PC cards that are inserted in the

system.   Remove any  PC  cards,  then  press  any  key.

CSALLOC  then   displays  the   contents  of   the  file

CSALLOC.INI, which  contains information  similar to the

following (the  information that  is listed  depends  on

your system configuration):

-12-

Example

Scanning upper memory for Read Only Memory (ROM)

ROM detected.

XMS driver detected.

High memory is present.

Allocated Upper Memory Block(s).

EMM driver is not present.

Scanning memory for RAM.

Scanning I/O Ports.

Scanning IRQs.

MEM=D000-DFFF

IO=108-1EF,1F8-377,380-3BF,3E8-3EF,970-977,B70-B77,D70-D77,F70-F77

IRQ=3,A-B,D,F

#

# *** Resource modifications  should be made below this line. ***

#

RIO=17-177, 2E8-2EF, 2F8-2FF, 370-377, 3E8-3FF

MEMEXCLUDE=C000-CFFF, E000-EFFF

Note      For a description of the resource lines (MEM=,

IO=, etc.), see Chapter 5.

You  can   cause  resources  to  be  included,

excluded,  reserved,   or  shared,  either  by

modifying  CSALLOC.INI,   or  by   using   the

Configuration Utility.   For more information,

see Chapter 5.

Using CARDINFO

CARDINFO is a DOS program that scans the PCMCIA slots on

your computer  and lists  information about the cards in

these slots.   Also,  if any  of the slots contain cards

when  you   start  your  computer,  CARDINFO  lists  any

warnings or  error messages  that may have occurred when

CardSoft attempted to configure these cards.

When to Run CARDINFO

CARDINFO can  be run  at any time.  Normally, you should

run CARDINFO if:

-  You  need  to  know  the  types  of  cards  that  are

currently inserted in your PCMCIA slots.

-  You need  to know  the I/O  Ports, IRQs,  and  Memory

areas that  are being  used by  your PC  cards.  This

information may be needed if you are installing other

components on  your system,  and you  need to know if

there is  going to  be a  conflict  between  the  new

component and a PC card that you are currently using.

("Conflict" means  that the  new component  and a  PC

card are  trying to use the same resource, such as an

IRQ interrupt; if this happens, either the PC card or

the new  component would  have to  be reconfigured to

use other resources that are available.)

-  You want  to turn  off (or turn on) power to a PCMCIA

slot that contains a PC card.

-13-

-  You  want   to  display   manufacturer  and   product

information about your PC cards.

-  You need  to know  the drive letter for your ATA Hard

Disk or ATA Flash Disk card.

-  You need  to know  the latest error that occurred for

an inserted card.

How to Run CARDINFO

CARDINFO can  be run  in several modes from the CardSoft

subdirectory DOS  command line.   To  run it,  change to

your CardSoft  directory (cd cardsoft), then type one of

the following  commands at the DOS prompt.  CARDINFO can

be run  from within  a Microsoft Windows DOS window; you

do not have to exit Windows to run CARDINFO.

cardinfo                Runs  CARDINFO   in  non-verbose

mode.     When  you  enter  this

command  at   the  DOS   prompt,

information   similar   to   the

following is displayed:

Slot 0

Card Type=Token Ring

Manufacturer=<card vendor name>

Product Name=Token Ring

Slot 1

Slot 1 is empty

cardinfo /v | more      Runs CARDINFO  in verbose  mode,

which  displays  more  extensive

information  about   the  PCMCIA

slots in  your  computer.    (We

recommend that  you use  the DOS

"more" switch with this command,

since   the    information   may

require more  than one screen to

be displayed).    The  following

section contains  an example  of

CARDINFO information  using  the

/v  switch.      The   type   of

information  that  is  displayed

depends on  the  type  of  cards

that your PCMCIA slots contain.

cardinfo /c             Provides      the      following

additional     Card     Services

information.

-  Card Services Release Number

-  Vendor Revision Number

-  Number of slots

-  Vendor copyright information

-14-

This  command   gives  you   the

ability to view manufacturer and

product information  for PC card

client software  other than  the

CardID component of CardSoft.

cardinfo /off <:slot>   Turns off  power to  all  PCMCIA

slots.

cardinfo /on <:slot>    Turns on  power  to  all  PCMCIA

slots.     For  both   of  these

commands, you have the option to

specify  a  slot  number.    For

example, cardinfo  /off:1  turns

off power  to Slot 1 only.  This

enables you to turn off power to

any slot  not  in  use,  without

removing the  card in that slot,

thereby conserving system power.

cardinfo /?             Displays    information    about

CARDINFO switches.

Sample CARDINFO Information

This section lists an example of the type of information

that is  displayed when you enter the cardinfo /v + more

command.   The exact  information that is listed depends

on the  PC cards  that are  currently  inserted  in  the

system, and the number of PCMCIA slots in the system.

Note      If you  have CardSoft  with optional Microsoft

Flash File System support, information on each

of the card-specific Memory Technology Drivers

(MTDs) is also displayed.

Client Information for handle 9B67:

Client Revision = 0.01

CS Support Level = 2.1

Revision Date = 02-07-1994

Client Name = "SRAM MTD"

Vendor Name = "SystemSoft Corporation"

Client Information for handle 9B3E:

Client Revision = 0.01

CS Support Level = 2.1

Revision Date = 03-14-1994

Client Name = "MTDDRV"

Vendor Name = "SystemSoft Corporation"

Slot 0:

[Card Information]

Card Type = "ATA Disk"  (Drive D:)

Manufacturer = <Vendor Name>

Product Name = <Vendor Product Name>

-15-

[Configuration Info]

Configuring client handle is 94D7

Memory + I/O Interface, Vcc 50, Vpp1 120, Vpp2 120

Config base 0200, Config values:

Option value: 42

Copy value: 00

First I/O Range 170-177, 16-bit

Second I/O Range 378-37F, 16-bit

No IRQ is assigned to this card.

Using the Configuration Utility

The Configuration Utility can be used to

-  Set or  change the IRQs and COM Port assignment order

for your fax/modem cards.

-  Set or  change the  I/O Port address, IRQ, and memory

areas that  your network  cards will  use, as well as

enable your  network cards  for  a  Fast  Token  Ring

configuration.

-  Select the  address (Primary, Secondary, or Any) that

your ATA  cards are  going to use to communicate with

the system.

-  Select the  type of video display (Color, Monochrome,

or  LCD)   you  want   to  use   when  you   run  the

Configuration Utility.

Note      The   Configuration    Utility    (CONFIG.EXE)

modifies the  CARDID.INI and CSALLOC.INI files

(the initiator  files for CardID and CSALLOC).

As a  result, whenever you are done using this

utility, you  have to  restart your system for

any of your changes to take effect.

Configuration Utility Help

On-line help  is available for many of the Configuration

Utility fields.  To access on-line help for a particular

field, position  the cursor  in the  field (or highlight

the field),  then press  F1  (or  click  on  the  <Help>

button, if available).

Running the Configuration Utility

You need  to run  the Configuration  Utility ONLY if you

are having  trouble using  a PC  card, or if you want to

customize  your   system.    To  run  the  Configuration

Utility:

1. From the DOS prompt (C:\), type cd cardsoft to change

to the CardSoft subdirectory.

2. Type config  and press  Enter.   The following screen

appears.

-16-

The menus on this screen contain the following items:

File                 Utility                Display

Edit Configuration   Resource Allocation    Color

Save Configuration                          Monochrome

Exit                                        LCD

Accessing Menu Items

Each menu  bar item  contains  a  pull  down  menu  with

various items  to choose  from.   To access  a pull down

menu, click  the left  mouse button on the menu name, or

press Alt + the highlighted key.  For example, to access

the File  menu, click  on the word File or press <Alt-F>

from the  keyboard.   When the  pull down  menu appears,

select a menu item by:

-  Clicking on it with the left mouse button

-  Pressing the down arrow key to highlight the item and

then pressing <Enter>, or

-  Pressing the  key that corresponds to the highlighted

letter.

Editing PC Card Configuration Settings

The  Configuration  Utility  enables  you  to  edit  the

general configuration  parameters for  fax/modem, modem,

network, and  ATA cards.  These configuration parameters

include items  such as  COM Port,  Network Port, and ATA

Port information, as well as general parameters (such as

having the  system beep  when a  card is inserted into a

slot).

Note      If your  PC cards are working properly, you do

not have  to edit  the PC  card  configuration

settings.  You should edit these settings only

if you  have a  PC card  that is  not  working

properly, or  if you  are an advanced user and

you want to custom-configure your system.

To edit PC card configuration parameters:

1. Select Edit  Configuration from  the File  pull  down

menu.  The Edit Configuration menu is displayed.

���������������������������������������Ŀ

�           Edit Configuration          �

�                                       �

�        <Modem Card Configuration>     �

�        <Netware Card Information>     �

�        <ATA Card Configuration>       �

�        <Card Insertion Parameters>    �

�                                       �

�������<OK>�����<Canel>���< Help>��������

2. If you  are configuring  a fax/modem  or modem  card,

select Modem Card Configuration.

-17-

If you are configuring a network card, select Network

Card Information.

If you  are configuring  an ATA card, select ATA Card

Configuration.

3. Click on  <OK>.   The following sections describe the

screens for each of these functions.

4. When done  configuring the  port information  for the

card, select  Card Insertion  Parameters.   The  Card

Insertion Parameters screen is displayed.

If "Continue  on Configuration  Failure" is  enabled,

CardID continues  to search card library records when

a card configuration is attempted and fails.

If "Beep  on Card  Events in  DOS" is  selected,  the

system emits  a beep  while in DOS whenever a card is

inserted or removed, or when there is a configuration

conflict.

If "Beep  on Card  Events in Windows is selected, the

system emits  a beep while in Windows whenever a card

is  inserted   or  removed,   or  when   there  is  a

configuration conflict.

Note      By default,  the events  that cause a beep are

card    insertion,     card    removal,    and

configuration conflict.    You  can,  however,

manually specify the events that cause a beep,

as  well   as  the   environment  (DOS  and/or

Windows) in  which beeps  occur, by  adding  a

/BEEPTYPES switch,  followed by  a hexadecimal

value, to  the "devicehigh=c:\cardsoft\cs.exe"

line in  CONFIG.SYS (see  Chapter 5  for  more

information).

An 'X' indicates that the feature is enabled.

�����������������������������������������Ŀ

�        Card Insertion Parameters        �

�                                         �

�   [ ] Continue on configuration failure �

�   [ ] Beep on Card Events in DOS        �

�   [ ] Beep on Card Events in Windows    �

�                                         �

�������<OK>�����<Canel>�����< Help>��������

5. When done  defining the  PC card configuration, click

on <OK>  from the Edit Configuration screen to return

to the main menu.

-18-

Modem Card Configuration

The Modem  Card Configuration screen is where you define

the assignment  of communication  ports (COM Ports), COM

Port addresses,  and IRQ  interrupts for  your modem and

fax/modem cards.   You  can define  I/O  ports  and  IRQ

values for as many as four COM Ports, and you can define

the COM  Port Assignment  Order (the  order in which the

ports  will   be  selected   for  configuring  Modem  or

Fax/Modem cards).

To change  the COM Port Order, position the cursor under

the value and type in the new value.

To change  the IRQ,  position the cursor under the value

and type  the new  value.  Do not change the 3-digit COM

Port address values.

������������������������������������������������������������������Ŀ

�            Modem  Card  Configuration                            �

�                                                                  �

� ���COM Port Order���Ŀ         ���COM Port Definition�����Ŀ     �

� �   First:  Port 1   �         �  Port 1: 3F8, IRQ 4       �     �

� �  Second:  Port 2   �         �  Port 2: 2F8, IRQ 3       �     �

� �   Third:  Port 3   �         �  Port 3: 3E8, IRQ 4       �     �

� �  Fourth:  Port 4   �         �  Port 4: 2E8, IRQ 3       �     �

� ����������������������         �����������������������������     �

�                                                                  �

� COM port Assignment Order      ���Other Setting�����������Ŀ     �

� (.) Insertion order            �  [X] Enable Audio         �     �

� ( ) Socket order               �  [X] Enable Ring Indicate �     �

�                                �����������������������������     �

����������< OK >��������<Cancel>�������< Help >���������������������

To select  the Assignment  Order, click  on the  desired

item, or  highlight it  and press  the space  bar.   The

selected item is marked with a bullet ().

To enable/disable  Audio or  Ring Indicate, click on the

item, or  highlight it  and press  the  space  bar  ('X'

indicates it is enabled).

COM Port Assignment Order

The COM Port Assignment Order settings determine whether

COM ports are assigned based on the order in which cards

are inserted,  or based  on the  PCMCIA slot  into which

they are inserted.

-19-

If you  select Insertion order, the first modem card you

insert into  a PCMCIA  slot would  be configured  as the

"First" port  (as listed in the COM Port Order box), the

second inserted card would be configured as the "Second"

port, etc.   For  example, if you insert a modem card in

Slot 1,  then you  insert a  card in  Slot 0, the Slot 1

card would  be configured  as the  "First" port, and the

Slot 0  card would  be configured  as the "Second" port.

In the  sample screen  on the previous page, the "First"

port is  Port 1  (COM 1).   If  you were  to change  the

"First" setting  to Port  4, the  first card  you insert

would be  assigned the  Port 4 (COM 4) values, which are

address 2E8 and IRQ 3.

If you select Socket order, the modem card you insert in

your first PCMCIA slot (Slot 0) would be configured with

the Port  1 values,  the modem  card you  insert in your

second PCMCIA slot (Slot 1) would be configured with the

Port 2  values, etc.   These  assignments would  be made

regardless  of   the  order  in  which  the  cards  were

inserted.   For example,  if you  insert a modem card in

Slot 0  after you  have already inserted a modem card in

Slot 1,  the Slot  0 card  would be  configured with the

"First" port  values, and  the  Slot  1  card  would  be

configured with the "Second" port values.

COM Port Definition

The   COM   Port   Definition   box   determines   which

communication address  and IRQ will be used for each COM

Port.  For example, in the sample screen on the previous

page, Port  1 is  assigned  COM  Port  address  3F8  and

interrupt IRQ  4.   You can  change the  IRQ assigned to

each port,  but you  should never  change the  COM  Port

addresses that  are listed.    If  you  change  the  IRQ

setting, always  change it  to a value that is listed as

available by  CSALLOC.   Also, do  not change the IRQ to

the same  value that is assigned to the Network Card IRQ

(see "Network Card Information").

COM Port Order

The COM  Port Order  settings determine  which COM Ports

are  "First",  "Second",  "Third",  or  "Fourth".    For

example, the  default settings are First=Port 1 (COM 1),

Second=Port  2  (COM  2),  Third=Port  3  (COM  3),  and

Fourth=Port 4  (COM 4).   You  can, however,  change the

order so  that First=Port  2, Second=Port  3, etc.   You

would do  this if  you need  a particular  COM Port  for

another device (such as a mouse), and you do not want to

use that COM Port for a modem card.  For example, if you

want to reserve COM 1 for your system mouse, enter "Port

1" for  "Fourth", and  "Port 2"  for "First".  That way,

COM 2  will be  the first port that your system tries to

use for your modem card.

-20-

Audio

When you enable Audio, you can hear all the normal audio

beeps that  are emitted when the modem (or fax/modem) is

attempting to  connect to a remote modem (or fax/modem).

When Audio is disabled, you do not hear these connection

beeps.

Ring Indicate

Ring Indicate  controls whether or not your system wakes

up from  a Suspend  mode when a call comes in to a modem

or fax  card.   If Ring Indicate is enabled, a system in

Suspend mode  will wake  up from  the Suspend  state  to

receive the  call.   If Ring  Indicate is not enabled, a

system in  Suspend mode  will remain  suspended, and the

call will be ignored.

Note      The Ring  Indicate feature is not supported by

all modem or fax/modem cards.

Network Card Information

The Network Card Information screen is where you specify

the base  I/O address, IRQ, and up to two memory windows

for use  with a  network card.   The base I/O address is

specified in  hexadecimal.   Except for  rare cases, the

value should  be  between  100h  and  3FFh.    The  IRQ,

specified in  decimal, may be any value between 3 and 15

that is  available on your computer.  The memory windows

are paragraph  values between C000h and EF00h (normally,

these values  are in  the range  D000-DFFF, which is the

memory range  that is excluded from use by your system's

memory manager,  as described  in Chapter 1).  Since the

last two digits must be zero, you only have to enter the

first two  digits (normally,  the appropriate  values to

use are D000 for Memory 1 and D400 for Memory 2, or D800

for Memory 1 and DC00 for Memory 2).

Note      Unless you  have a  specific reason  for doing

so, we  recommend that  you do  not change the

displayed values  for Port, IRQ, Memory 1, and

Memory 2.   Refer  to the  documentation  that

came with the card to determine if it requires

specific settings.

The Fast  Token Ring  field is used to specify the speed

for a  Token Ring card.  When this field is enabled, the

Token Ring  speed is set to 16 Megabits per second; when

disabled, the  Token Ring speed is set to 4 Megabits per

second.   You should  not enable this setting unless you

are connected to a Fast Token Ring network (your network

administrator can  tell you  whether or  not  you  are).

Fast Token  Ring is  not always  displayed, depending on

the type of system you have.

-21-

��������������������������������������������������������Ŀ

�             Network Card Configuration                 �

�                                                        �

�  Port 300  IRQ 5   Memory 1  d800  Memory 2 dc00       �

�                                                        �

�            [ ]  Fast Token Ring                        �

�                                                        �

��������< OK >�������<Cancel>������< Help >���������������

To edit  the information on this screen, simply position

the cursor on the value you want to change, then type in

the new  value.   To enable/disable  'Fast Token  Ring',

click on  the brackets  (or position  the cursor  in the

brackets and  press Enter); it is enabled when an 'X' is

displayed.  When done, click on <OK>.

ATA Card Configuration

The ATA  Card Configuration  screen is where you specify

how  CardID   will  attempt   to  configure   ATA  cards

(including Flash  disks and  rotating-media disks).  You

can specify  whether  CardID  should  try  the  standard

primary and  secondary ATA  addresses,  and  whether  it

should try  to use  the linear-address mode available on

most ATA  cards.   All three  settings can be enabled at

the same  time (this causes the software to use whatever

is available).   It is generally best to avoid selecting

only the  primary ATA  address, unless  you are  certain

there is  no disk  installed on  your computer that uses

that address.

An 'X' indicates that the item is enabled.

��������������������������������������������������������Ŀ

�                  ATA  Parameters                       �

�                                                        �

�  [ ]  Try Primary ATA address                          �

�  [ ]  Try Secondary ATA Address                        �

�  [ ]  Try Any Linear ATA Address                       �

�                                                        �

��������< OK >�������<Cancel>������< Help >���������������

To  enable/disable  an  item,  click  on  the  item,  or

highlight it  and press the space bar, to toggle the 'X'

on/off.

Saving PC Card Configuration

Once you  have edited  a PC card configuration, you must

save your  settings by selecting Save Configuration from

the File pull down menu.  The Save Configuration pick is

available only  if you  have edited  the general PC card

configuration settings.

-22-

Changing the Configuration Utility Display Mode

The Configuration  Utility can  be  displayed  in  three

modes: color, monochrome, or LCD.  To change the display

mode, select the Display pull down menu, then select the

type of  display you  want to  use.  The current display

mode is indicated by a check mark ().

-23-

-24-

Chapter 3

Using Removable Storage Cards

This chapter  describes how to prepare and use removable

storage cards  (that is,  ATA Hard  Disk/ATA Flash  Disk

cards and  SRAM  cards).    It  also  describes  how  to

determine which  drive letter(s)  you should be using to

access your removable storage cards.

Using ATA Hard Disk and ATA Flash Disk Cards

In order to use ATA Hard Disk or ATA Flash Disk cards on

your system,  your  CONFIG.SYS  file  must  contain  the

following line (in addition to the standard drivers that

are always required):

devicehigh=c:\cardsoft\atadrv.exe

Also, the  ATA card  has to be prepared before it can be

used.   To prepare the card, you have to run the ATAINIT

command from  the DOS  prompt, then  you have to use the

standard DOS FORMAT command to format the ATA card.  The

following sections describe how to do this.

ATAINIT

ATAINIT.EXE is  a disk partitioning utility that must be

used to  prepare any ATA card supported by ATADRV.  When

a new ATA card is inserted into a PCMCIA slot, it is not

recognized since  there is  no common method to find out

its physical  parameters (number  of sectors, cylinders,

etc.).   ATAINIT  interrogates  the  card  to  find  the

physical parameters to use, then prepares it for use.

Note      ATAINIT works  only with  ATA cards  that  are

supported by  the ATADRV  driver.   If you are

unable to  use your  ATA card,  it may  be  an

unsupported   card.       Check    with    the

documentation that came with the card.

ATA Drive Cards and Drive Letters

Just as  your system's hard disk has a drive letter (C:)

assigned to  it, so  do ATA  cards.  However, the letter

that is  assigned to  your ATA cards varies depending on

your system  configuration.   Basically, ATADRV  assigns

the next available drive letter on your system.

In most  cases, your  first ATA  card is  assigned drive

letter D:,  and the  second one  is assigned  E:.    If,

however, you  have multiple  drives already installed in

your system,  you may  already be using drive letters D:

and E: for these internal drives.

-25-

You can  find out which drive letters have been reserved

for your  ATA Drive  cards by using CARDINFO.  To do so,

insert the  ATA card into a PCMCIA slot.  Then, from the

C:\cardsoft directory, type cardinfo and press Enter.  A

listing similar to the following is displayed:

Slot 1:

[Card Information]

Card Type = "ATA Disk"  (Drive D:)

Manufacturer = <Vendor Name>

Product Name = <Vendor Product Name>

The drive letter is listed on the "Card Type" line.

Note      You can  configure the ATA driver as a "slave"

to MTDDRV.   If  you do  this, ATA cards share

drive  letters  with  Flash  Memory  and  SRAM

cards, and you can determine the correct drive

letter to  use by  typing mtddrv /? at the DOS

prompt from  within  the  CARDSOFT  directory.

For more  information, see  "DEVICEHIGH  Lines

Required for ATA Card Support" in Chapter 5.

You can also find out which drive letter to use when you

start your  system.   At system  startup,  a  series  of

messages scrolls up the screen as different software and

hardware drivers  are automatically  installed.   One of

these messages tells you which drive letter is the first

drive letter  that your ATA cards will be assigned.  You

can press  the PAUSE  key  to  stop  the  messages  from

scrolling so  that you  can read  them (to  continue the

system startup,  you then have to press the PRINT SCREEN

key).   As the system is starting, alternately press the

PAUSE and  PRINT SCREEN keys until you see the following

message:

SystemSoft PCMCIA IDE disk driver Version 1.01

Copyright  1992-1994  SystemSoft  Corporation.    All

rights reserved.

This driver  installed starting  at Drive  D:  for  2

unit(s).

The last  line in this message tells you the first drive

letter that  will be used for your ATA cards, as well as

the number  of drives  (units) that  are installed.  The

sample message  shown above indicates that there are two

drive letters   (D: and E:) reserved for ATA cards.  The

number of drive letters that is reserved is equal to the

number of PCMCIA slots on the system.

-26-

Formatting an ATA Drive

Caution!  Make sure  you are  using  the  correct  drive

letter when  preparing your  ATA card,  as any

information on  the specified drive is erased.

Refer to  the previous  section if you need to

know which drive letter to use.

To format a disk managed by ATADRV:

1. From the DOS prompt (C:\), type cd cardsoft and press

Enter.

2. Type

ATAINIT <drive letter>

and press  <Enter>.    Substitute  the  actual  drive

letter; if  your ATA  card has  been  assigned  drive

letter D:, you should type

ATAINIT D:

The following screen appears:

������������������������������������������������������������Ŀ

�         Fixed Cylinders: 160         <F1> - Help           �

�                   Heads: 2           <F2> - Quit           �

�  Physical Sectore/Track: 32                                �

�                                                            �

�  Here are the parameters that were returned, examine them  �

�  carefully to ensure they match what was expected.         �

�  Hit ENTER to accept and continue or <P> to Prompt.        �

��������������������������������������������������������������

Note      You can press the ESC key at any time to abort

ATAINIT processing.

3. Press  ENTER  to  accept  the  displayed  values  and

continue.   If, however, you are an experienced user,

and you want to enter specific drive parameters, type

P to display a screen that enables you to do so (then

follow the screen prompts).

4. The message  "Do  you  wish  a  compact  format?"  is

displayed.  In most cases, type N.

Note      If, however,  your ATA  card has a capacity of

4MB or less, type Y.

ATAINIT now prepares the card.  When the message

Partition table successfully written.

BPB Boot sector successfully wiped.

Proceed with a DOS "FORMAT /U" command.

appears, continue to Step 5.

-27-

5. Type

format <drive letter> /u

and press  Enter, where  <drive letter>  is the drive

letter assigned  to the  card.   For example,  if the

card is Drive D, type

format D: /u

Caution!  It is  very important that you use the correct

drive letter in this command, because all data

on the specified drive will be erased.

6. The message

WARNING: ALL DATA ON NON-REMOVABLE DISK

DRIVE D: WILL BE LOST!

Proceed with format (Y/N)?

appears; type  Y to  continue.  Wait for the drive to

be formatted, then go to Step 7.

7. When the message

Volume label (11 characters, ENTER for none)?

appears, type  in a volume label (this is simply your

own name  for the  drive) and  press Enter, or simply

press Enter  to leave  the volume  label blank  (most

people do not label their drives).

The next  message that  appears shows  you the  drive

capacity and other drive statistics.  This means that

your ATA card is now ready to be used.

Using SRAM Cards

In  order  to  use  SRAM  cards  on  your  system,  your

CONFIG.SYS file  must contain  the following  lines  (in

addition  to   the  standard  drivers  that  are  always

required):

devicehigh=c:\cardsoft\mtsram.exe

devicehigh=c:\cardsoft\mtddrv.exe

SRAM Cards and Drive Letters

In order  to use  your SRAM card, you need to know which

drive letter to use to access it:

1. From the  DOS command  prompt, type  cd cardsoft  and

press Enter to switch to the CardSoft directory.

-28-

2. Type mtddrv  /?  and  press  Enter.    The  following

message is displayed:

Drive E is partition number 00 for slot number 00.

Drive F is partition number 00 for slot number 01.

The drive  letters you  should use  are listed at the

end of  the message  that appears  (as shown  above).

The number  of drive  letters listed  depends on  the

number of PCMCIA slots in your system, and whether or

not MTDDRV is configured for multiple partitions (the

example above  is for a system with two PCMCIA slots,

configured for  one partition  per card).    In  this

example, if  you are  inserting an  SRAM card in your

first PCMCIA  slot (Slot  0),  you  would  use  drive

letter E: to access the card; if you are inserting an

SRAM card  in your  second PCMCIA  slot (Slot 1), you

would use drive letter F: to access the card.

Notes     Drive letters  are shared  by SRAM  and  Flash

Memory cards.  Using the example in Step 2, if

you insert  an SRAM card in Slot 0, it will be

assigned Drive E; if you insert a Flash Memory

card in  Slot 1,  it will use Drive F.  If you

remove the SRAM card from Slot 0, and insert a

second Flash Memory card in Slot 0, it will be

assigned Drive  E, because  that is  the drive

letter assigned  to Slot  0 for SRAM and Flash

Memory cards.

ATA cards DO NOT share drive letters with SRAM

and Flash Memory cards, unless you "slave" the

ATA driver  to MTDDRV.   For more information,

see Chapter 5.

Formatting SRAM Cards

Before you  can use  your SRAM cards, you have to format

them using  the DOS FORMAT command.  For example, if the

SRAM card is Drive F:, type

format F:

and press Enter.

For more  instructions on  using FORMAT,  see  your  DOS

User's Guide.

-29-

Creating a Bootable SRAM Card

SystemSoft provides  its CardBoot  product to enable you

to boot  your system  from an  SRAM card.  To do so, you

must use the following command to format the card:

format <drive letter:> /s /u

For example,  if the  drive letter  for the  card is E:,

type

format E: /s /u

and press Enter.

For more information on CardBoot, refer to Chapter 4.

For more information on FORMAT, refer to your DOS User's

Guide.

-30-

Part 2

CardSoft 3.1

Advanced User's Guide

Chapter 4

CardBoot

SystemSoft's CardBoot software enables you to:

-  Boot a PC from a PC Card

-  Create a bootable SRAM or Flash Memory card

CardBoot consists of the following components:

-  ENDBOOTR.COM

Boot Extension ROM

At system startup, the boot extension ROM code looks for

a PCIC-compatible adapter, and examines each slot of the

adapter for  a bootable  card.   If it finds no bootable

card, it  allows the PC to boot normally.  If it finds a

bootable card,  it prepares to make the boot area of the

card appear  to the  operating system  as Drive  A.  The

system  startup  then  proceeds  normally,  except  that

attempts to  read Drive  A are directed to the boot area

of the SRAM  card.

ENDBOOTR.COM

Once system  startup is  done, you  may want  to use the

actual Drive  A (diskette  drive) on your system.  To do

this, it  is necessary to run the ENDBOOTR program. This

program turns  off the intervention of disables the boot

extension ROM into disk reads, making the diskette drive

accessible again.

If  the   boot  code  resides  in  an  option  ROM,  run

ENDBOOTR.COM.   If you  are still  unable to access your

diskette drive, try the ENDBOOTR.COM command.

The ENDBOOT program can be executed from an AUTOEXEC.BAT

file located  on the PCMCIA card to automatically switch

Drive A  back to the system diskette drive by creating a

bootable memory  card.   To do so, perform the following

steps:

1. Insert the  bootable memory  card into a PCMCIA slot,

then restart your system.

2. From the DOS command prompt (A:\), type

edit autoexec.bat

and press Enter.

-31-

3. Move the  cursor to  the end  of  the  last  line  in

AUTOEXEC.BAT, then press Enter, and type endbootr.

Note      The ENDBOOT command should be the last command

in the AUTOEXEC.BAT file on the card

4. Save the  changes you  made to the AUTOEXEC.BAT file,

then Exit the edit utility.

Now, the  next time  you start  your system with this

memory card  inserted in  a PCMCIA  slot, the  system

will start  up from  the memory  card, then  it  will

automatically run  the ENDBOOT  program to  make your

diskette drive accessible again.

Boot Messages

The CardBoot  program can  display any  of the following

messages during system boot:

-  If the  boot  code  is  in  the  extension  ROM,  the

following  message  is  displayed  when  the  ROM  is

initialized:

PCMCIA Boot Services Option ROM

Copyright 1992  SystemSoft. All Rights Reserved

-  If the  boot code  is in the extension ROM and a PCIC

adapter is found, the following is displayed:

SystemSoft PC Card Boot Utility Version 1.02

-  When CardBoot  finds a  bootable  card,  one  of  the

following messages appears:

Found a bootable PCMCIA Card

or

Booting from Removable Media

-32-

Chapter 5

Customizing Your System

This  chapter  describes  the  different  ways  you  can

customize your system:

-  You  can   select  the   operating  environment   and

situations in which system beeps are emitted relating

to PC card events.

-  You can  modify  the  CardSoft  DEVICEHIGH  lines  in

CONFIG.SYS so  that you  are  installing  only  those

drivers that  are needed  for the  PC cards  you  are

using.

-  You can modify CSALLOC.INI to:

Exclude resources  from  being  scanned  by  CSALLOC,

thereby  preventing   CardSoft  from  trying  to  use

memory, IRQ,  or I/O  Port resources  that are needed

for  other  devices  or  drivers  installed  on  your

system.

Include resources that you know are available for use

by CardSoft, but are not being detected by CSALLOC.

Reserve resources that are specifically required by a

PC card that you are using.

Allow specific resources to be shared by CardSoft and

other system components.

Note      CSALLOC.INI can  be modified in two ways.  You

can  (a)  manually  edit  CSALLOC.INI  to  add

special "keywords"  that cause resources to be

reserved, included,  excluded, or  shared,  or

(b) use  the Resource  Allocation function  of

the Configuration  utility  to  change  system

resource assignments  (the Resource Allocation

function will then add the appropriate keyword

entries to CSALLOC.INI).

-33-

Using Keywords to Modify System Resource Assignments

CSALLOC.INI is  a file  that lists  the memory, IRQ, and

I/O Port resources that are available on your system for

use by  the CardSoft software.  It also informs CardSoft

which resources  are reserved  for use  by  specific  PC

cards that  you  have.    CSALLOC.INI  is  created  when

CardSoft is installed, and can be updated at any time by

running CSALLOC  from the DOS command line (described in

Chapter 2).

Note      Due to the fact that some system devices (such

as network  cards)  may  not  be  active  when

CSALLOC is  scanning for  available resources,

CSALLOC may  include resources that are needed

for the  inactive device,  thereby creating  a

resource  conflict  on  your  system.    Also,

system hardware  design and  security features

may  make   some  resources   undetectable  by

CSALLOC.

To resolve  this, several keywords can be manually added

to CSALLOC.INI  to exclude  resources from being used by

CardSoft.   Other keywords  also enable  you to  include

resources that  you know  are  available,  but  are  not

listed in  CSALLOC.INI.   Keywords that  enable  you  to

reserve resources  are also  available.   These keywords

can be  added to  the end of the CSALLOC.INI file (which

is located  in the  CardSoft directory)  using any  text

editor (such  as the MS-DOS EDIT program).  The keywords

are described in the following sections.

Note      Reserved resources  are those  resources  that

are specifically  required by  a  PC  card  in

order for  the card to function properly.  For

example, if  you have  a PCMCIA  network  card

that specifically requires IRQ5 and the memory

region D000-D7FF,  you would  have to  reserve

those  resources   to  prevent  CardSoft  from

assigning them  to other  PC cards  you may be

using.

MEM, IO, and IRQ Lines in CSALLOC.INI

The MEM  line in  CSALLOC.INI informs  CardSoft that the

listed  memory   areas  are  not  being  used,  and  are

available to CardSoft.  CardSoft uses these memory areas

to open  "windows" to  PC  cards  for  reading  from  or

writing to card memory.  Normally, the only memory range

that you should see on this line is D000-DFFF.

-34-

The IO  line in  CSALLOC.INI lists  ranges of  I/O  Port

addresses that  are available.   For  example, the value

108-16F means  that all  I/O  Port  addresses  from  108

through 16F  are available  to CardSoft.   If a range of

I/O Port  addresses is not listed, it will never be used

by CardSoft,  because CSALLOC  has determined that these

I/O Port  addresses  are  used  by  another  device  (or

application) on your system.

The IRQ  line in CSALLOC.INI lists all the IRQs that are

available to  CardSoft.  If a range of numbers is listed

(for example, 3-6), it means that all IRQs in that range

are available.   Any  IRQs that  are not  listed are not

available and  will never  by used  by CardSoft, because

CSALLOC  has   detected  that  they  have  already  been

assigned to something else.

Note      If a  resource is  not listed  on one of these

three lines,  you cannot use other keywords to

make it  a reserved  or shared  resource.  You

can, however,  use  an  "include"  keyword  to

include the resource; once it is included, you

can then  use other  keywords  to  make  it  a

reserved or shared resource.

IOEXCLUDE

IOEXCLUDE enables you to exclude I/O Port addresses from

being used  by CardSoft.   For example, if you know that

the I/O  Port address  range 1B8-1BF  is required  for a

sound card,  an ISA  LAN card, a joystick, or other such

device that  is installed in your system, but CSALLOC is

listing this  address range  on the  IO= line, you would

add the  following line  at the  end of your CSALLOC.INI

file:

IOEXCLUDE=1B8-1BF

This line  prevents CSALLOC  from scanning this range of

I/O Port addresses and, consequently, from listing it as

available.   If you are having a problem using a device,

refer to  the documentation you received with the device

to determine if it requires a specific I/O Port address.

IOINCLUDE

IOINCLUDE enables  you to include I/O Port addresses for

use by  CardSoft.  For example, if you know that the I/O

Port address  range 1F8-1FF  is not  being used  on your

system, but  this range is not listed on the IO= line of

CSALLOC.INI, you would add the following line at the end

of your CSALLOC.INI file:

IOINCLUDE=1F8-1FF

-35-

This line  causes  CSALLOC  to  include  this  I/O  Port

address range  as available  (it will  not, however,  be

listed on the IO= line when you run CSALLOC from the DOS

command line  to display the available resources on your

system).

IRQEXCLUDE

IRQEXCLUDE enables  you to  exclude IRQ  interrupts from

being used  by CardSoft.   For  example, if  you have  a

sound card, ISA LAN card, joystick, or other such device

that specifically  requires IRQ  5 to function properly,

but this  IRQ is  listed in  CSALLOC.INI as an available

IRQ, you  would add  the following  line at  the end  of

CSALLOC.INI to reserve IRQ 5 for your network card:

IRQEXCLUDE=5

This line  prevents CSALLOC  from scanning  IRQ  5  and,

consequently, from  listing it as an available resource.

If you are having a problem using a device, refer to the

documentation you  received with the device to determine

if it requires a specific IRQ in order to function.

IRQINCLUDE

IRQINCLUDE enables you to include IRQ interrupts for use

by CardSoft.  For example, if you know that IRQ 11 (B in

hexadecimal format)  is not  being used  on your system,

but  it  is  not  being  listed  in  CSALLOC.INI  as  an

available IRQ,  you would  add the following line at the

end of CSALLOC.INI to include this resource:

IRQINCLUDE=B

This line  causes  CSALLOC  to  include  IRQ  11  as  an

available IRQ  (it will  not, however,  be listed on the

IRQ= line when you run CSALLOC from the DOS command line

to display the available resources on your system).

MEMEXCLUDE

MEMEXCLUDE enables  you to  exclude memory  regions from

being scanned by CardSoft (and therefore being listed as

available).   Since CardSoft  normally requires only the

memory region (or part of the region) between D000-DFFF,

you can  manually exclude  any other memory regions that

are listed  on  the  MEM=  line  of  CSALLOC.INI.    For

example, if the MEM= line in CSALLOC.INI is

MEM=C800-CFFF, D000-DFFF, E000-EFFF

-36-

you  could   add  the  following  line  at  the  end  of

CSALLOC.INI to  prevent  memory  regions  C800-CFFF  and

E000-EFFF from  being scanned by CSALLOC, thereby making

these  memory   regions  available   to   other   system

components:

MEMEXCLUDE=C800-CFFF, E000-EFFF

After adding this line, the memory regions C800-CFFF and

E000-EFFF will no longer be listed on the MEM= line when

you run CSALLOC from the DOS command line to display the

available resources on your system.

MEMINCLUDE

MEMINCLUDE enables you to include memory regions for use

by CardSoft.   For  example, if you know that the memory

region D000-D7FF  is not  specifically needed  for other

system components,  but this memory region is not listed

on the  MEM= line  in CSALLOC.INI,  you  would  add  the

following line  at the  end of  your CSALLOC.INI file to

make this region available to CardSoft:

MEMINCLUDE=D000-D7FF

This line causes CSALLOC to include the D000-D7FF memory

region as a resource that CardSoft can use (it will not,

however, be listed on the MEM= line when you run CSALLOC

from the  DOS command  line  to  display  the  available

resources on your system).

RIO

RIO enables  you to  reserve I/O Port addresses that are

specifically needed  by a PC card that you are using, or

any other  system devices.   For  example, if you have a

PCMCIA network card that can only use I/O Port addresses

300-307, and  you have  an I/O  card that  uses only I/O

Port addresses 280-287, you would add the following line

at the  end of  CSALLOC.INI  to  prevent  CardSoft  from

assigning this  address range to other PC cards that you

may be using:

RIO=280-287, 300-307

Notes     If the  I/O Port  range you want to reserve is

not listed on the IO= line of CSALLOC.INI, you

must add  an IOINCLUDE  line in CSALLOC.INI to

make the  resource available  before  you  can

reserve it.

The following  IRQ resources are automatically

reserved on  most systems:  the hard drive I/O

addresses (170-177  and 370-377)  and the  COM

port I/O addresses (2E8-2EF, 2F8-2FF, 3E8-3EF,

and 3F8-3FF).

-37-

RIRQ

RIRQ enables  you to  reserve IRQ  interrupts  that  are

specifically needed  by a  PC card  that you  are using.

For example, if you have a PCMCIA network card that uses

only IRQ  12 (C  in  hexadecimal),  you  would  add  the

following line  at the  end of  CSALLOC.INI  to  prevent

CardSoft from  assigning this IRQ to other PC cards that

you may be using:

RIRQ=C

Note      If the  IRQ you  want to reserve is not listed

on the  IRQ= line of CSALLOC.INI, you must add

an  IRQINCLUDE   line  to  make  the  resource

available before you can reserve it.

RMEM

RMEM enables  you to  reserve memory  regions  that  are

specifically needed  by a  PC card  that you  are using.

For example,  if  a  PCMCIA  network  card  specifically

requires  the   memory  region  D800-DFFF  in  order  to

function properly,  you would  add the following line at

the  end   of  CSALLOC.INI   to  prevent  CardSoft  from

assigning this  memory region to other PC cards that you

may be using:

RMEM=D800-DFFF

Note      If the  memory region  you want  to reserve is

not listed  on the  MEM= line  of CSALLOC.INI,

you must  add a MEMINCLUDE line to CSALLOC.INI

to make the memory region available before you

can reserve it.

SIO

SIO enables  CardSoft to  share I/O  Port addresses with

other system  components.   For example,  if you  have a

modem application  that uses  COM port  address 2E8-2EF,

and you  want your PCMCIA modem card to use that address

when you  are using  the application,  you would add the

following line to CSALLOC.INI to enable CardSoft and the

modem application to share this I/O resource:

SIO=2E8-2EF

Note      If the  I/O Port  range you  want CardSoft  to

share  is  not  listed  on  the  IO=  line  of

CSALLOC.INI, you must add an IOINCLUDE line in

CSALLOC.INI to  make  the  resource  available

before you can make it shareable.

-38-

SIRQ

SIRQ enables CardSoft to share IRQ interrupts with other

system components.   For  example, if  you have  a modem

application that  uses IRQ  4, and  you want your PCMCIA

modem card  to also  use IRQ  4 when  you are using this

modem application,  you would  add the following line to

CSALLOC.INI to enable CardSoft and the modem application

to share this IRQ:

SIRQ=5

Note      If the  IRQ you  want to make shareable is not

listed on  the IRQ=  line of  CSALLOC.INI, you

must  add  an  IRQINCLUDE  line  to  make  the

resource available  before  you  can  make  it

shareable.

SMEM

SMEM enables CardSoft to share memory regions with other

system components.   For  example, if you have a network

application that  uses memory  region  D800-DFFF  for  a

memory window,  and you want your PCMCIA network card to

use  that   memory  region  when  you  use  the  network

application,  you   would  add  the  following  line  to

CSALLOC.INI  to   enable  CardSoft   and   the   network

application to share this memory region:

SMEM=D800-DFFF

Note      If  the   memory  region   you  want  to  make

shareable is  not listed  on the  MEM= line of

CSALLOC.INI, you must add a MEMINCLUDE line to

CSALLOC.INI  to   make   the   memory   region

available before you can reserve it.

Using CONFIG to Modify Resource Assignments

You can  use the  Resource Allocation  function  of  the

CONFIG Utility to customize system resource assignments,

instead of manually adding keywords to CSALLOC.INI.  The

Resource  Allocation  function  automatically  adds  the

appropriate keywords  to CSALLOC.INI,  based on  how you

assign resources.

To access the Resource Allocation function:

1. From the CARDSOFT subdirectory, type config and press

Enter.

-39-

2. From  the  Utility  pulldown  menu,  select  Resource

Allocation.  The following screen appears.

�������������������������������������������������������������������Ŀ

�                 Resource  Allocation                              �

�                                                                   �

�    Memory Addresses        IO Ports        IRQs                   �

�                       Scanned Resources                           �

�   ������������Ŀ     �������������Ŀ      ��������Ŀ    <  OK  >  �

�   � D000-DFFF  �     � 108-1EF     �      � 3      �              �

�   �            �     � 1F8-377     �      � 5      �    <Cancel>  �

�   �            �     � 380-3BF     �      � 9-13   �              �

�   �            �     � 3E8-3EF     �      �        �    < Help >  �

�   ��������������     ���������������      ����������              �

�                       Modified Resources                < Edit >  �

�   ������������Ŀ     �������������Ŀ      ��������Ŀ              �

�   � C000-CFFF X�     � 3F8-3FF R   �      �        �    < Scan >  �

�   � E000-EFFF X�     � 2E8-2EF R   �      �        �              �

�   �            �     � 2F8-2FF R   �      �        �              �

�   �            �     � 370-377 R   �      �        �              �

�   ��������������     ���������������      ����������              �

���������������������������������������������������������������������

The top  row of  boxes (Scanned  Resources) lists the

resources that  CSALLOC has  determined are available

for use  by CardSoft.  The information in these boxes

cannot be  modified; if you try to change them, three

beep tones are emitted.

The bottom row of boxes (Modified Resources) list all

resources that  are  marked  as  reserved,  included,

excluded,  or   shareable.     Resources  that   were

automatically excluded  or reserved when CardSoft was

installed should  already be displayed in these boxes

(normally, the  Memory and  I/O Port resources listed

in the sample screen are reserved/excluded).

The following  sections describe  the different  ways

you can modify resource assignments on your system.

Editing Modified Resources List

To edit the Modified Resources list:

1. On  the  Resource  Allocation  screen,  position  the

cursor  in  the  appropriate  resource  list  in  the

Modified Resource  section (bottom  section)  of  the

screen.   For example,  if you  want to  edit the I/O

Port assignments,  position the  cursor in the middle

box in the lower row of boxes.

2. If you want to add a range of resources to the list:

a.  Select <Edit> to display the Edit Options screen,

then select  <Add New>  to display  the  Add  New

Resource screen.

-40-

b.  Type in  the value  (or range of values) that you

want to  add to  the Modified Resource list.  For

example, if  you want  to add  the I/O port range

1E0-1E7, type  1E0 in  the left-most field, press

TAB, and type 1E7 in the right-most field.

c.  Use the TAB key to move to the resource type list

at the  bottom of  the screen.   Then use the  or

arrow key  to select  the resource type (Include,

Exclude, Reserved, or Shared).  For a description

of the  different resource  types, refer  to  the

keyword descriptions  at the  beginning  of  this

chapter.   For example, if you want to know about

reserved I/O  ports, refer  to the description of

the RIO keyword.

d.  Once   the    appropriate   resource    type   is

highlighted, press  the Space  Bar to  toggle the

bullet onto  your selection.   Then  select <OK>,

and proceed to Step 3.

If you  want to  change the  assignment of  a  listed

range of resources:

a.  Highlight the range of resources whose assignment

you want  to change  (for example, if you want to

change a  range of  I/O ports  from  Reserved  to

Shared,  highlight   the  desired  range  of  I/O

ports).

b.  Select <Edit> to display the Edit Options screen,

then  select   <Change>  to  display  the  Change

Resources screen.

c.  If you want to change part of the selected range,

type in  the range of values whose assignment you

want to change.

d.  Use the TAB key to move to the Resource Type list

at the bottom of the screen.

e.  Use the   or   arrow  key to  select the resource

type (Include,  Exclude, Reserved, or Shared) you

want to  assign to the specified resource.  For a

description  of  the  different  resource  types,

refer  to   the  keyword   descriptions  at   the

beginning of  this chapter.   For example, if you

want to  know about  reserved I/O ports, refer to

the description of the RIO keyword.

f.  Once   the    appropriate   resource    type   is

highlighted, press  the Space  Bar to  toggle the

bullet onto  your selection.   Then  select <OK>,

and proceed to Step 3.

If you  want to  delete a  resource from the list, or

part of a range of resources:

a.  Highlight the  range of  resources that  contains

the values you want to delete.

b.  Select <Edit> to display the Edit Options screen,

then  select   <Delete>  to  display  the  Delete

Resources screen.

-41-

c.  If you want to delete part of the selected range,

type in  the range  of values you want to delete.

Then select <OK>, and proceed to Step 3.

3. A confirmation  dialog box  appears.   Select <OK> to

complete the  operation and  return to  the  Resource

Allocation screen,  or select  <Cancel> to  return to

the Resource  Allocation screen  without  making  the

change.   Repeat Step  2 for each resource assignment

you want to modify.

4. When you are done modifying the list, select <OK>.

5. A dialog  box appears.   Select  Save  to  save  your

modifications.   After the  save,  select  <Done>  to

return to the CONFIG main screen, or select <Open> to

make more modifications to the Resource List.

Using a Customized .INI File

By  default,   CardSoft   gets   resource   availability

information  from   the  file  CSALLOC.INI.    You  may,

however,  want   to  create   a  customized  version  of

CSALLOC.INI to  enable you to automatically load special

resource configurations.   To do so, use any text editor

to create  the file,  and name  the file something other

than CSALLOC.INI  (e.g., MYFILE.INI).   Then  modify the

CSALLOC line  in CONFIG.SYS  to include the name of your

customized .INI file, as shown here:

devicehigh=c:\cardsoft\csalloc.exe

c:\cardsoft\myfile.ini

Configuring System Beeps

By default,  CardSoft is configured to emit system beeps

in both  DOS and Windows whenever you insert or remove a

card, or  if a  configuration conflict  is detected when

you insert a card.

You can,  however, select  the situations  in which  you

want CardSoft  to produce  a beep by adding a /BEEPTYPES

switch, followed  by  a  two-digit  hex  value,  to  the

devicehigh=c:\cardsoft\cs.exe line  in CONFIG.SYS (e.g.,

devicehigh=c:\cardsoft\cs.exe /BEEPTYPES CF).  The valid

hex values  that you can use are listed here.  The first

digit controls  the operating environment (DOS, Windows,

or both)  in which  beeps are emitted.  The second digit

controls the  types  of  events  (card  insertion,  card

removal, configuration  conflict, and artificial events)

that cause  a beep  to be  emitted.   If either digit in

this  value  is  0,  no  beeps  are  emitted  under  any

circumstances, regardless of the other digit.

-42-

First Digit

C = Beep in both DOS and Windows

8 = Beep in DOS only

4 = Beep in Windows only

0 = No beeps (even when event type beeps are selected)

Second Digit

F = Beep on  card insertion, card removal, configuration

conflict, and artificial events

E = Beep  on   card   insertion,   card   removal,   and

configuration conflict

D = Beep on card insertion, card removal, and artificial

events

C = Beep on card insertion and card removal

B = Beep on  card insertion, configuration conflict, and

artificial events

A = Beep on card insertion and configuration conflict

9 = Beep on card insertion and artificial events

8 = Beep on card insertion only

7 = Beep on  card removal,  configuration conflict,  and

artificial events

6 = Beep on card removal and configuration conflict

5 = Beep on card removal and artificial events

4 = Beep on card removal only

3 = Beep on configuration conflict and artificial events

2 = Beep on configuration conflict only

1 = Beep on artificial events only

0 = No  beeps   (even  if   operating   environment   is

specified)

Note      The term "artificial events" refers to logical

(as opposed to physical) events.  For example,

if  the   system  enters  a  power  management

Suspend state,  any cards  that  are  inserted

into  PCMCIA   slots  are  logically  removed.

Similarly, when  the system  resumes from  its

Suspend state,  any  cards  that  are  already

physically inserted  into a slot are logically

inserted.   Both  of  these  situations  would

cause a  beep to  be  emitted  if  "artificial

events" is selected as a beep event.

Customizing CONFIG.SYS

When CardSoft  is installed,  DEVICEHIGH lines are added

to your  CONFIG.SYS file  to install the various drivers

needed to  support the  different types of PC cards that

are available.   However,  depending on  the types of PC

cards you  use, you may not need to install all of these

drivers in  your system's memory.  You can prevent these

unneeded drivers  from being installed by removing their

devicehigh lines  from CONFIG.SYS,  or by adding a "rem"

statement to the beginning of the devicehigh line.

-43-

Required DEVICEHIGH Lines

The following  DEVICEHIGH lines  must always be included

in  CONFIG.SYS   in  order   for  CardSoft  to  function

properly.   You should  never delete  or "rem" out these

lines.

devicehigh=c:\cardsoft\<socket services driver>*

devicehigh=c:\cardsoft\cs.exe

devicehigh=c:\cardsoft\csalloc.exe

<additional devicehigh  lines needed to support specific

card types must be inserted here>

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

*  The socket  services driver that appears here depends

on the PCMCIA controller your system uses.

This minimum  configuration enables  you to  use  modem,

fax/modem,  network,   and  other   types  of   I/O   or

communications cards  without installing  any additional

CardSoft drivers.

DEVICEHIGH Lines Required for ATA Card Support

In order  to support  ATA drive cards and ATA Flash Disk

cards, your  CONFIG.SYS file  must contain the following

additional lines:

devicehigh=c:\cardsoft\atadrv.exe

If you  want to  "slave" the  ATA driver  to MTDDRV, you

need the following CONFIG.SYS lines.  When you slave the

ATA driver  to MTDDRV,  it allows  you  to  share  drive

letters between  SRAM, Flash Memory, and ATA cards.  The

value you  use with  the /S switch depends on the number

of PCMCIA  slots in  your system.  This example is for a

system with one PCMCIA slot (/S:1).

devicehigh=c:\cardsoft\atadrv.exe /S:1

devicehigh=c:\cardsoft\mtddrv.exe

DEVICEHIGH Lines Required for SRAM Card Support

To support SRAM cards, your CONFIG.SYS file must contain

the following additional lines:

devicehigh=c:\cardsoft\mtsram.exe

devicehigh=c:\cardsoft\mtddrv.exe

-44-

DEVICEHIGH Lines Required for Flash Memory Card Support

To support Flash Memory cards, your CONFIG.SYS file must

contain the following additional lines:

devicehigh=c:\cardsoft\mtaa.exe

devicehigh=c:\cardsoft\mtab.exe

devicehigh=c:\cardsoft\mti1.exe

devicehigh=c:\cardsoft\mti2p.exe

devicehigh=c:\cardsoft\mtddrv.exe

devicehigh=c:\cardsoft\ssmsflsh.sys

Note      You will  also need the optional MS FFS to get

Flash cards to operate properly.

If You Have DOS 6.0 ...

If you  have MS-DOS  6.0 or  greater installed  on  your

system, you can set up your CONFIG.SYS file to display a

menu  that   will  enable   you  to  choose  the  driver

configuration you  need during  a session.  For example,

if you  only need  to load network card support, you can

set up a section in your CONFIG.SYS file that loads only

those  CardSoft  drivers  that  are  needed  to  support

network cards.   If  you want  to use  an ATA  card at a

later time,  you can  have CONFIG.SYS  load  only  those

drivers that  are needed to support ATA cards.  By doing

so, you  can use  your system's memory more efficiently,

since you  won't be loading unneeded drivers into memory

when you start your system.

Following is  a sample  CONFIG.SYS file  that displays a

menu enabling  you to  load drivers to support all types

of cards,  or specific  drivers to  support a particular

type of PC card.  If you need more information on how to

set up a menu in your CONFIG.SYS file, refer to your DOS

manual.

Note      This example  is for  a system  using a Cirrus

PCMCIA controller  and CardSoft  with optional

Microsoft Flash  File System  support.    Your

CONFIG.SYS file  may differ  depending on your

PCMCIA  controller  and  whether  or  not  you

received  CardSoft   with  optional  Microsoft

Flash File System support.

Do not  include the  comment statements listed

in the right column; they are provided only to

assist  you   in  understanding   the  various

sections of this example.

-45-

Example

; This group of lines defines a standard DOS startup.

; These lines contain the commands that are normally run

; from your CONFIG.SYS file.  These lines may differ

; based on your system's configuration.

device=c:\dos\setver.exe

device=c:\dos\himem.sys

files = 40

buffers = 40

dos=high

shell=c:\dos\command.com /p /e:1024

stacks=9,256

[menu]

; This group of lines defines the menu that will be

; displayed when you start your system.

menuitem=normal, No CardSoft

menuitem=cs_all, Complete CardSoft

menuitem=cs_io, Basic CardSoft installation

menuitem=cs_ata, CardSoft w/ATA Support

menuitem=cs_flash, CardSoft w/Flash Support

menuitem=cs_sram, CardSoft w/SRAM Support

menucolor=2, 7

[normal]

; No entries are needed in this section.

[cs_all]

; This group of lines loads complete CardSoft support,

; enabling you to use all types of PC cards, including

; Flash memory cards.

devicehigh=c:\cardsoft\<socket services driver>

devicehigh=c:\cardsoft\cs.exe

devicehigh=c:\cardsoft\csalloc.exe

devicehigh=c:\cardsoft\atadrv.exe

devicehigh=c:\cardsoft\mtaa.exe

devicehigh=c:\cardsoft\mtab.exe

devicehigh=c:\cardsoft\mti1.exe

devicehigh=c:\cardsoft\mti2p.exe

devicehigh=c:\cardsoft\mtsram.exe

devicehigh=c:\cardsoft\mtddrv.exe

devicehigh=c:\cardsoft\ssmsflsh.sys

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

[cs_io]

; This group of lines loads CardSoft with I/O card

; support only (i.e., network, fax/modem, modem, and

; other types of communication cards).

devicehigh=c:\cardsoft<socket services driver>

devicehigh=c:\cardsoft\cs.exe

devicehigh=c:\cardsoft\csalloc.exe

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

-46-

[cs_ata]

; This group of lines loads CardSoft with ATA card and

; I/O card support only.

devicehigh=c:\cardsoft\<socket services driver>

devicehigh=c:\cardsoft\cs.exe

devicehigh=c:\cardsoft\csalloc.exe

devicehigh=c:\cardsoft\atadrv.exe

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

[cs_sram]

; This group of lines loads CardSoft with SRAM card and

; I/O card support only.

devicehigh=c:\cardsoft\<socket services driver>

devicehigh=c:\cardsoft\cs.exe

devicehigh=c:\cardsoft\csalloc.exe

c:\cardsoft\csalloc.ini

devicehigh=c:\cardsoft\mtsram.exe

devicehigh=c:\cardsoft\mtddrv.exe

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

[cs_flash]

; This group of lines loads CardSoft with Flash Memory

; card and I/O card support only.

devicehigh=c:\cardsoft\<socket services driver>

devicehigh=c:\cardsoft\cs.exe

devicehigh=c:\cardsoft\csalloc.exe

devicehigh=c:\cardsoft\mtaa.exe

devicehigh=c:\cardsoft\mtab.exe

devicehigh=c:\cardsoft\mti1.exe

devicehigh=c:\cardsoft\mti2p.exe

devicehigh=c:\cardsoft\mtddrv.exe

devicehigh=c:\cardsoft\ssmsflsh.sys

devicehigh=c:\cardsoft\cardid.exe

install=c:\cardsoft\cs_apm.exe

-47-