INTRODUCE

These drivers are VGA high resolution drivers (Version 3.1.7) for CHIPS &

TECHNIQUE 655XX VGA controller.

TABLE OF CONTENTS

Software Drivers ....................................................  2

[1]. Windows ....................................................  8

[2]. AutoCAD R12 ................................................  11

[3]. Lotus 1-2-3 ................................................  13

[4]. Lotus Symphony .............................................  13

[5]. VESA .......................................................  15

[6]. Word .......................................................  16

[7]. WordPerfect ................................................  17

SOFTWARE DRIVERS

This document describes the operation and  installation  of	the  software

drivers supplied on the Display Driver Diskettes  that  are	shipped  with

your VGA adapter.

Your VGA adapter is based on the CHIPS VGA Flat Panel/CRT  controller  and

is fully IBM VGA compatible.   This	controller  offers  a  large  set  of

extended functions and higher resolutions.  If you intend to use  your VGA

adapter in standard VGA modes only, you do not  need  to  install  any  of

these drivers.   Since your VGA adapter is fully compatible, it  does  not

require any special drivers to operate in standard modes.

The purpose of the enclosed software drivers is to take advantage  of  the

extended features of the CHIPS VGA Flat Panel/CRT controller.  These capa-

bilities include:

.  High performance in Microsoft Windows

.  Resolutions up to 1280x1024 in graphics modes with 16 colors

.  Resolutions up to 1024x768 in graphics modes with 256 colors

.  640x480 resolution in graphics modes with 32K, 64K and 16M colors

.  132 column text mode

Hardware Configuration

Some of the high-resolution drivers provided in  this  package  will  work

only in certain system configurations.   If	a  driver  does  not  display

correctly, try the following:

1.  Change the display controller to CRT-only mode, rather than flat panel

or simultaneous display mode.   Some high-resolution drivers will dis-

play correctly only in CRT mode.

2.  If a high-resolution mode is not supported on your system, try using a

lower-resolution mode.	For example, 1024x768 mode will not  work  on

some systems, but 800x600 mode is supported on most.

3.  The 65510 Flat Panel VGA Controller supports only those	drivers  that

can display on a 640x480 panel.	 See the following section on  Driver

Resolutions and Filenames for a list of the drivers that can  be  used

on the 65510.

4.  The 65535 and 65540 Flat Panel VGA Controllers support additional dri-

vers that function with the F65535 and F65540 only.  See the following

section on Driver Resolutions  and  Filenames  for  a  list  of	these

drivers.

5.  The 65545 Flat Panel VGA Controller support  additional	drivers  that

function with the F65545 only.  See the following  section  on  Driver

Resolutions and Filenames for a list of these drivers.

These software drivers support the following software applications in  the

filenames and resolutions listed:

������������������������������������������������������������������������Ŀ

� Application      Filename	       Resolution      Colors	 Notes	    �

������������������������������������������������������������������������Ĵ

� Windows  3.1							    �

�		      LINEAR4.DRV      640x480	       16	 incl. 510  �

�				       800x600	       16		    �

�				       1024x768        16		    �

�				       1280x1024       16		    �

�		      LINEAR8.DRV      640x480	       256	 incl. 510  �

�				       800x600	       256		    �

�				       1024x768        256	 535/540    �

�		      LINEAR15.DRV     640x480	       32K	 535/540    �

�		      LINEAR16.DRV     640x480	       64K	 535/540    �

�		      LINEAR24.DRV     640x480	       16M	 540	    �

�									    �

� Windows  3.1 - accelerated drivers for the F65545 only		    �

�		      VIDGX4.DRV       640x480	       16		    �

�				       800x600	       16		    �

�				       1024x768        16		    �

�				       1280x1024       16		    �

�		      VIDGX8.DRV       640x480	       256		    �

�				       800x600	       256		    �

�				       1024x768        256		    �

�		      VIDGX15.DRV      640x480	       32K		    �

�		      VIDGX16.DRV      640x480	       64K		    �

�		      VIDGX24.DRV      640x480	       16M		    �

��������������������������������������������������������������������������

Notes:  incl. 510	  Including the 65510:

The 65510 controller supports  only  these  drivers

that can display on a 640x480 panel.	These drivers

function with all other 655XX controllers.

535/540	  These drivers function with  the  65535  and	65540

controllers only.

540		  These drivers function with  the  65540  controller

only.

These software drivers support the following software applications in  the

filenames and resolutions listed (continued):

������������������������������������������������������������������������Ŀ

� Application      Filename	       Resolution      Colors	 Notes	    �

������������������������������������������������������������������������Ĵ

� AutoCAD  R12     RCTURBOC.EXP     640x480	       16	 incl. 510  �

�				       800x600	       16		    �

�				       1024x768        16		    �

�				       1280x1024       16		    �

�				       640x480	       256	 incl. 510  �

�				       800x600	       256		    �

�				       1024x768        256	 535/54X    �

�				       640x480	       32K	 535/54X    �

�				       640x480	       64K	 535/54X    �

�				       640x480	       16M	 54X	    �

������������������������������������������������������������������������Ĵ

� Lotus 1-2-3  2.0 and						    �

� Symphony  1.0/1.1 V132X25.DRV     132x25 (Text)   16		    �

�		       V132X50.DRV     132x50 (Text)   16		    �

������������������������������������������������������������������������Ĵ

� VESA  1.2	       VESA.COM        800x600	       16		    �

�				       1024x768        16		    �

�				       640x400	       256		    �

�				       640x480	       256		    �

�				       800x600	       256		    �

�				       1024x768        256	 535/54X    �

�				       640x480	       32K	 535/54X    �

�				       640x480	       64K	 535/54X    �

������������������������������������������������������������������������Ĵ

� Word  5.0	       VGA600.VID      800x600	       16		    �

�		       VGA768.VID      1024x768        16		    �

������������������������������������������������������������������������Ĵ

� Word  5.5	       VGA55600.VID    800x600	       16		    �

�		       VGA55768.VID    1024x768        16		    �

������������������������������������������������������������������������Ĵ

� WordPerfect  5.0  CHIPS600.WPD    800x600	       16		    �

�		       CHIPS768.WPD    1024x768        16		    �

������������������������������������������������������������������������Ĵ

� WordPerfect  5.1  VGA600.VRS      800x600	       16		    �

�		       VGA768.VRS      1024x768        16		    �

��������������������������������������������������������������������������

Notes:  incl. 510	  Including the 65510:

The 65510 controller supports  only  these  drivers

that can display on a 640x480 panel.	These drivers

function with all other 655XX controllers.

535/54X	  These drivers function with the  65535,  65540  and

65545 controllers only.

54X		  These drivers function with  the  65540  and	65545

controllers only.

Notational conventions

Certain conventions are used to describe the commands entered by the  user

and the response from the computer.	The  following conventions  are  used

throughout the manual:

Commands entered by the user are shown in a boldface font. Example:

COPY  A:*.*	C:

At the end of each line entered by the  user,  the  Enter  key  should  be

pressed.  This is indicated by the following symbol:

<ENTER>

Sometimes the user is required to press certain  function  keys.  This  is

indicated by enclosing the function key name in brackets.  Examples:

<F10>	   (the F10 key)

<END>	   (the "End" key)

<HOME>	   (the "Home" key)

Output from the system to the user is shown in a normal font.  Example:

C:\123

Menu alternatives are shown in a boldface italic font.  Example:

Option 1

Filenames and program names are also shown  in  a  boldface	italic	font.

Example:

AUTOEXEC.BAT

Necessary prerequisites

The instructions in this manual assume that you understand elementary con-

cepts of MS-DOS and the IBM Personal Computer.  Before you attempt to ins-

tall any driver or utility, you should:

.  Know how to copy files from a floppy disk to a directory	on  the  hard

disk.

.  Understand the MS-DOS directory structure.

.  Know how to format a floppy disk.

If you are uncertain about any of these concepts, please refer to the  DOS

or OS/2 user reference guides for more  information,  before  you  proceed

with the installation.

Before you begin

Before you begin the driver software installation,  you should make backup

copies of the Display Driver Diskettes.  Store the  original  disks	in  a

safe place.

Make sure you know the version of the application for which	you  are  in-

stalling drivers.  Your  Display  Driver  Diskettes	contain  drivers  for

several versions of certain applications.   For  your  driver  to  operate

properly, you must install the driver for your version of the  application

program.

[1]. Windows

These drivers are designed to work with  Microsoft  Windows	Version  3.1.

You may install these drivers either through Windows or in DOS.

Driver installation - Windows Setup

Step 1:     Install Windows as you normally would for a VGA display.   Run

Windows to make sure that it is working correctly.

Step 2:     Place the Display Driver Diskette #1 in drive A.   In  Windows

Program Manager, choose File from the Options Menu.  Then from

the pull-down menu, choose Run . . . .  At  the	Command  Line

prompt, type A:\WINSETUP.  Press the <ENTER> key or  click  OK

to begin the installation.  At this point  the  setup  program

locates the directory where Windows is installed.  For  proper

operation, the drivers must be installed in the	Windows  sub-

directory.  Press <ENTER> to complete the installation.	 Once

completed, the Display Driver Control  Panel  appears  on  the

screen.	 This Control Panel allows you to select and load the

installed drivers.

Another method of installing these drivers is through the File

Manager.  Click on Drive A:, and then double-click on WINSETUP

.EXE to begin installation.

Changing Display Drivers from Windows

To change display drivers from Windows, select the Windows Setup icon from

the Main window.   You will be  shown  the  current	setup  configuration.

Select Change System Settings from the Option menu.	Click on the arrow at

the end of the Display line.   You will be shown a list of display drivers

.  Click on the driver you want to select it.  Then click on the OK button

.  Follow the directions to complete the setup.

Changing Color Schemes

After you change display drivers, you may notice  that  the	color  scheme

used by Windows looks strange.  This is  because  different	drivers  have

different default colors.  You can correct this by choosing the same color

scheme or a new color scheme.   First select the Control  Panel  from  the

Main window.  Select the Color icon.  You will be shown the current	color

scheme.  Choose a new color scheme and click the OK button.

Driver installation - DOS Setup

Step 1:     Install Windows as you normally would for a VGA display.   Run

Windows to make sure that it is working correctly.   Then exit

from Windows.

Step 2:     Place the Display  Driver  Diskette #1  in  drive  A.  Type A:

<ENTER> to make this the default drive.	Type SETUP <ENTER> to

run the driver SETUP program.  Press any key  to  get  to  the

applications list.  Using the arrow keys, select Windows  Ver-

sion 3.1 and press the <ENTER> key.  Press the <ENTER> key  to

select All Resolutions, then press  <END>  to  begin  the  in-

stallation.  At this point you will be asked for the  path  to

your Windows Sysetm directory (default C:\WINDOWS).   When the

installation is complete, press any  key  to  continue.	Press

<ESC>  followed by Y to exit to DOS.

Step 3:     Change to the directory where you installed Windows (usually C

:\WINDOWS).

Step 4:     Type SETUP <ENTER> to run the Windows Setup program.   It will

show the current Windows configuration.	 Use the up arrow key

to move to the Display line and press <ENTER>.  A list of dis-

play drivers will be shown.   Use the arrow keys to select#one

of the drivers starting with an asterisk (*) and press <ENTER>

.

Step 5:     Follow the directions on the screen to complete the setup.  In

most cases,  you may press <ENTER> to accept the suggested op-

tion.   When Setup is done,  it will return to DOS.   Type WIN

<ENTER> to start Windows with the new display driver.

Changing Display Drivers from DOS

To change display drivers from DOS, change to the  Windows  directory  and

run Setup, repeating steps 4 and 5 from the previous  page.	 Besides  the

special display drivers marked by an asterisk (*), you should be  able  to

use the following standard drivers:

VGA		  640x480, 16 colors

Super VGA	  800x600, 16 colors

Panning Drivers

Special panning drivers are provided to allow high-resolution modes to  be

displayed on a flat panel or CRT.   These drivers will show a section of a

larger screen, and will automatically pan or scroll the screen horizontal-

ly and vertically when the mouse reaches the edge of the display.

[2]. AutoCAD R12

These drivers are designed to work with  Autodesk AutoCAD R12.   They con-

form to the Autodesk Device Interface (ADI) for Rendering drivers and Dis-

play drivers.  These display list drivers accelerate redraw, pan, and zoom

functions.

Driver installation

To install the drivers, follow these instructions:

Step 1:     Place  the  Display  Driver  Diskette  #1  in  drive  A.  Type

A:<ENTER> to make this the default drive.   Type SETUP <ENTER>

to run the SETUP program.   Press any key to get to the appli-

cations list.  Using the arrow keys, select AutoCAD Release 12

and press <ENTER>.  This will display a list of supported dri-

ver resolutions.   Using the arrow keys and the	<ENTER>  key,

select the resolutions that are appropriate for your  monitor.

When all of the desirede resolutions have been selected, press

<END> to begin the installation.  At this point	you  will  be

asked for a drive and directory	to  copy  the  driver  files.

Enter the dirve and  directory  that  contains  the  installed

AutoCAD R12.  You  may  be  asked  to  change  display  driver

diskettes, and if the destination directory does not exist you

will be asked for confirmation.	When the installation is com-

plete, press any key to continue.   Press <ESC>	followed by Y

to exit to DOS.

Step 2:     Go to  the  AutoCAD  directory  where  the  new	drivers  were

installed and run the driver installation  program  by  typing

ACAD12 -r <ENTER>.   This program will configure your  AutoCAD

R12 to use the new display drivers.  Select TurboDLD Classic.

Configuring TurboDLD Driver

Select Configure Video Display.  In Display	Device	Configuration  choose

Select Graphics Board/Resolution.  Then  choose  Select  Display  Graphics

Board.   After choosing a Graphics Board, go to Select Display Resolution.

After selecting Display Resolution, save the new configuration and  return

to the main menu.

Basic Configuration Menu

This menu allows you to modify:

. Number of AutoCAD Command Lines

. Font Size			     6x8/8x8/8x14/8x16/12x20/12x24

. Dual Screen		     Enable/Disable

User Interface Configuration

Double Click Interval Time

.  BP Button

.  BP Highlight		      Patt Line/Xor Rect/Both

.  BP Refresh		      Enable/Disable

.  BP Cache			      Enable/Disable

Expert Configuration Menu

This menu allows you to modify:

.  Display List		      Enable/Disable

.  Drawing Cache		      Enable/Disable

.  Use Acad 31 bit space?	      Yes/No

.  Internal Command Echo	      Enable/Disable

.  BP Zoom Mode		      Freeze/Float

.  Regen Mode		      Incremental/Fast

If you have previously installed a  driver  different  from	the  TurboDLD

driver, please note the following:

.  After installing the TurboDLD Classic, when running the Render Command,

AutoCAD R12 will take you into AVE_RENDER  to  reconfigure  Render  for

your new driver.

[3]. Lotus 1-2-3

[4]. Lotus Symphony

These drivers are designed to work with Lotus 1-2-3 Version 2.0, 2.01  and

2.2, and with Lotus Symphony Version 1.0 and 1.1.

Driver installation

To install the Lotus drivers, follow this procedure:

Step 1:     Place Display Driver Diskette #1 into drive A.  Make A the de-

fault drive by typing A:<ENTER>.  Run  the  SETUP  program  by

typing SETUP <ENTER>.  Press any key  to  display  a  list  of

supported applications.	 Use the arrow keys to select  Lotus/

Symphony and press <ENTER>.  A list of supported screen	reso-

lutions will be displayed.   Use the arrow key to  select  the

desired screen resolution and press <ENTER> ( make  sure  your

monitor is able to display  the	resolution  desired ).	Press

<END> to begin the driver  installation	process.   A  default

drive and directory path will displayed.   Use  the  backspace

key to erase this default and type in the 123  directory.   At

this point you may be prompted to  insert  one  of  the	other

driver diskettes.   You also may be asked to create the target

directory if it does not already exist.	 After the files have

been installed.	Press <ESC> followed by Y  to  exit  to  DOS.

Copy all the files that were just  created  on  the  temporary

directory onto a formatted floppy diskette.

Step 2:     Go to your 123 directory and start the  installation  program.

Type the following commands:

C: <ENTER>

INSTALL <ENTER>

Step 3:     The Lotus installation  program	will  load  and  present  the

installation menu.  From this menu, select  Advanced  Options.

From the Advanced Options menu, select  Add  New  Drivers  To

Library From the Add New Drivers Menu, select  Modify  Current

Driver Set.  From the Modify  Driver  Set  Menu,  select  Text

Display.  From the Text Display menu, select one of drivers.

Step 4:     After the selection of the appropriate VGA display driver, you

will need to exit this menu  and  return  to  the  Main	Lotus

Installation Menu.  Do this by selecting Return To Menu.

Step 5:     At the Main Lotus Installation Menu, select Save Changes

Step 6:     At this point the Installation Menu will prompt	you  for  the

name of your new Lotus configuration file.  The	Lotus  system

will prompt you with the default value -- 123.SET.  You do not

have to use this name, however.	You may want to use  a	file-

name that indicates the resolution of the driver it  contains.

For example, if you installed the 132 column by 25 line driver

, you could name this driver 132X25.SET.   Or if you installed

the 80 by 50 driver, you may want to call the file 80X50.SET.

Step 7:     The installation of your Lotus 1-2-3 driver is  now  complete.

You will need to exit the Lotus installation program  at  this

point.  At the main Lotus Installation Menu, select Exit.

Note:       If you use a different name of the driver  set  than  123.SET,

you have to remember to place the filename of your driver set

on the command line when you start Lotus 1-2-3.  For example,

if you named your driver set 132X25.SET, give t he  following

command to start Lotus 1-2-3:

123  132X25.SET <ENTER>

[5]. VESA

The Video Electronics Standards Association (VESA) has created a  standard

for a Super VGA BIOS Extension (VBE).  This defines	a  standard  software

interface to allow application program to set and control  extended	video

modes, such as 800x600 graphics, on video adapters  from  different	manu-

factures.

The VESA driver adds this Super VGA BIOS Extension to the  VGA BIOS.   Any

application program which supports the VESA standard driver interface  can

be used with this driver.  This VESA driver conforms  to  the  VESA	Super

VGA Standard #VS891001.

Driver installation

Step 1:     Place the Display Driver Diskette #1 into drive A.  Make A the

default drive by typing A:<ENTER>.  Run the SETUP  program  by

typing SETUP <ENTER>.  Press any key  to  display  a  list  of

supported applications.	Use the arrow  keys  to  select  VESA

Drive Version 1.2 and press <ENTER>.  Press the <ENTER> key to

select  All  Resolutions,   then  press	<END>  to  begin  the

installatin .  A default drive  and  directory  path  will  be

displayed.  Use the backspace key to erase this and type in  a

directory that is in the directory  path (such  as  C:\BIN  or

C:\UTILS).  At this point you may be prompted to insert one of

the  other  driver  diskettes.	After  the  files  have  been

installed, press any key to return to the  list	of  supported

appolications.  Press <ESC> followed by Y to exit to DOS.

Step 2:     To install the VESA driver, type either VESA <ENTER> or VESA +

<ENTER> at the DOS prompt.   The optional + command line para-

meter enables all of the available modes.  Make sure that your

monitor is capable of displaying these high  resolution	modes

before enabling them.

������������������������������������������������������������������������Ŀ

� NOTE:   If the video BIOS already supports VBE extended video modes,   �

�	     DO NOT use this driver.  Run the VTEST.EXE program to see if   �

�	     the video BIOS supports the VBE modes.			    �

��������������������������������������������������������������������������

[6]. Word

These drivers are designed to work with Microsoft  Word  Version  5.0  and

5.5.

Driver installation

If you have already installed Word  on  your  computer  go  to  Step 2  to

install the new video driver.

Step 1:     Install Word as you normally would.

Step 2:     After you complete the Word  installation, place  the  Display

Driver Diskette #1 into drive A.   Make A the default drive by

typing A:<ENTER>.  Run  the  SETUP  program  by	typing	SETUP

<ENTER>.  Press any key to display a list of supported  appli-

cations.  Use the arrow keys  to  select  the  desired  screen

resolution and press <ENTER> (make sure your monitor  is  able

to display the resolution desired).   Press <END> to begin the

driver installation process.  A default	drive  and  directory

path will be displayed.	Use the backspace key to  erase  this

and thpe in your Word directory.  At this  point  you  may  be

prompted to insert one of the  other  drver  diskettes.	After

the file have been installed, press any key to return  to  the

list of supported applications.	Press <ESC> followed by Y  to

exit to DOS.

Step 3:     Copy the driver file for the desired resolution that was  just

installed to SCREEN.VID.  (The driver file names are listed on

page 7.)

[7]. WordPerfect

These drivers are designed to work with WordPerfect Version	5.0  or  5.1.

They support 132-column  display  in  editing  mode,  and  high-resolution

graphics display in PreView mode.

Driver installation

Step 1:     Place Display Driver Diskette #1  into  drive A.  Make  A  the

default drive by typing A:<ENTER>.  Run the SETUP  program  by

typing SETUP <ENTER>.  Press any key  to  display  a  list  of

supported  applications.    Use	the  arrow  keys  to   select

WordPerfect and press <ENTER>.  A llist	of  supported  screen

resolutions wil be displayed.  Use the arrow key to select the

desired screen resolution and press <ENTER> ( make  sure  your

monitor is able to display  the	resolution  desired ).	Press

<END> to begin the  driver  installation  process.   A  defaul

drive and directory path will be displayed.  Use the backspace

key to erase this default and type in the  WordPerfect  direc-

tory.  At this point you may be asked to  insert  one  of  the

other drver diskettes.  After the file  have  been  installed,

press any key to return to the list of supported applications.

Press <ESC> followed by Y to exit to DOS.

Step 2:     Start WordPerfect and press <SHIFT>+<F1> to  enter  the	setup

menu.  Select D for Display and G for Graphics Screen Type and

then choose the desired Chips VGA resolution.

Configuring WordPerfect 5.0 for 132 columns

Follow these instructions to configure WordPerfect 5.0 for 132 column text

mode:

Step 1:     Use the SETCOL program to set 132 columns and  25  rows.  Give

the following command:

SETCOL  132, 25 <ENTER>

Step 2:     Start WordPerfect.  The program will detect the number of rows

and columns automatically.  If for some reason WordPerfect  is

unable to adapt to 132 columns by 25  rows, start  WordPerfect

with the following command:

WP  /SS=25,132 <ENTER>

Configuring WordPerfect 5.1 for 132 columns

Start WordPerfect and press <SHIFT>+<F1> to enter the setup menu.   Select

D for Display and T for Text Screen Type and then select Chips 132 Columns

Text.