PhoenixCARD Manager Plus

Version 3.20

User's Manual

Technologies, Ltd.

October 26, 1995

Proprietary Information

Information specific to the design contained in this document is proprietary to

Phoenix Technologies, Ltd. It is against the law to copy software on any medium

except as specifically allowed in the license or nondisclosure agreement.

Copyright

* 1994-1995 Phoenix Technologies, LTD.  All rights reserved.  This document

may not be copied, reproduced, transmitted, electronically or mechanically, stored

in a retrieval system, or translated into any computer language in any form or by

any means without the prior written permission of Phoenix Technologies, Ltd.,

2575 McCabe Way, Irvine, CA, 92714

Disclaimers

Phoenix Technologies Ltd., makes no representations or warranties with respect to

the design and documentation described in this manual.  Phoenix Technologies

Ltd., especially disclaims any implied warranties of merchant ability or fitness for

any particular purpose.  Further, this design and documentation is subject to

change without notice.

Trademarks

Phoenix is a registered trademark of Phoenix Technologies, Ltd.

Novell is a trademark of Novell, Inc.

Microsoft, MS, MS-DOS, Microsoft Flash File System I and II, and Windows are

registered trademarks of Microsoft Corporation.

Intel, Intel Series I And II Flash Cards are registered trademarks of Intel

Corporation.

IBM is a registered trademark of the International Business Machines

Corporation.

Document Number:  PCM3.2.0.10.26.95

Table of Contents

Chapter 1.  Introduction

Features

How to Use This Manual

Conventions

Chapter 2.  Installation

PCM Plus Installation and Setup

Advanced Mode Installation

Chapter 3.  Configuration

PCM For DOS Configuration

Advanced Information

Configure

Information

Option

Windows Configuration

Running PCMWin

Advanced Information

Configure

Information

Options

Chapter 4.  Advanced Configuration Considerations

Multiple Boot Support

PCM Plus Advanced Utilities

PCMFDISK.EXE

PCMRMAN.EXE

PCMWARN.EXE

Appendix A. Deinstallation

Manual Deinstallation

Removing Old PCM Plus Files

Removing Card Soft Files

Removing CardWare Files

Removing IBM PCMCIA Files

Completing the Deinstallation

Appendix B. Drivers and Memory Management

PCM Plus Requirements

Memory Management

Minimizing Memory Usage

PCM Plus Drivers

CNFIGNAM.EXE

PCMATA.SYS

Card Services

PCMMTD.EXE

PCMFFCS.EXE

PCMRMAN.SYS

PCMSCD.EXE

Socket Services

Appendix C. Troubleshooting

Index       Refer to any topic in this book

Chapter 1.  Introduction

The PhoenixCARD Manager Plus (PCM Plus) enables personal and portable

computers to use a wide range of PC cards that conform to the standards of

PCMCIA (Personal Computer Memory Card International Association). In the

past, the installation and configuration of a PCMCIA card often proved to be a

cumbersome and frustrating task.  For each and every instance of removing or

inserting a PCMCIA card, a user had to reconfigure the system so that the

computer could accept the new card. Unfortunately, sometimes the computer still

could not recognize a card.

PCM Plus eliminates the difficulty of installing a PCMCIA card. Because PCM

Plus provides simple menu selections, there is no need to learn a bookful of

commands filled with arcane utility switch options. PCM Plus works with most

cards_whether they are flash memory, SRAM Memory, network, fax/modem, or

IDE hard drives (ATA).

PCM Plus provides easy-to-use setup and configuration programs. Since PCM

Plus can instantaneously setup and reconfigure a system, a user does not have to

close an application and begin all over each time a new card is used. So, if you

need to swap out a card while in the middle of running a program, go ahead...

make PCM Plus' day!

* Features

PCM Plus offers these features:

�A setup program that enables quick and easy installation, while also providing

a full range of customization features for the sophisticated user.

�An easy-to-use deinstallation utility for removing older versions of PCMCIA

software.

�Mouse support to allow for easy selection of options and menus in DOS.

�Support for the DOS 6.0 Multiple Boot Option.

�Support for Microsoft's Flash File System II (FFSII).

�Easy-to-use information/configuration utilities for managing PC Cards in

both DOS and Microsoft Windows versions.

�Hot insertion and removal of PC cards without the need to reboot.

�Support for Power Management, including Intel/Microsoft's Advanced Power

Management (APM) 1.1.

�Support for the largest number of PCMCIA cards: Flash Memory, SRAM

Memory, Network/LAN, Fax/Modem, ATA, hard disks, SCSI, and other I/O

Cards.

�Bundled with Novell's DPMS.SYS utility, PCM Plus now loads its drivers

into Extended Memory (XMS) to maximize the availability of conventional

memory (first 640K).

* How to Use This Manual

.Install all PCM Plus programs                        .....Chapter 2

.Configure PCMCIA Cards                               .....Chapter 3

.Use PCM Plus Utilities                               .....Chapter 4

.Use PCM Plus with DOS 6.0 Multiple Configuration     .....Chapter 4

.Deinstall Existing PCMCIA Management Programs        .....Appendix A

.Install and Use Advanced Phoenix Drivers             .....Appendix B

.Optimize memory use with PCM Plus                    .....Appendix B

.Troubleshooting PCM Plus                             .....Appendix C

.Refer to any topic in this book                      .....Index

* Conventions

The table below describes notations and typographic conventions used throughout

this manual.

Format :        Convention

Meaning

UPPERCASE

a required element in a command line, such as the

command itself.

Italics

an optional parameter in a command line.

<Brackets>

keys on the keyboard; for example, <Enter> is the Enter

key and <F1> is the F1 function key.

Chapter 2. Installation

The instructions below make first-time installation of PCM Plus quick and easy.

Before installing PCM Plus, make sure the computer meets at least these minimum

requirements:

�2 MB of RAM memory.

�1 MB of free hard disk space.

�80386 microprocessor.

Additionally, the PCM Plus install program requires the following information:

�The number of PCMCIA sockets on your computer.

�The full path to the directory in which Windows is installed.  The directory

containing the WIN.INI and SYSTEM.INI files is the Windows directory.

*Note	If you plan to use Flash cards with MS-Flash file system, you must

install the flash file system now.

* PCM Plus Installation and Setup

1.Before installing PCM Plus, make a backup copy of the distribution diskette.

2.Insert the distribution diskette into drive A and type A:\INSTALL <Enter>

at the DOS prompt.

PhoenixCARD Manager Installation

����������������������������������������������������������������Ŀ

�                                                                �

�                PhoenixCARD Manager Version 3.20                �

�                                                                �

�                  PCMCIA Installation Utility                   �

�                         Revision 2.00-r01                      �

�                                                                �

�       Copyright(c) 1993, 1994, Phoenix Technologies Ltd.       �

�                      All Rights Reserved.                      �

�                                                                �

�                   Press any key to continue                    �

������������������������������������������������������������������

*Note  If the system is running DOS 6.0 with the Multiple Boot option, PCM

Plus will be installed for the current configuration.  See Chapter 4

for details on how PCM Plus works with DOS 6.0 Multiple Boot option.

If the Install program detects multiple configurations on your system, it displays

this warning:

PhoenixCARD Manager Installation

�������������������������� Warning! ������������������������ͻ

�                                                            �

� It has been determined that your system is capable of      �

� multiple boot configurations.  To be able to use the PCM+  �

� utilities it is necessary that they are installed and      �

� used with current boot configuration.  If you do not wish  �

� to use the current configuration, please exit this program �

� now, and reboot with a proper boot configuration.          �

�                                                            �

�                 Press any key to continue                  �

������������������������������������������������������������ͼ

To install PCM Plus under the current configuration, press <Enter>. To install

PCM Plus under a different configuration, exit from the Install program and

re-boot to the configuration under which you intend to use PCM Plus. If

necessary, you can install PCM Plus under all configurations. However, you must

run the setup program, PCMSETUP, for each configuration.

Optionally, you can install PCM Plus under a [COMMON] configuration which

will load the PCM Plus drivers every time the system is booted regardless of the

configuration. However, if limiting memory usage is a consideration, then you

should install PCM Plus separately for each configuration.

Because PCM Plus manipulates the EMM386 memory manager, and to optimize

memory usage, you should install EMM386 under each configuration rather than

under the [COMMON] configuration.

Next, if other PCMCIA software has been previously installed under the same

multi-boot configuration on the system, PCM Plus detects that software and

displays the following:

PhoenixCARD Manager Installation

���������������������������� Error! ���������������������������Ŀ

�                                                               �

�  It has been determined that your system has another version  �

�  of PCMCIA utilities installed.  You cannot continue          �

�  installation until the other version of PCMCIA utilities     �

�  have been removed.  Would you like PCM+ to de-install        �

�  these utilities at this time ?                               �

�                                                               �

�                            �������Ŀ                          �

�                            �  Yes  �                          �

�                            �  No   �                          �

�                            ���������                          �

�                                                               �

�����������������������������������������������������������������

If you choose No, PCM Plus terminates the installation and returns to DOS. If you

select Yes, the Deinstallation utility will guide you through:

(a)Removing or remarking out previously installed PCMCIA device drivers.

(b)Changing Windows system files.

(c)Reviewing the changes.

(d)Accepting or rejecting the changes.

*Note  PCM Plus cannot be installed while other PCMCIA software is still

loaded in the current configuration on the system. For details about

deinstallation, see Appendix A.

If PCM Plus does not detect previously installed PCMCIA software, it displays

this screen:

PhoenixCARD Manager Installation

������������������� PCMCIA Controller Found �����������������ͻ

�                                                             �

� The following controller has been detected in your system:  �

�                                                             �

�               Cirrus 6722 or 6720 or 6710                       �

�                                                             �

� Only the files needed for this controller will be installed �

� to your system.                                             �

�                                                             �

�                                                             �

�                                                             �

�                  Press any key to continue                  �

�������������������������������������������������������������ͼ

The PCMCIA Controller screen displays the PCMCIA controller that PCM Plus

has determined to be in the computer. Use the help message (press F1) for

information on which controller types are currently recognized by PCM Plus.

After you press any key, the installation continues with the Destination Drive and

Path screen:

PhoenixCARD Manager Installation

����������������� Destination Drive and Path ���������������Ŀ

�                                                            �

� Installation program is ready to copy files from your      �

� installation diskette to your hard drive.  Please specify  �

� the full path, including the drive, where you would like   �

� the PCM+ utilities installed:                              �

�                                                            �

�   C:\PCM320                                                �

�                                                            �

� Installation will require at least the following amount of �

� free disk space:                                           �

�                                                            �

�                         912 kBytes                         �

�                                                            �

��������������������������������������������������������������

In this screen, specify the name of the directory in which to install the PCM Plus

files. You can accept the default PCM320 directory by pressing <Enter>, or you

can type in a different pathname and then press <Enter>.

After you specify the destination pathname, PCM Plus begins copying files and

displays the following screen:

PhoenixCARD Manager Installation

��������������������� Copying Files �������������������Ŀ

�                                                       �

�                          40%                          �

�  �������������������������������������������������Ŀ  �

�  �                                                 �  �

�  ���������������������������������������������������  �

�   0%                                           100%   �

�             Now copying file PCMRMAN.EXE              �

���������������������������������������������������������

The Copying Files screen shows the progress of the installation.

When PCM Plus has finished copying all necessary files, it displays this message:

PhoenixCARD Manager Installation

���������������������� Files Copied ���������������������Ŀ

�                                                         �

� All files have been successfully copied to your system. �

� Installation is now ready to setup your system.         �

� The CONFIG.SYS, AUTOEXEC.BAT and Windows' SYSTEM.INI    �

� and PROGRAM.INI will be updated.                        �

�                                                         �

�                                                         �

�                                                         �

�                                                         �

�                Press any key to continue                �

�����������������������������������������������������������

When  you press any key after the files have been successfully copied, PCM Plus

continues with the Setup Program:

PhoenixCard Manager Installation

������������������������� PCMCIA Setup! �����������������������Ŀ

�                                                               �

� Welcome to PCMSETUP, the PCMCIA installation program.         �

� PCMSETUP will prompt you through the process of configuring   �

� your system to use PCMCIA cards. If you have any questions    �

� during PCMSETUP, press F1 for Help.                           �

�                                                               �

� PCMSETUP has determined that this is a first time             �

� installation. Your system should now be configured as you     �

� intend to operate it. If you have bypassed loading of device  �

� drivers, or if your system is not configured as you intend to �

� operate it, you must to exit this program, configure your     �

� system appropriately, restart your system, and then run       �

� PCMSETUP again.                                               �

�                   Press any key to continue                   �

�����������������������������������������������������������������

*Note  To modify settings or to add capabilities that you do not initially

install, you can run the Setup Program at any time after installation

by typing PCMSETUP at the DOS prompt.

If PCM Plus displays another message, follow the instructions provided therein.

For more information about messages, refer to Appendix C. Troubleshooting.

After you press any key, PCM Plus continues with the following screen:

Phoenix Card Manager Installation

����������������������� Installation Mode ���������������������Ŀ

�                                                               �

� Setup offers two modes of installation entitled "Quick Mode"  �

� and "Advanced Mode."                                          �

�                                                               �

� Quick Mode uses defaults values wherever possible.            �

� In Advanced mode setup allows you to access more detailed     �

� information about PCM Plus and completely customize its       �

� configuration.                                                �

�                                                               �

�                       ����������������Ŀ                      �

�                       �  Quick Mode    �                      �

�                       � Advanced Mode  �                      �

�                       ������������������                      �

�                                                               �

�����������������������������������������������������������������

The Quick Mode option is highlighted. Press <Enter> to select the Quick Mode.

This provides the fastest method for installing and setting up PCM Plus.

To customize the system setup, select the Advanced Mode option. Advanced Mode

allows the specifying of values for such options as IRQS, I/O addresses, memory,

and others. In most cases, the default values provided by Quick Mode are

sufficient. To designate other values, go to Advanced Mode Installation

for instructions. Of course, you can run PCMSETUP at a later time to

customize or add other values.

If you select Quick Mode, PCM Plus displays the Socket Information screen.

Phoenix Card Manager Installation

���������������������� Socket Information ���������������������Ŀ

�                                                               �

� Setup requires information on your PCMCIA hardware.           �

� Please enter the number of sockets located on your            �

� computer.  If your notebook computer is installed in          �

� a "docking bay" station please include the number of          �

� sockets attached to the bay as well.                          �

�                                                               �

� Number of sockets: 1                                          �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

The sockets are the PCMCIA ports in the computer. Most computers provide one

or two of these ports. Some computers may have more than one socket situated in a

single docking bay. PCM Plus automatically determines the number of sockets in

the computer and presents that number in the Socket Information screen.

If the number of sockets is correct, press <Enter>. If the number is not correct,

enter the correct number then press <Enter>.

After you specify the number of sockets, PCM Plus displays the Microsoft

Windows screen with Yes highlighted:

Phoenix Card Manager Installation

����������������������� Microsoft Windows ���������������������Ŀ

�                                                               �

� Setup modifies certain configuration files in your            �

� Windows directory in order for PCM Plus to work properly.     �

� Do you plan on using PCMCIA cards under Windows?              �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

If Windows is installed on the system, just press <Enter>. If Windows is not

installed on the system, use the down arrow key to highlight No, then press

<Enter>.

If Windows is installed on your system, PCM Plus then requests the complete

Windows pathname.

PhoenixCard Manager Installation

����������������������� Microsoft Windows ���������������������Ŀ

�                                                               �

� Please enter the complete pathname for Windows 3.1:           �

�                                                               �

� Full path: C:\WINDOWS                                         �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

The Windows path defaults to C:\WINDOWS. If this is correct, press <Enter>. If

this is not correct, change it to the correct path name and then press <Enter>.

After you specify the complete path to Windows, PCM Plus displays the Flash

File System screen with Yes highlighted:

Phoenix Card Manager Installation

����������������������� Flash File System ���������������������Ŀ

�                                                               �

� PCM Plus offers optional Flash File Support for PCMCIA        �

� type memory cards.  Would you like Setup to install           �

� this feature?                                                 �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

Microsoft's Flash File System for PCMCIA memory cards is the best known

program for formatting and maintaining files on the card. Most unformatted flash

cards on the market support this program. Press <Enter> to accept Yes.

If the flash card is already formatted, or if another formatting and file management

utility will be used, highlight No then press< Enter>.

After you select Yes in the Flash File System screen, PCM Plus requests the

complete path to the Flash File system:

PhoenixCard Manager Installation

����������������������� Flash File System ���������������������Ŀ

�                                                               �

� Please enter the full path for the Flash File System:         �

�                                                               �

� Full path: C:\PCM320                                          �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

The Flash File System path defaults to the C:\PCM320 directory.  If this is

acceptable, press <Enter>. If this is not correct, enter the correct path, such as

C:\FFSII, then press <Enter>.

If PCMSETUP displays the following warning after you enter the path to the Flash

File System:

�������������������������� Warning! ������������������������ͻ

� The file MS-FLASH.SYS is missing from your Flash File      �

� System directory.  PCM+ requires this file in order for    �

� the Flash File System to function.  To add FLash File      �

� suport, please copy this file as soon as possible.         �

�                                                            �

�                 Press any key to continue                  �

������������������������������������������������������������ͼ

copy the file MS-FLASH.SYS from the distribution diskette to the Flash File

System directory, and then rerun PCMSETUP.

PCM Plus then displays the Communication Ports screen with Yes highlighted.

Phoenix Card Manager Installation

���������������������� Communication Ports ��������������������Ŀ

�                                                               �

� Do you wish to use PCMCIA Modem/FAX cards in your system?     �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�����������������������������������������������������������������

Press <Enter> if you intend on using a PCMCIA modem/fax card. Otherwise,

highlight No and press <Enter>.

If you select Yes, PCM Plus displays the Communication Ports screen with COM1

(or the first available COM port on the system) highlighted:

Phoenix Card Manager Installation

���������������������� Communication Ports ��������������������Ŀ

�                                                               �

� In order for PCM+ to install correctly a communications       �

� port on your computer needs to be assigned to the             �

� Modem/FAX card.  Please select the communications port        �

� to use:                                                       �

�                                                               �

�                            ������Ŀ                           �

�                            � COM1 �                           �

�                            � COM2 �                           �

�                            � COM3 �                           �

�                            � COM4 �                           �

�                            ��������                           �

�                                                               �

�����������������������������������������������������������������

Select COM1, COM2, COM3 or COM4 by using the up or down arrow keys, then

press <Enter>.

To determine the port designation, check your computer. Many laptop and

portable computers, and most desktop computers, already have one serial port,

usually designated COM1. Inserting a fax/modem card provides a new serial port

which will normally be designated as COM2.

*Note  If you designate a COM port which is already in use, PCMSETUP

issues a warning and requests that another COM port be

designated.

After you specify the port designation, PCM Plus displays the ATA Socket screen

with Yes highlighted.

Phoenix Card Manager Installation

�������������������������� ATA Socket �������������������������Ŀ

�                                                               �

� PCM+ supports the use of ATA type cards which represent       �

� the latest in hard disk technology!                           �

�                                                               �

� Do you wish to use ATA cards?                                 �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

If ATA-type cards will be used, press <Enter>. Otherwise, highlight No and press

<Enter>.

Next, PCM Plus displays the PCMCIA Network Card Selection screen.

Phoenix Card Manager Installation

��������������������� PCMCIA Card Selection �������������������Ŀ

�                                                               �

� Please tag the PCMCIA card(s) of your choice.  Your           �

� selections will be inserted into the "active" section         �

� of your PCMCIA initialization file.                           �

�                                                               �

�                   � PCMCIA Card Selection �                   �

�                   �  IBM TOKEN RING       �                   �

�                   �  IBM Corp. Ethernet   �                   �

�                   �������������������������                   �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

Since the PCM or PCMWIN utilities will dynamically configure the system when

a network card is installed, it is not mandatory to select any network type at this

time. Simply press <Enter> to exit this screen.

To specify either Token Ring or Ethernet network for a PCMCIA network card,

follow these instructions:

(a)Use the up or down arrow to highlight one of the network types.

(b)Press the spacebar to select the option. An arrow appears to the left of the

selection. To deselect that option, press the spacebar again.

(c)Press <Enter> to confirm the choices.

At this point, PCM Plus may issue a power management warning if APM or

POWER.EXE has not been loaded. Press <Enter> to move on to the final screen.

Refer to Appendix C, Troubleshooting, for information about this, or any other

warning which PCM Plus may display.

Finally, PCM Plus displays the Setup complete screen with Yes highlighted:

PhoenixCard Manager Installation

������������������������ Setup complete! ����������������������Ŀ

�                                                               �

� PCM+ Setup is now complete! If you are unsure of the changes  �

� you have made you may exit the PCM+ Setup program without     �

� saving your changes by selecting "No".  Otherwise, to save    �

� and exit please select "Yes."                                 �

� NOTE: Please do not remove PCM+ REM statements from files.    �

�                                                               �

� Please, be sure to reboot the system to activate the changes. �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�����������������������������������������������������������������

Press <Enter> to complete the installation.  PCMSETUP updates the PCM.INI

file, PROGMAN.INI, CONFIG.SYS, WIN.INI and SYSTEM.INI files as

appropriate. It also adds the PCM Plus directory to the PATH statement in

AUTOEXEC.BAT.The program then exits to DOS.

* Adevanced Mode Installation

You may select Advanced Mode directly, or optionally, you may select the

Quick Mode during installation, but later run the setup program by typing

PCMSETUP at the DOS prompt. At that time, you can then modify or add any of

the options listed in the table below. To change the default settings for

any of the following values, select the Advanced Mode option (either during

installation or after you install PCM Plus).

Format :        Option

Settings that can be changed through Advanced Mode

PCM Plus

Resources

- IRQ setting

- CIS Window Address

Card Interaction

- Token Ring Speed

- Card Settle Time

Flash File System

- Memory Window Base Address

- Window Size

- Number of Partitions

- Number of erase queues

ATA Cards

- Spin down time

- ATA socket number

- Number of partitions

To use Advanced Mode installation, complete the following steps:

1.If you have already installed PCM Plus, type PCMSETUP at the DOS prompt.

2.Select Advanced Mode from the Installation Mode screen. Selecting Advanced

Mode displays the Socket Information screen, as shown below.

Phoenix Card Manager Installation

���������������������� Socket Information ���������������������Ŀ

�                                                               �

� Setup requires information on your PCMCIA hardware.           �

� Please enter the number of sockets located on your            �

� computer (1-8).  If your notebook computer is installed       �

� in a "docking bay" station please include the number of       �

� sockets attached to the bay as well.                          �

�                                                               �

� Number of sockets: 1                                          �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

The sockets are PCMCIA ports in the computer. Most computers provide one

or two of these ports. PCM Plus automatically determines the number of

sockets in the computer and displays that number on the screen.

If the number of sockets is correct, press <Enter>. If the number is not correct,

enter the correct number then press <Enter>.

3.Next, PCM Plus displays the Microsoft Windows screen with Yes highlighted.

Phoenix Card Manager Installation

����������������������� Microsoft Windows ���������������������Ŀ

�                                                               �

� Setup modifies certain configuration files in your            �

� Windows directory in order for PCM Plus to work properly.     �

� Do you plan on using PCMCIA cards under Windows?              �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

To use PCM Plus under Windows, select Yes and press <Enter>.

4.Provide the complete Windows pathname in the screen below:

PhoenixCard Manager Installation

����������������������� Microsoft Windows ���������������������Ŀ

�                                                               �

� Please enter the complete pathname for Windows 3.1:           �

�                                                               �

� Full path: C:\WINDOWS                                         �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

The Windows path defaults to C:\WINDOWS. If this is correct, press

<Enter>. If this is not correct, change it to the correct path name and then

press <Enter>.

After you specify the complete path to Windows, PCMSETUP displays the

Advanced Setup screen, which includes the following information:

(a)IRQ setting used by the PCM Card Services driver to determine when a

card is inserted or extracted

(b)Required CIS window address

(c)Token Ring speed to be used (if Token Ring is the network type)

(d)Card Settle Time

Phoenix Card Manager Installation

������������������������ Advanced Setup �����������������������Ŀ

�                                                               �

� IRQ ................. (0-15)        :  15                     �

� CIS Window Address .. (C800H-EF00H) :  C800                   �

� Token Ring Speed .... (4/16)        :  4                      �

� Card Settle Time .... (ms)          :  660                    �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

PCM Plus Setup automatically detects the controller in your system. For each

controller, there is a corresponding default IRQ that PCM Plus displays  in

the Advanced Setup window.

The default IRQ for each supported controller is shown in the table below.

Format :        Controller and model

Bus Type

Suggested Card Event IRQ

Cirrus Logic 6729 PPHA

PCI

11

Cirrus Logic 6710/6720/6722

ISA

11

Vadem BG-365/465/468

ISA

11

5.From the Advanced Setup window, select the required values. Use the <Tab>

key or the up/down arrow keys to go from field to field. Type in the required

values at each field. When finished, press <Enter> to accept the displayed

values.

*Note  If a value conflicts with an existing driver or device (e.g., if the IRQ

selected in the Advanced Setup window is already being used by

another device), PCM Plus displays a conflict warning. Select a

value which is available, or change the setting of the device that is

using the current value. For more information about conflicts, refer

to Appendix B and Appendix C.

The program then queries whether to install Flash File Support.

Phoenix Card Manager Installation

����������������������� Flash File System ���������������������Ŀ

�                                                               �

� PCM Plus offers optional Flash File Support for PCMCIA        �

� type memory cards.  Would you like Setup to install           �

� this feature?                                                 �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�����������������������������������������������������������������

After you select Yes in the Flash File System screen, PCM Plus requests the

complete path to the Flash File system:

PhoenixCard Manager Installation

����������������������� Flash File System ���������������������Ŀ

�                                                               �

� Please enter the full path for the Flash File System:         �

�                                                               �

� Full path: C:\PCM320                                          �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

6.The Flash File System path defaults to the C:\PCM320 directory.  If this is

acceptable, press <Enter>. If this is not correct, enter the correct path, such as

C:\FFSII, then press <Enter>.

After you enter the complete path, the program displays the following screen.

Phoenix Card Manager Installation

����������������������� Flash File System ���������������������Ŀ

�                                                               �

� Memory Window Base Address (C800H-EE00H) : CC00               �

� Window size (4 - 64 Kb in 4K steps)      : 16                 �

� Number of partitions       (1-5)         : 1                  �

� Number of erase queues     (1-5)         : 1                  �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

The Flash File System screen includes specific information about the Flash

Card:

(a)memory window base address

(b)window size

(c)number of partitions on this Flash card

(d)number of erase queues.

7.From within the Flash File System screen, use the <Tab> key or the up/down

arrow keys to go from field to field. Type in the required values at each field.

When finished, press <Enter> to accept the displayed values.

The program then displays the Communication Ports screen with Yes

highlighted.

Phoenix Card Manager Installation

���������������������� Communication Ports ��������������������Ŀ

�                                                               �

� Do you wish to use PCMCIA Modem/FAX cards in your             �

� system?                                                       �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

If you are going to use modem/fax cards with PCM Plus, press <Enter>.

Otherwise, highlight No and press <Enter>.

If Yes was specified to use modem/fax cards, PCM Plus displays the

Communication Ports screen with COM2 highlighted:

Phoenix Card Manager Installation

���������������������� Communication Ports ��������������������Ŀ

�                                                               �

� In order for PCM+ to install correctly a communications       �

� port on your computer needs to be assigned to the             �

� Modem/FAX card.  Please select the communications port        �

� to use:                                                       �

�                                                               �

�                            ������Ŀ                           �

�                            � COM1 �                           �

�                            � COM2 �                           �

�                            � COM3 �                           �

�                            � COM4 �                           �

�                            ��������                           �

�                                                               �

�����������������������������������������������������������������

8.Select COM1, COM2, COM3 or COM4 by using the up or down arrow keys,

then press <Enter>.

To determine the port designation, check the computer. Many laptop and

portable computers, and most desktop computers, already have one serial port,

usually designated COM1. Inserting a fax/modem card provides a new serial

port which will normally be designated as COM2.

*Note	 If you designate a COM port which is already in use, PCMSETUP

issues a warning and requests that another COM port be

designated.

Next, the program displays the ATA Socket screen with Yes highlighted.

Phoenix Card Manager Installation

�������������������������� ATA Socket �������������������������Ŀ

�                                                               �

� PCM+ supports the use of ATA type cards which represent       �

� the latest in hard disk technology!                           �

�                                                               �

� Do you wish to use ATA cards?                                 �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�����������������������������������������������������������������

9.If you are going to use ATA-type cards, press <Enter>. Otherwise, highlight

No and press <Enter>.

If you selected Yes for the use of ATA-type cards, PCM Plus displays the

following screen:

Phoenix Card Manager Installation

�������������������������� ATA Socket �������������������������Ŀ

�                                                               �

� The PCM+ ATA driver is capable of spinning down an ATA        �

� disk after a period of inactivity.  Please enter the          �

� number of minutes you will allow for the drive to be          �

� inactive. (Range 0-29)                                        �

�                                                               �

� ATA Drive spin down time: 5                                   �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

10.The spin down time is the period of inactivity that the ATA driver allows

before spinning down. This feature can save considerable power if the system

contains an ATA drive card. The lower the value, the fewer the minutes of

inactivity the driver allows before the it halts the ATA. You can accept the

default value that is highlighted, or you can specify another value between 0

and 29 minutes. After you designate or accept the default value, press <Enter>

to continue.

Next, the program displays the following screen:

Phoenix Card Manager Installation

�������������������������� ATA Socket �������������������������Ŀ

�                                                               �

� ATA support for Socket #0?                                    �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

11.Press <Enter> to confirm the slot with which you plan to use the ATA card.

After you confirm the slot for the ATA card, PCM Plus displays the following

screen:

Phoenix Card Manager Installation

�������������������������� ATA Socket �������������������������Ŀ

�                                                               �

� ATA support for Socket #0?                                    �

�                                                               �

� Number of partitions on ATA: 1                                �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

12.Specify the number of partitions the ATA drive will have. PCM Plus permits a

maximum of four partitions per ATA card.

*Note  PCMSETUP repeats the previous two screens for each socket in

the system.

Next, the program displays the PCMCIA Network Card Selection screen, as

follows:

Phoenix Card Manager Installation

��������������������� PCMCIA Card Selection �������������������Ŀ

�                                                               �

� Please tag the PCMCIA card(s) of your choice.  Your           �

� selections will be inserted into the "active" section         �

� of your PCMCIA initialization file.                           �

�                                                               �

�                   � PCMCIA Card Selection �                   �

�                   �  IBM TOKEN RING       �                   �

�                   �  IBM Corp. Ethernet   �                   �

�                   �������������������������                   �

�                                                               �

�                                                               �

�                                                               �

�����������������������������������������������������������������

Since both the PCM and PCMWIN utilities dynamically configure the system

when a network card is installed, it is not mandatory to select any network

type at this time. Simply press <Enter> to exit this screen.

However, if you must specify either Token Ring or Ethernet network for a

PCMCIA network card, follow these instructions:

(a)Use the up or down arrow to highlight one of the network types.

(b)Press the spacebar to select the option. An arrow appears to the left of the

selection. To deselect that option, press the spacebar again.

(c)Press <Enter> to confirm the choices.

At this point in the installation, PCM Plus may issue a power management

warning if APM or POWER.EXE has not been loaded. Press <Enter> to

move on to the final screen. Refer to Appendix C, Troubleshooting, for

information about this, or any other warning which PCM Plus may display.

PCM Plus then displays the Setup complete screen with Yes highlighted.

PhoenixCard Manager Installation

������������������������ Setup complete! ����������������������Ŀ

�                                                               �

� PCM+ Setup is now complete! If you are unsure of the changes  �

� you have made you may exit the PCM+ Setup program without     �

� saving your changes by selecting "No".  Otherwise, to save    �

� and exit please select "Yes."                                 �

� NOTE: Please do not remove PCM+ REM statements from files.    �

�                                                               �

� Please, be sure to reboot the system to activate the changes. �

�                                                               �

�                           ��������Ŀ                          �

�                           �  Yes   �                          �

�                           �  No    �                          �

�                           ����������                          �

�                                                               �

�����������������������������������������������������������������

13.Press <Enter> to complete the installation.  PCMSETUP updates the

PCM.INI file, PROGMAN.INI, CONFIG.SYS, WIN.INI and SYSTEM.INI

files as appropriate. It also adds the PCM Plus directory to the PATH

statement in AUTOEXEC.BAT.

This concludes the Advanced Mode setup.

*Note  Remember, you can run PCMSETUP again later to add or modify

any options that you do not initially select. To do so, simply type

PCMSETUP at the DOS prompt from the directory into which you

initially installed PCM Plus

Chapter 3. PC Card Configuration

Once installed, PCM Plus recognizes and dynamically configures most PCMCIA

cards when they are inserted. However, there may be instances when PCM Plus

either does not recognize the card or is unable to assign configuration resources

that do not conflict with other programs in the system.

To solve this problem, PCM Plus includes two configuration programs:

DOS-based PCM, and Windows-based PCMWin. These configuration programs

provide fast and easy configuration for PCMCIA I/O cards (including modem and

LAN cards). Both programs are explained in this chapter.

*Note  Although the configuration program will display information about

all card types, it can only configure I/O cards. PCM cannot

configure other types of cards, such as ATA or flash memory

cards. If PCM reports a non-I/O card as unconfigured, PCM Plus

will be unable to operate it. Equally important, PCM Plus cannot re-

configure I/O cards that were originally configured by their own

proprietary client driver.

When you run the configuration utilities, PCM Plus displays various windows and

menus which contain two types of fields:  those which are for viewing only and

those which you can modify. These fields are described in the two tables that

follow below. Refer to these tables when you use the configuration utilities and

their corresponding options.

Read-Only Fields:

Socket Number

The socket where the card currently resides. If the computer possesses more than

one PCMCIA socket and currently has more than one card inserted, use the

PgUp/PgDn keys to display data about cards in the previous or next socket.

Config. Number

The configuration number assigned to the current list of parameters and the total

number of configurations available to program. Use the arrow keys to switch

between them.

Format :        Manufacturer

The manufacturer's name.

Model

The model type or number. For instance, if this is a LAN card, this field identifies

whether it is an Ethernet or Token Ring card. This field can also show a serial

number, as in the previous example.

Type

The card type, such as Network LAN adapter, Flash Memory, Modem/Fax, etc.

Compliance

The version of the PCMCIA standard with which the card complies.

Configuration Loaded

Indicates that PCM Plus will use this set of configuration values to operate the

card.

Configuration Registers

The starting addresses for the register(s) where the configuration information is

stored (in hex).

Fields that can be edited:

Memory Window

When applicable, the address range where the memory window resides for this

card. This field is followed by a descriptor for the access type, 16-bit or 8-bit.  The

descriptor cannot be changed.

I/O Window

The I/O window address range for this card.  This field is followed by a descriptor

for the access type, 16-bit or 8-bit.

IRQ

The interrupt request level for this card.

* PCM For DOS Configuration

If PCM Plus has been installed for the Windows environment, go to PCMWin for

details about the Windows configuration program. To use the PCM Plus configuration

program for DOS, follow these steps:

1.At the DOS prompt, type this command:

PCM <Enter>

You will see the screen on below.

View   Information   Configure   Option   Help

�������������� General Information Per Socket ������������Ŀ

�                                                          ^

�   Socket Number: 1                                       �

�          Status: Card configured successfully            �

�    Manufacturer: RI_PICA_A                               �

�           Model: RC96ACL                                 �

�            Type: Serial Port COM2                        �

� Memory Window 1: None                                    �

� Memory Window 2: None                                    �

�    I/O Window 1: 2F8h - 2FFh                             �

�    I/O Window 2: None                                    �

�             IRQ: 3                                       �

�                                                          _

������������������������������������������������������������

Figure 3-1. General Information Per Socket window for an I/O Card.

The General Information Per Socket window displays information about the

card in the designated socket. Figure 3-1 shows how PCM displays the

General Information Per Socket window for an I/O card.

For a  memory card, the General Information Per Socket window is similar to

the following:

View   Information   Configure   Option   Help

�������������� General Information Per Socket ������������Ŀ

�                                                          ^

�   Socket Number: 1                                       �

�          Status: Write Protect is off                    �

�      Technology: FLASH                                   �

�            Size: 4096 KBytes                             �

�     File System: Unformatted or Unknown File System      �

�            Type: Memory                                  �

�                                                          �

�                                                          �

�                                                          �

�                                                          _

������������������������������������������������������������

Figure 3-2. General Information Per Socket window for a memory card.

In Figure 3-2, the fields may indicate the following:

Status:	In the case of SRAM, this field may indicate that the Battery

is good, dead, or low.

In the case of a FLASH card, this field will indicate Write

Technology: FLASH or SRAM

File System: Unformatted or Unknown..., FFS2 or TFFS

If more than one socket and card are installed in the computer, use the

PgUp/PgDn keys to view them.

2.Check the Status line. This line displays the current status of the resident

card. If PCM Plus configured an I/O card automatically, the status reads Card

configured successfully. If the program did not recognize an I/O card, or if the

I/O card has no Card Information Structure (CIS), the line reads

Unconfigured and the card must be configured manually with the PCM

configuration program.

In the case of an I/O card being Unconfigured, the Status may also indicate

that the requested configuration, I/O, IRQ, or Memory is unavailable. These

resources are unavailable, when indicated as such, because PCM Plus

excluded them to avoid hardware conflicts. The Status displays the detailed

cause of the failure to configure when PCM Plus recognizes an I/O card but

cannot configure it.

When you insert or remove a card while PCM is running, the program

displays one of these two messages:

�������Card Event Occurred�����ͻ

�                               �

� PC Card inserted in Socket 0  �

�                               �

�������������������������������ͼ

�������Card Event Occurred�����ͻ

�                               �

� PC Card removed from Socket 0 �

�                               �

�������������������������������ͼ

3.Proceed to the section Configure to:

(a)define an I/O card that PCM Plus cannot recognize.

(b)assign a new set of configuration values to an existing I/O card.

(c)edit an existing configuration.

4.To exit, press Alt-V and select the Exit option, either by highlighting Exit and

pressing <Enter> or by simply pressing <x>.

* Advanced Information

Not all information about a specific card is available in the General Information

window. However, the Advanced Information screen includes additional

information about a card, such as:

�	Register Base Address 		�	Option Register

�	Status Register 		�	Pin Replacement Register

�	Copy Register

Access the Advanced Information screen, as follows:

1.At the PCM main menu, press Alt-V and select Advanced Information.

View   Information   Configure   Option   Help

�������������� General Information Per Socket ������������Ŀ

� ������������� Advanced Information Per Socket ������������Ŀ

� �                                                          ^

� �         Socket Number: 0                                 �

� � Register Base Address: 200h                              �

� �       Option Register: 21h                               �

� �       Status Register: 18h                               �

� �  Pin Replac. Register: None                              �

� �         Copy Register: None                              �

� �                                                          �

� �                                                          �

�ĳ                                                          _

������������������������������������������������������������

Figure 3-3. Advanced Information. Sample for an I/O Card

Figure 3-3 is applicable to an I/O card. In the case of a memory card,

Advanced Information is similar to the following:

View   Information   Configure   Option   Help

�������������� General Information Per Socket ������������Ŀ

� ������������� Advanced Information Per Socket ������������Ŀ

� �                                                          ^

� �         Socket Number: 0                                 �

� �      Memory Component: Intel 28F008 (Series 2)           �

� �                                                          �

� �                                                          �

� �                                                          �

� �                                                          �

� �                                                          �

� �                                                          �

�ĳ                                                          _

������������������������������������������������������������

Figure 3-4.  Advanced Information. Sample for a Memory Card

In Figure 3-4 above, the Memory Component indicates the technology

recognized by PCM Plus. In the case of an SRAM card, the Memory

Component will indicate SRAM. PCM Plus recognizes memory cards by Intel,

AMD, and Mitsubishi.

2.After you finish reviewing the Advanced Information screen, press <Esc> to

return to the main menu.

* Configure

To configure or reconfigure a card, follow these steps:

1.At the main menu, press <Alt> < C>, and select one of the two following

options:

�������������������������Ŀ

� Add Card to List        �

� Edit Config Parameters  �

���������������������������

2.Before you can modify its configuration values, a card must be included in the

Card List.  Select Add Card to List before assigning a set of configuration

values to an inserted card that does not appear in the Card List.

To edit the configuration values of a card that is already in the Card List,

select Edit Config Parameters.

.Add Card to List

Adding a card to the list is often not necessary. Only cards that do not comply with

the PCMCIA standard (such as those manufactured before 1994) should be added

to the Card List. Further, you should add a card to the list only if PCM Plus fails

to automatically configure that card. LAN cards are normally configured by the

LAN's device driver and are almost never added to the Card List.

1.Select the Add Card to List if PCM cannot identify a card, or to define

custom card configuration values.

View   Information   Configure   Option   Help

������������������� Add New Card to List �����������������Ŀ

�                                                          ^

�           Socket Number:  0                              �

�ĳ          Config. Number:  1 of 5                         �

� ��������������������������                                 �

� �            Manufacturer: RI_PICA_A                       �

� �                   Model:  RC96ACL                        �

� �                   Type :  Serial Port COM1               �

� �              Compliance:  4.1                            �

� ��������������������������                                 �

� �    Configuration Loaded:  NO                             �

� �         Memory Window 1:  None                           �

� �         Memory Window 2:  None                           �

� �            I/O Window 1:  03F8h-03FFh (8 bits)           �

� �            I/O Window 2:  None                           �

� �                     IRQ:  4 (Level)                      �

� ��������������������������                                 �

�ĳ Configuration Registers:  20h, 0h, 0h, 0h                �

�                                                          _

������������������������������������������������������������

Figure 3-5. Sample Add New Card to List screen.

An installed card often has several card configuration possibilities, with each

configuration defining a different set of values for memory, I/O, or IRQ

settings.

2.Press the up or down arrow key to display configuration settings already

defined for this card.

Within the Add New Card to List screen (Figure 3-5), the I/O or Memory

Window fields may display a wildcard ????. The acceptable ranges for

replacing ???? are as follows:

Memory Window : from A000h to EF00h

I/O Window : from 100h to 3FFh

If you do not enter any values, PCM Plus displays an error message when it

attempts to validate the configuration and requests that you enter a value for

????.

3.Select the desired configuration by pressing <Enter>. PCM then requests

whether to validate the selection with this message:

Do you wish to validate the configuration?

No

Yes

4.Select Yes to test the selected configuration. If there are any conflicts, PCM

reports them and allows you to change the conflicting value. If there are no

conflicting values, the program reports the configuration as successful.

Select No to bypass validation and testing.

PCM then displays the following message:

Do you wish to save it?

No

Yes

5.Select No to return to the main configuration menu without saving the new

values.

Select Yes to save the new values. These parameters take effect immediately.

PCM Plus automatically writes the configuration information to the

initialization file, PCM.INI.

*Note  An I/O card usually fails to configure because of resource conflicts.

Use the PCMRMAN utility to determine available PCM Plus resources and/or

reserve system resources for PCM Plus. Once the resources required by the

I/O card are made available to PCM Plus, return to PCM and try again.

.Edit Config Parameters

1.Select Edit Config Parameters to edit the configuration values of a card

already added to the Card List.  PCM displays the following screen:

View   Information   Configure   Option   Help

����������������� Edit Card Configuration ����������������Ŀ

�                                                          ^

�           Socket Number:  0                              �

�ĳ          Config. Number:  1 of 5                         �

� ��������������������������                                 �

� �            Manufacturer:  RI_PICA_A                      �

� �                   Model:  RC96ACL                        �

� �                   Type :  Serial Port COM1               �

� �              Compliance:  4.1                            �

� ��������������������������                                 �

� �    Configuration Loaded:  NO                             �

� �         Memory Window 1:  None                           �

� �         Memory Window 2:  None                           �

� �            I/O Window 1:  03F8h-03FFh (8 bits)           �

� �            I/O Window 2:  None                           �

� �                     IRQ:  4 (Level)                      �

� ��������������������������                                 �

�ĳ Configuration Registers:  20h, 0h, 0h, 0h                �

�                                                          _

������������������������������������������������������������

Some of the lines in the Edit Config Parameters window may be edited,

while other lines are read-only.

2.From the Edit Config Parameters screen, press the up/down arrow keys to

display configuration settings already defined for a card.

3.Press the <Tab> or down arrow key to move into the fields that can be edited.

Use the left or right arrow keys to move within a field. Press the up arrow or

<Shift> + <Tab> keys to move backwards through the fields.

4.Change a field by typing in new values.

Press <Esc> at any time to return to the Config. Number field. Use the up or

down arrow key to change the configuration set to be edited. Use the <Tab>

key to return to the fields that can be edited.

5.When all the values required to configure a card have been defined, press

<Enter>. PCM then displays this message:

Do you wish to validate the configuration?

No

Yes

6.Select Yes to test the configuration values. If there are any conflicts, PCM

reports them and allows you to change the conflicting value. If there are no

conflicting values, the program reports the configuration as successful. Select

No to bypass the validation and testing process.

PCM then requests whether to save the configuration:

Do you wish to save it?

No

Yes

7.Select No to return to the main configuration screen without saving the new

values.

Select Yes to save the new values. These parameters take effect immediately.

PCM Plus automatically writes the changes  to the initialization file,

PCM.INI.

* Information

PCM Plus also permits access to a list of I/O cards, the configuration sets that

have been saved to the PCM.INI initialization file, and a list of active PCMCIA

client drivers. Such access makes it easy to quickly review configuration

information on all I/O cards previously configured under PCM Plus, as well as

information about the PCMCIA client drivers.

.Card List

The card list displays all PCMCIA cards currently enabled for use under PCM

Plus together with specific configuration information. Not all cards need to be

listed in this database in order to be used by PCM Plus. Normally, LAN,

Fax/Modem, and other cards that use multiple system resources should be added

to the configuration list. To add a card to the configuration list, select Add Card

to List from the Configure option as shown in the previous section. All fields

shown in this list are explained in the Configure section earlier in this chapter. To

review the card list, follow these steps.

1.At the PCM main menu, press Alt-I to open this pulldown menu:

�����������������Ŀ

� Card List       �

� Client Info     �

�������������������

2.Selecting Card List produces the following window:

View   Information   Configure   Option   Help

�������������� General Information Per Socket ������������Ŀ

� ���������������������� Select PC Card ��������������������Ŀ

� �[IBM                    ][TOKEN RING                   ]  �

� �[RI_PICA_A              ][RC96ACL                      ]  �

� �[RI_PICA_A              ][RC96ACL                      ]  �

� ������������������������������������������������������������

�                                                          �

� Memory Window 1: None                                    �

� Memory Window 2: None                                    �

�    I/O Window 1: 2F8h - 2FFh                             �

�    I/O Window 2: None                                    �

�             IRQ: 3                                       �

�                                                          �

�                                                          _

������������������������������������������������������������

3.Use the up/down arrow keys to highlight the desired card and press <Enter>.

PCM then displays the following screen:

View   Information   Configure   Option   Help

��������������� General Information Per Socket �������������Ŀ

� �������������� PreView PC Card Configuration �������������Ŀ

� �                                                          ^

� �           Card Number:  1 of 2                           �

� �        Config. Number:  1 of 2                           �

� ������������������������                                   �

� �          Manufacturer:  IBM                              �

� �                 Model:  TOKEN RING                       �

� �            Compliance:  4.1                              �

� ������������������������                                   �

� �  Configuration Loaded:  YES                              �

� �       Memory Window 1:  D800h - DBFFh                    �

� �       Memory Window 2:  D000h - D1FFh                    �

� �          I/O Window 1:  A20h - A23h                      �

�ĳ          I/O Window 2:  None                             �

�                   IRQ:  9 (Level)                        _

������������������������������������������������������������

4.Use the up or down arrow keys to review other sets of configuration values

stored in this list for a specific card. After you finished reviewing the list,

press <Esc> to return to the main menu.

.Client Information

The Client Program Information lists the PCM Plus and other third party client

drivers currently loaded and provides specific information on each client.

Information about each client driver includes:  Client number, Client type, Client

name, Client vendor, CS Level, Client Release (and the release date), Client

Handle. To review the client information, follow these steps:

1.At the PCM main menu, pressing Alt-I opens this pulldown menu:

�����������������Ŀ

� Card List       �

� Client Info     �

�������������������

2.Selecting Client Info produces the following screen:

View   Information   Configure   Option   Help

�������������� General Information Per Socket ������������Ŀ

� ���������������� Client Program Information ��������������Ŀ

� �                                                          ^

� �  Client Number:  1 of 4                                  �

� �    Client Type:  I/O Client                              �

� �    Client Name:  PCMSCD                                  �

� �  Client Vendor:  Phoenix Technologies, Ltd.              �

� �       CS Level:  2.10                                    �

� � Client Release:  3.10   11/06/1994                       �

� �  Client Handle:  8aa6                                    �

� �                                                          _

� ������������������������������������������������������������

�                                                          �

�                                                          �

������������������������������������������������������������

3.Use the up or down arrow keys to review other clients.

4.After you finished reviewing the list, press <Esc> to return to the main menu.

* Option

Option enables the program to confirm whenever a PCMCIA card is inserted or

removed from the computer. To activate this option:

1.From the PCM main menu, pressing Alt-O opens the following pulldown

message:

��������������������������Ŀ

�  Message On Card Events  �

����������������������������

2.Press <Enter>. PCM displays this message:

.Card Events Enabled

Once enabled, PCM Plus displays a message each time a card is inserted or

withdrawn from a PCMCIA socket, or when a card is successfully configured.

The messages look like:

�������Card Event Occurred�����ͻ

�                               �

� PC Card inserted in Socket 0  �

�                               �

�������������������������������ͼ

�������Card Event Occurred�����ͻ

�                               �

� PC Card removed from Socket 0 �

�                               �

�������������������������������ͼ

When Options is activated, a check mark (*) appears before the pulldown

message. To disable this option, simply repeat steps 1 and 2. PCM displays a

message to indicate that the Card Events feature has been disabled and the

check mark disappears from the pulldown message.

* Windows Configuration

PCMSETUP automatically installs PCMWin if Windows is installed on the

computer. The installation program inserts a PCM Plus program group into

Windows Program Manager and places the following PCMWin icon inside:

*Note  PCMWin does not work in Windows standard mode. To run

PCMWin, use only 386 Enhanced mode.

* Running PCMWin

PCMWin is very similar to its DOS counterpart, PCM.

1.Bring up Windows and select the PCMWin symbol either by double-clicking

on the symbol with the mouse or highlighting the symbol and pressing

<Enter>.

The initial PCMWin screen displays information on the card presently

inserted. To check other sockets, either use the up/down arrow keys or click

on the Previous/Next button with your mouse.

In the case of a memory card, PCMWin displays different fields that include

information as indicated below:

Status : In the case of SRAM, this field may indicate that the

Battery is good, dead, or low.

In the case of a FLASH card, this field will indicate Write

Protect is off or on.

Technology :  FLASH or SRAM

File System : Unformatted or Unknown..., FFS2 or TFFS

2.Check the Status line. This line displays the current status of the resident

card. If PCM Plus configured an I/O card automatically, the status reads

Configured. If the program did not recognize the I/O card, this line reads

Unconfigured, in which case, must configure manually the card by using

PCMWin.

In the case of an I/O card being Unconfigured, the Status may also indicate

that the requested configuration, I/O, IRQ, or Memory is unavailable. These

resources are unavailable, when indicated as such, because PCM Plus

excluded them to avoid hardware conflicts. The Status displays the detailed

cause of the failure to configure when PCM Plus recognizes an I/O card but

cannot configure it.

3.Proceed to the section, to do any of the following:

(a)Configure a card when PCM Plus cannot identify it;

(b)Add a different set of configuration parameters to an existing card;

(c)Reconfigure the card when it is necessary to change values previously

selected by the program.

4.To exit, press Alt-V and select Exit or drag down the View menu with the

mouse and click on Exit.

* Advanced Information

Advanced Information includes additional details that are not available in the

General Information window, such as:

�Memory Window 			�I/O Window

�IRQ Level			�Register Base Address

�Option Register		�Status Register

�Pin Replacement Register	�Copy Register

For information about the window addresses and register parameters associated

with a specific card, access the Advanced Information screen as follows:

1.At the PCMWin main menu, press Alt-V and select Advanced Information, or

use the mouse to drag down the View menu and select the Advanced

Information option.

In the case of a memory card, PCMWin displays two fields: Socket Number

and Memory Component. Memory Component indicates the technology

recognized by PCM Plus. In the case of an SRAM card, the Memory

Component will indicate SRAM. PCM Plus recognizes memory cards by Intel,

AMD, and Mitsubishi.

2.After you finish reviewing the screen, press <Esc> to return to the main menu.

* Configure

To configure or reconfigure a card, do the following:

*At the main menu, press Alt - C or use the mouse to select the Configure

option.

*Select Add Card to List when PCM Plus cannot recognize a card, or

when it is necessary to add a set of configuration parameters to a newly-

inserted card.

*Select Edit Card Configurations to edit the configuration values of a

card that is already saved in the PCM.INI list.

.Add Card to List

Adding a card to the list is often not necessary. Only cards that do not comply with

the PCMCIA standard (such as those manufactured before 1994) should be added

to the Card List. Further, you should add a card to the list only if PCM Plus fails

to automatically configure that card. LAN cards are normally configured by the

LAN's device driver and are almost never added to the Card List.

1.If PCMWin cannot identify a card just installed, or if it is necessary to add

another card configuration to the list of existing configurations, select the

Add Card to List option.

Each installed card can have several card configurations that define specific

memory, I/O, or IRQ settings. Select the preferred set of configuration values.

2.Press the up/down arrow keys or select the Previous/Next buttons to display

configuration settings already defined for a specific card.

Within the Add Card screen, the I/O or Memory Window fields may display a wildcard ????.

The acceptable ranges for replacing ???? are as follows:

Memory Window : from A000h to EF00h

I/O Window : from 100h to 3FFh

If you do not enter any values, PCM Plus displays an error message when it

attempts to validate the configuration and requests that you enter a value for

????.

3.Select the desired configuration by pressing <Enter>.

4.Select Yes to test the configuration values just entered. If PCMWin finds any

conflicts, it reports them and allows you to change the conflicting value. If

there are no conflicting values, the program reports the configuration as

successful.

Select No to bypass the validation and testing process. However, bypassing

the validation is not recommended.

5.Select No to return to the main configuration menu without saving the new

values.

Select Yes to save the new values. PCMWin automatically writes the added

configuration information to the PCM Plus initialization file, PCM.INI. The

next time you insert this card, PCM Plus will configure the card in the manner

that you saved the configuration.

*Note	 An I/O card usually fails to configure because of resource conflicts.

Use the PCMRMAN utility to determine available resources, and/or to reserve

system resources for PCM Plus. After you make the resources required by the

I/O card available to PCM Plus, return to PCMWin and retry configuring the

card.

.Edit Config Parameters

1.To edit an existing configuration, select Edit Config Parameters from the

Configure pulldown menu.

Only the values which appear below the Compliance field may be edited.

These include the Configuration Loaded, Memory Window, I/O Window,

IRQ, and Configuration Registers fields. The remaining fields are provided

for informational purposes only.

2.Press the up/down arrow keys or select the Previous/Next buttons to display

configuration settings already defined for the specific card.

3.Use the <Tab> key to move from field to field, the <Shift> + <Tab> keys to

move backwards through the fields, or the mouse to highlight a field.

4.Change a field by typing in new values. Select the Configuration Loaded

field by clicking the mouse button once; to deselect the field, click again.

5.When all the required changes to values for this card have been entered, either

select another configuration to add or edit by clicking on Next or Previous, or

confirm the configuration and exit to the main menu by clicking on OK.

6.Select Yes to test the configuration values just entered. If PCMWin finds any

conflicts, it reports them and allows you to change the conflicting value. If

there are no conflicting values, the program reports the configuration as

successful.

Select No to bypass the validation and testing process. However, bypassing

the validation is not recommended.

7.Select No to return to the main configuration menu without saving the new

values.

Select Yes to save the new values. The next time you insert this card, PCM

Plus will configure the card in the manner that you saved the configuration.

* Information

PCMWin also permits access to a list of I/O cards, the configuration sets that have

been saved to the PCM.INI initialization file, and a list of active PCMCIA client

drivers. Such access makes it easy to quickly review configuration information on

all I/O cards previously configured under PCM Plus, as well as information about

the PCMCIA client drivers.

.Card List

The card list displays all PCMCIA cards currently enabled for use under PCM

Plus together with specific configuration information. Not all cards need to be

listed in this database in order to be used by PCM Plus. Normally, LAN,

Fax/Modem, and other cards that use multiple system resources should be added

to the configuration list. To add a card to the configuration list, select Add Card

to List from the Configure option as shown in the previous section.

To review the card list, follow these steps.

1.At the main menu, press Alt-I or use the mouse to pulldown this menu.

2.Selecting Card List produces.

3.Select the required card type.

4.Select Next or Previous to review other card configurations in this list.

5.After you finish reviewing the list, press <Esc> or select OK to return to the

main menu.

.Client Information

The Client Program Information lists the PCM client drivers currently loaded and

provides specific information on each client, including:

�Client type		�Client name

�Client vendor		�CS Level

�Client Release (and the release date)

To review the Client Information, do the following:

1.At the PCMWin main menu, press Alt-I or click on the Information option

with the mouse. PCMWin opens the pulldown menu.

2.Selecting Client Info produces.

3.Click on Previous or Next to review other clients stored in the Client

Information window.

4.After you finish reviewing the Client Information, press <Esc> or select OK

to return to the main menu.

* Options

Options allows the program to:

�Confirm when a PCMCIA card has been inserted or removed from the computer.

�Run PCMWin minimized.

�Automatically launch a particular application when a specific card is inserted.

To activate Options:

1.From the main menu, press Alt-O to activate this pulldown message.

2.Select one of these options as required.

.Message on Card Events

Message on Card Events is a toggle feature. If enabled, a check mark (*) appears

to the left of the option. Click on the option to either activate or deactivate it.

When this option is enabled, PCM Plus displays a message each time a card is

inserted or extracted from a PCMCIA socket, or when a card is successfully

configured.

.Run Minimized

Run Minimized is also a toggle feature. If enabled, a check mark (*) appears to the

left of the option. Click on the option to either activate or deactivate it. Select it

again and the option is disabled. When Run Minimized is enabled, PCMWin

comes up as an icon whenever it is selected.

.Associations

Associations enables the specification of an associated program whenever a card is

inserted. For example, most modem cards are used with a communication program

such as the Windows Terminal program. By using the Associations option, the

communication program starts whenever the modem card is inserted.

*Note	Associations will work only when PCMWin is active (minimized or otherwise).

To use this option:

1.Press Alt-O or select Options from the PCMWin main menu.

2.Select Associations to activate the menu.

The card number, manufacturer name, and the model for this card are

displayed in the first three lines. This information is necessary for locating the

program needed to associate with a specific card.

3.To enable the start-up of the associated program, click in the Enabled check

box. An X appears to indicate that the association is enabled.

4.Use the <Tab> key or the mouse to move to the File field. Enter the exact

pathname and executable file that will start the associated program.

If the path or name of the program is not known, use Select File to browse

through the files until the needed file is located. (Most executable files have

an .EXE extension.)

5.Use the Previous or Next buttons to edit or create other card-program

associations.

6.When finished, select OK.

Chapter 4. Advanced Configuration

Considerations

PCM Plus consists of more than just setup and configuration programs, although

these two programs will be used most frequently. PCM Plus also includes an

assortment of utilities, drivers, and adaptations to complement any PCMCIA

environment. This chapter addresses the following areas which you may encounter

while using PCM Plus:

�Using PCM Plus within a multiple configuration/boot environment.

�Using PCM Plus' standalone utilities.

* Multiple Boot Support

PCM Plus supports DOS 6.0's multiple boot option, which enables the selection

of a specific configuration when the system is booted. For more information on

setting up the CONFIG.SYS file for multiple boot support, refer to the DOS 6.0

manual.

When started, the PCM Plus installation program checks the operating system

version.  If PCM Plus detects DOS 6.0, it scans the CONFIG.SYS file for the

[menu] section.  If  the install program cannot find the multiple configuration

option in CONFIG.SYS, it installs the PCM Plus software just as it would for any

single-boot version of DOS.

However, if the multiple configuration option is detected, PCMSETUP checks the

CONFIG variable in the environment to determine the current configuration name.

The program installs PCM Plus under the configuration name that PCMSETUP

determined to be current.

*Note  If at bootup you bypassed a configuration option at the Startup

Menu by pressing F5, the PCM Plus installation and setup program

cannot be run. An error message appears and the program ends.

Reboot the system and select a valid configuration. To check all of

the current configuration settings on the system, type SET at the

DOS prompt. The configuration name appears as

CONFIG=configname, where configname is the name selected at

boot time for a specific configuration.

PCM Plus installs and sets up its files for the current configuration only. Any

other configuration environment is not modified.  For example, if the computer

contains two boot configuration options, test1 and test2, and PCM Plus was

installed for test1 only, the CONFIG.SYS file might look like the following:

[menu]

menuitem=test1

menuitem=test2

[test1]

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\PCM3\CNFIGNAM.EXE /test1

DEVICE=C:\PCM3\PCMSS.EXE

DEVICE=C:\PCM3\PCMCSFUL.EXE

DEVICE=C:\PCM3\PCMRMAN.SYS

DEVICE=C:\PCM3\PCMSCD.EXE

DEVICE=C:\PCM3\PCMATA.SYS

DEVICE=C:\PCM3\PCMMTD.EXE

DEVICE=C:\PCM3\DBLFLASH.EXE

DEVICE=C:\PCM3\MS-FLASH.SYS

[test2]

DEVICE=C:\DOS\HIMEM.SYS

DEVICE=C:\DOS\EMM386.EXE NOEMS X=C800-C8FF X=CC00-CFFF X=C900-CCFF

[common]

DEVICEHIGH=C:\DOS\SETVER.EXE

DOS=HIGH,UMB

FILES=30

STACKS=9,256

Figure 3-9. Sample Multiple Boot CONFIG.SYS File after Installation of PCM Plus

Notice that PCM Plus has only changed the [test1] configuration files, while

[test2] remains unchanged. If EMM386.EXE appears under the [common]

configuration block, then PCMSETUP will modify EMM386.EXE. To install the

software for a different configuration, boot the system to that configuration and

run PCMSETUP. The PCM Plus initialization file (PCM.INI) will contain a

section for each configuration environment installed with PCM Plus software.

* PCM Plus Advanced Utilities

The installation program loads several additional programs to the PCM Plus

directory. The table below lists these files and their uses. Each program is

explained in detail later in this chapter.

Format :        File

Function

PCMFDISK

Edits partition table for ATA PC cards.

PCMRMAN

1.Manages and displays PCM Plus system resources.

2.Changes runtime values for allocation of system

resources.

PCMWARN

Written to the WIN.INI during installation, PCMWARN

runs whenever Windows is loaded and issues a warning if

Windows is running in Standard Mode. (PCM Plus

cannot be run in Windows Standard Mode.)

* PCMFDISK

PCMFDISK, like the DOS FDISK utility, sets the partition table for ATA rotating

disks and solid-state mass-storage PCMCIA cards.

*Warning  PCMFDISK will destroy all data on the device if new partitions are

created. DO NOT use PCMFDISK on SRAM PC Cards formatted

for use with PCMATA (the Phoenix ATA-IDE device driver)

because PCMATA treats SRAM cards more like floppy disk drives

than hard disk drives.

PCMSETUP permits the specification of the maximum number of partitions to

allow on ATA cards.  By default, all ATA cards will have the minimum of one

partition. Use the PCMFDISK utility to optimize the settings from PCMSETUP.

After inserting the ATA PC Card, run PCMFDISK to set (or reset) the following:

�Number of partitions

�Number of drive heads

�The number of sectors per track.

PCMFDISK calculates the resulting drive size. Use PCMFDISK also to adjust

partition settings for enhancing the performance of the disk. Consult the

documentation accompanying the ATA PC Card for the optimum configuration.

To run PCMFDISK, type the following at the DOS prompt:

PCMFDISK <Enter>

The initial display comes into view as follows:

Save   Modify   Help

����������������������� Partition and Geometry ���������������������Ŀ

�                                                                    �

�                          Socket Number: 0                          �

�                                                                    �

�  Partition  System   Size     Usage   Head   Cyl   Sec   Sectors   �

�  ���������������������������������������������������������������   �

�      1       FAT16    10mb    100%      1      0    1     20448    �

�      2                                                             �

�      3                                                             �

�      4                                                             �

�                                                                    �

�                              Geometry                              �

�                                                                    �

�           Heads: 2   Cylinders: 320   Sectors/Track: 32            �

�                                                                    �

�             Total Disk Size: 10 Mbytes (20480 Sectors)             �

�                                                                    �

�                                                                    �

����������������������������������������������������������������������

The Partition and Geometry screen presents information on the current

partition(s) existing on an ATA card. This screen provides information on each

partition, including:  the socket number where the card is located; the system type;

the partition size; the amount of the partition currently being used; and the number

of heads, cylinders, and sectors used by the partition. Below the partition

information, the PCMFDISK utility provides the geometry for the total ATA disk.

To access the pull-down menu options (Save, Modify, Help), press <Alt>  and the

highlighted letter simultaneously.

To modify the Partition or Geometry of the installed ATA card, do one of  the

following:

1.Create a new partition. First, delete existing partitions with Delete Partition,

then create new partitions with Create Partitions.

2.Add a new partition to existing partitions. Use Create Partitions to define a

new space. This new partition is limited to the size of the disk minus the

existing partition size. If the Usage field on the main screen displays 100%,

the disk does not have enough free space to add a new partition.

3.Change the default geometry for the ATA disk to optimize access speed and

overall performance. Use the Geometry option.

.Delete Partition

1.Press Alt-M to open this pulldown menu:

�������������������Ŀ

� Create Partition  �

� Delete Partition  �

� Geometry          �

���������������������

2.Press D or use the down arrow key to select Delete Partition and press

<Enter>. PCMFDISK displays the following message:

������� Delete Partition ������Ŀ

�                               �

� Partition 1 will be deleted.  �

�                               �

�    Press ENTER to confirm     �

�     or press ESC to abort     �

���������������������������������

If more than one partition exists on the card, PCMFDISK deletes the last

partition first. For example, if two partitions exist on the disk, Partition 2 is

deleted before Partition 1.

3.Press <Enter> to confirm deletion of the partition, or press <ESC> to abort

deletion and return to the main screen.

*Note	 Changes made to the partition table with the Modify option are not

committed to the disk until the Write Partition option under Save is

selected.

.Create Partition

Use this option to create a partition entry in the first empty partition slot.  A

maximum of four partition entries are allowed per ATA card. The number of

partitions which PCMATA can access depends on how PCMATA is set up.

PCMFDISK displays the number of partitions set by PCMSETUP when

Environment from under the Help menu is selected. Selecting the Quick mode

from PCMSETUP limits the number of partitions which PCMATA can access to

only one. Consequently, no matter how many partitions are created with

PCMFDISK, only the first partition may be accessed.

However, if access to more than the one partition is required, return to

PCMSETUP and run the Advanced mode version. The Advanced mode in

PCMSETUP allows stipulating the number of partitions on any ATA card in a

particular socket. This stipulation in the advanced mode of PCMSETUP sets the

maximum number of partitions that can be accessed.

1.Press Alt-M to open the following pulldown menu:

�������������������Ŀ

� Create Partition  �

� Delete Partition  �

� Geometry          �

���������������������

2.Press <C> or <Enter>.  PCMFDISK displays the following window:

3.Press <Enter> to accept the partition size shown in the highlighted field (this

value reflects the total size of the Mbytes remaining on the disk), or type in a

new size and press <Enter>. The Partition table reappears with the new

partition table values.

4.To create another partition, repeat steps 1 through 3.

*Note  Changes made to the partition table with the Modify option are not

committed to the disk until you select the Write Partition option

under Save.

.Geometry

Usually, the disk geometry does not require any changes. The default geometry has

the fastest access speed. However, some ATA cards allow changes to their

geometry.

*Warning  After you change the geometry, the partition table may become

invalid and you may need to reset it. Also, some ATA cards will fail

to write and become unstable, in which case you must remove and

reinsert the card to reset the state.

To change the Geometry, follow these steps.

1.Press Alt-M to open this pulldown menu:

�������������������Ŀ

� Create Partition  �

� Delete Partition  �

� Geometry          �

���������������������

2.Press G or use the down arrow key to highlight Geometry, and press <Enter>.

PCMFDISK displays this window:

����������� Modify Geometry ����������Ŀ

�                                      �

�       For advanced users only        �

�         Press F1 for detail          �

�                                      �

�          Heads: 2                    �

�  Sectors/Track: 32                   �

����������������������������������������

3.Type in a new value for either Heads, Sector/Tracks, or both, as required. Use

the arrow key to highlight the desired field. Press <Enter> after typing in the

new value(s). PCMFDISK displays this subwindow:

�Write Geometry�

�   No         �

�   Yes        �

��������������ͼ

4.Select Yes to change the disk geometry. Select No to ignore the changes and

return to the main menu. If Yes is selected, the heads and sector per track

values will reflect the new values.

.Save

No modifications to the ATA disk's partition tables become actual until Write

Partition is selected. This saves all the changes and deletes/creates a new partition

table for the disk.  To use this option:

������������������Ŀ

� Write Partition  �

� Exit             �

��������������������

2.Press <Enter>. PCMFDISK displays this warning:

Save   Modify   Help

����������������������� Partition and Geometry ���������������������Ŀ

�                                                                    �

�                          Socket Number: 0                          �

�                                                                    �

�  Partition  System   Size     Usage   Head   Cyl   Sec   Sectors   �

�  �������������������������������������������������������Ŀ������   �

�      1  � Warning: Writing the partition table may cause �  448    �

�      2  �  the loss of all data currently on the card.   �         �

�      3  �       Please format drive(s) after exit.       �         �

�      4  �                                                �         �

�         �     Do you wish to write partition table?      �         �

�         ������������������������������������������������ĳ         �

�         �                       No                       �         �

�         �                       Yes                      �         �

�         ��������������������������������������������������         �

�                                                                    �

�             Total Disk Size: 10 Mbytes (20480 Sectors)             �

�                                                                    �

����������������������������������������������������������������������

3.Select Yes to confirm the write. The partition information is written/changed.

4.Exit to DOS and format the drive with the DOS FORMAT command. At the DOS prompt type:

FORMAT d: /U

Where d is the drive letter.

The partition number and drive letter mapping is under sequential and

ascending order.

.Exit

To exit from PCMFDISK and return to the DOS prompt:

1.Press Alt-S.

2.Use the down arrow key to highlight Exit and press <Enter>, or press <X>.

.Help

PCMFDISK provides three areas of help under the Help menu, including:

�Contents - A short overview of PCMFDISK and what it can change.

�About PCMFDISK - The utility's version numbers.

�Environment - Information about this disk's specific environment. This

includes the number of partitions set by PCMSETUP.

* PCMRMAN.EXE

The PCM Plus includes two resource management programs (PCMRMAN.EXE

and PCMRMAN.SYS) that perform two distinct functions. PCMRMAN.SYS is

memory resident device driver that automatically loads from CONFIG.SYS and

maintains an internal database of resources. PCMRMAN.EXE executes from the

DOS prompt. You can run PCMRMAN.EXE in two ways:

�As a command line utility, it can change the current run-time values for the

allocation of system resources. These changes remain in effect until the user

reboots the system.

Generally, a user should first use the command line version before using the

stand-alone version. By doing so, a user can see how the system responds to

the modified resource allocations before making those modifications

permanent.

�As a stand-alone utility, it manages and displays the resources allocated by

PCM Plus, such as memory, I/O addresses and interrupts. Changes made with

this version of PCMRMAN.EXE are written to the .INI only after the user

reboots the system.

*Warning  The distribution of system resources is a highly technical

procedure. Use PCMRMAN.EXE with extreme caution. When used

improperly, it can adversely affect how your computer operates

with PCMCIA cards.

Command Line PCMRMAN.EXE

When appropriate, you can run a command line version of PCMRMAN.EXE

Changes made through this version take effect immediately but only last for the

duration of the current session. Use the command line version of PCMRMAN to:

*Assign or exclude memory ranges

*Assign or exclude I/O addresses

*Assign or exclude interrupts.

.When to Use

Some practical uses of this version of PCMRMAN include:

*With the map feature, ensure that the resources required to operate the next

PCMCIA card are available before you insert the card.

*Determine if a resource conflict with PCM Plus is causing erratic behavior in

the system, such as "lock-ups" or performance degradation.

*Before making permanent changes to PCM Plus, determine if a card fails to

configure because of unavailable resources or an incompatibility with PCM

Plus.

.Executing

By executing PCMRMAN.EXE with the switches described below, you can

override the availability of resources. All changes made with these switches remain

in effect until you reboot the computer. When you reboot the system, the resources

revert to their previous allocations. To permanently reassign resources, you must

enter these changes to the allocations through the stand-alone version of

PCMRMAN.

Resource availability will determine a PCMCIA card's successful configuration.

Excluding a resource from use by PCM Plus may result in the PCMCIA card not

being able to operate.

To invoke the PCMRMAN command line version, type the following at the DOS

prompt:

PCMRMAN parameters

where parameters are arguments shown in the table below. Upper portions of a

range are optional and ranges are inclusive.

*Note  If you do not include at least one parameter on the command line,

then PCMRMAN will assume the Stand-alone mode.

Format :        Argument

Possible Values

/MEM=xx-yy

Permits PCM Plus to use memory range xx00:0000 to

yy00:0FFF in hexadecimal. Argument range is A0h to

FFh.

/XMEM=xx-yy

Excludes PCM Plus from using memory range xx00:0000

to yy00:0FFF.

/IO=xxx-yyy

Permits PCM Plus to use the I/O range specified.

Argument ranges must be between 100h and 3ffh.

/XIO=xxx-yyy

Excludes PCM Plus from using the I/O range specified.

/IRQ=x-y

Permits PCM Plus to use IRQ in the range x to y.

Argument range is between 0 and 15.

/XIRQ=x-y

Excludes PCM Plus from using IRQ in the range x to y.

/MAP

Displays a map of card services and system resources.

/?

Displays the help screen.

For example, this command line:

PCMRMAN /MEM=C0-C8 /XMEM=D0-D8 /IO=2F0-2FF /XIO=300-3FF /XIRQ=6

assigns a memory range of C000:0000 through C800:0FFF while excluding ranges

D000:0000 through D800:0FFF. Similarly, the command line includes I/O range

2F0 through 2FF while excluding range 300 through 3FF, and explicitly excludes

IRQ level 6 from PCMRMAN's scope.

Map of System Resources

When you execute the command line version with /MAP, PCMRMAN displays a

summary of resource availability, as shown below, for the current PCM Plus

session.

Copyright(c) 1993,1994 Phoenix Technologies Ltd.

All Rights Reserved.

PCM Plus Resource Allocation Map

Upper Memory ( =Available to PCM+, I=In use by PCM+, U=Unavailable):

0 1 2 3 4 5 6 7  8 9 A B C D E F

A  U U U U U U U U  U U U U U U U U

B  U U U U U U U U  U U U U U U U U

C  U U U U U U U U  I         U U U

D  I I I I I I      U U U U U U U U

E  U U U U U U U U  U U U U U U U U

F  U U U U U U U U  U U U U U U U U

IO Port ranges:

AVAILABLE: 100-1EF,1F8-377,37C-3BF,3E0-3EF

IN USE by PCM+:

UNAVAILABLE: 1F0-1F7,378-37B,3C0-3DF,3F0-3FF

IRQ Allocations:

AVAILABLE: 03-04,07,09

IN USE by PCM+: 05

UNAVAILABLE: 00-02,06,08,0A-0F

Figure 4-1. Resource Allocation Map

Upper Memory

The Upper Memory Area (the 384K space between 640K and 1MB) is graphically

represented as a map of the memory addresses from A0000h to FFFFFh (see

Figure 4-1 above). Each block represents 4K of memory. Therefore, the first block

designates how the memory at A000:0000 to A0FF:0000 is being used. Following

are descriptions of all the types of information contained within the Resource

Allocation Map display.

=Available to PCM+

PCM Plus will allocate this 4K block of memory to a PCMCIA Card. Memory can

only be allocated in multiples of 4K blocks. If a card's configuration requests

unavailable memory, use the PCM or PCMWIN utility to change the configuration

so that it requests the memory that this map shows as Available. (See Chapter 3 in

this manual for details about the PCM and PCMWin utilities.)

I=In use by PCM+

A PCMCIA card is currently using this memory. If you try to reallocate memory

that another card is already using, PCM Plus will issue an IN USE error and will

deny your request. When you execute PCMRMAN /MAP, PCM Plus dynamically

determines the In Use designation. After you remove the card that is using the

memory in question, PCMRMAN /MAP will display that memory as Available.

*Note  There will always be at least one memory block in use by PCM

Plus for internal purposes, regardless of whether or not a PCMCIA

card is in the system.

U=Unavailable

Generally, the Upper Memory Area is used by the system BIOS, video cards,

networks, hard drives, MS-DOS and some memory managers (such as Quarterdeck

QEMM and Qualitas 386MAX). The memory that these devices and programs use

is not available to PCM Plus. If you try to assign this Unavailable memory to a

PCMCIA card, a resource conflict will result and the card will not configure

successfully.

I/O Port Ranges

The computer's CPU uses the I/O ports to communicate with such system

components as the keyboard, serial ports, mouse, printer, etc. Each port has a start

and end address associated with an I/O device. The software that operates these

devices will communicate with them through the respective port address. Examples

of PCMCIA cards that use an I/O range are ATAs, Fax/Modems, and Network

Cards.

Available

When an I/O range is marked as Available, PCM Plus will grant a request for that

range of I/O addresses. Successful configuration results in the card to which PCM

Plus assigns the requested I/O range.

*Note  PCM Plus grants a request only if the entire I/O range is available.

Unavailable

PCMCIA cards cannot use I/O addresses that are taken by other devices, such as a

mouse or keyboard. PCM Plus will deny any request for an unavailable I/O

address. PCMCIA cards that request unavailable I/O addresses will not configure

successfully.

In Use by PCM+

PCM Plus dynamically determines the In Use designation, and bases its

determination on the type of PCMCIA card that is present in the computer. After

you remove the card that is using the I/O address, PCMRMAN /MAP changes the

designation form In Use to Available.

IRQ Allocations

Hardware devices (hard disk, system timer, keyboard, et al) must each be serviced

by a unique hardware interrupt (IRQs or Interrupt ReQuest lines). Examples of

PCMCIA cards that require an IRQ are Network cards and Fax/Modem cards.

Available

You can assign an available IRQ to a PCMCIA card. PCM Plus will grant

requests for these available IRQs and thereby enable a card to configure

successfully.

Unavailable

The IRQs that are taken by these devices cannot be used by PCMCIA cards and

are therefore unavailable to PCM+. PCMCIA cards that request an IRQ that is

unavailable will be denied. This will prevent the card from configuring.

In Use by PCM+

PCM Plus dynamically determines the In Use designation, and bases its

determination on the type of PCMCIA card that is present in the computer. After

you remove the card that is using the IRQ, PCMRMAN /MAP changes the

designation of the corresponding IRQ from In Use to Available.

If you remove all PCMCIA cards from the system and PCMRMAN still reports an

IRQ as IN USE, then PCM Plus is using that IRQ for internal purposes. PCM Plus

does not permit you to override a resource designated as IN USE.

*Caution  Do not attempt to make Unavailable resources Available. Making

them available may cause resource conflicts that result in erratic

behavior such as computer "lock-ups" or performance degradation.

Stand Alone PCMRMAN.EXE

To run the stand-alone version of PCMRMAN.EXE:

1.Remove any PCMCIA cards from the computer.

2.At the DOS prompt, type:

PCMRMAN

PCM Plus displays a title screen. Press any key.

PCMRMAN displays a warning screen. Read this carefully before continuing.

Press any key. PCMRMAN then opens its main screen:

File   Display   Modify   Help

������������������������ Upper Memory Usage ����������������������Ŀ

�       Memory Range       System Status         User Override     ^

�       ������������������������������������������������������     �

�         A000-A0FF         Available             None             �

�         A100-A1FF         Available             None             �

�         A200-A2FF         Available             None             �

�         A300-A3FF         Available             None             �

�         A400-A4FF         Available             None             �

�         A500-A5FF         Available             None             �

�         A600-A6FF         Available             None             �

�         A700-A7FF         Available             None             �

�         A800-A8FF         Available             None             �

�         A900-A9FF         Available             None             �

�         AA00-AAFF         Available             None             �

�         AB00-ABFF         Available             None             �

�         AC00-ACFF         Available             None             �

�         AD00-ADFF         Available             None             �

�         AE00-AEFF         Available             None             �

�         AF00-AFFF         Available             None             _

��������������������������������������������������������������������

This read-only screen displays the upper memory ranges used by PCM Plus.

Use the PgUp/PgDn keys to review all of the memory ranges available to

PCM Plus. The System Status column indicates whether the computer itself is

currently using a range (in which case Unavailable is displayed) or whether

that range is Available for use by PCM Plus.

The User Override column indicates whether the user has modified the

corresponding range. The User Override column also displays the availability

of a resource. Resource availability is indicated by the User Override. If User

Override designates a resource as Included, then that resource will be

available to PCM Plus. If User Override designates a resource as Excluded,

then that resource will not be available to PCM Plus. If the value is none, then

the availability of the resource is determined by System Status.

Given the values in the System Status and User Override columns in the

PCMRMAN main screen, the table below indicates whether PCM Plus can

use the corresponding resource the next time the computer boots.

System Status

User Override

Can PCM+ Use?

Available or

Unavailable

Included

Yes

Available or

Unavailable

Excluded

No

Available

None

Yes

Unavailable

None

No

Display

The PCMRMAN main screen displays three types of resources:

�Memory ranges

.I/O ranges

�IRQ number

PCMRMAN defaults to displaying the Memory Ranges. The Display option

determines the type of resource displayed. To change the display, do the following

from the PCMRMAN main screen:

1.Press Alt-D to open this pulldown menu:

���������������Ŀ

�Memory Ranges  �

�I/O Ranges     �

�IRQ Number     �

�����������������

2.Use the up/down arrow keys to highlight the desired resource and press

<Enter>; or press the highlighted letter of the desired option, where

M = Memory Ranges, I = I/O Ranges, and Q = IRQ Number.

Selecting I/O Ranges produces the following:

File   Display   Modify   Help

������������������������� I/O Usage ������������������������Ŀ

�      I/O Range      System Status         User Override    �

�      ��������������������������������������������������    �

�      100h-1EFh        Available             None           �

�      1F0h-1F7h        Unavailable           None           �

�      1F8h-377h        Available             None           �

�      378h-37Bh        Unavailable           None           �

�      37Ch-3BFh        Available             None           �

�      3C0h-3DEh        Unavailable           None           �

�      3DFh-3EFh        Available             None           �

�      3F0h-3F6h        Unavailable           None           �

�      3F7h-3F7h        Available             None           �

�      3F8h-3FFh        Unavailable           None           �

�                                                            �

�                                                            �

�                                                            �

�                                                            �

�                                                            �

�                                                            �

��������������������������������������������������������������

This screen shows all the I/O ranges available to PCM Plus and their current

status.

Selecting IRQ Number produces the following:

File   Display   Modify   Help

��������������������������� IRQ Usage �������������������������Ŀ

�     IRQ Number       System Status         User Override      �

�     ����������������������������������������������������      �

�        IRQ 00           Unavailable           None            �

�        IRQ 01           Unavailable           None            �

�        IRQ 02           Unavailable           None            �

�        IRQ 03           Available             None            �

�        IRQ 04           Available             None            �

�        IRQ 05           Available             None            �

�        IRQ 06           Unavailable           None            �

�        IRQ 07           Available             None            �

�        IRQ 08           Available             None            �

�        IRQ 09           Unavailable           None            �

�        IRQ 10           Available             None            �

�        IRQ 11           Available             None            �

�        IRQ 12           Unavailable           None            �

�        IRQ 13           Unavailable           None            �

�        IRQ 14           Unavailable           None            �

�        IRQ 15           Unavailable           None            �

�����������������������������������������������������������������

This screen shows all the IRQ levels available to PCM Plus and their current

status.

.Modify

The Modify option provides a means to change resources (Memory range, I/O

range, and IRQ number) that are available or unavailable to PCM Plus.

To modify PCM Plus resources:

1.Press Alt-M from the PCMRMAN main screen to open this pulldown menu:

���������������Ŀ

�Memory Ranges  �

�I/O Ranges     �

�IRQ Number     �

�����������������

2.Select a resource to modify. Use the up/down arrow keys to highlight the

desired resource; or press the highlighted letter of the desired option, where

M = Memory Ranges, I = I/O Ranges, and Q = IRQ Number.

The screen that PCMRMAN displays, and the procedure for making changes,

will each vary depending on the option selected.

.Memory Ranges

1.Select Memory Ranges from the Modify submenu to display the following

screen:

File   Display   Modify   Help

������������������������ Upper Memory Usage ����������������������Ŀ

�       Memory Range       System Status         User Override     ^

�       ������������������������������������������������������     �

�         A000-A0FF         Available             None             �

�         A100-A1FF         Available             None             �

�         A200-A2FF         Available             None             �

�         A300-A3FF         Available             None             �

�         A400-A4FF         Available             None             �

�         A500-A5FF         Available             None             �

�         A600-A6FF         Available             None             �

�         A700-A7FF         Available             None             �

�         A800-A8FF         Available             None             �

�         A900-A9FF         Available             None             �

�         AA00-AAFF         Available             None             �

�         AB00-ABFF         Available             None             �

�         AC00-ACFF         Available             None             �

�         AD00-ADFF         Available             None             �

�         AE00-AEFF         Available             None             �

�         AF00-AFFF         Available             None             _

��������������������������������������������������������������������

PCMRMAN automatically highlights the first line in the User Override

column.

Each memory range block is listed in sizes of 4K, and the display reflects the

beginning and ending hexadecimal address. If both a memory manager and

PCM Plus have been installed, the available ranges or levels were

automatically determined during the installation process.

2.Pressing the spacebar produces a new value in the highlighted field. Press the

spacebar until a desired value appears. There are three values:

Value

Definition

None

Allow the availability of this resource to be determined by

whether or not the computer itself is using it.

Excluded

This range has been specifically excluded from use by

Included

PCM Plus has access to this range.

3.Use the up or down arrow key to move to the previous or next field. Use the

PgUp or PgDn key to move to the previous or next page.

4.When all of the desired User Override fields have been modified, press

<Enter> to accept these changes. The display screen reflects the changes.

.I/O Ranges

1.Select I/O Ranges from the Modify submenu to display this screen:

File   Display   Modify   Help

������������������������� I/O Usage ������������������������Ŀ

�      I/O Range      System Status         User Override    �

�      ��������������������������������������������������    �

�      100h-1EFh        Available             None           �

�      1F0h-1F7h        Unavailable           None           �

�      1F8h������������ Edit I/O Range ����������Ŀ          �

�      378h�                                      �          �

�      37Ch�  Please enter the I/O range that you �          �

�      3C0h�  would like to modify:               �          �

�      3DFh�                                      �          �

�      3F0h�  Start Range: 100                    �          �

�      3F7h�    End Range:                        �          �

�      3F8h�                                      �          �

�          �  User Override: None                 �          �

�          ����������������������������������������          �

�                                                            �

��������������������������������������������������������������

If you have both a memory manager and PCM Plus installed on your

computer, the PCM Plus installation utility automatically determined the

available ranges or levels.

2.Select the type of modification desired, then type in the new beginning and

ending ranges. When the User Override field is highlighted, use the spacebar

to change the value. Use the up/down arrow or <Tab> keys to move between

these fields.

3.When you are finished modifying the values, press <Enter> to confirm the

choices. The display screen will reflect the modifications.

.IRQ Number

1.Select IRQ Number from the Modify submenu to display this screen:

File   Display   Modify   Help

��������������������������� IRQ Usage �������������������������Ŀ

�     IRQ Number       System Status         User Override      �

�     ����������������������������������������������������      �

�        IRQ 00           Unavailable           None            �

�        IRQ 01           Unavailable           None            �

�        IRQ 02           Unavailable           None            �

�        IRQ 03           Available             None            �

�        IRQ 04           Available             None            �

�        IRQ 05           Available             None            �

�        IRQ 06           Unavailable           None            �

�        IRQ 07           Available             None            �

�        IRQ 08           Available             None            �

�        IRQ 09           Unavailable           None            �

�        IRQ 10           Available             None            �

�        IRQ 11           Available             None            �

�        IRQ 12           Unavailable           None            �

�        IRQ 13           Unavailable           None            �

�        IRQ 14           Unavailable           None            �

�        IRQ 15           Unavailable           None            �

�����������������������������������������������������������������

The first line in the User Override column is highlighted.

If you have both a memory manager and PCM Plus installed on your

computer, the PCM Plus installation utility automatically determined the

available ranges or levels.

2.Pressing the spacebar produces a new value in the highlighted field. Press the

spacebar until the desired value appears.

3.Use the up or down arrow key to move to the previous or next field. Use the

PgUp or PgDn key to move to the previous or next page.

4.When all of the desired User Override fields have been modified, press

<Enter> to accept these changes. The display screen reflects the changes.

Saving the Configuration

.To save changes made to the Memory, I/O or IRQ values, follow this procedure:

1.Press Alt-F from the PCMRMAN main screen. A pulldown menu appears:

�������������������Ŀ

�Save Configuration �

�Exit               �

���������������������

2.Press C or <Enter>. PCMRMAN displays this message:

���������������������������������Ŀ

� Save Configuration To INI File? �

���������������������������������ĳ

�               No                �

�               Yes               �

�����������������������������������

3.Select No to ignore the changes and return to the main PCMRMAN menu.

Select Yes to save the changes. The new values take effect when you reboot

the computer.

.Exit

To exit from the stand-alone version of PCMRMAN:

1.From the PCMRMAN main screen, press Alt-F or pull down the File menu

with the mouse. Click on Exit or press X. If you modified one of the

resources, PCMRMAN displays this message:

���������������������������������Ŀ

� Save Modification Before Exit ? �

���������������������������������ĳ

�               No                �

�               Yes               �

�����������������������������������

2.Select No to ignore the changes and return to DOS.

Select Yes to save the configuration modifications to the PCM.INI file and

exit to DOS.

.Help

Select Help to review basic PCMRMAN procedures, or use the context-sensitive

help key, F1.

PCMWARN.EXE

PCMWin cannot be invoked in Standard Mode. When invoked, PCMWARN

issues a warning if Windows is run in Standard Mode. PCMWARN should be

included in the WIN.INI file. Even though it can be activated as a runtime program

through Windows, PCMWARN generally runs every time Windows is loaded.

Usually, PCMWin writes the required line to the WIN.INI file. However, if the

line is not there, insert it in the WIN.INI file using this procedure:

1.Use a text editor to open the WIN.INI file which is under the /WINDOWS

directory.

2.Find the [windows] section in WIN.INI. (This should be at the top of the

file.) Enter the following in the [windows] section:

run=path\PCMWARN.EXE

where path is the directory path where PCMWARN.EXE is located. (Usually

this is in the default PCM Plus directory, PCM320.)

3.Save the changes to WIN.INI and exit Windows. PCMWARN will be loaded

when Windows is brought up again.

Appendix A. Deinstallation

Deinstallation is required when other PCMCIA support software has previously

been installed on the computer. If other PCMCIA software is already installed on

your computer, PCM Plus issues this message during installation:

PhoenixCARD Manager Installation

���������������������������� Error! ���������������������������Ŀ

�                                                               �

�  It has been determined that your system has another version  �

�  of PCMCIA utilities installed.  You cannot continue          �

�  installation until the other version of PCMCIA utilities     �

�  have been removed.  Would you like PCM+ to de-install        �

�  these utilities at this time ?                               �

�                                                               �

�                            �������Ŀ                          �

�                            �  Yes  �                          �

�                            �  No   �                          �

�                            ���������                          �

�                                                               �

�����������������������������������������������������������������

*Note  PCM Plus cannot be installed while other PCMCIA software is still

loaded in the system. The PCM Plus De-Install utility will

automatically remove previously installed PCMCIA software

(specifically, Card Services and Socket Services).

To deinstall the old software, highlight Yes and press <Enter>.  The Deinstall

utility displays the following:

PhoenixCARD Manager De-Installation

����������������������������������������������������������������Ŀ

�                                                                �

�                PhoenixCARD Manager Version 3.20                �

�                                                                �

�                                                                �

�                                                                �

�                 PCMCIA De-Installation Utility                 �

�                         Revision 2.00                          �

�                                                                �

�       Copyright(c) 1993, 1994, Phoenix Technologies Ltd.       �

�                       All Rights Reserved.                     �

�                                                                �

�                                                                �

�                    Press any key to continue                   �

�                                                                �

������������������������������������������������������������������

After you press <Enter>, Deinstall displays the following:

PhoenixCARD Manager De-Installation

��������������������������� WARNING! �������������������������Ŀ

�                                                              ^

� THERE ARE CONDITIONS UNDER WHICH THIS PROGRAM MAY FAIL TO    �

� UPDATE YOUR SYSTEM FILES PROPERLY.  PLEASE READ YOUR MANUAL  �

� CAREFULLY BEFORE PROCEEDING.                                 �

�                                                              �

� CREATING BOOTABLE DISKETTE.  It may be advisable to create   �

� a bootable diskette. Make sure that you have a bootable      �

� diskette available, so that, in the unlikely event that      �

� problems occur, due to conflict with unsupported utilities,  �

� you can re-boot your system.                                 �

�                                                              �

� GENERAL.  This program allows you to disable PCMCIA          �

� utilities, already on your system, so that the latest        �

� version of PCM+ can be installed.  Utilities are disabled by �

� removing, from your system files, all directives which were  �

� added there by the PCM+ INSTALL.EXE utility.  These are      �

� typically in CONFIG.SYS, AUTOEXEC.BAT, SYSTEM.INI, WIN.INI   �

� or PROGMAN.INI files.  Program may not succeed if these      �

� files have been edited by user, or by other utilities, or if _

����������������������������������������������������������������

Read the above message carefully. When you have finished reviewing the message,

press <Enter> to continue.

Deinstall then gives you the option of either commenting out or removing the lines

associated with the previously installed software and drivers.

PhoenixCARD Manager De-Installation

����������������� Remove or Comment Out ����������������Ŀ

�                                                        �

� To disable PCMCIA utilities, changes must be made      �

� to your CONFIG.SYS file, and possibly other files.     �

� The lines which need to be made inactive, can either   �

� be removed from the file or they can be commented out. �

�                                                        �

� Would you like to remove such lines from the file,     �

� or would you like to place a comment in front of       �

� such lines?                                            �

�                                                        �

�                 ��������������������Ŀ                 �

�                 � Comment-out lines  �                 �

�                 �    Remove lines    �                 �

�                 ����������������������                 �

�                                                        �

����������������������������������������������������������

If your system is running DOS 6.0 with the multiple boot option, Deinstall

displays the following:

PhoenixCARD Manager De-Installation

������������� De-Install from All Configurations ������������Ŀ

�                                                             �

� It has been determined that your system is capable          �

� of multiple boot configurations.  You have a choice         �

� of disabling drivers in current or all boot configurations. �

� (If you are using Windows, and previous version of PCM      �

� included Windows drivers, then you need to disable          �

� drivers in ALL configurations.)                             �

�                                                             �

� Would you like to disable drivers in all configurations?    �

�                                                             �

�            �����������������������������������Ŀ            �

�            �   Disable in All Configurations   �            �

�            � Disable in Current Configuration  �            �

�            �������������������������������������            �

�                                                             �

���������������������������������������������������������������

As the previous message indicates, you should disable drivers in all

configurations. Next, Deinstall displays the following:

PhoenixCARD Manager De-Installation

���������������������� Windows Directory ��������������������Ŀ

�                                                             �

� Some PCMCIA utilities modify Windows' system files.  Do you �

� have Microsoft Windows installed on your system?            �

�                                                             �

�                        ������������Ŀ                       �

�                        �    Yes     �                       �

�                        �    No      �                       �

�                        ��������������                       �

�                                                             �

���������������������������������������������������������������

If you are running Windows on your system, highlight Yes and press <Enter>.

Deinstall then requests the complete path to Windows:

PhoenixCARD Manager De-Installation

����������������������� Windows Directory���������������������Ŀ

�                                                              �

� Some PCMCIA utilities modify Windows' system files.  Please  �

� enter the directory where Microsoft Windows is installed:    �

�                                                              �

�                                                              �

�   C:\WINDOWS                                                 �

�                                                              �

�                                                              �

�                                                              �

�                                                              �

�                                                              �

�                                                              �

�                                                              �

�                                                              �

����������������������������������������������������������������

Next, Deinstall gives you the option of viewing the changes which the program

will make to the system files:

�������������������������� VIEW CHANGES ������������������������Ŀ

�                                                                �

� In order to de-install PCMCIA utilities several of your system �

� files have been modified.                                      �

�                                                                �

� Would you like to view the modifications made to these files?  �

� (This will allow you to accept or reject the changes).         �

�                                                                �

�                         ������������Ŀ                         �

�                         �    Yes     �                         �

�                         �    No      �                         �

�                         ��������������                         �

�                                                                �

������������������������������������������������������������������

If you select Yes to view the changes, Deinstall walks you through all the system

files, line-by-line.

First, it displays your CONFIG.SYS file, similar to the following:

PhoenixCARD Manager De-Installation

��������������������������� CONFIG.SYS ��������������������������Ŀ

� KEEP    [menu]                                                  ^

� KEEP    menuitem=swedish                                        �

� KEEP    menuitem=spanish                                        �

� KEEP    menuitem=german                                         �

� KEEP    menuitem=italian                                        �

� KEEP    menuitem=french                                         �

� KEEP    [common]                                                �

� KEEP    device=c:\dos\power.exe                                 �

� KEEP    device=c:\windows\himem.sys                             �

� KEEP    dos=high,umb                                            �

� KEEP    device=c:\dos\setver.exe                                �

� KEEP    BUFFERS=45                                              �

� KEEP    FILES=40                                                �

� KEEP    STACKS=9,256                                            �

� KEEP    shell =  c:\command.com c:\ /p /e:800                   �

� KEEP    [swedish]                                               �

� KEEP    device=c:\dos\emm386.exe noems                          �

� KEEP    [spanish]                                               �

� COMMENT device=c:\dos\emm386.exe noems X=C800-C8FF X=CC00-CFFF  �

� ADD     DEVICE=C:\DOS\EMM386.EXE NOEMS                          �

� KEEP    REM de PCM+ V3.0  X=C800-C8FF X=CC00-CFFF               �

� KEEP                                                            �

� COMMENT DEVICE=C:\SPANISH\CNFIGNAM.EXE /SPANISH                 �

�������������������������������������������������������������������

In the above example, which includes Multiple Boot options, KEEP indicates an

unchanged line, COMMENT indicates a REMarked-out line, and ADD indicates a

line which PCM Plus will install into the file. Use your mouse to click on the

scroll bar on the right of the window to scroll through each line of the file.

Next, Deinstall displays your SYSTEM.INI, which may be similar to the

following:

PhoenixCARD Manager De-Installation

������������������������������ SYSTEM.INI �����������������������������Ŀ

� KEEP    [boot]                                                        ^

� KEEP    shell=progman.exe                                             �

� KEEP    mouse.drv=mouse.drv                                           �

� KEEP    comm.drv=comm.drv                                             �

� KEEP    keyboard.drv=keyboard.drv                                     �

� KEEP    system.drv=system.drv                                         �

� KEEP    386grabber=vga.3gr                                            �

� KEEP    device=*vfd                                                   �

� KEEP    device=*parity                                                �

� KEEP    device=*biosxlat                                              �

� COMMENT ; by PCM+  by PCM+  device=*vcd                               �

� COMMENT COM4BASE=2E8                                                  �

� KEEP    PCMCIAComm4=TRUE                                              �

� KEEP    pcmciatimer=TRUE                                              �

� COMMENT EMMExclude=C800-DFFF                                          �

� KEEP    device=*vmcpd                                                 �

� KEEP    device=*combuff                                               �

� KEEP    device=*cdpscsi                                               �

� KEEP    local=CON                                                     �

� KEEP    FileSysChange=off                                             �

� KEEP    PagingFile=C:\WINDOWS\WIN386.SWP                              �

� KEEP    MaxPagingFileSize=12288                                       �

� KEEP    pcmcianotify=true                                             �

� KEEP    pcmciacom4=true                                               �

� COMMENT device=C:\FRENCH\PCMVCD.386                                   �

� ADD     device=*vcd                                                   �

� COMMENT EmmExclude=C800-C8FF CC00-CFFF                                �

�������������������������������������������������������������������������

Scroll through the file line-by-line, and when your are finished reviewing the

changes, press <Enter>.

Next, Deinstall walks you through your WIN.INI file, which may be similar to the

following:

PhoenixCARD Manager De-Installation

���������������������������������� WIN.INI ���������������������������������Ŀ

� KEEP    [windows]                                                          ^

� KEEP    spooler=yes                                                        �

� KEEP    load=                                                              �

� COMMENT run=C:\FRENCH\PCMWARN.EXE                                          �

� ADD     RUN=                                                               �

� KEEP    Beep=yes                                                           �

� KEEP    NullPort=None                                                      �

� KEEP    device=                                                            �

� KEEP    BorderWidth=3                                                      �

� KEEP    CursorBlinkRate=530                                                �

� KEEP    DoubleClickSpeed=452                                               �

� KEEP    Programs=com exe bat pif                                           �

� KEEP    Documents=                                                         �

� KEEP    DeviceNotSelectedTimeout=15                                        �

� KEEP    TransmissionRetryTimeout=45                                        �

� KEEP    KeyboardDelay=2                                                    �

� KEEP    KeyboardSpeed=31                                                   �

� KEEP    ScreenSaveActive=0                                                 �

� KEEP    ScreenSaveTimeOut=120                                              _

�۱���������������������������������������������������������������������������

After you have scrolled through the file and accepted the changes by pressing

<Enter>, Deinstall then takes you through the changes in your PROGRAM.INI

file which may look like:

PhoenixCARD Manager De-Installation

������������������������� PROGMAN.INI ������������������������Ŀ

� KEEP    [Settings]                                           �

� KEEP    Window=68 48 580 384 3                               �

� KEEP    display.drv=vga.drv                                  �

� COMMENT Order= 5 3 4 2 1                                     �

� ADD     ORDER= 3 4 2 1                                       �

� KEEP    SaveSettings=1                                       �

� KEEP    AutoArrange=1                                        �

� KEEP                                                         �

� KEEP    [Groups]                                             �

� KEEP    Group1=C:\WINDOWS\MAIN.GRP                           �

� KEEP    Group2=C:\WINDOWS\ACCESSOR.GRP                       �

� KEEP    Group3=C:\WINDOWS\GAMES.GRP                          �

� KEEP    Group4=C:\WINDOWS\STARTUP.GRP                        �

� COMMENT Group5=C:\ITALIAN\SAVE\PCMPLUS.GRP                   �

����������������������������������������������������������������

After you have finished scrolling through the changes and you want to accept the

changes press <Enter>.

Deinstall then displays the following window to summarize the system files that it

has modified:

PhoenixCARD Manager De-Installation

������������������������� CHANGES MADE �����������������������Ŀ

�                                                              �

� CONFIG.SYS   : Was MODIFIED                                  �

� CONFIG.103   : Contains the ORIGINAL data (backup)           �

�                                                              �

� AUTOEXEC.BAT : Was not used                                  �

�                                                              �

�                                                              �

� SYSTEM.INI   : Was MODIFIED                                  �

� SYSTEM.103   : Contains the ORIGINAL data (backup)           �

�                                                              �

� WIN.INI      : Was MODIFIED                                  �

� WIN.103      : Contains the ORIGINAL data (backup)           �

�                                                              �

� PROGMAN.INI  : Was MODIFIED                                  �

� PROGMAN.103  : Contains the ORIGINAL data (backup)           �

����������������������������������������������������������������

Deinstall saves your old files by adding a .10n extension, where n is the number

of previously backed up files plus one. The first time that Deinstall saves an old

file, it adds the extension .101, and the next time it saves the file with an

extension of.102, and so on. Consequently, you will have access to all of your

previous system files, regardless of how many time you use Deinstall.

*Note	In the example used in this section of the manual, the

AUTOEXEC.BAT file was not modified. However, if changes are

needed in your AUTOEXEC.BAT, Deinstall will treat it as it would

the other system files it modifies.

After you press <Enter> or <Esc> while in the CHANGES MADE screen,

Deinstall displays this final screen:

PhoenixCARD Manager De-Installation

������������������������ RE-BOOT SYSTEM �����������������������Ŀ

�                                                               �

� All of the system files have been properly updated.  However, �

� changes will not take effect, and PCMCIA drivers will not be  �

� disabled until your system is re-booted.                      �

� Be sure to remove any diskettes before rebooting.             �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�                                                               �

�     PLEASE RE-BOOT YOUR SYSTEM AFTER RETURNING TO DOS         �

�                                                               �

�����������������������������������������������������������������

Modifications will not take effect until you exit to DOS (by pressing <Enter> or

<Esc>) and reboot your system.

After you reboot your system, run the install program to complete the installation

of PCM Plus.

* Manual Deinstallation

This section is provided if you should decide to manually deinstall older versions

of PCMCIA software, in which case you may need to modify all of the following

files:

�CONFIG.SYS

.AUTOEXEC.BAT

�SYSTEM.INI

�WIN.INI

�PROGRAM.INI

1.There are two ways to edit the above system files:

Run EDIT from DOS by typing in the following command for each of the four

files:

EDIT filename <Enter>

where filename is one of the files to be edited. You would have to issue the

edit command once for each file, or a total of five times.

Or, a simpler method is to invoke Windows. From the File menu within

Program Manager, select the Run option and type this command in the

Command Line box:

SYSEDIT

SYSEDIT opens all relevant files simultaneously.

2.Scan CONFIG.SYS for old PCMCIA drivers.

3.Review the following syntax examples for editing the CONFIG.SYS,

AUTOEXEC.BAT, WIN.INI, SYSTEM.INI and PROGRAM.INI files.

(a)Remove old driver lines from CONFIG.SYS and AUTOEXEC.BAT by

writing the rem command at the beginning of the line, like this:

rem DEVICE=C:\PCMPLUS\PCMSS.EXE

(b)Remove old lines from the WIN.INI and SYSTEM.INI file by placing a

semicolon (;) at the beginning of the line in this manner:

;PCMCIACom4=TRUE

* Removing Old PCM Plus Files

All previous versions of PCM Plus reside in the \PCMPLUS directory. PCM Plus

uses the directory \PCM3 as its new default directory.

Examples of Editing CONFIG.SYS

Example 1

Before

DEVICE=C:\PCMPLUS\PCMSS.EXE

DEVICE=C:\PCMPLUS\PCMCSFUL.EXE

DEVICE=C:\PCMPLUS\PCMRMAN.EXE

DEVICE=C:\PCMPLUS\PCMMTD.EXE

DEVICE=C:\PCMPLUS\PCMSCD.EXE

DEVICE=C:\PCMPLUS\PCMATA.EXE

DEVICE=C:\PCMPLUS\PCMMTD.EXE

After

rem DEVICE=C:\PCMPLUS\PCMSS.EXE

rem DEVICE=C:\PCMPLUS\PCMCSFUL.EXE

rem DEVICE=C:\PCMPLUS\PCMRMAN.EXE

rem DEVICE=C:\PCMPLUS\PCMMTD.EXE

rem DEVICE=C:\PCMPLUS\PCMSCD.EXE

rem DEVICE=C:\PCMPLUS\PCMATA.EXE

rem DEVICE=C:\PCMPLUS\PCMMTD.EXE

Example 2

Before

DEVICE=C:\PCMPLUS\PCMSS.EXE

DEVICE=C:\PCMPLUS\PCMCCU.EXE

After

rem DEVICE=C:\PCMPLUS\PCMSS.EXE

rem DEVICE=C:\PCMPLUS\PCMCCU.EXE

Example of Editing SYSTEM.INI

Before

[386Enh]

;*** PATCHED BY PCM+ EZINSTALL ***

device=pcmvcd.386

PCMCIACom4=TRUE

COM4BASE=2E8

EMMExclude=C800-C8FF

ENMExclude=D000-EFFF

;*** PCM+ STOP ***

;*** PATCHED BY PCM+ EZINSTALL ***

[PCMCIA]

COM04=TRUE

;*** PCM+ STOP ***

;*** PATCHED BY PCM+ EZINSTALL ***

run=PCMWARN.EXE

;*** PCM+ STOP ***

After

[386Enh]

;*** PATCHED BY PCM+ EZINSTALL ***

;device=pcmvcd.386

;PCMCIACom4=TRUE

;COM4BASE=2E8

;EMMExclude=C800-C8FF

;EMMExclude=D000-EFFF

;*** PCM+ STOP ***

;*** PATCHED BY PCM+ EZINSTALL ***

[PCMCIA]

;COM04=TRUE

;*** PCM+ STOP ***

;*** PATCHED BY PCM+ EZINSTALL ***

;run=PCMWARN.EXE

;*** PCM+ STOP ***

* Removing Card Soft Files

After editing the CONFIG.SYS file, add the two lines to the SYSTEM.INI file as

shown on below.

Examples of Editing CONFIG.SYS

Example 1

Before

DEVICE=C:\CARDSOFT\SS365SL.EXE

DEVICE=C:\CARDSOFT\CS.EXE

DEVICE=C:\CARDSOFT\CSALLLOC.EXE

DEVICE=C:\CARDSOFT\ATADRV.EXE

DEVICE=C:\CARDSOFT\SRAMDRV.EXE

DEVICE=C:\CARDSOFT\CARDID.EXE

DEVICE=C:\PCMPLUS\PCMMTD.EXE

After

rem DEVICE=C:\CARDSOFT\SS365SL.EXE

rem DEVICE=C:\CARDSOFT\CS.EXE

rem DEVICE=C:\CARDSOFT\CSALLLOC.EXE

rem DEVICE=C:\CARDSOFT\ATADRV.EXE

rem DEVICE=C:\CARDSOFT\SRAMDRV.EXE

rem DEVICE=C:\CARDSOFT\CARDID.EXE

rem DEVICE=C:\PCMPLUS\PCMMTD.EXE

Example 2

Before

DEVICE=C:\CARDSOFT\SS365SL.EXE

DEVICE=C:\CARDSOFT\CS.EXE

DEVICE=C:\CARDSOFT\CSALLLOC.EXE

DEVICE=C:\CARDSOFT\ATADRV.EXE

DEVICE=C:\CARDSOFT\MEMDRV.EXE

DEVICE=C:\CARDSOFT\MS_FLASH.SYS

DEVICE=C:\CARDSOFT\CARDID.EXE

DEVICE=C:\CARDSOFT\CS_AP.EXE

After

rem DEVICE=C:\CARDSOFT\SS365SL.EXE

rem DEVICE=C:\CARDSOFT\CS.EXE

rem DEVICE=C:\CARDSOFT\CSALLLOC.EXE

rem DEVICE=C:\CARDSOFT\ATADRV.EXE

rem DEVICE=C:\CARDSOFT\MEMDRV.EXE

rem DEVICE=C:\CARDSOFT\MS_FLASH.SYS

rem DEVICE=C:\CARDSOFT\CARDID.EXE

rem DEVICE=C:\CARDSOFT\CS_AP.EXE

Example of Editing SYSTEM.INI

*Note  *.vcd indicates to add any file with the VCD extension

Before

[boot]

comm.drv=C:\CARDVIEW\SSCOM.DRV

drivers=mmsystem.dll C:\CARDVIEW\SSWINCS.DLL

[386Enh]

device=C:\CARDVIEW\SSVRDD.386

device=C:\CARDVIEW\SSVCD.386

COM3NONREMOVABLE=FALSE

COM4NONREMOVABLE=FALSE

After

[boot]

;comm.drv=C:\CARDVIEW\SSCOM.DRV

comm.drv=comm.drv            (add line)

;drivers=mmsystem.dll C:\CARDVIEW\SSWINCS.DLL

[386Enh]

device=*.vcd                          (add line)

;device=C:\CARDVIEW\SSVRDD.386

;device=C:\CARDVIEW\SSVCD.386

;COM3NONREMOVABLE=FALSE

;COM4NONREMOVABLE=FALSE

* Removing CardWare Files

To remove old CardWare files, both the CONFIG.SYS and the WIN.INI files

require the omission of certain lines.

Example of Editing CONFIG.SYS

Before

rem -- Following lines added by CardWare Setup

DEVICE=C:\CARDWARE\PCSS.EXE

DEVICE=C:\CARDWARE\PCCS.EXE

DEVICE=C:\CARDWARE\PCENABLE.EXE

DEVICE=C:\CARDWARE\PCDISK.EXE

rem -- Previous lines added by CardWare Setup

After

rem -- Following lines added by CardWare Setup

rem DEVICE=C:\CARDWARE\PCSS.EXE

rem DEVICE=C:\CARDWARE\PCCS.EXE

rem DEVICE=C:\CARDWARE\PCENABLE.EXE

rem DEVICE=C:\CARDWARE\PCDISK.EXE

rem -- Previous lines added by CardWare Setup

Example of Editing WIN.INI

Before

[CardWare]

INI=C:\CARDWARE\CARDWARE.INI

After

;[CardWare]

;INI=C:\CARDWARE\CARDWARE.INI

* Removing IBM PCMCIA Files

IBM PCMCIA files may reside in either the \PCMCIA or \DOS subdirectory.

Examples of Editing CONFIG.SYS

Example 1

Before

DEVICE=C:\PCMCIA\IBMDSS02.SYS

DEVICE=C:\PCMCIA\IBMDOSCS.SYS

DEVICE=C:\PCMCIA\DICRMU01.SYS

After

rem DEVICE=C:\PCMCIA\IBMDSS02.SYS

rem DEVICE=C:\PCMCIA\IBMDOSCS.SYS

rem DEVICE=C:\PCMCIA\DICRMU01.SYS

Example 2

Before

DEVICE=C:\DOS\IBMDSS02.SYS

DEVICE=C:\DOS\IBMDOSCS.SYS

DEVICE=C:\DOS\DICRMU01.SYS

After

rem DEVICE=C:\DOS\IBMDSS02.SYS

rem DEVICE=C:\DOS\IBMDOSCS.SYS

rem DEVICE=C:\DOS\DICRMU01.SYS

* Completing the Deinstallation

After removing previously installed software files as indicated in the preceding

examples, complete the deinstallation with this procedure:

1.Check AUTOEXEC.BAT. Delete older versions of PCM Plus from the

PATH= or SET PATH= statement. For example:

PATH=C:\ C:\DOS C:\PCMPLUS

becomes

PATH=C:\ C:\DOS

In the above example, the old PCM Plus directory, C:\PCMPLUS, was

deleted. The new installation program will then add the revised PCM Plus to

AUTOEXEC.BAT.

Similarly, remove all path designations for other versions of PCMCIA support

products previously installed on the system.

2.Save all revised files.

3.Exit to DOS and reboot. The newly revised files are loaded.

4.Reinstall PCM Plus using the directions in Chapter 2, Installation.

5.Normally during installation, PCM Plus automatically adds lines to the

resident memory manager (such as EMM386) which accommodate PCM Plus'

memory requirements. However, if PCM Plus displays any messages

indicating that it cannot find sufficient memory regions to load the program,

or if the memory manager is not EMM386, reconfigure the memory manager

before reinstalling PCM Plus. Follow this procedure:

(a)From DOS or Windows, open the CONFIG.SYS, SYSTEM.INI, and

PCM.INI files. Look for lines applying to the memory manager.

(b)Edit the file to exclude memory regions in reserved or Upper Memory

that PCM Plus might require, or

Rerun your memory manager and specify these reserved areas.

Each memory manager is different.  Check the memory manager's user

guide if the system is not running EMM386.

For more information on memory management and the use of PCM Plus

drivers, refer to the memory manager's user guide or Appendix B,

Memory Management in this manual.

(c)Rerun the PCM Plus installation program.

Appendix B. Drivers and Memory Management

In addition to utilities and programs, PCM Plus also includes drivers that provide

necessary services for PCMCIA cards. The PCM Plus installation program

automatically loads most of these drivers. However, these drivers may require some

fine-tuning for maximum performance.

The best method for optimizing driver performance is to increase the efficiency of

memory usage.  This appendix provides details on how to optimize the

performance of the drivers that PCM Plus installs on the computer.

*Caution  In the normal course of operation, the default settings for the drivers

should be adequate for most systems. Editing these settings requires

some knowledge and experience in memory optimization.

* PCM Plus Requirements

The table below provides information on drivers required by PCM Plus for each

card.

Format :        PCM PLUS Filename

PCMCIA Cards (ATA IDE, Flash Memory, LAN, Fax/Modem, SRAM Memory)

PCMSS.EXE

Socket Services

*

PCMCSFUL.EXE

Card Services

*

PCMCS.EXE

Card Services

*

PCMSCD.EXE

LAN or FAX/Modem

*

PCMATA.SYS

IDE Hard-Disk Emulation

*

PCMFFCS.EXE

PC-Flash support

*

PCMMTD.EXE

PC-Flash support

*

�All configurations require the installation of PCMSS.EXE and

PCMCSFUL.EXE (or PCMCS.EXE), which must be loaded after the

CNFIGNAM driver in the CONFIG.SYS file.

�PCM Plus supports multiple ATA cards.

�PCM Plus PC-Flash support requires the installation of the Microsoft Flash

File System (MSFLASH.SYS).

�SRAM Memory cards may be configured under the ATA driver along with

hard disks, or as PC Flash cards under the Flash Memory driver, only if the

required drivers are installed.

* Memory Management

All drivers are mapped to specific memory sites in the computer's RAM. Available

sites are shown below.

1FFFFFF - FFFFFF...

up to 64MB

Extended Memory

0E0000 -0FFFFF

128KB

System BIOS

0C0000 - 0DFFFF

128KB

I/O adapter ROM

Upper Memory

(UMB)

0A0000 -0BFFFF

128KB

Video RAM

000000 - 09FFFF

640KB

Conventional (base) memory

Conventional

Memory

Most of the first 640KB of RAM, or conventional memory, is available for

applications_depending on the DOS version and the device drivers that are

loaded. The remainder of the first megabyte (the upper 384KB), or upper memory,

is reserved for video and system BIOS, device drivers, etc. The area above the first

1MB, or extended memory, is accessible through memory managers which support

data storage of up to a theoretical limit of 4 gigabytes (a 486 system can physically

handle 64MB).

When you install PCM Plus, the install program automatically uses Novell's

DPMS.SYS to load the required PCMCIA drivers into extended memory. With

these drivers located in extended memory, your system will have significantly more

conventional memory available. The table below shows how much conventional

memory you can save with DPMS.SYS. Optionally, you can override this memory

saving option by using the /RM parameter with PCMATA.SYS, PCMCS.EXE,

PCMCSFUL.EXE, or PCMSCD.EXE. (See the section, PCM Plus Drivers, for a complete

description of PCM Plus drivers and corresponding parameters you can use.)

PCM Plus stores the CIS (Card Information Structure) data of the corresponding

PCMICA card in upper memory. The amount of UMB memory the CIS uses and

the default CIS window address are also shown in the table below.

Format :        Driver

Conventional Memory used

UMB Memory

used by CIS

Default CIS

Window Address

PCMSS.EXE

6K

0K

PCMCS.EXE

38k

4K

/ADDR=A000 - BFFF

PCMCSFUL.EXE

5K

51k

4K

/ADDR=D800

PCMSCD.EXE

2K

17K

0K

PCMATA.SYS

1K

11K

4K

/ADDR=D9

PCMFFCS.EXE

15k

0K

/BASE=DC

PCMMTD.EXE

14k

4 - 12K

(per socket)

Novell DPMS.SYS

2K

In addition, consider:

�The PCMATA driver window is required for PCMATA to support SRAM

cards. Specify the PCMATA driver window address in the ATA Socket menus

by running PCMSETUP in Advanced Mode. You may also specify the

PCMATA driver window address with the /ADDR option.

�Some PCMCIA Token Ring cards default to using memory address space

beginning at D000. Check the PCMCIA card documentation for the card's

upper memory requirements to assure that no memory collisions occur,

especially with PCMMTD.EXE.

* Minimizing Memory Usage

To reduce memory usage, follow these recommendations:

1.Load only those drivers that are absolutely required for the cards which are

going to be used. For example, if modem cards are used exclusively, load only

the PCMSS, PCMCS, and PCMSCD drivers.

2.Make all upper memory block addresses contiguous to avoid memory holes.

3.Use Microsoft's MSD program to maximize memory usage.

* PCM Plus Drivers

PCM Plus normally installs these drivers:

Format :        Driver

Purpose

CNFIGNAM.EXE

Specifies which PCM Plus configuration should be

accessed from the PCM.INI file.

PCMATA.SYS

Used with SRAM, rotating disk cards, or solid-state mass

storage cards, for IDE hard disk emulation.

PCMCSFUL.EXE or PCMCS.EXE

Supports Card Services. Works at the operating system

level to manage all PCMCIA cards. PCM Plus installs

PCMSFUL only if you plan to use Flash memory cards.

Otherwise, it installs PCMCS.

PCMFFCS.EXE

Acts as high-level bridge between MSFLASH.SYS  and

Card Services.

PCMMTD.EXE

Supports the Microsoft Flash File System (FFS) protocol.

Used with SRAM and Flash memory cards.

PCMRMAN.SYS

Initializes Card Services resource data base.

PCMSCD.EXE

Used with Fax/Modem and LAN cards.

PCMSS.EXE

Supports Socket Services. Works at the BIOS level as an

interface between the PCMCIA card socket and the Card

Services program.

The PCM Plus installation program automatically installs drivers for the specified

cards. The program also assigns parameters to the driver, indicating such things as

memory allocation, slot assignment, and address size.

Normally, these original assignments will not require any changes. However, to

optimize system performance or to add a new driver for a newly-acquired PCMCIA

card, one of these drivers may need to be added to the CONFIG.SYS file. This

section provides specific information on all the PCM Plus drivers currently

available.

*Note  Drivers provided by third party suppliers, including

MS-FLASH.SYS and DPMS.SYS, are not defined in this section.

Syntax used in this section is shown below.

COMMAND {required} /switch

where:

COMMAND	is the required word to type or invoke for running the

driver or program.

{required}	is a required option for invoking the command. (Not

all utilities require a command option.)

/switch	is an option which can be omitted. Normally a default

setting already exists.

* CNFIGNAM.EXE

The CNFIGNAM.EXE device driver specifies which PCM Plus configuration

parameters to use based on the selected boot configuration. The PCM Plus setup

utility adds CNFIGNAM.EXE to the CONFIG.SYS file.

*Note  If the system is not running DOS 6.0 in multiple boot mode, only

one configuration option exists.

PCM.INI defines system resource requirements for all boot configurations for

which PCM Plus has been installed. When a boot configuration is specified after

startup, CNFIGNAM identifies the selection as CONFIG.SYS is loaded and

removes the relevant configuration information from the PCM.INI file.

Syntax:

CNFIGNAM.EXE /config_name

Arguments:

/config_name	Identifies the PCM Plus configuration to use.  The

PCM.INI file may contain multiple setup

configurations for PCM Plus. Each of these

configurations is identified and separated by square

brackets: [ ].

Example:

CNFIGNAME.EXE /NEW

where /NEW is the name of the boot configuration selected at system startup.

Discussion:

Typing CNFIGNAM at the DOS prompt without an argument produces a report on

the current boot configuration name for PCM Plus.

* PCMATA.SYS

PCMATA.SYS, the Phoenix ATA-IDE device driver, acts as an I/O client to Card

Services. PCMATA.SYS enables the system to access ATA-configured PCMCIA

cards as IDE hard drive devices using an IDE partition table, and maps the first

socket (Socket 0) to the first available drive letter (usually D:) and the second

socket (Socket 1) to the next available drive letter (usually E:), and so on.

PCMATA.SYS loads from the CONFIG.SYS file and the pcmata= line appears in

the PCM.INI file.

Syntax:

PCMATA.SYS /Sn=x /SRAM /ADDR=nn /SD=mm /RM /IO=999

Arguments:

/Sn=x		Where n specifies the socket number and x designates

the number of partitions allocated for that socket. Each

socket requires its own argument. For example, /S1=2

indicates that socket 1 has 2 partitions. When no

socket is specified, the program configures all installed

sockets with one partition. Socket numbers start at

zero (0).

/SRAM	Enables PCMATA to access SRAM cards which

require 16K of upper memory to configure properly.

/ADDR=nn	Where nn specifies the base address of the 16K

memory-mapped window required to configure SRAM

cards. When no address is specified, Card Services

assigns an available address.

/SD=mm	Spin down the drive after mm minutes. When no time

is specified, Card Services assigns no spin down.

/IO=xxx	Where 0 < xxx * 999. Use alternate 16 bytes of I/O

memory for PCMCIA ATA drives. As an example, for

the Texas Instruments T1 4000M, /IO=170.

/RM		Forces real mode, and disables DOS Protected Modes

Services (DPMS). As a default, PCMATA.SYS uses

DPMS and loads most of its executable code into

extended memory to maximize available conventional

memory.

Example:

PCMATA.SYS /S0=1 /SD=1 /SRAM /ADDR=D0 /IO=179 /RM

Discussion:

PCMCIA Plus enables both the ATA driver (PCMATA.SYS) and the Flash File

driver (PCMMTD.EXE) to read SRAM cards. Adding the /SRAM option to

PCMATA and loading PCMMTD permits access to one SRAM card with two

drivers. As long as the cards have been formatted properly with DOS FORMAT or

contain a FAT-compatible disk structure, PCMATA can access the following

PCMCIA Cards:

�	PCMCIA ATA IDE fixed disks that have been properly configured. If

necessary, use PCMFDISK to assign partitions then format the disk.

�	PCMCIA 1.0/2.0 compliant SRAM PC card (Read/Write)

�	PCMCIA 1.0/2.0 compliant OTP, Mask ROM, EEPROM, or flash card (Read

Only).

If the system has more than one socket, all of the sockets can be used for ATA

disk cards as long as not more than one card lacks a CIS.

* Card Services

The PCM Plus Card Services driver (PCMCSFUL.EXE or PCMCS.EXE)

functions as an extension to the operating system by coordinating access to the

PCMCIA Cards and allocating system resources among client drivers. The Card

Services driver must be loaded from CONFIG.SYS directly after Socket Services

(PCMSS.EXE). Card Services has the following responsibilities:

�	Manage all available resources for PCMCIA cards. When you insert a

PCMCIA card into a socket, Card Services determines if it can provide the

resources required by that card's configuration.

�	Manage client drivers that are written for specific PC cards. These client

drivers are registered with Card Services during initialization (system boot-

up). Card Services then provides the registered client drivers with pertinent

information when PC card events occur (card insertion or extraction) as well

as status changes, such as battery low, card ready, and card locked.

The DEVICE=PCMCS(FUL).EXE line appears in the CONFIG.SYS file.

*Note	If you DO NOT plan to use Flash memory cards, then PCM Plus employs

PCMCS.EXE in place of PCMCSFUL.EXE as its Card Services driver.

Syntax:

PCMCS.EXE /ADDR=nn /CLIENTS=nn /MCA /NOBEEP /NOPM /PMOFF /POLL /REGIONS=n /RM /WAIT=n

/XIRQ=xxxx

Arguments:

/ADDR=nn	Sets internal memory window to address xx00:0000

(first available).

/CLIENTS=n	Allows n clients to register.

/FLASH 	Enables Flash File System Compatibility.

/IRQ=n	Enables IRQ n for status-change interrupts.

/MCA		Forces Micro channel compatibility mode.

/NOPM	Ignores all Power-Management events.

/NOBEEP	Turns off the tone emitted when cards are inserted or

removed.

/PMOFF	Disables sending Power Management events to

Clients.

/POLL	Uses polling instead of interrupts for card events.

/REGIONS=n	Uses up to n different memory regions at a time.

/RM		Forces real mode, and disables DOS Protected Modes

Services (DPMS). As a default, Card Services uses

DPMS and loads most of its executable code into

extended memory to maximize available conventional

memory.

/WAIT=n	Delays n 18.2 ms periods before configuring I/O card.

/XIRQ=xxxx	Bitmask of unavailable interrupts.

*Note	You cannot use the /IRQ and /POLL arguments simultaneously.

These arguments are mutually exclusive.

Discussion:

/ADDR=nn specifies the PC Card Configuration Address. This option defines the

starting segment address for Card Services to configure a PCMCIA Card. You

must locate the segment nn within the first 1MB address space, and you must

specify the start address of a 2-digit hexadecimal segment address. The granularity

of nn is dependent on the socket controller. However, the minimum address

granularity allowed is 4K. Leaving out the argument causes Card Services to

default to the first available address beginning at C000.

Address Range:			/ADDR= C0h-F0h.

/CLIENTS=n specifies the maximum number of supported client drivers that can

register with Card Services. Each client driver requires Card Services to allocate

60 bytes of memory.

Default Value, if not specified:		/CLIENTS=10

/FLASH enables Flash File System Compatibility. Use this option to make the

Phoenix Flash File system drivers (PCMMTD.EXE) more reliable. If you have

problems using the Flash File System, include this option.

/IRQ=n specifies the Card Services IRQ level. This option defines the IRQ

resource that Card Services uses for PCMCIA card events such as insertion and

removal. This IRQ is unrelated to the communication (COM:) ports.

Interrupt Range:					/IRQ=8-15

Default Value if not specified:		/IRQ=10

/MCA enables Micro Channel Mode. This option handles interrupts and card

events as if it were working on a Micro Channel Bus. Use this option if Card

Services cannot automatically detect MCA.

/NOPM causes the program to ignore Power Management. This option disables all

Card Services power management, and prevents Card Services from monitoring

power-management events. Use this option to prevent collisions with non-APM-

compliant power management schemes.

/PMOFF disables the Power Management Option provided with Card Services. If

you specify the /PMOFF option, then Card Services unilaterally powers-down

PCMCIA Cards on SUSPEND messages. On RESUME messages, Card Services

powers-up and reconfigures the PC cards (if there is an associated client driver) by

sending an artificial insertion message.

/POLL enables Poll Card Events. This option enables sense-driven, rather than

interrupt-driven, card events. This switch is useful for noisy hardware

environments, or when an IRQ is not available. This mode is automatically enabled

when an MCA is detected or when the /MCA option is used. When more than one

adapter is present, use /POLL to avoid multiple interrupts for monitoring card

events.

/RM forces real mode operation. As a default, PCM Plus loads PCMATA.SYS

into extended memory. Though not recommended, you can use the /RM option to

load PCMATA.SYS into conventional memory and disable DPMS. If you use the

/RM parameter, PCM Plus does not require Novell's DPMS.SYS.

/WAIT=n specifies the Card Settle Time. The Wait parameter, a value specified in

system timer ticks (18.2 per second), determines the required delay period from the

time a PC card is inserted to the time at which it becomes accessible. This option

is useful for PC Cards that have longer settle times.

Default value: 	/WAIT=1

Range:			/WAIT= 1-100.

/XIRQ=xxxx specifies system IRQs to exclude. On a bare IRQ system, as shown in

the table below, Card Services can use only IRQ10, IRQ11, IRQ12, or IRQ15 for

card events. Fully loaded systems, however, such as multimedia systems, often

require these same IRQ levels. In such cases, use /XIRQ to exclude these IRQs

from Card Service consideration and to avoid conflicts.

The variable xxxx decodes to these IRQs:

\XIRQ=0400

Excludes Card Services from using IRQ10

\XIRQ=0800

Excludes Card Services from using IRQ11

\XIRQ=8C00

Excludes Card Services from using IRQ10, IRQ11, and IRQ15, leaving

only IRQ12 available.

The table below shows the devices to which the IRQ levels are dedicated.

IRQ0

Timer Output

IRQ1

Keyboard

IRQ2

Route to Interrupt Controller 2, IRQ8 to 15

IRQ3

Serial Port COM2

IRQ4

Serial Port COM1

IRQ5

Parallel Printer Port 2

IRQ6

Floppy-disk controller

IRQ7

Parallel Printer Port 2

IRQ8

Real-Time Clock

IRQ9

Software redirect to IRQ2 (INT 0AH)

IRQ10

Reserved (default PC Card events)

IRQ11

Reserved

IRQ12

Reserved

IRQ13

80x87 Math Coprocessor

IRQ14

Hard-Disk controller

IRQ15

Some Hard-Drive/SCSI controllers

* PCMMTD.EXE

The PCMMTD.EXE driver enables PCM Plus to support industry-standard

SRAM cards, as well as Intel Series I and II Flash Cards (and compatible cards)

and Advanced Micro Devices (AMD) cards Series A, B, and C. PCMSETUP

automatically installs PCMMTD and PCMFFCS if Flash File System cards will

be employed.

PCMMTD.EXE does not use any arguments or parameters.

* PCMFFCS.EXE

PCMFFCS acts as a high-level bridge between Microsoft's Flash File System

driver (MSFLASH.SYS) and Card Services. Phoenix's Flash File support driver,

PCMMTD.EXE, mediates between Card Services and the actual flash card, and

performs low-level functions such as read, write, copy, and erase. This division of

high and low level tasks allows PCM Plus to read, and map to the same drive

letter, both ATA and Flash memory. Additionally, PCMFFCS allows PCM Plus to

map the first socket (Socket 0) to the first available drive letter (usually D:) and

the second socket (Socket 1) to the next available drive letter (usually E:), and so

on.

The PCMFFCS= argument appears in PCM.INI and has the following syntax.

PCMFFCS  /QUEUE=n  /Sn=x

Arguments:

/QUEUE=n	Specifies the number of concurrent erasures for cards

that support background erasure. Options are 1-9.

/Sn=x 		Where n specifies the socket number and x designates

the number of partitions allocated for that socket. Each

socket requires its own argument. For example, /S1=2

indicates that socket 1 has 2 partitions. When no

socket is specified, the program configures all installed

sockets with one partition. Sockets are numbered

starting at zero (0).

* PCMRMAN.SYS

Card Services cannot determine either the allocated system resources or to which

I/O addresses, IRQs, and UMBs those resources are assigned. PCMRMAN.SYS

loads after the Card Services driver, then scans I/O and RAM space (in the UMB

from A000-FFFF), as well as IRQ assignments. It then checks the PCM.INI file

under [INSTALL.Resources] to determine the manufacturer overrides. Once these

are established, PCMRMAN.SYS passes all of the information to Card Services

which then makes device allocations.

During installation, PCM Plus automatically copies PCMRMAN.SYS to the

appropriate directory and writes it into the CONFIG.SYS file.

Syntax:

PCMRMAN /NOSCAN

Arguments:

/NOSCAN	Disable the PCMRMAN.SYS scan which determines

system resources. Before using /NOSCAN, make

certain that every system resource is either included

with or excluded from PCM Plus in the

[INSTALL.Resources] section of the PCM.INI file.

Otherwise, do not include this parameter.

* PCMSCD.EXE

The Phoenix Super Client Driver, a Card Services client, increases the efficiency

of resource acquisition by requesting system resources (memory, I/O, IRQ) from

Card Services. When requested in this way, those resources appear to the system

as resident for use by software.

PCMSCD must reside in the same directory as the active PCM.INI. PCMSCD

reads PCM.INI to determine the boot options and the configuration information

used for enabling PCMCIA cards.

When PCMSCD is booted, selected card configurations are loaded from the

initialization file into a list maintained by PCMSCD. When a PC Card is inserted,

PCMSCD identifies the card and looks for a possible configuration in that list. If

it finds a configuration that matches the card, PCMSCD attempts the

configuration. If it cannot find a matching configuration, PCMSCD examines the

CIS configurations on the card. If the card is a generic modem/fax/serial card,

PCMSCD tries to enable the card as such. If a configuration is successful,

PCMSCD beeps once.

*Note	As a default, the program emits a high-toned beep when a card is

inserted or successfully configured. When a card is extracted,

PCMSCD emits a lower-pitched beep.

PCM Plus automatically loads PCMSCD.EXE into the CONFIG.SYS file and

writes the PCMSCD= argument to the PCM.INI files.

Syntax:

PCMSCD /NOBEEP /NOMS /NODB /NW /RM

Arguments:

/NOBEEP	Disable beeps when a LAN or FAX/Modem PC Card

is configured.

/NOMS	Do not save modem state information on power down.

/NODB	Disable internal card information database.

/NOMODEM	Disable the generic configuration for modems. System

no longer automatically recognizes FAX/modem PC

Cards.

/NW		Do not wait on error/warning message.

/RM		Forces real mode, and disables DOS Protected Modes

Services (DPMS). As a default, PCMSCD uses DPMS

and loads most of its executable code into extended

memory to maximize available conventional memory.

* Socket Services

During installation of PCM Plus, the program loads the Socket Services device

driver PCMSS.EXE into CONFIG.SYS. To modify the device driver options for

Socket Services, use PCMSETUP Advanced Mode or edit the PCMSS= argument

in PCM.INI.

Syntax:

PCMSS /RS=n /RW=n /APOFF /LOCK

Arguments:

RS=n		Reserves socket n. No argument indicates none.

RW=n		Reserves socket-window controller n. No argument

indicates none.

/APOFF	Disables hardware auto-power mode.

/LOCK	Enables socket locking features.

For more information about the functionality of Socket Services, refer to the

PCMCIA Socket Services 2.0 Interface Specification.

Example:

PCMSS /RW=3

Discussion:

/RS=n1 n2 specifies the number of reserved sockets. The Reserve Socket option

prevents Socket Services from colliding with other dedicated PCMCIA software

that directly access the socket hardware. Also, use this option to limit access to a

socket that may not be accessible in the system. /RS reserves a maximum of two

sockets.

/RW=n1 n2 reserves a socket controller window. Reserve either I/O or memory

windows to prevent conflicts with software that are not allocating resources from

Card Services.  n specifies the window number starting at zero on the first socket.

There is no distinction between I/O and memory windows since the number and

type of windows is controller-specific. /RW reserves a maximum of two windows.

/APOFF disables Auto-Power Mode on some socket controllers. Use this option

for maximum compatibility on MCA-bus machines or for controllers that have

problems with auto-power mode.

/LOCK enables Socket Locking. This switch only works if your PCMCIA

hardware has socket-locking capability.

Appendix C. Troubleshooting

PCM Plus provides a large number of help and error messages. The Help messages will

simplify the installation, setup, and configuration process. The Error messages will assist

users in understanding not only what is wrong but, more importantly, how to fix it. Most of

the messages in PCM Plus are self-explanatory. The following tables provide additional

explantions for some of the messages.

PCM Plus Installation

PCM Plus may display these messages during the installation program:

Format :        Message

What to do ?

APM and POWER.EXE are not loaded.

Copy APM over to the /DOS directory. POWER.EXE should already be in the /DOS

directory. Define the location of both files in the CONFIG.SYS file.

Could not allocate memory!

Not enough memory space found. Reallocate memory resources either by editing

CONFIG.SYS or by rerunning the memory manager. Refer to Appendix B, Memory Management.

Directory does not exist!

PCMSETUP could not find the specified directory. Create a new directory.

Error! It has been determined that your system has another version of PCMCIA utilities installed.

Remove all old versions of PCMCIA utilities from CONFIG.SYS and AUTOEXEC.BAT. See

Appendix A, Deinstallation.

PCM+ Drivers already installed!

PCM Plus is already installed. Do not reinstall it.

PCM+ Subsequent Installation!

PCM Plus files are already loaded. REMark out all references to previous versions of

PCM Plus in CONFIG.SYS and AUTOEXEC.BAT. See Appendix A, Deinstallation.

Warning! It has been determined that your system is capable of multiple boot configurations.

If the PCM Plus installation program discovers MS-DOS 6.x with multiple boot configurations,

it queries whether to install PCM Plus on this particular boot configuration or another. To

load PCM Plus on a different boot configuration, exit the install program, reboot the system,

and specify the desired configuration.

PCM Configuration

Format :        Message

What to do ?

ERROR: Card Services is not installed !!!

The PCMCSFUL (or PCMCS) file is not properly installed. Either rerun the Install program or

manually copy PCMCSFUL.EXE (or PCMCS.EXE) if you do not plan to use Flash memory cards) from

the distribution diskette to the current PCM Plus directory (usually \PCM3).

ERROR: PCM cannot run under MS-Windows.

Use PCMWIN.EXE instead. Install and use PCMWIN. See Chapter 2, Installation, and Chapter 3,

Configuration.

Requested configuration unavailable

One (or more) of the configuration changes to this card is not available. Make another

selection or leave the configuration at its default values.

Requested I/O unavailable

The I/O setting assigned to this card is not available, probably because it is being used

by another device. Either select another I/O setting, accept the default, or reallocate

the I/O of the device that currently controls it.

Requested IRQ unavailable

The IRQ level assigned to this card is not available, probably because it is being used

by another device. Either select another IRQ, accept the default, or reallocate the IRQ

of the device that currently controls it.

Requested Memory unavailable

The memory range setting assigned to this card is not available probably because it is

being used by another device. Either select another range setting, accept the default,

or reallocate the memory of the device that currently controls it .

Unavailable COM

The assigned COM value is already being used by another device. Either reassign this

card port to another COM or reassign the port that already has this value.

Unconfigured

The inserted card does not have a CIS. Use PCM to assign parameters to the card. If

the card is an I/O card, PCM will not be able to run it until the card is configured.

If the inserted card is a Flash Card, ignore this message. In most cases, PCM Plus

will still run the Flash Card.

PCM Windows Configuration

Format :        Message

What to do ?

PCMCSFUL.EXE (or PCMCS.EXE) not loaded.

The card services driver is not loaded. Either rerun the Install program or manually

copy PCMCSFUL.EXE (or PCMCS.EXE) from the distribution diskette to the appropriate

PCM Plus directory (usually \PCM3). If problems persist, check the path name

designated in the CONFIG.SYS file and change it if necessary.

PCMSCD.EXE not loaded.

The super client driver is not loaded. Either rerun PCM installation or manually copy

PCMSCD.EXE from the distribution diskette to the appropriate PCM Plus directory

(usually \PCM3).

Unable to locate PCM.INI.

The program cannot find PCM.INI. Rerun PCM installation.

Unable to allocate enough memory.

PCM Plus could not find sufficient memory. Run the memory manager to optimize memory

use (see Appendix B) or remove some of the TSRs from memory. If multiple applications

are currently running in Windows, close those applications and try again.

Requested configuration unavailable

One (or more) of the configuration changes to this card is not available. If possible,

make another selection or leave the configuration at its default values.

Requested I/O unavailable

The I/O setting assigned to this card is not available, probably because it is being

used by another device. Either select another I/O setting, accept the default, or

reallocate the I/O of the device that currently controls it.

Requested IRQ unavailable

The IRQ level assigned to this card is not available, probably because it is being

used by another device. Either select another IRQ, accept the default, or reallocate

the IRQ of the device that currently controls it.

Requested Memory unavailable

The memory range setting assigned to this card is not available probably because it

is being used by another device. Either select another range setting, accept the

default, or reallocate the memory of the device that currently controls it.

PCMWIN can only add I/O cards, i.e. Fax/Modem, LAN, SCSI, Sound Card.

It is not necessary to configure this card. PCMWIN

automatically configures media cards such as Flash

cards and ATA disk cards. Only I/O cards have to be

configured.

Additional Error Messages

Format :        Message

What to do ?

FLASHCMP.EXE is missing.

The computer could not find FLASHCMP.EXE in the directory specified by CONFIG.SYS.

Either edit the CONFIG.SYS file to reflect the correct path or copy FLASHCMP.EXE

from the distribution diskette to the appropriate directory.

MS-FLASH.SYS is missing.

The computer could not find MS-FLASH.SYS in the directory specified by CONFIG.SYS.

Either edit the CONFIG.SYS file to reflect the correct path or copy MS-FLASH.SYS

from the distribution diskette to the appropriate directory.

Socket Services not loaded

The file PCMSS.EXE is not loaded. Either rerun PCM Plus installation or copy

PCMSS.EXE from the distribution diskette to the appropriate directory. If problems

persist, check the path name designated in the CONFIG.SYS file and change it if

necessary.

Socket Services version must be 2.0 or greater

The copy of PCMSS.EXE loaded on the machine is not up-to-date. Copy PCMSS.EXE

from the distribution diskette to the directory where the old version of PCMSS

currently resides; Or, reinstall PCM Plus with an updated version.

Incorrect command-line parameter(s) detected

While loading one of the PCM Plus programs or drivers, an incorrect parameter

was detected. Check the CONFIG.SYS, PCM.INI, and WIN.INI files for incorrect

switch settings. Refer to Appendix B, Drivers and Memory Management.

ERROR: Socket number specified over logical sockets in the system!

More sockets were defined than the computer supports. Edit CONFIG.SYS and PCM.INI

to reflect correct number of ports, or use PCM to reconfigure the system. Refer

to the section, PCMATA.SYS, in Appendix B for more details.

ERROR: Syntax error in command line!

The program tried to load a driver that had the wrong syntax in its CONFIG.SYS or

PCM.INI line. Refer to driver syntax descriptions in Appendix B.

No ATA Card Detected

PCMFDISK was unable to detect an ATA card. Rerun PCM or PCMWIN to define the ATA card.

Fail to read partition table

PCMFDISK could not read the partition table on the memory card. Run PCMFDISK to

allocate partitions. See Chapter 4 for details.

At most 4 partitions per ATA card While running PCMFDISK, an attempt was made to

define more than four partitions on the memory card. Only four partitions are allowed.

See Chapter 4 for details.

No space to create a partition

The memory card already has all of its space allocated to partitions. To reassign space,

run PCMFDISK, delete existing partitions, and then reallocate the space to new partitions.

For details, refer to Chapter 4.

ERROR: PCMSCD requires 80386 processor or above.

PCMSCD and PCM Plus cannot be run on a computer possessing less than an 80386 processor.

ERROR: When you want to install the IBM token ring card and memory card simultaneously.

Please insert the token ring card first, then the memory card.