PHDISK 2.X -- Phoenix NoteBIOS 4.0(tm) Save-to-Disk Partition Utility

------------------------------------------------------------------------------

PHDISK.EXE is the Phoenix NoteBIOS 4.0(tm) Save-to-Disk Utility.

This utility prepares a hard disk for the suspend-to-disk (STD) feature,

including both alternatives of creating either a file or a partition

for the storage of the suspend/resume operation.

PHDISK.EXE should be run in the MS-DOS Mode. You must boot your system

in the MS-DOS Mode then run this program to reserve or remove the disk

space for the 0-volt suspend and resume function.

To properly run PHDISK.EXE in Windows 95, you must start the system in

the MS-DOS Mode as well. This program will not work from the MS-DOS

PROMPT in Windows 95.

After the execution of the 'DELETE' or 'CREATE' option, You need to

restart the system for the completion of the specified action.

The syntax of PHDISK is shown as follows.

Usage: PHDISK [option]

Options:

/CREATE [size] (/FILE or /PARTITION) -- Create STD file or partition.

/DELETE (/FILE or /PARTITION) -- Delete existing STD file or partition.

/INFO			      -- Information on STD disk area(s).

/REFORMAT /PARTITION	      -- Reformat existing STD partition.

Note: If PHDISK is invoked without any parameters, it  displays the

amount of disk storage required for the suspend to hard disk

feature and the current amount of storage available in the

hard disk.

OPTION				 DESCRIPTION

--------------------------   -----------------------------------------------

/CREATE [size] /FILE	       Create a suspend-to-disk file SAVE2DSK.BIN

of a specified size (in kilobytes). If the

size parameter is not specified, PHDISK will

allocate the minimum size required for the

suspend to disk feature.

/CREATE [size] /PARTITION     Create a suspend-to-disk partition of a

specified size (in kilobytes). If the size

parameter is not specified, PHDISK will

allocate the minimum size required for the

suspend to disk feature. This option will

mark out any bad blocks found during the

format operation.

The exact amount of disk storage required for

the suspend to disk may be calculated as follows:

Required storage = physical system memory size +

video memory size + 1K.

/REFORMAT /PARTITION	       Reformat an existing suspend-to-disk partition.

Use this option if a previous suspend-to-disk

terminated in a read or write error. The option

will also mark out any bad blocks found during

the format operation.

/DELETE /FILE		       Delete the suspend-to-disk file or partition

or		       and reclaim any space used by this feature.

/DELETE /PARTITION	       Use this option in conjunction with /CREATE to

change the size of the suspend-to-disk.

/INFO			       Display extended information about the suspend

to hard disk.