AudioDriveÒ for WindowsÔ 3.1x

Version 4.16.19

Drivers Release Notes

March 28, 1996

# Contents

Contents

About This Release

Release Information

Release Description	2

LIMITATIONS	3

# About This Release

ESS Technology is providing updated drivers for the AudioDrive® family of sound chip products in this Drivers Release. These drivers are complete updates to prior ESS releases for MicrosoftÒ WindowsÔ.

# Release Information

Product:	Device Drivers Version 4.16.19 for WindowsÔ 3.1x

and Windows for WorkgroupsÔ 3.11

Drivers:	ES488 (with OPL2), ES1488, ES688 (with OPL3), ES1688,

ES1688/ES968, ES1788, ES1888

Release Date:	March 28, 1996

# Release Description

## Version 4.16.19 Changes

The following changes have been incorporated:

- Warning and error messages from the device drivers are by default disabled. Those messages are now handled by the Windows Operating System and do not require localization.

- Improved the resolution step mapping between the hardware volume control and the software master volume control for the ES1788 and ES1888 solution.

## Version 4.16.12 Changes

The following changes have been incorporated:

- ESFM driver : eliminated extra note that shows up at the end of MIDI files playback.

- ES1888 driver : separated Mixer from Record Control; thus, allowing users to hear the output from the mixer while a recording is in progress.  This also allows users to adjust the mixer volumes while doing a recording via the record channel.

- ES1788 and ES1888 drivers : fixed external volume control.  Symptom was when the balance node of the master volume control is at the left most state, pushing any external hardware volume control button would cause the master volume to drop to 0.

## Version 4.16.11 Changes

The following changes have been incorporated:

- ES1888 driver : eliminated pops at the beginning and end of ES1888 wave file playback.

# Limitations

The ES1888 drivers no longer permit recording from FM.  In turn, Full-Duplex support is provided for applications such as Internet-Phone, Speaker-Phone, etc.