# S02-400 BIOS Release Note

Version 3E01 – Jan. 22, 2003 / Checksum=13BCh: 1. Bug Fixed: “If you switch the KVM to another server on the switch you will not be able to switch back to the G300.”

Version: 3D01 – Jan. 10, 2003 / Checksum=20A1h: - Bug Fixed: “Only three SCSI HDDs are indentified in BIOS Setup when connecting 4 HDDs onto LSI 1010 SCSI card.”

Version: 3C01 – Nov. 25, 2002 / Checksum=9E15h: - Change the version number from 3B08 to 3C01.

Version: 3B08 – Nov. 18, 2002 / Checksum=E4E2h: 1.Remove the barcode <F9> functionality. 2.Upgrade the microcode for Pentium 4 processors. 3.Bug Fixed: “System cannot boot with Hyper-Threading processor.”

Version: 3B07 – Oct. 17, 2002 / Checksum=3CF4h: - Fix PXE boot fail when insert LSI 2 channel SCSI card in some BOOT sequence. - Fix “New event log” does not disappear when user clear event log in BIOS setup or by PnP function call.

Version: 3B06 – Oct. 11, 2002 / Checksum=EBAAh: - Fix system hang if exit BIOS setup without saving change. - Prompt “New Event Log” found at POST whenever a new event is logged. Keep prompt this message until user read system event log in BIOS setup. - Update CPU frequency table and CPU temperature limitation for new P4 CPU, up to 3.2GHz.

Version: 3B05 - Patch system hang in PCI_SIG test. - Upgrade onboard LAN Intel 82540 IBA to version 1.1.09 - Allow displaying 24 character serial number in BIOS setup/product menu.

Version: 3B04 Fix product and baseboard information in BIOS setup\production information.

Version: 3B03 - Fix some SMBIOS field could not modify by SMBIOS utility. - Correct baseboard information in BIOS setup\product information.

Version: 3B02 Fix system hang in the beginning of FreeBSD installation.

Version: 3B01 - Fix system hang after exit setup with discard changes in 3A10. - Support secure mode booting function. When this function activated: - PS2/keyboard will be locked with user password. Mouse is disabled. - Power button is disabled. - Console redirection input is disabled. Enter correct user password will enable the input from console redirection. Press ESC-L in terminal to disable the input from remote terminal. - The local keyboard num lock, caps lock and scroll lock led flashing in sequence. - Floppy boot will wait for keyboard unlock. - The secure mode could be reentrant by press F12 or F11, depend on the user’s setting in BIOS setup\security. - Support “AC power recovery” event log. - Fill device information in PCI PERR event log. - Remove “Mark as Read” option in BIOS setup\Advanced\Event log Configuration. - Program Northwood P4 sample CPU to default ratio.

Version: 3A10 - Disable DMA in Linux Red Hat 7.3 - Remove DMA option for 3rd IDE devices in BIOS setup menu. - Update all CPU micro code - Record DIMM number into ECC event log. - Implement memory reduction, which will disable the faulty memory banks at next system boot up. For detailed, please refer 2.2 in Acer BIOS Specification for T13A, version 1.6. - A POST_memory_resize event is generated when the BIOS detected a faulty memory bank. - Port 80 will show 60h, 61h, 62h, 63h when the BIOS detected DIMM A,B,C,D faulty. - If all memory banks are disabled, the BIOS will enable one DIMM to boot up the system. The priority is DIMM A, B, C, D. - Disabled memory bank will have value 7Eh in offset 09h, 0Ah, SMBIOS type 6. - The disabled bank will show ‘Disabled’ in BIOS setup MAIN – system information – Total memory sub menu. - Adjust setup menu according to 2.5.2 in Acer BIOS Specification for T13A, version 1.6. - Fix Pentium 4 string displayed in processor type in BIOS setup menu. - Correct and modify SMBIOS information as following: - Remove type 18. - Correct offset 11h,12h as non-interleave in type 20. - Fix offset 05h, bank connection info in type 6. - Set offset 0Bh, type 6 to 04h. - Let max cache size equal to installed size in type 7. - Modify offset 17h, supported event log type descriptors in type 15.

Version: 3A09 - Fix POST CPU display string. - Fix F9 barcode write.

Version: 3A08 - Fix error of CDROM disappear in 3rd IDE channel. - Fix SMBIOS offset 0Bh, type 5, supported memory type to ECC, DIMM, SDRAM. - Fix auto-wake up problem. Remove “Eanble S4 RTC wakeup” setup item.

Version: 3A07 - Remove 3rd IDE mode programming in ACPI OS. The setting will be the same with DOS, which is PIO mode.

Version: 3A06 - Fix SMBIOS - UUID field in Type 1 could be write by DMI_2027.exe - Runtime update state in following type. The runtime information must be read by PnP calling method, not table convention method. - Chassis security state: offset 09h, boot-up state field in Type 3. - Fan status: offset 06h of Type 27. - Voltage status: offset 05h of Type 26. - Temperature status: offset 05h of Type 28 - Add SMBIOS 2.3.2 field in type 0, 2, 3 - Modify memory type to DDR in offset 12h, Type 17. - Modify CPU external clock to 100 in offset 12h, Type 4 - Configure 3rd IDE channel in PIO mode in legacy environment. Remove 3rd IDE UDMA configuration in ACPI environment. - Remove POST checkpoint display. - Remove 03E8 option item in setup COM port configuration. This port is conflicted with 3rd IDE.