# S02-533 BIOS Release Note

Version 3K03 - Fri 09-12-2003 / Checksum = A0A5h - Bug Fixed: “If you switch the KVM to another server on the switch you will not be able to switch back to the G301.”

Version 3K02 – Aug. 21, 2003 / Checksum=A8Ech: - Fixed event log appear random code. Note : When you flash to the new BIOS, please do the following procedure : 1.Please "Load Optimal defaults" or "F9". 2.Select "Exit Saving Change" or "F10" (Don't select other function)

Version 3K01 – Aug. 07, 2003 / Checksum=A54Ch: - Update BIOS version from 3J02 to 3K01 . Note : When you flash to the new BIOS, please do the following procedure : 1.Please "Load Optimal defaults" or "F9". 2.Select "Exit Saving Change" or "F10" (Don't select other function)

Version 3J03 – Aug. 07, 2003 / Checksum=D836 h: - Replace software IDE Raid firmware from Rev2.7.08041808A to Rev2.7.05291737I. Note : When you flash to the new BIOS, please do the following procedure : 1.Please "Load Optimal defaults" or "F9". 2.Select "Exit Saving Change" or "F10" (Don't select other function)

Version 3J02 – Aug. 06, 2003 / Checksum=ABF1 h: - Update software IDE Raid firmware from Rev2.7.05291737I to Rev2.7.08041808A. 2. Add front fan alarm option in Setup menu. Note : When you flash to the new BIOS, please do the following procedure : 1.Please "Load Optimal defaults" or "F9". 2.Select "Exit Saving Change" or "F10" (Don't select other function)

Version 3J01 – Jun. 23, 2003 / Checksum=CD65h: 1. Update the firmware for software IDE RAID. 2. Update the SMBIOS type 3. 3. Update the 3.06G CPU shutdown temperature to 80℃. 4. Update the POST chipset string from :”GCLE” to “GCSL”. Note : When you flash to the new BIOS, please do the following procedure : 1.Please "Load Optimal defaults" or "F9". 2.Select "Exit Saving Change" or "F10" (Don't select other function)

Version 3I04 – May 29, 2003 / Checksum=858Eh: 1. Correct CPU ratio string in BIOS setup menu.

Version 3I03 – May 23, 2003 / Checksum=CA75h: 1. Change the firmware for software IDE RAID to the old version. ( the same with 3H06)

Version 3I02 – May 16, 2003 / Checksum=252Fh: 1. Problem Solved : - Solve the ARMC card that has additional floppy drive issue in W2K when set the virtual floppy to the first boot priority. - Update the string “Vtt” to “A_Vtt” during POST. - Update the firmware for software IDE RAID for new version. - Remove the string “00000…..” in DMI table Manufacture area. - Solve the LSI MegaRAID card can’t boot issue.

Version 3I01 – May 5, 2003 / Checksum=10E1h: 1. Problem Solved : - Update the CPU ratio display method. - Add the DMI table Manufacture string “ACER”. - Update the string “Vtt” to “A_Vtt”. - Update the string “2.5V” to “Vcc 2.5V”, “3.3V” to “Vcc 3.3V”, “5V” to “Vcc 5V”, “12V” to “Vcc 12V”. - Update the microcode for D1 stepping. - Update the firmware for software IDE RAID.

Version 3H06 – Apr. 7, 2003 / Checksum=824Bh: 1. Problem Solved : - Update the string from “ssis” to “Chassis”. - Solve the HCT 11.1 ACPI error issue. 2. Known issue : - There were extra local site floppy devices appeared when setting AMI virtual floppy as first boot device.

Version 3H05 – Apr. 3, 2003 / Checksum=BE22h: 1. Problem Solved: - Update the string from “CPU #1 Temp” to “CPU Temp”. - Update the string from “System Fan 1” to “Front Fan”. - Update the string from “System Fan 2” to “Rear Fan”.

Version 3H04 – Apr. 1, 2003 / Checksum=4467h: 1. Problem Solved: - Install G2 card and reboot system, the system will hang. - The “system health monitoring hardware” page in the setup will crazy. - When install G2 card, and enter the Win2K, the floppy & CD-ROM will show - Update the error string from “Vdd IMB 1.5V” to “LAN 1.5V”. - Update the error string from “LAN 2.5V” to “VGA 2.5V”. - Update the error string from “1.8V” to “Vdd IMB 1.5V”.

Version 3H03 – Mar. 4, 2003 / Checksum=007Bh: 1. As requested by STN, when G2 card is installed, - HWM SMI# is to be disabled prior to booting OS, but it still works during POST; - SMBIOS Type 26/27/28 structures are to be hidden. 2. Problem Solved: “The system got blue hang when running ASM.”

Version 3H02 – Feb. 19, 2003 / Checksum=B218h: - Bug Fixed: “The system got blue hang when running ASM.” - Bug Fixed: “P4 3.06GHz processor was displayed as 3.1GHz by the BIOS.”

Version 3H01 – Feb. 10, 2003 / Checksum=D317h: - Defect ID 4497 Solved: “Unreported Memory and I/O port of HCT11.1a test failed under .NET Enterprise.” - G2 Card Multi-Master Issue Solved: - Issue SUSPEND/RESUME command upon and after accessing I2C bus. - Change the USB Legacy Support in Setup to ‘Auto’ defaultly. - Let PCI VGA card has higher priority than onboard VGA device. - Phase in the new micro code for P4 C1 stepping processor.

Version 3G01 – Jan. 22, 2003 / Checksum=0F86h: - Bug Fixed: “If you switch the KVM to another server on the switch you will not be able to switch back to the G301.”

Version: 3F01 – Jan. 10, 2003 / Checksum=C4B2h: - Bug Fixed: “Only three SCSI HDDs are indentified in BIOS Setup when connecting 4 HDDs onto LSI 1010 SCSI card.”

Version 3E01 – Dec. 24, 2002 / Checksum=A6D6h: - As requested by STN, change the System Product Name from “Altos G300 Series” to “Altos G301”.

Version 3D01 – Dec. 18, 2002 / Checksum=B4C7h: - Defect ID 4305 Solved: “Cpu speed post error ‘2.50 GHz , @ 2530 MHz’ when use 2.50G/400 CPU.” - As requested by STN, change the System Product Name from “Altos G301” to “Altos G300 Series”.

Version 3C02 – Dec. 10, 2002 / Checksum=AABAh: - Phase in LSI IDE-RAID BIOS/CTRL-M Version 2.6.12091041I/2.6.1126I: - The Write Cache setting is fixed at "Disabled"; - Use proper way to utilize EBDA to solve Netware installation issue. - Bug Fixed: “Fail to log the System Fan 2 event.” - Change PCI Slot 1 attribute of SMBIOS data from 3.3V to 5V and restore PCI Slot 5 attribute to 3.3V.

Version 3C01 – Dec. 5, 2002 / Checksum=A78Bh: 1. Defect ID 4065 Solved: “Windows NT4.0 installation can't be finished under raid func. Enable.” 2. Phase in LSI IDE-RAID BIOS/CTRL-M Version 2.6.11271727I/2.6.1126I: - Write Cache: No option is fixed at "Disabled"; - DMA: No option is fixed at "Enabled". - As requested by STN, - add Hyper-Threading menu item in Setup; - add CPU Ratio menu item in Setup; - change the Setup string from "Total Processor" to "CPU Socket 1 Installed"; - replace the Setup string "Mouse"/"Keyboard" with "Mouse (PS/2, USB)"/"Keyboard (PS/2, USB)"; - add “Front Side Bus Speed” in System Information of Setup; - change the Setup string “LSI Software RAID” to “IDE Software RAID”; - change the PCI Slot 5 attribute of SMBIOS data from 3.3V to 5V. - add Memory Test menu item in Setup. - Bug Fixed: “Wrong L2 Cache size was displayed in System Information of Setup for the Northwood-core Celeron processor.”

Version 2B02 – Nov. 18, 2002 / Checksum=4C31h: - Modify SSID from 0025h to 0029h. To modify the SSID of 82540, please update the EEPROM contents. - Remove the barcode <F9> functionality during POST. - Restore the name of all fans to original design.

Version 2B01 – Nov. 4, 2002 / Checksum=2F96h: - Turn off system failed LED when case closed. Note that system failed LED remains ON if any other event occurs. - Fix PXE could not boot when insert a 2 channel LSI SCSI adaptor. - Update CPU micro code. - Modify SSID and SVID to “0025” and “1025” in CSB6 and VGA. To modify the SSID and SVID of 82540, please update the EEPROM of onboard LAN. - Fix memory reduction does not work in some memory configuration. - Fix a “?” character appear in 3rd Master in system summary menu. - Update CPU shut down temperature.

Version 2A03 – 10/14/2002 / Checksum=9B6Eh: - Support FSB 400 CPU. - Support hyper-threading CPU.

Version 2A02: - Modify product name as “G300” - Allow 24 characters serial number in product information. - Monitor 3 Fan statuses in following location: LM81A(0x58) Fan_2=System Fan 1(Front FAN) LM81B(0x5A) Fan_1=System Fan 2, Fan_2=CPU FAN

Version 2A01: First release for T13A FSB533. - Modify product name as “G301”, baseboard name as “T13A FSB533” - Support GCSL 2.2, CSB6 1.2 - Install windows 2000 without 3rd IDE driver. - Enable bus master and DMA for 3rd IDE channel. - Modify H.W.M. Fan location according to HW specification. The BIOS will monitor 4 Fan statuses. - Support LSI Mega RAID over CSB6 1.2. Now could install ACPI mode Windows 2000 on RAID.