UPSD BIOS ENGINEERING

Intel Corporation, 5200 NE Elam Young Parkway
Hillsboro, OR  97124-6497

DATE:	Nov 20, 2006

PRODUCT:	Nob Hill Server Custom BIOS

### Build 1334 (Release Candidate)

About This BIOS:

- Nov 20, 2006

- NH72310J.86B.1334.2006.1119.2030

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB, M34F6404.PDB, M04F6507.PDB.

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed issue where recovery was not working.

Known Errata:

For boards with BIOS build between 0674 and 1178, you must first flash to BIOS build 0674 and then flash to 1178 or newer BIOS otherwise the setup will display incorrect information.

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1333 (Release Candidate)

About This BIOS:

- Nov 17, 2006

- NH72310J.86B.1333.2006.1116.1638

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB, M34F6404.PDB, M04F6507.PDB.

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Added microcode F65.

- Fixed ITK workspace issue and WinPE flash (with ITK .BIO) issue. (Tibet 978593)

- Fixed AMCC 3ware 9500S OpRom does not display during POS on PCI-X slot. (Tibet 975980)

- Fixed AMCC 3ware 9550SX Oprom does not display during POS on PCI-X slot.

- Added support for Windows Vista SLP 2.0 for standard product.

- Added fix Hot Lava Hifn 6 Port Card NIC Card hang on system boot.

- Improved PCI OpRom memory map size determination method.

- Fixed slow recovery which take 60-120 seconds.

- Fixed issue where some CompactFlash IDE adapters are not detected by the system BIOS.

- Fixed issue where USB 2.0 speed was not working under BIOS USB legacy support.

- Added support for duplicating SLP string to multiple location with a single BIOS.

- Fixed dual display of message pop-up box in Setup.

- Fixed the key that directly go to set a new password when user not key in any old password.

- Fixed Key Help alignment & format in setup.

- Fixed Discard Changes pop-up message box display in setup.

- Fixed illegal pop-up message box display in setup.

- Fixed Setup Browser would not refresh with F9 default Language.

- Fixed issue where Iflash, reboot method (/rb), would fail to update the BIOS ID.

- Fixed intermittent system hang at POST code 92.

- Fixed consecutive read and/or writes of password supplied secure variables.

- Fixed a problem where the variable update would fail. Symptoms seen is that the system would hang at POST code 5Ah.

- Fixed microcode block being corrupted after BIOS update.

- Fixed “CMOS Timer Not Set” event has incorrect event count and incorrect time stamps.

- Fix for Intel® Integrator Toolkit.

- Fixed issue where Default Modules were not getting loaded upon Iflash /md switch.

- Added Thermal Diode support for new processors.

- Added feature to allow changing USB flash drive DOS emulation from BIOS SETUP.

- Added workaround for issue where IDER CD-ROM device was intermittently not being detected.

- Clear wake event status from PCIe Root Port registers.

- Fixed issue where system may hang in DOS accessing USB flash drives.

- Fixed POST delay at POST code 5A when certain CD-ROM drives are attached to the system.

- Fixed issue where a device behind multiple bridges (2+) will get an invalid IRQ assignment.

- Fixed issue where system would hang with certain third-party IDE controllers installed in the system.

- Changed the code to not reset video mode for every OPROM.

- Fixed SMBIOS Type 16 Memory Error Correction reporting.

- Added code to prevent programming of the remap registers on Lakeport RVP.

- Added support for IDCC Version 2.1.00.

- Fixed the issue where IDCC will not launch if FSB margins set higher than 10%

- Added SMBIOS Processor Family for Core(TM)2 processor family.

- Added CPU voltage override support for new CPUs.

- Added a new feature for System Wake from S5.

- Protect the AC Power Lost status bit during the early init to avoid messing up the boot path.

- Fixed Sankyo ATM card reader causing POST delay when USB Legacy ”Enabled”.

Known Errata:

For boards with BIOS build between 0674 and 1178, you must first flash to BIOS build 0674 and then flash to 1178 or newer BIOS otherwise the setup will display incorrect information.

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1297 (Release Candidate)

About This BIOS:

- Aug 24, 2006

- NH72310J.86B.1297.2006.0823.1733

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB, M34F6404.

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed for Remote Management card appears to hang system with NH1104 (Tibet 964886).

- Updated new logo from EPSD.

- Removed OSB support.

Known Errata:

For boards with BIOS build between 0674 and 1178, you must first flash to BIOS build 0674 and then flash to 1178 or newer BIOS otherwise the setup will display incorrect information.

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1179 (Release Candidate)

About This BIOS:

- May 09, 2006

- NH72310J.86B.1179.2006.0508.2328

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB, M34F6404.

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed to block system recovery to other BIOS.

- Fixed a problem where the microcode updates did not work if the flash update was using no reboot method.

- Fixed BIOS Version update failure during Iflash update.

Known Errata:

For boards with BIOS build between 0674 and 1178, you must first flash to BIOS build 0674 and then flash to 1178 or newer BIOS otherwise the setup will display incorrect information.

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1178 (Release Candidate)

About This BIOS:

- May 05, 2006

- NH72310J.86B.1178.2006.0505.0909

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB, M34F6404.

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Added microcode M34F6406.

- Fixed hard disk boot order not available in BIOS Setup. (Tibet 963666).

- Fixed dual core cache size is not fully detected. (Tibet 963134)

- Fixed floppy non-functional when Emulex LP10000 installed. (Tibet 962127)

- Fixed USB mouse doesn’t work in SRCS16 Config. (Tibet 951448)

- Fixed SRCS16 keyboard doesn’t work without mouse in OpROM. (Tibet 951447)

- Fixed Adapter 39320 hangs in legacy slots. (Tibet 962111)

- Fixed system fans run at high speed only with Presler Processor installed. (Tibet 963133).

- Fixed SW single processor mode description wrong in BIOS. (Tibet 960955)

- Added MPS Table support for SCO OSR 5.0.7 with SMP for LC sku. (Tibet 947694)

- Fixed CPU microcode did not get updated for dual core CPU.

- Fixed CPU Speed display in setup main page shows incorrect value.

Known Errata:

For boards with BIOS build between 0674 and 1178, you must first flash to BIOS build 0674 and then flash to 1178 or newer BIOS otherwise the setup will display incorrect information.

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1104 (Release Candidate)

About This BIOS:

- March 22, 2006

- NH72310J.86B.1104.2006.0322.1002

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Removed Beta microcode M07F6005.PDB (Tibet 961713).

Known Errata:

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1103 (Release Candidate)

About This BIOS:

- March 21, 2006

- NH72310J.86B.1103.2006.0321.1535

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB,M07F6005.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed ICH7 SATA RAID 0 array going offline before OS load (Tibet 960479).

- Fixed entering Option Rom setup for SRCSAS18E (Tibet 960648).

Known Errata:

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1063 (Release Candidate)

About This BIOS:

- March 8, 2006

- NH72310J.86B.1063.2006.0308.1033

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,MBDF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A04.PDB,M04F620F.PDB,M9DF4703.PDB,M07F6005.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed System hangs with TCC Post Card.

- Fixed Adaptec 39320A appears to Hang in Option Rom both Slot 5 and 6 (LX).

- Fixed LSI 22320R Appears to have problem in Slot 5 only with Slot okay.

- Fixed Emulex LP10000 Channel 0 Not ready issue (Tibet 959000).

- Qlogic 2360 will boot with updated Qlogic Option Rom (Tibet 949927).

- Updated Microcode (Tibet 958833). If microcode is not loaded after changing processors you must reflash the BIOS with the new processor. Use “Iflash /rb /pf NH1063C.bio” at the dos prompt.

Known Errata:

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

- USB mouse will not work in LSI based Raid option rom setup without Ps2 mouse when CDROM is attached.

- May experience problems when an ATI based Add in Video card is used.

### Build 1026 (Release Candidate)

About This BIOS:

- February 14, 2006

- NH72310J.86B.1026.2006.0214.1033

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,M9DF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A03.PDB,M04F620D.PDB,M9DF4703.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed SRCS16 Option Rom setup Keyboard doesn’t work without mouse (Tibet 951447).

- Fixed SRCS16 Option rom setup Usb mouse doesn’t work (Tibet 951448).

- Fixed Hard drives not available when SRCU42E is in slot 4 (Tibet 957438).

- Fixed PS2 mouse in LSI320-1 Option rom Setup (Tibet 951659).

- Fixed Console redirection hangs exiting setup on LC (Tibet 955659).

- Fixed System hangs with TCC Post Card.

Known Errata:

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- Adaptec 39320A appears to Hang in Option Rom both Slot 5 and 6 (LX).

- LSI 22320R Appears to have problem in Slot 5 only with Slot okay.

- LSI 320-1 No Usb mouse in WebBios setup. Firmware 1L37 Bios G119, PS2 mouse good Tibet 951659.

### Build 0962 (Release Candidate)

About This BIOS:

- January 13, 2006

- NH72310J.86B.0962.2006.0113.1117

- SATA RAID info:

  - LSI RAID for SATA - v5.4.11101321I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB,M9DF4117.PDB,M9DF4305.PDB,M9DF4406.PDB,MBDF4903.PDB,M5CF4A03.PDB,M04F620D.PDB,M9DF4703.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Changed Console redirection to the Onboard serial port baud rate to 57600.

- Added SMBIOS Type15 structures for LanDesk (Tibet 943272).

- Added additional Help Text to Setup screens (Tibet 944387).

- Added separate Network Boot options for PCIe Lan and PCI Lan Controllers (Tibet 944389).

- Fixed entry into SRCU41L and ARCS16 Option ROM (Tibet 945330, 951442).

- Added Timeout waiting for keyboard input after BIOS messages, i.e. Chassis Intrusion (Tibet 945778).

- Added automatic Disable of On board ATI Video Controller when Add-in Video device installed (Tibet 946126).

- Added support for additional processors (Tibet 950268).

- Removed Manual Aggressive Setup Option (Tibet 950688).

- Changed Non-Fatal PCIe errors to not cause SERR (Tibet 952830).

- Changed sequence to display Enter Setup prompt causing issues with KVM Keyboard switch (Tibet 953292).

- Removed Setup Option “Manual Control” for memory.

- Removed Setup Boot Option “Advanced”.

- Changed ICH7 RAID Option Rom version.

- Fixed 39320A SCSI Card Device for Channel A operation.

Known Errata:

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

- Sawtell SRCS16 Option Rom setup requires matched mouse and Keyboard (Both PS2 or Both USB).

### Build 0734 (Release Candidate)

About This BIOS:

- November 23, 2005

- NH72310J.86B.0734.2005.1123.1649

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Added Console redirection to the Onboard serial port(Tibet 949048).

- Added processor entries to the MP table (Tibet 947694).

- Corrected APM function GetPowerStatus (Tibet 947695).

- Added USB Ports Enable/Disable (Tibet 946688).

- Fixed Serial Port Disable not disabling the serial port (Tibet 949508).

- Fixed BIOS Boot Order display of some SCSI cards (Tibet 947717).

- Moved location of F2 to enter setup display (Tibet 949057).

- Added Uqi for iTK toolkit not working (Tibet 950623).

- Added timeout for message displays waiting for user input (Tibet 945778).

Known Errata:

- Console Redirection of the VGA display not routed to Serial port, You will not see Add-in cards video display.

### Build 0674 (Release Candidate)

About This BIOS:

- October 11, 2005

- NH72310J.86B.0674.2005.1011.1851

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed WOL from S4 and Tabor voltage backfeed to ICH while in S4.

### Build 0663 (Release Candidate)

About This BIOS:

- October 07, 2005

- NH72310J.86B.0663.2005.1007.1516

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Enabled PERR and Serr on PCIx to cause NMI resulting in Blue Screen (Tibet 938470).

- Fixed Issue where 39320A Setup wouldn’t allow configuration or Channel A (Tibet 945271).

- Fixed PME detection on PCIx Bus (Tibet 942001).

- Added Hardware Monitoring setup page to LC Sku (Tibet 946678).

- Fixed Red Hat EL 4.0 Install and Boot problems on LC SKU (Tibet 947711, 947722).

Known Errata:

- XP installation fails without Service Packs (Tibet 944010).

### Build 0626 (Release Candidate)

About This BIOS:

- September 28, 2005

- NH72310J.86B.0626.2005.0928.1934

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Fixed Tabor back driving VCC and VCC3 while in S5 with wake on   Lan enabled (Tibet 947156).

- Set network boot disabled as default (Tibet 946694).

- Set Default Hard Disk pre-delay to 5, enables longer display of BIOS Splash screen (Tibet 946655).

Known Errata:

- XP installation fails without Service Packs (Tibet 944010).

### Build 0615 (Release Candidate)

About This BIOS:

- September 20, 2005

- NH72310J.86B.0615.2005.0920.2142

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

- AMT Firmware version 1.1.36.

- Intel(R) Active Management Technology BIOS Extension v1.2.3.0000

New Fixes/Features:

- Enabled AMT bios extensions (Tibet 947230).

- Fixed 33MHz PCI cards in PCI-X slot hanging system.

Known Errata:

- XP installation fails without Service Packs (Tibet 944010).

### Build 0595 (Release Candidate)

About This BIOS:

- September 15, 2005

- NH72310J.86B.0595.2005.0915.2157

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Fixed issue where addin cards did not work in PCI Slot 2 in Manufacturing Mode (Tibet 946424).

- Corrected SMBIOS System Slots Information (Type 9) and On Board Devices Information (Type 10) (Tibet 945834).

- Changed minimum fan tach settings for heceta in SIO (Tibet 946657).

Known Errata:

- XP installation fails without Service Packs (Tibet 944010).

- 33MHz PCI cards may not work in PCI-X slot and the system may hang.

### Build 0570 (Beta)

About This BIOS:

- September 13, 2005

- NH72310J.86B.0570.2005.0913.1756

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Corrected SMBIOS System Slots Information (Type 9) and On Board Devices Information (Type 10) (Tibet 945834).

- Changed minimum tach settings for Heceta’s on LC and LX (Tibet 945850).

- Fixed SCRU42X (Chilito2) and SCSI ICPVotex cards entering option rom setup (Tibet 946111 and 946135)

- Fixed Windows 2003 Adaptec 39160 in PCI-X riser Card (Tibet 938814).

- Fixed Windows 2003 finding hard drives in LX Slots 5 and 6 (Tibet 944009).

- Fixed Display of Adaptec cards in Slots 5 and 6 (Tibet 945271).

- Fixed Display of Adaptec ASR-2110S (Phantom) card in Boot menu Normal mode (Tibet 946134).

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- XP installation fails without Service Packs (Tibet 944010).

- 33MHz PCI cards may not work in PCI-X slot and the system may hang.

### Build 0525 (Beta)

About This BIOS:

- September 2, 2005

- NH72310J.86B.0525.2005.0902.1536

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Enabled PCI-X/PXH on NobHill LX platforms.

- Fixed issue where the BIOS did not match Silk Screen for memory on FAB B board (Tibet 941994).

- Fixed no video on first boot at Functional Test after In Circuit Testing (Tibet 946141).

- Changed manufacturing default for PCIe Compliance to Enable.

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- XP installation fails without Service Packs (Tibet 944010).

- 33MHz PCI cards may not work in PCI-X slot and the system may hang.

### Build 0497C (Release Candidate)

About This BIOS:

- August 24, 2005

- NH72310J.86B.0497.2005.0824.1452

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Fixed low throughput on PCI bus. (Tibet 944096).

- Fixed issue where Windows installation or boot hung with Intel Chilito-2 (SCRU42X) RAID card. (Tibet 944008).

- Removed Hardware Monitoring page from the BIOS Setup (Tibet 945441).

- Fixed WHQL ACPI Verifier failure on HCT 12.1.01 Windows 2003 SP1 32bit (Tibet 944497).

- Changed Product Guide message when no boot devices found in the system from "Desktop" to "Server" (Tibet 944402).

- Allowed Ctrl+Alt+Del when no boot devices detected in the system (Tibet 944402).

- Enabled Pulse Width Stretching for 3-wire fan headers.

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- XP installation fails without Service Packs (Tibet 944010).

### Build 0489C (Release Candidate)

About This BIOS:

- August 18, 2005

- NH72310J.86B.0489.2005.0818.1716

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tabor)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Fixed PXE boot via Tabor LAN (Tibet 945190).

- Added support for PERR/SERR events detection and logging (Tibet 942037).

- Fixed reading of dowling fan tachs at manufacturing (Tibet 944104).

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- Unable to install Win2003 to Chilito-2 on NobHill Fab-B (Tibet 944008).

- XP installation fails without Service Packs (Tibet 944010).

### Build 0467C (Release Candidate)

About This BIOS:

- August 15, 2005

- NH72310J.86B.0467.2005.0815.1849

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent FE v4.1.17 (Ekron)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Fixed problem with QuickPost post card hang at checkpoint 58.

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- Unable to install Win2003 to Chilito-2 on NobHill Fab-B (Tibet 944008).

- XP installation fails without Service Packs (Tibet 944010).

- Perr Serr event detection and logging

### Build 0462C (Release Candidate)

About This BIOS:

- August 12, 2005

- NH72310J.86B.0462.2005.0812.1558

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent FE v4.1.17 (Ekron)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Changed Subsystem Vendor Id of onboard ATI RN50 video controller to 0x8086 (Tibet 943983).

- Added support for RMC POST complete signal (Tibet 940161).

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- Unable to install Win2003 to Chilito-2 on NobHill Fab-B (Tibet 944008).

- XP installation fails without Service Packs (Tibet 944010).

### Build 0454C (Release Candidate)

About This BIOS:

- August 11, 2005

- NH72310J.86B.0454.2005.0811.1712

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent FE v4.1.17 (Ekron)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.2.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Fixed issue where the device could not install correct driver (Tibet 943976).

- Added BBS Popup Menu (Boot Menu), invoked with <ESC> hotkey during POST (Tibet 943741).

- Removed references to "Desktop" from the BIOS Setup (Tibet 943740).

- Added support for PERR/SERR events detection and logging (Tibet 942037).

- Fixed issue where Fibre Channel cards Qlogic 2362 and Emulex 10000ex did not show up in Setup boot order (Tibet 942553).

- Fixed issue where sometimes ECC errors were not detected and logged (Tibet 941993).

- Updated Memory Reference Code to version 1.2 (Tibet 943674).

- Changed manufacturing default of the "Primary Video Adaptor" setup option to External PCI Graphics (Tibet 940719).

- Changed spread mode of the clock to down spread (Tibet 944455).

- Added workaround to fix boot hangs with locked CPU at 1066MHz FSB (Tibet 944106).

- Fixed issue where unused PCI slot clocks were not turned off (Tibet 938808).

- Turned off unused PCIE ports (Ports 2, 3 and 4) in the South Bridge.

- Changed defaults of “USB Boot” setup option to Enable.

- Moved “ZIP Emulation Type” setup option to Boot menu and changed its defaults to Floppy.

- Changed defaults of “Wake on LAN from S5” option to Stay Off.

- Fixed EIST setup option based on processor capability.

- Changed defaults of "Chassis Intrusion" setup option to Enable.

- Fixed SATA Configuration setup option based on ICH features capability.

Known Errata:

- Can't pass MVL 4-corner when testing 4GB & 8GB of total memory with 2GB parts (Tibet 942555).

- Cert-WHQL Invalid vendor ID for ATI ES1000 (Tibet 943983).

- Unable to install Win2003 to Chilito-2 on NobHill Fab-B (Tibet 944008).

- XP installation fails without Service Packs (Tibet 944010).

- RMC POST complete signal failure (Tibet 940161).

### Build 0444B (Beta)

About This BIOS:

- August 5, 2005

- NH72310J.86B.0444.2005.0805.1021

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent FE v4.1.17 (Ekron)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.0.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Added 2nd onboard LAN enable/disable option in Setup (Tibet 936819).

- Fixed issue where unused PCI slot clocks were not turned off (Tibet 938808).

- Fixed Windows XP installation hang problem at PCI.SYS (Tibet 944010).

- Added fan control support (Tibet 943246).

- Removed “ACPI Suspend State” option from the BIOS Setup.

Known Errata:

- PCI Slot 2 is off unless there is a card in PCI Slot 1.

- Tabor Lan is disabled on shutdown restart.

### Build 0400B (Beta)

About This BIOS:

- July 28, 2005

- NH72310J.86B.0400.2005.0728.1802

- SATA RAID info:

  - LSI RAID for SATA - v5.4.05262138I

- Initializing Intel(R) Boot Agent FE v4.1.17 (Ekron)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.0.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB, M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- LSI 22320, LSI Megaraid SATA 150-6 fixes for multiple CV/DV configurations (Tibet 942505, 942507).

- New GPIO Programming for Tabor enable disable.

- Linux install of RHLE with 4 SATA Drives (Tibet 942510).

- Fixed enter of Intel SRCU42L-Chilito OPROM Setup (Tibet 942554).

- Fixed SMBIOS tables in BIOS (Tibet 942588).

- Fixed Event logs display of wrong date/year (Tibet 939495).

- ECC is set as default (Tibet 941995).

- BIOS Suspend state defaults to S1 (Tibet 943226).

- ASF removed from BIOS Setup (Tibet 943219).

- Front panel lights blink in standby (Tibet 936104).

Known Errata:

- Tabor Lan is disabled on shutdown restart.

### Build 0287 (Beta)

About This BIOS:

- July 8, 2005

- NH72310J.86B.0287.2005.0708.1725

- SATA RAID info:

  - LSI RAID for SATA - v5.4.052621381

- Initializing Intel(R) Boot Agent FE v4.1.17 (Ekron)

- Initializing Intel(R) Boot Agent GE v1.2.28 (Tekoa)

- Memory reference code equivalent to MRC 1.0.

- CPU Patches: M1DF3417.PDB, MBDF4117.PDB,M9DF4305.PDB, M9DF4406.PDB

New Fixes/Features:

- Mukilteo (7230) Chipset support

- System bus support 1066/800/533

- Memory support DDR2 667/533 up to 8 GB

- Added new Server logo 800X600 BMP

- Fixed issues with LSI Mega Raid card

  - Properly detected during boot

  - Yellow bangs in device manager

- Fixed issue where system would hang when shutting down from windows.

- New GPIO Programming for Tabor enable disable.

Known Errata:

- Tabor Lan is disabled on shutdown restart.