| f | TITLE |
| --- | --- |

| DATE: | July 31, 2026 |
| --- | --- |
| TO: |  |
| SUBJECT: | 5DXSL BIOS Release Notes version P06 |

# About This Release

Build Date	: December 26, 2008

# BIOS Components/Contents

Processor stepping(s) supported: supports Intel® Pentium(R), Conroe, Core(TM) 2 Duo, Core(TM)2 Extreme, Kentsfield series processors

System hardware configurations supported: 5DXSL-RH Rev: 1.0

Microcode updates versions:

| ###### CPUID | ###### Stepping | ###### Microcode ###### Update ID |
| --- | --- | --- |
| 06F2 | L2 (Conroe) | 56 |
| 06F6 | B2 (Conroe) | C6 |
| 06FB | G0 (Conroe) | B6 |
| 06FD | M0 (Conroe) | A3 |
| 06FB | G0 (Kentsfield) | B6 |
| 10661 | A1 (Conroe-L) | 32 |
| 10676 | C0 (Wolfdale) | 60B |
| 10676 | C0 (YorkField) | 60B |
| 10677 | C1/M1 (YorkField) | 703 |
| 1067A | E0 (Wolfdale) | A07 |
| 1067A | E0/R0 (YorkField) | A07 |

OPROM update versions:

| ###### OPROM | ###### Version |
| --- | --- |
| A0 VBIOS | 01.08.07 |
| A1 VBIOS | 01.09.10 |
| AHCI | 01.05.80 without Port multiplier support |
| LSI SATA RAID | A.05.10121518R |
| PXE | 10.4.6 |

MRC version:

| ###### MRC | ###### Version |
| --- | --- |
| MRC | 1.41 |

# Installation Notes

IMPORTANT NOTES:

- Please extract the 5DXSL_P06.zip to a bootable diskette that can boot to real DOS

- Please don’t load HIMEM.SYS or any other memory management SW in the diskette

BIOS UPDATE INSTRUCTIONS:

- Insert bios update diskette to floppy drive

- Power on system and wait for boot into DOS

- Execute F.BAT for bios update

- After bios flash finish, it’ll display a message

- Press a key then system will reboot automatically

BIOS Version CHECK INSTRUCTIONS:

- Power on system and press F2 during POST

- Enter CMOS setup utility and go to Server\System Management

- The bios version shows on the first item

# Known Issues/Workarounds

# Features added in this release

# Features removed in this release

# Issues fixed in this version

- Fix: PCI slot information is lost (PCI IRQ table issue) with DTM solution.

- Fix: ATI VGA HD 3450 can’t display.

# How to make LOGO data

# LOGO update instruction