# Workaround for installing NW5.1 SP6+ on systems with ICH5R chipset

Purpose:  NW5.1 OS Installer is unable to see the on-board ICH5R SATA controller. Hence, a workaround was devised to install the OS on SATA drives connected to this controller.

Workaround:

- The install program prompts the user for the Slot No when the storage driver “Megaride.ham” is installed from a floppy drive or some other location.

- The slot no is found by loading the driver on the system console (for instance, if the driver is being loaded from the floppy drive, type a:megaride at the system console) and then choose "No Selection", to avoid loading the driver.

- The slot number is now entered in the prompt and the megaride driver loads successfully.

- The Ideata.ham driver on NW5.1 SP7 does not work with the on-board CD-ROM drive. So, the user should install using the latest Ideata.ham. This driver can be copied from NW6.0 or NW6.5 install CD.

- Ensure, the next screen shows “Idehd” and “Idecd” as device drivers for the storage devices.

- Continue the installation as usual.

Note: On rebooting the system, it has been noticed on Intel Grand Prairie platform, the slot number for the onboard ICH5R SATA controller changes. The system will prompt the user with the correct slot no, hit enter to accept it and modify the slot no for megaride.ham in the “startup.ncf”.

Editing startup.ncf: Startup.ncf file can be modified through the nwconfig utility. Type “nwconfig” on the system console to start it. Select nwconfig -> NCF files Options -> Edit startup.ncf. Change the line “LOAD MEGARIDE.HAM SLOT=xxx” to “LOAD MEGARIDE.HAM SLOT=yyy”. Hit F10 to save the file. Hit “Esc” twice to quit “nwconfig”.

Impact: This workaround is for NW5.1 OS installation only. NW6.0 and NW6.5 do not require this workaround for OS installation. It has no performance or functionality impact. Further this workaround could be revised with Novell’s input in the future.