Tony Lee	Apr. 22st, 2005

Acer Sirius
 BIOS Release Document

Version:

Release: 	D1

| ###### Change ID | ###### Detailed Change Description | ###### Reason |
| --- | --- | --- |
| # D1 | Initial revision. | Date: 04/22/2005 Implement: 1. Boot device selection menu w/ F12 hotkey 2. Hot-Key F10 to load BIOS default at/before POST 3. Support Processor(s) type , CPU Frequency (including Internal and external bus frequency)., CPU ID, CPU Stepping, CPU Voltage auto detection 4. ACPI –aware OS [Disabled] or [Enabled] support in BIOS Setup 5. Power Button Mode support for [Instant On/Off] 6. S.M.A.R.T support in BIOS Setup [Auto], [Disabled], [Enabled] 7. Memory Configuration support in BIOS Setup 8. Manufacture Name in BIOS Setup or SMBIOS should be all “Acer” 9. IMPI DEVICE INFORMATION (TYPE 38) 10. Support UUID in SMBIOS Use LAN MAC address for UUID |
| # D2 |  | Date: 04/29/2005 Implement: - Change Setup Menu OnBoardLan to OnBoardNIC - Add Clear IPMI SEL - Add IPMI LogFull - Add Send MAC Address to BMC. - Add Logo.(But not correctly) - Add Power NOW. |
| # D3 |  | Date: 05/06/2005 Implement: - POST ECC log in event log (multi-bit error) - Chagne Setup Menu Style - Display processor type, frequency, and cache size related information during POST if quiet boot is disabled |
| # D4 |  | Date: 05/13/2005 Implement: - Fan Speed Control - Memory single bit ECC check - CPU 1 (present: show, no present, no show), CPU2 (present: show, no present, no show), - Add UUID in setup menu. - Add Serial No. in setup menu. - Fixed setup menu. |
| # D5 | Checksum:9ADC | Date: 05/20/2005 Implement: 1. Display correct memory size in the BIOS Setup. 2. Remove CPU 2 information from BIOS Setup if only one CPU exists. 3. Remove DIMM 5 - DIMM 8 information from BIOS Setup if only one CPU exists. 4. Support HostRAID feature. 5. PXE BIOS default setting is disabled, SCSI BIOS default setting is enabled. 6. Fix the bug that system can't install Windows Server 2003. |
| # D6 | Checksum:67E0 | Date: 05/26/2005 Implement: - Enable/disable the NMI assert feature - Enable/disable add-on card's I/O ROM - Display full CPU string, don't be truncated by scroll bar - Display the help messages for PowerNow and NX Support - The options of IDE type should match with the help messages - The options of COM port and parallel port should match with the help messages - The mode options of parallel port should match with the help messages, - Display the correct L1 and L2 cache information for dual core CPU - Fixed “After configuring MegaRAID card, cannot press Ctrl+Alt+Del to reboot” - Fixed “Without BMC module, BIOS menu shows FRU Revision.” - Fixed “informatiom in Device Manage show 'unknow' and only two CPU.” - Fixed “BIOS post error: System CMOS checksum bad” - Fixed “Processor Type only shows 'AMD Athlon(tm) or Opteron(tm)' for dual core CPU” |
| # P00 | D6 rename to P00 |  |
| # P00c | Checksum:A2C8 | Date: 06/02/2005 Implement: 1. Can't display Acer logo during POST. 2. Can't distinguish single core CPU from dual core CPU, the BIOS Setup displays 128KB X 2 for L1 cache and 1024KB X 2 for L2 cache if single core CPU installed. 3. The help message of Power Now exceeds the boundary. 4. The help message "OS Controlled" of serial port A and B still exists. |
| # P01 | P00c rename to P01 Checksum:CA56 |  |
| # P01.03 | Checksum:E90D | Date: 06/02/2005 Implement: - Fix Sys Fan1 2000 RPM can’t detect. - Fix Lpt can’t display in windows. - Remove the NX Support item from - Display full UUID information in the BIOS Setup - Remove or modify POST messages when quiet boot is disabled |
| # P01.06 | Checksum:E023 | Date: 06/17/2005 Implement: 1. UID string not full display at BIOS set up screen. 2. BIOS version only show P01 not P01.03 3. SDR version always show “SIRIUS SDR002” not 0.02 4. Change fan speed control to 62℃ & down at 57℃ & re-test thermal result. 5. While Enable Node interleave , the setup menu will show DIMM not install. 6. Memory error post wrong DIMM location..... |
| # P02 | Checksum:F3B8 | Rename P01.06 to P02. |
| # P02.01 | Checksum:BFFC | Date: 06/22/2005 Implement: 1. There is a garbage character in the BIOS version of BIOS Setup. 2. The SDR version is always 0.02, please check this issue with Marco. 3. The boot device string issue |
| # P03 | Checksum:A893 | Rename P02.01 to P03 |
| # P03.01 | Checksum:EFEF | Date: 06/29/2005 Implement: - Remove the H/W monitoring page from BIOS Setup. - Remove "Dual Core", but add "model number" for dual core CPU. 3.Change the default setting of printer port to ECP mode. |
| # P03.02 | Checksum:33EE | Date: 07/08/2005 Implement: - Display full CPU string(two lines) in the BIOS Setup if dual core CPU installed. - Support 8GB transfer rate for Hyper Transport. - Hide "Event Log Configuration" if the system has no BMC module. - Hide "Onboard SCSI ROM" if the system has no SCSI module. - Add the SRAT option into BIOS Setup. - Found the root cause which the system will reboot if press Ctrl+H to enter LSI MegaRAID BIOS Setup. |
| # P03.03 | Checksum:31E4 | Date: 07/12/2005 Implement: 1.The BIOS Setup doesn't display the correct CPU information if two different CPUs installed. 2. Add a new option to enable/disable the A/C lost log feature. 3. Send the command to BMC to record the event log if the memory ECC error occurs. 4. Send the command to BMC to record the event log if the POST error occurs. 5. Provide the handy drive samples for GBT BIOS team.Support handy drive to be drive A. |
| # P03.04 | Checksum:42A5 | Date: 07/15/2005 Implement: - The uncorrectable ECC event log will be recorded if discharge CMOS then boot up the system. - Support 8GB transfer rate for Hyper Transport. |
| # P03.05 | Checksum:73C4 | Date: 07/22/2005 Implement: - Pause the system during POST if two different CPUs installed. - The BIOS Setup doesn't display the correct CPU information if two different CPUs installed. - Pause the system during POST if the event log buffer is full. - Remove the SRAT option from BIOS Setup. |
| # P03.06 | Checksum:0372 | Date: 07/29/2005 Implement: - Change the detect different CPUs code function. - Fixed phlash16 doesn’t protect dmi information issue. - Implement ACPI PSS control method for PowerNow feature. - Pause the system during POST if the event log buffer is full. |
| # P03.07 | Checksum:1BCB | Date: 08/09/2005 Implement: - Fixed Mixer CPU issue. 2. Modify SMBIOS type 17. 3. Modify Some DMI Information. 4. Modify Power Now enable/disable function. 5. Modify SRAT depend on NodeInterleave function. |
| # P03.08 | Checksum:7F23 | Date: 08/11/2005 Implement: - Change the option string from "ACPI PSS Table" to "Power Now". - Rreport the actual slot status, available or in use, in the SMBIOS, does not report unknown for slot status. - Fixed the DIMM location in the SMBIOS does not match the BIOS Setup information. 4. In SMBIOS type 3, BIOS does not reserve the buffer, space characters, for version and serial number, for asset tab number, please reserve 32 space characters for it. 5. Fixed the BIOS version in the SMBIOS is incorrect. 6. Fixed the HyperTransport speed for memory is downgraded to 600MHz, it should 800MHz, and the HyperTransport speed for CPU is downgraded to 800MHz, it should be 1000MHz. 7. Remove FRU setup menu item. |
| # P03.09 | Checksum:0C18 | Date: 08/25/2005 Implement: - The SDR version is incorrect in the BIOS Setup. 2. Change the device locator(SMBIOS type 17) to be same as BIOS Setup. S0 -> DIMM 1 S1 -> DIMM 2 S2 -> DIMM 3 S3 -> DIMM 4 S4 -> DIMM 5 S5 -> DIMM 6 S6 -> DIMM 7 S7 -> DIMM 8 3.Fill in the correct memory speed 400MHz or 333MHz in the SMBIOS type 17 4. Should SMBIOS type 15(system event log) be removed because POST error and ECC error are recorded by BMC, these two event logs won't be recorded by BIOS. |
| # P03.10 | Checksum:8205 | Date: 09/07/2005 Implement: 1. Keep DMI type 15. 2. Don't erase DMI type 1 information when update new BIOS. 3. Add the hint message during POST for BIOS setup utility. 4. Change the sensor number for ECC and POST error event logs. |
| # P03.11 | Checksum:8E0B | Date: 09/21/2005 Implement: 1. Modify Bios and Bmc protocol |
| # P04 | Checksum:6492 | Date: 09/30/2005 Rename from P0311 |
| # P0401 | Checksum:E5FF | Date: 09/30/2005 Implement: - Add Redundant Fan function. - Fixed LSI SCSI Card PS/2 issue |
| # P0402 | Checksum:1BA0 | Date: 11/05/2005 Implement: - Modify redundant fan feature. - Modify Power Fan 1 speed. |
| # P0403 | Checksum:9B6C | Rename p0402 to p0403 |
| # P0404 | Checksum:7061 | Date: 11/07/2005 Implement: 1.Modify some code to help IPMI fix the hdd event log issue. |
| # P0405 | Checksum:84b6 | Date: 1115/2005 Implement: 1. Fixed boot to Win2003 and boot by handy drive hang up issue which base on P0404 solution. |
| # P05 | Checksum:A84E | Date: 11/24/2005 Implement: 1. Rename from P04.05 to P05 |