|  | TITLE |
| --- | --- |

| DATE: | July 31, 2026 |
| --- | --- |
| TO: |  |
| SUBJECT: | ACER 7BESH BIOS Release Notes version P20 |

# About This Release

Build Date	: July 31, 2026

# BIOS Components/Contents

Processor stepping(s) supported: supports Intel® Dempsey series processors

System hardware configurations supported: 7BESH Rev: 1.1 & 2.1

Microcode update versions:

| ###### CPUID | ###### Stepping | ###### Microcode ###### Update ID |
| --- | --- | --- |
| F63 | B2 | 05 |
| F64 | C0 | 02 |
| F64 | C1 | 02 |
| 6F5 | B1 | 33 |
| 6F6 | B2 | C9 |
| 6FB | G0 | B4 |
| F65 | D0 | 08 |
| 6F5(C) | B1 | 33 |
| 6F7(C) | B3 | 67 |
| 6FB(C) | G0 | B4 |
| 10671(H) | A1 | 106 |
| 10671(W) | A1 | 106 |
| 676(H) | C0 | 606 |
| 676(W) | C0 | 606 |
| 674(W) | B1 | 404 |
| 67A(H)(Wolf) | E0 | A07 |

OPROM update version :

| ###### OPROM | ###### Version |
| --- | --- |
| VGA | VER008.005.023.000 |
| AHCI | 1.07 |
| Intel SATA RAID | v5.6.2.1002 |
| SAS | v0105171109 |
| PXE | v1.2.36 |

# Installation Notes

IMPORTANT NOTES:

- Please extract the ACER_G540_P20.zip to a bootable diskette that can boot to real DOS

- Please don’t load HIMEM.SYS or any other memory management SW in the diskette

BIOS UPDATE INSTRUCTIONS:

- Insert bios update diskette to floppy drive

- Power on system and wait for boot into DOS

- Execute F.BAT for bios update

- After bios flash finish, it’ll display a message

- Press a key then system will reboot automatically

BIOS Version CHECK INSTRUCTIONS:

- Power on system and press F2 during POST

- Enter CMOS setup utility and go to Server\System Management

- The bios version shows on the first item

# Known Issues/Workarounds

# Features added in this release

# Features removed in this release

# Issues fixed in this version

- Fixed some Harpertown CPU can't show "Execute Disable Bit" item under BIOS setup    menu. (CPU CPUID:1067A)

# How to make LOGO data

Not support yet

# LOGO udpate instruction

Not support yet