|  | TITLE |
| --- | --- |

| DATE: | July 29, 2026 |
| --- | --- |
| TO: |  |
| SUBJECT: | 3CESL BIOS Release Notes version P10 |

# About This Release

Build Date	: May 24, 2007

# BIOS Components/Contents

Processor stepping(s) supported: supports AMD® Dual processors

System hardware configurations supported: 3CESL Rev: 1.3

Microcode update versions:

| ###### CPUID | ###### Stepping | ###### Microcode ###### Update ID |
| --- | --- | --- |
| 0F11 | F1 | 62 |
| 0F12 | F2 | 62 |
| 0F13 | F3 | 62 |
| 0F32 | F2 | 62 |
| 0F33 | F3 | 62 |
| 0F20 | B0 | 33 |
| 0F21 | B1 | 33 |
| 0F2A | BA | 33 |
| 0F22 | B2 | 33 |

OPROM update version :

| ###### OPROM | ###### Version |
| --- | --- |
| VGA | 1.07.05 |
| NVRAID | 6.7.7 |
| PXE | 244.0538 |

AGESA version :

V3.1.1.0

# Installation Notes

IMPORTANT NOTES:

- Please extract the 3CESL _P10.zip to a bootable diskette that can boot to real DOS

- Please don’t load HIMEM.SYS or any other memory management SW in the diskette

BIOS UPDATE INSTRUCTIONS:

- Insert bios update diskette to floppy drive

- Power on system and wait for boot into DOS

- Execute F.BAT for bios update

- After bios flash finish, it’ll display a message

- Press a key then system will reboot automatically

BIOS Version CHECK INSTRUCTIONS:

- Power on system and press F2 during POST

- Enter CMOS setup utility and go to Server\System Management

- The bios version shows on the first item

# Known Issues/Workarounds

# Features added in this release

# Features removed in this release

# Issues fixed in this version

- To avert when get DQS data from ESCD, the block size is invalid and it occur dead loop in POST_TP=28h

- Correct SVM solution

- Add POST error message for DMI event log area is full

# How to make LOGO data

Not implement

# LOGO udpate instruction

Not implement

# How to update MAC address

If you have mac address 00:14:85:2A:87:FD, please follow below step to update it.

1. Create macgen.txt

2. Input 0014852A87FD at the first line of macgen.txt

3. issue macgen.exe on DOS mode

# How to make BIOS recovery disk

If you want to make BIOS recovery disk, please follow below step to create it.

- Uncompress crisis.zip

- Put a floppy into floppy disk device

- Execute wincris.exe

# How to update GBIA module

If you want to update GBIA module, please follow below step to update it.

- Execute buildbia.exe on DOS mode to create GBIA.BIN

- Execute upgbia.exe on DOS mode to update GBIA module into BIOS rom.